Copyright (C) 1992, 1995 Aladdin Enterprises.  All rights reserved.

This file is part of Aladdin Ghostscript.

Aladdin Ghostscript is distributed with NO WARRANTY OF ANY KIND.  No author

or distributor accepts any responsibility for the consequences of using it,

or for whether it serves any particular purpose or works at all, unless he

or she says so in writing.  Refer to the Aladdin Ghostscript Free Public

License (the "License") for full details.

Every copy of Aladdin Ghostscript must include a copy of the License,

normally in a plain ASCII text file named PUBLIC.  The License grants you

the right to copy, modify and redistribute Aladdin Ghostscript, but only

under certain conditions described in the License.  Among other things, the

License requires that the copyright notice and this notice be preserved on

all copies.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

This file, devices.doc, gives more detailed documentation about

certain specific devices for which Ghostscript can produce output.

For an overview of Ghostscript and a list of the documentation files, see

README.

Devices for which this file currently contains documentation:

SPARCprinter

HP DeskJet 520, 540, and 560C

HP DeskJet 500C & 550C

HP PaintJet, XL, and XL300

DEC LJ250

Apple Dot Matrix Printer (and Imagewriter)

Epson Stylus Color Printer

### ------------------------- The SPARCprinter ------------------------- ###

This section was written by Martin Schulte.

Introduction

------------

The SPARCprinter is is connected to SPARCStation via a special SBUS card's

video inferface, the picture is composed on the host and only a bitmap is

send to the printer unit.

Together with a SPARCprinter, you always buy (as far as I know) software

that enables you to do postscript-printing on your SPARCPrinter.

So, the need for a Ghostscript-Interface to the SPARCPrinter seems low,

but on the other hand some Postscript drawings are not correctly printed

with SUN's software: on some pages occured a thin vertical line of rubbish

(reproducable), on some Mathematica drawings the text at the axes wasn't

rotated.

I tried all of these with Ghostscript and always got the expected results.

However, replacing proprietary software should never be a bad idea.

The problem is that there has yet been no effort to make the SPARCPrinter-

driver behave like a BSD output-filter, I made my tests using the script

mentioned under Installation.

Installation

------------

Add sparc.dev to DEVICE_DEVS and compile ghostscript as described in

make.doc.

Afterwards, you can use the following script (the way of handling standard

input versus filename-arguments doesn't look very clever, has anyone a

better idea ?) to print if you substitute <GSPATH> by the place where you

installed the ghostscript binary:

outcmd1='/vol/local/lib/troff2/psxlate -r'

outcmd2='<GSPATH> -I/home/schulte/gs252 -sDEVICE=sparc -sOUTPUTFILE=/dev/lpvi0 -'

if [ $# -eq 0 ]

then

$outcmd1 | $outcmd2

else

cat $* | $outcmd1 | $outcmd2

fi

Problems

--------

Since /dev/lpvi can only be opened for exclusive use, another job having

opened it (engine_ctl_sparc or another ghostscript as the most probable

canidates) will cause to stop ghostscript with "Error: /invalidfileaccess

in --.outputpage--"

In case of common printer problems like out of paper, a warning describing

the reason will be printed to stdout, the driver will try to access again

and again each five seconds.

Due to a problem with the device-driver (in the kernel) the reason of

printer failure is not always correctly reported to program.  This is the

case at least if you open the top cover (Error in the display: E5).  Look

to the display at the printer if a "Printer problem with unknown reason"

is reported.

Fatal errors will cause the print-job to be terminated.

### ------------------------------ End --------------------------------- ###

### ------------------- H-P color inkjet printers ---------------------- ###

###  (DeskJet 500C, DeskJet 550C, PaintJet, PaintJet XL, PaintJet XL300  ###

###  and the DEC LJ250 which can operate in a Paintjet-compatible mode)  ###

This section was written by George Cameron.

Information and tips on usage for the drivers contained in gdevcdj.c

====================================================================

OVERVIEW:

There are 6 generic drivers contained in the source module:

1 - cdj500:      HP DeskJet 500C and 540C

2 - cdj550:      HP DeskJet 550C and 560C

3 - pjxl300:     HP PaintJet XL300 and DeskJet 1200C

4 - pjtest:      HP PaintJet

5 - pjxltest:    HP PaintJet XL

6 - declj250:    DEC LJ250

All of these drivers have 8-bit (monochrome), 16-bit and 24-bit

(colour) and for the DJ 550C 32-bit, (colour, cmyk mode)

options in addition to standard colour and mono drivers.

It is also possible to set various printer-specific parameters

from the gs command line, eg.

gs -sDEVICE=cdeskjet -dBitsPerPixel=16 -dDepletion=1 -dShingling=2 tiger.ps

NB/ The old names cdeskjet, cdjcolor and cdjmono drivers have been retained;

however, their functionality duplicates that available using the above

drivers (and cdeskjet is identical to cdj500), ie. we can use:

gs -sDEVICE=cdj500 -dBitsPerPixel=24 ...	for cdjcolor, and

gs -sDEVICE=cdj500 -dBitsPerPixel=1  ...	for cdjmono

DEFAULT PAPER SIZE:

If the preprocessor symbol A4 is defined, the default paper size is the

European A4 size; otherwise it is the U.S. letter size (8.5"x11"). Other

paper sizes (including A3 for the PaintJet XL and PaintJet XL300) may be

specified on the command line as explained in the Ghostscript documentation.

DEFAULT BITS-PER-PIXEL:

If the preprocessor symbol BITSPERPIXEL is defined as an integer (see below

for the range of allowable values), this number will be used to define the

default bits-per-pixel (ie. bit depth) for the generic drivers. If the

symbol is not defined, the default is set to 24 bits per pixel. It is

of course still possible to specify the value from the command line, as

described below. Note also that the cdeskjet, cdjcolor and cdjmono

drivers are unaffected by setting this symbol, as their default settings

are predefined to be 1, 3 and 24 respectively.

DESKJET PHYSICAL LIMITS:

Maximum printing width = 2400 dots = 8". The printer manuals say that the

maximum recommended printing height on the page is 10.3", but since this

is obviously not true for A4 paper, and I have been unable to detect any

problems in printing longer page lengths, this would seem to be a rather

artificial restriction.

All Deskjets have 1/2" unprintable bottom margin, due to the mechanical

arrangement used to grab the paper. Side margins are approximately 0.25"

for US Letter paper, and 0.15" for A4.

COMMAND LINE PARAMETERS:

Several printer 'properties' have been implemented for these printers.

Those available so far are all integer quantities, and thus may be

specified as eg.

gs -dBitsPerPixel=32 -dShingling=1 ...

which sets the BitsPerPixel parameter to 32 and the Shingling parameter

to 1.

BITS-PER-PIXEL:

All of the drivers in gdevcdj.c accept a command line option to set the

BitsPerPixel property. This gives considerable flexibility in choosing

various trade-offs between speed/quality/colour etc. The valid numbers

are:

1:	This is a standard Ghostscript monochrome driver, and uses

black ink (by installing the separate mono cartridge in

the case of the DeskJet 500C, or automatically for the

other printers)

3:	A standard Ghostscript colour driver, using internal

dithering. This is fast to compute and to print, but

the clustered dithering can lose some detail and

colour fidelity.

8:	An 'error-diffusion' monochrome driver which uses

Floyd-Steinberg dithering to print greyscale images.

The patterns are much more randomised than with the

normal clustered dithering, but the data files can

be much larger and somewhat slower to print.

16:	This is a 'cheaper' version of the following (24-bit)

driver, which generates a Floyd-Steinberg colour dithered

output using the minimum amount of memory (this may be

helpful when using IBM PC's when Ghostscript has not

been compiled using a 32-bit 386-style compiler). The

quality can be almost as good as the 24-bit version.

24:	A high-quality colour driver using Floyd-Steinberg dithering

for maximum detail and colour range. However it is very

memory intensive and thus can be slow to compute (and it

tends to produce rather larger raw data files, so they

can also be slower to print).

32:	This is for the DeskJet 550C only, which uses the black

cartridge and the colour cartridge simultaneously (ie.

CMYK printing). This printer can be both faster and give

higher quality than the DeskJet 500C, because of the

true black ink. (Note that the 24-bit mode also permits

CMYK printing on this printer, and uses less memory. Any

differences between 24-bit and 32-bit should be very small.)

DESKJET PROPERTIES:

The addional properties available for the DeskJets are:

BlackCorrect	(int)	  /* Colour correction to give

* better blacks when using the DJ500C

* in colour mode, eg. the default of 4

* reduces the cyan component to 4/5

* Range accepted: 0 - 9 (0 = none) */

Shingling	(int)	  /* Interlaced, multi-pass printing

* 0 = none, 1 = 50%, 2 = 25%, 2 is

* best & slowest */

Depletion	(int)	  /* 'Intelligent' dot-removal

* 0 = none, 1 = 25%, 2 = 50%, 1 best

* for graphics?

* Use 0 for transparencies */

PAINTJET XL300/PAINTJET XL PROPERTIES:

PrintQuality	(int)	  /* Mechanical print quality

* -1 = fast, 0 = normal, 1 = presentation

* Fast mode reduces ink usage and uses

* single-pass operation for some media

* types. Presentation uses more ink and

* max number of passes, ie. slowest

* printing for highest quality */

RenderType	(int)	  /* 0 = driver does dithering

* 1 = snap to primaries

* 2 = snap black -> white, others to black

* 3 = ordered dither

* 4 = error diffusion

* 5 = monochrome ordered dither

* 6 = monochrome error diffusion

* 7 = cluster ordered dither

* 8 = monochrome cluster ordered dither

* 9 = user-defined dither (not supported)

* 10 = monochrome user-defined dither ns. */

PAINTJET PROPERTIES:

No additional properties

GAMMA CORRECTION:

One consequence of using Floyd-Steinberg dithering rather than Ghostscript's

default clustered ordered dither is that it is much more obvious that the

ink dots are rather larger on the page than their nominal 1/180" or 1/300"

size (clustering the dots tends to minimise this effect). Thus it is often

the case that the printed result is rather too dark. A simple empirical

correction for this may be achieved by preceding the actual postscript

file to be printed by a short file which effectively sets the gamma for

the device, eg.

gs ... gamma.ps colorpic.ps -c quit

where gamma.ps is

%!

{0.333 exp} dup dup currenttransfer setcolortransfer

This example sets the gamma for r, g, and b to 3, which seems to work

reasonably well in practice.

GENERAL TIPS:

For all the above printers, the paper is critically important to the

final results. Smoother, less fibrous paper is generally better (and

suggested types are given in the printer manuals). In particular, the

special ink-jet paper can make a big difference; the colours are

brighter, but most importantly, there is almost no colour bleed, even

with adjacent areas of very heavy inking. Similarly, the special coated

transparencies also work well (and ordinary transparencies do not work

at all!)

The unix-lpr.sh provides one example of setting up a multi-option

colour postscript lpr queue on Unix systems, and includes the ability

to choose a range of different colour options and printer accounting

and error logging.

CAVEAT EMPTOR!:

It is not always easy for me to test all of these drivers, as the only

colour printer I have here is the DeskJet 500C. I rely on others testing

drivers for the additional machines and reporting their findings back to

me.

HP's 600x300 dpi resolution-enhanced mode for inkjet printers

=============================================================

This feature is available on HP's more recent inkjet printers,

including the Deskjet 520 (mono) 540 (mono or colour) and 560C (mono

and colour).

The colour and monochrome drivers for the HP deskjet 550c are

(probably) the best you will get for use with ghostscript, for the

following reasons:

These printers do not offer true 600x300 dpi resolution. Those that

print in colour are strictly 300x300 dpi in colour mode, while in mono

mode there is a pseudo 600x300 dot mode, with the restriction that you

can't print two adjacent dots. Thus, in effect what you have is 600 dpi

dot positioning, but on average you don't get more dots per line.

What this does give is the possibility to have eg. sharper character

outlines, as you can place dots on the edges nearer to their ideal

positions - this is why it is worth doing.

However, HP will not support user-level programming of this

resolution-enhanced mode, one reason being that (I understand) all the

dot spacing has to be done by the driver, and if you get it wrong, you

can actually damage the print head.

To summarise, you may lose a smidgin of (potential) text clarity using

the 550c drivers (cdj550, cdjcolor, cdjmono etc.), but other than that,

they are the ones for the job.

### ------------------------------ End --------------------------------- ###

### ------------------- Apple Dot Matrix Printer  ---------------------- ###

This section was written by Mark Wedel.

The Dot Matrix Driver (DMP) driver is a simple driver I wrote.  It

could more more efficient, but it seems to print the images fine.

The Dot Matrix Printer was a parallel predecessor to the Imagewriter

printer.  As far as I know, the Imagewriter commands are a superset

to those of the Dot Matrix printer, so the driver should work fine at

generating output that can be printed on Imagewriters.

A few notes (from the gdevadmp.c file):

* To print out images, it sets the printer for unidirection printing

* and 15 cpi (120 dpi). IT sets line feed to 1/9 of an inch (72 dpi).

* When finished, it sets things back to bidirection print, 1/8" line

* feeds, and 12 cpi.  There does not appear to be a way to reset

* things to initial values.

*

* This code does not set for 8 bit characters (which is required). It

* also assumes that carriage return/newline is needed, and not just

* carriage return.  These are all switch settings on the DMP, and

* I have configured them for 8 bit data and cr only.

*

* You can search for the strings Init and Reset (in devdemp.c) to find the

* strings that set up the printer and clear things when finished, and change

* them to meet your needs.

*

* Also, you need to make sure that the printer daemon (assuming unix)

* doesn't change the data as it is being printed.  I have set my

* printcap file (sunos 4.1.1) with the string:

* ms=pass8,-opost

* and it works fine.

Mark Wedel

master@cats.ucsc.edu

### ------------------------------ End --------------------------------- ###

### ------------------ The Epson Stylus Color printer ------------------ ###

Epson Stylus-Color Driver, contributed by Gunther Hess (address: see below)

The Ghostscript-Device is stcolor and the driver resides in the

file named gdevstc.c. It supports the following Options, besides

the standard ones:

BitsPerPixel   1, 4, 8, 24   Default: -dBitsPerPixel=24

SpotSize       4 floats      Default: -dSpotSize='{1.0 1.0 1.0 1.0}'

OutputCode     Bitfield      Default: -dOutputCode=24 (Bidir,SoftWeave)

4 -> Microweave

8 -> SoftWeave

16 -> Bidirectional

512 -> Messages

Valid Resolutions:

-r180x90, -r180x180, -r180x360,    -r180x720

-r360x90, -r360x180, -r360x360(D), -r360x720

-r720x90, -r720x180, -r720x360,    -r720x720

Default: -r360x360

An example for 720x720Dpi-Freaks, to be used with special paper:

gs -sDEVICE=stcolor -r720x720 -dSpotSize='{2.6 2.4 2.6 2}'

NOTE: Due to a buggy behaviour during testing, a compile-time option to

disable setting SpotSize from the command-line was provided. If this

option was used, one should not use 720x720 DpI - perhaps except for

overhead-transparents. This applies to versions 1.10/1.11.

[If I'd knew what's going on, I'd fixed it. :-(]

A C K N O W L E D G E M E N T S:

********************************

Two other driver-sources were used when coding this driver:

gdevcdj.c, contributed by

George Cameron      - g.cameron@biomed.abdn.ac.ukis

Koert Zeilstra      - koert@zen.cais.com

Eckhard Rueggeberg  - eckhard@ts.go.dlr.de

and gdevescp.c, contributed by

Richard Brown       - rab@tauon.ph.unimelb.edu.au

The Ghostscript-Interface and part of the Floyd-Steinberg algorithm are

almost copies from the Deskjet-Driver and some hints for the Printer-

Interface are from the Stylus 800 driver.

While I wish to thank the authors of the drivers mentioned above, they

are by no means responsible for bugs in the stcolor-driver - just for

the features.

|

|Duisburg 30-JAN-1995, Gunther Hess

|

|Up to 15-MAR-1995 E-Mail:  hess@ims.fhg.de

|After March '95, one should use snail-mail or phone:

|

|Gunther Hess                  phone: ++49 203 376273

|Richard Wagner Strasse 112

|D-47057 Duisburg

|Germany

|

******************************************************************************

EXCUSE:

I'm just an owner of the Epson Stylus-Color, I've only the owners-manual.

So I'm unable to use the advanced features of the printer, since my local

Epson-Dealer cannot deliver a Programmers-Manual since December, 7th.

FEATURES:

The driver generates runlength-encoded data suitable for the

Epson Stylus-Color - that famous 720x720DpI paper-inker.

Any mix of X-Resolutions out of 180/360/720DpI and Y-Resolutions out

of 90/180/360/720DpI is supported.

Under normal circumstances the printer does not allow Y-Resolutions

larger than the X-Resolution. This combinations are handeled by the

SoftWeave-Code [(TM) :-)], which is a software-pendant for the

Microweave-Feature of the printer. IMHO the combinations 360x720 or

720x360 are a real *MUST*, both of them can only be reached by

SoftWeave. Since 720x720DpI-Printing produces just wonderful black,

even on the "Special 720DpI-Paper" and takes much more printing time.

The option BitsPerPixel controls wether Black&White- or Color-Printing

will happen and wether GhostScripts-Dithering or the Drivers Floyd-

Steinberg-Algorithm will be used. The Default is to use Floyd-Steinberg

and color-printing. There are four modes:

BitsPerPixel =  1    -> monochrome, GS-Dithering

BitsPerPixel =  4    -> color,      GS-Dithering

BitsPerPixel =  8    -> monochrome, Floyd-Steinberg Error-Diffusion

BitsPerPixel = 24(D) -> color,      Floyd-Steinberg Error-Diffusion

If the Floyd-Steinberg-Algorithm is used, there is another option of

the Driver: "SpotSize". SpotSize is an array of four floats, that defines

the area, that an dot of ink covers, measured in Pixels, for each ink.

The order of the values is:

-dSpotSize='{Black Cyan Magenta Yellow}'

A real 720x720DpI-Printer should work with the default of

SpotSize='{1 1 1 1}', but that's not true for the Epson Stylus-Color.

Unless one increses SpotSize to approximately 2.0 or above, very well

inked (Special-) Paper is the result. Since SpotSizes can be adjusted

for each individual color, this option can be `abused' for some kind

of color-correction. (My favourites: see the example above)

|

| It's a matter of taste, but I think Floyd-Steinberg produces better

| pictures - at the expense of computation time. My 486/DX2 is >90%

| loaded when processing data for the printer, and this is almost independend

| of the image-complexity. On the other Hand the Ghostscript-Algorithms

| are much faster and there is not that much difference at the high

| resolutions. Unfortunatley at 720x720DpI Floyd-Steinberg is required

| to "emulate" the Depletion-Mode of the HP-Printers, but fortunately

| the advanage above 360x720 (perferably) or 720x360 is just marginal.

|

OutputCode is another driver-option, which is actually a Bit-Field

and its main purpose is debugging of the Driver. Extensive use of this

option is not recomended, since the coding may change.

The current coding is:

Bits 0-1 : Compression-Type (currently unused, due to lack of manual)

0: plain, 1: runlength(D), 2: delta-row, 3: tiff

Bits 2-3 : Weave-Mode

0: none,  4: Hardware-Microweave, 8: Software-Weave(D),

12: definitly illegal

Bit  4:   0: unidirectional-Printing, 16: bidirectional-Printing(D)

Bit  9:   0: silence(D), 512: Debugging-Messages on.

Except for the `Weave-Mode' this stuff should be self-explanatory.

I'd like to mention, that bidirectional-printing gains significant

speed (45%) without loss of print-quality.

The Weave-Mode is a special-feature of the Epson-Printer. It is a

multi-pass printing-mode, where the head moves down head-height/#passes

between the passes. This causes the 'Zebra-Look' to vanish and should

be used. Only some resolutions are supported, so this mode was implemented

in software too. Besides the effect of more resolutions, SoftWeave gains

significant speed in 720x720DpI, which is an 8-Pass mode, where the printer

cannot buffer enough rows to print 15-rows (at 90DpI) per pass (?). But on

some popular Operating-Systems, Softweave might be troublesome too, since

the buffersize in 720x720DpI-Mode is approximately 500k.

T E S T S (version 1.11)

(1.10)

DpI    BPP  OC                       i486DX2 Real-Time

180x 90  1  513  none/uni golfer.ps     2.26s   0:02.59 (All bufferd)

180x 90  4  513  none/uni golfer.ps     5.05s   0:05.43 (All bufferd)

180x180  8  533 micro/bi  golfer.ps    11.15s   0:25.50 (Mostly bufferd)

180x360 24    -  soft/bi  escher.ps    72.038   2:04.98 (Partly bufferd)

180x720 24  529  soft/bi  colorcir.ps  81.19s   3:08.46 (SS='{2 2 2 1}')

360x 90  1  533  none/bi  colorcir.ps   5.65s   0:12.25 (Mostly bufferd)

360x180  8  529  none/bi  golfer.ps    18.29s   0:32.29 (Mostly bufferd)

360x360 24  533 micro/bi  kathy.ps    215.57s   3:40.68

360x720  1    -  soft/bi  kathy.ps    108.91s   4:38.65 (Dither-Bug)

720x 90  8  513  none/uni golfer.ps    22.44s   1:08.89 (Mostly bufferd)

720x180 24  529  soft/bi  escher.ps   169.32s   2:00.53 (Partly buffered)

720x360 24  529  soft/bi  kathy.ps    299.48s   6:08.93

720x720 24  537  soft/bi  tropics.ps  748.71s  13:37.57 (SS='{2.6 2.4 2.6 2}')

720x720 24  533 micro/bi  tropics.ps  774.48s  33:21.83 (SS='{2.6 2.4 2.6 2}')

Notes:

A)  OC (OutputCode) is not necessarily related to used mode, since I tested

the automatic switching too.

B)  kathy & tropics are full-page color-images with 24Bit rgb-Data.

C)  Amazingly the CP-Time of SoftWeave at 720x720 is less than MicroWeave,

this might be a result of the more efficient usage of the loop-codes

when filling the buffer. (SoftWeave processes about 15 scans at a time,

while MicroWeave reads scan by scan)

### ------------------------------ End --------------------------------- ###