THE INPUT SEQUENCE

Mininec has been created with a good set of prompts, which makes data

input relatively easy. The following reviews the input requirements in

the sequence they are encountered. It is assumed that MNsetup.doc has

been read first.

Each of the following headers represents a major input request from

Mininec. A star after an input quantity indicates that the entry

causes some of the immediately following input possibilities to be

ignored.

Note that most input requests have protection tests. Most ask for a

repeat of input, but a few cause a jump to a menu. Requests for a

letter require a capital.

After printing a header, data input requests start.

OUTPUT TO CONSOLE=C *, PRINTER=P *, Disk=D

-Enter selected letter

FILENAME

-Enter the filename (and filepath if disk storage is to be

used for output and/or geometry input)

FREQUENCY

-Enter desired frequency in MHz. A zero causes the frequency to

be set to 299.8 MHz.

ENVIRONMENT

-Enter 1 for free space conditions *, -1 for ground plane

NUMBER OF MEDIA

-Enter 0 for ideal ground *, 1 to 5 for number of media

TYPE OF BOUNDARY

-Enter 1=Linear, 2=Circular

MEDIA

-Enter, in order, dilectric constant, conductivity in

mhos/meter, for each media

NUMBER OF RADIAL WIRES IN GROUND SCREEN

-Enter 0=no screen *, or number

RADIUS OF RADIAL WIRES

-Enter radius, meters

X OR R COORDINATE OF NEXT MEDIA

-Enter distance, meters

HEIGHT OF MEDIA

-Enter height relative to 0,0,0 point, meters

(Note: at this point, a check is made for existence of the named file

+".GEO" in the disk designated by filepath; it is loaded if present

*)

NO. OF WIRES

-Enter number; 0= *

NO. OF SEGMENTS

-Enter number

END ONE COORDINATES

-Enter X, Y, Z in meters

END TWO COORDINATES

-Enter X, Y, Z in meters

RADIUS

-enter radius, meters

(For each wire)

(Note: at this point wire connections are made, and a table of

connections is output)

CHANGE WIRE NO.

-Input Y for a change or N

(Note: after finishing with input of last wire a second geometry table

is output. Check that location of segment to be fed or loaded is

correct, and that connections are correct)

CHANGE GEOMETRY

-Enter N= no change, or Y= repeat at NO. Wires

NO. OF SOURCES

-Enter number of drive feed points; 1 is assumed if a lesser

number is entered

PULSE, VOLTAGE, PHASE

-Enter number of segment to be driven, drive in volts, phase

in degrees for each driven segment

NUMBER OF LOADS

-Enter number of segments to be loaded, 0=none *

S-PARAMATER LOAD

-Enter Y if needed, or N *

PULSE NUMBER, ORDER OF S

-Enter number of the segment to be loaded, highest S

exponent

NUMERATOR, DENOMINATOR COEFFICIENTS OF S

-Enter coefficients, ohms, henries, farads, as appropriate,

each loaded segment

PULSE, RESISTANCE, REACTANCE

-Enter number of segment to be loaded, load resistance, load

reactance, ohms, each loaded segment

(Note: A menu is now displayed. Several entries allow input changes,

by jump to the input routines above. In some cases following inputs

also must be repeated. The options which produce output and require

further input follow)

C=COMPUTE/DISPLAY CURRENTS

(Note: current computation is required for any form of solution. If C

is not entered the currents will be computed but not displayed. A

table of feed point(s) conditions and power will be output in all

cases. If C has been entered, this is followed by a tabulation of

current in each segment.

P=COMPUTE FAR-FIELD PATTERNS

VARIABLES IN DBI OR VOLTS/METER

-Enter D * for dbi or V for field intensity, in

uvolts/meter

CHANGE POWER LEVEL

-Enter N * for no change, or Y

NEW POWER LEVEL

-Enter value, 0 or less= no change *

RADIAL DISTANCE

-Enter distance to calculation point, meters, 0 or less= no

change *

ZENITH ANGLE

-Enter initial angle in degrees from zenith, increment per

step, number of calculation steps

AZIMUTH ANGLE

-Enter initial angle in degrees from zero, increment per

step, number of calculation steps

(Note: Zero degrees azimuth is along plus X-axis, zero degrees zenith

is along plus Z-axis.  Minus angles are allowable. For complete

coverage of the free space pattern, 0 to +180 zenith and 0 to 360

azimuth is required. Choose increment to show smallest lobe width.)

FILE FAR FIELD DATA

-Enter Y for disk copy or N  *

FILENAME

-Enter filepath + filename

N=COMPUTE NEAR FIELDS

ELECTRIC OR MAGNETIC

-Enter E=electric, H=magnetic

FIELD LOCATION(S)

-Enter initial location from 0,0,0, increment distance,

number of steps

CHANGE POWER LEVEL

-Enter Y=change or N *

NEW POWER LEVEL

-Enter new level, watts, 0=old

SAVE TO A FILE Y/N

-Enter Y for a disk file or N= No *

FILENAME

-Enter filepath + filename

The extended versions of MININEC include one or more additional

routines

VARIABLE FREQUENCY

Has inputs to set the initial frequency, the increment and the

number of increments. Does not recalculate the magnitude of load when

the frequency steps.

VARIABLE LOADS

Has inputs to set the initial reactive load, the increment and

the number of increments. Does not allow resistive components in the

variable load.

OPTIMIZER

Has input to select maximization of gain or front to back ratio.

Requires reflector to be first element, then radiator, then directors.

Varies reflector size to maximize the selected quantity, then varies

director sizes in the same way. All directors use same increment.

(Caution: Maximum gain, maximum F/B and high bandwidth occur at

different design points. The maximum gain condition often gives narrow

bandwidth and poor F/B ratio. Maximum F/B may give reduced gain and

reduced bandwidth. Always recheck the optimized design for all

inportant performance parameters. Several trials may be needed to

obtain a satisfactory compromise design.)