======================================================================

0.0.0

TNIMAGE.DOC

Description of TN-Image Image Analysis Software

Latest version:  2.19

Latest revision: 03/15/1996

Ordering:     T.J. Nelson

Box 275

11140 Rockville Pike

Rockville, MD 20852

USA

e-mail:       tjnelson@las1.ninds.nih.gov

or

Compuserve: 71461,1473

or

71461.1473@compuserve.com

ftp sites:

las2.ninds.nih.gov: /pub/tnimage

las1.ninds.nih.gov: /pub/tnimage

A collection of sample images, mostly in TIF format,

is also available at these sites. These are also the

ftp sites for NPLOT, the scientific graphics plotting

program, and TNSHELL, the powerful DOS shell.

The program is also available at SimTel mirror sites.

BBS sites:  Compuserve, IBMAPP, library 10

America On Line

======================================================================

1.0.0  Introduction

Welcome to TN-Image!

TN-Image is an advanced image analysis program oriented toward

scientific and technical image analysis and editing. This manual

describes the various funtions of TN-Image. Not all the features

of TN-Image are available on the shareware version; however, it

is still a powerful, functional image editing program.

1.0.1  About this document

This is the text-only manual for the DOS version of TN-Image and

is no longer current.  Consult the PostScript version of the manual

(tnimage.ps) for more up-to-date information.

1.1.0   Contents

Introduction..................................................1.0.0

Contents...................................................1.1.0

Summary.......................................................2.0.0

Usage License.................................................3.0.0

Differences between shareware and registered versions......3.1.0

Submitting bug reports.....................................3.2.0

Technical support..........................................3.3.0

Upgrade policy................................................4.0.0

System Requirements...........................................5.0.0

Overview......................................................6.0.0

File types supported.......................................6.1.0

Pixel intensities..........................................6.2.0

Menu bars..................................................6.3.0

Tutorial files.............................................6.4.0

Operation.....................................................7.0.0

Command line options ......................................7.1.0

Unix command line options................................7.1.1

Video cards supported......................................7.2.0

Screen modes supported.....................................7.3.0

Basic operation............................................7.4.0

Operating under low-memory conditions......................7.5.0

Commonly asked questions...................................7.6.0

Batch-mode processing......................................7.7.0

Editing text files.........................................7.8.0

Menu Options..................................................8.0.0

File Menu..................................................8.1.0

Load Image.............................................8.1.1

Reading CT, X-ray and NMR images and raw bytes......8.1.1.1

Reading Imaging Systems files.......................8.1.1.2

Reading Images from Macintosh systems...............8.1.1.3

Reading raw ASCII images............................8.1.1.4

Save Image.............................................8.1.2

Printing Images........................................8.1.3

B/W printing........................................8.1.3.1

Color printing......................................8.1.3.2

Printing under Unix.................................8.1.3.3

Printing to a file or different printer.............8.1.3.4

Change Title...........................................8.1.4

Save scan data.........................................8.1.5

Create/Resize Image....................................8.1.6

Switch to..............................................8.1.7

Create File Format.....................................8.1.8

Save FFT...............................................8.1.9

Erase FFT..............................................8.1.10

Unload image...........................................8.1.11

DOS Command............................................8.1.12

Quit...................................................8.1.13

Load FFT...............................................8.1.14

Image menu................................................8.2.0

Delete region..........................................8.2.1

Crop...................................................8.2.2

Erase background.......................................8.2.3

Move...................................................8.2.4

Copy...................................................8.2.5

Change size............................................8.2.6

Rotate image...........................................8.2.7

Flip horizontally......................................8.2.8

Flip vertically........................................8.2.9

Back up................................................8.2.10

Restore................................................8.2.11

Process menu..............................................8.3.0

Filter.................................................8.3.1

High pass...........................................8.3.1.1

Low pass............................................8.3.1.2

Background subtract.................................8.3.1.3

Background flatten (de-trending)....................8.3.1.4

Noise reduction (median) filter.....................8.3.1.5

Laplace edge enhancement............................8.3.1.6

Sobel edge enhancement..............................8.3.1.7

Edge detection......................................8.3.1.8

Adjustable parameters...............................8.3.1.9

Warp...................................................8.3.2

Measure................................................8.3.3

Distance............................................8.3.3.1

Angle...............................................8.3.3.2

Calibration............................................8.3.4

Spot Densitometry......................................8.3.5

Area Measurements...................................8.3.5.1

Strip Densitometry.....................................8.3.6

Plotting densitometry results and other data........8.3.7

Peak areas..........................................8.3.8

Trace curve............................................8.3.9

Mathematical pixel operations..........................8.3.10

FFT (2D Fourier Transform).............................8.3.11

Convolution.........................................8.3.11.1

Deconvolution and image reconstruction..............8.3.11.2

Digital filtering using FFT.........................8.3.11.3

Macros.................................................8.3.12

Macro command summary...............................8.3.12.1

Macro programming guide.............................8.3.12.2

Color Menu................................................8.4.0

Set color..............................................8.4.1

Color intensity/brightness.............................8.4.2

Changing brightness using the palette bar...........8.4.2.1

Changing brightness using the menus.................8.4.2.2

Contrast...............................................8.4.3

Changing contrast using the palette bar.............8.4.3.1

Changing contrast using the menus...................8.4.3.2

Palette (colormap).....................................8.4.4

Select palette......................................8.4.4.1

Grayscale intensity mapping.........................8.4.4.2

Read palette from disk..............................8.4.4.3

Save palette to disk................................8.4.4.4

Create palette......................................8.4.4.5

Invert palette......................................8.4.4.6

Rotate palette......................................8.4.4.7

Restore original palette............................8.4.4.8

Remap colors...........................................8.4.5

Invert colors..........................................8.4.6

Change color depth.....................................8.4.7

Color->gray scale......................................8.4.8

Gray scale->color......................................8.4.9

Histogram..............................................8.4.10

Change pixel value.....................................8.4.11

Draw menu.................................................8.5.0

Set color..............................................8.5.1

Line...................................................8.5.2

Circle.................................................8.5.3

Box....................................................8.5.4

Arrow..................................................8.5.5

Curve..................................................8.5.6

Bezier curve.........................................8.5.6.1

B-spline curve.......................................8.5.6.2

Least-squares line...................................8.5.6.3

Polygon..............................................8.5.6.4

Text direction.........................................8.5.7

Font...................................................8.5.8

Sketch.................................................8.5.9

Fill region............................................8.5.10

Gradient fill .......................................8.5.10.1

Paint region...........................................8.5.11

Diffuse spray..........................................8.5.12

Fine spray.............................................8.5.13

Add border.............................................8.5.14

About menu................................................8.6.0

About the program......................................8.6.1

About the file.........................................8.6.2

About the image........................................8.6.3

How to register........................................8.6.4

Configure menu............................................8.7.0

Show menu bar..........................................8.7.1

Show menu bar 2........................................8.7.2

Show palette...........................................8.7.3

Redraw menu bar........................................8.7.4

Redraw palette.........................................8.7.5

Show O.D. table........................................8.7.6

Pixel interact mode....................................8.7.7

Configure............................................. 8.7.8

Automatic undo......................................8.7.8.1

Spray factor........................................8.7.8.2

Cursor movement rate................................8.7.8.3

Color settings......................................8.7.8.4

Significant digits..................................8.7.8.5

Crawl delay.........................................8.7.8.6

Crawl density.......................................8.7.8.7

Active color planes.................................8.7.8.8

Help menu.................................................8.8.0

Miscellaneous features....................................8.9.0

Frame rate test........................................8.9.1

Windows and OS/2 compatibility...............................9.0.0

Changing Computers..........................................10.0.0

Problems....................................................11.0.0

Limitations and known bugs..................................12.0.0

Error messages..............................................13.0.0

Trademark disclaimers.......................................14.0.0

2.0.0   Summary

|---------------------------------------------------------------------|

|                                                                     |

| TN-Image allows you to view, edit,and print PCX, IMG, TIF (both     |

| Macintosh and PC), JPEG, BMP, GIF, and TGA images, of any depth     |

| between 1 and 32 bits per pixel, color or monochrome, as well as raw|

| binary images and user-definable image formats.  You can edit,      |

| cut/paste, add text, adjust color intensity or contrast, superimpose|

| multiple images, and create drawings using lines, circles, splines  |

| and Bezier curves. It handles 8 bit/pixel (indexed-color), as well  |

| as 15,16, 24 and 32-bit per pixel screen modes. In the pseudo-color |

| modes, you can change the color palette to one of 10,000 pre-defined|

| palettes or graphically create a new one. TN-Image is completely    |

| menu-driven with an extremely user-friendly graphical user          |

| interface. Dialog boxes and graphs can all be clicked-and-dragged to|

| one side so as not to obscure the image. Image size is limited only |

| by available memory or disk space (whichever is greater).           |

|                                                                     |

| TN-Image is particularly useful for scientific image analysis,      |

| with densitometry, background subtraction, molecular weight calib-  |

| ration, and automatic detection of peaks, bands and spots in your   |

| image for quantitative measurement of optical density. Images deeper|

| than 8 bits/pixel, such as medical grayscale images, can be viewed  |

| with a sliding scale to enhance any particular intensity region.    |

| Images can be rotated, resized, warped, flipped, color-remapped, or |

| filtered (with sharpening, blurring, 3 types of edge enhancement,   |

| freeze-fracture effect (shadow  sharpening), background subtract,or |

| noise filter). Quantitative data, peak area tables, or color histo- |

| grams can be exported to an ASCII file with a single mouse click.   |

|                                                                     |

| Areas or parts of images can be filled with a solid color or a color|

| gradient, copied to another region, superimposed, subtracted, XOR'd,|

| added, averaged or erased. All operations function seamlessly across|

| multiple images regardless of their color depth. Images can be up-  |

| or down-converted into a different color depth.                     |

|                                                                     |

| It is particularly useful to biochemists and molecular biologists   |

| who wish to analyze SDS gels, blots, and DNA gels. The blot can     |

| simply be scanned in any scanner, then imported as an image file    |

| into TN-Image and labeled, analyzed, quantitated, and printed on a  |

| laser printer. Printed blot images and quantitative results have    |

| already appeared in several scientific papers. The print quality    |

| is optimized for high resolution, and gives extremely high quality  |

| output on a laser or inkjet printer up to 2400 dpi.                 |

|                                                                     |

| Images can be Fourier-transformed and the real or imaginary compon- |

| ents can be viewed, edited and modified. Pairs of images can also be|

| convoluted and deconvoluted. Power spectra can also be viewed.      |

|                                                                     |

| TN-Image can use up to 2GB of extended and virtual memory. Up to 512|

| images can be viewed and edited simultaneously. The program requires|

| an 80386 with a minimum of 4MB of RAM, DOS 3.2 or above or a        |

| supported version of Unix, and a Super VGA  monitor. It handles all |

| VESA screen modes, including 1600x1200                              |

| resolution, in 256, 32768, 65536, and 16 million (true color) modes.|

| A mouse is required. Laser or inkjet printer is required for        |

| printing. TN-Image can also run in a DOS box under Windows or OS/2  |

| if desired.                                                         |

|                                                                     |

|---------------------------------------------------------------------|

3.0.0  Usage license

*****************************NOTICE********************************

The author makes no claims or warranties concerning the accuracy

of the information presented herein, including, but not limited to,

any implied warranties of fitness or suitability for any purpose

and is not responsible for any loss, damage, inaccurate medical

diagnosis, or personal injury occurring out of or in connection with

the use of this product. In no event shall the author, or any other

party who may have redistributed the product as permitted above, be

liable for any damages, including any general, special, incidental, or

consequential damages arising out of the use or inability to use this

product, including but not limited to loss of data or data being

rendered inaccurate or losses sustained by you or third parties or

any failure of the product to operate with any other programs, even

if such holder or other party has been advised of the possibility of

such damages.

***********************************************************************

The following applies only to the DOS version of TN-Image.

The program TN-Image is not free software or public domain software,

but shareware. However, you may use the shareware version without

charge for a limited time (up to one month) to evaluate its suitability

for your needs. AFTER THAT TIME, YOU ARE REQUIRED TO REGISTER YOUR

COPY OR DISCONTINUE USING IT.

To register, send your $25 registration fee to the above address. A copy

of the registered version will be mailed to you, or if you prefer, sent

electronically. Please indicate where you obtained your copy (e.g., by

ftp'ing, from  Compuserve, etc.). This information will be used to help

ensure that future upgrades can be posted to that location. Requests for

modifications to the program (such as supporting new features, new types

of SVGA  cards, etc), and assistance, are welcome from all registered

users. An order form is included in the file TNORDER.FRM.

If for some reason you are unable to send your registration to a

P.O. box, feel free to contact me for a street address.

The shareware version of TN-Image, including this manual, may be

freely redistributed, copied, and uploaded to computer networks or

bulletin boards without restriction, provided that the following

conditions are met:

(1) This documentation file and all other files included in the

TN-Image package, including the sample images and tutorial files,

must be included in their entirety whenever the program is distributed.

Any modifications of the documentation must be clearly indicated as such.

(2) The executable file TNIMAGE.EXE must not be modified in any way

without express written permission from the author.

(3) No algorithm used in this program or any part of the user

interface may be patented or trademarked by any party.

(4) No additional restrictions  may be added to the above-mentioned

limitations. In particular, vendors, end users and distributors may

not be prohibited from distributing, uploading, or using the share-

ware version of this package without any charge or restriction other

than those stated in this license.

(5) TN-Image may also be distributed in any form, including CD-ROM,

on a commercial for-profit basis, as long as the package is clearly

marked as being "shareware" or "containing shareware items".

In contradistinction to conditions (4) and (5) above, the registered

version of TN-Image may not be distributed by any party without the

express written permission from the author. Inquiries regarding site

licenses, from commercial enterprises wishing to market this program,

or persons wishing to obtain a customized version of the program on

a contract basis should be directed to the above mail or e-mail address.

3.1.0   Differences between shareware and registered versions

The following applies only to the DOS version of TN-Image.

The main limitations of the shareware version is that it can only

handle two images at a time, and that advanced features such as

densitometry and 2D Fourier transforms are not available in the

shareware version. Some features that require creation of a new image,

such as image rotation, will not work if there are already two images

present. Functions such as Fourier deconvolution, which requires

several image buffers, will also not work. Reading and writing of

JPEG images, which requires virtual memory, does not work.

Depending on the amount of memory in your computer, some of the

higher resolution screen modes also may not work in the Shareware

version.

After registering your copy of TN-Image, you will receive a free

copy of the full TN-Image "Professional" version. This program

contains all the features of the shareware version, as well as the

following enhancements:

(1) Improved capacity - handles up to 512 images simultaneously

instead of only two.

(2) 2D-Fourier transform and Fourier convolution/deconvolution of

images. View the image "DECONVOL.TIF" for a brief tutorial on

2D-Fourier transforms, convolution, and deconvolution.

(3) A much wider selection of predefined palettes, as well as the

ability to interactively create new palettes, save, and load them

from disk.

(4) Many more types of filters (see Sec. 8.3.1).

(5) Virtual memory, for much larger file handling (up to 3.5 GB).

(6) New features which are constantly being added (see "Planned

Enhancements" below). By the time you read this, there will

certainly be many additional new features, most of which you

will unquestionably be in dire need of.

(7) Densitometry, area analysis, and peak area measurement.

View the images "DENSITOM.PCX" and "DENSITO2.TIF" for a tutorial

on these features.

(8) Ability to create custom file formats or save images in a

custom format.

(9) Reading and writing of JPEG files. JPEG is not included in the

shareware version because it requires virtual memory, and also

adds about 100k to the size of the program.

(10) Color and grayscale printing of images.

(11) Powerful macro language and batch mode operation.

(12) Registered users also are entitled to unlimited free technical

support via e-mail.

(13) Liberal upgrade policy (see below).

Note:

Because of the tremendous volume of registrations that we have

received, is is not possible to answer questions from

unregistered users unless you are having basic trouble such as

"blank screen", "won't start", etc.

An order blank is included in the file TNORDER.FRM.

3.2.0  Submitting bug reports

If TN-Image crashes, hangs, draws the screen incorrectly or fails

to read or write a valid image, it is probably a bug. I would greatly

appreciate receiving bug reports from all registered and unregistered

users. Bug reports should be sent to the above Internet e-mail address

for fastest response, or alternatively to the P.O. box listed above.

If you send a bug report, please run the command "tnimage -diag" first

and write down any relevant information. Also indicate your computer

type, video card, and monitor type, what resolution you are trying to

use, and the operating system and OS version you are using, and any

other relevant information.  Most importantly, indicate what version

of TN-Image you are using, and the sequence of actions that cause the

problem. If the problem only occurs with a specific image, you may also

need to send a copy of the image file. The easiest way to do this is to

upload it by ftp into the 'uploads' directory on las2.ninds.nih.gov. All

bug reports will receive immediate attention. If you are a registered

user, you will also receive a replacement disk if I am able to reproduce

the bug.

3.3.0  Obtaining technical support

The following applies only to the DOS version of TN-Image.

Registered users can obtain technical support by sending e-mail to

the above Internet or Compuserve e-mail addresses, or by sending

regular mail to the P.O. box mentioned above. You will get the

fastest response using the Internet e-mail address.

4.0.0  Upgrade policy

The following applies only to the DOS version of TN-Image.

Registered users may receive an unlimited number of upgrades at

any time within two years of the date of registration, by sending

$5 to cover postage and diskette costs to the mailing address

above. Alternatively, you can request that the update be e-mailed (in

uuencoded format) to your e-mail address, or sent to your ftp server.

(It is not necessary to pay in this case.)

After the two year period, upgrades may be requested at any time,

for a price of 1/2 the original purchase price. You may request that

the upgrade be sent by e-mail, ftp, or by regular mail.

Read the file TNIMAGE.NEW, available at the ftp site, for news about

the latest features and changes in TN-Image.

5.0.0   System  Requirements

TN-Image has the following minimum system requirements:

1. MS- or PC-DOS above version 3.2, Windows, or a supported

version of Unix with X11 (Unix version is available separately).

2. A minimum of 4 MB of RAM, preferably 8 MB. 8MB is strongly recommended

and may be required in future versions (If you have 4 MB of RAM

or less, see "Operating under low-memory conditions" below).

3. A computer with an 80386 or higher CPU. (TN-Image runs in protected

mode). A co-processor is recommended for 80386's if you will be

performing FFT's.

4. A Super VGA card with at least 1 MB of video memory, and capable

of handling 256 or more colors.

You MUST have at least 1 megabyte of video memory on your Super

VGA card to run TN-Image in 1024 x 768 pixel mode. (To run

TN-Image in lower-resolution modes, see "Command-line options"

below).

5. A super VGA monitor capable of displaying 256 colors. The

default screen mode for TN-Image is 800 x 600 pixels. If you don't

know whether your monitor can display at this resolution, it is

advisable to check first, either in your monitor documentation, or

using the diagnostics program that should have come with your video

card. WARNING: It is the responsibility of the video card, not this

program, to determine whether your monitor can handle any given screen

mode. The only way for the program to determine whether it is safe to

set a given screen mode is to query your video card. Unfortunately,

not all video cards respond with accurate information. If your video

card supports this screen mode but your monitor doesn't, you could

experience a computer crash or even (theoretically) damage your

monitor. This has never happened in all the months of extensive

testing of the program on a variety of monitors. Nonetheless:

THE AUTHOR ASSUMES NO RESPONSIBILITY FOR ANY PROBLEMS

CAUSED BY RUNNING THIS PROGRAM IN A COMPUTER WHICH HAS A

SUPER VGA CARD ATTACHED TO A REGULAR (NON-SUPER VGA) MONITOR!

If you have a super VGA monitor but it cannot handle 800x600 pixel

mode, TN-Image can also use other video modes. See "Command-line

options" below.

6. A mouse. It is advisable to obtain a mouse driver that is

compatible with super VGA modes, such as Logitech's mouse driver

version 6.12 or later. Earlier mouse drivers will work, but you

will not have as fine control of the mouse as with later versions.

A mouse driver is needed even if TN-Image is run in a Windows DOS box.

7. TN-Image should be run from a hard disk. If run from a floppy, do

not remove the floppy while the program is running.

8. A PCL laser or inkjet printer is required to obtain hard copies. (H/P

Laserjet IV compatible or a similar 600 dpi laser printer is recommended).

A 300 dpi printer will also work, but will give inferior quality prints.

A H/P compatible color laser or inkjet printer is required for color

printing.

6.0.0   Overview

TN-Image is an image editing program  for  PC-compatibles.  You can

read  and  write  image files from disk, edit and combine them, add

text or  various  graphic  elements,  adjust  the  colors,  perform

densitometry,  quantitatively  scan trapezoidal regions or lines in

the image, draw graphs, and print images  at  several  resolutions.

Unlike the image editing programs which run under Windows, or those

which may  come  with  a scanner, TN-Image allows you to view 8- to

32-bit images in their entirety.

Images are stored in RAM and on disk  using  virtual  memory.  This

means  that if the program begins to run slowly, it is probably due

to insufficient RAM. Up to 512 different images can be  edited  and

be on the screen simultaneously (2 in the Shareware version).  This

limit may be lower if you have limited RAM and disk space.  You can

obtain  more  virtual  memory by using a disk utility to unfragment

your hard disk. Images can overlap without  interfering  with  each

other or the  background.  You  can use the mouse to scroll through

the image in adjustable steps.

6.1.0  Image file types supported

TN-Image reads and writes image files in TIF, JPEG, PCX, TGA, BMP, as

well as the IMA format produced by the publication-quality graphics

shareware program NPLOT (available at the ftp sites las2.ninds.nih.gov

and las1.ninds.nih.gov) and the IMG format produced by frame-grabbers,

as well as GEM IMG and several others listed below. Any other generic

file type can also be read, provided that:

1. you know the number of x and y pixels in the image

2. you know the no. of bits per pixel.

3. the image stores its pixels in the usual scanning order (left

to right, top to bottom).

In addition, you can create new image file formats to read and write

images in virtually any uncompressed format that uses fixed-size

headers.

The IMG format produced by TN-Image is the same as that produced by

most frame grabbers: 4 bytes indicating the x and y size, followed

by the image data.  This is not the same as the GEM "IMG" format,

which is seldom used for image analysis. TN-Image can read GEM IMG

Raster files, however.

For densitometry, it is recommended that images be stored in TIF

format because this format allows the image to provide a calibration

curve to accurately convert pixel intensity to optical density (O.D.).

This is important for quantitative applications. Most scanners provide

this information. If not, TN-Image will still function, but the results

will be in uncorrected pixel intensity values instead of O.D. (This

consideration does not apply for color images).

***NOTE*** Targa (TGA) files must have the extension .TGA to be readable.

***NOTE ON COMPRESSED TIF FILES***

As of Version 2.11, creation of compressed TIF image files is not

supported in TN-Image.  LZW- and PackBits-compressed TIF files are,

however, readable.  The reason for this discrepancy is the choice

by Aldus Corporation of LZW compression, which is patented by Unisys,

for the compression algorithm.  The most recent TIFF documentation

indicates that LZW is now being phased out of TIFF. This means that

LZW-compressed TIF files will no longer be a valid image format.

Technical information -

The following file subtypes/features in all valid permutations have

been tested to be readable by TN-Image:

TIFF files:

1 to 32 bit/pixel TIF-B,G,P, and R files

(B=black/white, G=gray scale, P=palette color, R=RGB full color)

Tiles/Strips/CMYK

Intel and Macintosh TIF

PCX files:

1,2,4,and 8 bit/pixel PCX

IMG files:

1,2,4,and 8 bit/pixel IMG

Other:

4 bit IMA  (from VGA or EGA mode NPLOT and NPDATA)

Unformatted 1 to 32 bit/pixel raw binary images (color

and monochrome)

JPEG:

Color JPG (JFIF)     (registered version only)

Targa:

8, 16, and 24 bit/pixel TGA

BMP:

4,8, and 24 bit BMP (Windows 3.x subtype)

GIF

GEM IMG

The following file subtypes can be created by TN-Image:

1 to 32 bit PC TIF-B,G,P,and R, incl. CMYK extension(uncompressed)

1 to 32 bit raw bytes

1 to 32 bit custom files for PC, Mac, and MVS mainframe OS's

1 and 8 bit PCX

1 and 8 bit IMG

24 bit JPG (JFIF subtype)  (registered version only)

8,16, and 24 bit TGA

8 and 24 bit BMP (Windows 3.x subtype)

The following file types may be also supported if enough requests are

received:

Creation of 1 to 32 bit Mac TIF

OS/2 BMP

PNG

6.2.0   Pixel intensities

When you move the mouse in TN-Image, the upper right part of the menu

bar shows the x and y coordinates and the value of the pixel at the

mouse position. This is convenient when manipulating the image, since

image manipulation uses simple mathematical functions to transform

the image. Thus, to make the image lighter, you can "add" a value

of 50 to each pixel. To increase contrast, you can multiply each

pixel value by a constant factor. This ensures that quantitative

manipulations on the image remain directly traceable to the

original data.

6.3.0  Description of Menu Bars (DOS Version)

TN-Image displays a variety of information in the menu bars. One

or both of these lines can be hidden to create a less cluttered

display. The menus are still active even when the menu bars are

hidden.

Top line:

File  Image..etc    - The main menus - use the mouse to select

the desired option.

�                   - Foreground color indicator

# # # #             - Clicking on these small triangles will

move the topmost image in the indicated

direction. The distance is determined

by the "Cursor movement rate" (see below).

r255g255b255        - The red, green, and blue components of

the current pixel, i.e., the pixel the

mouse is pointing to. For 8 bit/pixel

images, the red, green, and blue components

of the palette entry for that color are

shown.

i=32767             - The intensity value of the current pixel,

This is only displayed if TN-Image is in

a mode with a wide enough screen width.

This value will be different for images

with different numbers of bits per pixel.

Second line:

0.12345             - The current pixel value, scaled from 0 to 1

and corrected for bits/pixel of the point.

123 456             - The next 2 numbers are the x and y

coordinates of the current pixel.

Size 8              - The cursor movement rate. This is the

distance by which the mouse will move when

an arrow key is pressed, and the distance

an image will move when you click one of

the small triangles. The cursor size can

be changed  by pressing the gray (+) or (-)

keys, or selecting "Configure...

Cursor movement rate".

8 bpp               - The bits per pixel of the current pixel,

calculated for the topmost image.

16 bits/pixel       - The rightmost item is the bits/pixel of

the screen.

------------        - A click box is visible when the program

| Vertical |          is in a state such as "vertical text" mode

------------          or "drawing" mode. Clicking on this box

returns to normal mode.

6.4.0 Tutorial files

Five tutorial files are included with TN-Image:

DENSITOM.PCX - an image file demonstrating strip densitometry.

DENSITO2.TIF - an image file demonstrating spot densitometry.

DECONVOL.TIF - an image file demonstrating convolution and

deconvolution using FFT (Part 1).

DECONVO2.TIF - an image file demonstrating convolution and

deconvolution using FFT (Part 2). (Included in

registered version).

FILTER.TIF   - an image file demonstrating digital filtering

using FFT.

7.0.0   Operation

To start TN-Image, type TNIMAGE. If VESA BIOS is present, TN-Image

will run using the functions defined in the VESA specification.

Otherwise, TN-Image will try to use video card-specific functions.

If no VESA BIOS is present and the card is unsupported, TN-Image

will terminate. VESA 1.2 or above is required to run in 15-, 16-,

24-, and 32-bit/pixel color modes, since in the earlier VESA specif-

ication, modes 110 to 11b were text modes.

7.1.0   Command-line options  (DOS version)

-n        =  Skip introductory screen in 8-bit modes.

-vesa     =  forces TN-Image to use VESA BIOS even for Trident or

Tseng Labs cards.

-oldvesa  =  forces TN-Image to use functions for version 1.0 VESA

BIOS (slightly slower, but required on some cards).

-trident  =  Forces TN-Image to use Trident-specific calls, even

if VESA BIOS is present.

-tseng    =  Forces TN-Image to use Tseng-specific calls, even if

VESA BIOS is present.

-diamond  =  Forces TN-Image to use Tseng-specific calls, even if

VESA BIOS is present and sets the program to use

Diamond Speedstar mode. This option is experimental and

will be subsumed under the "Tseng" option after we obtain

a Tseng-based Diamond card for testing. Feedback as to

whether this option actually works will be appreciated.

-w32      =  Forces TN-Image to use Tseng-specific calls, even if

VESA BIOS is present and sets the program to use

W32 chip mode. This option is experimental and

will be subsumed under the "Tseng" option after we obtain

a card with the Tseng ET4000W32 chip for testing.

Feedback as to whether this option works will be

appreciated.

-S3       =  Force TN-Image to control these chips directly, even if

-ATI         VESA BIOS is present. NOT CURRENTLY IMPLEMENTED.

-XGA

-diag     =  Diagnostic mode (Prints status messages at each stage

or         of setting your video mode). Useful if you have

-diagnose    difficulty starting TN-Image on your computer. At each

step, you need to press a key to continue.  PLEASE

REPORT THIS INFORMATION WHEN REQUESTING TECHNICAL

ASSISTANCE.

-fyu      =  This option is for debugging use only.

-macro <macro_file>

=  Automatically runs the specified macro file. See "Batch

mode processing"

-files <file_list>

=  Specifies a list of image files to be automatically

loaded. Using a text editor, create an ordinary text

file containing a list of the files to be read, with one

filename on each line, for example:

t*.tif

c:\images\abc.bmp

hubble.img

Then start TN-Image with the command:

tnimage -files test.lst

substituting the name of your list for "test.lst". Do

not put a dash before the filename of the list file.

Wildcard characters such as '*' or '?' are also

permissible. Up to 512 images can be loaded from

the list file (if you have the Registered version).

If you only need to load a few files, it may be easier to

use the "<filename>" option (below) instead.

<filename> <filename> ...

=  Reads the specified file(s). You can load as many files

as you can cram onto the DOS command line.

If you have a large number of files, it may be easier

to use the "-files" option (above).

Do not precede the filename with a dash ("-"). Filenames

may contain path specifications and wild cards (*  or ?).

Examples:

TNIMAGE my.gif      = Loads my.gif

TNIMAGE *.tif       = Loads all TIF files in

current directory.

TNIMAGE c:\images\*.tif b:\abc.img

= Loads all TIF files from

the C:\images directory,

then file abc.img from

drive B.

tnimage -mode 103 *.tif -files file.lst -vesa

= Any combination of command

line options is also accept-

able.

-mode xxx =  Start up in screen mode xxx, where xxx is a

hexadecimal number indicating the desired screen mode

(see below).

Some monitors cannot handle the highest resolution modes

and will fail to `sync' or display a white screen.

If this happens, press Alt-X two or three times to escape

from the program. You can determine which screen modes

your computer can handle by checking in the manual that

came with your video card and monitor. READ THE WARNING

BELOW BEFORE USING THIS OPTION!

Example:  tnimage -mode 111

-xres xxx = Start up in a VESA screen mode with x resolution of xxx,

where xxx is the no. of horizontal pixels to display

per scan line. 8 bits/pixel is automatically selected

unless specified with the -bpp option.

Some monitors cannot handle the highest resolution modes

and will fail to `sync' or display a white screen.

If this happens, press Alt-X two or three times to escape

from the program. You can determine which screen modes

your computer can handle by checking in the manual that

came with your video card and monitor. READ THE WARNING

BELOW BEFORE USING THIS OPTION!

-bpp xxx =  Start up in a VESA screen mode of xxx bits/pixel. (Used

in conjunction with the -xres option).

? or     =  List the available screen modes.

-?

The options can be in any order, except that if a parameter

is needed, it must follow the option.

The arguments must be separated by spaces.

Examples:

To run TN-Image in VESA 1024x768 mode, type:

TNIMAGE -vesa -mode 105 <Enter>

or

TNIMAGE -mode 105   (The word "Vesa" is only

required if you happen

to have a Tseng or

Trident chip and you

want to bypass direct

chip addressing).

If you have an old card, specifying "-vesa" will cause display

problems. If problems occur, use:

TNIMAGE -oldvesa

or

TNIMAGE -oldvesa -mode 105   to select 1024x768 mode.

To run TN-Image on a computer equipped with a

Trident card, in 640 x 480 mode, type:

TNIMAGE -mode 5d

or

TNIMAGE -trident -mode 5d

To run TN-Image on a computer equipped with a card

containing a Tseng Labs ET4000 chip, in 800 x 600 mode, type:

TNIMAGE -mode 30

or

TNIMAGE -tseng -mode 30

To run TN-Image and automatically load and display

the image "test.img", type

TNIMAGE test.img

or

TNIMAGE -file test.img

To run TN-Image in a 1600 x 1200 resolution at 8 bits/pixel, type:

TNIMAGE -xres 1600

or

TNIMAGE -xres 1600 -bpp 8

To run TN-Image in a 1600 x 1200 resolution at 24 bits/pixel (true

color), type:

TNIMAGE -xres 1600 -bpp 24

The command line options are stored in the file TN-Image.INI,

so the next time you start TN-Image, it is not necessary to

specify the screen mode.

7.1.1  Unix Command-line options

-n        =  Skip introductory screen in 8-bit modes.

-fyu      =  This option is for debugging use only.

-macro <macro_file>

=  Automatically runs the specified macro file. See "Batch

mode processing"

-files <file_list>

=  Specifies a list of image files to be automatically

loaded. Using a text editor, create an ordinary text

file containing a list of the files to be read, with one

filename on each line, for example:

t*.tif

~/images/abc.bmp

hubble.img

Then start TN-Image with the command:

xtnimage -files test.lst

substituting the name of your list for "test.lst". Do

not put a dash before the filename of the list file.

Wildcard characters such as '*' or '?' are also

permissible. Up to 512 images can be loaded from

the list file (if you have the Registered version).

If you only need to load a few files, it may be easier to

use the "<filename>" option (below) instead.

<filename> <filename> ...

=  Reads the specified file(s). You can load as many files

as you can cram onto the DOS command line.

If you have a large number of files, it may be easier

to use the "-files" option (above).

Do not precede the filename with a dash ("-"). Filenames

may contain path specifications and wild cards (*  or ?).

Examples:

xtnimage my.gif      = Loads my.gif

xtnimage *.tif       = Loads all TIF files in

current directory.

xtnimage ~/images/*.tif /mnt/abc.img

= Loads all TIF files from

the /images directory,

then file abc.img from

/mnt.

Any combination of command

line options is also accept-

able.

? or     =  List the available options.

-?

-fg or -foreground = Specify foreground color

-bg or -background = Specify background color

Example:  xtnimage -fg white -bg MistyRose3 &

-geometry <geometry-string>

Example:  xtnimage -geometry 400x500+100+100

-display <screen-IP-address>

Example:  xtnimage -display 192.168.1.1:0.0 &

See ' man X ' for more details.

The options can be in any order, except that if a parameter

is needed, it must follow the option.

The arguments must be separated by spaces.

To run TN-Image and automatically load and display

the image "test.img", type

xtnimage test.img

7.2.0  Video cards supported (DOS version)

Currently two SVGA chips are directly supported:

a) Trident 8800, 8900, or more recent chips. Trident cards seem

to be popular in many local computer stores.

b) Tseng Labs ET4000  or more recent chips. Note: the older Tseng

ET3000 chip is not supported. The ET4000 chip is in many cards

made by a variety of manufacturers.

If these two chips are not detected, TN-Image will use VESA

super VGA functions.

Other cards will also work if they are VESA-compatible. To use a

VESA-compatible card, you may need to first install a VESA  BIOS

extension,  which is usually a small program that came with your

video card, and may be named  "VVESA.COM"  (for  Mach32  cards),

"VESART.COM"   (for   Realtek  cards),  or  something   similar.

Installing this VESA BIOS means you have to run the VESA program

before running TNEDIT. This is conveniently done by  putting  it

in  your  AUTOEXEC.BAT file. Newer cards often have VESA already

installed in the card in ROM so no action needs to be taken.

Certain early video cards interact with the mouse cursor, making

smooth movement of the mouse difficult. Some older mouse drivers

also cannot handle SVGA modes smoothly. It should be possible to

position  the  mouse  at  each  point  on  the  screen.  If  you

experience  difficulty  reaching  odd-numbered  pixels,  try  to

obtain  a  newer  mouse  driver. If this doesn't help, it may be

necessary to upgrade to a better  SVGA  card.  This  problem  is

known to occur with some RTG3106-based Realtek cards.

If you have one of the above video cards and TN-Image still

doesn't work on your computer, please let us know. We will try

to fix it for you.

ATI video card users:

Computers with Mach 32 ATI cards may have problems running TN-Image

in true-color modes from within a DOS box. The only known solution

is to run TN-Image from DOS, or use TN-Image in an 8 bit/pixel

mode (e.g., mode 103). (See "Command-line options" above).

If your ATI video card won't go into desired screen mode, even though

the manual states that mode is supported, run ATI's "Install" program

again to verify the specified video mode is activated. If this doesn't

work, it is possible that the computer was turned on before turning on

the power to the monitor. ATI cards only examine the monitor type at

power-up, and could become confused if the monitor is off during a cold

boot-up, and refuse to set high-resolution modes.

The display in TN-Image is corrupted when using ATI Mach 64 cards

if the UNIVBE TSR is present.

7.3.0  Screen modes supported  (DOS version)

************************* WARNING! ****************************

CHECK YOUR MONITOR MANUAL *BEFORE* ATTEMPTING TO RUN TN-IMAGE

IN A SCREEN MODE OTHER THAN THE DEFAULT MODE!  THE AUTHOR IS

NOT RESPONSIBLE FOR ANY MONITOR DAMAGE CAUSED BY ATTEMPTING TO

SET YOUR MONITOR TO A MODE WHICH IT IS UNABLE TO HANDLE. MOST

VIDEO CARDS WILL REFUSE TO SET AN UNSAFE MODE, BUT SOME WILL!

**************************************************************

The mode value `xxx' on the command line can be one of the following:

Video card/chip   Mode     Resolution    Colors   Video RAM  Bits per

Needed     Pixel�

---------------  ------    ----------   --------  ---------  --------

Tseng ET4000/W32   2E       640 x 480         256  1/2 meg     8

30       800 x 600         256  1/2 meg     8

38      1024 x 768         256  1 meg       8

2E1 +    640 x 480      32,768  1 meg      15

301 +    800 x 600      32,768  1 meg      15

381 +   1024 x 768      32,768  2 meg      15

2E2 +    640 x 480      65,536  1 meg      16

302 +    800 x 600      65,536  1 meg      16

382 +   1024 x 768      65,536  2 meg      16

2E3 +    640 x 480  16,777,216  1 meg      24

303 +    800 x 600  16,777,216  2 meg      24

383 +   1024 x 768  16,777,216  3 meg      24

Trident 8800/8900  5D       640 x 480         256  1/2 meg     8

5E       800 x 600         256  1/2 meg     8

62      1024 x 768         256  1 meg       8

VESA              100       640 x 400         256� 256 k       8

101       640 x 480         256  1/2 meg     8

103       800 x 600         256  1/2 meg     8

105      1024 x 768         256  1 meg       8

107      1280 x 1024        256  2 meg       8

110       640 x 480      32,768� 1 meg      15

111       640 x 480      65,536  1 meg      16

112       640 x 480  16,777,216  1 meg      24-32

113       800 x 600      32,768� 1 meg      15

114       800 x 600      65,536  1 meg      16

115       800 x 600  16,777,216* 2 meg      24-32

116      1024 x 768      32,768� 2 meg      15

117      1024 x 768      65,536  2 meg      16

118      1024 x 768  16,777,216* 3 meg      24-32

119      1280 x 1024     32,768� 3 meg      15

11a      1280 x 1024     65,536  3 meg      16

11b      1280 x 1024 16,777,216* 4 meg      24-32

120�     1600 x 1200        256  2 meg       8

1600 x 1200     65,536  4 meg      16

1600 x 1200 16,777,216* 6 meg      24-32

* Most cards use mode 115, 118, and 11b as 32-bit/pixel

modes for speed purposes, but the maximum no.of colors

is still only 16,777,216.

+ Experimental - awaiting testing with W32 card.

Any feedback as to whether these modes work is

appreciated.

� Some video cards, such as the #9GXE, do not support any 15-bit

screen modes.

� #9GXE64 card does not support this mode.

� Numbering for screen modes above 11b may vary from card to card.

All VESA modes with 8 or more bits/pixel are available using the

-xres and -bpp options (Sec.7.1.0).

If after trying the above options, you still cannot get TN-Image

to run on your machine, you can try utilities such as UNIVBE,

which can permit your Super VGA card to use VESA modes.

If your monitor makes a buzzing or squeaking sound and/or fails to

synchronize (evidenced by jagged diagonal lines or other unusual

effects), turn the monitor off immediately and press Alt-X several

times to stop the program. This could occur if your video card reports

that it is safe to set a given screen mode when in fact it is not

safe. Although it should be the responsibility of the video card to

determine whether it is safe to set a given screen mode, this

feature is absent from many cards. All SVGA monitors, however,

should be capable of running safely at the default screen mode

(800x600 pixels).

===========================================================================

7.4.0   Basic operations

Most operations, such as filtering, saving images to disk, etc. use

a "selected region" which has to be selected before you perform the

operation.  To select a region, move the mouse to one corner of the

desired area, click and drag to the other corner, then release  the

mouse  button.  A rectangle shows the selected region while you are

dragging. This selected region stays in effect until you  (1)  quit

the  program, (2) select another region, (3) pick "unselect region"

from the menu, or (4) double-click on an image to select an  entire

image.  If  no  region has been selected, or it was unselected, the

region defaults to the entire screen. If you  make  changes  to  or

select  a  region  that  is  partly  on  an image and partly on the

background, the portion of your changes that  falls  on  the  image

will  remain  with  the  image,  and  the  rest  will  stick to the

background.

The  background is maintained independently of the images. Thus, if

an image is  moved  over  the  background  or  another  image,  the

background  is  not  disturbed. When many images are simultaneously

present, it is often convenient to keep most of them out of the way

by stacking them up or moving them out of the  visible  region, off

the edge of the screen. If an image gets lost off the edge, you can

find its coordinates by selecting "About the program" or "About the

image".

You can select an entire image by double-clicking on that image. To

change to a different image, single-click on  the  other  image  to

bring  it  to  the  foreground  or double-click on it to select it.

Selecting an image means that, all subsequent  operations  will  be

performed on the entirety of that image.

You can move an image around by clicking on the small arrows on the

top  menu  bar.  The  distance by which a single click on the arrow

moves the image is adjustable by pressing the gray "+" or "-" keys,

or by selecting "Cursor movement rate" from the "Configuration" menu.

When the mouse cursor passes over an edge of an image,  it  changes

from a diagonal arrow to double arrows. If you click the left mouse

button  at  this  point, the image will be "grabbed". Keep pressing

the mouse button, move to a new location,  and  then  release.  The

image is then moved to the new location. The same is true for mess-

age boxes, graphs, dialog boxes, the color palette window,  and in-

formation windows.

=========================================================================

7.5.0   Operating under low-memory conditions

If you have 4 MB or less of RAM, or use large images or a lot of

images simultaneously, or work in a Windows DOS box, eventually you

may run out of memory.

There are several things you can do to increase available memory:

(1)  In  the  "Config"  menu,  turn off the "Automatic undo" option

before loading any images. This will double  the  amount  of  space

available for images, but TN-Image will not create an "undo" buffer

for each image. This means you will not be able to  erase  anything

from  the image (such as text or graphic elements) without creating

blank areas on the image. You can still back up and restore  images

manually, however.

(2) Start TN-Image in a  lower  screen  mode.  In  lower resolution

modes,  less  memory is needed to manipulate images and background.

(See under "Command-line options"). Try using 8-bit/pixel modes

instead of true color modes. TN-Image reserves a fixed amount of

memory for drawing dialog boxes and error messages. The amount of

reserved memory is also screen mode-dependent.

(3) In TN-Image registered version, memory is limited  only  by the

available hard disk space. You can create more  virtual  memory  by

using a disk utility to unfragment your hard disk, or you can erase

unneeded files.

(4)  If high resolution is not essential, images can be loaded in a

small size.  Loading an image with  "X size=50"  and  "Y size=50"

uses  only  �  as  much  memory  as  a  full-size image. (See under

"File menu...Load image").

(5). If you have more than one image  loaded,  unload  one  of  the

images using "File... Unload image". This removes it and its backup

from memory.

(6). Erase FFT's as soon as possible by selecting "File...Erase FFT".

Fourier transforms use large amounts of memory.

7.6.0  Commonly asked questions

Q. How do I add text?

A. Move the mouse cursor to the desired location and begin typing.

It is not necessary to click the mouse before typing. Make sure

the mouse does not move while you are typing. The text will stick

to the foremost image, or if no image is present, to the background.

Unix versions can also select the font, font size, and weight.

Q. How do I undo an operation?

A. Select "Process...Restore". The image will be restored to its state

at the last time you backed it up. If you never backed it up, it will

be restored to its original state -- unless the "Auto-Undo" option

has been turned off.

Q. How do I turn off box- or line-drawing mode?

A. Click anywhere on the top menu bar or pull down any menu. This

automatically turns off everything. Alternatively, click on the

small rectangle which displays the current mode on the right side

of the menu bar.

Q. Is there an easy way to change the color of text, lines, etc.?

A. Yes, click the left mouse button on the palette to select the

foreground color. Click the right mouse button on the palette to

select the background color.

Q. When filtering, changing contrast, etc. of color images, why does

junk sometimes briefly appear on the screen?

A. If TN-Image is in an 8-bit screen mode, manipulating 16 and 24-bit

images temporarily causes the image to be filled with 16 or 24 bit

data, which appears as random noise because of the discontinuous

palette. After the operation is finished, the palette is automatically

recalculated, and the image will appear normal again.

Q. How can I combine two or more pseudocolor images into one image so

that they have the same palette?

A. Start TN-Image in a color mode by typing:   tnimage -xres 800 -bpp 24

(for example). Load both images, then select "Create image" and select

a region covering both images, using the mouse. Select "Change

image depth" to convert the image to 1 byte (8 bits) per pixel. A new

8-bit palette will be generated to fit both images.

Q. Why is <xxx> image file format not supported?

A. Some image formats (such as GEM IMG) are not commonly used in image

analysis. Others (such as lossless JPEG, LZW-compressed TIF, and

creation of GIFs) are not supported because of patent restrictions.

JPEG is not included in the Shareware version because it requires

virtual memory. Requests for other formats are welcome.

Q. Is a Windows-9x version of TN-Image being planned?

A. This has not been decided. For now, TN-Image has an icon and PIF file,

and works perfectly in a DOS box. TN-Image is very demanding on the

hardware and will always run better from the DOS command line than

in Windows.

Q. Is a Windows-3.1 version of TN-Image being planned?

A. No.

Q. Is a Macintosh version of TN-Image being planned?

A. No.

Q. Is a Unix (X-Window) version of TN-Image being planned?

A. TN-Image is currently being ported to the X Window system. Several

UNIX versions, including Irix, SunOS, and Linux will be available.

Contact the author for availability dates. Registered users will be

able to obtain a free copy by ftp or by mail for a nominal charge.

Q. Will lower-resolution screen modes (such as 640 x 480 x 16 colors)

ever be supported?

A. No. Reducing the number of colors to 16 would render any quantitative

analysis of the image almost meaningless. Users are advised against

using 4-bit images for any scientific work, as many such images that

have previously appeared in the literature are now being questioned.

7.7.0  Batch mode processing

TN-Image can also be run in "batch" mode for unattended operation.

This is useful for operations that are time-consuming or need to be done

every day. You can use a cron-like utility or a DOS command scheduler

such as TNSHELL (available from the author) to automatically carry out

any sequence of operations at (for example) 3 a.m. on Thursdays or to

automatically load a series of images before you come to work. The

computer's mouse and video card must still be present, and if your monitor

is turned on the images will still appear on the display.

Procedure:

1. Create a macro in TN-Image and save it to disk. The last command in

the macro should be "exit" to ensure TN-Image quits when finished.

The command "messages 0" can also be used to ensure that any error

messages are ignored instead of causing the program to wait for an

acknowledgement (Make sure the macro works before using this!).

2. Using TNSHELL (or other program), set the following DOS command to

be executed at the desired time (e.g., 3 a.m. every Saturday):

TNIMAGE -MACRO <macro_file>

where <macro_file> is the name of your macro (without brackets).

See "Macro programming guide" for examples of useful batch macros.

7.8.0  Editing text files

The Macro Editor in TN-Image can also be used as a text editor for small

(up to 256 lines) text files. This can be used to take notes on your

image analysis, create color-remapping files, etc. as well as writing

macros.

=========================================================================

8.0.0  Menu options

Menus, dialog boxes, message boxes, and click boxes can all be moved

by moving the mouse to the edge of the object and clicking the

left mouse button.  Move to the desired location and release the

mouse button.

8.1.0  File menu

8.1.1  Load image

Reads an image from a disk file.

Vertical/Horizontal - Selects whether the image should be

loaded in its normal orientation

(vertical) or rotated 90� (horizontal).

Positive/Negative   - Selects whether the image is to be loaded

normally or as a negative (color-inverted)

image.

Filename            - Name of the image to be read. The previous

20 filenames are kept on a stack and can be

selected with the up and down arrow keys.

Alternatively, if you press <Enter> when no

filename is showing, you can select a file

from a menu. Wildcard (* or ?) characters

are permissible; thus, "*.pcx" is a valid

filename, and will cause all matching files

to be loaded.

X position          - Horizontal starting pixel position to place

the image (in pixels).

Y position          - Vertical starting position to place the

image (in pixels).

X size              - If x size is 100, the image is loaded in

its actual size. If set to some value

between 0 and 100, the image is shrunk in

the x direction.

Y size              - If y size is 100, the image is loaded in

its actual size. If set to some value

between 0 and 100, the image is shrunk in

the y direction.

Auto File Type/ Raw bytes -

Select `raw bytes' if you wish to read the image as a series of

bytes and override the automatic image file type detection. (If

the  image  is  an unknown format, "raw bytes" mode is selected

automatically).

Invert byte order   -

Certain other imaging programs save the RGB values in the wrong

order.  This  could  also  happen  with  certain  video  cards.

Clicking this option will switch the byte order to the  correct

value.

CMYK->RGB           -

If  the  TIF  file  happens to be a 32-bit CMYK image, TN-Image

will  automatically  convert  it  to  RGB  format  for  display

purposes. Occasionally, an image file header says the image  is

a  CMYK  image but in actuality it is not. Un-clicking this

option will cause TN-Image to treat the image as if it were  an

ordinary  RGB  image.  This option has no effect on images less

than 32 bits/pixel.

Convert to gray scale

Changes the image to shades of gray  instead  of  its  original

colors. This is particularly useful for CT or MRI images of greater

than 8 bits per pixel, because the gray scale can be remapped

as a "sliding scale" to enhance specific regions of the image.

Color reduction     -

Quantization/Fit current palette/None

If TN-Image is in an 8-bit/pixel (indexed-color) mode, when a

color image is loaded the number of colors must be reduced to

256 for display purposes. This can be done by "quantizing"  the

image or by trying to fit the image to the currently-selected

palette. Even though the quantization algorithm used is new and

one of the fastest known, converting the image to 8 bits/pixel

often still takes much longer than reading it from the disk.

Quantizing is more general than palette fitting, and is guar-

anteed to give a viewable image. Many GIF files were produced

with quantizing.  However, because it changes the palette, all

other images currently being viewed will become "garbage" if

TN-Image is in an 8-bit mode (Of course they are not really

garbage, but temporarily unviewable). Clicking on an image restores

the palette for that image.

Palette fitting uses less memory than quantizing. If the error

message "Insufficient memory to convert to 8 bits/pixel" appears,

try changing the color reduction method to "fit current palette".

This can be quite slow, however, for certain types of images.

Because quantized palettes are usually not a continuous flow

of colors, it may not always be possible to perform all types of

quantitative analysis or filtering on a quantized 8-bit image.

The best solution is to select "Change color depth" and convert

the  image to a 24 bits/pixel color image. The colors will then

work as expected during filtering.

Fitting to the current palette does  not  affect  the  other

images  on  the  screen,  and  gives  a  result  that  is still

quantitatively analyzable and filterable. Also, if the original

colors in the image are similar to those in  the  palette,  the

result  will  be a much smoother image. However, if the palette

is substantially different, none of the colors  may  match  up.

(Of course, you can easily change the palette and repeat). This

method incidentally uses much less memory than quantizing.

If  TN-Image  is  in a color mode, this option has no effect

except when saving images to disk when "8-bit/pixel palette" is

specified as the file type. Color images  are  filtered  in  an

identical  manner regardless of whether or not TN-Image is in a

color mode. The palette is also used in  color  modes  when  an

image  with  8 or less bits/pixel (i.e., a indexed-color image)

is loaded.

All 8,16,24, and 32-bit images are kept in memory in  their

original  color  depth,  regardless of the current screen mode.

Other images are converted at read time  to  the  next  highest

multiple  of  8. This means that you can safely edit full-color

images even if your computer's video card can only  handle  256

colors. Editing operations also are handled in a manner approp-

riate for each image. Thus, for example, you can select an area

encompassing both an 8-bit image and a 24-bit image, and filter

them  simultaneously,  with no problems. TN-Image automatically

knows the color depth of each pixel being processed and handles

it appropriately.

See also under "Color settings" for more details.

Images between 1 and 7 bits/pixel are automatically converted

to 8 or 16 bits/pixel when they are loaded. The conversion

makes use of whichever palette is selected. If you have not

specifically changed the palette, the default "spectrum" palette

will be used. This can result in an unexpected color appearance.

8 bit/pixel images are a special case, because they are usually

saved with their own palette.

If TN-Image cannot identify the image type, or finds an invalid

file, it asks if you want to try to read it anyway as raw bytes.

It then makes a guess as to the x and y sizes, and  bits/pixel.

You should correct these as needed. In order to read raw bytes,

the image should have a constant no. of bytes per scan line, be

uncompressed,  and  have  the  image rows stored in consecutive

order. You may also have to skip a number of bytes to  get  the

image to line up horizontally. This number has to be determined

experimentally. If the file is not really an image file, in all

likelihood you will just get junk on the screen.

This feature allows you to import other types of files that are

not  normally  thought  of  as  images.  For example, to read a

database of 16 x  16  Chinese  characters,  use  the  following

parameters:

Filename:     ziku  (or name of the database)

x pixels:       16

y pixels:   <length of file � 16 > , e.g. 15000 or so.

bits/pixel:      1

bytes to skip:   0

The  database  will  be  loaded  as  a  long  strip  of Chinese

characters, which then can be cut and  pasted  as  desired.  Of

course, this is not the most efficient way to edit Chinese, but

it demonstrates the usefulness of the feature.

See the following section for a detailed procedure for reading

raw bytes and creating custom formats.

If a single-color image containing between 1 and 8 bits/pixel is

loaded, the values are automatically filtered through the currently-

selected palette. If the image itself contains a palette, the palette

in the image is used. This is not done for single-color images greater

than 8 bits/pixel, because of the large size of the palette table that

would be required (usually, only 8-bit images actually contain a

palette).

8.1.1.1 Reading CT, X-ray, or MRI images, or raw bytes

Below is a detailed example for reading images produced by X-ray,

CT or nuclear magnetic resonance scanners and other equipment. These

machines typically use unusual file formats not recognized by TN-Image,

but usually the images can easily be read as "raw bytes".

The same procedure is applicable to any non-compressed file of

unknown type.

NOTE:

The Registered version of TN-Image can be used to create a "custom"

image file format, which is a superior method for reading medical image

files, because once the custom format is set up, reading the image is

transparent to the user, obviating the necessity for reading "raw

bytes" as described here (See "Create File Format").

(1) Select "File...Load image" and enter the image file name.

(2) Click on "Raw bytes".

(3) Click on "OK".

(4) TN-Image will make a guess at the image width. If the image width

is known, enter the image width and height. Otherwise, if the guess

turns out to be wrong, use the auto-increment method described below

to determine the width. In this case, the width happens to be 381.

(5) Select the bits/pixel of the image. This is usually 12 or 16 for

CT images (MAMOGRAM.LUM is 16 bits).

(6) (Optional): If the length of the image file header is known,

enter this  value under "skip bytes". Otherwise, leave it as 0.

(7) Click on "OK" to load the image.

(8) Once the image is loaded, if the skip bytes were incorrect in

step 6, the image border will be visible as a vertical strip in

the image. Use the mouse to locate the X-position of this strip.

Unload the image and re-load it, using the X-position * bytes/pixel

as an estimage for skip bytes.

(10) If the image was color, it may be necessary to add 1,2, or 3

additional skip bytes to get the colors to line up correctly.

(11) If the image is grayscale, the grayscale mapping can be adjusted

by selecting "Color...Change palette...Grayscale brightness/

contrast". This allows you to interactively adjust the appearance

of the image as a sliding scale, to highlight areas of different

intensity (See "Grayscale brightness" below).

(12) The image can now be saved in a conventional file format.

Determining the width of an unknown image:

TN-Image has a unique feature for determining the width of an

image whose dimensions are completely unknown. This procedure is

substituted for step (4) above:

(4a) Make a conservative guess of the image width. For instance, if

the image is known to be square, and it is 16 bits (2 bytes)

per pixel, use

width =  sqrt( file size / 2 )

as your starting estimate.

(4b) Click on "Auto increment". This will cause the width to be

increased with each scan line.

(4c) For "Increment", enter "1" or "2".

(4d) Click on "OK" to load the image. The image should consist of

a swirling pattern, with fuzzy lines in a horizontal "U" shape

like this:

\\\   \   \\

\\\   \   \\

\\\   \   \\*

///   /   //

///   /   //

Usually, the image is slightly clearer at the vertex (marked

with a *). If so, find the Y-coordinate of the vertex using

the mouse. Subtract 32 (the size of the menu bar) from this

value and add the result to your previous estimate of the

image width.

(4e) Unload the image.

(4f) Turn off "Auto increment" and load the image again. The image

should now be recognizable. If the image slopes to the right

as you move down the screen, this means the value is still too

low. Increase the image width slightly and try again. If it

slopes left, the value is too high.

If the image is still not recognizable after these steps,

try changing the byte order, or check the "Convert->gray scale"

option.

If you selected the wrong number of bits/pixel or y size for

a "raw bytes" image, the automatic grayscale detection

may estimate tne maximum and minimum gray levels incorrectly.

This will result in an image that is too dark or too light.

This can be easily corrected by remapping the gray levels

(See "Grayscale intensity mapping").

8.1.1.2  Reading Imaging Systems format files

The .IMG files produced by software from Imaging Systems Co. are

readable by TN-Image; however, their 4-byte header gives a value

for the image width that is off by 1. Thus, in order to read the

image properly, it is necessary to read the image as "raw bytes".

After manually changing the width from 511 to 512, the file will

read normally.

8.1.1.3  Reading Images from Macintoshes

Files imported from Macintosh via Mac-TCP are corrupted by the

addition of an extra header. This often creates the impression

that PC-based programs are incapable of reading Macintosh images.

TN-Image, however, automatically detects the presence of the header

and eliminates it before reading any image.

8.1.1.4  Reading Raw ASCII images

Raw ASCII images consisting of a series of integers are readable

by TN-Image. To read an image in ASCII format, you must select

"Raw ASCII" as the File Type in the Load Menu. Then set the

remaining parameters the same way as for "Raw Bytes" (Sec. 8.1.1.1).

Some of the options (such as bit packing) do not apply to ASCII files

and their setting is ignored.

8.1.2  Save image

Transfers an image or selected portion of the screen to a disk file.

File format

Format in which to save the image. See "Custom format" below

for details on creating files in your own customized format).

Save entire image/selected region -

If "entire image" is selected, the currently-selected image will

be saved in its entirety. Otherwise, only the currently-selected

screen region (which can beyond an image or contain  2  or  more

images)  will  be  saved. The actual size of the region saved is

displayed after the file is written.

Bits/pixel -

You  can  select  standard  values  of  1,8,15,16,24,   or   32

bits/pixel,  or  select  "other"  to  specify  any  other value

between 1 and 32. The resulting file size  is  proportional  to

the  bits/pixel.  This  can result in a considerable savings in

disk space. Monochrome (1 bit/pixel) is ideal for line drawings

and graphs. Don't forget  that  saving  an  image  in  a  lower

bits/pixel than it was created causes a loss in color accuracy.

For  example, if you save an 8-bit image as a 3 bit/pixel image

file, the colors in the image will look slightly different from

the original colors when you load it back. If you save it as  a

monochrome  (1  bit/pixel)  image,  all pixels above 50% of the

maximum color will come  out  white  and  those  below  50%  of

maximum will come out black.

NOTE - Some file formats (e.g., GIF and PCX) only allow certain

values  for  bits/pixel and number of colors. TN-Image displays

an error message if an illegal value is selected.

WARNING - Due to limitations in  the  capabilities  of  some

other  programs,  saving images using "non-standard" bits/pixel

or using "non-standard" numbers of colors can result  in  image

files  that  are  not readable by some other image viewers. For

example, one well-known  shareware file interconversion program

can only handle 4,8, and 24 bit/pixel TIF images, but  not  the

common  15  and 16 bit images. One commercial Macintosh program

can only handle 8,15,24, or 32 bit/pixel  images,  but  not  16

bits/pixel.  Very few other programs can handle odd values such

as 12 bits/pixel. However, these values can be extremely useful

and widely used in handling radiological, electron  microscope,

or MRI images, which are often of unusual pixel depths.

TN-Image  will  display  an appropriate warning if you select a

"non-standard" file format.

Treat data as Color / Gray scale -

When  saving  images  greater  than  8  bits per pixel as TIF or

Custom files, it is sometimes useful to save the image as if the

image data represent shades of gray instead of colors. Selecting

"gray scale" causes TN-Image to treat the image as if each pixel

value was already a "luminosity", and skips the conversion  from

RGB to luminosity. This setting is ignored if the image is saved

as  8 or less bits/pixel, or if it is saved as a color image (by

selecting "2" or "3" as the number of primary colors). Normally,

this setting should be kept on "Color".

Filename            -

The file name under which the image will be saved (can include

drive and path). The previous 20 filenames are kept on a stack

and can be selected with the up and down arrow keys.

Image no.           -

Indicates  the  currently-selected  image which will be saved if

you have selected "save entire image". Has no effect if you  are

saving   a   "selected   region".  This  option  is  mainly  for

informative purposes and does not normally need to be changed.

Extra TIF Param.(for `Other') -

If you select "Other", it is also possible to customize several

additional parameters. Currently, these values only have an effect

for TIF and "custom" file formats.

No.of primary colors(1-4) -

If you select "1", the image will  be  converted  to  grayscale

("luminosity")  before  saving,  and  the red, green, blue, and

black bits/pixel specified below are  ignored.  If  you  select

more  than  1  color,  the  number of bits for each color, when

added together, must equal the total number of  bits/pixel  you

specified  under  "other  bits/pixel".  If they don't, TN-Image

displays a message to that effect.

Red bits/pixel     No.of bits of red to save.

Green bits/pixel   No.of bits of green to save.

Blue bits/pixel    No.of bits of blue to save.

Black bits/pixel   No.of bits of black to save (CMYK files only).

For example, to save only 4 bits each of the red and blue planes

of your image, set the following values:

`Other' bits/pixel (on left side of

dialog box)   = 8

Number of primary colors = 2

Red bits/pixel   = 4

Blue bits/pixel  = 4

Green bits/pixel = 0

Black bits/pixel = 0

NOTE: TN-Image currently cannot display more than 8  bits/pixel

of each color (i.e., a total of 24 bits/pixel).

This is due mainly to limitations in the current

generation  of  PC  video cards. Thus, there is no advantage in

setting the bits/pixel for any one color higher than "8".

You can only specify black bits/pixel if the "cmyk"  option  is

checked.

RGB->CMYK  -

Convert the image from "RGB" to "CMYK" (cyan, magenta, yellow &

black) format when saving. (When reading a CMYK  image,  it  is

automatically  converted  to RGB, since there cannot be a "CMYK

computer monitor"). If you are using a non-standard  bits/pixel

mode,  you also need to specify the number of red, blue, green,

and black  bits/pixel  (the  values  will  be  changed  to  the

corresponding   c,m,y,   and   k   values).  If  you  select  a

non-standard format, TN-Image will display a  warning  to  that

effect.  Selecting  CMYK  is only allowed for the 32 bit/ pixel

standard mode and the "Other" (non-standard) mode.  If you  try

to  save an image with CMYK checked in any other mode, TN-Image

will  display  an  error  message.  This  is  because  the  TIF

specification (as of version  6.0)  does  not  recommend  using

lower color depths for CMYK.

This option is useful for exporting the image to other programs

which cannot create their own CMYK images for printing.

JPEG quality factor -

An integer between 1 and 99. Values above 90 or below 10 are not

recommended. Smaller values result in greater compression and

more signal loss.

NOTE: Jpeg is a lossy file format. This means an image saved in

JPEG format will lose some information. Images consisting of text

and sharp lines are not good candidates for Jpeg compression,

because the compression algorithm is optimized for realistic

scenes such as images from photographs. Files should only be

saved in Jpeg format when all image processing has been completed,

and the smallest possible size is needed.

Also, some settings, such as red, green, blue, and black bits/

pixel, and CMYK conversion, are ignored for JPEG.

8.1.3   Printing images

Prints the image or selected portion of the screen on a laser printer.

A laser or inkjet printer that uses a page control language similar

to Hewlett-Packard's PCL3 or PCL5, or any PostScript printer

supporting PostScript Level 2 or above is required. (PostScript

Level 2 is not to be confused with "PostScript Type 2 fonts",

which don't exist).

8.1.3.1 Printing Black-and-White or gray scale images

Most  laser  printers  are optimized to print text. This means the

pixels are printed by the printer as dark as possible and adjacent

pixels partially overlap. The effect that  this  has  on  graphics

printing  is  that  the  darkest  50%  of the image will appear as

black. For example, if you are printing at 8x8 dithering mode  (64

gray  levels),  shades  1-32 will appear white to black and shades

33-64 will all appear black. Thus, your gray-scale  resolution  is

effectively  cut in half. Although you can make your image lighter

before printing or select a lighter setting from TN-Image, the only

way to avoid any loss of resolution is  to  set  your  printer  to

print  lighter.  This  is  easy  to  do but varies from printer to

printer. See your printer manual for the procedure.

To  select  a  lighter  print  intensity  from  TN-Image,   select

`file...print' from the menu, then click on `printer color adjustment".

Adjust the red, green, or blue intensity factors as desired. 128 is

normal (dark) printing. Any other values will be multiplied by

the data going to the printer to make it lighter or darker.

If you are using a monochrome printer, be sure to select "B/W or

Grayscale". The red, green, and blue intensity factors will be

averaged for grayscale printing.

Currently, only laser and inkjet (bubble jet) printers are supported.

No support is planned for dot-matrix printers, because the

image quality would be unacceptably low.

Print quality can often be improved by using glossy paper sold for

pen plotters instead of regular paper.

----Column 1---

Vertical/Horizontal   - Selects whether image will be printed

in "portrait" (vertical) or "landscape"

(horizontal) mode.

Positive/Negative     - Print normally (white on the screen =

white on the paper) or as a negative.

Print entire image/ print selected area

- Select whether to print the entire

currently-selected image, or the

currently-selected screen area which

may span 2 or more images.

----Column 2---

No. of copies         - Number of copies of the image to print.

Not all printers accept this command.

Printer Device/file name - See Sec. 8.1.3.4

Color adjustment      - Darkness of printed image (255=darkest).

The red, green, and blue factors are averaged

when "printing type" is "B/W".

Vert. position (inches)- Offsets the image by the specified no. of

inches down the page.

Horiz.position (inches)- Offsets the image by the specified no. of

inches to the right across the page.

Media type               - Optimizes the output for the type of

paper being used.

The following table shows the differences

used for different media types:

Media    Print        No. of   Recommended

Type     Quality      passes*  Depletion*

-----    -------      -------  -----------

Plain     Draft          1       --

Normal         2      25% w/gamma

Presentation   4      50% w/gamma

Bond      Draft          1       --

Normal         2      25% w/gamma

Presentation   4      50% w/gamma

Premium   Draft          1       --

Normal         2      25% w/gamma

Presentation   4      50% w/gamma

Glossy    Draft          2      25%

Normal         4      25%

Presentation   4       0%

Transpar- Draft          2      25%

ency    Normal         4      25%

Presentation   4       0%

On certain printers, e.g., H/P 540, the

number of passes and the depletion are

determined automatically by the printer

and may differ from the value in the table.

On some other printers, depletion and no.

of passes cannot be changed, and setting

them to different values has no effect.

* Recommended by H/P.

For PCL printers, such as the HP LaserJet and most inkjet printers,

the following options also apply:

Resolution            - Selects the desired dots/inch resolution

to print the image. Normally this is

set to the highest value your printer

can handle (see discussion above).

Typical values are:

150 dpi     300 dpi

600 dpi    1200 dpi

This number must be set _exactly_.

For example, a 300 dpi printer may ignore

the setting, or it may print nothing at all

if the resolution is set at 299 or 301.

The resolution is also used to estimate the

default size for PostScript output.

Dither size           - Determines the number of separate gray or

color levels that can be printed:

dithering

size        no.of gray/color levels

----------    -----------------------

1x1              2

2x2              4

4x4             16

8x8             64

16x16           256

The size of the printed image also increases

proportionately with the dithering size.

No "error propagation" is performed, in order

to maximize resolution for text.

Print Quality

Draft

Normal

Presentation

- Selects the quality of printout. Presentation

quality takes longer, but gives slightly

better results.

Not all printers support this option.

Depletion -              - Improves image quality by removing a

Printer Default        certain percentage of the pixels. It is

None (0%)              recommended to leave this setting at

25%                    "printer default" unless unsatisfactory

50%                    results are obtained.

25% + gamma corr.        The highest two settings also apply

50% + gamma corr.      gamma correction to the printed image, if

this feature is supported by your printer.

Gamma correction produces a smoother

transition from one color to the next.

See the table above for recommended

depletion levels.

Not all printers support this option.

Printer gray balance     - If checked, the internal gray balance

adjustment in the printer will be activated

if present. This is supposed to adjust the

colors so that blacks appear black instead

of dark green. Do not check this box if

you have changed any of the values under

"Printer color adjustment" (see above),

since this would result in correcting the

gray balance twice. If your printer does

not support this option, checking this

will have no effect.

For PostScript printers, the following options apply:

Horizontal image size (inches)

Vertical image size (inches)

- The output will be scaled accordingly to

shrink or enlarge the image to the specified

size. The default values are calculated on

the basis of the DPI value entered under

"Printer resolution".

Rotation (Degrees)      - The printout will be rotated

counterclockwise by the specified angle.

The angle is centered at the lower left

corner of the paper. Note that a rotation

may cause part of the image to be off the

page unless 'horizontal position' is also

adjusted.

Interpolate             - If checked, and if the output resolution

of the printer is high enough, additional

pixels will be added to smooth the output.

NOTE: PostScript Level 2 or above is required.

8.1.3.2 Color printing

Color printing uses the same parameters as B/W printing, except

you must select RGB, CMY, or CMYK color instead of grayscale.

Color printing can take a much longer time than B/W printing,

and requires more printer memory on a laser printer. Use B/W mode

whenever possible. The choice between the color types is subjective;

however, CMYK generally gives darker blacks.

NOTE: You do not have to convert your image to CMYK format to

print it in CMYK mode.

Printing type -          - Selects color printing mode.

B/W (grayscale)       - Recommended for non-color printers and

for monochrome images. The pixel values

are sent to the printer without

filtering them through the color

palette. Thus, if you made the image

lighter or darker by dragging the

palette, these changes will not be

reflected in the printout.

RGB /indexed color    - Not recommended for PCL inkjet printers,

because some PCL printers insist on

interpreting this command to mean that

all pixels not explicitly defined are

to be printed as "black". This causes

a wide black stripe to be printed next

to the image, and causes printing to

take a much longer time.

For PostScript printers, this is a

good setting to use for both grayscale

and 8-bit color images.

If the image is 8 bits, the colors

or gray levels are determined by the

palette. If it is a true-color image,

the RGB values are used directly.

Prints using red, green, and blue inks

if available (not supported by all

printers).

CMY color             - Recommended selection for PCL color

printers.

Not recommended for black & white printers.

CMYK color            - For color printers - gives darker blacks

but takes 33% longer.

Printer color adjustment - Adjusts each color to allow you to match

the printer output with the screen. For

each color, a value of 128 (the default)

is "normal". Increasing the value will

increase the intensity of that color.

If CMY or CMYK is selected, the cyan,

magenta, and yellow inks are changed.

If RGB is selected, the red, green, and

blue inks are changed.

If B/W-grayscale is selected, the black

value is taken as the average of the

three color values.

NOTE: For CMYK printing, it is important

to keep the 3 colors balanced so that

the average is 128. Otherwise, the black

ink, whose value is derived from the 3

pigments, will also become lighter or

darker.

8.1.3.3 Printing under Unix

If you have multiple users on your system, or if you do not have

write permission for the printer, it is recommended to change this

to a regular file, e.g., "temp.print" and print the file later using

lpr.

Warning: if a device is specified that is not a printer or a

file, bad things will happen.

8.1.3.4  Printing to a file or another printer

TN-Image does not yet have the capability of printing over a

network. Therefore, for networked printers, it is necessary to

create a 'print file' and send the print file to the printer

using other software. This can be done by entering a filename

instead of a printer name under "Printer device/file name" in

the "Print..." menu. This will create a print file which can be

sent to the printer later, such as at night when it is less busy.

In DOS, printing of a file can be done using the command:

copy /b print.tmp lpt1

However, copy /b does not work correctly on some printers. In this

case, you can use 'tnprint.exe', which is a replacement for DOS's

'print.exe'. This program is available at the ftp site.

In Unix, printing of a file is done using the command:

lpr -Pmyprinter print.tmp

To schedule the printing at a specific time, use a program such as

TNSHELL in DOS, or 'cron' or 'at' in Unix.

If a printer device is entered, the image will be printed on that

device. "Printer device" refers to the printer name as it is known

to the operating system, for example:

DOS version:    lpt1   lpt2  prn

Unix version:   /dev/lp0  /dev/lp1 /dev/lp2 /dev/printer, etc.

8.1.4 Change Title

Changes the name of the image. This must be a valid filename.

If the title contains illegal characters (such as spaces or

control characters in the DOS version), they are automatically

removed and an error message is displayed.

8.1.5  Save Scan Data

Saves the data from the most recent strip densitometry scan

in a disk file. You are prompted for the filename. Do

not confuse this with "save image" - This will not save

a "scanned image"!

8.1.6  Create/Resize Image

You can create a new image by clicking and dragging with the mouse,

setting a fixed size and position, copying another image, or

resizing another image.

For 'fixed size', you must enter the x and y size (in pixels), and

the x and y position for the upper left corner of the image. The

screen contents at the specified location will be copied into the

new image.

For 'use mouse', use the mouse to select the region for the new image.

The screen contents at that location will be copied into the new

image. The other parameters will be ignored.

For 'copy another image', enter the image number of the image to

duplicate and the x and y position for the upper left corner of the

new image.

For 'resize another image', enter the image number of the image to

duplicate, the new x and y size (in pixels), and the x and y position

for the upper left corner of the new image. Any space beyond the

copied image will be set to black.

8.1.7  Switch to

Make some other image the currently-selected, foreground image.

This option has been replaced by "About the program...", which

can also switch images.

8.1.8  Create file format

Defines a new image file format. This allows you to add up to 100

new formats to TN-Image, or create your own customized format.

Once defined, the new format is automatically recognized by

TN-Image and can be loaded and saved without any special action

on your part.

If you know certain basic details about the image format,

or have a sample image file already in that format, you can easily

create an image file in almost any file format imaginable, which

will be readable by any Intel-based PC, Macintosh, IBM mainframe,

and many UNIX systems.

Custom formats are particularly useful if reading medical images

from an X-ray scanner, which vary from one manufacturer to the other.

Currently, only uncompressed custom formats are available.

Of course, with this much flexibility it is impossible to test

every possible combination of parameters on every destination platform,

so you should always make a test image before proceeding to make

sure the file is readable on your non-PC system.

*IMPORTANT*

Always test to make sure your custom file is readable on the target

system before erasing the original!

The information you need to know is:

(1) Bits/pixel that the destination system expects. This can be

calculated from the number of colors on the screen, using the formula

No. of colors =  2 ^ (Bits/pixel).

Thus, if there are only 2 colors (black and white), there is 1 bit

per pixel.

(2) Destination system - If the destination computer is not a

PC, Mac, or IBM mainframe, pick whichever seems closest. Many UNIX

systems use "big-endian"-type processors similar to the Mac. VMS

systems use "little-endian" processors similar to the PC.

(3) Size of the image file header. To get this information, you

can either guess, or view your sample image with a hex editor.

Often it is a power of 2 (32, 128, etc).

(4) Offset of the x and y size - This is the most difficult step

if you are unfamiliar with hex notation. If you can't make a guess,

if you are a registered user you can send us a copy of your sample

image and we will determine these numbers for you.

(5) The no. of primary colors (1 for grayscale or 3 for color).

Example 1: Creating a file format for images from a Lumisys X-ray

scanner.  No sample image file is needed.

1. Select "File...Create file format".

2. Set the following settings in the dialog box:

Target platform      -  Mac

Bit packing          -  None

Format Identification-  By Extension

Extension            -  LUM

Identifier           -  (none)(leave blank)

Bytes to skip        -  2048

Use header           -  (not checked)

Header file          -  (none)(leave blank)

Header bytes         -  0

Default bits/pixel   -  16

Default No.of colors -  1

3. Set the following file offsets:

x size               -  806

y size               -  808

Bits/pixel           -  810

All others           -  -1

Care must be taken not to allow the offsets (which take 2 bytes

each) to overlap with each other or with your identifier. The

identifier itself cannot exceed 40 characters. If an identifier

is specified, the image file must contain it at the specified

offset. If an identifier is not specified, the image file must

have the specified 3-letter file extension ("LUM" in this case).

"Bytes to skip" must be a multiple of 2 in this case, because

there are 2 bytes (16 bits) per pixel. If you skip an odd number

of bytes, the image will look strange because the light and dark

areas are partially reversed.

Similarly, if the wrong target platform is selected, the image

could look posterized or grainy, because the bytes are being put

into the wrong pixels. It is sometimes necessary to experiment

if you have an unknown image format.

4. Then click on "OK". The new file format has been created.

5. Reading and creating images in the new format will now be

transparent. The format descriptor file "FORMATS" can be used on

the shareware version of TN-Image. The shareware version cannot,

however, create files in the new format or create new formats.

Example 2: Creating an IMDS file. IMDS is a little-known and rarely

used monochrome format similar to GEM's IMG format, except that IMDS

was used on IBM mainframes running MVS, and there seems to be no

documentation for it.

1. Select "File...Create file format".

2. Set the following settings in the dialog box:

Target platform      -  MVS

Bit packing          -  TIF-like

Format Identification-  By Extension

Extension            -  IMG

Identifier           -  (none)

Bytes to skip        -  0

Use header           -  � (checked)

Header file          -  <Name of a sample IMDS file>

Header bytes         -  32

Default bits/pixel   -  1

Default No.of colors -  1

3. Set the following file offsets:

x size               -  12

y size               -  14

All others           -  -1

4. Then click on "OK". The new file format has been created.

5. Click on an image, or select something on the screen, and

save it in the new IMG format (by clicking on "Format" then

the last "IMG" item. Your sample IMDS file must always be

present to create image files in this format.

6. Test to make sure the image is readable. Since TN-Image

can already read this format, simply select "File...Read"

and press <Enter>. If you set the above settings correctly,

the image should appear as a black-and-white version of

your original.

The default bits/pixel and default no.of colors are needed only if

the these values are not in the header.

Example 3: Define a file format for raw byte images.

1. Select "File...Create file format".

2. Set the following settings in the dialog box:

Extension            -  RAW

Target platform      -  PC

Bit packing          -  None

Format Identification-  By Extension

Identifier           -  (none)

Bytes to skip        -  0

Use header           -  (unchecked)

Header file          -  (none)

Header bytes         -  0

Default bits/pixel   -  8

Default No.of colors -  1

Default x size       -  382 (substitute actual size)

Default y size       -  362 (substitute actual size)

Default red bpp      -  0

Default green bpp    -  0

Default blue bpp     -  0

Default black bpp    -  0

3. Check the file offsets to make sure they are all -1.

4. Then click on "OK". The new file format has been created.

Because "by extension" was specified, the raw image must have

the extension .RAW to be identified.

8.1.9 Save FFT

Saves the currently-selected FFT matrix to disk in ASCII format.

This allows you to precisely change any desired frequencies,

then reload the FFT and reverse-transform.

Warning: These files can be quite large.

8.1.10 Erase FFT

Erases the matrix used in storing the FFT, if a Fourier transform

has been performed on the image, thus freeing up a considerable

amount of memory. It will not erase the Fourier transformed image

that appears on the screen. If no FFT's have been performed, this

has no effect.

8.1.11  Unload image

Removes the currently-selected image from the screen and frees up its

memory. Also erases the image's backup and its FFT if present.

If your image is large, and there are other images above it in RAM,

on rare occasions it may not be possible to free the memory right

away. In this case, it is occasionally necessary to select "unload

image" a second time.

8.1.12 DOS Command

Shell to DOS to execute a command or run a different program. Or,

to obtain a DOS command line, enter "Command". Type "Exit" to

return to TN-Image. DOS requires a small amount of memory below

1 megabyte to exit commands. Thus you could get an "insufficient

memory" error even if there is a lot of high memory.

This function is, of course, not available in the Unix version.

8.1.13  Quit

Return to operating system. If any images have been modified, you

are given an opportunity to save them. Alt-X also can be used to quit.

8.1.14  Load FFT

Reads a Fourier-transformed image stored in ASCII format.

The file format is as follows:

FFT of <image name>

xsize <x size in pixels>

ysize <y size in pixels>

Real

<Real frequencies from row 1>

...

<Real frequencies from row n>

Imaginary

<Imag frequencies from row 1>

...

<Imag frequencies from row n>

This format differs from the format used in TN-Image prior to

version 2.18. The header in these earlier files must be changed

before they can be loaded.

8.2.0  Image menu

8.2.1  Delete region

Erase part or all of an image or background. After selecting `delete

region', move to one corner of the region to erase, and click and

drag to the other corner. This rectangular region will be set to

the background color. (If there is another image behind the topmost

image, the obscured part will be unaffected.)

8.2.2  Crop

Similar to `delete region', except erases everything except the

currently-selected region, including  the  background  and  any

portions  of  any  image currently visible on the screen. If an

image is obscured by another image, the obscured  part  is  not

cropped.

8.2.3  Erase background

Removes  everything  on  the  screen  that is not a part of an

image, and sets it to the background color.

8.2.4  Move

Move part of an image or drawing to another region.

After selecting `move', click and drag to select the region to move.

The region is cut and pasted in the new location as soon as you

release the mouse button. If the pasted portion falls partly on an

image and partly on the background, the portion that falls

on the image sticks to the image and the part that falls on the

background sticks to the background.

You can change the mode of interaction of the moved region with the

background or other images by changing the "Pixel interaction mode"

from the "Config" menu.  For example, if the pixel interaction

mode is "subtract" the moved region will be subtracted from

whatever is there. (See "Pixel interaction mode" below).

8.2.5  Copy

Copy part of an image or drawing to another region.

After selecting `copy', click and drag to select the region to move.

The region is copied and pasted in the new location as soon as you

release the mouse button.  If the pasted portion falls partly on an

image and partly on the background, the portion that falls

on the image sticks to the image and the part that falls on the

background sticks to the background.

You can change the mode of interaction of the copied region with the

background or other images by changing the "Pixel interaction mode"

from the "Config" menu.  For example, if the pixel interaction

mode is "subtract" the copied region will be subtracted from

whatever is there. (See "Pixel interaction mode" below).

8.2.6  Change size

Creates a new image of a different size than the currently-selected

image. You can specify different size factors for the x and y

dimensions.  However, both x and y factors must be either greater

than or less than 1.0. For example, setting the x factor to 1.1 and

y factor to 4.78 creates a vertically elongated image.

If the  image  does  not  contain  fine  detail,  it  is  often

convenient  to shrink the image to x and y sizes of 0.5 or less

before saving it. Setting the x and y factors to 0.5  decreases

the  file  size  by 75%. Conversely, enlarging the image allows

you to fine-tune each pixel value before shrinking it  back  to

normal size. (Note that in this case, you will have 3 images in

memory:  the original, the enlarged image, and the new shrunken

modified image. It is recommended to delete the enlarged images

as soon as they  are  not  needed,  to  avoid  running  out  of

memory.)

8.2.7  Rotate image

Creates a new image identical to the currently-selected image,

except that it is rotated by 90�.

8.2.8 Flip horizontally

Converts  the  currently-selected  image  or  region into a

left-right mirror image of itself.

8.2.9 Flip vertically

Turns the currently-selected image upside-down.

8.2.10   Backup

Creates a backup copy of the current image in memory. This is

recommended before manipulating the image or adding text, in

case you don't like the results.  Selecting "Restore" will retrieve

the screen back onto the screen.

8.2.11   Restore

Replaces the current image with the backed-up copy if one exists.

8.3.0  Process menu

8.3.1  Filter

Modifies  the currently-selected image, or the selected screen

region, without creating  a  new  image.  The  original  image

should  be  backed  up  before filtering it, in case you don't

like the results. The amount of filtering is adjustable from 1

to 10 (maximum).

8.3.1.1  High-pass filter (Sharpening)

Extracts  only  the  highest spatial frequencies in the image.

More low frequency components are saved if a larger 'kernel' is

selected. If the image contains few high-frequency components,

and you selected too much sharpening, the result may appear

dark or black. You can compensate for this by  increasing  the

contrast of the image after filtering it.

8.3.1.2 Low-pass filtering

Low-pass filtering eliminates sharp edges from the image,

causing a blurring effect. The total intensity is unchanged,

but is spread throughout the neighboring pixels.

8.3.1.3 Background subtract

This is similar to high-pass filtering except it removes only

the lowest frequencies.  This has the effect of removing uneven

background in the image. As with high-pass filtering, it can

make the image appear darker. Selecting a larger kernel removes

less of the low frequencies, but will require much more processing

time. A preferred method is to use the "kernel multiplier".

Selecting a multiplier of 5 with a 3x3 kernel produces results

similar to using a multiplier of 1 with a 15x15 kernel, but is

many times faster. For this reason, it is recommended to set

the kernel to 3x3 for background subtract.

You can specify whether "black" or "white" pixels should be

considered as the background. If you select "white", the image

will generally get lighter in regions of unvarying pixel intensity.

If the background value is "black", the image will get darker

wherever the pixel intensity does not vary. You can compensate

for this by increasing the contrast.

Background subtraction will also trash any text in the image.

See "Background flatten" below for an alternative (and faster) way

to remove background gradients.

8.3.1.4  Background Flatten (de-trending)

This filter removes large-scale gradients from the image or

selected region by measuring the average pixel value in each

corner and then adding or subtracting a value to equalize the

overall intensity. This could also be done manually, by creating

a gradient region on the screen and adding the image to it (see

"Gradient fill"). However, the manual method is not 2-dimensional.

In contrast to Background Subtraction, this filter does not trash

text.

8.3.1.5  Noise filtering (median filtering)

Median filtering removes extraneous pixels from the image.

This is useful if the image contains noise which consists

of little dots that are clearly outside the range of the other

pixels. If a given pixel varies by more than a certain amount

from the pixels around it, median filtering substitutes the

median of the neighboring pixels, thereby eliminating noise from

the image. The image is otherwise unaffected. The range in � pixel

value units, outside of which a value is to be considered noise,

can be changed. A higher value results in less noise removal.

Excessive noise filtering can make an image appear posterized.

A setting of 0 will create a smoothing effect.

8.3.1.6 Laplace edge enhancement

This filter finds any edges in the image whose length is equal

or greater than the kernel size. The edges are then increased

in intensity while non-edge regions are eliminated. The effect

on text is to create an outline of the text.

8.3.1.7 Sobel edge enhancement

This filter is similar to Laplace edge enhancement except that

the effects are gentler. A Sobel filter enhances the color or

intensity gradient, so that areas of constant color become black.

8.3.1.8 Edge detection and edge sharpening

Edge detect �            Detects and enhances edges in the image,

Edge detect �            preferentially in the horizontal or

vertical direction.

Sharpen �

Sharpen /                Sharpens the selected region or image

Sharpen �                preferentially in the indicated direction,

Sharpen \                creating a `freeze-fracture' 3-D effect.

This is particularly useful for images

of biological specimens that have low

contrast, since it highlights the overall

shape of the cells.

8.3.1.9 Adjustable parameters

Kernel

The kernel is the number of pixels used in calculating a  pixel

in the new image. The processing time needed increases with the

square of the kernel size.

Kernel multiply factor

A "kernel multiply  factor" allows you to select an arbitrarily

large convolution kernel without an increase in computation time.

This is possible because usually only a small sample of the

surrounding pixels are really needed to calculate the new pixel

value. For most types of filters, a 3x3 kernel with a kernel

multipier of 3 gives the same results as a 9x9 kernel, but is 3

times faster.  For sharpening and blurring, this doesn't work,

and the factor is automatically set to 1.

Amount of filtering

The amount of filtering applied to the image, from 1 (minimal

filtering) to 10 (maximal filtering).

Range (median filter)

For  median  filters,  you can specify a range of pixel values

outside of which the pixel will be considered to  be  a  noise

pixel.  For instance, if the median value of all the pixels in

a given area is 123, and the range is set  to  �10,  then  the

pixel will be replaced with the median value (123) only if its

value higher than 133 or less than 113.

8.3.2  Warp

Corrects distortion in the image. Currently works in vertical

direction only.

(1) Select a series of control points to define the distortion

to be corrected. For example, if you have  an  SDS  gel  in

which  the  bands  are U-shaped, trace out a U-shaped curve

somewhere on the image that follows the U shape.

(2) Press any key. This fixes the curve in place (it becomes

invisible).

(3) With the mouse, select a y-value somewhere below the lowest

point in the curve which you traced out.

(4) Click the left mouse button at the desired y-value.

(5) Each vertical line on the screen will be shifted up or down so

that each point on the curve you traced out is lined up with the

selected y-value.

Future versions of TN-Image will have a more elaborate, 2-dimensional

distortion-removal feature. Contact the author for availability

dates.

8.3.3   Measure...

8.3.3.1 Distance measurement

Measure distance between two points. Click the left mouse button,

move to the end point, and release the mouse. Click on the top

menu bar or press <ESC> when finished.

Before making distance measurements, you can calibrate your image

as described in Sec.8.3.9, using "2D linear calibration" mode.

This will cause the results to be in known units (such as

centimeters) rather than pixels.

If the image has been calibrated in a "1D" mode, distance measure-

ments are reported as uncalibrated pixel distances.

8.3.3.2 Angle measurement

Measure the angle between 2 lines. Same as distance measurement

except you must click and drag twice. The angle is always given

as the most acute of the two angles between the two lines.

8.3.4  Calibrate

Calibrates the image or screen x and y coordinates. To calibrate

an image, select "New calibration" from the menu, then click on the

known calibration points in your image and enter the calibration

values. The calibration points (which appear as small squares) can

be dragged to new locations if desired. When finished, press any key.

A dialog box will appear; select "linear" or "logarithmic"

calibration, and change the title if desired.  A small window will

appear displaying the calibrated value for the current mouse position.

The calibration does not have to be vertical or horizontal, but can

be in any direction on the screen.

To end calibration mode, select "Calibrate" again and select

"Hide calibration". To restore the previous calibrations, select

"Unhide calibration".

Calibrating an image affects distance measurements and strip

densitometry area calculations.

Each image, as well as the background, can be calibrated separately

and can have a different title.

Four types of calibration are possible:

(1) 1-D linear calibration: Measures the distance from a line

perpendicular to the best-fit line passing through your calibration

points. This calibration is useful for images of isoelectric

focusing gels, or other situations where the distance in one

direction is the parameter of interest.

(2) 1-D logarithmic calibration: Same as (1) except distances are

logarithmic (For example, if 10 pixels corresponds to 1 unit of

distance, 20 pixels would be 10 units, and 30 pixels would be

100 units). This calibration is suitable for acrylamide and

agarose gel electrophoresis images.

(3) 1-D 2nd-order polynomial calibration: Same as (1) except

distances are fitted to a quadratic equation. Not really very

useful, but it was easy to program.

(4) 2-D linear calibration: Distances are calculated as the distance

from the point (0,0). This calibration type is suitable for

calibrating to objects of known size on the image (Sec. 8.3.4),

for example if a ruler was included in the image.

EXAMPLE: To calibrate an SDS-PAGE gel for molecular weight standards:

(1) Select "New calibration".

(2) Click on "OK" to start calibration.

(4) Click on each size standard in the image, then enter the

corresponding molecular weight. Each point will be shown by a

small box. If you make a mistake, you can go back and drag

any of the boxes to a new location. A best-fit line is

automatically drawn connecting the boxes. Make sure that this

line is in the same orientation as the lanes in your gel

(i.e., vertical or near vertical if your lanes are straight).

(5) When finished, press a key. The boxes and line will disappear.

(6) Select "logarithmic" calibration from the dialog box.

(7) Change title to "Molecular weight" (optional).

(8) The molecular weight is now displayed continuously. You can

move the display window to any location by clicking on its edge

and dragging to the new location.

Example 2:

Calibrating coordinates on the image to centimeters using a ruler

in the image:

(1)  Select  Process...Calibration...New Calibration.

(2)  Click  OK.

(3)  Click on the ruler markings on the strip in your image.

A small box indicates the calibration point. Move the box by

clicking-and-dragging if necessary. After releasing the mouse

button, enter the centimeter value. Repeat for at least 2 more

calibration points. The program uses least squares calculations

to find the best calibration fit to your points.

(4)  When finished, press <ESC>.

(5)  Select "2D Linear" from the menu, change the title to

"Centimeters", and click OK.

(6)  The image is now calibrated. Each image, as well as the back-

ground, can have a different calibration. The distance is

measured as the distance from 0,0 as defined by your

calibration points. You can now perform calibrated distance

measurements.

8.3.5  Spot Densitometry

Performs densitometric analysis on parts of the image.

See also "Strip densitometry".

For a tutorial, view the image DENSITOM.PCX.

In  high-color and true color modes, the image is treated as a 15,

16,or 24-bit deep monochrome image during densitometry. This is

the traditional method of densitometric analysis (as opposed to

analyzing each color separately). NOTE: If a monochrome or 8-bit

image is converted to color, the original correspondence of pixel

values to optical density (if one existed), may be lost.

Pixel compensation - If checked, this will cause TN-Image to use

in  its  calculations  the  actual  optical  density value that

corresponds to each pixel, instead of the raw pixel value. This

information is often provided by digital scanners in  the  form

of a `gray response curve' that is embedded in the TIF file. It

is  recommended  that pixel compensation be checked (�) for all

densitometry.

Pixel compensation is not applicable for color (15,16,24, or

32-bit/pixel) images.

Maximum signal - Selects whether pixel value corresponding to

`black' or `white' (0 or 255 in 8 bpp mode) is to be considered

the strongest signal. If you have an image of a protein gel,

for instance, the bands appear black and maximum signal should

be set to "black". For DNA gels, the bands appear white and

maximum signal should be set to "white".

NOTE: Selecting "Black" as the maximum signal will cause the

reported values to be calculated as 1 - (measured value). Thus,

if the average uncorrected pixel intensity of a spot is 0.25,

the density (with "pixel compensation" off) will be 0.75.

Area selection - Automatic: Automatically finds the boundary of

the region. Move to a point near the middle of the region  to

be analyzed  and  click  the left mouse button. TN-Image will

automatically determine the boundaries  of  the  area  to  be

densitometrically  measured.  This boundary is defined by the

set of contiguous pixels whose value is between  the  maximum

value (which can be selected above as "black" or "white") and

the background value (which can be selected to any value). It

is  important  that  there  is no unbroken trail of pixels in

this range that  extends  out  of  the  region  of  interest,

otherwise  an  area  larger  than desired may be selected. As

long as you continue clicking, new areas will be selected and

analyzed. To stop densitometric analysis, either click on the

right mouse button, or click somewhere on the menu bar at the

top of the screen. If the background is set  inappropriately,

the  quantitated  region  may  be too small or too large. The

densitometric operation can be aborted by pressing the  <ESC>

key.

Manual: In this mode, you must select (by clicking and dragging)

a rectangular region to be analyzed each time.

Fixed size: In this mode, you select (by clicking and dragging)

a rectangular region the first time. From then on, as long

as densitometry is active, when you click on a point, a

region of size and shape identical to the first region will

be analyzed. This region will be centered at the point where

you click.

After each analysis, a message box appears with the results.

This is : (1) The area (total number of pixels) analyzed; (2)

The total signal measured (either in total pixel values or total

O.D. units, depending on whether pixel compensation is active);

(3) The average signal, i.e., the quotient of (2)/(1); (4) The

corrected total signal - background. For most purposes, including

gel analysis, the number of interest is (2) or (4), because you

need to measure the total amount of absorbing (or fluorescing)

material in the entire band. For other purposes, you may be more

interested in the signal density or concentration per unit area

on the image. The area is of interest in photomicrographs.

Background  -  Select the pixel value which is to be considered

the `background level'. The background level is important  in

automatic  area  selection as described above. The background

value can also be automatically subtracted from  the  results

if desired.

Calibration factor - If a value is entered here, the result is

multiplied by that number. This is useful in converting to

micrograms of protein, for example, on an image of an SDS gel.

Subtract background - If checked, the background value will  be

automatically subtracted from each pixel during calculations.

NOTE: See the warning under "Contrast".

8.3.5.1 Area measurements

Making area measurements:

1. Before starting, obtain a histogram on the image to find the

desired background value for use with automatic spot detection

(select "Color...Histogram"). Alternatively Use "Draw...Sketch"

before starting, and completely encircle the objects to be

measured with a white line. Outlining the objects manually

will obviate the need to select an appropriate background value.

2. Select "Image...Spot densitometry". Leave all the settings at

their defaults except for the Background Value.

3. If you wish TN-Image to identify the spot automatically, set

the background value to the pixel value that corresponds to the

brightest pixel in the background. Otherwise, if you have outlined

the regions beforehand, set the background value to any large

number (for example, if you outlined the areas with white

(=255), set the background to 254).

4. Click on OK, then click again to dismiss the message box.

5. Click anywhere inside the object to measure. The area being

measured will temporarily be painted in black, then a list box

will appear displaying the area.

6. Click on the background (or the upper left corner of the list box)

to hide the list box. Then click on the next object to measure.

7. Press <Esc> when done.

8.3.6  Strip Densitometry

See also "Spot densitometry"

For a tutorial, view the image file DENSITO2.TIF.

Strip densitometry is not as exciting as it sounds. It is similar

to spot densitometry, but is easier to perform and interpret because

it is not necessary for TN-Image to find the edges of the object.

If the image has been calibrated, moving the cursor over the plot

area of the graph causes two different x-values are printed. The

upper x value is the data point number, and the lower x value is

the calibrated value for the center pixel of the line along which

the strip densitometry was performed. If the image has not been

calibrated, the two x values will be identical. Clicking-and-

dragging within the plot selects a portion of the graph for area

calculation. The selected area is highlighted in reverse color.

If the image has been calibrated, this area (printed at the lower

right of the graph) is also calculated from the calibrated values.

To obtain a densitometric tracing of a trapezoidal region, select

"Strip densitometry"  from the "image" menu. Do not confuse this

option with obtaining a scan from a scanner, which would require

a hardware interface, which is not provided with TN-Image. The

options are:

Select coordinates     - If `select coordinates' is checked, you must

select the region to scan each time.

Repeat prev. scan      - If `repeat previous scan' is checked, the same

region as the last time will be

re-scanned without you having to select it.

This is useful if you modified the image in

some way, such as by filtering it, in order

to see the effects on scanning.

Maximum signal         - select whether a "black" or "white" pixel is to

Black                    be regarded as the most intense value. Note:

White                    in 8 bit/pixel modes(monochrome/pseudocolor),

this will be affected by the optical density

table which is often included in TIF files,

if `pixel compensation' is set to `on'.

Most scanners set their optical density

table so that `0' is the maximum optical

density. (To find out whether your image

file has an O.D. table, select "Configure...

Show O.D. table".)

Pixel compensation     - if set to `on', the optical density table in

On                       the image file is used to correct the pixel

Off                      values from a range of { 0,255 } to values

that are proportional to the actual optical

density of the original image. This table is

usually generated by a scanner and is found

mainly in TIF files.

It is recommended to set pixel compensation to

`on' when analyzing images from a scanner.

This setting is automatically turned "off" in

the color modes, because these modes have

enough dynamic range that a gray scale table

is not needed.

Scan type              - This option determines what type of region you

Snap to 90�              can select for scanning. Selecting `snap to

Permit diagonal scan     90�' causes the starting and ending edges of

Fixed width=1            the scan to "snap" to either vertical or

horizontal, whichever is closest.  This is

fastest because it does not require any of the

anti-aliasing calculations which are needed

when you scan diagonally. However, if your

image contains objects that are oriented

diagonally, some resolution would be lost in

the scan because the objects will be scanned

at an angle to their true orientation.

The first two points always determine the

orientation of scanning. Scanning is always

done along a line parallel to these 2 points.

The 3rd and 4th points determine the direction

and shape of the region to scan. The area

being scanned is indicated by a sweeping

wave of temporarily inverted pixels.

Selecting `permit diagonal scan' allows you

to select any trapezoidal area with no re-

strictions on the starting angle. However,

the angle of the far side of the trapezoid is

always adjusted to make it parallel to the

starting side. This avoids confusion that

would otherwise be caused by having a small

`tail' at the end if the starting and ending

sides were not parallel.

Because of the grid nature of the screen,

this option is slower because it is

necessary to correct for `aliasing' caused

by pixels being at unpredictable distances

from the starting point. However, you can

scan objects that are oriented in any direc-

tion. To compensate for the slower speed,

the pixels are no longer inverted as they

were in earlier versions of TN-Image.

'Fixed width' only requires that you select

the two end points instead of 4. Densitometry

is performed in a rectangular region between

the two points using the specified width in

pixels (which can be 0 to 200). If the region

is not perfectly vertical or horizontal, anti-

aliasing is performed.

This method is the most useful for obtaining

results which need to be compared to each

other, or when the density of a very narrow

region (such as a line) is desired. Of course,

a narrower region will tend to be noisier

than a wide one.

'Fixed width' densitometry is slower than

the other two methods because more floating

point calculations and memory accesses are

needed. However, it may be more accurate.

����Options����

Automatically save scan

- If this option is checked, each scan will

automatically be saved in an ASCII file.

Filename for scan    - A default filename of "1.scn" is provided.

If the 1st 8 characters of the filename

consist entirely of digits, the program will

automatically increment the filename after

each scan (For example, if the starting

filename was " 1000.dat", subsequent scans

would be saved under 1001.dat, 1002.dat,

etc.). If the filename contains letters,

the filename must be typed in each time.

You can also save the most recent scan at

anytime by selecting "save scan" from the

"File" menu.

Plot results         - If checked, the scan results will

automatically be plotted on the screen

after each scan. Clicking the "OK" button

on the graph makes the graph disappear and

returns you to scanning mode. The graph is

always automatically scaled in the Y

direction to fill the entire box. When the

graph is visible, the data can be saved to

disk, a background curve can be subtracted

from the data, or the graph can be captured

into a new image (See "Plotting densitometry

results and other data").

Pause to show region-  If checked, the program pauses after drawing

the 4 boxes, allowing you to verify that you

selected the correct region. Press a key to

continue.

Procedure:

1. Select "Image...Strip densitometry" from the menu.

2. Click on "Maximum signal=black" or "Maximum signal=white" depending

on whether the features of interest are darker or lighter than

the background.

3. Click on "Snap to 90�" , "Diagonal scan", or "Fixed width"

(see above). If you select "Fixed width", enter the desired width

in pixels.

4. Click on "OK", then click anywhere to start.

5a. Click once on each corner of the rectangular area to be scanned,

in a clockwise direction, so that 4 boxes appear on the screen.

The region will be scanned from the first 2 to the last 2 boxes.

5b. For 'Fixed-width' densitometry, click at the center of the starting

and ending points, so that 2 boxes appear on the screen.

6. If any of the boxes are in the wrong position, or the lines

connecting them are crossed, click and drag the boxes to the

correct position. (NOTE: if the lines are crossed, the algorithm

automatically swaps the coordinates to un-cross them).

7. Press any key, or click anywhere to start scanning. The scan

results will be automatically plotted if "plot results" is checked.

The densitometry scan can be saved to disk or captured into a

new image from the plot window.

8. Click on "Save to disk" to save the densitometry tracing in an

ASCII file.

9. Click on "OK" to scan the next region, or press <ESC> or click

the right mouse button to stop.

NOTE: See the warning under "Contrast".

8.3.7  Plotting densitometry results and other data

There are several useful options available

in the plot mode:

Save to Disk

------------

Saves the currently-displayed graph into an ASCII file.

Smooth

------

Performs Gaussian smoothing on the data being displayed.

Auto Baseline

-------------

Automatically calculates an optimal curved baseline and

subtracts it from the data. The smoothness of the calculated

baseline can be changed by clicking on "BL smoothness". The

calculated baseline is displayed for 1 second before

subtracting it.

Manual Baseline

---------------

Allows you to select a baseline manually. Click at several

locations on the graph where you want the baseline to go. Small

squares will appear to indicate where you clicked. You can

click-and-drag these squares to move them. When finished, press

a key to subtract the baseline. The baseline curve is constructed

using a B-spline (see `B-spline curve'). The squares can be anywhere

on the screen, and do not have to be inside the plot. Also, you can

subtract a baseline from only a portion of the data by placing boxes

only under the portion you wish to change.  However, any negative

numbers  created  by baseline subtraction are truncated to 0's.

Up to 200 boxes (`control points') can be used.

Capture Image

-------------

Clicking `capture' allows you to capture all or part of the

graph into a new image. After clicking on `capture', use the

mouse to select the desired portion of the graph. A new image

which includes that region is then automatically created. The

new image is put into the background, behind the image you are

currently scanning, so that you can continue with the next

scan.

Help

----

Calls a help screen.

OK

--

Ends plotting, returns you to densitometry mode.

8.3.8  Peak Areas

Finds the x and y coordinates of each peak and  calculates  the

peak  area.  The  integration  limits  are shown as short black

lines on either side of each peak. No baseline  subtraction  is

carried  out  before calculating peak areas. Thus, best results

are usually obtained  if  you  first  subtract  a  baseline  by

clicking on `Auto.baseline' or `Manual baseline'.

When the list of peaks is displayed, most of the click buttons

on the left side of the plot are still active, but now apply to the

peak list. This includes:

Save to disk

Capture image

Help

OK

The other buttons (`peak areas',`smooth', `manual baseline', and

`auto baseline') are inactivated while displaying the list.

You can also close the peak area list by clicking on the `-' symbol

in the upper left corner.

Area calculations:  While the graph is displayed, you can also

manually measure peak areas by selecting the desired region

of the graph. To select a region, click with the left mouse

button and drag horizontally within the graph area. When the

mouse button is released, the total area of the selected region

is automatically displayed.

8.3.9  Trace curve

If you have an image of a graph, chromatogram, etc., TN-Image can

convert this into an ASCII file. Currently, TN-Image only traces

in 1 dimension, and from left to right. Contour tracings will be

implemented later.

Procedure:

1. First, clean up the image to remove any stray specks, labels, or

other points that could be confused with the graph. The entire

image must be clean, including the edges. The trace will jump

to the darkest y-value point for each x value, regardless of where

it is located on the image, even if it is not visible on the screen.

Use "Paint region" if necessary to eliminate white areas from the

image.

2. Select "Image...Trace curve".

3. If you want the trace to be saved automatically, check "Save trace".

4. If you check "Plot trace", the data will be displayed. You will then

be able to click on the "Save" button to save it. While the graph

is visible, you can smooth it, subtract a background, measure

areas, etc. (See "Plotting densitometry results").

5. If the image is a dark graph on a white background, click on "Color

to track= black". Otherwise, select "white".

6. Click on "OK", then move the mouse cursor to the left end of the

curve and click the left mouse button.

A cross-hair cursor indicates what is being traced out.

Click on "Cancel" when finished.

Sometimes, sharpening or thresholding the image can improve the

tracing. If there are extremely sharp peaks in the image, it may

be necessary to manually create a channel between the ascending and

descending parts of the peak to force the trace to follow to the

top. This can be done by selecting "Draw" from the menu, or pressing

<F2> (=manual draw), and carefully moving around with the cursor keys

(Be sure to back up your image before starting).

Checking "Debug" waits for a keypress after each x increment and

prints the x, y, and the pixel value being tracked. This is helpful

if stray noise pixels are accidentally getting included in the scan.

Only pixels on the selected image are included in the trace. Thus,

in order to trace something in the background, you must convert it

to an image first (use "File...Create/resize image"). Conversely,

if the image contains a white area (or black area, if "black" is

selected) that is is brighter than the area being traced, the cursor

will jump to this area instead of tracing your curve. If the

bright area is beyond the edge of the screen, the cursor will jump

to the edge of the screen. If this occurs, it will be necessary to

delete the offending area before tracing anything (The "backspace"

key is the most convenient way to do this).

8.3.10  Mathematical pixel operations (Math)

Although it is possible to add or multiply pixel values by any

factor by using the "brightness" or "contrast" menu items,

TN-Image also has a more powerful tool - mathematical pixel

operations.

Using the "math" menu, you can enter any mathematical formula

to transform each pixel value in one pass. This formula could

be any legal mathematical equation consisting of constants,

variables, and operators from the following list:

Constants -  any real number (floating point or integer)

Variables -  one of 4 pre-defined variables:

i  =  intensity (total pixel value)

r  =  red component of the pixel

g  =  green component of the pixel

b  =  blue component of the pixel

Operators -     () =  parentheses

^  =  exponentiation

*  =  multiplication

/  =  division

+  =  addition

-  =  subtraction

For example, if you entered the following equations:

r=r+3

g=(r/4)+1.2345

b=g-b^2

each pixel would have its red component increased by 3, its

green component would be set equal to red/4 + 1.2345, and blue

would be set equal to green minus (blue squared).

To edit the equation, use the mouse or arrow keys to highlight

the desired row. Press <Enter> and edit the equation. Click on

<OK> or press <Enter> again when finished.

To perform "gamma correction" to your image, enter the following

equations:

r = r^1.8

g = g^1.8

b = b^1.8 ,

where 1.8 is the gamma correction factor.  For monochrome, a higher

value (e.g., 2.35) is often used.

8.3.11  Transform (FFT)

Performs a Fast Fourier Transform on the image or selected

region. This can use a lot of memory because the frequencies are

stored as double-precision complex numbers, which require 16

bytes for each pixel. Thus, an FFT is 4 times as expensive as

a 32-bit image. Additionally, the mathematics of the FFT requires

that the image be first enlarged so that each dimension is the next

higher power of two. For example, if your image is 129 x 67 pixels,

TN-Image has to create a new buffer of 256 x 128 pixels (524,000

bytes) to carry out the calculations.

The FFT result is a matrix which is displayed as a new 128 bit/pixel

image. This new image can be saved, unloaded, annotated, filtered,

and manipulated like any other image. Any changes you make

to this image (such as adding text) are also converted to the

appropriate floating point numbers and inserted at the appropriate

position into the displayed component (real or imaginary) of the

FFT matrix. Therefore, changing pixels on the displayed image could

change the real component of the FFT, the imaginary component, or

both. After editing the image to enhance or eliminate specific

spatial frequencies, you can then perform a reverse FFT to obtain

the filtered result.

Forward/Reverse/Change display only

Selects whether to carry out a forward or reverse FFT, or to

merely change which component (original image, imaginary, real,

or power spectrum) is being displayed. No actual change is made

to the FFT data.

Real/Imaginary/Power spectrum

Selects whether to display the real, imaginary, or power spectrum

component of the transformed image, or only the original image.

NOTE: If you select "imaginary" or "power  spectrum", the displayed

image will appear black if you perform a FFT followed by a reverse

FFT. This is because the original data do not have an imaginary

component. The original data are not lost!

Although a complete description of the many applications of

FFT's and deconvolution is beyond the scope of the manual, there

are a few TN-Image- specific points worth remembering:

(1).  Regardless  of  the  screen  mode  or color depth of the

image, the FFT and deconvolution algorithms treat  all  pixels

as  monochrome.  Thus, a 24-bit/pixel image is treated as a 24

bit deep grayscale image. In other words, the colors  are  not

deconvoluted separately in the current version of TN-Image.

(2). The grayscale mapping algorithm sets the most negative

FFT result to black and the most positive FFT result to white.

Thus, zero will be some shade of gray.

(3)  Editing  power  spectra will lead to unpredictable results

in your image.

(4) Select "About...About the image" to view some of the FFT

parameters. These include:

Minimum FFT value (which is mapped to black)

Maximum FFT value (which is mapped to white)

FFT=0 color value (the color to which FFT values of zero are mapped)

FFT=0 RGB values (the corresponding red, green, and blue components

of the FFT=0 color value).

These  numbers can be used to calculate the color values needed

to edit the image in the frequency domain to remove or accentuate

specific fequencies. The filtered image can then be reverse-

transformed to obtain the result.

(5) FFT's are displayed in the conventional manner,  i.e.  with

lowest  positive  frequencies  at  the  top  and  left,  lowest

negative  frequencies  on  the right and bottom, and Nyquist in

the center, with the exception that only the real or  imaginary

components are shown, as illustrated in the diagram below:

f=0 f=1/N  ...  f=�  ...   f=-1/N

f=0    -------------------------------

|\             |           :  |

f=1/N  | 0,0 freq.    |           :  |

|              |           : \|

.     |              |           :  \

.     |              |           :  |\

.     |              |           :  | \

|              |           :  |  This region of FFT is

f=�1/2 |--------------*--------------|  not visible if image size

|              |\          :  |  is not a power of 2

|              |  Nyquist  :  |

|              |  frequency:  |

.     |              |           :\ |

.     |              |           : \|

.     |..............|...........:  \

|              |              |\

|              |              | image boundary if image size

f=-1/N ------------------------------- is not a power of 2

If the original image size is not an exact power of 2, a portion of

the FFT spectrum will not be visible.  In this case, the Nyquist

frequency will not be in the exact center.  If desired, you can

change the image size before calculating the FFT with the

"File...Create/resize image" option.

(6) FFT's should be done with as few other images present as possible.

Because an FFT is extremely memory-intensive, if TN-Image is forced

to use virtual memory, the FFT may take a very, very long time.

(7) When deconvoluting or convoluting, the two images should be

approximately the same size for optimal results.

(8) Avoid image sizes that are just larger than a power of 2, such

as 129 x 129 pixels. This will cause the complex array for FFT to

be allocated as the next higher power of 2 (i.e., 256 x 256).

(9) The screen mappings of FFT intensity values are automatically

rescaled whenever the FFT image is redrawn. Thus, changing the

contrast of an FFT'd image will appear to have no effect.

8.3.11.1 Convolution

View the image DECONVOL.TIF for a quick tutorial on convolution

and deconvolution.

Convolution      -

Convolutes two images. Convolution is the same as multiplication

in the frequency domain. Therefore, the resultant image will have

the characteristics of both images. In the simplest possible example,

a sharp image convolved with an image of a blurred point will become

blurry. An image convolved with a single point is unchanged.

8.3.11.2 Deconvolution and image reconstruction

Deconvolution is a form of image reconstruction which was made

famous by successfully being used on the distorted images from

the Hubble space telescope. As with convolution, two images are

required. Typically, one of the two images is a point spread

function, or psf, which represents the effect of distortion on

a single pixel, and the other is the image from which this distortion

is to be removed. The two images must be approximately the same size.

For example, if your image is blurred by spherical distortion from

a bad lens, you would create an image the same size as your blurry

test image, but consisting of a single point in the upper left of the

image on a black background, subjected to the same blurring effect.

If the point spread function is known accurately, after deconvolution

the blur will be removed. In effect, a priori knowledge from outside

the image is used to remove the distortion.

In principle, any degraded image can be reconstructed by

deconvolution if the point spread function is known.  If a photo

is blurred due to movement in one direction, the point spread

function would be a line in the direction of motion whose length

depends on the amount of movement. In practice, deconvolution will

not work for every image.

This method is far more powerful than using a "sharpening" filter,

because any arbitrary type of degradation of the image can be

eliminated. In practice, noise (especially noise in the psf) or

low precision in the FFT will cause inaccuracies in the recovered

image. For this reason, FFT's in TN-Image are maintained as

double-precision numbers. Also, if the spatial frequency of the

psf at any point happens to be zero, it will also be impossible

to reconstruct that point (since it would require dividing

by zero). Adding a little noise to the psf will ensure that all

frequencies are non-zero. Thus, sometimes it is necessary to

tolerate some noise in the psf to get good image recovery.

Important: The point spread function must be centered at the upper

left corner of the image to be used as a psf. Otherwise, a

translation effect will also be convoluted into or out of the image.

Exercise: Try deconvoluting fft1.tif and fft2.tif. It should be

possible to reconstruct the original image in fft1.tif, which was

a small square, despite the complicated point spread function in

fft2.tif which renders the distorted image unrecognizable. The more

complicated the point spread function, the more difficult it is to

recover a noise-free image.

8.3.11.3 Digital Filtering of images using the FFT

Digital filtering can be easily done while the FFT spectrum is being

displayed, by changing the pixels which correspond to the frequencies

you wish to change. This is most conveniently done by setting the

background  color to "black" and using <delete> or <backspace> to

erase the frequencies you wish to remove; or by selecting "sketch"

(<F2>) and adding points at the desired frequencies by drawing with

the mouse. It is necessary to change both the positive and negative

frequencies to achieve good filtering. This means the entire FFT

matrix must be visible, i.e., the original image must be close to

a power of 2 in each dimension.

Example: High-pass filtering

1. Make sure the original image size is a power of 2.

2. Set background color to black by right-clicking on "black" in

the palette.

3. Forward-FFT the image.

4. Delete the low-frequency regions (as shown with a 'L') with the

'delete' key.

5. Alternatively, use the mouse to select these regions and use

"paint region", "change contrast" or "color/intensity" to make

them darker.

6. Optionally, you can also enhance the higher frequencies by

using "enhance contrast" in the high-frequency region (shown

by 'H'.

7. Drawing on a FFT'd image only affects the component being

displayed ("real" by default). Apply the same filtering

procedure to the imaginary component by selecting "FFT...

change display...imaginary" and repeat the changes.

LLLL---------------------LLLLL

LLLLL          |          LLLLL

|              |              |

|              |              |

|              |              |

|          HHHHHHHHH          |

|----------HHHHHHHHH----------|

|          HHHHHHHHH          |

|              |              |

|              |              |

|              |              |

LLLLL          |          LLLLL

LLLLL---------------------LLLLL

5. Reverse-FFT to obtain the filtered image. Note: Changing the

zero-frequency pixel (at the upper left corner of the FFT)

will have drastic effects on the image.

8.3.12 Macros

TN-Image can execute a series of predefined commands. This is

particularly useful when a large number of images need to be

processed in an identical manner. Technically, the macros in

TN-Image are not macros but small programs. The macro editor

can also edit small text files (up to 255 lines), making it useful

as a clipboard for making notes.

Macro commands consist of the text of one of the menu items,

followed by a series of parameters which are usually numeric.

Any menu item from any menu can be part of a macro, with the

following caveats:

1. Spaces should be replaced by underlines (_) or omitted.

2. Periods and check marks should be omitted.

3. Capitalization is not significant.

The numbering of command parameters corresponds to the

order in which the item would appear in the menu or dialog box.

It is not necessary to give all the parameters. If a parameter

is not given, it will be unchanged from the last time the command

was executed.  Menu items useable as macros include:

Load_image  Save_image  Print_image  Save_scan_data

Create/Resize_Image

Create_File_Format Erase_FFT Unload_image  DOS_Command Quit

Delete_region  Crop  Erase_background  Move  Copy  Spot_densitometry

Strip_densitometry  Trace_curve  Rotate_image Flip_horiz

Flip_vertically Filter  Change_size  Warp  Measure  calibration

Pixel_math  FFT   Backup  Restore  Macro  Set_color  Color/brightness

Contrast  Change_palette Color_settings

Remap_colors  Invert_colors  Change_color_depth

Color->gray_scale   Gray_scale->color   Histogram

Line   Circle   Box   Arrow   Line_style

Text_direction   Sketch   Line_width   Fill_region

Paint_region    Curve  Diffuse_Spray   fine_Spray  Add border

About_the_program    About_the_file    About_the_image

How_to_Register  Show_menu_bar   Menu_bar_2   Show_palette

Automatic_undo  Redraw_menu_bar  Show_OD_table  Pixel_interact_mode

Spray_factor    Cursor_size     Signif_digits  Help

Of course, some of these (such as "sketch" and "help") are not too

useful in a macro.

In addition to menu items, the following non-menu commands (described

later) can also be used:

beep

load

save

print

fill

dos

color

brightness

size

select_region

select_image

goto

loop

palette

exit

messages

convert

file_format

unload_all

resize_image

copy_image

You can cause the macro to ask for any parameter by substituting the

value with a question mark followed (with no spaces) by a prompt

string. For example, for the  `load' command, the first parameter

is the filename. If instead of putting an actual filename in the

macro, such as

load myimage.tif

you use

load ?Filename

the program will ask you to enter a filename each time, by presenting

a message box like the following:

--------------------------------------

|Filename                            |

|____________________________________|

|                                    |

|                                    |

|                                    |

|                                    |

--------------------------------------

At this point, you can type the filename and press <Enter> to

continue. The prompt string cannot contain spaces, because a space

is interpreted as the beginning of the next parameter. There can

be any number of prompts for each command.

NOTE: no checking is done on the response to make sure it is a valid

answer. In the case of a filename, the macro interpreter itself does

not check whether the answer is a valid filename or even that a

filename is actually required for this parameter. Similarly, no

checking of your commands is done to make sure the macro is a valid

command. A command that is invalid, for example, by being misspelled,

will either be skipped, or if it generates a fatal error, will cause

the macro to be stopped.

8.3.12.1   Macro command summary

This is a partial listing of commands which require parameters.

Parameters must be specified in the indicated sequence and

separated by spaces. All parameters must be on the same line as

the command. Each command must be on a separate line.

Anything that appears in any of the menus may also be used as a

command. Those commands that do not require parameters are not

listed here. Some, such as 'automatic_undo' toggle a feature on

or off. For these commands, use the same command a second time

to turn the feature back on.

Unless noted otherwise, all parameters can be omitted. This

causes the current default setting to be used. Parameters cannot

be skipped. Thus, if you wish to set parameter #3, for example,

you also must set parameters #1 and #2.

If a fatal error occurs, the macro is automatically terminated

and an error message is displayed showing what line caused the error.

Command   Parameter# Refers to   Value Meaning

-------   ---------- ----------- ----- -------

filter        1      filter type:  0   low pass

1   high pass

2   laplace

3   background subtract

4   background flatten

5   noise (median)

6   sharpen /

7   sharpen |

8   sharpen -

9   edge detect -

10  edge detect |

11  sobel

2      kernel size:  1   3x3

2   5x5

3   9x9

4   15x15

3      amount of filtering (1-10)

4      range (for median filter) (any integer)

5      kernel multiplier (any integer)

6      maximum background:

0    black

1    white

Examples: filter 5 1 3 10 1 1

Filters the currently-selected

image or area using a 3x3 median

filter, with a filtering level

of 3, a range of �10, kernel

multiplier of 1, and white declared

as the background.

filter 11 2

Filters the currently-selected

image or area using a 5x5 Sobel

filter.

select_region        Sets boundary for other operations such

as contrast. The coordinate values are

relative to the upper left corner of the

screen (= 0,0). Select_region is cancelled

by select_image and vice versa. All 4

parameters must be given.

1      upper left x coord.

2      upper left y coord.

3      lower right x coord.

4      lower right y coord.

Example:  select_region 100 100 200 200

This has the same effect as clicking-and-

dragging the mouse from (100,100) to (200,200).

select_image  1      image no.  NOTE: Image numbering changes

unpredictably when an image is

unloaded. This is because images

are rearranged to compact the

free space.

Example:

select_image 5

Has the same effect as double-clicking on

image #5.

load          1      filename (can include wildcards)

2      x position for upper left corner

3      y position for upper left corner

4      x size (0 to 1) (1=full size)

5      y size (0 to 1)

The remaining arguments (6 to 15) are used only for

reading raw images.

6      platform    0 = PC

1 = Mac

2 = IBM

7      color type  0 = RGB color

1 = grayscale or pseudocolor

2 = CMYK color

8      bit packing 0 = TIF-like

1 = GIF-like

2 = none

9      x size in pixels

10     y size in pixels

11     no.of bytes to skip at start of file

12     bits/pixel of image

13     red bits/pixel of image

14     green bits/pixel of image

15     blue bits/pixel of image

16     black bits/pixel of image

Examples:  load myimage.tif 100 100 1 1

Reads myimage.tif from disk, positions it at

(100,100), at full size.

load *.gif

Reads all .GIF files from disk. Since no

position or size is specified, the values

set by the previous command will be used.

The most recently loaded image becomes

the currently-selected image for other

operations.

Example: Load an image of raw bytes.

file_format 9

load raw.img 0 0 1 1 0 1 0 382 362 10 8

The first command is necessary to override

the automatic file detection, which would

otherwise display a message "Invalid file

type".

The second command loads the image at x=0,

y=0, x size=100%, y size=100%, platform=PC,

color type=pseudocolor, x size=382 pixels,

y size = 362 pixels, skip 10 bytes, and

interpret data as 8 bits/pixel.

If a large number of raw byte images having

identical parameters are to be read, it is

easier to create a custom file type and

define these values as the defaults. This

will eliminate repetitive retyping of the

arguments.

save          1      filename

2      file format  -see under "file_format" for

permissible values

3      bits/pixel  (Any value between 1-32. Must

be a legal value for the specified format

otherwise it is a fatal error. If omitted,

it uses the image's current bpp.)

Example:

save test.tif 0

Creates a TIF file named "test.tif" using

the currently-selected area or currently-

selected image.

If no arguments are given, the image's

original filename and format will be used.

Use `file_format' to override the default

image format. You must change the extension

manually in case the file is to be read by

file viewers, which may require a

specific file extension. If the image was

created within TN-Image it will not have a

filename until you save it. If a macro tries

to save a file without specifying a filename,

a fatal error occurs, terminating the macro.

color,

brightness

1      change in brightness for monochrome portion

of selected area if any (If the entire area

is color, this parameter is ignored).

2      change in red for color portion if any

3      change in green for color portion if any

4      change in blue for color portion if any

If the entire area is monochrome, parameters

2-4 are ignored.

Example: To increase the red in a color image by 10:

color 0 10 0 0

The color, brightness, and color_brightness commands

are synonymous.

contrast

1      change in contrast for monochrome portion

if any (1 = no change)(If the entire area

is color, this parameter is ignored).

2      change in red contrast for color portion

3      change in green contrast for color portion

4      change in blue contrast for color portion

If the entire area is monochrome, parameters

2-4 are ignored.

Example: To increase blue contrast in a color image

by a factor of 1.5:

contrast 0 0 0 1.5

convert,

change_color_depth

1   bits/pixel to convert image to (must be 8, 16,

24, or 32).

change_palette,

palette       1   Palette number   1 - gray scale

2 - spectrum

3 - multi colors 1

4 - multi colors 2

5 - multi colors 3

6 - RGBI

7 - black to green

8 - zebra

-1 - Other (randomly-generated)

0 - Other (e.g., inverse)

unload        (No parameters, unloads currently selected image,

Use 'select_image' command to select an image.)

unload_all    Unloads all images. If `messages' is `on', you are

prompted to save any image that has been modified.

quit          Quits the program and asks if you want to save

your changes.

exit          Quits the program WITHOUT asking if you want

to save your changes. Useful when running TN-Image

macros in batch mode (See "Batch-mode processing").

goto          1      line number in macro to execute next.

A jump to a lower line number causes an

infinite loop. Press <ESC> to stop the

macro.

loop          Causes the macro to be repeated. Each time

a message box appears asking if you want to

repeat the macro - you must type a 'y' or 'n'.

pixel_interact_mode  Set

1      Pixel inter-  1 Overwrite

action mode   2 Maximum

3 Minimum

4 Add

5 Subtract new - old

6 Subtract old - new

7 XOR

8 Average

9 Superimpose

dos

dos_command   1-5    Execute a DOS command

Example:     dos pkunzip image.zip

messages      1      Disables messages, including all requests

for confirmation and requests for filenames.

Messages are automatically re-enabled after

the macro ends. Useful in batch mode. Turning

messages off is dangerous and should only be

used in fully-debugged macros. If messages

are turned off, a macro could overwrite

files, unload images, create a file in a

non-standard format, or exit to DOS without

asking for confirmation.

0  Messages off

1  Messages on

Example:    messages 0

fft           1      Direction     1  Forward

2  Reverse

3  Convolute 2 images

4  Deconvolute 2 images

5  Change display only

2      Display       1  Real

2  Imaginary

3  Power spectrum

3      Image #1 (may be left blank for FFT of

currently-selected image)

4      Image #2 (required for deconvoluton)

Examples:    fft 1 1     (forward fft)

fft 4 1 7 8 (deconvolute images

7 and 8)

macro         1      Macro file name. Control is transferred

to the first line of the new macro. After

the new macro is finished, the old macro

continues. There is no limit on the depth

of nested macros except as imposed by

available memory. Do not create recursive

macros (macros which call themselves

directly or indirectly).

file_format   1      Sets the default file format for loading

and saving images.

-1  (automatically detect)

0  NONE

1  TIF

2  PCX

3  IMA

4  IMG (Frame grabber)

5  GIF (GIF87a)

6  IMM

7  GEM

8  IMDS

9  RAW

11 JPG

12 TARGA (Targa RLE)

16 BMP (Windows bitmap)

23 ASCII

100 first custom format

101 2nd custom format

...etc.

This will supercede the image's existing

default file format for the duration of the

macro.

Example:    file_format 5

= sets default format to GIF

create_image

resize_image

1      Method for obtaining image coordinates.

1 Use mouse

2 Use specified coordinates

3 Copy another image

4 Resize another image using

specified coordinates

2      Border         0 No border

1 Add border

3      x size (in pixel units)

4      y size (in pixel units)

5      x position (0,0 = upper left)

6      y position (0,0 = upper left)

7      No. of image to copy. Remember that the image

number changes unpredictably if an image

is unloaded. If the image no. does not exist,

it is an error.

8.3.12.2   Macro Programming Guide

-----------------------

Here is a brief tutorial on some of the finer points on creating

useful macros. In general a macro should reset the program to its

original state.  If a macro does something different the second

time it is executed, the cause is almost always a failure to reset

the original conditions.  In a real macro, you should also always

execute  "select_image" or "load" first to make sure the operation

is performed on the correct image.

Example macro #1: Nested macros

Macro "beep.mac"

Line #    Command    Note: the words "Line #" and "Command"

------    -------    and the numbers 1,2, and 3 are not part

1       beep       of the macro. In this example, the macro

2       beep       would consist of only 3 words ("beep"),

3       beep       one on each line.

Macro "test.mac"

Line #    Command

------    -------

1       macro beep.mac

2       beep

Executing "test.mac" should cause a total of 4 beeps.

A macro must never call itself - this would cause a system hang.

Example macro #2: Converting GIF image to TIF format interactively

Line #    Command

------    -------

1       load ?Filename

2       messages 0

3       file_format 0

4       save

5       messages 1

6       loop

Line 1 - Prompts you for a filename each time (because of the

question mark).

Line 2 - Resets default file format to 0 (TIF). If this line is

omitted, the file format would default to the original format

of each image.

Line 4 - Saves the image in TIF format. Don't forget to rename

the file extension to TIF later. You could also use:

save ?Filename_to_save_as ?Format

which would ask you for both the filename and the format

each time.

Line 6 - Unconditional loop back to line 1. Press <ESC> to stop.

Example macro #3: Converting GIF image to TIF format in batch mode

Line #    Command

------    -------

1       messages 0

2       load *.gif

3       file_format 0

4       save

This macro could be executed at 3 a.m. by setting the following

timed events in TNSHELL or other command scheduler:

Event 1:  cd c:\tnimage              3:00am

Event 2:  tnimage -macro convert.mac 3:01am

Event 3:  ren *.gif *.tif            4:00am

Event 4:  cd c:\                     4:01am

Event 3 could also be replaced by a new line in the macro:

dos ren *.gif *.tif

Make sure there is enough free disk space before starting this

macro.

Example macro #3: Subtracting two images

Line #    Command

------    -------

1       pixel_interact_mode 1

2       load cells.tif 0 0 1 1

3       pixel_interact_mode 5

4       load cells.tif 1 1 1 1

5       pixel_interact_mode 1

6       contrast 1.85

This macro loads an image, then subtracts the same image

after offsetting it by (1,1), then increases the contrast.

All 6 lines are essential for a good macro.

Line 1: The pixel interaction mode is set to 1 (overwrite)

in case a previous operation changed it.

Line 2: All 4 parameters (x offset, y offset, x size, and y

size) must be given. Otherwise, if the macro was executed again,

the default parameters would be { 1,1,1,1 } because of line 4.

This would cause the image to be placed at (1,1) and then

subtracted from itself, giving solid black.

Line 3: The pixel interaction mode is set to 5 (subtract).

It is usually helpful to change the screen background color to

0 (black) before doing this.

Line 4: Load the image to subtract.

Line 5: Reset the pixel interaction mode to overwrite.

Line 6: Increase the contrast by a factor of 1.85.

Be careful using "DOS command" in combination with "messages 0".

If there is insufficient low DOS memory to execute the command,

"messages 0" may cause you to miss the fact that the command

was not executed.

8.4.0   Color menu

Palettes are only needed for indexed-color (8-bit/pixel) images.

If an 8-bit (256-color) image which lacks a palette of its own

is loaded and converted to color, the most recently-selected

palette will be used to perform the conversion.

8.4.1   Invert palette

Switches the palette so that all images become a negative.

Changes the appearance only, and has no effect on the actual

pixel values in the image.

8.4.2   Color intensity / brightness

There are 3 ways to change the brightness: you can drag the

palette bar to change the color LUT (palette), or you can select

"Color intensity" or "Change pixel values" from the menu.

8.4.2.1 Changing brightness using the palette bar

This method is very fast, but only works on images that have a

continuous colormap, and only in 8-bit/pixel screen modes.

Files with a discontinuous colormap should be adjusted using the

"Color intensity" menu option instead of the palette bar.

Click on the top part of the bar and drag down to "squash" the

colors downward, or click on the bottom part of the bar and drag

up to squash the colors upward. This increases the contrast in

all images of the screen.

Then, click in the middle of the bar and drag up or down to adjust

the brightness as desired. The currently-selected image will be

set to contain the new palette.

If you change the palette from the menu, or select "restore original

palette", the brightness and contrast settings return to normal.

8.4.2.2 Changing brightness and color using the menus

This method is also fast, and allows you to change the red, green,

blue, and intensity values independently. The action is different

depending on the screen mode.

For 8 bit/pixel modes, the colormap (palette) of the image is

changed. The pixel values in the original image is unaffected.

It is not possible to change the color of only part of the image

with this method. To change a portion of the image, you must either

use "Change pixel value" or convert the image to 24 bits/pixel

first.

For true-color screen modes, the action is the same as "Change

pixel value" (see below).  A value is added or subtracted for the

red, green, and blue components of the currently-selected image or

screen  region, making it darker or lighter or altering its color

balance. The original colors can be restored by selecting

"Image..Restore".

8.4.3   Contrast

There are also two ways to change the contrast: you can drag the

palette bar to change the color LUT, or you can select "Contrast"

from the menu.

Changing the LUT is much faster, but only works in 8-bit

(pseudocolor) screen modes, and it always changes the entire screen.

Selecting "Contrast" from the menu works in all screen modes, and

can be used to change contrast in selected regions of an image.

8.4.3.1 Changing contrast using the palette bar

Click on the top part of the bar and drag down to "squash" the

colors downward, or click on the bottom part of the bar and drag

up to squash the colors upward. This increases the contrast in

all images of the screen.

Then, click in the middle of the bar and drag up or down to adjust

the brightness as desired. The currently-selected image will be

set to contain the new palette.

If you change the palette from the menu, the brightness and contrast

settings return to normal.

8.4.3.2 Changing contrast using the menus

For 8 bit/pixel images:

Multiplies all pixels in the currently-selected image or screen

region by a value (100=no change), increasing the contrast.

Note that if the contrast or brightness is increased or decreased

too much, information will be lost from the image due to saturation.

***NOTE***WARNING***CAUTION***ATTENTION****READ THIS**************

|                                                                |

| When using 8  bit/pixel  modes,  adjusting  the  contrast,  or |

| making  the  image  lighter  or  darker,  may  invalidate  the |

| correspondence  between  pixel values and the original optical |

| density in the image. This will occur if the  O.D.  table  for |

| the  image is non-linear and densitometry  is done with `pixel |

| correction' set to `on'. The  numerical  results  obtained  by |

| scanning   under  those  conditions  could  be  scientifically |

| invalid.                                                       |

|                                                                |

| If you plan to quantitatively analyze your image after altering|

| its contrast or brightness, first check that the O.D. table for|

| the image is a straight line (Select `Configure...Show O.D.    |

| table'). Alternatively, set `pixel compensation' to `off'      |

| before any quantitative analysis.                              |

|                                                                |

| This warning does not apply to changes made by dragging the    |

| palette bar.                                                   |

******************************************************************

For color images:

Multiplies the r,g, and b components  of  all  pixels  in  the

currently-selected  image  or screen region by separate values

(100=no change), increasing the contrast.

If the selected region has both color and monochrome pixels,

you can select independent contrast factors for the monochrome

intensity and the red, green, and blue components of the color

region.

8.4.4   Change palette (colormap)

In 8-bit/pixel modes, each pixel value is translated through a

"palette  table"  which you can easily modify in TN-Image. The

new table is saved along with the image (for TIF,BMP,and TGA

files) so the next time you load the image, the display changes

to the modified palette. Changing the palette has two advantages

over other methods of changing brightness or color: (1) it is

much faster, and (2) it is always reversible since it never

affects the original image data. The disadvantages are that

changing the palette only works in pseudocolor screen modes,

and it always changes the entire screen. If you need to brighten

just a portion of the screen, use the "color intensity/brightness"

menu option.

In color modes, the current palette table is used as a guide

to determine how an monochrome image is converted to color when

it is loaded from disk, but does not otherwise affect the display.

8.4.4.1 Select palette

You can select from a variety  of  different  color  palettes.

Changing  the  palette  has  no effect on the image when it is

stored or densitometrically  analyzed. However, it can be used

to enhance the visibility of details in the image. Each image

can have a different palette.

The following pre-set palettes are available:

Gray scale     - continuous shading of 256 levels of gray.

Multi-color 1  - a  continuous  blend  of  colors useful for

enhancing low-contrast images.

Multi-color 2  - a  discontinuous  sequence  of  short  color

gradients

Multi-color 3  - a discontinuous sequence of color gradients

arranged oppositely from multi-color 2.

RGBI           - 4 continuous gradients, 0-63=red, 64-127=green,

128-192=blue and 193-255=gray scale.

Spectrum       - A single continuous gradient from blue through the

colors of the spectrum to red and then white.

Black to green - Same as gray scale except black to green

instead of black to white.

Zebra          - Alternating black and white - useful for finding

very subtle patterns in the image.  Creates a

contour-line effect.

Brightness     - Lightens or darkens palette, increasing or

decreasing color saturation.

Other  - User-selected palette. Select the desired number with

the mouse to select from  a  wide  range  of

different computer-generated palettes.

1-100     = Various gray scales

101-1000  = randomly-generated gradients. The

amount of color gradually increases

in intervals of 100.

1000-10000= continuous, smooth  gradients without

sharp breaks in the colors.  The number

of light and dark cycles varies from

1 upwards.  Some are a single color,

others  are  gray scales, and others

are multiple colors.

Example: 2345 = shades of `gold'

8.4.4.2 Grayscale intensity mapping

Grayscale images of greater than 8 bits/pixel often contain subtle

details which would not ordinarily be visible. This option permits

interactively changing the way the pixels are mapped to the screen,

creating, in effect, a "sliding scale".

This sliding scale uses 4 parameters: the maximum and minimum

values in the image to display, and the maximum and minimum

values on the screen to which these are mapped (these latter 2

do not normally need to be changed). When a monochrome

image is first loaded, TN-Image automatically determines the

maximum and minimum values in the image to allow the entire image

to be viewed. By selecting "Grayscale brightness", you can use the

mouse to select the actual range of values to be displayed.

For example, if the image has 12 bits of grayscale depth, it will

be possible to set the maximum and minimum to any number between 0

and 4096. Setting the numbers close together enhances the contrast

in that intensity range. Similarly, making the numbers small

enhances the contrast in the darker areas of the image. To make

the change visible, click on "Preview".

The maximum can also be lower than the minimum, creating an

inverse image. Each image on the screen can have a different

brightness/contrast level.

Note: Color images must be converted to monochrome before this

feature can be used.

8.4.4.3  Read palette

Reads  a  palette file from disk. Palette files are ASCII text

files which can be created with  a  text  editor,  or  created

graphically within TN-Image (see `Create palette').

If  you  create  a  palette  file  manually, your palette file

should consist of 4 columns of  up  to  256  rows.  The  first

column  is  the  color number or `index', an integer between 0

and 255, indicating which  color.  The  other  3  columns  are

integers  between  0  and  63, which define the amount of red,

green, and blue respectively for each color. For example:

0    0    0   63

1   63   63   63

255    7    7    7

This file would set color 0 to light blue, color 1  to  bright

white,  and  color 255 to dark gray. All other colors would be

unchanged (Normally,  a  palette  file  would  have  255  rows

instead of only 3).

8.4.4.4 Save palette

Saves the current palette into a disk file.

8.4.4.5 Create palette

Allows you to graphically create a custom palette. There are 3

boxes,  one  for  red,  green,  and  blue,  which  graphically

represent  the  red, green, and blue intensities of each color

in the palette. To change the  palette,  click  on  the  graph

corresponding  to  the desired color and trace out the desired

new graph while holding the left mouse button down. The  graph

and  palette  will both adjust in real time to reflect the new

values.

Clicking `smooth' causes the computer to automatically  smooth

out  the  newly-drawn curve to create a more even dispersal of

colors in your palette. When you are satisfied  with  the  new

palette, click on "OK".

8.4.4.6 Invert palette

Inverts the color look-up table so the image appears as a

negative. This differs from "invert colors" in that inverting

the palette has no effect on the actual image data. The two

extreme colors (black and white) are not switched.

8.4.4.7 Rotate palette

Rotates the palette table through all 255 colors. This effectively

increases the number of predefined palettes to 2.5 million. Useful

in achieving fine control of the appearance of the image and

enhancing subtle details.

If  you find a number that gives a useful effect, write the number

down for future reference or save it using "Save Palette". The actual

pattern of colors in the palette is not easily predictable from the

color number. (Saving the currently-displayed screen or a portion

of it as a TIF or PCX file, or selecting `Save palette' will also

save the palette.)

8.4.4.8 Restore original palette

Restores the palette the image had when it was originally loaded

from disk. Useful when you accidentally rotate the palette of

a GIF file.

8.4.5   Remap colors - Changes the colors in an image by substituting

from a map file. It acts differently on pseudocolor and color images.

8-bit/pixel images:

Changes the pixels in the image or screen region by substituting

from a set of values specified in an ASCII file. This file should

consist of a list of 2 columns of numbers.  The 1st column is the

pixel value to change, the 2nd column should be the desired new

values. For example:

1         255

2         200

3         180

4         175

255           0

This  file would change all pixels with a value of 1 to pixels

with a value of 255, those with values of 2 to 200, 3 to  180,

4  to 175, and 255 to 0. Any other pixels would be unaffected.

Typically in a remap file, you would remap all 255 colors, but

this is not essential.  Be  sure  colors  0  and  255  do  not

accidentally get remapped to the same value, otherwise it will

be impossible to use the menus.

The most common use of this is to reduce the number of separate

intensity values in an image, by eliminating those values in regions

that are not of interest. This can greatly improve the compress-

ibility of an image.

Another use is to stretch the contrast. Since the new intensity

values can be known in advance, contrast enhancement can be done

on multiple images in an exactly reproducible manner.

The Macro Editor in the Registered version can be used to create

remap files without leaving TN-Image.

Color images:

Interchanges r,g, and b color values. This could be used, for

example, to convert an image to a single color or to correct

for color errors in a corrupted image file.

8.4.6   Invert colors

Changes  the  image into a negative. This differs from "invert

palette" in that the pixel values themselves are changed.

8.4.7   Change color depth

Converts the selected image to a different number of bits per

pixel. This option only works on entire images. When working

with pseudo-color images, it is necessary to use this option to

convert them to color images before filtering them (see "Filtering

color images") or using other operations that assume the image

is color. Images can be converted from any depth (8,16,24,or

32 bits per pixel) to any other depth.

If the starting image is an 8-bit image with a palette, the

palette colors are used to select the appropriate colors for

the new image. These can be easily changed (see "Change palette").

If the starting image is a color image, and it is being converted

to 8 bits/pixel, the currently-selected  palettization method is

used to calculate the colors.  The default method is "quantization".

This can be changed (see "Color Settings").

Once converted, writing the image to disk will cause it to be

saved in its new depth.

8.4.8   Color -> Gray scale

Converts the image from color or pseudo-color to gray scale

(luminosity). The relative contributions to luminosity from

the red, green, and blue components are determined by the

"Luminosity factors" which can be changed by selecting

"Color...Color settings".

The default is:    value = .299*r + .587*g + .114*b

The results for 8-bit images may be different from the results

of merely changing the palette to gray scale, because for 8-bit

images, the conversion creates a new palette to match the intensities

in the image as closely as possible.

8.4.9   Gray scale -> color

Deactivates grayscale mapping, reactivates color manipulation

features, and tries to restore previous color information if

possible.

8.4.10  Histogram

Plots a histogram of the no. of  pixels  with  a  given  value

within  the  selected  area  vs. the pixel value. Quite useful

when trying to optimize  the  contrast  or  brightness  of  an

image.  Clicking  on  the  edge of the histogram allows you to

move it to some other screen location in the  same  manner  as

moving the dialog boxes and click boxes.

Clicking  `capture'  allows  you to capture all or part of the

graph into a new image. After clicking on `capture',  use  the

mouse  to select the desired portion of the graph. A new image

which includes that region is then automatically created.

Clicking `subtract baseline' allows you to subtract a baseline

from the histogram. Select the desired baseline by clicking as

many times as  desired  to  create  the  baseline  curve.  The

baseline  curve  is  constructed  using  a B-spline (see under

`B-spline curve'). You can modify  the  curve  as  desired  by

clicking and dragging the small boxes (which represent control

points  for  the curve). Up to 200 control points can be used.

The control points do not have to be within the graph  region;

however,  any negative numbers created by baseline subtraction

are truncated to 0's.

See  `strip densitometry' above for more details on the plot

functions.

Note:  When a graph is being displayed, smoothing, subtracting

baseline, etc., affect the displayed data only,  and  have  no

effect on the image.

8.4.11 Change pixel value

This menu option was previously called "Change brightness/contrast".

The effect is consistent across all image types and screen modes,

but is slower in 8-bit modes than the "Change brightness" option.

For 8 bit/pixel images:

This adds a value (-255  to 255) to all pixels in the currently-

selected image or screen region, generally making it darker or

lighter. (The actual effect on the image will depend on the

palette).

NOTE: See the warning under "Contrast".

For color images:

This adds or subtracts a value for the red, green, and blue

components of the currently-selected image or  screen  region,

making it darker or lighter or altering its color balance.

8.5.0   Draw  menu

Most of the drawing operations listed below put TN-Image in the

drawing mode, in which each mouse click creates another graphic

element. To return to normal mode, click anywhere on the top menu

bar.

8.5.1   Set color

Allows you to change the foreground and background colors. The

foreground color is used for all text, lines, circles,  spray,

arrows,  etc.  The  background color is used for the backspace

and delete keys, and when erasing an image from the screen.

You can also change the foreground color by clicking the  left

mouse  button when the mouse cursor is pointing to the desired

color in the `palette  display'  (the  long  vertical  bar  or

square  at the right of the screen). Similarly, the background

color can be changed by clicking the right mouse button on the

palette display.

8.5.2   Line

Draws lines. Click at the starting position, drag to  the  end

position,  and  then  release  to  draw a straight line. Click

anywhere on the top menu bar to return to normal mode.

8.5.3   Circle             - Draws circles of the specified size.

8.5.4   Box                - Draws boxes by the click-and-drag method.

8.5.5   Arrow              - Draws an arrow. After selecting `arrow',

move the mouse cursor to the desired

location and click the left mouse button.

An arrow will be drawn at that position.

8.5.6  Curve  - Draws a curve or line defined by user-selected

control points.

8.5.6.1  Bezier curve    -

Draws  a  smooth  curve. Click on the desired locations for the

control points. At least 3 points are required to draw a curve.

The position of each control point is indicated by a small box.

The control points can be moved interactively by clicking on an

existing box and  dragging  it  to  a  new  location.  Clicking

outside  a  box  creates  a  new  control  point.  When you are

satisfied with the shape of the curve, pressing any key  erases

the  small  boxes and makes the Bezier curve permanent. You can

have a maximum of 199 control points.

8.5.6.2  B-spline curve

Similar to Bezier curve, except has a maximum  of  196  control

points.  B-spline  curves differ from Bezier curves in that the

curve tends to be closer to the control points and  sharper  in

appearance.

8.5.6.3 Least-squares line - Fits a straight line between the control

points using a least-squares algorithm.

8.5.6.4  Polygon  -  Draws a polygon through the control points.

8.5.7   Text direction     - Toggles between normal and 90� rotated text.

8.5.8  Font

Currently, fonts are only available on the Unix version. You can

select from one of the 10 default fonts typically available on X11,

or click on "other" to enter a font specification in X11's format

(e.g., *lucida-medium-r*100*). See the X man pages for additional

information.  You can also select the stroke weight (medium or bold),

or the point size (10 -24).  Checking "vertical text" causes the text

to be printed sideways.  Checking "opaque text" causes the background

to also be drawn, using the current background color.

8.5.9   Sketch             - In sketch mode, whenever the left mouse button

is pressed, a continuous line is drawn on

the screen. Sketch mode is also toggled

on/off by the F2 key.

8.5.10  Fill region        - Flood fill

Fill type

Solid           - fills a region with a constant color

Horiz.Gradient  - fills a region with colors increasing or

decreasing left to right

Vert.Gradient   - fills a region with colors increasing or

decreasing top to bottom

Solid fill color  - Used for `solid' fills only. Specifies the

color to be used when filling.

Max border color  -

Min border color

In  a  flood  fill, all pixels in a given region bounded by a

"border color" are changed. Outside the border, the image  is

unchanged.  In  TN-Image, instead of one single border color,

you can select a range of colors. Any pixel within that range

will act as a border. For example, if  you  set  Max.  border

color = 200 and Min. border color = 127, then only the region

containing  pixels  less than 127 or greater than 200 will be

filled. This is useful since many real-world  images  do  not

contain  a single continuous line of pixels which could serve

as a "border".

If you try to fill a bounded region (such as a box), the color

of the boundary must be between the minimum and maximum border

colors. Otherwise, the color will spill out of the region and

possibly ruin other images on the screen.

8.5.10.1 Gradient fill

Grad start coordinate

Grad end coordinate -

Specifies the starting and ending referende coordinates that will

be used to calculate the color. The closer the starting and ending

coordinates are to each other, the steeper the gradient. The "Start

Gradient Color" will be used at the Start Coordinate and the "End

Gradient Color will be used at the End Coordinate. All other colors

will be calculated from these 2 points. Note that filling itself

is bounded by the Border Color and not by the Start and End

coordinates.

For horizontal gradients, use x values for the start and end

coordinates; when making vertical gradients, use y values.

For 8 bit/pixel screen modes:

Start gradient color

End gradient color  - Selects the intensity value that is to be

used at the starting and ending coordinates,

respectively.

For other bit/pixel screen modes:

Start gradient red

End gradient red

Start gradient green

End gradient green

Start gradient blue

End gradient blue   - Selects the RGB color value that is to be

used at the starting and ending coordinates,

respectively.

These parameters specify the color values to use at the gradient

starting and ending coordinates. Note that the  "starting" color

can  be  larger  or smaller than the "ending"  color. This gives

greater flexibility, especially in true-color modes, for creating

a blend of one color into another.

Sometimes,  as when filling the inside of letters in text, it is

desirable to have the appearance of the same gradient  extending

out  of  sight  between  the letters. This can be done easily in

TN-Image, simply by not changing the starting and ending gradient

colors. Clicking inside each area to be filled sequentially will

cause  the  same  colors  to be used as if the entire region had

been filled with a gradient.

Special care should be given to starting and ending r,g,  and  b

values  when  working  in  15-  and  16-color  modes. You should

specify slightly different starting and ending values for each color

in order to avoid a  "stepped"  appearance of the gradient. This

prevents all 3 colors from being incremented at the  same  place

on the screen, due to the fact that each color only has 31 or 63

discrete values.

Example 1:

Creating a rectangular area with a gradient increasing in brightness

from left to right.

1. Start TN-Image in mode 103 by typing:

tnimage -mode 103

2. Click on the palette to select a light color, e.g., 250.

3. Select "Draw...Box" and draw a box. Note that the box is drawn in

color 250.

4. Make a note of the starting and ending x coordinates of the box

(Shown on the menu bar).

5. Select "Draw...Fill region".

6. Click on "Horizontal gradient".

7. For "Grad start coordinate" enter the leftmost x position of

the box.

8. For "Grad end coordinate" enter the rightmost x position of

the box.

9. Enter 251 and 249 for "Max border color" and "Min border color",

respectively. The flood fill will stop at any pixel that is a

249, 250, or 251.

10.For "Start gradient color" and "End gradient color" enter 0 and

255, respectively. This will cause the color to change from 0

to 255 as x changes from the starting to ending coordinate of

the box.

11.Click on "OK".

12.Move the mouse to any point inside the box and click. The box

should fill up with the gradient.

Example 2:

Removing a gradient background from an image (flattening the

background).

If an image is too light on one side and too dark on the other,

one way to fix it is by adding or subtracting an artificial gradient.

In the Registered version, this can be done automatically, by

selecting "Filter...Background Flatten".

1. Move all images off to one side to create a blank area on the

screen.

2. Select "Color...Set colors" and change the background color to

black (0) and the foreground color to white (e.g.,255).

3. Draw a box as in Example 1, slightly larger than the image that

has the uneven background.

4. Set the Grad Start and Grad End Coordinates to match the position

of the box.

5. Set the Max and Min Border Colors so that the color of the box

is between the Max and Min Border Color.

6. Set the Start and End Gradient Colors to values which, when added

to the corresponding parts of the image, will equalize the

intensity. For instance, if the image colors are mostly 10-20 on

the left and 90-100 on the right, set the Start and End Gradient

Colors to 80 and 0, respectively. The imarge should be dark enough

so that adding these colors will saturate the image. If the image

is too light already, you should set the Start and End Gradient

Colors to 0 and 80, respectively, and use "Subtract" instead of

"Add" below.

7. Fill the box with the gradient.

8. Move your image back onto the screen (don't cover the gradient).

9. Select "Configure...Set pixel mode" and click on "Add".

10. Select "Image...Copy" and click-and-drag to copy a region from

your image. Paste it on top of the gradient. The image will be

"added" to the gradient.

11. Select "Configure...Set pixel mode" and set the pixel mode back

to "Overwrite".

12. Select "File...Create Image" and select the area you just pasted.

This will put the modified area into an image buffer.

13. Select "Image...Erase Background" to clean up the desktop.

8.5.11  Paint Region

Sets a rectangular  region  to  the  foreground  color.  After

selecting  `paint region', move the mouse cursor to one corner

and click and drag to the other corner. The selected area will

be painted.

8.5.12  Diffuse spray

Creates  a  spray-paint  effect. Click the left mouse button at

the center of the region to be sprayed.  The  area  covered  is

determined by the "spray factor" in the "configuration" menu.

8.5.13  Fine spray

Creates a solid painting effect. Click the left mouse button at

the  center  of  the  region to be sprayed. The area covered is

determined by the "spray factor" in the "configuration" menu.

8.5.14  Add border  - Puts a border around the current image. The border

color is selectable.

8.6.0   About menu

8.6.1   About the program - Displays the version number, amount of

free memory, the number of images

currently loaded, the number of images that

have been backed up, and the x and y starting

position of the current (top) image. You can

also use this option to switch to a different

image, by selecting the desired image from

the list with the arrow keys or mouse, and

pressing <Enter> or clicking on <OK>.

8.6.2   About the file    - Displays a variety of technical information

about your image file. This is useful in

diagnosing problems with reading a possibly

corrupted file or in getting parameters from

an unknown file format such as that produced by

certain specialized frame grabbers.

8.6.3   About the image   - Displays a variety of information about the

currently-selected image, including whether

it is backed up, has an FFT, its original

filename and image depth, etc.

8.6.4   How to register   - Displays vital information concerning how to

register your copy of TN-Image and obtain a

more powerful version along with technical

support.

8.7.0   Configure menu

8.7.1   Show menu bar (DOS)

If checked, the menu bar at the top will be continuously visible.

Sometimes it is  convenient to make it invisible, as when

photographing the screen.  Even when invisible, clicking on the

appropriate region will still activate the menu.

8.7.2   Show menu bar 2   - Displays additional information. Toggles

the second menu bar on/off.  (DOS)

8.7.3   Show palette - If checked, a vertical palette strip or box will

be drawn.  (DOS)

8.7.4   Redraw menu bar   - Re-draws the menu bar, in case some

operation wrote on top of it. (DOS)

8.7.5   Redraw palette

Click the left mouse button to indicate where the new palette strip

should go. The palette is redrawn. The palette is always kept in a

window of its own, separate from the images.  The palette can be used

to select the foreground and background colors (by clicking the left

or right mouse buttons, respectively, on the desired color).

8.7.6   Show O.D. table   -

Draws a graph showing the correspondence between pixel values (0

to 255) and optical  density,  as  calculated  by  the  scanner.

Typically,  only  TIF  files  from  scanners  have an O.D. table

(referred to in TIF files as a `gray  response  curve'.  If  the

image  does  not provide one, TN-Image will create a linear O.D.

table. (Note: most Macintosh TIF files do not have O.D. tables.)

The O.D. table is only  meaningful  in  monochrome/indexed-color

modes  (8  bits/pixel).  It is also known as a "gamma correction

table".  Each image can have a separate O.D. table.

Changing this O.D. table is possible but not recommended.

8.7.7   Pixel interact mode -

Selects how 2 pixels interact with each other when  you  copy  and

paste parts of an image, read an image from disk, or add graphical

elements.

Overwrite - Default mode. The new pixel erases the old pixel. This

is by far the fastest mode.

Maximum   - The larger of the 2 values is used.

Minimum   - The smaller of the 2 values is used.

Add       - The 2 values are added (up to the maximum value).

Subtract  - The new value is subtracted from the old value (with a

minimum result of 0).

XOR       - The 2 values are XOR'd with each other.

Average   - The 2 values are averaged.

Superimpose - The new pixel replaces the old pixel unless the new pixel's

value is 0, in which case the old pixel is unaffected (allows

superimposing images with parts "masked out").

Example 1: Finding subtle differences between two images (also creates a

silhouette effect).

(1). Load an image into TN-Image.

(2). Select "Pixel interaction mode...Subtract".

(3). Select "Load image" and change "x position" and "y position" to 2.

(4). Click on "OK".

(5). You now have a subtracted image. (It may be necessary to make it

lighter or increase its contrast.)

Example 2: Creating contour maps of an image.

(1). Load an image into TN-Image.

(2). Select "Pixel interaction mode...XOR".

(3). Select `copy', and click-and-drag to select a portion of the image.

(4). Release the mouse button.

(5). Move the mouse to a location 1 pixel to the right and 1 pixel

below the original position and click the mouse button to put

the copy in place.

(6). You now have a crude contour plot of your image. The gradient

sensitivity of the contour plot can be decreased by using an

offset of 2 or more pixels in step (5). You can also "threshold"

the plot by subtracting a value ("make darker") and then adding

it back("make lighter").

Example 3: Reducing chunkiness in image.

(1). Read an image into TN-Image.

(2). Select "Pixel interaction mode...Average".

(3). Select "Load image" and change "x position" and "y position" to 2.

(4). Click on "OK".

(5). You now have an image that is smoother without becoming blurred.

Don't forget to change "Pixel interact mode" back to "Overwrite"

afterwards.

NOTE: Don't forget that, when loading an image, unless the

interact mode is "overwrite", the image will interact not only

with other images but also with the background in areas where no

image is present. Thus it may be helpful to set the background

color to black first, to avoid unexpected results.

8.7.8  Configure...

Configures the following options:

8.7.8.1   Automatic undo    - If checked, automatically creates an

"undo buffer" when a new image is read.

This can also be done manually, by

selecting "Process...Backup image".

If you make a mistake when filtering or

adding text to the image, you can

restore the most recent backup from the

undo buffer by selecting "Process...Restore".

8.7.8.2  Spray factor      - Controls the area affected by "fine

spray" and "diffuse spray" (in "Draw"

menu).

8.7.8.3  Cursor movement rate  - Controls (a) distance moved by the mouse

cursor when the arrow keys are pressed,

and (b) the distance which an image moves

when you click one of the 4 arrows at

the top of the screen.

8.7.8.4  Color settings    - Changes the default parameters used by TN-Image

when converting images.

Color reduction method - used when converting a color image to

pseudo-color (8 bit/pixel) modes.

Quantization   - (Default) Calculates the optimal set of 256

colors to match the image as closely as

possible.

Fit current palette

- Calculates the closest match to the currently-

selected palette. This method can give a

smoother result than quantization or it can give

garbage, depending on how closely the colors in

the palette match the image.

If there are several images that need to be

converted to the same palette, the best procedure

is to convert the first one using quantization

then convert the rest by fitting to the current

palette.

8.7.8.5  Significant digits  - Changes the number of significant digits to

display when using a floating-point number.

8.7.8.6  Crawl delay      - Selects the speed of the "crawling" of the box

that indicates the selected area. A value of

4000 disables crawling. A lower value causes

faster crawling, but too low a value will cause

flicker on some displays, and slows down the

mouse cursor on slower machines.

8.7.8.7 Crawl density     - Selects the number of dots in the "crawling box"

that indicates the selected area. A smaller

value makes the box easier to see, but slows

down the mouse cursor on slower machines.

8.7.8.8 Active color planes

- Selects which colors (red, green, or blue) are

capable of being modified. For example, if "red"

and "green" are un-checked, smoothing the image

would only smooth the blue component, while the

red and green components were unaffected. This

option only has an effect when TN-Image is

running in high-color or true color screen

modes (i.e., 15 to 32 bits/pixel).

8.8.0  Help  menu

Displays the Help screen. Context-sensitive help is also available

for several topics by clicking on the "Help" button.

8.9.0  Miscellaneous features

Alt-E   -  Erase image

Erases the currently-selected image (after asking you).

Useful if memory somehow gets filled with images and there

is insufficient memory to open a window. This can sometimes

happen in a Windows DOS box but should not happen in DOS.

Alt-R   -  Rebuild screen display

If for some reason the display is incorrect, Alt-R will

cause it to be rebuilt.

F2      -  Toggle Sketch mode on/off

8.9.1  Alt-T    -   Test video

Tests the maximum frame rate of your video system.

Here are some typical frame rates (fps) in DOS on a 80486 33MHz PC

with ISA:

Screen mode

Video card        mode 100   101   103   105   107   111   112   115

xres 640   640   800  1024  1280   640   640   800

yres 400   480   600   768  1024   480   480   600

bpp    8     8     8     8     8    16    32    32

-------------------    ----  ----  ----  ----  ----  ----  ----  ----

#9GXE64-2MB VRAM S3     -    7.69  4.54  4.34  3.33  2.63  2.50  1.00

ATI Expression Mach64  12.5  9.09  5.26  5.55  1.27  2.63  1.59  0.89

Kelvin 64 (Cirrus) 2MB 16.7 12.50  8.33  5.00  3.33  3.22  1.89  2.00

Tseng ET4000 8-bit*     4.3  3.57  2.22  1.61   -     -     -     -

Trident 8900 1MB       10.0  8.33  4.34   -     -     -     -     -

Realtek 1MB RTG3106     8.3  6.66  4.00  2.85   -     -     -     -

* = Card in bad shape

- = Unable to set mode

Speed is not the most important parameter, however. On a high-quality

monitor, cheaper cards may appear fuzzy or jittery or have weird-

looking fonts. PCI bus video cards are in general about 5-10 times

faster. Frame rates will be much lower in Unix X11 versions and

Windows versions (if one becomes available) of the program.

9.0.0  Windows and OS/2 compatibility

It is recommended to use the included PIF file (TNIMAGE.PIF) when

running TN-Image under Windows.

The suggested procedure is:

(1) Copy and unzip TN-Image in the desired location, e.g.,

C:\TNIMAGE.

(2) In Windows, Select "File...New program item"

(3) Set "Description" to "TN-Image".

(4) Set "Command line" to C:\TNIMAGE\TNIMAGE.PIF

(5) Set "Working directory" to C:\TNIMAGE

(6) Set "Shortcut Key" to "none"

(7) Leave "Run minimized" unchecked.

(8) Select "Change icon" and enter "C:\TNIMAGE\TNIMAGE.ICO"

(9) Click on "OK".

Virtual Memory

Because Windows takes over virtual memory, running TN-Image

under Windows will affect virtual memory usage. Before starting

TN-Image, make sure the amount of Virtual Memory selected in the

Windows Control Panel is greater than the amount of RAM in your

computer. Failure to do this can result in frequent "Out of memory"

errors. Windows' virtual memory is much slower than the VM built

into TN-Image. Hence, there is no particular advantage to running

TN-Image under Windows. Also, as with all applications, speed

under Windows will be markedly slower than in DOS.

A peculiarity of Windows is that if you have the wrong mouse

driver installed in Windows, you can lose control of the mouse when

you run a mouse-based DOS program such as TN-Image. If this

happens, run the "setup" program in Windows to ensure  that

Windows is set up to use the mouse driver that is appropriate for

your mouse. For example, if you have a Logitech mouse, using the

Microsoft mouse driver may cause the mouse to become immobilized

when you return from TN-Image.  Because Windows uses a different

mouse driver from DOS, the mouse  may work under Windows but not

in a DOS box. If the mouse doesn't work right when running under

Windows, you almost certainly need a new (or different) mouse driver.

An alternative approach which sometimes works for some reason is to

change Windows from its default VGA screen mode to a 256 color mode.

Windows 3.1 also has an "unexpected feature" of sometimes not

redrawing its own screen when returning from DOS graphics programs

such as TN-Image. One solution is to "minimize" the Windows screen

after returning from TN-Image and then immediately "Restore" it. This

forces Windows to redraw its screen.

Computers with ATI Mach 32 cards have problems running TN-Image

in true-color mode from within a DOS box. In this case, the only

solution is to run TN-Image from DOS, or use TN-Image in an 8 bit/pixel

mode (e.g., mode 103). (See "Command-line options" above). Mach 64

cards are a little better (they can handle 16 bit/pixel modes, but

still can't handle 24 bit modes in a DOS box). Most other cards have

no problems.

For both OS/2 and Windows, TN-Image must be run in "full screen" mode.

For OS/2, it may be necessary to change the DOS box parameters to

get TN-Image to run properly.  TN-Image may not run on all

OS/2 configurations, because it needs to access the hardware.

NOTE: NO TECHNICAL SUPPORT IS PROVIDED FOR TN-IMAGE RUNNING UNDER OS/2.

NOTE:

In a Windows DOS box, the virtual memory manager in TN-Image

sometimes gets confused if Windows is set up for a small amount

of virtual memory.  This causes the "Free Memory" display to be

incorrect, and more importantly, allows you to completely fill up

memory with images, making it impossible to use the menus. If this

happens, you can still erase an image by clicking on the image you

want to erase and pressing Alt-E.  We are trying to find a better

solution for this problem. In the meantime, it is recommended that

Window's virtual memory be set to a size greater than the amount

of RAM in your computer.

10.0.0  Changing Computers

If you experience problems after moving TN-Image to a different

computer, or after changing video cards, erase the file TNIMAGE.INI.

This file contains TN-Image's video mode selection, and may inhibit

automatic detection of the video chip in your new computer. TN-Image

will revise the file automatically if it detects an incompatibility.

11.0.0  Problems

Problem: Strange colors on screen or TN-Image only using top 1/12

or 1/7 of the screen.

Solution: Some other Super VGA programs and communication programs

(such as some versions of Carbon Copy) contain an "unexpected

feature" that results in the computer being left in a state that

prevents proper functioning of TN-Image with certain video cards.

The DOS shell program 1Dirplus and one HPLC data-acquisition

program also have a similar feature which can interfere with the

palette registers. If TN-Image suddenly begins to run incorrectly

after running one of these programs, reboot your computer and

start TN-Image again. This can also occur when running TN-Image

as a DOS box in OS/2. Change the DOS box parameters to correct

the problem.

Problem: Entire screen compressed into upper 1/7 of the screen.

Solution: (1) Try starting TN-Image with the command line:

TNIMAGE -OLDVESA

(2) If that doesn't work, you probably are using an outdated VESA

VBE TSR (Terminate-and-stay-resident program). Obtain a new version

from the video card manufacturer. Note that most new video cards

already have VESA BIOS built in, and don't need a TSR. Try running

TN-Image without it. If nothing works, contact the author for

assistance.

Problem: Lower part of screen flickering (upper 2/3 of screen is

okay, lower 1/3 is "snow" or "static".

Solution: This occurs when the video card reports to TN-Image that

it is OK to set the specified high-resolution screen mode, but in

fact the card has insufficient video RAM to handle it. This will

occur on certain Cirrus-based cards. To solve the problem, it is

necessary to install additional memory in your video card. Using

TN-Image when this problem occurs could result in a system crash.

Problem: Screen is completely white.

Solution: This occurs when using one of the "experimental" Tseng

modes, if the video card is unable to handle the specified screen

mode. Press Alt-X two or three times to return to DOS, then use

the "mode" command line option to run TN-Image in a different

resolution.

Problem: Multiple copies of Menu Bar.

Solution: This is usually caused by an old video card with VESA

1.0 BIOS. Start TN-Image again using the command line:

TNIMAGE -OLDVESA

This can also happen if the UNIVBE TSR is present.

UNIVBE must be removed in this case.

Problem: Printer prints part of image, then gives an error message

or goes into a "form feeding frenzy".

Solution: This can occur on laser printers if your printer has

insufficient internal memory to handle the image. You will have

to install additional printer memory, print a smaller image or

print at a lower resolution or dither size. Generally, �4 MB is

required to print a 8x10 inch page in black-and-white at 600 dpi.

Color printing requires 3 or 4 times as much. Unfortunately, there

is no way for TN-Image to test how much memory the printer has. For

some reason, printer manufacturers have also made it impossible

to print an image in small segments. This problem does not occur

on inkjet printers.

This will also occur if printing is attempted on a dot-matrix

printer.

Problem: Mouse gets "lost" when running TN-Image from a Windows

DOS box.

Solution: This occurs with some Microsoft and possibly other mouse

drivers. Exit Windows, and replace your mouse driver with a newer

version or with a Logitech mouse driver. Then use Windows' Setup

program to change to the new mouse. Alternatively, use Windows'

Setup program to change Windows' screen display to 256 colors

(It's not known why this helps).

Problem: Screen is not restored correctly after running TN-Image

from a Windows DOS box.

Solution: Install a different video driver in Windows using

Windows' Setup utility.

Problem: Mouse jumps around on the screen, or exhibits jerky

movement as if trapped in a grid.

Solution: Obtain a new mouse driver. This can usually be obtained

from a bulletin board or ftp site operated by the mouse manufacturer.

Alternatively, Logitech mouse drivers can work on many mice. Older

mouse drivers often have trouble in SVGA modes. Microsoft mouse

drivers are particularly prone to these sorts of troubles. Some

old SVGA cards, such as Realtek video cards, create difficulties

with the mouse, making it possible to reach only x and y coordinates

that are divisible by 4. In this case, the video card must be

replaced.

Problem: Screen is garbage when running TN-Image from a Windows

DOS box, even though there is lot of video RAM.

Solution: This is a problem in computers with ATI video cards in

Windows for Work Groups for some screen resolution modes.

Run TN-Image at a lower color depth by changing the "Command Line"

entry in Windows' "Properties" menu to:

C:\TNIMAGE\TNIMAGE.PIF -mode 103

Alternatively, run TN-Image from DOS.

Problem: Unusual behavior or display colors immediately after

upgrading from a previous version of TN-Image.

Solution: Delete the configuration file TNIMAGE.INI.

Problem: Message "Invalid image file" appears after transferring

an image file from a Unix or mainframe system.

Solution: The most common cause of this problem is the failure to

use "binary" mode when sending the image file over the network.

If you are using Kermit, give the command

set file type binary

before getting the image. If you are using ftp, give the command

binary

before getting the image. It is very difficult to fix an image file

that has been corrupted by sending it in text mode.

Problem: Loading or processing of images is slow.

Solutions:(1) Speed can be greatly increased by starting TN-Image in

a screen mode that has the same color depth as the images. Analyzing

24-bit color images in 8-bit mode, for example, will be slow because

the entire image projected to the screen has to be re-palettized every

time any operation changes one or more pixels.

(2). If your hard disk light goes on during an image processing

operation, this means TN-Image is paging out to disk. This can be

reduced by installing more memory in your computer.

(3). Speed can be approximately doubled by running TN-Image

from DOS instead of in Windows or by removing any expanded-memory

managers such as 386^Max or (especially) EMM386.

Problem: Palette rotation in startup screen is slow or jerky.

Solution: If you are running the Unix version, this can happen if

another copy of xtnimage is running, or if the mouse is moved

suddenly during startup. In the DOS version, this is usually

caused by a slow video card.

Problem: "Insufficient memory" message occurs even though "About the

program..." indicates there should be enough memory.

Solution: Turn "Automatic undo" option off. This will cut the storage

requirements in half. Also, a certain amount of memory is also

reserved by TN-Image for dialog boxes, etc. and is unavailable.

If this happens in Windows, you must increase the "Virtual Memory"

setting in Windows ("Control panel..386 Enhanced..Virtual memory.)".

Problem: Video card won't go into desired screen mode, even though

video card manual states that mode is supported.

Solution: (1) If ATI card: Run ATI's "Install" program again to verify

the specified video mode is activated. (2) Run TN-Image's diagnostics

by typing:   tnimage -diag  to determine the problem, which is usually

caused by insufficient RAM or a monitor limitation.

(3) Another possibility is that the computer was turned on before

turning on the power to the monitor. Some video cards only examine

the monitor type at power-up, will become confused if the monitor

is off during a cold boot-up, and refuse to set high-resolution modes.

Problem:  Monitor makes a buzzing sound and fails to sync on startup

(causing jagged diagonal lines on the screen).

Solution: Turn the monitor off immediately and press Alt-X several

times to stop the program. Restart TN-Image using a lower screen mode.

This problem could occur if your video card reports that it is safe to

set a given screen mode when in fact it is not safe. Only one card has

been found so far that does this, when setting a 1280x1024 resolution

mode.  Prolonged operation in this condition could harm your monitor.

Problem: A grayscale image appears completely black or white.

Solution: This is usually caused by the automatic grayscale mapping

feature misinterpreting the upper and lower gray levels of your image.

If the image is being read as "raw bytes", make sure you have selected

the correct number of bits/pixel, x size and y size. If the y size

you entered is too large, TN-Image may read beyond the end of the

image and incorrectly estimate the maximum and minimum levels.

To correct this, select "Color...Set palette...Grayscale mapping"

and change the maximum and minimum values to correspond to those in

the image. (For example, if the image is known to be 12 bits deep,

the maximum should be approximately 2^12 or 4096.)

Problem: "Mouse required" message occurs when starting TN-Image,

even though Windows runs OK with the mouse.

Solution: Windows has its own separate mouse driver. It is still

necessary to load a mouse driver for DOS applications. Run the

program "mouse.com" that came with your mouse.

Problem: When typing text on the screen, it comes out in the wrong

color.

Solution:  The available colors are determined by the color depth,

the gray scale mapping parameters and palette (if present) of the

image you are typing on. For example, if an 8-bit image with a

particular palette is loaded, you can only add colors that exist

in that palette, even if TN-Image is in a 24-bit color mode. This

prevents you from accidentally adding a color which is impossible

to save with the image. If all else fails, press Alt-R to rebuild

the screen display.

Problem: The message "Converting from 24 to 8 bits/pixel" stays on

the screen for a long time.

Solution: Select "Color...Color settings" and make sure "Color reduction

method" is set to "Quantization" and not "Fit current palette" which

can be slow.

Problem: When reading color images, the colors are wrong, or it takes

a long time.

Solution: Select "Color...Color settings" and make sure "Color reduction

method" is set to "Quantization" and not "Fit current palette" which

may give incorrect colors.

Problem: After reading a GIF file or a color image, subsequent images

in other formats appear to be "garbage".

Solution: This only occurs in 8-bit/pixel screen modes. When an image

that does not have its own palette is loaded, the palette is not

automatically changed. If the current palette is discontinuous, the

new image may appear to contain garbage (of course, it is not really

garbage). Simply click anywhere on the background, or use the "Change

palette" menu option to change to a continuous palette.

Problem: Medical grayscale images appear grainy or posterized; or

appear as strange shades of blue instead of gray.

Solution: The wrong target platform may have been selected. Select

"File...Create custom format" and change the "Target platform" and/or

"Bytes to skip" for the specified format until a smooth image is

obtained.

If you are using the Shareware version, you will need to read the

image as a "raw" file and tweak the image parameters manually.

Problem: Deconvolution result is completely black, completely white,

or garbage.

Solution: This can be caused by (a) using an inappropriate point

spread function, (b) using a point spread function that has zero or

near-zero intensities at one or more frequencies, or (c) wrap-around

effects. Sometimes, adding noise to the point spread function will

increase the intensity at the missing frequencies and solve the problem.

Alternatively, try a less ambitious point spread function. Try dumping

the FFT data to disk and reading it with a text editor. If the numbers

are all "-nan"'s, this is a sign that the intensities were too low

and a different psf must be used. Some images simply don't work well

with deconvolution.

Problem: "Fill Region" was selected, but nothing happened when mouse

was clicked on the region to fill.

Solution: Make sure the area being filled is darker (lower intensity

value) than BOTH the Max. Border Color and the Min. Border Color.

Filling will stop whenever any color between these two values is

encountered.

Problem: Mouse cursor moving by itself.

Solution: Make sure mouse is on a level surface.

Problem: "out-of-memory" problems running TN-Image in a Windows DOS box.

Solution: This is usually caused by an incorrect setting in

your virtual memory. Try the following procedure to fix it:

1. Click on Windows Control Panel

2. Click on "386 Enhanced"

3. Click on "Virtual Memory"

4. If the size of the Virtual memory is less than the amount of

RAM in your computer, problems may occur. Change the amount of

Virtual Memory to the maximum recommended by Windows; or exit

Windows and run TN-Image from DOS.

Problem: Spots floating across screen, or jagged flickering lines.

Solution: You are working too hard, go lie down.

12.0.0  Limitations and known bugs

In the Unix version, the width or height of the viewable area cannot

be made larger than the width or height of the screen. Color sharing

with other applications could be friendlier in the Unix version.

When making the main window smaller in the Unix version, the actual

resizing is not done until the window is re-exposed.

When an FFT is present, it is impossible to flip the image.

13.0.0  Error messages

All versions

------------

Insufficient conventional memory to run program

Insufficient extended memory to run program

These errors usually mean an extended memory manager is

required but not present. The simplest solution is to find

the file HIMEM.SYS (which comes with DOS) and install it

in your CONFIG.SYS file, e.g., add the line

C:\DOS\HIMEM.SYS

to CONFIG.SYS (make sure HIMEM.SYS is in the DOS

directory before doing this).

Also, check to see if you have an expanded memory

manager installed. These programs sometimes block access

to portions of video memory or high memory.

Not enough memory

Insufficient memory to perform the requested action (see

"Operating under low-memory conditions" above). This message

should not occur in the Registered version of TN-Image,

unless there is insufficient disk space.

Insufficient memory to convert to 8 bits/pixel

A color map table could not be generated for the image. When

this happens, the image cannot be converted to the screen

resolution, leaving "garbage" on the screen.  Try changing

`color reduction method' to "fit current palette". This

method uses less memory. Alternatively, start TN-Image in a

color mode and try again. This message should not occur in

the Registered version of TN-Image, unless there is insufficient

disk space.

Internal error [...]

Contact author for assistance, specifying the exact message

and the circumstances under which it occurred.

Can't compact free space

This can occur when you try to unload an extremely large image.

The only consequence is that the maximum amount of free

memory could not be recovered. Usually, erasing the image

again will free up the space.

Too many images

The limit of 512 images (or 2 images for the Shareware version)

was exceeded while attempting to read an image from disk

or create a new image. Don't forget that a new image is also

created when you try to rotate or resize an image or carry out

an FFT.

Image was not backed up

This only occurs when you select "Restore" when

the "Auto undo buffer" option was un-checked, so that new

images were not automatically backed up. Although un-checking

`auto undo buffer' creates more free memory, it means that it

will be impossible to undo any changes by selecting "restore".

Function not available in shareware version

The function is only available in the Registered version of

the program.

Function not available

The function has not yet been implemented. Contact the author

for availability dates.

Error - mixed expand and shrink operations#

You cannot shrink an image in one dimension and enlarge it

in another simultaneously, it must be done in two passes.

Error: Bad OD table, Turn pixel compensation off

Densitometry could not be performed because part or all of

the selected region was on an image whose O.D. table mapped

two or more pixel intensity values to the same O.D. You must

either repair the OD table, or select "No pixel compensation".

#Bad scan parameters

This occurs if the starting and ending points of your

densitometry scan are the same, or if the region could

not be scanned for some reason. Select the region again.

#No scans to save#

"Save Scan" was selected from the menu, before "Strip

densitometry".

Can't create file!#

An invalid file name was specified, or the filename was the

same as some existing file that DOS has marked as "read-only",

or a "directory".

Can't find file!

The specified filename does not exist, or is marked by

the operating system as "hidden". Use a utility such as

NSHELL to un-hide the file.

Only the 1st image in your file will be displayed

Your TIFF file contains more than one image.

Sorry, can't read xxx compressed TIF images

TIF files are sometimes compressed using a weird form of

compression not supported by TN-Image.

Not a valid TIF file

The TIFF file is corrupted and unreadable, or is an unusual

non-standard TIF format.

Warning: File transfer aborted#

You aborted the operation of saving the image to a file by

pressing <ESC> or clicking on "Cancel"; or, saving of the

file was aborted by the program for some reason.

An error occurred#

A "Critical error" occurred in connection with a file transfer.

This normally leads to the message "Abort, Retry, Ignore?".

This occurs if you try to save a file to a floppy drive that

does not exist, does not have a disk in it, has an open

door, etc. It can also happen if a problem is found with your

disk.

No images to save

You selected "Save image", but no images were currently loaded.

#Printer not responding!#

Printer I/O error!#

Printer out of paper

The printer is off-line, powered off, or something.

Unable to read disk

The specified drive letter referred to a non-existent

disk; or there was a disk drive failure. Occasionally,

CD ROM drives will spuriously give this error.

Invalid GIF file#

Error decoding GIF file#

Unable to read GIF file#     #

The GIF file is corrupted and unreadable. The most common

reason for this is that, when you downloaded the file, you

forgot to set the communications package to "binary" mode.

For example, in ftp, before getting the file, type:

binary

In Kermit, before telling the remote Kermit to send the

file, type:

set file type binary

You may also need to make sure your local computer is set

to "binary" mode. The command for this differs for each

communications package.

Number of colors doesn't add up

Change the bits/pixel on a color

or number of primary colors

You have selected an incompatible combination of options.

The number of primary colors must correspond to the number

of colors which have non-zero bits/pixel. For example, if

you select "1" as the number of primary colors, only one

of the 4 entries (red, green, blue, or black) can have a

non-zero number of bits. The rest must be 0.

RGB bits/pixel don't add up to total

Change total bits/pixel or number of colors

You have selected an incompatible combination of options.

The sum of the red, green, blue, and black bits to be used

to save an image must be equal to the total bits/pixel

selected. For example, if you select to save the image as

17 bits/pixel, the red + blue + green + black bits must

also total up to 17.

If you are reading a raw 8-bit monochrome image, the "color

type" must be set to "Gray Scale/Pseudo".

You must have only 1 primary color

to save data as grayscale

You have selected an incompatible combination of options.

If you want to save the image as "Gray scale" image data,

you must also change the "no. of primary colors" to 1.

Non-standard image format - is this ok?

This means the file format selected is not standard, and

some other programs may have difficulty reading the file

that is created.

Not enough bits for CMYK

You must either select 32 bits/pixel mode, or "other bpp"

mode, in order to create a CMYK file. If you select "other

bpp", you need to specify the number of bits to use for

black as well as red, green, and blue.

You must select CMYK to use black bits

Setting a non-zero value for "black bits/pixel" is only

allowed if you have also selected "CMYK".

Must have 4 colors for CMYK

If you selected "other bpp", you need to also select "4"

as the number of primary colors in order to create a CMYK file.

PCX files must be 8 bits/pixel

TN-Image can only create 8 bit/pixel PCX files. You must

convert your image to 8 bits/pixel before saving it as a

PCX file.

IMG files must be 1 or 8 bits/pixel

TN-Image can only create 1 or 8 bit/pixel IMG files. You must

convert your image to 8 bits/pixel before saving it as an

IMG file, or select "monochrome".

File is an unsupported type of PCX file.

The PCX file had too many bit planes, or was not 1 or 8

bits/pixel.

No header file specified

This only occurs when "Create custom file" is selected.

If you specify a non-zero number of header bytes to copy

from a file, you must also specify a file name.

Specified header bytes exceeds length

of header file

This only occurs when "Create custom file" is selected.

You selected a number of bytes to copy into the header of

a custom file format which exceeded the file size of the

specified file.

Bad gradient parameters

One or more of the coordinates specified for a gradient fill,

or one of the boundary colors, was bad. Reselect another

similar area and try again.

Invalid parameters,

Setting kernel multiplier to 1

For some types of filtering, the kernel multiplier must be

set to 1. TN-Image has done this automatically.

Kernel size is larger than selected area

The size of the kernel to be used for for filtering, multiplied

by the kernel multiplier, is larger than the image. This

would result in no filtering, so the filtering was stopped.

You may have accidentally click-and-dragged a 1x1 area.

Try re-selecting the area to filter again.

Bad scan parameters

A trapezoidal region selected for scanning happened to have

an illegal combination of values. Select a slightly different

region and try again.

Error converting image

Contact the author for assistance.

Too many peaks

The number of peaks exceeded the size of the list TN-Image

allocated for them.  Contact the author to obtain an

upgrade which can handle more peaks.

Can't find help file

The help file, TNIMAGE.HLP, was not in the starting directory.

Bad regression order

This is usually caused by trying to draw a curve or calibrate

an image using only one calibration or control point. At least

3 control points must be selected.

No unique solution

Division by zero

These messages indicate that it was impossible to calibrate the

image using the existing calibration points.  Calibrate the

image again using slightly different control points.

Error: negative value in logarithm

When calibrating an image in logarithm mode, all calibration

values must be positive.

You must select "Other" or "Custom"

to save an image with non-standard parameters

TN-Image detected a non-standard value entered for bits/pixel

or number of colors. To protect against accidentally creating

non-standard files, you must specifically check the "Custom"

file format or the "Other" bits/pixel selection.

Error: need two images to convolute

Convolution or deconvolution of images was selected, but

less than 2 images were present.

nn Divisions by zero detected

During deconvolution of two images, if the frequency of the

2nd image is zero at any point, division by zero occurs.

If there are a large number of these, it usually means the

2nd image is not entirely appropriate to use for deconvolution.

Can't open list file

A file containing a list of images was expected, because of

the "-FILES" option, but either no file list name was given

or it was an invalid file name.

Extension must be TGA for Targa format

Since there is no foolproof way to determine whether a file

is really in TGA format, TN-Image will classify any file with

a TGA extension as a Targa file.  If you specified some other

extension, the file would be unreadable. Thus, TN-Image does

not allow you to save the file with any extension other than

"TGA".

Unknown error - nothing was saved!

This message should not occur in normal use. Contact the

author for assistance.

Selected bit/pixel values will be ignored

Images are automatically converted to 24 bits/pixel before

saving in JPEG format. Other values are illegal and are ignored.

Color information will be lost!

You should convert image to 8 bits/pixel first

You are about to save a color image in a file format that does

not have color information. When loaded back, the new image

might not have the same colors as the original. Select "Change

image depth" and quantize the image first by converting it to

1 byte per pixel.

Error: overlapping offsets

While creating a custom image format, you cannot set two or

more file offsets to overlap the same byte position. Each

offset occupies 2 bytes.

Error: offset exceeds 1024

While creating a custom image format, you cannot set a file

offset to a number greater than 1024.

Macro terminated at line xxx

A fatal error occurred while executing a macro. The macro

was terminated at the specified line. Commands following the

offending line were not executed. A fatal error can include

the following:

Out of memory

File not found

No images available for the operation

Can't create file

Critical error  (e.g., disk drive door open)

File was zero length (after creating an image file, the

resulting file was checked and found to be empty).

DOS/Windows-specific error messages

-----------------------------------

Fatal error, DPMI host does not support 32 bit applications

Fatal error, 80386 processor is required

Fatal error Previously installed software is neither VCPI nor DPMI

compatible

Fatal error allocating DOS memory

16 bit code and data are too large

Fatal error, insufficient conventional memory

Cannot enable the A20 line, XMS memory manager required

FATAL error, XMS memory corrupted

16 bit code is too large

DPMI failed to enter protected mode

DPMI operating system error

One or more of these messages will occur if you do not have

an 80386 or higher processor, or if some other program is

providing DPMI services in a way that is incompatible with

TN-Image.  This might occur if you are running some unusual

type of memory manager or have an incompatible type of CPU

chip. Certain hardware problems can also cause these

messages. Try booting from a floppy or removing memory

managers from AUTOEXEC.BAT.

Fatal error reading disk��

FATAL error during virtual memory disk IO

These messages mean that something bad happened to your disk

or to the swap file used for virtual memory. Try to free

up more disk space and try again. This can also happen if

you run TN-Image Registered version from a floppy, and

then remove the floppy.

Mouse required

No mouse driver was found.  Run the file "MOUSE.COM"

or "MOUSE.EXE" (a file that came with your mouse) and try

again.

VESA not present

VESA BIOS not found

These messages mean that no VESA driver was found. This

driver is a file that should come with your super VGA card,

with a name like "VESA.COM". Install the VESA driver and try

again. This message will also occur if you do not have a super

VGA card.

Can't set VESA mode

#Unsupported VESA mode

These messages mean that even if you have a VESA-compatible

card, it cannot handle the selected screen mode. See under

"Command line options" above for instructions on setting other

screen modes.

#Unsafe/unable to set VESA mode

Your monitor reported that it is unable to handle the

scan rate for the specified resolution, or the video card

refused to set the video mode for some other reason.

Trident card not found

This only occurs if you specified "Trident" on the command

line and no Trident card was present.

Unsafe to test for Tseng chip

This occurs if you specified "Tseng" and no Tseng Labs chip

was found. Certain non-Tseng chips cause a system lock-up if

you try to check for a Tseng mode, thus the program did not

try to set the Tseng mode.

VGA mode set OK - Unrecognized SVGA chip

TN-Image found that you have a VGA card, but it is not

VESA-compatible and not a Tseng or Trident card. Probably

it is not capable of super VGA modes.

Can't open graphics device

No grapics card was found.

Unable to set video mode

You specified a video mode on the command line that is not

supported. See "Supported video modes" above.

You either have insufficient video memory or an ET3000 chip

#You need 1MB of RAM and an ET4000 chip

A Tseng Labs card was detected, but it is either too old

(ET3000 chips are not supported) or does not have enough

video RAM. On most cards, it is a simple matter to

purchase and install an additional video RAM chip.

Error: Monochrome chip

A monochrome Tseng Labs chip was found on your video card.

You need to upgrade to a newer card.

Error: ET3000 chip

An ET3000 Tseng Labs chip was found on your video card.

You need to upgrade to a newer card.

Error: S3 not currently supported, use VESA

Error: ATI chip not supported, use VESA

Error: XGA not currently supported

The "-S3", "-ATI", and "-XGA" options for directly programming

these chips are not available yet.

**Warning: unusual value**

An unexpected value was obtained from the VESA BIOS in your

video card. TN-Image may not function correctly.

#Unsupported Tseng mode

A Tseng mode was specified which could not be set by your

video card or is not supported by TN-Image.

#Unsupported Trident mode

A Trident mode was specified which could not be set by your

video card or is not supported by TN-Image.

#Wrong switch setting on Trident video card

#Check your manual to ensure your card is configured correctly

A Trident card was detected, but was configured incorrectly.

Please consult your video card manual or computer dealer.

#Insufficient video memory in Trident card#

1MB video ram required

A Trident card was detected, but was found to have less

than 1 MB of video RAM. Take the card to your dealer to

have additional RAM installed.

#Unable to set video mode on Trident card

#(Error code=%x)

A Trident card was detected, but the video mode could not

be set for an unknown reason. Consult the author or your

computer dealer.

#Sorry, your type of SVGA card is not supported

TN-Image does not support your type of SVGA card; or, you

did not load your VESA BIOS before running TN-Image. This

is a file that should come with your super VGA card,

with a name like "VESA.COM". Run this program and try again.

This message may also occur if you do not have a super VGA

card.     #

#DPMI host can't lock error handling code!

The DOS protected mode interface supplier (i.e., Windows,

386^Max, etc) made an error.

The simplest solution is to find the file HIMEM.SYS

(which comes with DOS) and install it in your CONFIG.SYS

file, e.g., add the line

C:\DOS\HIMEM.SYS   .

Then, run the program from the DOS command line instead

of within Windows.

UNIX-specific error messages

----------------------------

There are a huge number of these. Below are the most common.

Can't open display

X11 must be running.

No visuals found

No appropriate visual

X11 is running in an unsupported screen mode.

Error allocating size hints

Error allocating class hint

Error creating XTextProperty

Error allocating Window manager hints

Can't allocate colors

XCreateImage failed

error in dialogbox

14.0.0  Trademark disclaimers

Graphics Interchange format and GIF (SM) are service marks of

Compuserve Incorporated.

This software is based in part on the work of the Independent JPEG Group.

Windows and UNIX are trademarks of Microsoft Corporation and X/Open

Company, respectively.

PostScript is a trademark of Adobe Systems, Inc.

This software was developed as an independent, unfunded project and

has no association with the United States Government, the National

Institutes of Health, or the National Institute of Neurological

Disorders and Stroke.

15.0.0 Index

Subject                  Section

--------------------    -----------

2D Fourier Transform     8.3.11

about menu               8.6.0

about the file           8.6.2

about the image          8.6.3

about the program        8.6.1

angle                    8.3.3.2

area measurements        8.3.5.1

area selection           6.3.0

arrow                    8.5.5

ATI                      7.2.0

automatic undo           8.7.8.1

B-spline curve           8.5.6.2

back up                  8.2.10

background               7.4.0

background flatten       8.3.1.4

background subtract      8.3.1.3

background, erase        8.2.3

baseline                 8.3.7

basic operation          7.4.0

Bezier curve             8.5.6.1

bits/pixel               6.1.0

bits/pixel               8.1.2

black and white printing 8.1.3.1

BMP image files          6.1.0

border                   8.5.14

box                      8.5.4

brightness               8.4.2

brightness               8.4.2.1

brightness               8.4.2.2

bug reports              3.2.0

bugs                     12.0.0

calibration              8.3.4

capture                  8.3.7

change size              8.2.6

changing computers       10.0.0

changing title           8.1.4

Chinese                  8.1.1

circle                   8.5.3

CMY, printing            8.1.3.2

CMYK                     8.1.1

CMYK                     8.1.2

CMYK, printing           8.1.3.2

color adjustment         8.4.2

color, command line      7.1.1

color depth              8.4.7

color menu               8.4.0

color printing           8.1.3.2

color reduction          8.4.7

color reduction          8.7.8.4

color settings           8.7.8.4

color, setting           8.5.1

color->gray scale        8.4.8

colormap, see palette

colors, inverting        8.4.6

colors, remapping        8.4.5

command line options     7.1.0

command line options,Unix 7.1.1

commonly asked questions 7.6.0

Compuserve               0.0.0

Compuserve               14.0.0

configure menu           8.7.0

contents                 1.1.0

contrast                 8.4.3

contrast                 8.4.3.1

contrast                 8.4.3.2

convolution              8.3.11.1

copy                     8.2.5

crawl delay              8.7.8.6

crawl density            8.7.8.7

creating file format     8.1.8

creating image           8.1.6

crop                     8.2.2

CT images                8.1.1.1

cursor movement rate     8.7.8.3

curve                    8.5.6

curve                    8.5.6.1

curve                    8.5.6.2

custom format files      8.1.8

de-trending              8.3.1.4

DECONVO2.TIF             6.4.0

DECONVOL.TIF             6.4.0

deconvolution            8.3.11.2

deleting region          8.2.1

DENSITO2.TIF             6.4.0

DENSITOM.PCX             6.4.0

densitometry             8.3.5

densitometry             8.3.6

depletion                8.1.3.2

depth                    8.4.7

Diamond                  7.1.0

diffuse spray            8.5.12

diffuse spray factor     8.7.8.2

digital filtering        8.3.11.3

direction                8.5.7

disclaimers              14.0.0

disclaimers              3.0.0

display, command line    7.1.1

display, problems with   11.0.0

distance                 8.3.3.1

dither, printing         8.1.3.1

DOS Command              8.1.12

draw menu                8.5.0

e-mail address           0.0.0

edge detection           8.3.1.8

edge enhancement         8.3.1.6

edge enhancement         8.3.1.7

editing text files       7.8.0

equations                8.3.10

erase background         8.2.3

erasing image            8.1.11

error messages           13.0.0

features                 8.9.0

FFT                      8.3.11

FFT, erasing             8.1.10

FFT, loading             8.1.14

FFT, saving              8.1.9

file format, creating    8.1.8

file menu                8.1.0

file types               6.1.0

fill region              8.5.10

filter                   8.3.1

filtering color images   8.3.1.10

fine spray               8.5.13

flip horizontally        8.2.8

flip vertically          8.2.9

ftp sites                0.0.0

ftp sites                6.1.0

frame rate test          8.9.1

frame rates              8.9.0

gamma correction         8.7.6

garbage collection       13.0.0

Gem Img files            6.1.0

geometry                 7.1.1

GIF                      6.1.0

GIF                      8.3.1.0

GIF                      7.6.0

GIF files, problems with 11.0.0

gradient fill            8.5.10.1

gray balance, printer    8.1.3.2

gray scale               8.4.8

gray scale               8.4.9

gray scale->color        8.4.9

grayscale                8.4.4.2

help menu                8.8.0

high pass filter         8.3.1.1

HIMEM.SYS                13.0.0

histogram                8.4.10

how to register          8.6.4

Hubble                   8.3.11.2

Ima files                6.1.0

image menu               8.2.0

imaginary                8.3.11

Imaging Systems files    8.1.1.2

IMDS                     8.1.8

Img files                6.1.0

index                    15.0.0

intensity                8.4.2

intensity mapping        8.4.4.2

interact                 8.7.7

interpolation            8.1.3.1

introduction             1.0.0

invert colors            8.4.6

invert palette           8.4.4.6

JFIF                     6.1.0

JPEG                     3.1.0

JPEG                     8.1.2

JPEG image files         6.1.0

kernel multiply factor   8.3.1.9

Laplace edge enhancement 8.3.1.6

least-squares            8.5.6.3

license                  3.0.0

lighter, printing        8.1.3.1

limitations              12.0.0

line                     8.5.2

line, least squares      8.5.6.3

linear calibration       8.3.4

load image               8.1.1

logarithmic calibration  8.3.4

low pass                 8.3.1.2

low-memory conditions    7.5.0

Lumisys                  8.1.8

Macintosh , images from  8.1.1.3

macro command summary    8.3.12.1

macro programming guide  8.3.12.2

macros                   8.3.12

mapping, intensity       8.4.4.2

mathematical             8.3.10

measure                  8.3.3

media type               8.1.3.2

median filter            8.3.1.5

menu bar                 8.7.1

menu bar                 8.7.2

menu bar                 8.7.4

menu bars                6.3.0

menu options             8.0.0

menu, invisible after loading image 11.0.0

miscellaneous features   8.9.0

mode                     7.1.0

mouse                    5.0.0

mouse, jumpy             7.2.0

mouse, problems with     11.0.0

move                     8.2.4

moving image             6.3.0

MVS                      8.1.8

NMR images               8.1.1.1

noise reduction filter   8.3.1.5

Nyquist                  8.3.11

O.D. table               8.7.6

operation                7.0.0

ordering                 0.0.0

OS/2                     9.0.0

overview                 6.0.0

paint region             8.5.11

palette                  8.7.3

palette                  8.7.5

palette bar              8.4.2.1

palette bar              8.4.3.1

palette, creating        8.4.4.5

palette, fitting         8.1.0

palette, fitting         8.7.8.4

palette, inverting       8.4.4.6

palette, options         8.4.4

palette, reading         8.4.4.3

palette, restore         8.4.4.8

palette, rotating        8.4.4.7

palette, saving          8.4.4.4

palette, selecting       8.4.4.1

paper type               8.1.3.2

PCL                      5.0.0

PCL                      8.1.3

peak areas               8.3.8

pixel compensation       8.3.5

pixel compensation       8.3.6

pixel intensities        6.2.0

pixel interact mode      8.7.7

pixel operations         8.3.10

pixel value, changing    8.4.2.2

pixel value, changing    8.4.11

planes, color            8.7.8.8

plot                     8.3.7

plotting data            8.3.7

point spread function    8.3.11.2

polygon                  8.5.6.4

polynomial  calibration  8.3.4

Postscript               5.0.0

Postscript               8.1.3.1

power spectrum           8.3.11

printer                  5.0.0

printer, problems with   11.0.0

printing images          8.1.3

print quality            8.1.3.1

problems                 11.0.0

process menu             8.3.0

quantization             8.1.1

quantization             8.7.8.4

questions, common        7.6.0

quit                     8.1.13

raw ASCII images         8.1.1.4

raw bytes                8.1.1

raw bytes                8.1.1.1

real                     8.3.11

Realtek                  7.2.0

reconstruction           8.3.11.2

redraw menu bar          8.7.4

redraw palette           8.7.5

register                 8.6.4

registered               3.1.0

registered versions      3.1.0

registered, features     3.1.0

registration             3.0.0

remap colors             8.4.5

resizing image           8.1.6

resolution, printing     8.1.3.1

restore                  8.2.11

restore palette          8.4.4.8

RGB                      8.1.1

RGB                      8.1.2

RGB, printing            8.1.3.2

rotate palette           8.4.4.7

rotating image           8.2.7

S3                       7.1.0

saving images            8.1.2

saving scan data         8.1.6

scan                     8.3.9

screen modes supported   7.3.0

select box               8.7.8.6

selecting region         7.4.0

selecting region         8.1.1

set color                8.5.1

settings, color          8.7.8.4

shareware                3.0.0

shareware version        3.1.0

shareware version, limitations   3.1.0

show menu bar            8.7.1

show menu bar 2          8.7.2

show palette             8.7.3

significant digits       8.7.8.5

site license             3.0.0

size, change             8.2.6

sketch                   8.5.9

sliding scale            8.4.4.2

Sobel edge enhancement   8.3.1.7

spline                   8.5.6.2

spot densitometry        8.3.5

spots/ jagged lines      11.0.0

spray                    8.5.12

spray                    8.5.13

spray factor             8.7.8.2

strip densitometry       8.3.6

summary                  2.0.0

super VGA                5.0.0

superposing images       8.7.7

switch to...             8.1.7

system requirements      5.0.0

Targa                    6.1.0

technical support        3.3.0

test                     8.9.1

text direction           8.5.7

text files               7.8.0

TIFF                     6.1.0

TIFF parameters, extra   8.1.2

trace curve              8.3.9

trademarks               14.0.0

Trident                  7.1.0

Trident                  7.2.0

Trident                  7.3.0

Tseng                    7.1.0

Tseng                    7.2.0

Tseng                    7.3.0

tutorial files           6.4.0

undo                     8.2.11

undo, automatic          8.7.8.1

Univbe                   11.0.0

Univbe, problems with    11.0.0

Unix                     7.6.0

Unix                     7.1.1

Unix, printing           8.1.3.3

unknown image files      6.1.0

unloading image          8.1.11

upgrade policy           4.0.0

usage license            3.0.0

VESA                     7.0.0

VESA                     7.3.0

video cards supported    7.2.0

video cards, frame rates 8.9.0

video RAM requirements   5.0.0

video, problems with     11.0.0

virtual memory           3.1.0

warp                     8.3.2

warranty                 3.0.0

Windows                  5.0.0

Windows                  9.0.0

Windows 9x               7.6.0

X-ray images             8.1.1.1

XGA                      7.1.0