WHAT SCSI-ACCELERATOR IS ALL ABOUT.The accelerator works only on a MacPlus! It enhances the throughput of I/Ooperations for so called blind read and write operations. Nothing else isaffected.The reason that the performance of these operations can be enhanced isthat Apple's code to do these operations is (and must be) suited to handlea variety of disk types. Some of these are slower than others. In the followingdiscussion we will talk about read operations only, but the discussion appliesjust as well to write operations.When transfering data to or from a SCSI device, there is no support for hardwarehandshake on the MacPlus (there is some on the other Macintoshes). Because of this,the only really safe way to do the I/O is to poll the SCSI chip for the arrival ofa new byte each time you need one. Of course this is slow. Therefore Appleintroduced the "blind" operations. For blind operations, the system only waits forthe arrival of the first byte, the rest of them are read in a small loop which lookslike this:	@1	move.b	(a1),(a2)+		dbra	d6,@1This means that after every transfer of a single byte, the Macintosh waits at leastthe time to execute the DBRA (about 10 cycles) before fetching the next byte. Thisis long enough for even the slowest hard disk that Apple anticipated to have thenext byte ready. In fact, most hard disk can deal with a lot less! Reduction ofthis "dead" time can be achieved by unfolding the loop, i.e. reducing the looptrip count and putting more than a single move.b instruction in the loop body.Of course, if we put two moves right next to each other, we get the fastesttransfer that is possible (knowing that we do not have a DMA controller). Thismight be too fast for some hard disks, so it may well be that we have to put oneor more NOP instructions between each two move.b instructions. The execution timeof a NOP is only 4 cycles however, much less than for the DBRA and thus, throughputcan be increased even if we need 2 NOPs per move.b. By increasing the number ofmove.b instructions in the loop body, we further decrease the looping overhead,leading to increased performance, but of course, also to more use of memory forthe code.Since the loop illustrated is 6 code bytes long, there is just enough place toreplace the loop with a JSR instruction to a patched version of the loop that appliesthese techniques. This is exactly what the SCSI-Accelerator does. The reason thatthis does not improve performance on the Mac SE or II/IIx is that those machinessupport a sort of pseudo DMA transfer mode that already takes care of getting inthe bytes as soon as they arrive. For this reason, the accelerator init refusesto install itself on anything other than a Mac Plus.HOW TO USE IT.As said before, if you do not operate a Mac Plus, forget it, it will not install!If you are using a MacPlus, the thing to do is to find out what kind of unfoldedloop will still work for your configuration. This depends mainly on two things:disk type(s) and processor (in case you operate an accelerator board). For thisreason, several variants of the init code have been provided. They are all namedSCSI-Accel-r<x>w<y>s<z>. Here the <x>, <y> and <z> are single digits meaning:<x>		The number of NOPs inserted after each move.b in the loop body for reading.<y>		The number of NOPs inserted after each move.b in the loop body for writing.<z>		The number of move.b instructions in the loop body is equal to 2^<z>This means that in general you should use the init with the lowest <x> and <y>that still works with your configuration. Once you know which one to use, the nextthing to do is to decide how you want to trade of memory and speed by choosingthe <z> that you want to use. In general z=5 works quite well.You find out which <x> and <y> version to use by putting one of the INITs inyour system folder and rebooting. If the boot works it is quite likely that thatversion works for you. Test this by duplicating a file with the finder.You should start to test this with a variant with <x> and <y> large. If it worksyou can progressively try out lower numbers. Don't worry, during boot the diskis only read and even if the Mac crashed (which is the usual symptom of a <x>,<y>which is too low), no real harm will be done.The code in this patch should work with all disks (providing you choose theright <x> and <y>. Disks with block sizes that are a multiple of 2^<z> workfastest, but any other block size will be handled correctly (for those of youthat have disks that operate with tags). The original accelerator worked onlywith 512 bytes/block disks.HOW GOOD DOES IT WORK?Since the SCSI manager read and write blind operations are patched, you will notget improved performance if your disk's driver does not use the SCSI manager. Inthat case it is quite likely that the driver will already do the same kind ofoptimalization as suggested here, so there won't be much to gain here.Supposing your driver *does* use the SCSI manager, performance will improve,depending on the type of disk you have. An example:Using a HD SC80 (Quantum Q280) disk with my own custom driver that *does* usethe SCSI manager we get the following DiskTimer II results:Variant		Reads	Writes	Seeksnone		106		105		18r0w0s5		63		67		18r1w1s5		83		82		18The SC 80 is a disk which is formatted with a 1:1 interleave. Its controllercaches a complete disk track though, hence the almost twofold improvement.For a Rodime RO632 (Some Apple HD 20's) and the same driver we get:Variant		Reads	Writes	Seeksnone		160		161		52r0w0s5		109		107		52r1w1s5		110		108		52This disk is quite a bit slower, but still there is improvement. What I have notyet tested is what happens if, in addition to using r0w0s5, I also reformat thedisk with a lower interleave factor. It might very well be that because ofthe improved data transfer rate, the Mac Plus can keep up with an interleave thatis one lower. In that case, performance would improve even more.Apple's drivers also use the SCSI manager, so the INIT should at least work withthat software. It is known that some Rodime drivers bypass the SCSI manager. Youwill have to try and measure to see wether or not the INIT works for you. In general:just try and see. Included in this package is disktimer II so that you can checkthe results yourself.Have fun, DolfDolf StarreveldDepartment of Mathematics and COmputer ScienceUniversity of AmsterdamKruislaan 4091098 SJ  AmsterdamThe NetherlandsPhone: +31 20 592 5022E-mail: dolf@uva.UUCP (...!uunet!mcvax!hp4nl!uva!dolf for oldies)