���������������������������������������������������������������������������Ŀ

� ���� Using TERMINATE with WINDOWS 3.1 or 3.11 ��������������������������� �

�����������������������������������������������������������������������������

NOTE: It seems as if "Windows 3.11 for Workgroups" does not need any of the

following changes.

To install Terminate under Windows, just create a group and create three

items in the group. Use the 3 PIF files to start the programs, so you don't

have to worry about setting them up yourself. You could use the icons found

in this directory as well.

Windows 3.1 installation tips

=============================

To use Terminate with Windows, we provide TERMINAT.PIF and TERMINAT.ICO.

But you must also make some adjustments to the Windows setup first.

Edit SYSTEM.INI

���������������

Under [386Enh]:

Com1AutoAssign=0

In the Windows menu: Main / Control Panel / 386 Enhanced:

it should read NEVER WARN at the comport you use.

The comport used for modem must not be autoassigned by Windows. If you

use COM4, just enter Com4AutoAssign=0

Change the values to match the correct address for the comports.

COM1BASE=3F8h

COM2BASE=2F8h

COM3BASE=3E8h

COM4BASE=2E8h

For other values that can be changed try reading the Windows documentation.

COMIrqSharing=true (This only has effect on EISA and Microchannel machines)

COMBoostTime=10

COM1Buffer=1024

COM2Buffer=1024

COM3Buffer=1024

COM4Buffer=1024

If your modem has a 16550 UART, you will also have to install another

communication driver to support 16550 UART FIFO in DOS-windows.

Use a driver such as CHCOMB.386 (CHCOMB.ARJ which can be downloaded from

DAN BBS) or CyberCom.

To use the soundblaster card, enter under [386Enh]:

DMABUFFERSIZE=059

If you have many problems using the Windows coms-driver, you could use the

communication ports directly by changing WIN.INI to:

COM1:=COM1.DOS or

COM2:=COM2.DOS or

COM3:=COM3.DOS or

COM4:=COM4.DOS

Information from Microsoft:

���������������������������

Windows cannot use a serial port if the address is to the right of the

missing port. An example of this problem could be that the modem is on COM4

and COM3 is not physically present.

Without a physical COM3, the BIOS will find COM4 on address 02E8 and will

call it COM3 in the BIOS field. This confuses Windows, so it will not find

COM4 and think that COM3 is on 02E8/IRQ4.

This behaviour is normal for many BIOS's and can be fixed with COMxIRQ=-1

in SYSTEM.INI under the [386Enh] section. For this example, the following

line should be inserted in SYSTEM.INI:

COM3IRQ=-1