"Personal Paint  -  10. Problem Solving"

10. Problem Solving

10.1 Memory Shortage

10.2 Problems with Disks

10.3 Problems with Printers

10.4 Problems with Screen Modes and Video Devices

10.        Problem Solving

This chapter describes some of the most common problems which may be

encountered when using the computer and its peripherals, explaining how to

solve these difficulties, and what to do in order to prevent them from the

beginning.

10.1       Memory Shortage

Computer memory is one of those few resources which, regardless of the

amount which is available, tend to be exploited to the limits. The most

common obstacles which may be encountered when using Personal Paint are

likely to be connected with insufficient Chip RAM.

Sections 1.3.2 and 4.11 respectively contain important information on

RAM in general and on Personal Paint's Memory Information function. As

indicated in those sections, the information about memory which is

displayed on the Workbench title bar may be misleading, as it sums up

fragmented memory as if it were a contiguous block.

A low memory condition may be signalled by an explicit message (e.g.

"There is not enough memory", or "Insufficient Chip RAM"), or by symptoms

like menus not appearing as usual, windows which cannot be resized or

repositioned, screen flashes indicating failures to accomplish a given

task, etc.

Chip memory may be made available by freeing the undo buffer (selecting

the Undo tool with the right mouse button - section 3.1.10), clearing the

image twice (by clicking twice on the Clear tool - section 3.1.9),

choosing a screen mode having less resolution or colors (sections 1.3.2

and 4.5), freeing the current environment (section 4.8.5), freeing brushes

(section 5.6), freeing the stencil (section 7.2.7), switching off the

audio (section 8.4), closing the Workbench screen (section 8.16),

terminating other programs, etc.

Typing "Avail FLUSH" in a Shell window, or selecting FlushLibs in the

Workbench Debug menu (which becomes available if the Workbench is

activated with a "LoadWB -DEBUG" command) frees all RAM-resident system

resources which are not being used any more. This may free some

considerable memory if several large fonts were used in previous sessions,

or if some complex libraries used by other programs are not needed any

more. When the system is in critical memory conditions, it usually

performs these actions automatically.

In an emergency (e.g. if the main screen cannot be opened), Personal

Paint may automatically and gradually try to free some memory by taking

one or more of the actions mentioned above.

10.2       Problems with Disks

The Amiga operating system may perform another disk write operation

some seconds after a file is stored. For this reason, a disk should never

be ejected from the drive (nor should the computer be switched off) for

some seconds after the disk drive activity indicator light has gone off.

Electrical noise can also cause problems. Lights, monitors and other

electrical devices connected to the same power line as the computer should

never be switched on or off while data is being written to the disk.

Errors are first signalled by messages from the Amiga operating system,

and then by the program which is trying to save the data. The most

frequent sources of errors are: disk is write protected, disk is full,

disk is damaged or not formatted (sections 1.3.4 and 1.6). Unlike 5.25"

disks, 3.5" disks are write protected if it is possible to see through the

write-protect hole.

If, while a file is being written to, the Amiga operating system

displays a message indicating that a read/write error has occurred, the

first thing to do is to eject the disk and set the write protect tab.

Precious data can be lost by trying to write to the disk after such an

error message. If the write protect tab is not set, the system may try to

write to the disk even if no write operation has been requested by the

user (especially if the disk is damaged). After the disk has been

reinserted, all the system messages should be cancelled with the mouse

(unless a "Software Error" message appears). The program which was saving

the data will display its own error message. The data which could not be

stored because of the error should be saved to another disk. Any other

material which is displayed or stored in RAM should be saved as well, so

that it can be recovered if its copy on the damaged disk has been

corrupted. Next, a copy of the damaged disk should be made (section 1.6).

If the standard system copy program does not work, because of the errors

present on the disk, other copiers should be used.

A disk recovery program like DiskSalv will probably be able to restore

most files, with the likely exception of the one which was being written

to when the error occurred. It is possible that different disk recovery

programs will produce different results. The disk on which the error

occurred should not be used again, unless the cause of the error is known

not to be hardware dependent. An error may occur again on the same disk,

especially if a single sided or poor quality disk is used, or if the disk

is dirty, or the magnetic surface is scratched. Scratches are usually

visible when light is reflected from the disk surface. The surface itself

should never be touched or contaminated.

Some minor errors are automatically corrected by a process of the Amiga

operating system called validating. If, for example, a write operation is

interrupted for some reason (e.g. a power failure, or the removal of a

disk), and no other damage occurs to the disk, the Amiga operating system

tries to validate the disk the next time that it is inserted in a drive

(or mounted, if it is a hard disk). The validation process can take a few

seconds or several minutes, depending on the size of the volume. If the

disk is write protected the validating process has to be repeated every

time the disk is inserted (or mounted). While the disk is being validated,

a "Disk not validated" system message is displayed if a program tries to

access that disk. The "Cancel" option of the message should not be

selected, as the message disappears automatically as soon as the

validating procedure terminates. It is possible to read from a disk which

has not been validated, but no data can be saved to it, since the data

structures which mark the parts of the disks which are free are not

updated.

The most serious errors can be caused by physically damaged or

defective disks. The main cause of such errors is dirt. Coffee poured over

disks lying on the table, sticky drops of orange soda "raining" from a

nearby glass or a few grains of sand inside the disk case can be very

dangerous. Unless the magnetic coating of the disk has been scratched or

corroded, the data can still be recovered. Most liquids leave the cookie

(the foil coated with magnetic media inside the disk) intact, but must be

removed as soon as possible. The cookie must be taken out of the disk case

before it can be cleaned. To do so, a demagnetized blade can be used to

carefully open the case. The metal (or plastic) shutter of a 3.5" disk can

be removed by hand. The cookie can be washed under running water and dried

with a clean, soft cloth. The clean cookie can finally be inserted into a

new case (the shutter is not needed), from where it can be copied to a

new, intact disk. The procedure described here is not guaranteed to

succeed. Improper handling can damage the disk and/or the disk drive.

10.3       Problems with Printers

Some suggestions to improve the quality of the printed output are

listed here. Possible solutions to many problems which may occur during a

print operation are also contained in this section.

The parameters of the Amiga Preferences program must be perfectly set.

The most common cause of errors is the wrong choice of printer driver. The

printer driver specified in Preferences must be the correct driver for the

printer which is used. All printer settings of the Preferences program

must be correctly set and stored on the Workbench disk. Not all printers

use the same codes and control sequences.

If the parallel port is used for a peer-to-peer connection between

computers (using a network filing system like "ParNet") then the Printer

Port setting in the Amiga Printer Preferences should be set to Serial, or

any device other than that used by the network. Use of the Amiga printer

device on the same line as the network might lock the network. Programs

like Personal Paint might need to access the printer device on startup,

even if no explicit printing is performed. Although no data is sent to the

printer by this process, this operation may lock the network and the

printer device.

When the name of the printer driver has been selected with the

Preferences program, the programs which need to use the printer will be

able to use the correct printer codes, and Amiga will activate the

appropriate printer driver. Personal Paint's printer settings may be used

to change most system printer parameters, except selecting the printer

driver itself.

By default, printer drivers are stored in the "Printers" drawer, which

is contained in the "Devs" drawers of the Workbench and Extras disks. The

Amiga operating system searches for printer drivers in the "DEVS:Printers"

and "PRINTERS" logical units. A driver stored only on the Extras disk must

be copied to the Workbench disk before it can be used. The InstallPrinter

program, on 1.3 Workbench disks, allows the user to copy the drivers from

the Extras disk to the Workbench disk in a very easy way. On operating

systems starting from version 2.1, unused printer drivers are placed in

the "Storage/Printers" drawer, from where they can be moved into "Devs"

using the mouse when necessary. Some of the most used printer drivers are

stored on Personal Paint's program disk. If that disk is used as the

Workbench disk, more updated or more specific printer drivers can be

copied to it.

If there is no driver for a particular printer model, a similar or

compatible driver can be used. Most impact printers are compatible with

one or more of the Epson (ESC/P), Nec (Pinwriter) or IBM (Proprinter)

standards. Laser printers can generally work in the HP LaserJet and/or

PostScript standard. When more than one emulation mode is available, the

selection should match the selected Preferences printer driver. The

documentation enclosed with the Amiga and with the printer should be read

to determine which is the most suitable driver for a particular printer.

Appendix D of this guide also lists several suggested drivers for the most

used printers. The printer retailer or manufacturer should be contacted if

no information can be found in the handbooks, and the existing drivers do

not work. Drivers which were included with versions of the operating

system before 1.3, as well as versions labelled Alpha, Beta, Gamma and

Omega might not work properly with most software.

The Printer Preferences program can be started with the mouse from

Workbench. If the screen has been closed, the Workbench parameter

described in section 8.16 can be set to open it again. If the Workbench is

still closed, the program should be terminated (section 4.12) to free the

memory necessary to open the Workbench screen.

If text characters are printed instead of an image, or undesired

characters appear in the text printed by a word processor, the wrong

driver was probably selected. As an example, with some inappropriate

driver/printer combinations a 'P' or 'K' character (not part of the

document) is printed at the beginning of each print operation. The page

format (A4, Letter, Wide Tractor, etc.) must also be set correctly in

either Personal Paint's or the system's Page/Printer Preferences. Again,

it is not necessary to use the system Printer Preferences, if these values

are set through Personal Paint.

When a print operation is interrupted (section 4.3), some printers may

continue to wait for the remaining image data to be sent. Software reset

commands, as well as margin, size and graphical data of subsequent prints

could be interpreted as binary data belonging to the first image. This

could result in subsequent images being printed at the wrong position, or

in the wrong size, or "trash" data being printed. To avoid this, it is

sufficient to reset the printer from its control panel (or switch it off

and turn it on again, if it lacks a reset function) after interrupting a

graphical print.

Sometimes it may be necessary to change the default factory settings of

the printer. Some printers have small DIP-switches grouped into banks,

located at the rear of the printer or under the cover. Other printers

allow the user to change the parameters by accessing the printer memory

settings through the control panel. A parameter which is often not set

appropriately is the automatic line feed function, which should be

disabled to work with most printer drivers. This has to be done if the

graphic print is interrupted by wide horizontal lines, or if the line

spacing of the text is twice the spacing which was set through the

parameters of the word processor. If accents or other particular national

characters of a specific language are not printed, and the correct driver

was used, it is possible that the character set default setting (e.g.

"Italic"/"Epson" versus "Standard"/"IBM") or the language character set

should be modified. Personal Paint only prints in graphics mode, but

specific text-mode imperfections may indicate more general problems.

If it is possible to choose between tractor (perforated paper) or

friction paper feed, it should be noted that friction feed is usually much

more accurate over short distances (e.g. single sheets), especially on

low-cost printers. On some printers friction feed may not grab a sheet

stiffly enough if the paper is too light or too thick (forcing the feed

mechanism). A discontinuous, asymmetric and imprecise paper feed may be

caused by the wrong paper type.

The presence of thin, light, horizontal lines on the printed output

(graphic or text) can be reduced by selecting the friction feed and/or

narrowing the printer head to the paper. If the inked ribbon streaks the

paper, or dirties the margins of the paper, the printer head is probably

too close to the surface of the sheet. On some printers the presence of

thin horizontal lines can be reduced by selecting a Custom Paper Size,

and/or Single paper type rather than Fanfold in the Preferences program.

The internal power supply of some printers is not sufficient to guarantee

a homogeneous pressure of all needles on the ribbon. Some drivers may let

the printer use only part of their pins (generally the upper 16 of 24) if

some parameters in Preferences are set as described above. These drivers

make an improper (but surely appreciated) use of some Preferences

parameters, which would otherwise remain unused, like Paper Type. The

Threshold setting is also sometimes used for different purposes (like

adjusting color and lightness correction in the HP DeskJet printer).

As strange as this may sound, graphics are often printed faster (and

with a higher degree of precision) in mono-directional mode than in

bi-directional mode. The mode can be selected manually or through software

control on most printers. Black and white graphic print is the fastest,

while gray level and color printing take longer.

Some settings, like Smoothing, may be activated to reduce diagonal

"steps" in the graphic prints. This usually slows down the printing.

On some color printers a black ribbon can (or must) be used to print

black text or black and white or gray graphics. In this case, only black

ink ribbons, usually more affordable than color ribbons, need be bought.

If the printed colors appear different from what is displayed, it may

be useful to apply color, brightness and contrast corrections (section

7.1.4).

A printing density which is too high (e.g. 300 to 360 dpi, rather than

50 to 150 dpi) may cause the printout to be too dark and smeared

(especially on ink-jet printers), as well as enhance undesired horizontal

stripes.

Similarly, PostScript printouts may become too dark and have very

little different shades of gray when the density (LPI) is too high.

Section 4.3.3.3 has more on this. In case of doubt, the printer's default

values should be used (Personal Paint DPI and LPI set to 0, Default Spot,

angles set to a value greater than 360). The Moir� phenomenon, which

causes undesired patterns to appear, is discussed in section 4.3.3.11.2.

Another frequent problem with PostScript printers has to do with the

serial connection between the computer and the printer. If the serial port

is used, some wires of the cable must be crossed and connected in a

different way than other, frequently used, one-to-one connections. To

allow for different types of handshaking and interactive operation, the

following wiring could be used: 1-1 (input and output pins with the same

number are connected directly), 2x3 (pin two is connected with pin three

on the other side, and vice versa), 4x5, 7-7, 6+8x20 (pins 6 and 8 on each

part are joined, and connected to the opposite pin 20). The other signals

are unused. On the software side, it is important that both the printer

and the computer use the same Baud Rate (usually 9600 or 19200), Data Bits

(e.g. 8), Parity (e.g. Even), Stop Bits (e.g. 1) and Handshaking (e.g.

XOn/XOff). These settings can usually be set from the Amiga Serial

Preferences and the printer's control panel.

Finally, a warning addressed to the owners of Amiga 1000 models. This

computer requires a non-standard parallel printer cable. Connecting the

printer cable of another Amiga model, or a standard Centronics cable may

damage the equipment.

10.4       Problems with Screen Modes and Video Devices

The main symptom of most of the problems described here are: a black

screen, flickering or unstable picture, monitor makes strange sounds and

impossible to select certain video modes or colors.

With Amiga operating systems starting from version 2.0, it is possible

to specify the monitors which can be used. Each monitor has an associated

set of screen (or video) modes. This is important, as different Amiga

computers may employ more or less powerful or flexible graphic chip sets

and monitor hardware. All display modes which are supported by the

hardware and judged to be of good quality should be activated by copying

the associated Workbench icon into the "Wbstartup" (version 2.0) or

"Devs/Monitors" (version 2.1 and 3.x) system drawers. Otherwise, Personal

Paint would not be able to use those modes. Conversely, some video modes

could have been inadvertently selected. For example, having activated the

"Multiscan" monitor on an Amiga with a monitor capable of displaying only

NTSC or PAL pictures may produce unreadable displays when loading some

images. If this happens, the monitor files in excess should be moved back

from the "Wbstartup" or "Devs/Monitors" directory into "MonitorStore"

(Workbench 2.0) or "Storage/Monitors" (versions 2.1 and beyond).

Images created on Advanced Graphics Architecture systems may not look

the same when displayed on earlier Amigas, even if the same amount of

colors is used. This is caused by the fact that more color combinations

are available for the color palette on AA systems. Personal Paint's

palette, color reduction and remapping functions will use the "coarser"

color resolution to guarantee a more correct representation of the image

if the Original or Advanced Chip Type options are selected in the boot

menu of AA systems.

If a display card is used, and the screen does not appear to be updated

properly by some graphics operations, or if a system error occurs when a

new video mode is selected, the RTG option (section 4.5.5) should be

activated.

Some graphics boards which make it possible to display screens in 256

colors or more have a color palette resolution of less than 24 bits (i.e.

less than 16 million colors). Since the standard for 256-color images is a

24-bit color palette, it may happen that images with such a color

resolution may not be rendered accurately by these cards. Personal Paint's

Remap function (section 7.3) can be applied after loading an image to

restore the original color variations. A sophisticated remapping technique

such as Floyd-Steinberg should be used (section 8.7).

Double-Scan and other higher bandwidth video modes (as are generated by

Flicker Fixer and Display Enhancer devices) require an adequate monitor

(MultiSync/multiscan, VGA, etc.) to be displayed properly. The Flicker

Fixer or Display Enhancer may have to be disabled when using some of the

highest bandwidth video modes. At the opposite end of the range, the

original Amiga video modes (e.g. "plain" PAL or NTSC) may have a bandwidth

which is narrower than the minimum of some multiscan monitors. This is a

particular problem on Advanced Graphics Architecture Amiga systems, where

several, completely different, video modes may have to be used (and the

lower horizontal scan frequencies are not corrected by a Flicker Fixer or

Display Enhancer). Commodore provides special multiscan monitors capable

of handling most video modes. Other monitors may work, or may be

re-calibrated by a qualified technician to accept lower frequency signals

(typically, problems may arise at horizontal scan frequencies below 30 kHz

and vertical refresh rates below 55 Hz).

Section 4.1 ("Load Image") has more on video modes.