SiS IDE Driver Installation For DOS

=====================================

01) Run INSTALL.EXE in the install utility diskette to install

SiS IDE DOS PIO mode driver or SiS IDE DMA mode driver.

02) INSTALL.EXE program will add the following line in CONFIG.SYS file :

"DEVICE=C:\SISIDE\SISDR.SYS"    for PIO mode driver or

"DEVICE=C:\SISIDE\SISDR.SYS /M" for DMA mode driver.

03) Users can use /A /R and /M parameters to set the wait states and

IDE BUS MASTER  CONFIGURATION SPACE BASE ADDRESS manually.

For example:

"DEVICE=C:\SISIDE\SISDR.SYS /A3456 /R6543 /M:4000"

means active time for channel 0 master is 3*30ns

channel 0 slave  is 4*30ns

channel 1 master is 5*30ns

channel 1 slave  is 6*30ns

recovery   time   for channel 0 master is 6*30ns

channel 0 slave  is 5*30ns

channel 1 master is 4*30ns

channel 1 slave  is 3*30ns

and the CONFIGURATION SPACE BASE ADDRESS is 4000h.

04) Hard Drives' default wait states are set by using Hard Drive

internal information automatically. The recommended value table

for different mode Hard Drives is as follow :

Hard Disk Mode    4       3       2      1      0

==================================================

Recovery Time     1       3       8      A      0

Active Time       3       3       6      6      0

==================================================

Note that the range for Recovery Time is 0-F

and for  Active  Time is 0-7.

05) After IDE DOS driver installed. Some mode 4 hard disks may cause the

system hang when we reboot the system. Users can avoid this by downing

the speed in manual way as described in 03) 04).

For example, change

"DEVICE=C:\SISIDE\SISDR.SYS" into

"DEVICE=C:\SISIDE\SISDR.SYS /A3 /R3"