Digital Local Management Application

User's Guide

____________________________________________________

This guide describes the use of DEClma to manage the

DEC FDDIcontroller installed in specific operating-

system environments described herein.

Supersession/Update Information:This is a new manual.

Software Version:               V1.0

________________________________________________________________

December 1992

The information in this document is subject to change

without notice and should not be construed as a commitment

by Digital Equipment Corporation. Digital Equipment

Corporation assumes no responsibility for any errors that

may appear in this document.

Restricted Rights: Use, duplication or disclosure by the

U.S Government is subject to restrictions as set forth in

subparagraph (c) (1) (ii) of DFARS 252.227-7013, or in FAR

52.227-19, as applicable.

Copyright 1992 by Digital Equipment Corporation

All Rights Reserved. Unpublished rights reserved

under the copyright laws of the United States.

The following are trademarks of Digital Equipment

Corporation:

DEC           DECndu                    ULTRIX

DECbridge     DECstation                UNIBUS

DECconnect    DECsystem                 UWS

DECelms       DECUS                     VAX

DECmcc        PDP

DIGITAL

AT, OS/2 and IBM are registered trademarks of International

Business Machines Corporation.

Microsoft, LAN Manager, and MS-DOS are registered

trademarks of Microsoft Corporation.

Novell and NetWare are registered trademarks of Novell,

Incorporated.

SCO is a registered trademark of Santa Cruz Operations,

Incorporated.

UNIX is a registered trademark of UNIX Systems

Laboratories, Incorporated.

This manual was produced by Telecommunications and Networks

Publications.

This document was prepared using VAX DOCUMENT, Version 2.0.

_________________________________________________________________

Contents

Preface...................................................      v

1  Product Overview

2  Before Running DEClma

2.1  Installation Requirements........................    2-1

3  DEClma Commands

3.1  Initiating DEClma................................    3-1

3.2  CLEAR COUNTERS...................................    3-2

3.3  EXIT.............................................    3-2

3.4  HELP [command]...................................    3-3

3.5  SELECT device_name...............................    3-5

3.5.1  SELECT device_name (SCO UNIX) .................    3-5

3.5.2  SELECT device_name (Novell) ...................    3-7

3.5.3  SELECT device_name (DOS and OS/2) .............    3-9

3.6  SET object_name [object_value]...................   3-10

3.7  SHOW ATTACHMENT..................................   3-13

3.8  SHOW COUNTERS....................................   3-14

3.9  SHOW ERRORLOG....................................   3-15

3.10 SHOW EVENTS......................................   3-17

3.11 SHOW FDX.........................................   3-18

3.12 SHOW INTERFACE...................................   3-19

3.13 SHOW MAC.........................................   3-20

3.14 SHOW PORT........................................   3-22

3.15 SHOW SMT.........................................   3-23

3.16 SHOW VERSION.....................................   3-23

3.17 QUIT.............................................   3-23

4  Error Messages

Index

iii

Examples

3-1     CLEAR COUNTERS Command Syntax .................    3-2

3-2     Output Generated by the HELP SHOW SMT

Command........................................    3-3

3-3     Output Generated by the HELP Command ..........    3-4

3-4     Output Generated by the SELECT Command

(SCO UNIX).....................................    3-6

3-5     Novell device_names (Device ID) ...............    3-7

3-6     Output Generated by the SELECT Command

(Novell).......................................    3-8

3-7     Output Generated by the SELECT Command

(DOS and OS/2).................................    3-9

3-8     Using the SET Command .........................   3-12

3-9     Output Generated by the SHOW ATTACHMENT

Command........................................   3-13

3-10    Output Generated by the SHOW COUNTERS

Command........................................   3-14

3-11    Output Generated by the SHOW ERRORLOG

Command........................................   3-15

3-12    Output Generated by the SHOW EVENTS Command ...   3-17

3-13    Output Generated by the SHOW FDX Command ......   3-18

3-14    Output Generated by the SHOW INTERFACE

Command........................................   3-19

3-15    Output Generated by the SHOW MAC Command ......   3-20

3-16    Output Generated by the SHOW PORT Command .....   3-22

3-17    Output Generated by the SHOW SMT Command ......   3-23

Tables

3-1     Configurable SNMP Objects .....................   3-12

iv

_________________________________________________________________

Preface

This manual explains how to:

o  Initiate DEClma

o  Use the DEClma commands

o  Interpret error messages

Intended Audience

This manual is for the network manager responsible for

configuring and assessing the status of DEC FDDIcontroller

interfaces.

The network manager should have a basic knowledge of the

IETF SNMP FDDI MIB, MIB-II and DEC Vendor MIB to interpret

DEClma status displays and status requests.

Product Related Documentation

The following documents provide additional applicable

information:

o  DEC FDDIcontroller/EISA Installation Manual Order No.

EK-DEFEA-IN.

Describes the installation of the hardware, driver

software, board configuration, and troubleshooting.

o  The following RFC documents are related Management

Information Base (MIB) specifications and standards

that you can obtain directly over the Internet from host

NIC.DDN.MIL by using the File Transfer Protocol (FTP)

to log in to the host with the user name anonymous as

requested. Next, change to the rfc subdirectory using

the commmand "cd rfc". Retrieve an RFC of interest with

a "get rfc" command.

v

For example, to obtain an ASCII text file of RFC 1285,

enter the command "get rfc1285"<Return>.

-  RFC1285 defines the FDDI MIB.

-  RFC1155 defines the SMI mechanisms for describing and

naming objects.

-  RFC1156 defines the MIB-I core set of TCP/IP managed

objects.

-  RFC1213 defines MIB-II (an evolution of MIB-I)

managed objects.

-  RFC1157 defines the Simple Network Management

Protocol (SNMP).

Additionally, to receive a copy of Digital's MIB

extensions, check the appropriate box on the DEC

FDDIcontroller Registration Card supplied with the

controller, and mail the card to the DEC FDDI Product

Group, 550 King Street, LKG2-1/Z3, Littleton, MA

01460-1289.

Conventions Used in This Guide

The following conventions apply to this manual:

Convention         Meaning

<Return>           Press the RETURN key.

UPPERCASE          Uppercase letters in command lines

indicate keywords that must be entered.

You can enter them in either uppercase

or lowercase. You can abbreviate command

keywords to the first three characters

or the minimum unique abbreviation.

[]                 Square brackets in command syntax

statements indicate the enclosed value

is optional. Do not type the brackets.

vi

1

_________________________________________________________________

Product Overview

DEClma is a standalone utility that allows you to manage

DEC FDDIcontroller interfaces. Using simple command lines,

you can access the FDDI (SMT, MAC, PORT, and ATTACHMENT)

and INTERFACE group objects, as described in the IETF SNMP

FDDI MIB, MIB-II and DEC Vendor MIB extensions.

You can manage objects contained in the above MIBs as

follows:

o  Display objects in the following groups:

-  INTERFACE (MIB-II and DEC MIB extensions)

-  SMT (SNMP FDDI MIB and DEC MIB extensions)

-  MAC (SNMP FDDI MIB and DEC MIB extensions)

-  PORT (SNMP FDDI MIB and DEC MIB extensions)

-  ATTACHMENT (SNMP FDDI MIB)

-  FDX (DEC MIB extensions)

o  Display counter objects from all of the groups just

listed

o  Set configurable parameters where valid for an FDDI

object

o  Display network events

o  Display and/or dump the contents of the error logger to

a file.

DEClma is currently available for managing FDDI data links

with the following operating systems:

o  Novell NetWare 386 (Server and Client)

o  SCO UNIX

o  NDIS 2.01 (DOS and OS/2)

Product Overview 1-1

2

_________________________________________________________________

Before Running DEClma

This chapter describes the installation requirements and

considerations for running DEClma on a local FDDI-connected

PC host, workstation or fileserver.

2.1 Installation Requirements

Before you install and run DEClma, verify that the

FDDIcontroller and the appropriate driver for your

operating system environment are properly installed. Refer

to the DEC FDDIcontroller Installation manual for details.

The DEClma utility and this manual are located along with

the device driver in the platform-specific subdirectories

of the distribution diskettes as described in the

Installation manual. Each platform subdirectory also

contains instructions in its README.TXT file for installing

DEClma. These platform-specific procedures must be

completed before you can run DEClma.

Before Running DEClma 2-1

3

_________________________________________________________________

DEClma Commands

This chapter describes the DEClma commands and how to

initiate them. The commands are listed in alphabetical

order.

DEClma displays appropriate messages for invalid commands

or completion of the command (refer to Chapter 4).

3.1 Initiating DEClma

Once you install DEClma, you can issue commands in two

ways. You can enter a single DEClma command from the

operating-system prompt as shown in the following examples:

# declma dfe0 show interface<Return>

This command displays the Interface Group objects for

device dfe0.

# declma dfe0 show counters<Return>

This command displays all of the SNMP counters for device

dfe0.

# declma dfe0 set treq 9<Return>

This command sets the MACTReq object to 9 ms for device

dfe0.

The keyword "declma" precedes each command, followed by the

FDDI device name and the DEClma command.

Alternately, you can enter commands interactively. If the

"declma" keyword is entered with no arguments

DEClma Commands 3-1

DEClma Commands

3.1 Initiating DEClma

#  declma<Return>

DEClma returns the following prompt

DEClma>

You can then enter any DEClma command. Upon command

completion, DEClma returns to the DEClma prompt and waits

for another command. Exit the interactive session by

entering either of the exit commands (EXIT or QUIT).

3.2 CLEAR COUNTERS

This command simulates setting all the counter objects for

the selected device to zero for this interactive session

(DEClma> prompt). (This command is not functional from

the command line mode.) Since all of the SNMP counter

objects are read only, the CLEAR COUNTERS command does

not actually zero the values of the controller objects.

Instead, the function is a relative reset and the counter

values are incremented from zero (a delta value) each time

they are clocked. The SHOW COUNTERS command displays

the values of the counters relative to the last CLEAR

COUNTERS command. Example 3-1 shows the syntax for the

CLEAR COUNTERS command.

------------------------------------------

Example 3-1 CLEAR COUNTERS Command Syntax

------------------------------------------

DEClma> clear counters <Return>

DEClma>

3.3 EXIT

This command closes the communication channel (if one

is open) for the selected device and terminates the

application.

3-2 DEClma Commands

DEClma Commands

3.4 HELP [command]

3.4 HELP [command]

This command displays information about a DEClma command.

Enter the keyword HELP followed by the DEClma command of

interest. Example 3-2 shows the output generated by a HELP

SHOW SMT command. To display a list of all DEClma commands,

enter HELP with no arguments as shown in Example 3-3.

---------------------------------------------------------

Example 3-2 Output Generated by the HELP SHOW SMT Command

---------------------------------------------------------

DEClma> help show smt

SHOW SMT :

This command displays SNMP objects from the SMT group of the FDDI and

DEC Vendor MIBs.  A device must be selected prior to issuing this

command.

DEClma Commands 3-3

DEClma Commands

3.4 HELP [command]

--------------------------------------------------------------------

Example 3-3 Output Generated by the HELP Command

--------------------------------------------------------------------

DEClma> help

SELECT device      - Opens a communication channel to selected device

device - System name assigned to identify device as a resource

SHOW INTERFACE     - Displays objects from SNMP INTERFACE group

SHOW SMT           - Displays objects from SNMP SMT group

SHOW MAC           - Displays objects from SNMP MAC group

SHOW PORT          - Displays objects from SNMP PORT group

SHOW ATTACHMENT    - Displays objects from SNMP ATTACHMENT group

SHOW FDX           - Displays objects from DEC Extended FDX group

SHOW COUNTERS      - Displays all SNMP group counters

SHOW ERRORLOG      - Displays driver-maintained error log

SHOW EVENTS        - Displays prior eight (8) unsolicited events

SHOW VERSION       - Displays this version of DEClma

SET object [value] - Sets writeable SNMP objects to new values

object - Name of SNMP object to be set or "ALL" to set default values

value  - New value of object; not included with "SET ALL" command

CLEAR COUNTERS   - Clears all SNMP group counters

HELP [command]   - Displays HELP screens

command - When present, a HELP screen will appear for the given cmd

When not present, this HELP screen is displayed

EXIT - Closes selected communication channel and terminates application

QUIT - Same as EXIT

DEClma>

3-4 DEClma Commands

DEClma Commands

3.5 SELECT device_name

3.5 SELECT device_name

This command allows you to select the controller to be

managed. The SET, SHOW, and CLEAR commands require that you

open a device before issuing the command. Since DEClma

can access only one device at a time, commands issued

interactively (DEClma> prompt) always refer to the last

device selected. Notice that you do not have to specify

whether the device is of single attachment station (SAS) or

dual attachment station (DAS) type. DEClma determines the

controller type automatically.

device_name

The device_name specification is required to SELECT the

device. The device associated with this name is opened and

subsequent commands then apply to this controller. DEClma

displays an appropriate error message if the FDDI device

cannot be opened.

The device_name specification differs depending on your

operating-system environment. Section 3.5.1 describes the

controller device names for SCO UNIX users. For Novell

server and client users, refer to Section 3.5.2.  DOS and

OS/2 users should refer to Section 3.5.3.

3.5.1 SELECT device_name (SCO UNIX)

There are three recognized device names. These devices

are dfe0, dfe1, and dfe2. If you select a recognized

device that is not configured in your system, DEClma

responds with the error message:

DEClma: Device could not be opened

Example 3-4 shows the output generated by this command.

DEClma Commands 3-5

DEClma Commands

3.5 SELECT device_name

---------------------------------------------------------------------

Example 3-4. Output Generated by the SELECT Command (SCO UNIX)

---------------------------------------------------------------------

DEClma> select dfe0 <Return>

DEClma: Device opened successfully

DEClma> select <Return>

DEClma: Device name missing

Recognized device names are:

dfe0, dfe1, dfe2

DEClma> select xyz0 <Return>

DEClma: Invalid device id; type SELECT for a list of valid device id's

3-6 DEClma Commands

DEClma Commands

3.5 SELECT device_name

3.5.2 SELECT device_name (Novell)

In the Novell environment, you specify a device_name

(Device ID) with a unique integer derived automatically

by the controller slot number, the short name, and the

frame type. To display a list of your valid devices, enter

select <Return>

The list is similar to the following:

---------------------------------------------

Example 3-5 Novell device_names (Device ID)

---------------------------------------------

Device Id   Slot   Short Name   Frame Type

1          3       DEFEA     FDDI_SNAP

2          4       DEFEA     FDDI_802.2

3          4       DEFEA     FDDI_SNAP

If you enter an invalid Device ID for a device_name, DEClma

responds with the error message:

DEClma: Invalid device id; type SELECT for a list of valid

device id's

Example 3-6 shows the output generated by this command.

DEClma Commands 3-7

DEClma Commands

3.5 SELECT device_name

----------------------------------------------------------------

Example 3-6 Output Generated by the SELECT Command (Novell)

----------------------------------------------------------------

DEClma> select 1 <Return>

DEClma: Device opened successfully

DEClma> select <Return>

DEClma: Device name missing

Available boards:

Device Id   Slot   Short Name   Frame Type

1          3       DEFEA     FDDI_SNAP

2          4       DEFEA     FDDI_802.2

3          4       DEFEA     FDDI_SNAP

DEClma> select 9 <Return>

DEClma: Invalid device id; type SELECT for a list of valid device id's

3-8 DEClma Commands

DEClma Commands

3.5 SELECT device_name

3.5.3 SELECT device_name (DOS and OS/2)

There is one recognized device name, DEFEA$.  If you

select the device and it is not configured in your system,

DEClma will respond with the error message:

DEClma: Device could not be opened

Example 3-7 shows the output generated by this command.

------------------------------------------------------------------

Example 3-7 Output Generated by the SELECT Command (DOS and OS/2)

------------------------------------------------------------------

DEClma> select DEFEA$ <Return>

DEClma: Device opened successfully

DEClma> select <Return>

DEClma: Device name missing

Recognized device names are:

DEFEA$ - DEC FDDIcontroller/EISA

DEClma> select abc <Return>

DEClma: Invalid device id; type SELECT for a list of valid device id's

DEClma Commands 3-9

DEClma Commands

3.6 SET object_name [object_value]

3.6 SET object_name [object_value]

This command allows you to set configurable objects within

the INTERFACE, SMT, MAC, PORT, ATTACHMENT, and FDX groups

for the selected device. You specify the object name to be

set and an appropriate object value. In addition, DEClma

allows you to use the predefined default value for a

particular object or to set all objects to their default

values. The specification of the command and all arguments

must be correct or the command will be unsuccessful. DEClma

will neither prompt you for the object value, nor reprompt

you if the value is out of range or invalid. DEClma displays

a message if the command was successful or an appropriate

error message if the command fails. Table 3-1 lists the

configurable SNMP objects with their default values. This

table is also available on line. From within DEClma, enter

HELP SET <Return> to display the table. Additionally,

Example 3-8 shows some typical SET commands.

object_name

This required argument specifies the SNMP object name to

be set. DEClma verifies that the object name is valid,

unique, and is configurable as listed in Table 3-1, column

1 (Object Name). You can specify the special object_name

ALL to set all configurable objects to their default

values. If you specify ALL, then an object_value is not

entered (see Example 3-8). DEClma returns and displays an

appropriate error message if the object name is missing,

unknown, ambiguous, or not a configurable SNMP object.

object_value

This argument specifies a new value for the object. DEClma

verifies that the object_value is valid, unique, and within

the range specified in the MIBs. As stated in the previous

paragraph, the object_value is omitted when the SET ALL

command is entered. DEClma displays an appropriate error

message if the object value is missing, unknown, ambiguous,

extraneous, or out of the specified range.

3-10 DEClma Commands

DEClma Commands

3.6 SET object_name [object_value]

Value Type

This is the type of value entered at command line.

String

One argument value string (such as true, false)

Integer

One argument integer value.

Real

One argument real number specified in milliseconds.

Real sets SNMP objects that are in terms of fdditime

(80-ns units).

Default Value

The default value for the SNMP object as specified in the

SNMP FDDI MIB, MIB-II, and DEC MIB extensions.

DEClma Commands 3-11

DEClma Commands

3.6 SET object_name [object_value]

---------------------------------------------------

Table_3-1 Configurable SNMP Objects

---------------------------------------------------

Object Name            Value Type    Default Value

-----------            ----------    -------------

TReq                   Real          8.00000 ms

RingPurgerEnable       String        false

RestrictedTokenTimeout Real          1000.00000 ms

FDXEnable              String        false

MACLoopTime            Real          200.00000 ms

LerCutoff              Integer       8

LerAlarm               Integer       8

-----------------------------------------------------------

Example 3-8 Using the SET Command

-----------------------------------------------------------

DEClma> set  treq 10.0 <Return>

DEClma: Set object command successful

DEClma> set maclooptime default <Return>

DEClma: Set object command successful

DEClma> set leralarm <Return>

DEClma: Value is missing

Default: 8

Range:   4 <= LerAlarm <= 15

DEClma> set all <Return>

DEClma: Set object command successful

DEClma> set maclooptime 100.0 <Return>

DEClma: Value is out of range

Default: 200.00000 ms

Range:   200.00000 ms <= MACLoopTime <= 10000.00000 ms

3-12 DEClma Commands

DEClma Commands

3.7 SHOW ATTACHMENT

3.7 SHOW ATTACHMENT

This command displays the ATTACHMENT Group objects

contained within the SNMP FDDI MIB for the selected device.

Example 3-9 shows the output generated by this command.

-----------------------------------------------------------

Example 3-9 Output Generated by the SHOW ATTACHMENT Command

-----------------------------------------------------------

DEClma> show attachment

Class                = single-attachment

OpticalBypassPresent = false

IMaxExpiration       = 40.00000 ms

InsertedStatus       = false

InsertPolicy         = true

DEClma>

DEClma Commands 3-13

DEClma Commands

3.8 SHOW COUNTERS

3.8 SHOW COUNTERS

This command displays the counter objects contained in the

INTERFACE, SMT, MAC, PORT, and ATTACHMENT groups for the

selected device. Example 3-10 shows the output generated by

this command.

----------------------------------------------------------

Example 3-10 Output Generated by the SHOW COUNTERS Command

----------------------------------------------------------

DEClma> show counters

InOctets            = 762,033,187    InUcastPkts       = 7,777,100

InNUcastPkts        = 460,737        InDiscards        = 0

InErrors            = 0              InUnknownProtos   = 1,178,525

OutOctets           = 11,171,477     OutUcastPkts      = 7,423,452

OutNUcastPkts       = 7,488,784      OutDiscards       = 248

OutErrors           = 2              BadFramesReceived = 0

ReceiveOverrun      = 0              OversizeFrames    = 0

TransmitFramesError = 2              TracesReceived    = 0

FrameCts = 2,833,001,394  RingInitializationsInitiated = 2

ErrorCts        = 0       RingInitializationsReceived  = 618

LostCts         = 8       RingBeaconingInitiated       = 2

RingPurgeErrors = 0       DuplicateAddressTestFailures = 0

FCIStripErrors  = 0       DuplicateTokensDetected      = 0

TracesInitiated      = 0            FrameStatusErrors  = 0

FrameAlignmentErrors = 0            TransmitUnderruns  = 0

LCTFailCts    = 0               LemRejectCts           = 0

LemCts        = 0               ConnectionsCompleted   = 3

TNEExpRejects = 0               ElasticityBufferErrors = 0

DEClma>

3-14 DEClma Commands

DEClma Commands

3.9 SHOW ERRORLOG

3.9 SHOW ERRORLOG

This command formats and displays the error log located

within the controller's EEPROM, and optionally writes the

log to a user-specified file.  This information can be used

by Digital service personnel to interpret controller errors.

Example 3-11 shows the output generated by this command.

----------------------------------------------------------------

Example 3-11 Output Generated by the SHOW ERRORLOG Command

----------------------------------------------------------------

DEClma> show errorlog

Enter the error log filename  (Press RETURN if none) :

Error Log Event Header -

Event Status: Valid

Caller Id:    Firmware

Firmware Event Information -

Timestamp:    42,085 seconds

Write Count:  1

Reserved Data:

00000000 00000001 00000002 00000003 00000004 00000005 00000006 00000007

00000008 00000009 0000000A 0000000B 0000000C 0000000D 0000000E 0000000F

00000010 00000011 00000012 00000013 00000014 00000015 00000016 00000017

00000018 00000019 0000001A 0000001B 0000001C 0000001D 0000001E 0000001F

00000020 00000021 00000022 00000023 00000024 00000025 00000026 00000027

00000028 00000029 0000002A 0000002B 0000002C 0000002D 0000002E 0000002F

00000030 00000031 00000032 00000033 00000034 00000035 00000036 00000037

00000038 00000039 0000003A 0000003B 0000003C 0000003D 0000003E 0000003F

00000040 00000041 00000042 00000043 00000044 00000045 00000046 00000047

00000048 00000049 0000004A 0000004B 0000004C 0000004D 0000004E 0000004F

00000050 00000051 00000052 00000053 00000054 00000055 00000056 00000057

00000058 00000059 0000005A 0000005B 0000005C 0000005D 0000005E 0000005F

00000060 00000061 00000062 00000063 00000064 00000065 00000066 00000067

00000068 00000069 0000006A 0000006B 0000006C 0000006D 0000006E

<PRESS RETURN TO CONTINUE OR "q" TO STOP DISPLAY>

(continued on next page)

DEClma Commands 3-15

DEClma Commands

3.9 SHOW ERRORLOG

------------------------------------------------------------------

Example 3-11 (Cont.) Output Generated by the SHOW ERRORLOG Command

------------------------------------------------------------------

Error Log Event Header -

Event Status: Valid

Caller Id:    Self-test Diagnostics

Diagnostics Event Information -

Timestamp:    56,091 seconds

Write Count:  1

FRU Imp Mask: 0

Test Id:      6

Reserved Data:

00000000 00000001 00000002 00000003 00000004 00000005

DEClma>

3-16 DEClma Commands

DEClma Commands

3.10 SHOW EVENTS

3.10 SHOW EVENTS

This command displays the latest network events detected

by the controller (up to 8). Example 3-12 shows the output

generated by this command.

--------------------------------------------------------

Example 3-12 Output Generated by the SHOW EVENTS Command

--------------------------------------------------------

DEClma> show events

Link Entity

Ring Initialization Initiated Event

Argument: Reason

RIReason: TRT Expired

Link Entity

Ring Initialization Received Event

DEClma>

DEClma Commands 3-17

DEClma Commands

3.11 SHOW FDX

3.11 SHOW FDX

This command displays the Full-Duplex (FDX) Group objects

contained within the DEC Vendor MIB for the selected

device. Example 3-13 shows the output generated by this

command.

-----------------------------------------------------

Example 3-13 Output Generated by the SHOW FDX Command

-----------------------------------------------------

DEClma> show fdx

FDXEnable    = false

FDXOp        = false

FDXState     = fdxIdle

DEClma>

3-18 DEClma Commands

DEClma Commands

3.12 SHOW INTERFACE

3.12 SHOW INTERFACE

This command displays the INTERFACE (if) Group objects

contained within the MIB-II and the DEC Vendor MIBs for the

selected device. Example 3-14 shows the output generated by

this command.

-------------------------------------------------------------

Example 3-14 Output Generated by the SHOW INTERFACE Command

-------------------------------------------------------------

DEClma> show interface

Descr           = DEC FDDIcontroller/EISA V1.0

Module Rev: 0       Firmware Rev: 1.0

Type            = fddi                Mtu         = 4,478 bytes

Speed           = 100,000,000 bits/S  PhysAddress = 08-00-2B-AA-BB-CC

AdminStatus     = up                  OperStatus    = up

LastChange      = 90,061 seconds      InOctets      = 762,898,462

InUcastPkts     = 7,785,494           InNUcastPkts  = 461,294

InDiscards      = 0                   InErrors      = 0

InUnknownProtos = 1,179,980           OutOctets     = 11,223,811

OutUcastPkts    = 7,431,397           OutNUcastPkts = 7,496,252

OutDiscards     = 248                 OutErrors     = 2

OutQLen         = 256 packets

Specific        = fddi

BadFramesReceived = 0                 ReceiveOverrun      = 0

OversizeFrames    = 0                 TransmitFramesError = 2

DEClma>

DEClma Commands 3-19

DEClma Commands

3.13 SHOW MAC

3.13 SHOW MAC

This command displays the MAC Group objects contained

within the SNMP FDDI and DEC Vendor MIBs for the selected

device. Example 3-15 shows the output of two screens

generated by this command. Press <Return> to display the

second screen.

-----------------------------------------------------------

Example 3-15 Output Generated by the SHOW MAC Command

-----------------------------------------------------------

DEClma> show mac

FrameStatusCapabilities   = FSC-Type0

TMaxGreatestLowerBound    = 165.00000 ms

TVXGreatestLowerBound     = 2.50000 ms

PathsAvailable     = Primary

CurrentPath        = primary         UpstreamNbr = 08-00-2B-AA-BB-DD

OldUpstreamNb      = 08-00-2B-AA-BB-EE  DupAddrTest = pass

PathsRequested     = Primary

DownstreamPORTType = s               SMTAddress  = 08-00-2B-AA-BB-CC

TReq  = 7.98720 ms                     TNeg     = 7.98720 ms

TMax  = 167.77216 ms                   TvxValue = 2.62144 ms

TMin  = 4.00000 ms

CurrentFrameStatus = FSC-Type0         FrameCts        = 2,975,621,201

ErrorCts           = 0                 LostCts         = 8

FrameErrorThreshold= 0                 FrameErrorRatio = 0

RMTState           = Ring_Op           DaFlag          = false

UnaDaFlag          = false             FrameCondition  = false

ChipSet            = 0                 Action          = other

<PRESS RETURN TO CONTINUE OR "q" TO STOP DISPLAY>

LinkState        = onRingRun             RingPurgerState = purgerOff

RingPurgerEnable = false                 RingPurgeErrors = 0

FrameStripMode   = bridgeStrip           FCIStripErrors  = 0

(continued on next page)

3-20 DEClma Commands

DEClma Commands

3.13 SHOW MAC

-------------------------------------------------------------

Example 3-15 (Cont.) Output Generated by the SHOW MAC Command

-------------------------------------------------------------

RingErrorReason                = ringInitInitiated

RingInitializationsInitiated   = 2

RingInitializationsReceived    = 618

RingBeaconingInitiated         = 2

DuplicateAddressTestFailures   = 0

DuplicateTokensDetected        = 0

UpstreamNbrDuplAddressFlag     = false

TracesInitiated                = 0

RestrictedTokenTimeout         = 1000.00000 ms

FrameStatusErrors              = 0

FrameAlignmentErrors           = 0

TransmitUnderruns              = 0

DEClma>

DEClma Commands 3-21

DEClma Commands

3.14 SHOW PORT

3.14 SHOW PORT

This command displays the PORT Group objects contained

within the SNMP FDDI and DEC Vendor MIBs for the selected

device. Example 3-16 shows the output generated by this

command.

--------------------------------------------------------

Example 3-16 Output Generated by the SHOW PORT Command

--------------------------------------------------------

DEClma> show port

PCType             = s

PCNeighbor         = m

ConnectionPolicies = none

RemoteMACIndicated = false

CEState            = Insert_P

PathsRequested     = Primary

MACPlacement       = 0

AvailablePaths     = Primary

MACLoopTime   = 200.00000 ms   TBMax        = 50.00000 ms

BSFlag        = false          LCTFailCts   = 0

LerEstimate   = 15             LemRejectCts = 0

LemCts        = 0              LerCutoff    = 8

LerAlarm      = 8              ConnectState = active

PCMState      = Active         PCWithhold   = none

LerCondition  = false          ChipSet      = 0

Action        = other

PMDType       = pmdMM          PHYState               = inuse

RejectReason  = noReason       ConnectionsCompleted   = 3

TNEExpRejects = 0              ElasticityBufferErrors = 0

DEClma>

3-22 DEClma Commands

DEClma Commands

3.15 SHOW SMT

3.15 SHOW SMT

This command displays the SMT Group objects contained

within the SNMP FDDI and Digital Vendor MIBs for the

selected device. Example 3-17 shows the output generated

by this command.

--------------------------------------------------------

Example 3-17 Output Generated by the SHOW SMT Command

--------------------------------------------------------

DEClma> show smt

StationId     = 00-00-08-00-2B-AA-BB-CC   OpVersionId = 2

HiVersionId   = 2                         LoVersionId = 2

MACCt         = 1 MAC                     NonMasterCt = 1 port

MasterCt      = 0 Mports

PathsAvailable     = Primary

ConfigCapabilities = none

ConfigPolicy       = none

ConnectionPolicy   = Reject A-A, B-B, M-M

TNotify       = 10 seconds       StatusReporting      = true

ECMState      = In               CFState              = Wrap_S

HoldState     = not-implemented  RemoteDisconnectFlag = false

StationAction = other

StationType   = sas              TracesReceived = 0

DEClma>

3.16 SHOW VERSION

This command displays the version of DEClma as follows:

Digital Local Management Application (DEClma) for SCO UNIX systems

Copyright, 1992 - Digital Equipment Corporation, Littleton, Mass.

Version 1.0 - December 1992

3.17 QUIT

This command, identical to the EXIT command, closes

the communication channel for the selected device and

terminates the application.

DEClma Commands 3-23

-----------------------------------------------------------------

4

_________________________________________________________________

Error Messages

This chapter alphabetically lists the error messages

produced by DEClma. Each message has an explanation and

a user action.

Messages

--------

- DEClma: Ambiguous command; type HELP for a list of valid

commands

Explanation: The first argument of a command corresponds

to more than one valid command.

User Action: Reenter the command with more of the valid

command letters to make the command unique.

- DEClma: Ambiguous object; type HELP SET for a list of valid

objects

Explanation: The object name argument specified within

a SET command corresponds to more than one valid object

name.

User Action: Reenter the command with more of the valid

command letters to make the command unique.

- DEClma: Cannot create errorlog file

Explanation: DEClma could not open the file with the

file name specified.

User Action: Enter a valid file name.

Error Messages 4-1

Error Messages

- DEClma: Command failed; could not allocate dynamic memory

Explanation: DEClma attempted to allocate dynamic

memory, but was unsuccessful. This event normally

occurs when DEClma runs under DOS with insufficient

conventional memory.

User Action: Before running DEClma, make as much of the

conventional memory resources available as possible.

- DEClma: Command failed; internal error

Explanation: If DEClma discovers an internal error

while processing a valid command, a software error in

DEClma, the device driver, the controller firmware, or

the system resources is likely.

User Action: If the condition persists, contact your

Digital sales representative.

- DEClma: Device could not be opened

Explanation: A device name is recognized, but is not

configured in your system.

User Action: Verify that the device is configured

properly in your system and reenter the command.

- DEClma: Device is already open

Explanation: A SELECT command was entered for a device

that was already open.

User Action: None.

- DEClma: Device name missing

Explanation: A device_name must be specified in a SELECT

command.

User Action: Reenter the SELECT command with the

appropriate device name.

4-2 Error Messages

Error Messages

- DEClma: Illegal object; type HELP SET for a list of valid

objects

Explanation: An unknown object name was specified as an

argument in a SET command.

User Action: Reenter a valid SET command.

- DEClma: Invalid device id; type SELECT for a list of valid

device id's

Explanation: An unrecognized device id was entered.

User Action: Reenter the SELECT command with a valid

device id.

- DEClma: No open device; type "SELECT device_name" to open a

device

Explanation: A SHOW, CLEAR, or SET command was issued

before you selected a device.

User Action: Enter a SELECT command with a valid device

specification argument and then reenter the desired

SHOW, CLEAR, or SET command.

- DEClma: Object name missing; type HELP SET for a list of

valid objects

Explanation: A SET command was entered without an object

name argument.

User Action: Reenter a valid SET command.

Error Messages 4-3

Error Messages

- DEClma: Object not settable; type HELP SET for a list of

valid objects

Explanation: A read only SNMP object_name was specified

in a SET command.

User Action: Reenter the SET command with a valid

object_name.

- DEClma: This object has no default value

Explanation: A SET object_name DEFAULT command was

entered and the object does not have a default value.

User Action: Reenter the SET command with appropriate

argument(s).

- DEClma: Too many parameters on command line

Explanation: An extraneous argument was entered for an

otherwise valid command.

User Action: Reenter the command with the appropriate

argument(s).

- DEClma: Unrecognized command; type HELP for a list of valid

commands

Explanation: The first argument in the command was

unrecognized by the parser.

User Action: Reenter a valid command.

- DEClma: Value is ambiguous

Explanation: The object value argument specified in a

SET command corresponds to more than one valid object

value string.

User Action: Reenter the SET command with more of the

value object value string to make the string unique.

4-4 Error Messages

Error Messages

- DEClma: Value is illegal

Explanation: An unknown object value string was entered

within the SET command.

User Action: Reenter the SET command with a valid object

value string.

- DEClma: Value is missing

Explanation: A SET command was entered without an object

value argument.

User Action: Reenter the SET command with a valid object

value argument.

- DEClma: Value is out of range

Explanation: The object value argument is outside the

allowable range of values for the object specified.

User Action: Reenter the command with a valid value.

Error Messages 4-5

_________________________________________________________________

Index

C______________________________   E______________________________

CLEAR COUNTERS command, 3-2       Error messages, 4-1

Command                           EXIT command, 3-2

clear counters, 3-2

exit, 3-2                       H

help, 3-3                       _______________________________

initiating DEClma, 3-1          HELP command, 3-3

interactive, 3-1

quit, 3-23                      I______________________________

select, 3-5                     Installation, 2-1

set, 3-10                       Interactive session, 3-1

show attachment, 3-13

show counters, 3-14             M

show errorlog, 3-15             _______________________________

show events, 3-17               Messages, error, 4-1

show fdx, 3-18                  MIB, v

show interface, 3-19

show mac, 3-20                  O______________________________

show port, 3-22                 Overview, product, 1-1

show smt, 3-23

show version, 3-23

D______________________________   Q______________________________

DEClma                            QUIT command, 3-23

commands, 3-1

Index-1

S______________________________     EVENTS command, 3-17

SELECT command, 3-5                 FDX command, 3-18

SET command, 3-10                   INTERFACE command, 3-19

SHOW                                MAC command, 3-20

ATTACHMENT command, 3-13           PORT command, 3-22

COUNTERS command, 3-14             SMT command, 3-23

ERRORLOG command, 3-15             VERSION command, 3-23

Index-2