Board Name:		Goliath

Series Number:	730

Date:			05/21/96

Goliath DIMM Memory Requirements

The memory modules in Goliath are different from the x36 SIMMs used by all other boards.

This is a new module defined for 8 byte wide common CAS memory.  This is a JEDEC standard and many memory module suppliers support it. Most new servers which require large memory capacity are designed to use DIMMs.

Specificatons are as follows:

| 3.3 volt | The memory chips used on the module must be 3.3 volt parts. |
| --- | --- |
| 168 pin | The module pinout is 168 pin (Simms have 72 pins). |
| x72 | It is a 72 data bit module. |
| ECC | “Raw” single CAS DRAM. There is no ECC (error correction code) logic on the module. ECC is implemented in the motherboard chipset. ECC does not require parity chips. |
| DIMM | Dual in-line memory module |
| Buffered | This uses 3.3 volt buffers (typically 163244 or 16244LVT) to buffer the CAS, address lines and write enables. |
| FPM | Goliath uses fast page mode (FPM). EDO is a superset of FPM. The EDO feature will not be used by the board. |
| 60ns | The speed required. |

DRAM Sizes	Typical sizes for DRAM modules are 1Mx72, 2Mx72, 4Mx72, 8Mx72 and 16Mx72.  A maximum of 8 modules can be used to provide 1 gigabyte total.  It is advised that no less than 128 megabytes be used in a Goliath system.  Do not mix module types or sizes within a bank. The modules should be no more than 1.5” to avoid a conflict with the PCI slots.

| Description | Manufacturer | Part Number |
| --- | --- | --- |
| 2MB x 72 (16 MB Module) | Micron | MT9LD272G-60 (FPM) |
|  | Micron | MT9LD272G-60X ( EDO) |
|  | Advantage | A16/ECCDIMM |
|  | Samsung | KMM372V213AJ6 |
|  | IBM | 11M2730HBE |
| 4MB x 72 (32 MB Module) | Micron | MT18LD472G-60X (EDO) |
|  | Advantage | A32ECCDIMM |
|  | Samsung | KMM372V400AK6 |
|  | Samsung | KMM372F400AK6 (EDO) |
|  | IBM | 11M4730CBD |
|  | NEC | MC-424000LAB72 |

Cont’d

| Description | Manufacturer | Part Number |
| --- | --- | --- |
| 8MB x 72 (64 MB Module) | Micron | MT36LD872G-60 (FPM) |
|  | Micron | MT36LD872G-60X (EDO) |
|  | Advantage | A64/ECCDIMM |
|  | Samsung | KMM372V803J6/AJ6 |
|  | IBM | 11M8730CBD |
| 16MB x 72 (128MB Module) | Advantage | A128ECCDIMM |
|  | IBM | 11M16730CBD |
|  | Samsung | KMM372V1680J6 |
|  | Samsung | KMM372V1680AJ6 |

| Supplier | Phone Number | Fax Number | Contact |
| --- | --- | --- | --- |
| Advantage Memory Corp | 800 245 5299 | 714 453 8158 | Dwayne Abrades |
| Centon | 714 855 9111 | 714 855 1144 | Paul Kindly/ Steve Rick |
| Samsung | 770 447 6154 | 770 447 6714 | Greg Mordecai |
| Netmemory | 714 631 5388 | 714 631 8724 | John Deters |

This information will be updated as we continue testing.  The part numbers for NEC and Hitachi modules may not be complete.  These parts have not been tested by us.  Please read the manufacturer’s specifications completely before using these parts.