Apollo III PCI Pentium

ISA Motherboard

User's Guide

MAN-752

7/28/96

© Copyright 1996 American Megatrends, Inc.

All rights reserved.

American Megatrends, Inc.

6145F Northbelt Parkway

Norcross, GA 30071

This publication contains proprietary information which is protected by copyright. No part of this publication may be reproduced, transcribed, stored in a retrieval system, translated into any language or computer language, or transmitted in any form whatsoever without the prior written consent of the publisher, American Megatrends, Inc.

Limited Warranty

Buyer agrees if this product proves to be defective, that American Megatrends, Inc. is only obligated to replace or refund the purchase price of this product at American Megatrends’ discretion according to the terms and conditions on the motherboard warranty card. American Megatrends shall not be liable in tort or contract for any loss or damage, direct, incidental or consequential. Please see the Warranty Registration Card shipped with this product for full warranty details.

Limitations of Liability

In no event shall American Megatrends be held liable for any loss, expenses, or damages of any kind whatsoever, whether direct, indirect, incidental, or consequential, arising from the design or use of this product or the support materials provided with the product.

Trademarks

Intel and Pentium are registered trademarks of Intel Corporation.

MS-DOS, Microsoft Word, and Microsoft are registered trademarks of Microsoft Corporation.

SMC is a registered trademark of Standard Microsystems Corporation.

IBM, AT, VGA, PS/2, OS/2, and EGA are registered trademarks of International Business Machines Corporation. XT and CGA are trademarks of International Business Machines Corporation.

Fujitsu is a registered trademark of Fujitsu America, Inc.

Motorola is a registered trademark of Motorola Corporation.

Hitachi is a registered trademark of Hitachi America, Ltd.

PNY is a registered trademark of PNY Corporation.

Oki is a registered trademark of Oki America, Inc.

NEC is a registered trademark of NEC Corporation.

Samsung is a registered trademark of Samsung Electronics Corporation.

Siemens is a trademark of Siemens Corporation.

Mitsubishi is a registered trademark of Mitsubishi Electronics of America.

Micron is a registered trademark of Micron Corporation.

SCO, UnixWare, and Unix are registered trademarks of The Santa Cruz Operation, Inc..

Toshiba is a registered trademark of Kabushiki Kaisha Toshiba.

VESA is a trademark of the Video Electronics Standards Association.

All other brand and product names are trademarks or registered trademarks of their respective companies.

Revision History

7/28/96	Initial release..

Table of Contents

1 Hardware Installation

Overview

Apollo III Dimensions

Apollo III Motherboard Layout

Step 1 Unpack the Motherboard

Set Jumpers

Avoid Static Electricity

Step 2 Configure CPU

Step 3 Install Memory

Step 4 Install the Motherboard

Step 5 Attach Cables

Step 6 Onboard I/O

Step 7 Attach Floppy Drive

Step 8 Attach IDE Drive

Step 9 Test and Configure

2 WINBIOS Setup

Section 1 Setup

Standard Setup

Advanced Setup

Chipset Setup

Power Management Setup

PCI/PnP Setup

Peripheral Setup

Section 2 Security

AMIBIOS Password Support

Setting a Password

Changing a Password

Anti-Virus

Section 3 Utility

Section 4 Default

3 Programming the Flash ROM

Bootblock Code Checkpoint Codes

A Specifications

Index

Preface

To the OEM	Thank you for purchasing the high performance American Megatrends Apollo III PCI Pentium ISA motherboard. This product is a state of the art motherboard that includes the famous AMIBIOS. It is assumed that you have also licensed the rights to use the American Megatrends documentation for the American Megatrends Apollo III motherboard

This manual was written for the OEM to assist in the proper installation and operation of this motherboard. This manual describes the specifications and features of the Apollo III motherboard. It explains how to assemble a system based on the Apollo III motherboard and how to use the AMIBIOS that is specifically designed for this motherboard.

This manual is not meant to be read by the computer owner who purchases a computer with this motherboard. It is assumed that you, the computer manufacturer, will use this manual as a sourcebook of information, and that parts of this manual will be included in the computer owner's manual.

Technical Support

If you need help installing, configuring, or running this product, call American Megatrends technical support at 770-246-8645. You can also send questions to tech support at:

support@american.megatrends.com.

American Megatrends BBS The American Megatrends BBS permits you to access technical information about American Megatrends motherboard, peripheral, and BIOS products. Product Engineering Change Notices, Tech Tips, and technical documentation are available on the BBS. Some parts of the BBS are not accessible to all callers. Call American Megatrends Technical Support at 770-246-8645 to find out how to access the BBS.

Phone Numbers	The BBS requires no parity, eight data bits, and one stop bit. The characteristics of the BBS phone numbers are:

| Phone Number | Characteristics |
| --- | --- |
| 770-246-8780 | 28,800 baud rate. Supports v.34. |
| 770-246-8781 | 28,800 baud rate. Supports v.34. |
| 770-246-8782 | Supports HST and v.42. |
| 770-246-8783 | Supports HST and v.42. |

Web Site	We invite you to access the American Megatrends world wide web site at:

http://www.megatrends.com.

Packing List

You should have received the following:

- an Apollo III Pentium PCI ISA motherboard,

- an optional USB cable and mounting bracket,

- two serial cables,

- one parallel cable,

- a Warranty Card, and

- the American Megatrends Apollo III Pentium ISA Motherboard User's Guide.

Static Electricity

The Apollo III motherboard can easily be damaged by static electricity. Make sure you take appropriate precautions against static electric discharge:

- wear a properly-grounded wristband while handling the Apollo III motherboard or any other electrical component,

- touch a grounded anti-static surface or a grounded metal fixture before handling the Apollo III motherboard,

- handle system components by the mounting bracket, if possible.

Batteries	Make sure you dispose of used batteries according to the battery manufacturer’s instructions. Improper use of batteries may cause an explosion. Make sure you follow the battery manufacturer’s instructions about using the battery.

Replace used batteries with the same type of battery or an equivalent recommended by the battery manufacturer.

# 1	Hardware Installation

### Overview

The American Megatrends Apollo III PCI Pentium ISA motherboard includes the following features.

CPU		The motherboard supports an Intel® Pentium™ 75, 90, 100, 120, 133, 150, 166, 180, 200 MHz or higher speed CPU operating at Standard, VR, or VRE voltages. The CPU speed and voltage are set via jumpers.

Upgrade CPU	You can install an Intel P54C series, P54CT, or P55C CPU.

Chipset		The Apollo III PCI Pentium ISA motherboard uses an Intel 82430HX, including CPU interface controller, advanced cache controller, integrated DRAM controller, synchronous ISA bus controller, PCI local bus interface, and integrated power management unit.

Expansion slots	The motherboard includes three 16-bit ISA expansion slots and four 32-bit PCI expansion slots. The PCI local bus throughput can be up to 132 megabytes per second.

L1 Internal Cache The Intel Pentium on the motherboard has an 8 KB data cache and an 8 KB instruction cache.

L2 Secondary Cache The motherboard has either 256 KB or 512 KB of synchronous PBSRAM, direct-mapped, write-back. L2 secondary cache memory.

Cont’d

### Overview, Continued

DRAM Cached 	If 256 KB of L2 secondary cache memory is installed on the motherboard, up to 128 MB of system memory can be cached.

If 512 KB of L2 secondary cache memory is installed on the motherboard, up to 256 MB of system memory can be cached.

System Memory	The motherboard supports up to 256 MB of system memory mounted on the motherboard in two banks of 72-pin 4 MB, 8 MB, or 16 MB SIMMs.

Use Fast Page Mode or Extended Data Out (EDO) SIMMs operating at 60 ns.

Fast ATA	The motherboard supports the Fast ATA specification using PIO mode 4 and multiword DMA mode 2.

System BIOS	This motherboard has a 128 KB AMIBIOS system BIOS on a Flash ROM with built-in WINBIOS Setup.

BIOS Shadowing The system BIOS is always copied from ROM to RAM for faster execution. The end user can shadow 16 KB ROM segments from C0000h – DCFFFh.

Motherboard Size  The Apollo III Pentium PCI ISA motherboard is 10 inches by 8.625 inches.

Cont’d

### Overview, Continued

AMIBIOS Features  AMIBIOS features include:

- IDE block mode support,

- IDE 32-bit data transfer support,

- IDE Programmed I/O mode 0, 1, 2, 3, and 4 support,

- IDE LBA mode support,

- APM (Advanced Power Management) and Flash BIOS hooks,

- EPA Green PC-compliant,

- PCI and Plug and Play (PnP) support, and

- DIM (Device Initialization Manager) support,

- DMI (Desktop Management Interface) support,

- ATAPI support,

- can boot from a CD-ROM drive,

- automatically detects system memory, cache memory, and IDE drive parameters,

- Intel NSP-compliant,

- Fast ATA IDE mode programming,

- Boot sector virus protection,

- instant-on support,

- automatically configures PnP and PCI devices.

IDE		The Apollo III motherboard has two 40-pin IDE connectors onboard that support up to four IDE drives. The IDE controller is on the PCI local bus.

Floppy		The Apollo III motherboard has an onboard floppy controller that supports up to two 360 KB, 720 KB, 1.2 MB, 1.44 MB, or 2.88 MB floppy drives.

Parallel Port	The Apollo III motherboard has an onboard ECP and EPP-capable parallel port connector.

Serial Ports	The Apollo III motherboard has two onboard serial port connectors and two 16550 UART serial ports.

Keyboard	The Apollo III motherboard Includes a standard miniDIN keyboard connector.

Cont’d

### Overview, Continued

Mouse		The Apollo III motherboard includes a 5-pin berg mouse connector.

USB		The Apollo III motherboard has two 4-pin USB connectors. USB allows future generations of USB-compliant peripheral devices to be automatically detected and configured through a single port. The AMIBIOS on the Apollo III motherboard provides complete USB system BIOS support.

Infrared	A 10-pin infrared connector is provided on the motherboard. The motherboard and AMIBIOS comply with the IrDA SIR infrared device standards and specifications.

Power Connectors  The Apollo III motherboard has three power supply connectors.

RTC/CMOS RAM  A real time clock and 128 bytes of CMOS RAM with a battery backup is provided on the motherboard.

Power Management  Power management services include:

- Green PC LED,

- power management signal to Green PC-aware power supplies,

- automatic IDE and video power down,

- monitor blanking,

- SMI (System Management Interrupt) support,

- APM, and

- system stop clock.

Cont’d

### Overview, Continued

PCI Slots	The motherboard conforms to the PCI Version 2.1 specification. The PCI slots are automatically configured by the AMIBIOS. The PCI slots operate synchronously with the CPU clock, as follows:

| CPU External Clock Frequency | PCI Expansion Slot Frequency |
| --- | --- |
| 66 MHz | 33 MHz |
| 60 MHz | 30 MHz |
| 50 MHz | 25 MHz |

Onboard I/O	The Apollo III Pentium ISA motherboard includes:

- two 40-pin IDE connectors on the PCI bus that support up to four IDE drives,

- a 34-pin floppy drive connector,

- two 10-pin serial port connectors (with 16550 UARTs),

- a 26-pin parallel port connector with ECP and EPP support,

- an infrared connectors,

- two USB connectors,

- a keyboard connector, and

- a PS/2 mouse connector.

### Apollo III Dimensions

### Installation Steps

| Step | Action | Turn to |
| --- | --- | --- |
| 1 | Unpack the motherboard. | Page 10 |
| 2 | Configure the CPU. | Page 12 |
| Select the CPU voltage. |  | Page 13 |
| Select the CPU speed. |  | Page 12 |
| Install the CPU. |  | Page 14 |
| 3 | Install memory. | Page 17 |
| Install system memory. |  | Page 17 |
| 4 | Install the motherboard. | Page 22 |
| 5 | Attach cables to connectors. | Page 25 |
| Connect the power supply. |  | Page 27 |
| Attach the keyboard cable. |  | Page 28 |
| Connect the mouse cable. |  | Page 30 |
| Attach cables. |  | Page 32 |
| 6 | Connect onboard I/O. | Page 34 |
|  | Connect the serial ports. | Page 34 |
|  | Connect the parallel port. | Page 36 |
| 7 | Connect floppy drive(s). | Page 37 |
| 8 | Connect the IDE drive(s). | Page 38 |
| 9 | Test and configure. | Page 44 |

Warning This motherboard contains sensitive electronic components that can be easily damaged by static electricity. Follow the instructions carefully to ensure correct installation and to avoid static damage.

### Apollo III Motherboard Layout

### Step 1 Unpack the Motherboard

| Step | Action |
| --- | --- |
| 1 | Inspect the cardboard carton for obvious damage. If damaged, call 770-246-8645. Leave the motherboard in its original packing. |
| 2 | Perform all unpacking and installation procedures on a ground-connected anti-static mat. Wear an anti-static wristband grounded at the same point as the anti-static mat. Or use a sheet of conductive aluminum foil grounded through a 1 megohm resistor instead of the anti-static mat. Similarly, a strip of conductive aluminum foil wrapped around the wrist and grounded through a 1 megohm resistor serves the same purpose as the wristband. |
| 3 | Inside the carton, the motherboard is packed in an anti-static bag, and sandwiched between sheets of sponge. Remove the sponge and the anti-static bag. Place the motherboard on a grounded anti-static surface component side up. Save the original packing material. |
| 4 | Inspect the motherboard for damage. Press down on all ICs mounted in sockets to verify proper seating. Do not apply power to the motherboard if it has been damaged. |
| 5 | If the motherboard is undamaged, it is ready to be installed. |

### Set Jumpers

Set all jumpers and install the CPU before placing the motherboard in the chassis. Set jumpers by placing a shunt (shorting bridge) on the designated pins of the jumper. A shunt and jumpers are shown below:

### Avoid Static Electricity

Static electricity can damage the motherboard and other computer components. Keep the motherboard in the anti-static bag until it is to be installed. Wear an anti-static wrist grounding strap before handling the motherboard. Make sure you stand on an anti-static mat when handling the motherboard.

Avoid contact with any component or connector on any adapter card, printed circuit board, or memory module. Handle these components by the mounting bracket.

### Step 2 Configure CPU

See the drawing on page 9 for the jumper locations.

Important Perform the following steps to configure the motherboard before installing a CPU.

External and Internal CPU Clock CLK1, CLK2, FREQ1, and FREQ2 are 3-pin bergs that set the CPU external and internal clock frequencies. FREQ1 and FREQ2 are next to the CPU. CLK1 and CLK2 are towards the middle of the motherboard. See the drawing on page 9 for the exact jumper locations.

| External | Internal | CLK1 | CLK2 | FREQ1 | FREQ2 |
| --- | --- | --- | --- | --- | --- |
| 66 MHz | 166 MHz | Short Pins 1-2 | Short Pins 2-3 | Short Pins 2-3 | Short Pins 2-3 |
| 60 MHz | 150 MHz | Short Pins 2-3 | Short Pins 1-2 | Short Pins 2-3 | Short Pins 2-3 |
| 66 MHz | 133 MHz | Short Pins 1-2 | Short Pins 2-3 | Short Pins 2-3 | Short Pins 1-2 |
| 60 MHz | 120 MHz | Short Pins 2-3 | Short Pins 1-2 | Short Pins 2-3 | Short Pins 1-2 |
| 66 MHz | 100 MHz | Short Pins 1-2 | Short Pins 2-3 | Short Pins 1-2 | Short Pins 1-2 |
| 60 MHz | 90 MHz | Short Pins 2-3 | Short Pins 1-2 | Short Pins 1-2 | Short Pins 1-2 |
| 50 MHz | 75 MHz | Short Pins 2-3 | Short Pins 2-3 | Short Pins 1-2 | Short Pins 1-2 |

Important Please contact American Megatrends technical support at 770-246-8645 if you need to support a CPU running at a higher speed.

Cont’d

### Step 2 Configure CPU, Continued

JVR Set CPU Voltage Install Intel Pentium CPUs that adhere to either the standard or VRE voltage specifications. VR1 and VR2 are 6-pin bergs that set CPU voltage. VR1 is near the top of the CPU socket. VR2 is next to the CPU, near the edge of the motherboard.

Standard and VR Voltage

VRE Voltage

Mixed Voltage

Cont’d

### Step 2 Configure CPU, Continued

Important If you are not sure about the voltage specification for the CPU that will be installed in this motherboard, please call Intel and make sure that you set the voltage jumpers correctly. Selecting the wrong voltage may damage the CPU.

Install CPU	Install the CPU in the ZIF (zero insertion force) socket by performing the following steps. The CPU socket is near one edge of the motherboard, as shown on page 9.

Warning Improper CPU installation can damage the CPU and the motherboard. You must follow the procedures in this section exactly as documented. Make sure you wear an antistatic wristband while installing the CPU. Follow all antistatic procedures described on page 11.

| Step | Action |
| --- | --- |
| 1 | Lift the lever on the ZIF socket. The empty CPU socket looks like this. 14 |
| 2 | Check for bent pins on the CPU. Gently straighten any bent pins with pliers. Place the CPU in the middle of the socket, as shown below. Make sure that pin 1 of the CPU is aligned with pin 1 of the socket. Make sure you are properly grounded while handling the CPU. |
| 3 | Complete installation by lifting the ZIF lever to the other side of the socket, as shown below. |

### Step 3 Install Memory

System Memory	The motherboard has four 32-bit SIMM – Single Inline Memory Module) sockets. You can use Fast Page Mode or EDO (Extended Data Out) SIMMs. Memory must be populated one bank at a time. Each bank has two sockets. Each bank must be populated with the same type of SIMM. If a 1 MB SIMM is installed in the first socket in Bank0, then the same type of 1 MB SIMM must be installed in the second Bank0 SIMM socket. Each socket can hold one 72-pin SIMM. You can use:

- 1 MB x 36,

- 4 MB x 36,

- 8 MB x 36, or

- 16 MB x 36 SIMMs.

The motherboard supports banks of fast page mode and EDO (Extended Data Out) memory together operating at 60 ns (RAS access time).

Memory Display	System memory is reported by AMIBIOS as it boots and again when the AMIBIOS System Configuration Screen is displayed just before the operating system boots. The memory displayed by AMIBIOS on the System Configuration Screen is 384 KB less than the total memory installed.

SIMM Part Numbers

| Type | Manufacturer | Part Number |
| --- | --- | --- |
| 4 MB x 36 | Micron | MT12D436M-6 |
| “ | Mitsubishi | MH4M36SAJ-6 |
| “ | Motorola | MCM36400S-60 |
| “ | PNY | P364000-60 |
| “ | Samsung | KMM5364100-6 |
| 8 MB x 36 | Motorola | MCM36800S-60 |
| “ | PNY | P368000-60 |
| “ | Samsung | KMM5368100-6 |

Cont’d

### Step 3 Install Memory, Continued

DRAM Configurations Valid memory configurations include:

| Total RAM | Bank0 | Bank0 | Bank1 | Bank1 |
| --- | --- | --- | --- | --- |
| 8 MB | 4 MB | 4 MB | None | None |
| 16 MB | 8 MB | 8 MB | None | None |
| 24 MB | 8 MB | 8 MB | 4 MB | 4 MB |
| 32 MB | 8 MB | 8 MB | 8 MB | 8 MB |
| 40 MB | 16 MB | 16 MB | 4 MB | 4 MB |
| 48 MB | 16 MB | 16 MB | 8 MB | 8 MB |
| 64 MB | 16 MB | 16 MB | 16 MB | 16 MB |
| 64 MB | 32 MB | 32 MB | None | None |
| 72 MB | 32 MB | 32 MB | 4 MB | 4 MB |
| 80 MB | 32 MB | 32 MB | 8 MB | 8 MB |
| 96 MB | 32 MB | 32 MB | 16 MB | 16 MB |
| 128 MB | 32 MB | 32 MB | 32 MB | 32 MB |
| 136 MB | 64 MB | 64 MB | 4 MB | 4 MB |
| 144 MB | 64 MB | 64 MB | 8 MB | 8 MB |
| 160 MB | 64 MB | 64 MB | 16 MB | 16 MB |
| 192 MB | 64 MB | 64 MB | 32 MB | 32 MB |
| 256 MB | 64 MB | 64 MB | 64 MB | 64 MB |

Cont’d

### Step 3 Install Memory, Continued

Installing SIMMs The motherboard has four x 36 SIMM sockets. These sockets can be filled with either 1 MB x 36, 4 MB x 36, 8 MB x 36, or 16 MB x 36 SIMMs.

Place the motherboard on an anti-static mat. With the component side of the SIMM facing you, firmly push the SIMM into the socket at an angle, then push it up. When properly inserted, the SIMM clicks into place as the latching pins engage. The SIMM installation process is shown below:

Cont’d

### Step 3 Install Memory, Continued

Configure Cache Memory The motherboard supports 256 KB or 512 KB of L2 3V Pipeline Burst SRAM secondary cache memory. Either 256 KB or 512 KB of Pipeline Burst Mode Static RAM L2 secondary cache memory is mounted on the motherboard. You cannot upgrade L2 secondary cache memory.

| L2 Cache Size | Cache DIP Type | Tag RAM Type |
| --- | --- | --- |
| 256 KB | 32 KB x 32 (M2 and M4) | 8 KB x 8 (M5) |
| 512 KB | 32 KB x 32 (M1, M2, M3, and M4) | 16 KB x 8 (M5, M6) |

Flash ROM Type You must specify the type of flash EPROM installed on the motherboard, as follows. EP1 and EP2 are next to the BIOS ROM between the ISA and PCI expansion slots.

| Flash ROM Type | EP1 | EP2 |
| --- | --- | --- |
| Intel 28F010 | Short Pins 1-2 | Short Pins 2-3 |
| Intel 28F001BX-T | Short Pins 1-2 | Short Pins 1-2 |
| SST 29EE010 (factory setting) | Short Pins 2-3 | Short Pins 2-3 |

### Step 4 Install the Motherboard

The motherboard mounting hole pattern is the same as the mounting hole pattern on the standard baby AT motherboard. Standoffs and mounting screws are not supplied with the motherboard. The chassis manufacturer should supply these parts.

| Step | Action |
| --- | --- |
| 1 | Place the chassis on an anti-static mat. Connect the chassis to ground to avoid static damage during installation. Connect an alligator clip with a wire lead to any unpainted part of the chassis. Ground the other end of the lead at the same point as the mat and the wristband. |
| 2 | Rotate the chassis so the front is to the right, and the rear is to the left. The side facing you is where the motherboard is mounted. The power supply is mounted at the far end of the chassis. |
| 3 | Hold the motherboard, component-side up, with the edge with the SIMM sockets toward you and the edge with the power supply connector away from you. The keyboard, mouse, and video connectors should be to the left. |
| 4 | Carefully slide the motherboard into the chassis. Make certain the edge connectors fit the ports in the rear of the chassis. The motherboard should rest level with the chassis. |
| 5 | Place the mounting screws in the holes provided and tighten them. If necessary, shift the motherboard slightly to align the mounting holes on the motherboard with the holes on the chassis. See the drawing on the next page. |

Warning If using metallic screws, make sure you use them only in the plated mounting holes. If using metallic screws, make sure the head of the screw fits completely inside the plated mounting holes. See the graphic on the following page.

Cont’d

### Step 4 Install Motherboard, Continued

### Step 5 Attach Cables

Connectors	The Apollo III motherboard includes many connectors. Connection instructions, illustrations of connectors, and pinouts are supplied in the following pages. A list of all connectors described in this section follows:

| Connector | Turn to |
| --- | --- |
| CPS Clear password | page 26 |
| Power supply connector | page 26 |
| Keyboard connector | page 28 |
| Mouse connector | page 30 |
| CPU Fan | page 31 |
| Infrared | page 32 |
| Remote Power connector | page 32 |
| USB connectors | page 32 |
| Speaker | page 33 |
| IDE LED | page 33 |
| Remote Power Switch | page 33 |
| Hardware Reset Switch | page 33 |
| Power LED (lit when power is on) | page 33 |
| Keyboard Lock | page 33 |
| Turbo LED (lit when high speed is active) | page 33 |
| Suspend Mode Switch | page 33 |
| Suspend LED (lit when system in suspend mode) | page 33 |
| Serial Port | page 36 |
| Parallel port | page 36 |
| Floppy drive connector | page 37 |
| IDE drive connectors | pages 40 through 43 |

Cont’d

### Step 5 Attach Cables, Continued

Cable Connector Ends	When connecting chassis connectors to the motherboard, make sure to connect the correct connector end. Most connector wires are color-coded. Match the color of the wires leaving the switch or LED to the same pin on the connector end. There may be more than one connector with the same color-coded wires. If so, follow the wire to the switch or LED. Pin 1 is indicated for all motherboard components by a white line. Pin 1 is always nearest to the white line.

CPS Clear Password If you forget the computer password, the only course of action is to erase the system configuration information stored in the NVRAM (Non-Volatile Random Access Memory). The computer password is stored in the system configuration data. Short the CPS pins together for two seconds. CPS is located near the bottom of PCI Slot4. All system memory will be erased. You must now turn the computer power on and run WINBIOS Setup to reset all system configuration information. Make sure that CPS remains OPEN unless you have to erase a forgotten password.

Cont’d

### Step 5 Attach Cables, Continued

Connect Power Supply  The power supply should match the physical configuration of the chassis. Make sure that the power switch is Off before assembly.

Before attaching all components, make sure that the proper voltage has been selected. Power supplies often can run on a wide range of voltages and must be set (usually via a switch) to the proper range. Use at least a 200 watt power supply, which should have built-in filters to suppress radiated emissions.

Power Cables	Attach the power supply cables to the power connector on the motherboard. AT-compatible power supplies have one twelve pin connector, as shown below.

Cont’d

### Step 5 Attach Cables, Continued

Connector Keys	The keys on the connector must be cut to fit on some power supplies, as shown below.

Power Connector Pinout

| Pin | Description |
| --- | --- |
| 1 | Power Good (Orange wire) (Not used) |
| 2 | VCC (Red wire) |
| 3 | +12 Volts (Yellow wire) |
| 4 | -12 Volts (Blue wire) |
| 5 | Ground (Black wire) |
| 6 | Ground (Black wire) |
| 7 | Ground (Black wire) |
| 8 | Ground (Black wire) |
| 9 | -5 Volts (White wire) |
| 10 | VCC (Red wire) |
| 11 | VCC (Red wire) |
| 12 | VCC (Red wire) |

Keyboard Cable	The keyboard attaches via a PS/2 keyboard connector, labeled KB_CONN.

| Pin | Assignments |
| --- | --- |
| 1 | Keyboard data |
| 2 | N/C |
| 3 | Ground |
| 4 | VCC |
| 5 | Keyboard clock |
| 6 | N/C |

Cont’d

### Step 5 Attach Cables, Continued

Mouse Cable	Attach the mouse connector cable supplied by American Megatrends to the five-pin mouse berg connector on the motherboard (labeled MS_COM), as shown below. Attach the standard 9-pin mouse connector at the other end of the mouse cable to the mouse connector port on the computer case. Incorrect mouse installation can cause the system to hang.

| Pin | Description |
| --- | --- |
| 1 | Mouse Clock |
| 2 | N/C |
| 3 | N/C |
| 4 | Mouse Data |
| 5 | Ground |

Cont’d

### Step 5 Attach Cables, Continued

When connecting chassis connectors to the motherboard, make sure to connect the correct connector end. Most connector wires are color-coded. Match the color of the wires leaving the switch or LED to the same pin on the connector end. There may be more than one connector with the same color-coded wires. If so, follow the wire to the switch or LED. Pin 1 is always indicated on the motherboard, as shown below:

CPU Fan	A three-pin berg labeled FAN attaches to the CPU fan. The FAN connector is next to the CPU socket.

| Pin | Description |
| --- | --- |
| 1 | Ground |
| 2 | +12V |
| 3 | Ground |

Cont’d

### Step 5 Attach Cables, Continued

IR Infrared	The 8-pin infrared connector (IR1) near PCI Slot 1 connects to an infrared port mounted on the computer chassis. It allows data transmission to any other device that supports the IrDA standards for infrared transmission.

Remote Control Power RPW_COM is a 2-pin berg next to the infrared connector that connects to the power supply for enabling system power when the remote power switch is turned on. Connect a 2-wire power cable to RPW_COM and attach the other end to the peripheral device.

USB Connectors  The Apollo III motherboard has two 4-pin headers (USB1 and USB2) that attach to a USB connector on the computer chassis. The USB port allows you to attach to a USB hub. The USB connector pinouts are the same for both USB connectors:

| Pin | Signal Description |
| --- | --- |
| 1 | VCC (Fused 5V) |
| 2 | - Data |
| 3 | + Data |
| 4 | Ground |

Cont’d

### Step 5 Attach Cables, Continued

Block Connector  The Apollo III motherboard has a 22-pin header that is used to connect the following offboard connectors. The header is on the corner of the motherboard near the CPU socket.

| Connector | Signal Descriptions |
| --- | --- |
| Speaker | Pin 12 VCC Pin 15 Speaker Data |
| IDE LED | Pin 16 LED Power Pin 17 IDE Active |
| Remote Power Switch (Power when low). | Pin 18 Remote Power Pin 19 Ground |
| Hardware Reset Switch | Pin 20 Hard Reset Pin 21 Ground |
| Power LED (lit when power is on) | Pin 1 +Power Pin 2 Ground |
| Keyboard Lock | Pin 4 Keyboard Lock Pin 5 Ground |
| Turbo LED (lit when low speed is active) | Pin 6 TURBO_LIT Pin 7 TURBO_LEDPWR |
| Suspend Mode Switch (Suspend when Low) | Pin 8 Suspend In Switch Pin 9 Ground |
| Suspend LED (lit when system in suspend mode) | Pin 10 Suspend Out LED+ Pin 11 Suspend Out LED- |

### Step 6 Onboard I/O

Onboard I/O	The Apollo III motherboard has:

- two serial ports (COM1 and COM2),

- a parallel port (LPT),

- an IDE controller on the PCI bus. The primary IDE connector is IDE1. The secondary connector is IDE2.

- a floppy controller (FDD).

The serial and parallel port connectors are described below. The floppy connector is described on page 37. The IDE connector is described on page 38.

Conflicts	AMIBIOS minimizes conflicts between onboard and offboard I/O devices.

AMIBIOS automatically checks the adapter cards installed in the expansion slots on the Apollo III motherboard for a hard disk or floppy controller and serial or parallel ports.

Serial Ports 	COM1 and COM2 are 10-pin connectors that provide an AT-compatible serial port interface. Connect the cables supplied with the motherboard to the 10-pin serial connectors. The serial port base I/O port address and other serial port settings can be selected in Peripheral Setup in WINBIOS Setup. The serial connector pinout is shown below.

| Pin | Description | Pin | Signal Description |
| --- | --- | --- | --- |
| 1 | Carrier Detect | 6 | Data Set Ready |
| 2 | Receive Data | 7 | Request to Send |
| 3 | Transmit Data | 8 | Clear to Send |
| 4 | Data Terminal Ready | 9 | Ring Indicator |
| 5 | Ground | 10 | CUT PIN |

Cont’d

### Step 6 Onboard I/O, Continued

Parallel Port	LPT is a 26-pin connector for a parallel port. The LPT pinout is shown below. Connect the 26-pin to DB25 cable provided with the motherboard to LPT. All parallel port settings can be configured through Peripheral Setup in WINBIOS Setup.

| Pin | Signal Description | Pin | Signal Description |
| --- | --- | --- | --- |
| 1 | STROBE# | 2 | PD0 |
| 3 | PD1 | 4 | PD2 |
| 5 | PD3 | 6 | PD4 |
| 7 | PD5 | 8 | PD6 |
| 9 | PD7 | 10 | ACK# |
| 11 | BUSY | 12 | PE |
| 13 | SLCT | 14 | AUTOFD# |
| 15 | ERROR# | 16 | INIT# |
| 17 | SLCTIN# | 18 | Ground |
| 19 | Ground | 20 | Ground |
| 21 | Ground | 22 | Ground |
| 23 | Ground | 24 | Ground |
| 25 | Ground | 26 | Ground |

### Step 7 Attach Floppy Drive

FDD is a 34-pin dual-inline berg. Connect the cable from the floppy drive to FDD, as shown below. The onboard floppy controller cannot be used if a hard disk card with a floppy controller is installed. Choose Standard Setup and Peripheral Setup to configure the floppy controller.

The motherboard supports up to two 720 KB, 1.44 MB, or 2.88 MB 3½" drives and 360 KB and 1.2 MB 5¼" drives. The connecting cable is a 34-pin ribbon connector with two 34-pin edge connectors for attaching the floppy disk drives. There is a small twist in the cable between the floppy connectors. The last (end) connector should be connected to floppy drive A: as shown below.

Cont’d

### Step 7 Attach Floppy Drive, Continued

Floppy Connector Pinout

| Pin | Use | Pin | Use |
| --- | --- | --- | --- |
| 1 | GND | 2 | DENSE1 |
| 3 | GND | 4 | N/C |
| 5 | GND | 6 | DRATE0 |
| 7 | GND | 8 | -INDEX |
| 9 | GND | 10 | -MOTOR0 |
| 11 | GND | 12 | -FDSEL1 |
| 13 | GND | 14 | -FDSEL0 |
| 15 | GND | 16 | -MOTOR1 |
| 17 | GND | 18 | DIR |
| 19 | GND | 20 | - |
| 21 | GND | 22 | -WDATA |
| 23 | GND | 24 | -WGATE |
| 25 | GND | 26 | -TRK0 |
| 27 | GND | 28 | -WRPROT |
| 29 | GND | 30 | -RDATA |
| 31 | GND | 32 | HDSEL |
| 33 | GND | 34 | DSKCHNG |

Twist in Floppy Cable

| Floppy B to A | Floppy B to A | Floppy B to A | Floppy B to A |
| --- | --- | --- | --- |
| 10 to 16 | 12 to 14 | 14 to 12 | 16 to 10 |
| 11 to 15 | 13 to 13 | 15 to 11 |  |

### Step 8 Attach IDE Drive

IDE Drives	Attach the IDE drives in the following manner. Choose Peripheral Setup in WINBIOS Setup to enable the onboard IDE controller.

Cont’d

### Step 8 Attach IDE Drive, Continued

Attach IDE Cable IDE1 is the primary IDE (Integrated Drive Electronics) hard disk drive connector. Both the primary master and the primary slave IDE drives must be connected by cable to IDE1, as shown below.

IDE1 is a 40-pin dual-inline berg that connects an IDE drive to the primary onboard IDE connector. This motherboard supports IDE Modes 0, 1, 2, 3, and 4, IDE prefetch, LBA (Logical Block Address) mode, high capacity drives (over 528 MB), 32-bit data transfer, and fast IDE transfer. These IDE features are configured in Peripheral Setup in the WINBIOS Setup utility. Disable the onboard IDE interface in Peripheral Setup to use an ISA ESDI, RLL, MFM, or SCSI hard disk drive controller.

Install IDE Drivers American Megatrends provides IDE drivers for the onboard IDE controller. These drivers are unnecessary if running in DOS, but must be loaded for other operating systems. These drivers have not been tested with Novell NetWare or SCO Unix.

| Operating System | Description |
| --- | --- |
| Windows 3.11 and Windows for Workgroups | Load the PIO drivers for 32-bit access. |
| Windows 95 | Always load the IDE drivers. |
| Windows NT v3.x | Always load the IDE drivers. |
| OS/2 | Always load the IDE drivers. |

Cont’d

### Step 8 Attach IDE Drive, Continued

IDE1 Pinout	The IDE1 pinout is:

| Pin | Use | Pin | Use |
| --- | --- | --- | --- |
| 1 | -RESET | 2 | GND |
| 3 | DATA7 | 4 | DATA8 |
| 5 | DATA6 | 6 | DATA9 |
| 7 | DATA5 | 8 | DATA10 |
| 9 | DATA4 | 10 | DATA11 |
| 11 | DATA3 | 12 | DATA12 |
| 13 | DATA2 | 14 | DATA13 |
| 15 | DATA1 | 16 | DATA14 |
| 17 | DATA0 | 18 | DATA15 |
| 19 | GND | 20 | KEY (N/C) |
| 21 | N/C | 22 | GND |
| 23 | -IOW | 24 | GND |
| 25 | -IOR | 26 | GND |
| 27 | IDERDY | 28 | ALE |
| 29 | N/C | 30 | GND |
| 31 | INT14 | 32 | -IOCS16 |
| 33 | HA1 | 34 | N/C |
| 35 | HA0 | 36 | HA2 |
| 37 | -CS0 | 38 | -CS1 |
| 39 | -IDEACT | 40 | GND |

IDE2 Secondary IDE Controller	IDE2, the secondary IDE connector, is a 40-pin dual-inline berg that connects the secondary primary and slave IDE drives to the secondary onboard IDE controller.

Attach the secondary master and slave IDE drives to IDE2 via a standard 40-pin IDE cable as shown on page 40.

Cont’d

### Step 8 Attach IDE Drive, Continued

IDE2 Pinout	The IDE2 pinout is:

| Pin | Use | Pin | Use |
| --- | --- | --- | --- |
| 1 | -RESET | 2 | GND |
| 3 | DATA7 | 4 | DATA8 |
| 5 | DATA6 | 6 | DATA9 |
| 7 | DATA5 | 8 | DATA10 |
| 9 | DATA4 | 10 | DATA11 |
| 11 | DATA3 | 12 | DATA12 |
| 13 | DATA2 | 14 | DATA13 |
| 15 | DATA1 | 16 | DATA14 |
| 17 | DATA0 | 18 | DATA15 |
| 19 | GND | 20 | KEY (N/C) |
| 21 | N/C | 22 | GND |
| 23 | -IOW | 24 | GND |
| 25 | -IOR | 26 | GND |
| 27 | IDERDY | 28 | ALE |
| 29 | N/C | 30 | GND |
| 31 | INT15 | 32 | -IOCS16 |
| 33 | HA1 | 34 | N/C |
| 35 | HA0 | 36 | HA2 |
| 37 | -CS2 | 38 | -CS3 |
| 39 | N/C | 40 | GND |

### Step 9 Test and Configure

Review the following points before powering up:

- make sure that all adapter cards are seated properly,

- make sure all connectors are properly installed,

- make sure the CPU is seated properly,

- make sure there are no screws or other foreign material on the motherboard,

- plug the system into a surge-protected power strip, and

- make sure blank back panels are installed on the back of the chassis to minimize RF emissions.

Start the Test	Plug everything in and turn on the switch. If there are any signs of a problem, turn off the unit immediately. Reinstall the connectors. Call Technical Support if there are problems.

BIOS Errors	If the system operates normally, a display should appear on the monitor. The BIOS Power On Self Test (POST) should execute.

If POST does not run successfully, it will beep or display error messages. Beeps indicate a serious problem with the system configuration or hardware. The Beep Code indicates the problem. AMIBIOS Beep Codes are defined in the AMIBIOS Technical Reference. Make sure the affected part is properly seated and connected. An error message is displayed if the error is less serious. Recheck the system configuration or the connections.

Configure the System  Run WINBIOS Setup. You must enter the requested information and save the configuration data in CMOS RAM. The system will then reset, run POST, and boot the operating system. See page 50 for information on configuring the computer.

# 2	WINBIOS Setup

In ISA and EISA computers, the system parameters (such as amount of memory, type of disk drives and video displays, and many other elements) are stored in CMOS RAM. Unlike the DRAM (dynamic random access memory) that is used for standard system memory, CMOS RAM requires very little power. When the computer is turned off, a back-up battery provides power to CMOS RAM, which retains the system parameters. Every time the computer is powered-on, the computer is configured with the values stored in CMOS RAM by the system BIOS, which gains control when the computer is powered on.

The system parameters are configured by a system BIOS Setup utility. Historically, BIOS Setup utilities have been character-based, required keyboard input, and have had user interfaces that were not very intuitive.

Graphical Interface American Megatrends has a new type of system BIOS Setup utility. WINBIOS Setup has a graphical user interface the end user can access using a mouse. The WINBIOS Setup code is so compact that it can reside on the same ROM as the system BIOS. The system configuration parameters are set by WINBIOS Setup.

Since WINBIOS Setup resides in the ROM BIOS, it is available each time the computer is turned on.

Starting WINBIOS Setup	As POST executes, the following appears:

Hit <DEL> if you want to run SETUP

Press <Del> to run WINBIOS Setup.

### Using a Mouse with WINBIOS Setup

WINBIOS Setup has a built-in mouse driver and can be accessed by either a serial mouse or PS/2-style mouse. WINBIOS Setup supports Microsoft-Compatible serial mice and all PS/2-type mice.

The mouse click functions are: single click to change or select both global and current fields and double click to perform an operation in the selected field.

### Using the Keyboard with WINBIOS Setup

WINBIOS has a built-in keyboard driver that uses simple keystroke combinations:

| Keystroke | Action |
| --- | --- |
| <Tab> | Change or select a global field. |
| <, , ,  | Change or select the current field. |
| <Enter> | Perform an operation in the current field. |
| + | Increment a value. |
| – | Decrement a value. |
| <Esc> | Abort any window function. |
| <PgUp> | Return to the previous page. |
| <PgDn> | Advance to the next page. |
| <Home> | Returns to the beginning of the text. |
| <End> | Advance to the end of the text. |
| <Ctrl><Alt><+> | Change to high speed. |
| <Ctrl><Alt><-> | Change to low speed. |

### WINBIOS Setup Menu

The WINBIOS Setup main menu is organized into four sections. Each of these sections corresponds to a section in this chapter.

Each section contains several icons. Clicking on each icon activates a specific AMIBIOS function. The WINBIOS Setup main windows and related functions are described on the next page.

Main Windows	The WINBIOS Setup main windows are:

- Setup, described in Section 1 on page 50, has icons that permit you to set system configuration options such as date, time, hard disk type, floppy type, and many others,

- Security, described in Section 2 beginning on page 71, has three icons that control AMIBIOS security features, and

- Utilities, described in Section 3 beginning on page 74, sets the screen color and allows language changes,

- Default, described in Section 4 beginning on page 75, this section has three icons that permit you to select a group of settings for all WINBIOS Setup options.

## Section 1 Setup

### Standard Setup

Standard Setup options are displayed by choosing the Standard icon from the WINBIOS Setup main menu. All Standard Setup options are described in this section.

Date/Time	Select the Date/Time icon. The current values for each category are displayed. Enter new values through the keyboard.

Floppy Drive A: and B:	Move the cursor to these fields via  and  and select the floppy type. The settings are 360 KB 5¼ inch, 1.2 MB 5¼ inch, 720 KB 3½ inch, 1.44 MB 3½ inch, or 2.88 MB 3½ inch.

Pri Master, Pri Slave Select one of these hard disk drive icons to configure the hard disk drive named in the option. A scrollable screen that lists all valid disk drive types is displayed. Select the correct type and press <Enter>.

Detecting IDE	AMIBIOS automatically finds all IDE hard disk drive parameters for IDE drives. Before automatically detecting IDE drives, make sure the Onboard IDE option in Peripheral Setup is set to Enabled.

AMIBIOS places the hard disk drive parameters it finds in the Pri Master or Pri Slave fields in Standard Setup.

Cont’d

### Standard Setup, Continued

Entering Drive Parameters You can also enter the hard disk drive parameters. The drive parameters are:

| Parameter | Description |
| --- | --- |
| Type | The number for a drive with certain identification parameters. |
| Cylinders | The number of cylinders in the disk drive. |
| Heads | The number of heads. |
| Write Precompensation | The size of a sector gets progressively smaller as the track diameter diminishes. Yet each sector must still hold 512 bytes. Write precompensation circuitry on the hard disk compensates for the physical difference in sector size by boosting the write current for sectors on inner tracks. This parameter is the track number where write precompensation begins. |
| Landing Zone | This number is the cylinder location where the heads will normally park when the system is shut down. |
| Sectors | The number of sectors per track. MFM drives have 17 sectors per track. RLL drives have 26 sectors per track. ESDI drives have 34 sectors per track. SCSI and IDE drive may have even more sectors per track. |
| Capacity | The formatted capacity of the drive is the number of heads times the number of cylinders times the number of sectors per track) times 512 (bytes per sector). |

Cont’d

### Standard Setup, Continued

Hard Disk Drive Types

| Type | Cylinders | Heads | Write Precompensation | Landing Zone | Sectors | Capacity |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 306 | 4 | 128 | 305 | 17 | 10 MB |
| 2 | 615 | 4 | 300 | 615 | 17 | 20 MB |
| 3 | 615 | 6 | 300 | 615 | 17 | 31 MB |
| 4 | 940 | 8 | 512 | 940 | 17 | 62 MB |
| 5 | 940 | 6 | 512 | 940 | 17 | 47 MB |
| 6 | 615 | 4 | 65535 | 615 | 17 | 20 MB |
| 7 | 462 | 8 | 256 | 511 | 17 | 31 MB |
| 8 | 733 | 5 | 65535 | 733 | 17 | 30 MB |
| 9 | 900 | 15 | 65535 | 901 | 17 | 112 MB |
| 10 | 820 | 3 | 65535 | 820 | 17 | 20 MB |
| 11 | 855 | 5 | 65535 | 855 | 17 | 35 MB |
| 12 | 855 | 7 | 65535 | 855 | 17 | 50 MB |
| 13 | 306 | 8 | 128 | 319 | 17 | 20 MB |
| 14 | 733 | 7 | 65535 | 733 | 17 | 43 MB |
| 16 | 612 | 4 | 0 | 663 | 17 | 20 MB |
| 17 | 977 | 5 | 300 | 977 | 17 | 41 MB |
| 18 | 977 | 7 | 65535 | 977 | 17 | 57 MB |
| 19 | 1024 | 7 | 512 | 1023 | 17 | 60 MB |
| 20 | 733 | 5 | 300 | 732 | 17 | 30 MB |
| 21 | 733 | 7 | 300 | 732 | 17 | 43 MB |
| 22 | 733 | 5 | 300 | 733 | 17 | 30 MB |
| 23 | 306 | 4 | 0 | 336 | 17 | 10 MB |
| 24 | 925 | 7 | 0 | 925 | 17 | 54 MB |
| 25 | 925 | 9 | 65535 | 925 | 17 | 69 MB |
| 26 | 754 | 7 | 754 | 754 | 17 | 44 MB |
| 27 | 754 | 11 | 65535 | 754 | 17 | 69 MB |
| 28 | 699 | 7 | 256 | 699 | 17 | 41 MB |
| 29 | 823 | 10 | 65535 | 823 | 17 | 68 MB |
| 30 | 918 | 7 | 918 | 918 | 17 | 53 MB |
| 31 | 1024 | 11 | 65535 | 1024 | 17 | 94 MB |
| 32 | 1024 | 15 | 65535 | 1024 | 17 | 128 MB |
| 33 | 1024 | 5 | 1024 | 1024 | 17 | 43 MB |
| 34 | 612 | 2 | 128 | 612 | 17 | 10 MB |
| 35 | 1024 | 9 | 65535 | 1024 | 17 | 77 MB |
| 36 | 1024 | 8 | 512 | 1024 | 17 | 68 MB |
| 37 | 615 | 8 | 128 | 615 | 17 | 41 MB |
| 38 | 987 | 3 | 987 | 987 | 17 | 25 MB |
| 39 | 987 | 7 | 987 | 987 | 17 | 57 MB |
| 40 | 820 | 6 | 820 | 820 | 17 | 41 MB |
| 41 | 977 | 5 | 977 | 977 | 17 | 41 MB |
| 42 | 981 | 5 | 981 | 981 | 17 | 41 MB |
| 43 | 830 | 7 | 512 | 830 | 17 | 48 MB |
| 44 | 830 | 10 | 65535 | 830 | 17 | 69 MB |
| 45 | 917 | 15 | 65535 | 918 | 17 | 114 MB |
| 46 | 1224 | 15 | 65535 | 1223 | 17 | 152 MB |
| IDE | AMIBIOS automatically enters drive parameters. |  |  |  |  |  |  |  |  |  |  |

### Advanced Setup

Advanced Setup options are displayed by choosing the Advanced icon from the WINBIOS Setup main menu. All Advanced Setup options are described in this section.

Quick Boot	Set this option to Enabled to instruct the BIOS to boot within five seconds after the computer is powered on. AMIBIOS does not run the memory diagnostics tests on memory above 1 MB if this option is set to Enabled. The settings are Enabled and Disabled. The Optimal default is Enabled. The Fail-Safe default is Disabled.

Pause on Config. Screen  This option specifies the length of time that the AMIBIOS configuration screen appears. The settings are 2 (seconds), 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, or Disabled. The Optimal and Fail-Safe default settings are Disabled.

Boot Up Sequence This option sets the sequence of boot drives (either floppy drive A, hard disk drive C:, or a CD-ROM drive.) AMIBIOS attempts to boot from after AMIBIOS POST completes. The three settings are C:,A:,CDROM, A:,C:CDROM, and CDROM,C:,A:. The Optimal default setting is C:,A:CDROM. The Fail-Safe default setting is A:,C:CDROM.

Boot Up Num Lock  Set this option to On to turn the Num Lock key On at system boot. The settings are On or Off. The Optimal and Fail-Safe default settings are On.

Floppy Drive Swap Set this option to Enabled to permit drives A: and B: to be swapped. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Disabled.

Cont’d

### Advanced Setup, Continued

Floppy Drive Seek Set this option to Enabled to instruct AMIBIOS to perform a Seek instruction on the floppy drive at system boot up. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Disabled.

PS/2 Mouse Support When this option is enabled, AMIBIOS supports a PS/2-type mouse. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Enabled.

Primary Display This option configures the type of monitor attached to the computer. The settings are EGA/VGA, CGA80x25, Mono, CGA40x25, or Absent. The Optimal and Fail-Safe default settings are VGA/EGA.

Password Check	This option enables the password check option every time the system boots or the end user runs Setup. If Always is chosen, a user password prompt appears every time the computer is turned on. If Setup is chosen, the password prompt appears if WINBIOS is executed. See page 71 for instructions on changing a password. The Optimal and Power-On defaults are Setup.

OS/2 Compatible Mode	Set this option to Enabled if running OS/2 operating system and using more than 64 MB of system memory on the motherboard. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Disabled.

External Cache 	Set this option to Enabled to enable L2 secondary (external) cache memory. The settings are Enabled or Disabled. The Optimal default setting is Enabled. The Fail-Safe default setting is Disabled.

Cont’d

### Advanced Setup, Continued

System BIOS Cacheable	When this option is set to Enabled, the contents of the F0000h system memory segment can be read from or written to L2 secondary cache memory. The contents of the F0000h memory segment are always copied from the BIOS ROM to system RAM for faster execution. The settings are Enabled or Disabled. The Optimal default setting is Enabled. The Fail-Safe default setting is Disabled.

C000,16K Shadow

C400,16K Shadow  This option controls the location of the contents of video ROM. The settings are:

| Setting | Description |
| --- | --- |
| Shadow | The contents of the video ROM area (C0000h - C7FFFh) are written to the corresponding address in RAM. |
| Cached | The contents of the video ROM area (C0000h - C7FFFh) are written to the corresponding RAM address and may be read from or written to cache memory. |
| Disabled | The video ROM is not copied to RAM. The contents of the video ROM cannot be read from or written to cache memory. |

The Optimal default setting is Cached. The Fail-Safe default setting is Disabled.

C800,16K Shadow

CC00,16K Shadow

D000,16K Shadow

D400,16K Shadow

D800,16K Shadow

DC00,16K Shadow  These options enable shadowing of the contents of the ROM area in the option title.

| Setting | Description |
| --- | --- |
| Shadow | The contents of the ROM area are written to the corresponding address in RAM for faster execution. |
| Cached | The contents of the ROM area are written to the corresponding RAM address and can be read from or written to cache memory. |
| Disabled | The ROM is not copied to RAM. The contents of the video ROM cannot be read from or written to cache memory. |

The Optimal and Fail-Safe settings are Disabled.

### Chipset Setup

Memory Hole	This option allows the end user to specify a memory hole. The settings are Disabled, 512-640K, or 15-16M (from 15 MB to 16 MB). The Optimal and Fail-Safe default settings are Disabled.

8-Bit I/O Recovery Time (SYSCLK)  This option specifies the length of a delay (specified in SYSCLKs) inserted before consecutive 8-bit I/O operations. The settings are 1 (SYSCLK), 2, 3, 4, 5, 6, 7, or 8. The Optimal and Fail-Safe defaults settings are 1.

16-Bit I/O Recovery Time (SYSCLK)  This option specifies the length of a delay (specified in SYSCLKs) inserted before consecutive 8-bit I/O operations. The settings are 1 (SYSCLK), 2, 3, 4, 5, 6, 7, or 8. The Optimal and Fail-Safe defaults settings are 1.

DRAM Timing	This option specifies the DRAM timings. The settings are Auto or Manual. When set to Auto, AMIBIOS automatically sets the DRAM timings to optimal values based on the CPU speed. When set to manual, you must set DRAM timings via the DRAM Read Burst Timing and DRAM Write Burst Timing Chipset Setup options. The Optimal and Fail-Safe default settings are Auto.

Turbo Rate Leadoff  Set this option to Enabled to enable the DRAM data pipeline transmissions for improved read leadoff timing performance. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings vary, depending on the CPU speed and DRAM type.

Cont’d

### Chipset Setup, Continued

DRAM Read Burst Timing  This option is only valid if the Chipset Setup DRAM Timing option has been set to Manual. This option specifies the DRAM read burst timings. The settings are x-4-4-4, x-3-3-3, or x-2-2-2. The Optimal and Fail-Safe default settings vary, depending on the CPU speed and DRAM type.

DRAM Write Burst Timing  This option is only valid if the Chipset Setup DRAM Timing option has been set to Manual. This option specifies the DRAM write burst timings. The settings are x-4-4-4, x-3-3-3, or x-2-2-2. The Optimal and Fail-Safe default settings vary, depending on the CPU speed and DRAM type.

Fast RAS to CAS Delay (Clocks)  This option specifies the length of a delay (in clocks) inserted between the assertion of the RAS# active signal and the CAS# active signal. The settings are 2 (CLKs) or 3. The Optimal and Fail-Safe default settings vary, depending on the CPU speed and DRAM type.

DRAM Leadoff Timing (DLT)   This option specifies the ratio of the DRAM read/write leadoff timings, RAS# precharge time, and refresh RAS# signal assertion. The settings are: 7/6/3/4, 6/5/3/4, 7/6/4/5, or 6/5/4/5. The Optimal and Fail-Safe default settings are 6/5/3/4.

Memory Address Drive Strength  This option sets the strength of the drive memory address and control signals. The settings are 8ma/8ma, 8ma/12ma, 12ma/8ma, 12ma/12ma, or Auto. Select Auto to allow AMIBIOS to automatically determine the correct signal strength. The Optimal and Fail-Safe default settings are Auto.

Cont’d

### Chipset Setup, Continued

Extended Cacheability  This option sets the maximum amount of system memory that can be read from or written to L2 secondary cache memory. The settings are 64 MB or 512 MB. The Optimal and Fail-Safe default settings are 64 MB.

Peer Concurrency Set this option to Enabled to permit the CPU to remain active while activity occurs on the PCI bus. The settings are Enabled or Disabled. The Optimal default setting is Enabled. The Fail-Safe default setting is Disabled.

Memory Parity/ECC Check  Use this option to enable DRAM parity checking or ECC (Error Checking and Correction). This option is only effective if the DRAM interface on the DRAM array mode installed in the computer is 72-bits wide. The settings are Disabled, Parity, or ECC. The Optimal and Fail-Safe default settings are Disabled.

USB Function Enable  Set this option to Enabled to enable USB (Universal Serial Bus) support. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Disabled.

USB Passive Release Enable  Set this option to Enabled if you have provided a programmable mechanism to meet the required USB master latency times. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Disabled.

### Power Management Setup

The AMIBIOS Setup options described in this section are selected by choosing the Power Management Setup icon from the Setup section on the AMIBIOS Setup main menu.

Power Management/APM Set this option to Enabled to enable the Intel Triton power management features and APM (Advanced Power Management). The settings are Enabled, Inst-On (instant-on), or Disabled. The Optimal and Fail-Safe default settings are Disabled.

Instant-On Timeout (Minute) This option specifies the length of a period of system inactivity while the computer is in Full power on state. When this length of time expires, AMIBIOS takes the computer to a lower power consumption state, but the computer can return to full power instantly when any system activity occurs. This option is only available if supported by the computer hardware. The settings are Disabled and 1 Min. through 15 Min in 1 minute intervals. The Optimal and Fail-Safe default settings are Disabled.

Green PC Monitor Power State This option specifies the power state that the green PC-compliant video monitor enters when AMIBIOS places it in a power saving state after the specified period of display inactivity has expired. The settings are Off, Standby, Suspend, or Disabled. The Optimal and Fail-Safe default settings are Standby.

Video Power Down Mode This option specifies the power conserving state that the VESA VGA video subsystem enters after the specified period of display inactivity has expired. The settings are Disabled, Standby, or Suspend. The Optimal and Fail-Safe default settings are Disabled.

Cont’d

### Power Management Setup, Continued

Hard Disk Power Down Mode This option specifies the power conserving state that the hard disk drive enters after the specified period of hard drive inactivity has expired. The settings are Disabled, Standby, or Suspend. The Optimal and Fail-Safe default settings are Disabled.

Hard Disk Timeout (Minute) This option specifies the length of a period of hard disk drive inactivity. When this length of time expires, the computer enters power-conserving state specified in the Hard Disk Power Down Mode option (see the previous page). The settings are Disabled and 1 Min. through 15 Min in 1 minute intervals. The Optimal and Fail-Safe default settings are Disabled.

Standby Timeout (Minute) This option specifies the length of a period of system inactivity while in Full power on state. When this length of time expires, the computer enters Standby power state. The settings are Disabled and 1 Min. through 15 Min in 1 minute intervals. The Optimal and Fail-Safe default settings are Disabled.

Suspend Timeout (Minute) This option specifies the length of a period of system inactivity while in Standby state. When this length of time expires, the computer enters Suspend power state. The settings are Disabled and 1 Min. through 15 Min in 1 minute intervals. The Optimal and Fail-Safe default settings are Disabled.

Cont’d

### Power Management Setup, Continued

Stop Clock Throttling Ratio This option specifies the speed at which the system clock runs in power saving states. The settings are expressed as a ratio between the normal CPU clock speed and the CPU clock speed when the computer is in the power-conserving state. The settings are 1:1, 1:2, 1:4, 1:8, 1:16, 1:32, 1:64, and 1:128. The Optimal and Fail-Safe defaults are 1:8.

IRQ3

IRQ4

IRQ5

IRQ7

IRQ9

IRQ10

IRQ11

IRQ12

IRQ14

IRQ15		When set to Monitor, these options enable event monitoring on the specified IRQ line. If set to Monitor and the computer is in a power saving state, AMIBIOS watches for activity on the specified IRQ line. The computer enters the full on power state if any activity occurs. AMIBIOS reloads the Standby and Suspend timeout timers if activity occurs on the specified IRQ line. The settings for each of these options are Ignore, Monitor, Wakeup, or Both. The defaults are:

| Option | Optimal | Fail-Safe |
| --- | --- | --- |
| IRQ3 | Both | Both |
| IRQ4 | Both | Both |
| IRQ5 | Ignore | Ignore |
| IRQ7 | Ignore | Ignore |
| IRQ9 | Ignore | Ignore |
| IRQ10 | Ignore | Ignore |
| IRQ11 | Ignore | Ignore |
| IRQ12 | Both | Both |
| IRQ14 | Both | Both |
| IRQ15 | Both | Both |

### PCI/PnP Setup

Choose the PCI/PnP Setup icon from the WINBIOS Setup screen to display the PCI and Plug and Play Setup options, described below.

Plug and Play-Aware OS Set this option to Yes if the operating system in this computer is aware of and follows the Plug and Play specification. Windows 95 is PnP-aware. The settings are Yes or No. The Optimal and Fail-Safe default settings are No.

PCI Slot-1 Latency Timer

PCI Slot-2 Latency Timer

PCI Slot-3 Latency Timer

PCI Slot-4 Latency Timer These options specify the latency timings (in PCI clocks) for PCI devices installed in the four PCI expansion slots. The settings are 32, 64, 96, 128, 160, 192, 224, or 248. The Optimal and Fail-Safe default settings are 64.

PCI VGA Palette Snoop When this option is set to Enabled, multiple VGA devices operating on different buses can handle data from the CPU on each set of palette registers on every video device. Bit 5 of the command register in the PCI device configuration space is the VGA Palette Snoop bit (0 is disabled). For example: if there are two VGA devices in the computer (one PCI and one ISA) and:

| VGA Snoop Bit | Action |
| --- | --- |
| Disabled | Data read and written by the CPU is only directed to the PCI VGA device's palette registers. |
| Enabled | Data read and written by the CPU is directed to the both the PCI VGA device's palette registers and the ISA VGA device palette registers, permitting the palette registers of both devices to be identical. |

This option must be set to Enabled if an installed ISA adapter card requires VGA palette snooping. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Disabled.

Cont’d

### PCI/PnP Setup, Continued

PCI Slot-1 IRQ Preference

PCI Slot-2 IRQ Preference

PCI Slot-3 IRQ Preference

PCI Slot-4 IRQ Preference These options specify the IRQ priority for PCI devices installed in the four PCI expansion slots. The settings are IRQ3, IRQ4, IRQ5, IRQ7, IRQ9, IRQ10, IRQ11, IRQ12, IRQ 14, and IRQ15, in priority order.

DMA Channel 0

DMA Channel 1

DMA Channel 3

DMA Channel 5

DMA Channel 6

DMA Channel 7	These options allow you to specify the bus type used by each DMA channel. The settings are PnP or ISA/EISA. The Optimal and Fail-Safe default settings are PnP.

Cont’d

### PCI/PnP Setup, Continued

IRQ3

IRQ4

IRQ5

IRQ7

IRQ9

IRQ10

IRQ11

IRQ14

IRQ15		These options specify the bus that the specified IRQ line is used on. These options allow you to reserve IRQs for legacy ISA adapter cards.

These options determine if AMIBIOS should remove an IRQ from the pool of available IRQs passed to devices that are configurable by the system BIOS. The available IRQ pool is determined by reading the ESCD NVRAM. If more IRQs must be removed from the pool, the end user can use these options to reserve the IRQ by assigning an ISA/EISA setting to it. Onboard I/O is configured by AMIBIOS. All IRQs used by onboard I/O are configured as PCI/PnP.

IRQ14 and 15 will not be available if the onboard Triton PCI IDE is enabled. If all IRQs are set to ISA/EISA and IRQ14 and 15 are allocated to the onboard PCI IDE, IRQ9 will still be available for PCI and PnP devices, because at least one IRQ must be available for PCI and PnP devices.

The settings are ISA/EISA or PCI/PnP. The Optimal and Fail-Safe default settings for IRQ3 through 7 are ISA/EISA. The Optimal and Fail-Safe default settings for IRQ0 through 15 are PCI/PnP.

PCI Peer-To-Peer Traffic  Set this option to Enabled to enable peer-to-peer transmission between PCI devices on the PCI local bus. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Disabled.

### Peripheral Setup

Choose the Peripheral Setup icon from the WINBIOS Setup screen to display the Peripheral Setup options, described below.

Onboard FDC	Set this option to Enabled to enable the floppy drive controller on the motherboard. The settings are Auto (AMIBIOS automatically determines if the floppy controller should be enabled), Enabled, or Disabled. The Optimal and Fail-Safe default settings are Enabled.

Onboard Serial Port1 This option specifies the base I/O port address of serial port 1. The settings are Auto (AMIBIOS automatically determines the correct base I/O port address), Disabled, 3F8h, 2F8h, 2E8h, or 3E8h. The Optimal default setting is 3F8h. The Fail-Safe default setting is Disabled.

Serial Port2 Mode  This option specifies the serial port 2 mode. The settings are Normal, IrDA 3/16 bit, or Sharp-IR. The Optimal and Fail-Safe default settings are Normal.

IR Duplex Mode  This option selects the infrared transmission method. The settings are Full or Half. The Optimal and Fail-Safe default settings are Full.

Onboard Serial Port2 This option specifies the base I/O port address of serial port 2. The settings are Auto (AMIBIOS automatically determines the correct base I/O port address), Disabled, 3F8h, 2F8h, 2E8h, or 3E8h. The Optimal default setting is 3F8h. The Fail-Safe default setting is Disabled.

Cont’d

### Peripheral Setup, Continued

Onboard Parallel Port This option specifies the base I/O port address of the parallel port on the motherboard. The settings are Auto (AMIBIOS automatically determines the correct base I/O port address), Disabled, 378h, 278h, or 3BCh. The Optimal and Fail-Safe default settings are 378h.

Parallel Port IRQ This option specifies the IRQ used by the parallel port. The settings are 5 or 7. The Optimal default setting is 7. The Fail-Safe default setting is Disabled.

Parallel Port Mode This option specifies the parallel port mode. The Optimal and Fail-Safe default settings are Normal. The settings are:

| Setting | Description |
| --- | --- |
| Normal | The normal parallel port mode is used. |
| Bi-Dir | Use this setting to support bidirectional transfers on the parallel port. |
| EPP | The parallel port can be used with devices that adhere to the Enhanced Parallel Port (EPP) specification. EPP uses the existing parallel port signals to provide asymmetric bidirectional data transfer driven by the host device. |
| ECP | The parallel port can be used with devices that adhere to the Extended Capabilities Port (ECP) specification. ECP uses the DMA protocol to achieve data transfer rates up to 2.5 Megabits per second. ECP provides symmetric bidirectional communication. |

Parallel Port DMA Channel This option is only available if the setting for the Parallel Port Mode option is Extended or ECP. This option sets the DMA channel used by the parallel port. The settings are (DMA Channel) 0, 1 or 3. The Optimal and Fail-Safe default settings are 3.

Cont’d

### Peripheral Setup, Continued

Onboard IDE 	This option specifies the IDE channel used by the onboard IDE controller. The settings are Disabled, Primary, Secondary, or Both. The Optimal and Fail-Safe default settings are Disabled.

PCI IDE Bus Master Set this option to Enabled if the onboard IDE controller is a PCI bus mastering device. The settings are Enabled or Disabled. The Optimal and Fail-Safe default settings are Enabled.

Offboard PCI IDE Card This option specifies the PCI expansion slot that the offboard PCI IDE Controller adapter card is installed in. If an offboard PCI IDE Controller adapter card is installed in a PCI expansion slot, the onboard IDE controller is automatically disabled. Choose Auto to let AMIBIOS automatically select the correct setting for this option. If Auto is selected, IRQs 14 and 15 are forced to the PCI expansion slot on the PCI local bus where the offboard PCI Controller adapter card is installed..

The settings are Disabled, Auto, Slot1, Slot2, Slot3, or Slot4. The Optimal and Fail-Safe default settings are Auto.

Offboard PCI IDE Primary IRQ

Offboard PCI IDE Secondary IRQ These options specify the PCI interrupt used by the primary and secondary IDE channels if an offboard IDE controller is installed in the computer. The settings are Disabled, Hardwired, INTA, INTB, INTC, or INTD. The Optimal and Fail-Safe default settings are Disabled.

## Section 2 Security

Three icons appear in this part of the WINBIOS Setup screen:

- Supervisor (Password),

- User (Password), and

- Anti-Virus (see page 73).

Two Levels of Passwords Both the Supervisor and the User icons configure password support. If you use both, the Supervisor password must be set first.

The system can be configured so that all users must enter a password every time the system boots or when WINBIOS Setup is executed, using either or both the Supervisor password or User password.

### AMIBIOS Password Support

The Supervisor and User icons activate two different levels of password security.

If WINBIOS Setup has an optional password feature. The system can be configured so that all users must enter a password every time the system boots or when WINBIOS Setup is executed.

### Setting a Password

The password check option is enabled in Advanced Setup (see page 55 by choosing either Always (the password prompt appears every time the system is powered on) or Setup (the password prompt appears only when WINBIOS is run). The password is encrypted and stored in NVRAM.

As shown on the above screen, you are prompted for a 1 – 6 character password. You can either type the password on the keyboard or select each letter of the password, one at a time, using the mouse. The password does not appear on the screen when typed. Make sure you write it down. If you forget it, you must drain NVRAM and reconfigure.

If You Do Not Want to Use a Password	Just press <Enter> when the password prompt appears.

### Changing a Password

Select the Supervisor or User icon from the Security section of the WINBIOS Setup main menu. Enter the password and press <Enter>. The screen does not display the characters entered. After the new password is entered, retype the new password as prompted and press <Enter>.

If the password confirmation is incorrect, an error message appears. If the new password is entered without error, press <Esc>. The password is stored in NVRAM after WINBIOS completes. The next time the system boots, a password prompt appears if the password function is present and enabled.

Remember the Password	Keep a record of the new password when the password is changed. If you forget the password, you must erase the system configuration information in NVRAM (Non-Volatile Random Access Memory). See page 26 for information about erasing system configuration information.

### Anti-Virus

When this icon is selected from the Security section of the WINBIOS Setup main menu, AMIBIOS issues a warning when any program (or virus) issues a Disk Format command or attempts to write to the boot sector of the hard disk drive. The settings are Enabled or Disabled. If enabled, the following appears when a write is attempted to the boot sector. You may have to type N several times to prevent the boot sector write.

Boot Sector Write!!!

Possible VIRUS: Continue (Y/N)? _

The following appears after any attempt to format any cylinder, head, or sector of any hard disk drive via the BIOS INT 13 Hard Disk Drive Service:

Format!!!

Possible VIRUS: Continue (Y/N)? _

## Section 3 Utility

The following icons appear in this section of the WINBIOS Setup main screen:

Color Set	Color Set sets the Setup screen colors.

Language	If this feature is enabled, you can select WINBIOS Setup messages in different languages.

## Section 4 Default

The icons in this section permit you to select a group of settings for all WINBIOS Setup options. Not only can you use these icons to quickly set system configuration parameters, you can choose a group of settings that have a better chance of working when the system is having configuration-related problems.

Original	Choose the Original icon to return to the system configuration values present in WINBIOS Setup when you first began this WINBIOS Setup session.

Optimal		You can load the optimal default settings for the WINBIOS by selecting the Optimal icon. The Optimal default settings are best-case values that should optimize system performance. If NVRAM is corrupted, the Optimal settings are loaded automatically.

Fail-Safe	You can load the Fail-Safe WINBIOS Setup option settings by selecting the Fail-Safe icon from the Default section of the WINBIOS Setup main menu.

The Fail-Safe settings provide far from optimal system performance, but are the most stable settings. Use this option as a diagnostic aid if the system is behaving erratically.

# 3	Programming the Flash ROM

All versions of the Apollo III motherboard use Flash EPROM to store the system BIOS. The advantage of Flash EPROM is the EPROM chip does not have to be replaced to update the BIOS. The end user can actually reprogram the BIOS, using a ROM file supplied by American Megatrends.

Programming the Flash EPROM

| Step | Action |
| --- | --- |
| 1 | Turn power off. Make sure the computer has a working speaker. |
| 2 | Insert the floppy disk with the S752P.ROM file in drive A:. |
| 3 | Before DOS boots, press and hold down the <Ctrl> and <Home> keys to reprogram the Flash EPROM-based AMIBIOS. The bootblock code immediately reads the A: drive, looking for the new BIOS information. |
| 4 | When the flash ROM has successfully been programmed, the computer will reboot. |

Bootblock BIOS Actions	When you reprogram from system boot, the bootblock BIOS code:

Reads S752P.ROM from the root directory of the floppy disk in drive A:.

Erases the Flash EPROM.

Programs the Flash EPROM with the data read from the floppy disk in drive A:.

Generates a CPU reset, rebooting the computer.

The bootblock part of the Flash EPROM is not programmed. Should you inadvertently open the disk drive door or turn power off to the computer while programming the Flash EPROM, the bootblock will be unaffected. Simply turn power back on and begin the Flash ROM programming process again.

Cont’d

### Programming the Flash ROM, Continued

S752P.ROM	S752P.ROM resides on a floppy disk and contains the updated main BIOS code. American Megatrends will provide this file when the AMIBIOS for the Apollo III ISA motherboard must be updated.

S752P.ROM must be present in the root directory of the floppy disk before the onboard Flash EPROM can be reprogrammed. The file that has the main BIOS code must be named S752P.ROM.

Sequence of Operation  The sequence of operation and expected behavior of the bootblock BIOS code is:

| Step | Expected behavior |
| --- | --- |
| 1 Look for floppy disk. | The system beeps one time before the BIOS attempts to read from floppy drive A:. |
| 2 Look for S752P.ROM on the floppy disk. | S752P.ROM must be in the root directory of the floppy disk in drive A:. There is no beep if successful. |
| 3 Read the floppy disk. | The floppy disk is read. There is no beep if this step is successful. |
| 4 Check for BIOS file size. | The BIOS file size is checked. There is no beep if this step is successful. |
| 5 Check for Flash EPROM. | The BIOS looks for an Intel i28F001BX-T Flash EPROM. It does not beep if this step is successful. |
| 6 Erase the Flash EPROM. | Two beeps sound when the BIOS begins erasing the Flash EPROM. |
| 7 Program the Flash EPROM. | Three beeps sound when the AMIFlash Code begins reprogramming the Flash EPROM. |
| 8 Continue programming the Flash EPROM. | Four beeps sound when reprogramming has been successfully completed. |
| 9 AMIFlash does a reset. | A CPU reset is generated to reboot the computer. |

### Programming the Flash ROM, Continued

Beep Codes	The bootblock code produces a series of beeps during Flash ROM programming to:

- signify completion of a step (as shown on the previous page), or to

- signal an error.

Error beeps are arranged in a coded sequence and have different meanings depending on when they occur. The error beep codes and when they can occur are:

| Number of Beeps | Description |
| --- | --- |
| 1 | Insert diskette in floppy drive A:. |
| 2 | The AMIBOOT.ROM file was not found in the root directory of the diskette in floppy drive A:. |
| 3 | Base memory error. |
| 4 | Flash program successful. |
| 5 | Floppy read error. |
| 6 | Keyboard controller BAT command failed. |
| 7 | No Flash EPROM detected. |
| 8 | Floppy controller failure. |
| 9 | Boot Block BIOS checksum error. |
| 10 | Flash erase error. |
| 11 | Flash program error. |
| 12 | AMIBOOT.ROM file size error. |
| Continuous beep | Flash Programming successful. Turn power off. The turn power on again to restart. |

### Bootblock Code Checkpoint Codes

| Code | Description |
| --- | --- |
| E0h | Verify the bootblock BIOS checksum. Disable the internal cache, DMA, and interrupt controllers. Initialize the system timer. Start memory refresh. |
| E1h | Initialize the chipset registers. Set the BIOS size to 128K. Make the 512 KB base memory available. |
| E2h | Test the base 64 KB of system memory. Send the BAT command to the keyboard controller. Make sure that <Ctrl> <Home> was pressed. Verify the main system BIOS checksum. |
| E3h | The main system BIOS is good. Transfer control to the main system BIOS. |
| E4h | Start the memory test. |
| E5h | The memory test is over. Initialize the interrupt vector table. |
| E6h | Initialize the DMA and interrupt controllers. |
| E7h | Determine the CPU internal clock frequency. |
| E8h | Initialize the I/O chipset, if any. |
| E9h | Program the CPU clock-dependent chip set parameters. |
| EAh | Enable the timer and the floppy diskette interrupt. Enable the internal cache. Copy the bootblock BIOS and pass control to the bootblock BIOS in the 0000h segment. |
| EDh | Initialize the floppy drive. |
| EEh | Look for a diskette in drive A:. Read the first sector of the diskette. |
| EFh | Floppy read error. |
| F0h | Search for AMIBOOT.ROM in the root directory of the floppy diskette in drive A:. |
| F1h | The AMIBOOT.ROM file is not in the root directory. |
| F2h | Read the FAT. Analyze the FAT to find the clusters occupied by the AMIBOOT.ROM. |
| F3h | Start reading the AMIBOOT.ROM file, cluster by cluster. |
| F4h | The AMIBOOT.ROM file is not the correct size. |
| F5h | Disable the internal cache. Raise the Vpp. Enable Flash write and reset the Flash ROM. |
| FBh | Detect the flash type. |
| FCh | Start erasing flash blocks. |
| FDh | Program the Flash ROM in the E0000-EFFFFh region. |
| FEh | Start programming Flash at F0000-FFFFF region. |
| FFh | Flash programming is successful. The computer reboots. |

# A	Specifications

| Item | Description |
| --- | --- |
| CPU | Intel Pentium 75, 90, 100, 120, 133, 150, 166, 180, 200 MHz or higher speed CPU |
| Upgrade CPU | Intel P54C series, P54CT, or P55C with an optional split voltage regulator module. |
| Chipset | Intel 82430HX, including CPU interface controller, advanced cache controller, integrated DRAM controller, synchronous ISA bus controller, PCI local bus interface, and integrated power management unit. |
| Expansion slots | Three ISA expansion slots Four PCI expansion slots |
| L1 internal cache memory | The Intel Pentium has 8 KB data cache and 8 KB instruction cache. |
| L2 secondary cache memory | 256 KB or 512 KB of synchronous PBSRAM, direct-mapped, write-back. |
| Cache memory /system memory | 256 KB caches up to 128 MB of system memory 512 KB caches up to 512 MB of system memory |
| Type of System Memory | Up to 512 MB mounted on the motherboard in two banks of 72-pin 4 MB, 8 MB, 16 MB, or 32 MB SIMMs. |
| System memory | Fast page mode or Extended Data Out (EDO) SIMMs operating at 70 ns. |
| Memory buffer | One level posted write memory buffer |
| Max. system memory | 128 MB |
| Fast ATA | Supports the Fast ATA specification using PIO mode 4 and multiword DMA mode 2. |
| System BIOS | This motherboard has a 128 KB AMIBIOS system BIOS on a Flash ROM with built-in WINBIOS Setup. |
| BIOS shadowing | The system BIOS is always copied from ROM to RAM for faster execution. The end user can shadow 16 KB ROM segments from C000h – DCFFFh. |
| AMIBIOS features | IDE block mode support, IDE 32-bit data transfer support, IDE Programmed I/O mode 0, 1, 2, 3, and 4 support, IDE LBA mode support, APM (Advanced Power Management) and Flash BIOS hooks, EPA Green PC-compliant, PCI and Plug and Play (PnP) support, and DIM (Device Initialization Manager) support, DMI (Desktop Management Interface) support, ATAPI support, can boot from a CD-ROM drive, automatically detects system memory, cache memory, and IDE drive parameters, Intel NSP-compliant, Fast ATA IDE mode programming, Boot sector virus protection, instant-on support, automatically configures PnP and PCI devices. |
| IDE | Provides two 40-pin IDE connectors onboard that support up to four IDE drives. IDE controller is on the PCI local bus. |
| Floppy | Onboard support for up to two 360 KB, 720 KB, 1.2 MB, 1.44 MB, or 2.88 MB floppy drives. |
| Parallel port | Onboard ECP and EPP-capable parallel port connector. |
| Serial ports | Two onboard serial port connectors. Two 16550 UART serial ports. |
| Keyboard | Includes a standard miniDIN keyboard connectors. |
| USB | Two USB connectors are provided on the motherboard. |
| Infrared | An infrared connector is provided on the motherboard |
| Mouse | Includes a 10-pin berg mouse connector. |
| Power supply | Includes three power supply connectors. |
| Real time clock/ CMOS RAM | A real time clock and 128 bytes of CMOS RAM with a battery backup is provided on the motherboard. |
| Power management | Power management services include: Green PC LED, power management signal to Green PC-aware power supplies, automatic IDE and video power down, monitor blanking, SMI (System Management Interrupt) support, APM, and system stop clock. |
| Speaker | Standard four-pin speaker connection. |

# Index

1

16-Bit I/O Recovery Time (SYSCLK), 46

3

32-bit data transfer support, 68

8

8-Bit I/O Recovery Time (SYSCLK), 46

A

Advanced Setup, 43

AMIBIOS features, 68

AMIFlash

Beep Codes, 65

Anti-Virus

WINBIOS Setup, 60

ATAPI support, 68

B

BBS, v

BIOS

Password Support, 58

BIOS Defaults, 62

BIOS Errors, 36

BIOS Security, 58

BIOS shadowing, 67

BIOS Utility, 61

Block Connector, 27

Block mode support, 68

Boot sector virus protection, 68

Boot Up Num Lock, 43

Boot Up Sequence, 43

C

C000,16K Shadow, 45

C400,16K Shadow, 45

C800,16K Shadow, 45

Cables

Connecting, 21, 25

Cache Memory

Configuring, 17

CC00,16K Shadow, 45

CD-ROM drive

booting from, 68

Chipset, 67

CN4 Mouse connector, 24

CN6 Pinout, 23

CN8 Parallel Port connector, 29

CN9 floppy connector, 30

Color Set, 61

COM1, 28

COM2, 28

Configuring System, 36

Conflicts, 28

Connectors, 20

Floppy disk, 30

IDE Hard Disk Drive, 33

Keyboard, 23

Serial ports, 28

SIMM, 16

CPS Clear Password, 21

CPU, 67

Configuring, 10

Installing, 12

CPU Fan connector, 25

D

D000,16K Shadow, 45

D400,16K Shadow, 45

D800,16K Shadow, 45

Date/Time, 40

DC00,16K Shadow, 45

Desktop Management Interface, 68

Device Initialization Manager, 68

DIM, 68

DMA Channel 0, 53

DMA Channel 1, 53

DMA Channel 3, 53

DMA Channel 5, 53

DMA Channel 6, 53

DMA Channel 7, 53

DMI, 68

DRAM Configurations, 15

DRAM Leadoff Timing (DLT), 47

DRAM Read Burst Timing, 47

DRAM Timing, 46

DRAM Write Burst Timing, 47

E

Expansion slots, 67

Extended Cacheability, 48

External Cache, 44

F

Fail-Safe BIOS Setup Settings

WINBIOS Setup, 62

Fast ATA, 67

Fast ATA IDE mode programming, 68

Fast RAS to CAS Delay (Clocks), 47

Figures

Floppy drive cable, 30

Floppy drive connector, 30

Power supply key cutting, 23

Flash EPROM

Programming, 63

Floppy Disk Connector

Pinout, 30

Floppy Drive A: and B, 40

Floppy Drive connector

Pinout, 31

Floppy Drive Seek, 44

Floppy Drive Swap, 43

G

Green PC, 68

Green PC Monitor Power State, 49

H

Hard Disk Drive Capacity, 41

Hard Disk Drive Parameters, 41

Hard Disk Power Down Mode, 50

Hard Disk Timeout (Minute), 50

I

IDE Drive connectors, 32

IDE drive parameters, 40

IDE Drivers, 33

IDE Hard Disk Connector, 33

IDE1 Pinout, 34

Infrared, 68

Install Memory, 14

Installation, 6

Installing Adapter Cards, 36

Installing the Motherboard, 18

Instant-on support, 68

Instant-On Timeout (Minute), 49

Intel 82430HX, 67

IR Duplex Mode, 55

IRQ10, 51, 54

IRQ11, 51, 54

IRQ12, 51

IRQ14, 51, 54

IRQ15, 51, 54

IRQ3, 51, 54

IRQ4, 51, 54

IRQ5, 51, 54

IRQ7, 51, 54

IRQ9, 51, 54

J

J1 Remote Control Power, 26

J4 Pinout, 35

J44 Green PC Power, 25

J7 CPU Fan, 25

K

Keyboard Cable, 23

Keyboard connector, 23

L

L1 internal cache memory, 67

L2 secondary cache memory, 67

Landing Zone, 41

Language, 61

LBA mode, 68

M

Memory

Reporting, 14

Memory Address Drive Strength, 47

Memory Hole, 46

Memory Parity/ECC Check, 48

Motherboard Size, 2

Mouse Cable, 24

Mouse Support, 44

N

NSP, 68

Number of Cylinders, 41

Number of Heads, 41

Number of Sectors, 41

O

Offboard PCI IDE Primary IRQ, 57

Offboard PCI IDE Secondary IRQ, 57

Offboard PCI/ISA IDE Card, 57

Onboard Adapters, 28

Onboard Floppy Controller, 55

Onboard I/O, 5

Onboard IDE, 57

Onboard Parallel Port, 56

Onboard Serial Port1, 55

Onboard Serial Port2, 55

Optimal default settings

WINBIOS Setup, 62

Original, 62

OS/2 Compatible Mode, 44

Overview, 1

P

Parallel Port, 29

Parallel Port Address, 56

Parallel Port DMA Channel, 56

Parallel Port Mode, 56

Password Check, 44

Password Support

Levels of, 58

Pause on Config. Screen, 43

PCI IDE Bus Master, 57

PCI Peer-To-Peer Traffic, 54

PCI Slot-1 IRQ Preference, 53

PCI Slot-1 Latency Timer, 52

PCI Slot-2 IRQ Preference, 53

PCI Slot-2 Latency Timer, 52

PCI Slot-3 IRQ Preference, 53

PCI Slot-3 Latency Timer, 52

PCI Slot-4 IRQ Preference, 53

PCI Slot-4 Latency Timer, 52

PCI specification, 5

PCI VGA Palette Snoop, 52

PCI/PnP Setup, 52

Peer Concurrency, 48

Pentium, 67

Peripheral Setup, 55

Onboard IDE, 32

Pinout

J4 Secondary IDE connector, 35

Parallel Port, 29

Serial ports, 28

Pinouts

Serial ports, 28

Plug and Play-Aware OS, 52

Power management, 68

Power Management Setup, 49

Power Management/APM, 49

Power Supply, 22

Pinouts, 23

Pri Master, 40

Pri Slave, 40

Primary Display, 44

Programmed I/O mode, 68

PS/2 Mouse Support, 44

PS/2-type mouse, 24

Q

Quick Boot, 43

R

Real time clock/, 68

Remote Control Power, 26

Reporting Memory, 14

Resource conflicts, 28

S

S752P.ROM, 64

S752P.ROM file, 63

Secondary IDE Controller, 34

Serial port

Pinout, 28

Serial Port1 Address, 55

Serial Port1 IRQ, 56

Serial Port2 Mode, 55

SIMM Part Numbers, 14

SIMMs

Installing, 16

Part numbers, 14

Size, 2

Speaker, 68

Specifications, 67

Standard Setup, 40

Standby Timeout (Minute), 50

Static Electricity, 9

Stop Clock Throttling Ratio, 51

Supervisor Password, 58

Suspend Timeout (Minute), 50

System BIOS, 67

System BIOS Cacheable, 45

System Memory, 67

Configuring, 14

System memory configurations, 15

T

Tables

Floppy pinout, 31

Hard Disk Drive Types, 42

I/O conflicts, 28

Serial port pinout, 28

Steps for installing motherboard, 18

Twist in floppy cable, 31

Technical Support, v

Test motherboard, 36

Testing System, 36

Turbo Rate Leadoff, 46

U

Unpack the Motherboard, 8

Upgrade CPU, 67

USB, 68

USB Function Enable, 48

USB Passive Release Enable, 48

User Password, 58

V

Video Power Down Mode, 49

Virus protection, 68

W

WINBIOS Setup, 37

Write Precompensation, 41