J-bond Computer System Corp.

[PCI400C-B]

Here is a list of these support files :

File Name       Description                size

******************************************************

readme.doc      this file		   2394

imsflash.exe    BIOS update utility        15917

032295.rom      new BIOS filename          131072

for system board (PCI400C-B)

************   Updating Your System BIOS   ************

To update your system BIOS, perform the steps described below. Please read

through all of the steps carefully before beginning.

1. Make sure there is a MXIC 28F010 compatible(e.g.,MX28F1000) flashable

BIOS EPROM on your system board.

2. Please jump JP19 = 2-3 and JP20 = 2-3 , before power-on.

3. Boot your system from a DOS system diskette that contains no config.sys

or autoexec.bat file. (If the diskette contains a config.sys or

autoexec.bat file, the BIOS updating program will not work properly.)

4. Place the BIOS update disk into drive A (or drive B). The BIOS update disk

contains THREE files: there are readme.doc, Imsflash.exe and the new BIOS

data file (032295.rom). Note the name of the file containing the new BIOS

data and then enter the following command:

a.   A:\IMSFLASH

b. You will see the following:

FLASH UPDATE Utility Version 1.10      Jun, 10 1994

Copyright (C) 1994, Integrated Micro Solutions Ins.

**** Flash Memory Menu *****

*                          *

* 1) AT29C010              *

* 2) AM29F010              *

* 3) CAT28F010             *

* 4) INTEL28F010           *

* 5) 28F010 Compatible     *

* q) exit                  *

*                          *

****************************

Enter selection:

Please select Item 5th (28F010 Compatible) for Selection.

c. After step b. The system will ask user to enter NEW BIOS file, then

please enter the 032295.rom

5. After Update BIOS is completed, please turn off the power and jump

JP19 = 1-2 , JP = 1-2 again, then all action for Update BIOS is done.

!! IMPORTANT: Do not power off or reset the computer while the BIOS updating

operation is in progress. If you do so, you will be unable to

start your system again.