MS-4144 WF53S BIOS Release

--------------------------

NEW FUNCTION OF THIS BIOS:

1.  Supports the new INTEL DX4-100 WB (&EW) CPU.

2.  This BIOS does not support PS/2 mouse.

Note:   This is a Plug 'n Play BIOS, and has to be used with SST or Winbond

Flash Memory.  If this BIOS is programmed into EPROM or other brands

of Flash Memory, there will be a problem with Windows 95 RESTART

function.

How to use the AWDFL301.EXE Utility

-----------------------------------

This utility is used to program a new BIOS into a Flash memory.  You have to

make sure first that the BIOS installed in your mainboard is Flash memory,

and not EPROM.

1.  Turn on your computer.

2.  Boot up clean, bypassing your AUTOEXEC.BAT and CONFIG.SYS files.

3.  Then, run the AWDFLASH.EXE file.

C:\>AWDFL301

The programming screen will be shown.  You will prompt to enter the file

name:

File name to program:  WF53S.BIN

At the bottom of the screen, the following message will appear:

Do you want to save BIOS (Y/N)?

Press "N" if you do not want to save the old BIOS.

Press "Y" if you want to save the old BIOS.  Enter file name to save.

File name to save: __________

After saving the old BIOS, the following message will appear at the

bottom of the screen:

Are you sure to program (Y/N)?

Press "N" if you do not want to program the new BIOS.

Press "Y" if you want to continue programming the new BIOS.  The new BIOS

will then be programmed.  When it has finished programming, the following

message will appear:

Flash EPROM Program Successful.

You have successfully programmed the new BIOS.