Sounding Board (tm) Sound Information File (SOUND.DOC)

----------------------------------------------

(C) 1992 ARTISOFT, Inc.

(Revised 7/27/92)

Your diskette contains--

Windows Driver Files:

VASBID.386       Windows enhanced mode driver

SNDBRD.DRV       Windows standard mode driver

SASBID.DLL       Windows Sounding Board driver

ASBCVT.DLL       Used by the Sounding Board driver

Windows ArtiSound Files:

ASR.EXE          ArtiSound (tm) software for playing and recording

sound while running under Windows.

ASR.HLP          Used by ArtiSound to display helpful information.

Windows Install Files:

INSTALL.EXE      Used to install ArtiSound and/or the Sounding

Board Driver.

REGLOAD.EXE      Used by the INSTALL.EXE.

INI.REG          Used by the INSTALL.EXE.

INSTAL1.INF      Used by the INSTALL.EXE.

INSTAL2.INF      Used by the INSTALL.EXE.

OEMSETUP.INF     Used by Windows to install the Sounding Board

driver.

Windows Programmers Files:

ASBCVT.LIB       Library of routines to convert between different

Sound file formats.

ASBCVT.H         Include file to be used with ASBCVT.LIB.

ASNDBRD.H        Include file to be used to interface with the

Sounding Board driver.

DOS Program Files:

HTEST.EXE        Diagnostic program for the handset

SOUNDBD.EXE      Driver software for the Sound adapter

LTEST.EXE        Diagnostic program for the RCA in/out jacks

RECORD.EXE       Program for testing the adapter's recording

functions for both the RCA input jack and handset.

SAY.EXE          Program for testing the adapter's playing

functions for both the RCA input jack and handset.

DOS Data Files:

ARTIBLAB.SPK     Sample message file for DOS.

ARTIBLAB.WAV     Sample message file for Windows.

TEST.DAT         Test data for HTEST.EXE and LTEST.EXE

.DOC Files:

SOUND.DOC        This file

ASBTEST.DOC      Documention for using HTEST.EXE, LTEST.EXE,

RECORD.EXE and SAY.EXE

DOS Install Files:

PCSETUP.BAT      File to copy the necessary DOS files to the

specified directory.

Thank you for purchasing the Sounding Board (tm) adapter. The printed

manual enclosed with your adapter describes software and hardware

installation in detail.

Four executable programs have been included on the distribution disk

which are not described in the manual. HTEST, LTEST, RECORD, and SAY

are primarily for diagnostic purposes and their operation is described in

ASBTEST.DOC. SOUNDBD.EXE is the only file which you must copy for

the sound adapter to function. You may find SAY and RECORD to be useful

as well.

Remember to make a backup of your distribution diskette before you begin

installation. Also, fill out and return your registration form, so that

you will be notified of upgrades, etc.

ADDENDUM TO USER'S MANUAL:

One compatibility problem has been noted that occurs when the sound

drivers are used on certain '386 machines with '286 BIOS (specifically,

Everex 386 machines). When these computers are used with background

software (specifically, disk-caching programs) which use extended

memory, the DMA channel page register values are overwritten during

the extended memory move operations. Since the sound adapter uses

DMA concurrently with the caching mechanism, Sound DMA is redirected

to incorrect memory addresses, resulting in random noise at the

earpiece during playback, and in system crashes during record operations.

The DMA page register appears to be stepped on during the switch into

protected mode, which in '286 BIOS is accomplished by resetting the

processor.

There is no real fix to this problem, other than correcting the

Everex motherboard implementation, using a '386 BIOS to avoid

the RESET, or replacing the BIOS "Move Extended Memory" function

with a TSR that implements it correctly.

Until such a fix is available, a switch has been added to the SOUNDBD

driver program which partially overcomes the problem. Simply install

with the keyword "FIX_DMA_PAGE" switch on the command line, as in:

SOUNDBD VERBOSE FIX_DMA_PAGE

This will cause sound activity to be temporarily suspended during

extended memory moves. The result will be noticable "glitches" in

playback and recording, but no noise or system crashes. You should

NOT use this switch unless you have the problem, as unnecessary

glitches will be introduced.

USING MC-SOUND FROM A DOS VIRTUAL MACHINE

=========================================

The SAY.EXE and RECORD.EXE programs cannot be run from a DOS virtual

machine. The low level driver is disabled when Windows starts. Upon

exit from Windows, the driver is re-enabled and can be used from DOS.

Use ArtiSound to play and record voice files when in Windows.

ADDITIONAL INFORMATION FOR PROGRAMMERS

======================================

Artisoft's device identification number has changed.  It is reflected in

the ASNBRD.H file.

ArtiSound now supports the extended wave format structure. This new

structure is defined as follows:

typedef struct extended_waveformat_tag {

WAVEFORMAT wf;

WORD  wBitPerSample;

WORD  cbExtraSize;

} EXTWAVEFORMAT;

Additional information on the new RIFF format is available from Compuserve

WINSDK forum or the Microsoft Multimedia BBS.