Running Dos Applications with the Tropez Plus

Description of Shortcuts:

The following shortcuts were copied to your Windows 95 desktop during the installation of the Tropez Plus.  The three new icons may be hidden behind existing applications or shortcuts, so you may have to move them aside to find the .PIF's (Program Information Files) below:

95dosmix:

This shortcut is used to reset the DOS volume levels when running DOS applications

(i.e. games) through a Windows 95 DOS box.  If you notice low sound effects or MIDI

volume, quit the application, return to the Windows 95 desktop, and run the 95dosmix

shortcut.  Try the application again and the levels should be fine.

Trop_dos: (referred to as Tropez_P in the manual)

This shortcut is used to enable the Tropez Plus in MS-DOS Mode.  Certain DOS

applications and games cannot be run through Windows 95 and will only work in

MS-DOS Mode.  In order for you to retain the audio functions of the Tropez Plus

in MS-DOS Mode, you must use a .PIF that will tell DOS how to initialize the card.

A custom Config.sys and Autoexec.bat are executed when the .PIF is run.  Detailed

instructions for configuring the .PIF are included in Appendix B of your Tropez Plus

manual.

Trop_cd:

This shortcut is similar to the Trop_dos, but also includes lines to enable a Mitsumi

4X CD-ROM drive in MS-DOS Mode.  The Mitsumi is shown as an example only.

You may insert your own DOS drivers for your CD-ROM drive using the instructions

in Appendix B.

TBConfig:

This shorcut will allow you to edit the Tbsdos.ini file to customize your system for MS-

DOS Mode.  Notes are provided in the Appendix B of your manual to assist you with 	changing the settings.  To view your settings, right click on the 'My Computer' icon and 	choose 'Sound, Video, and Game controllers' under the 'Device Manager' tab.  The 	'Resources' tab on the installed devices will show where Windows 95 has configured the 	card.

Note:  	This shortcut assumes that your boot drive is drive C:.  If your TBEACH/ 		DOSAPPS directory was copied to a different drive letter, you will need to

update the Tbsdos.ini shortcut before you can use it.

For further release notes on the Tropez Plus, view the README.DOC file from the 'Start Menu,' 'Programs,' 'Tropez Plus' section.