The CTS Serial Port Utilities (TM)

User Guide

(c) Copyright 1993-1996 Computer Telecommunication Systems, Inc.

All Rights Reserved.

No part of this User Guide may be reproduced, transmitted,

transcribed, stored in a retrieval system, or translated into any

language (natural or computer), in any form or by any means,

without the prior written permission of Computer

Telecommunication Systems, Inc. See the accompanying files

OEM.DOC, VENDOR.DOC, LICENSE.DOC, and SYSOP.DOC for specific

information pertaining to distribution of this shareware version

of the User Guide.

Information in this User Guide is subject to change without

notice and does not represent a commitment on the part of

Computer Telecommunication Systems, Inc.

Computer Telecommunication Systems, Inc. may make improvements

and/or changes in the User Guide and/or in Serial Port Utilities

at any time.

Trademarks and Copyrights

IBM(R) and PS/2(R) are registered trademarks of International

Business Machines Corporation.

Hayes(R) is a registered trademark, and ESP(TM) is a trademark of

Hayes Microcomputer Products, Inc.

Practical Peripherals(R) is a registered trademark of Practical

Peripherals, Inc.

The CTS Serial Port Utilities(TM), IRQInfo(TM) SwapIRQ(TM), and

AT Command Test(TM) are trademarks of Computer Telecommunication

Systems, Inc.

T/Port(TM) and Telcor(TM) are trademarks of Telcor Systems

Corporation.

Other names listed in this User Guide are the proprietary

trademarks of their respective companies.

Computer Telecommunication Systems, Inc.

3847 Foxwood Road, Suite 1000

Duluth, Georgia  30136-6100 USA

1-800-380-2666 (Orders)		             CompuServe: 76662,2315

1-770-263-8623 (Voice)             Internet: sales@compuserve.com

1-770-263-0124 (FAX)      World Wide Web: http://www.comminfo.com

ii

TABLE OF CONTENTS

Registration Options  . . . . . . . . . . . . . . . . . . . .  vi

Corporate  . . . . . . . . . . . . . . . . . . . . . . .  vi

Personal . . . . . . . . . . . . . . . . . . . . . . . .  vi

Personal PLUS  . . . . . . . . . . . . . . . . . . . . .  vi

Professional . . . . . . . . . . . . . . . . . . . . . . vii

Professional PLUS  . . . . . . . . . . . . . . . . . .  viii

Acknowledgments . . . . . . . . . . . . . . . . . . . . . . . xii

1.0 Introducing The CTS Serial Port Utilities . . . . . . . .   1

PORTINFO . . . . . . . . . . . . . . . . . . . . . . . .   2

BIOS_FIX . . . . . . . . . . . . . . . . . . . . . . . .   3

BUFFER . . . . . . . . . . . . . . . . . . . . . . . . .   3

COM_BPS  . . . . . . . . . . . . . . . . . . . . . . . .   3

COM_FMT  . . . . . . . . . . . . . . . . . . . . . . . .   4

DOS_COM  . . . . . . . . . . . . . . . . . . . . . . . .   4

DOS_SWAP . . . . . . . . . . . . . . . . . . . . . . . .   4

DTR  . . . . . . . . . . . . . . . . . . . . . . . . . .   4

FIND_COM . . . . . . . . . . . . . . . . . . . . . . . .   4

HANGUP . . . . . . . . . . . . . . . . . . . . . . . . .   4

IRQ  . . . . . . . . . . . . . . . . . . . . . . . . . .   4

RESETCOM . . . . . . . . . . . . . . . . . . . . . . . .   5

RESETMOD . . . . . . . . . . . . . . . . . . . . . . . .   5

RTS  . . . . . . . . . . . . . . . . . . . . . . . . . .   5

2.0 Short Instructions  . . . . . . . . . . . . . . . . . . .   7

Installation . . . . . . . . . . . . . . . . . . . . . .   7

Quick-Start  . . . . . . . . . . . . . . . . . . . . . .   7

3.0 The Command Line  . . . . . . . . . . . . . . . . . . . .   9

Serial Port Number Switches  . . . . . . . . . . . . . .  10

On / Off Switches  . . . . . . . . . . . . . . . . . . .  10

Parameter Switches . . . . . . . . . . . . . . . . . . .  10

Option Switches  . . . . . . . . . . . . . . . . . . . .  11

Default Values . . . . . . . . . . . . . . . . . . . . .  11

4.0 Defining Non-Standard Serial Ports  . . . . . . . . . . .  13

5.0 Testing The System With PortInfo  . . . . . . . . . . . .  15

Test All Serial Ports - /0 . . . . . . . . . . . . . . .  16

Test One Serial Port - /n, n<>0  . . . . . . . . . . . .  16

BIOS Update - /B . . . . . . . . . . . . . . . . . . . .  17

Quick Test - /Q  . . . . . . . . . . . . . . . . . . . .  17

Log File - /L, /LA, /LP  . . . . . . . . . . . . . . . .  17

Force a Mouse Reset on PortInfo Exit - /MR . . . . . . .  18

Short (or Summary) Report - /S . . . . . . . . . . . . .  18

Skip the IRQ Scan - /NoScan  . . . . . . . . . . . . . .  18

Search for ESP port - /ESP . . . . . . . . . . . . . . .  18

Advanced PortInfo Options  . . . . . . . . . . . . . . .  18

iii

/Batch  . . . . . . . . . . . . . . . . . . . . . .  19

/LT . . . . . . . . . . . . . . . . . . . . . . . .  19

/Modem  . . . . . . . . . . . . . . . . . . . . . .  19

/ModemID  . . . . . . . . . . . . . . . . . . . . .  19

/NoModem  . . . . . . . . . . . . . . . . . . . . .  19

/NoMouse  . . . . . . . . . . . . . . . . . . . . .  19

/NoWin  . . . . . . . . . . . . . . . . . . . . . .  19

/Plug . . . . . . . . . . . . . . . . . . . . . . .  20

/Win <Path> . . . . . . . . . . . . . . . . . . . .  20

PORTINFO.CFG File . . . . . . . . . . . . . . . . .  20

Network Considerations . . . . . . . . . . . . . . . . .  20

The PortInfo Summary Screen  . . . . . . . . . . . . . .  22

Port Configuration  . . . . . . . . . . . . . . . .  23

Port Address . . . . . . . . . . . . . . . . .  23

Interrupt (IRQ)  . . . . . . . . . . . . . . .  23

IRQ (Out2) Enabled . . . . . . . . . . . . . .  23

DOS uses port as . . . . . . . . . . . . . . .  24

Device Detected  . . . . . . . . . . . . . . .  24

FAX Service Class  . . . . . . . . . . . . . .  24

Type of Port . . . . . . . . . . . . . . . . .  25

Buffer - Trigger . . . . . . . . . . . . . . .  26

Port Settings . . . . . . . . . . . . . . . . . . .  27

Speed (BPS)  . . . . . . . . . . . . . . . . .  27

Bits / Character . . . . . . . . . . . . . . .  27

Parity . . . . . . . . . . . . . . . . . . . .  27

Stop Bits  . . . . . . . . . . . . . . . . . .  27

Hardware Signals  . . . . . . . . . . . . . . . . .  28

Clear to Send  . . . . . . . . . . . . . . . .  28

Data Set Ready . . . . . . . . . . . . . . . .  29

Ring Indicator . . . . . . . . . . . . . . . .  29

Data Carrier Detect  . . . . . . . . . . . . .  29

Data Terminal Ready  . . . . . . . . . . . . .  30

Request to Send  . . . . . . . . . . . . . . .  30

6.0 The Other Utilities . . . . . . . . . . . . . . . . . . .  31

BIOS_FIX . . . . . . . . . . . . . . . . . . . . . . . .  31

BUFFER . . . . . . . . . . . . . . . . . . . . . . . . .  31

COM_BPS  . . . . . . . . . . . . . . . . . . . . . . . .  32

COM_FMT  . . . . . . . . . . . . . . . . . . . . . . . .  32

DOS_COM  . . . . . . . . . . . . . . . . . . . . . . . .  32

DOS_SWAP . . . . . . . . . . . . . . . . . . . . . . . .  33

DTR  . . . . . . . . . . . . . . . . . . . . . . . . . .  33

Find_COM . . . . . . . . . . . . . . . . . . . . . . . .  33

HANGUP . . . . . . . . . . . . . . . . . . . . . . . . .  36

IRQ  . . . . . . . . . . . . . . . . . . . . . . . . . .  36

RESETCOM . . . . . . . . . . . . . . . . . . . . . . . .  37

RESETMOD . . . . . . . . . . . . . . . . . . . . . . . .  37

RTS  . . . . . . . . . . . . . . . . . . . . . . . . . .  37

7.0 PortInfo FAILURE and WARNING Messages . . . . . . . . . .  39

Port Failures  . . . . . . . . . . . . . . . . . . . . .  39

Message PI-1000 . . . . . . . . . . . . . . . . . .  39

iv

Interrupt Conflicts  . . . . . . . . . . . . . . . . . .  39

Message PI-2010 . . . . . . . . . . . . . . . . . .  40

Message PI-2020 . . . . . . . . . . . . . . . . . .  40

Message PI-2100 . . . . . . . . . . . . . . . . . .  40

Message PI-2110 . . . . . . . . . . . . . . . . . .  41

You have multiple ports at the same address  .  41

You have another device using the same

interrupt . . . . . . . . . . . . . . . .  42

The system will not let you share this

interrupt . . . . . . . . . . . . . . . .  42

Message PI-2200 . . . . . . . . . . . . . . . . . .  43

Message PI-2300 . . . . . . . . . . . . . . . . . .  43

Message PI-2400 . . . . . . . . . . . . . . . . . .  43

Message PI-2500 . . . . . . . . . . . . . . . . . .  44

Port Conflicts . . . . . . . . . . . . . . . . . . . . .  44

Message PI-3000 . . . . . . . . . . . . . . . . . .  45

Message PI-3100 . . . . . . . . . . . . . . . . . .  45

Message PI-3200 . . . . . . . . . . . . . . . . . .  45

Message PI-3500 . . . . . . . . . . . . . . . . . .  46

Mouse Conflicts  . . . . . . . . . . . . . . . . . . . .  46

Message PI-4000 . . . . . . . . . . . . . . . . . .  46

DOS Port Assignments . . . . . . . . . . . . . . . . . .  47

Message PI-5000-5040  . . . . . . . . . . . . . . .  47

Message PI-5050 . . . . . . . . . . . . . . . . . .  47

Other CTS Software Detected  . . . . . . . . . . . . . .  48

Message PI-6000 . . . . . . . . . . . . . . . . . .  48

Windows Communication Setup  . . . . . . . . . . . . . .  48

Message PI-7000 . . . . . . . . . . . . . . . . . .  48

Message PI-7050 . . . . . . . . . . . . . . . . . .  48

Message PI-7100 . . . . . . . . . . . . . . . . . .  49

Message PI-7200 . . . . . . . . . . . . . . . . . .  49

Message PI-7300 . . . . . . . . . . . . . . . . . .  49

8.0 PCMCIA (Card Modem) Considerations  . . . . . . . . . . .  49

9.0 Customer Support Policy . . . . . . . . . . . . . . . . .  51

10.0 About Computer Telecommunication Systems . . . . . . . .  53

OEM and Custom Programming Services  . . . . . . . . . .  53

Serial Port and Modem Installation Software . . . .  53

Custom "Pre-communication" testing  . . . . . . . .  53

IRQInfo  . . . . . . . . . . . . . . . . . . . . . . . .  53

SwapIRQ  . . . . . . . . . . . . . . . . . . . . . . . .  54

PortFix  . . . . . . . . . . . . . . . . . . . . . . . .  54

AT Command Test  . . . . . . . . . . . . . . . . . . . .  54

AT Command Test for EIA-602  . . . . . . . . . . . . . .  54

New Products . . . . . . . . . . . . . . . . . . . . . .  55

Appendix A: BONUS utilities MouseOFF and MouseON  . . . . . .  57

Appendix B: Introduction To Serial Ports  . . . . . . . . . .  59

What is a serial port? . . . . . . . . . . . . . . . . .  59

v

Serial Port Addresses for PC/XT/AT and Compatible

Computers . . . . . . . . . . . . . . . . . . . . .  60

Serial Port Addresses for MicroChannel Architecture

Computers . . . . . . . . . . . . . . . . . . . . .  60

Interrupts (IRQs)  . . . . . . . . . . . . . . . . . . .  61

Why are the DOS serial port assignments sometimes

different from one application to another?  . . . .  63

Appendix C: Types of Serial Ports . . . . . . . . . . . . . .  65

The 8250 and 8250B . . . . . . . . . . . . . . . . . . .  65

The 8250A  . . . . . . . . . . . . . . . . . . . . . . .  66

The 16450  . . . . . . . . . . . . . . . . . . . . . . .  66

The 16550  . . . . . . . . . . . . . . . . . . . . . . .  66

The 16550A . . . . . . . . . . . . . . . . . . . . . . .  66

The 16552  . . . . . . . . . . . . . . . . . . . . . . .  67

The StarTech 16650 UART  . . . . . . . . . . . . . . . .  67

Texas Instruments 16750 UART . . . . . . . . . . . . . .  67

The T/Port . . . . . . . . . . . . . . . . . . . . . . .  67

Emulated 16450 UARTs . . . . . . . . . . . . . . . . . .  67

Hayes ESP-I and ESP-II . . . . . . . . . . . . . . . . .  68

Practical Peripherals HSSP . . . . . . . . . . . . . . .  68

Appendix D: Sharing Interrupts (IRQs) . . . . . . . . . . . .  69

Appendix E: Using IRQ 2 and IRQ 9 . . . . . . . . . . . . . .  71

Appendix F: Avoiding or Reducing Overrun Errors . . . . . . .  73

What is an Overrun Error?  . . . . . . . . . . . . . . .  73

Why Overrun Errors Occur . . . . . . . . . . . . . . . .  73

Eliminating Overrun Errors . . . . . . . . . . . . . . .  75

Additional Questions about Overrun Errors  . . . . . . .  76

Appendix G: The EIA-232 Interface . . . . . . . . . . . . . .  77

EIA-232 Signalling Standards . . . . . . . . . . . . . .  78

EIA-232 Signal Lines Used in Normal Modem

Communications  . . . . . . . . . . . . . . . . . .  78

Signal Ground (GND) . . . . . . . . . . . . . . . .  78

Transmitted Data (TXD)  . . . . . . . . . . . . . .  78

Receive Data (RXD)  . . . . . . . . . . . . . . . .  78

Request To Send (RTS) . . . . . . . . . . . . . . .  79

Clear to Send (CTS) . . . . . . . . . . . . . . . .  79

Data Set Ready (DSR)  . . . . . . . . . . . . . . .  79

Data Terminal Ready (DTR) . . . . . . . . . . . . .  79

Ring Indicator (RI) . . . . . . . . . . . . . . . .  79

Carrier Detect (CD) . . . . . . . . . . . . . . . .  79

Table of Modem Signals . . . . . . . . . . . . . . . . .  80

CTS Tech Note 1: Serial Port and Modem Installation Guide . .  81

CTS Tech Note 2: Trouble-Shooting Serial Ports  . . . . . . .  87

vi

Registration Options

====================

Several different registration options are available:

** Corporate **

---------------

Multi-Copy and OEM license agreements are available for any or all of

the utilities included in the CTS Serial Port Utilities. Please see

OEM.DOC or contact CTS, Inc. for details.

** Personal **

--------------

This is the most economical registration for a single user. If you need

to use (or used) the SPU to diagnose a problem with your personal

computer, then this is the license for you. By registering, you obtain

personalized support to assist you with trouble-shooting your computer.

Details are provided in the User Guide section titled "Customer Support

Policy". As part of this license, you also receive:

*  Additional features in PortInfo includes the Windows Setup

Verification test that makes sure that Windows is setup correctly.

PortInfo suggests specific changes if any are required.

*  BIOS_FIX includes the Windows Setup Verification test.

*  30 minutes of personal assistance with your serial port problems.

*  The most current version of the CTS Serial Port Utilities.

*  A FREE CompuServe IntroPak which gets you online with CompuServe

without the regular signup fee plus a $15 usage credit.

*  Announcements about updates.

*  Special savings on other products from CTS. Details are listed just

before the order form!

The license provided with this option allows the registered user to use

the SPU on any computer personally owned or used exclusively by the

registered user.

** Personal PLUS **

-------------------

This registration option provides all of the benefits above plus:

*  A new step-by-step guide to installing a modem. This informative

document will help you make the right decisions about organizing

your serial ports and IRQs when you are installing or upgrading

your modem.

*  The serial port troubleshooting guide. This helpful document will

help you resolve the most common types of serial ports problems.

vii

*  A printed copy of the attractive 96 page, 8.5 x 11" User Guide,

including 30 pages of introductory and reference information not

included in the electronic manual. This additional information

includes serial port basics, types of serial ports, when and why

you need a 16550A UART, how and when you can share interrupts

between serial ports, details of the RS-232 interface, cable

wiring, and how IRQ 2 and IRQ 9 are used.

*  60 minutes of personal assistance with your serial port problems.

*  The MouseOn and MouseOff utilities. The "mouse utilities" allow a

mouse and a modem to share the same interrupt when running under

DOS.

The license provided with this option allows the registered user to use

the SPU on any computer personally owned or used exclusively by the

registered user.

** Professional **

------------------

This license is for consultants and corporate support specialists who

routinely deal with a multiple computers. Treat the SPU like your

screwdriver - that is, as long as it is in your possession and not

"left" (installed) at any other machine, you are in compliance with the

license. Only one individual is authorized to call for support.

This license option provides all of the benefits described above, plus:

*  A number of new "advanced options" are available in the PortInfo

utility designed specifically for the technical user. These include

the ability to create fully automated batch files to operate

PortInfo, a configuration file which allows operation without

command line switches, and the ability to force or suppress

different tests. See the "Advanced PortInfo" section for more

details.

*  The bonus utility "FindCOM" which locates any serial port installed

in the computer system. This utility does a "safe search" of the

I/O address area to find any 8250 compatible serial port at *any*

address. A very handy utility for those "unknown" machines.

*  90 minutes of technical support (one authorized caller per license)

*  Allows use of the SPU on multiple computers AS LONG AS the SPU is

ONLY used on ONE COMPUTER AT A TIME. Site licenses are available

for situations where multiple installed copies are required.

viii

** Professional PLUS **

-----------------------

Includes everything in the Professional License, PLUS:

*  A "toolkit copy" of the CTS SwapIRQ(tm) utility. SwapIRQ is our

$20 (retail) DOS utility that lets DOS applications use any IRQ

supported by a serial port or modem. A very handy utility for those

situations where an IRQ conflict could be resolved *if* a software

application could just use a different IRQ. Now it can!

SwapIRQ is a TSR which uses 784 bytes of memory and redirects

hardware-generated IRQs to the IRQ the software uses. SwapIRQ runs

in DOS, or in a DOS box under Windows and OS/2.

Note: each installed copy of SwapIRQ requires an additional

license. Multicopy discounts and site licenses are available if

required. Contact CTS for details.

*  A "toolkit copy" of the CTS PortFix utility ($10). PortFix is

another little DOS TSR utility (only 704 bytes!) which eliminates

lockups caused by some faulty UARTs shipped with PCI computers

during late 1993 and 1994. These faulty UARTs have also been used

in some addin cards...if you are experiencing occasional lockups,

particularly on the "second use" of the port, then this utility is

worth a shot!

ix

IRQInfo(tm)

-----------

This is a comprehensive Interrupt Mapping utility. IRQInfo detects

nearly all common devices and accurately displays their IRQs.

IRQInfo includes tests for serial and parallel ports, sound cards,

scanners, the mouse, hard and floppy disk drives, network cards,

SCSI devices...just about everything that uses an IRQ. Due to the

type of testing performed by IRQInfo, this utility runs in DOS only.

Pricing: Normal price is $28.95 (including S&H), but it is available

for only $24 when ordered with SwapIRQ or any CTSSPU license.

IRQInfo PRO (tm)

----------------

The professional version of IRQInfo includes support for even more

hardware devices that use IRQs and includes an integrated TSR module

that watches computer while you use it to determine if any of the

unused IRQs have a "hidden device" using them. IRQInfo PRO includes

a full color animated display as devices are detected, and

continually "learns" about your system as long as the TSR module is

loaded. NOTE: TSR module uses only 512 bytes.

Pricing: $35

SwapIRQ

-------

This utility is described in the Professional PLUS version. Normally

$24 (including shipping) but you can order it now for only $20.

PortFix

-------

This utility is described in the Professional PLUS version. Normally

$14 (including shipping) but you can order it now for only $10.

How to Pay for your order

=========================

We now accept Visa, MasterCard, American Express and Discover credit

cards, personal and business checks from US banks, money orders and we

can ship 2nd day AIR COD in the US (please add $10 for COD & increased

shipping charges, and be sure to write "COD" on the order form.) We

accept EuroCheqs, American Express and International Postal money orders

for our international customers.

We also accept purchase orders from established customers, Fortune 1000

corporations, accredited schools and US government agencies. Please note

that there is a $5 service fee on purchase orders for less than $50.

You may also order on CompuServe through the SWREG area - but be aware

that your handling charges are much higher when using SWREG.

The CTS Software Utilities Order Form

Name: ________________________________  Telephone: _____________________

Company: ________________________________   E-mail ID: ____________________

Address: __________________________________________________________________

__________________________________________________________________

__________________________________________________________________

QTY   Description                               Price           Subtotal

===   ===========                               ======          ========

The CTS Serial Port Utilities (CTSSPU)

___          Personal License . . . . . . . . . $20.00          ________

___          Personal PLUS License  . . . . . .  35.00          ________

___          Professional License . . . . . . .  59.00          ________

___          Professional PLUS License  . . . .  79.00          ________

___   SwapIRQ . . . . . . . . . . . . . . . . .  20.00          ________

___   PortFix . . . . . . . . . . . . . . . . .  10.00          ________

___   IRQInfo . . . . . . . . . . . . . . . . .  24.00          ________

___   IRQInfo PRO . . . . . . . . . . . . . . .  35.00          ________

========

Sub-Total     $________

Sales Tax:         State of Georgia residents add 5%     $________

Shipping and Handling:  USA/Canada, all products   $ 4.00

2nd Day COD (USA Only)   $15.00

International (CTSSPU Personal PLUS, PRO or PRO PLUS)    10.00

International (all other products)     4.00  $________

TOTAL (US dollars, please)   $________

PAYMENT METHOD:  Check __    Money Order __    EuroCheq __     Discover __

Visa __    MasterCard __    Am. Express __    Purchase Order ($50 min) __

========================================================================

CREDIT CARD INFORMATION

Name as it appears on the card: _________________________________

Card No. ________  ________  ________  ________  Expires  ___/___

Cardholder signature: ___________________________________________

Please send to, FAX, Email or call:

+--------------------------------------------------------------------+

|              Computer Telecommunication Systems, Inc.              |

|                   3847 Foxwood Road, Suite 1000                    |

|                  Duluth, Georgia  30136-6100 USA                   |

+--------------------------------+-----------------------------------+

|           Telephone            |          Electronic Mail          |

|           ---------            |          ---------------          |

|   Orders: 1-800-380-2666       |  Internet:sales@comminfo.com      |

|    Voice: 1-770-263-8623       |     or    http://www.comminfo.com |

|      FAX: 1-770-263-0124       |  CompuServe: 76662,2315           |

+--------------------------------+-----------------------------------+

Federal TaxID: 58-1880693. Prices are subject to change without notice.

xi

DISCLAIMER OF WARRANTY

USERS OF THIS SOFTWARE MUST ACCEPT THIS DISCLAIMER OF WARRANTY:

THIS SOFTWARE IS SUPPLIED AS IS. COMPUTER TELECOMMUNICATION

SYSTEMS DISCLAIMS ALL WARRANTIES, EITHER EXPRESS OR IMPLIED,

INCLUDING BUT NOT LIMITED TO IMPLIED WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE, WITH

RESPECT TO THE SOFTWARE AND THE ACCOMPANYING WRITTEN MATERIALS.

NO LIABILITY FOR CONSEQUENTIAL DAMAGES. IN NO EVENT SHALL

COMPUTER TELECOMMUNICATION SYSTEMS OR ITS SUPPLIERS BE LIABLE FOR

ANY DAMAGES WHATSOEVER (INCLUDING, WITHOUT LIMITATION, DAMAGES

FOR LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS OF

BUSINESS INFORMATION, OR OTHER PECUNIARY LOSS) ARISING OUT OF THE

USE OF OR INABILITY TO USE THIS COMPUTER TELECOMMUNICATION

SYSTEMS PRODUCT, EVEN IF COMPUTER TELECOMMUNICATION SYSTEMS HAS

BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES. BECAUSE SOME

STATES DO NOT ALLOW THE EXCLUSION OR LIMITATION OF LIABILITY FOR

CONSEQUENTIAL OR INCIDENTAL DAMAGES, THE ABOVE LIMITATION MAY NOT

APPLY TO YOU.

xii

Association Of Shareware Professionals Member

=============================================

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

This program is produced by a member of the Association of

Shareware Professionals (ASP). ASP wants to make sure that the

shareware principle works for you. If you are unable to resolve a

shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help. The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but

does not provide technical support for members' products.  Please

write to the ASP Ombudsman at 545 Grover Road, Muskegon, MI

49442-9427 USA, FAX 616-788-2765 or send a CompuServe message via

CompuServe Mail to ASP Ombudsman 70007,3536.

Acknowledgments

I wish to express my very special thanks to David Bittner, Ted

DeCastro, Wayne Fulton, Dan Landiss, Earle Robinson and Tom Wason

who made this application possible through their dedicated beta

testing. The CTS Serial Port Utilities would not have the same

features or reliability as they do without the many hours of

testing invested by these individuals.

In addition, there were individuals too numerous to list who

tested at least one version of the SPU and reported back

suggestions or verified that known problems were fixed. To each

of you, my thanks!

1

1.0 Introducing The CTS Serial Port Utilities

Serial ports (also known as comm ports) are painless when they

work, and a pain in the neck when they don't. They have plenty of

options and limitations. They need a unique address, an inactive

IRQ, and software which is properly configured to use it. No

wonder nearly everyone gets confused by serial ports.

The CTS Serial Port Utilities (SPU) is a collection of DOS

software utilities that provide you with control over the serial

ports in your computer. Most serial port problems can be resolved

by using these utilities. Each of the utilities support all

standard and any user-defined serial ports. User-defined ports

are defined in the command line or in the system environment

variables.

Why "Oh, NO! NOT ANOTHER?" serial port utility? Well, several

good reasons come to mind. The first is that there were a lot of

things still not done by other utilities. Can you name one which

detects multiple serial ports installed at the same address

(other than the SPU?) Or one that tells you when the monitor will

cause problems if COM4 is used? Want to know what the current

receiver trigger level is in the 16550A serial port? Better use

the SPU program PortInfo to find out! Nothing else out there

today will tell you!

The SPU even detects several types of "enhanced" serial ports,

including Telcors' T/Port , the Hayes ESP cards (both types), and

the Practical Peripherals HSSP. PortInfo even identifies some

internal modems that have "emulated UARTs". Also new in this

version is the ability to identify which ports have active

modems, and sometimes we can even identify the modem as internal

or external. FAX modems are also identified, with the FAX

class(es) supported listed.

More reasons? We spent a lot of time reviewing the utilities that

were out before SPU, and didn't find one that was both good and

full-featured. There were a few that were very good for limited

applications, but didn't properly test IRQ assignments, or were

limited in the serial ports they supported, or....well, you get

the idea. It just seemed like it was time for a common set of

utilities that tied everything together.

So the CTS Serial Port Utilities came into being. The utilities

all share a common command line interface, and they all use a

common set of environment variables you can set to define non-

standard serial ports. The SPU contains the following programs:

2

1.1 PORTINFO

PortInfo is a comprehensive utility which tests the computer

system to determine everything possible about your serial

ports, including interrupt and address conflicts. PortInfo

provides you with FAILURE and WARNING messages about actual

and possible conflicts.

PortInfo provides details on the following:

*   Multiple serial ports at the same address (eg, 2 Com1

ports)

*   Interrupt conflicts between a bus mouse and a serial

port

*   Shared interrupts (2 serial ports, or a serial port and

a serial mouse sharing an IRQ)

*   The local loopback test accurately identifies failed

serial ports that have failed

*   Conflicts between COM4 and 8514/A compatible monitors

*   Unusual default serial port setups (BIOS and DOS port

assignments)

*   Serial ports that cannot generate interrupts

*   Helps to determine when interrupts can be shared

*   Identifies which port(s) have active modems, and may

determine if the modem is internal or external.

*   FAXmodems are identified, with the FAX classes the

modem reports supporting included on the summary

report.

*   PortInfo can also set the system BIOS to match the

actual ports installed.

Other information provided:

*   Actual IRQ used by each port and the current IRQ status

*   DOS port assignments (com1-com4, mouse)

*   Port (UART) Identification (8250, 8250A/16450, 16550,

16550A, 16650, 16750, the T/Port, Hayes ESP (both

types), the Practical Peripherals HSSP, and some

"emulated UARTs"

*   FIFO status: If it exists, on / off, else 'None'

*   If the FIFO is ON, the current FIFO receiver trigger

level is displayed.

*   All programmable port parameters: speed, format,

parity, current RTS and DTR settings

*   Current status of modem lines: Ring Indicator, Carrier

Detect, Clear to Send, Data Set Ready, with indication

of when the lines are tied together (looped back).

3

Other features:

*   Ports are COMPLETELY restored to their original

settings

*   Supports standard serial ports 1-4 on PC/AT, 1-8 on

PS/2

*   Supports up to 8 user definable (port address and IRQ)

serial ports

*   Displays detailed messages about conflicts with

reference numbers to assist looking up the detailed

assistance in the section titled "PortInfo FAILURE and

WARNING Messages".

1.2 BIOS_FIX

The BIOS in many computer systems will not accurately detect

the serial (Com) ports in the computer. Many will never

detect Com3 or Com4, a few will fail to detect one or more

ports after a control-alt-del, and nearly all will

inaccurately the ports if they are not installed in sequence

(for example, Com4 will be assigned to Com3 if no real Com3

exists). This can cause any number of problems in DOS and/or

Windows operation. BIOS_FIX can be run from the AUTOEXEC.BAT

file to ensure that the serial ports are always installed

and initialized properly.

Registered versions of BIOS_FIX also test the Windows

SYSTEM.INI file to make sure that the port addresses and

IRQs are properly set for Windows operations. Any changes

that are required are displayed on the screen.

1.3 BUFFER

Buffer allows you to control the FIFO buffers in a 16550A or

compatible UART. You can turn the buffers on and off, and

set the receiver trigger level. See the section titled

"Testing Your Software with PortInfo...Buffer-Trigger" for

more information on UART FIFO receiver trigger levels.

1.4 COM_BPS

COM_BPS allows you to set a port data rate (in bits per

second, or BPS) to any standard value supported by the

serial port. This utility is normally used as a replacement

for the DOS set mode command.

4

1.5 COM_FMT

COM_FMT allows you to set a port character format to any

standard value supported by the serial port. This utility is

normally used with COM_BPS as a replacement for the DOS mode

command.

1.6 DOS_COM

You use DOS_COM to insert a serial port in the list of ports

used by DOS.

1.7 DOS_SWAP

You use DOS_SWAP to exchange two serial ports in the port

list used by DOS.

1.8 DTR

DTR gives you command line control of the Data Terminal

Ready (DTR) line of any serial port.

1.9 FIND_COM

FIND_COM performs an intelligent search of the system I/O

address space to locate serial ports that are not at

standard addresses. FIND_COM also includes a capability to

install any located ports into the BIOS data area.

NOTE: FIND_COM is provided with all Professional License

options.

1.10 HANGUP

HANGUP provides a simple utility to assist in forcing the

modem to hangup a connection by using a automatically using

different hangup techniques until the modem responds.

1.11 IRQ

IRQ allows you to enable and disable the interrupts from any

serial port.

5

1.12 RESETCOM

RESETCOM resets a port to a "power-up" configuration: Port

interrupts are disabled, data rate is set to 2400 bps,

character format to eight data bits, no parity, one stop

bit. If the port is a 16550A family device, then the buffers

(FIFOs) are disabled. This utility will often restore a port

that doesn't work after running an ill-behaved application.

1.13 RESETMOD

RESETMOD provides a simple utility to assist in forcing the

modem to respond to commands. If you have ever had to reboot

a computer to get a modem to response, then RESETMOD may be

able to help!

1.14 RTS

RTS gives you command line control of the Request to Send

(RTS) line of any serial port.

7

2.0 Short Instructions

2.1 Installation

Installation of the CTS Serial Port Utilities is easy. Just

create a subdirectory to hold all of the utilities, and then

copy the distribution diskette to the directory. Please

refer to your DOS User Guide if you need assistance with

creating a sub-directory.

2.2 Quick-Start

Each of the utilities provides a short help screen when you

enter the program name followed immediately by the return

key. This help screen includes a brief description of what

each utility does and the valid command line switches used

to control it. For example, if you wanted to see the help

screen for PortInfo, enter:

PortInfo <Enter>

After reading the help screen, you might decide to do the

basic system test with the command:

PortInfo /0 <Enter>

NOTE: This only works for standard serial port addresses.

Refer to the following documentation for information on how

to test serial ports located at any address.

Some internal modems occasionally require the computer to

have a hard reset before running PortInfo to obtain correct

results. If you observe a problem where PortInfo does not

detect an internal modem or the IRQ used by the modem, then

try using the ResetMod(em) utility before PortInfo. If it

still does not work, then either turn the computer off and

back on to perform a hard reset, or, if the computer has a

RESET button, press it. Then try running PortInfo again.

NOTE: a soft reset (control-alt-delete) is not sufficient to

reset these modems to a state where PortInfo can recognize

them. You must do a "hard reset" as described above.

9

3.0 The Command Line

The Serial Port Utilities are controlled by "command line

switches" that you enter on the command line. Command line

switches are program options that you place on the command line.

Each option is preceded by a slash character "/". These

"switches" control the SPU software the way a hardware switch

controls the options on a board.

The standard command line format is:

Program_Name /switch1 /switch2 ... <Enter>

For each of the utilities, you can enter either the program name

by itself or with a "/? command line switch" to display a brief

description of the utility and the switches used to control it.

For example, both of the following commands will display the help

screen for PortInfo:

PortInfo /? <Enter>

or

PortInfo <Enter>

where <Enter> means you should press the Enter or Return key.

There are some general rules for the command line switches:

1. You may enter the command line switches in any order UNLESS a

fixed sequence is defined for a particular utility.

2. You can enter the command line switches as uppercase or

lowercase letters.

3. You may optionally leave one space between different values in

a single command line switch.

To demonstrate, the following commands all do the same thing (it

sets the data rate to 9600 bps for COM1):

COM_BPS /1 /S9600 <Enter>

com_BPS /s 9600 /1 <Enter>

com_bps /1 /S 9600 <Enter>

com_bps /1/s9600 <Enter>

CoM_BpS/1/s9600 <Enter>

Choose a format to use that is comfortable for you. You will

receive a format error message if you enter a string the utility

doesn't understand.

10

There are four formats of switches in addition to the help

switch:

3.1 Serial Port Number Switches

Each of the utilities can work on a defined serial port

specified as a number, or on all serial ports that are

standard of defined for the CTS Serial Port Utilities (see

the next chapter for instructions on how to define a non-

standard port.)

The command line switch for port assignments is simply a "/"

(the switch command) followed by the port number. For

example, to run PortInfo on serial port one, you would

enter:

PortInfo /1 <Enter>

Specifying serial port zero (a non-existent port) will cause

the utility, to operate on all of the serial ports. For

example, to run PortInfo on all standard and user-defined

serial ports, you would enter the command:

PortInfo /0 <Enter>

3.2 On / Off Switches

Some utilities control a serial port function that is turned

on or off. For example, to turn "On" the DTR line for serial

port 2, you enter:

DTR /on /2 <Enter>       -OR-

DTR /ON /2 <Enter>       -OR-

DTR /2 /On <Enter>

Remember, they all do the same thing!

To turn a function off, you use the "/Off" switch rather

than "/On".

3.3 Parameter Switches

There are a number of these switches which you use to enter

a value. One example was shown above when we set the port

data rate to 9600 bps with the "/Sspeed" parameter switch.

Parameter switches are also used to define a non-standard

serial port, to set port formats, receive trigger levels for

a 16550A fifo, and to define a particular interrupt (IRQ) to

use. Check the help screen for each utility to see which

parameter switches are used.

11

3.4 Option Switches

These are a number of option switches used in PortInfo to

control how it operates. The switch "/Q" instructs PortInfo

to do a Quick test, and "/L" causes a Log file named

"PORTINFO.LOG" to be created containing an copy of all

messages displayed to the screen. The "/X" switch allows you

to eXclude IRQs ("/XIi") and port addresses ("/XAaaa" or

"/XAaaa-bbb") from the PortInfo test. See the Section

"Testing The System With PortInfo" for details.

3.5 Default Values

A number of the command switches have default values. All of

the programs will default to all ports (/0) if you any other

command. For example, "COM_BPS /S2400" is the same as

"COM_BPS /0 /S2400". Commands with On/Off toggles always

default to "On" - that is "DTR /0" is the same as "DTR /0

/On" or "DTR /On".

NOTE: you must specify one of the two commands to avoid

getting the help screen. Check the help screen for each of

the utilities to determine what defaults are available.

13

4.0 Defining Non-Standard Serial Ports

The CTS Serial Port Utilities will work with any serial port on

any interrupt (IRQ). However, if there are serial ports other

than the standard COM1-COM4, then you have to tell the SPU where

the ports are (that is, what address the port is installed on.)

You have two different ways to do this:

1. you can run each of the utilities using the serial port

address switch "/Aaaa", where "aaa" is the first address location

(the "base address") used by the serial port

2. you can define a system environment variable for each non-

standard serial port that will make the ports available to the

SPU via port numbers.

The CTS SPU recognizes an environment variable with the following

format:

CTSCOMn= /Aaaa

where:

"n" is the serial port number you are assigning to the Com

port

"aaa" is the first address location (the "base address") for

the serial port

NOTE: Make sure there is NO space between the com port number and

the "="

You define an environment variable with the DOS "SET" command,

which you can enter at the DOS command line or place into a batch

file (such as the AUTOEXEC.BAT file.) The DOS manual provides

documentation on the SET command.

Examples:

1. Define a port at base address 1E0 (hexadecimal) as COM5:

CTSCOM5= /A1E0

14

2. Define a port at base address 2E0 as COM6:

CTSCOM6= /a 2E0

Using these two examples, you may now access the two non-standard

serial ports as COM5 and COM6 in all utilities of the SPU. These

serial ports will also be used by any utility that is used with

the "/0" (all ports) command line.

NOTE: The CTSCOMn environment variables are not used by any other

applications except SwapIRQ, another CTS product which allows DOS

software to use any IRQ.

15

5.0 Testing The System With PortInfo

The first thing to do after installing the Serial Port Utilities

is to run PortInfo to do a full system test. If there is a mouse,

make sure that the mouse driver is loaded. PortInfo will discover

many common problems with the serial ports, including:

*  multiple ports installed at the same address

*  ports whose UARTs have failed

*  interrupt conflicts with other devices

*  conflicts between the mouse and serial ports

*  identifies which serial ports are sharing an interrupt

*  identifies serial ports which are not able to generate an

interrupt

*  possible problems with the DOS definitions for the serial

ports

*  Identifies some displays which conflict with the standard

COM4 serial port

PortInfo can also be used to update the system BIOS data area to

accurately reflect the system serial ports.

PortInfo (and all of the utilities) will run from DOS, or from a

DOS session within Windows or OS/2. However, the conflict testing

is very limited under Windows and OS/2, so it is recommended that

PortInfo be used for diagnosing problems primarily in the DOS

environment.

The summary screen provides information about the type of UART

used in the serial port and the current settings for the port.

You want to pay particular attention to the FAILURE and WARNING

messages that appear the first time you run PortInfo. All

messages and the summary screen can be captured to a "log file"

for later use or reference.

PortInfo reports a FAILURE message when there is a problem

detected with the system that will prevent correct operation.

Failures are typically reported for conflicts between a serial

port and another serial port or device. At times, a failure will

be reported by PortInfo when the port seems to work - this

generally happens because PortInfo tests the port at high speeds

(which "forces" many problems to appear), while the port is only

used at low speeds. You may get away with using such a port until

you try to move to higher speed operation.

A WARNING message is reported by PortInfo when there is no

immediate problem that you have to fix, but there is a situation

that could lead to a problem if you are not careful. For example,

if there are two different serial ports using the same interrupt,

a WARNING will be reported. If you are careful, you can often

share the IRQ between the ports - BUT the potential for a problem

exists.

16

Both warning and failure messages have a message number. Save the

message number(s) for reference to the section titled "PortInfo

FAILURE and WARNING Messages". You will find each message

explained in more detail with a description of the most common

solutions for the problem.

We've occasionally observed situations with internal modems that

require the computer to have a hard reset before running PortInfo

to obtain correct results. If you observe a problem where

PortInfo does not detect an internal modem or the IRQ used by the

modem, then either turn the computer off and back on to perform a

hard reset, or, if the computer has a RESET button, press it.

Then try running PortInfo again.

Also, if you have the Personal PLUS or any Professional version,

try using the "ResetMod" utility. ResetModem will perform the

most complete reset of the modem that is possible without turning

the power off.

NOTE: a soft reset (control-alt-delete) is not sufficient to

reset these modems to a state where PortInfo can recognize them.

You must do a "hard reset" as described above.

5.1 Test All Serial Ports - /0

To start PortInfo to do a complete test of all ports, enter

the following command:

C:> PortInfo /0 <Enter>

This tells PortInfo to do a full test of all standard serial

ports (remember, /0 is the command for all ports), and all

non-standard serial ports you have defined using the CTSCOMn

environment variables for the CTS Serial port Utilities.

NOTE: If you have a non-standard serial port installed, it

will not be tested unless you have defined a CTSCOMn

environment variable.

5.2 Test One Serial Port - /n, n<>0

You can also have PortInfo test only a single serial port.

You can tell PortInfo to test a single serial port in two

different ways. One way is to specify the serial port number

you want to test. For example, the following command will

test COM2:

C:> PortInfo /2 <Enter>

17

You can also test a serial port by specifying the serial

port address. This is the only way you can test a non-

standard serial port without a CTSCOMn environment variable.

For example, to test the port at address 01E0 (hex), you

enter:

C:> PortInfo /A 1E0 <Enter>

5.3 BIOS Update - /B

PortInfo will update the BIOS data area to accurately

include all serial ports in the computer system. Windows and

many DOS applications often fail if the BIOS data area is

not accurate. If you receive any "PI-5000" series error

messages, run PortInfo with the /B option. Combining "/B"

with "/S" (discussed below) offers an easy alternative to

the DOS_COM utility for AUTOEXEC.BAT file use.

5.4 Quick Test - /Q

You may tell PortInfo to do a "quick test" if you are in a

hurry to just check out a port setting. The quick test skips

most of the conflict tests, so it will generally not detect

port conflicts. You can specify the quick test for both a

single port test and the complete system test. To specify

the quick test, add a "/Q" (for Quick) to the command line

as shown below:

Example 1: Do a quick test on all ports:

C:> PortInfo /0 /Q <Enter>

Example 2: Do a quick test on COM3:

C:> PortInfo /3 /Q <Enter>

5.5 Log File - /L, /LA, /LP

PortInfo can create a copy of all messages and the summary

screen on the disk that you can use to refer to when

installing new equipment in the computer. When you use the

"/L" command line switch, PortInfo creates a file named

"PORTINFO.LOG" in the default directory. This file contains

all information that is displayed on the screen while

PortInfo runs. Use the command "/LA" to Append to an

existing log file (/LA will create a new log file if there

is not a PortInfo.LOG file available to append too.) The

command "/LP" will "create" the log file on the system

printer (LPT1).

18

Example: Do a full system test, and save the results in the

"PORTINFO.LOG" file:

C:> PortInfo /0 /L <Enter>

5.6 Force a Mouse Reset on PortInfo Exit - /MR

On occasion, a serial port mouse driver will need to be

reset after running PortInfo. If the mouse stops working

after running PortInfo, use the '/MR' command to have

PortInfo reset the mouse just before stopping.

5.7 Short (or Summary) Report - /S

PortInfo now includes a "short report" capability which does

a quick system test to simply identify the current serial

ports by Com Port number, IRQ (interrupt) used, the port

type and the current BIOS setting. Combining this option

with the "/B" command provides a fast way to automatically

set the comm port assignments and verify they match what you

think is installed.

Example: Do a quick system test, update the BIOS area, and

display the short report:

C:> PortInfo /S /B <Enter>

NOTE: the "/0" is not required here, as this the default

setting.

5.8 Skip the IRQ Scan - /NoScan

The "/NoScan" command bypasses the "Active Interrupt Scan"

normally done at the start of PortInfo. The active interrupt

scan is used by PortInfo to detect certain devices,

including network cards, that may be affected by PortInfo.

If there is a network card installed in the system being

tested, it is suggested that you do NOT use this option.

5.9 Search for ESP port - /ESP

The "/ESP" command will instruct PortInfo to search the

computer for a Hayes ESP-compatible serial port.

5.10 Advanced PortInfo Options

The following PortInfo options are available in the

different registered versions as indicated. Those indicated

as "Professional" are designed to allow fast checking of a

number of computer via different batch file options.

19

/Batch (Professional)

The "/Batch" command causes PortInfo to automatically accept

any options presented, and to continue running without user

intervention. Use of the /Batch option is normally done with

the "/LA" option, allowing the creation of batch files that

can perform a number of different operations while allowing

the user to review the test results with a text viewer.

/LT  (All)

The "/LT" command will cause PortInfo to do a "long test" to

search for a hard to find conflict. The long test takes

about 2 hours / port (this may be best run overnight!)

PortInfo will report any conflicts that are detected as well

as any "spurious" interrupts. Spurious interrupts may be

caused by a number of sources (including PCMCIA devices);

however, if several occur when testing one particular port,

then this may indicate a second port located at the same

address.

/Modem (Professional)

The "/Modem" command forces PortInfo to do the modem

detection test. This option bypasses the prompt requesting

permission from the user to continue and reminding the user

to turn on their modems.

/ModemID (All)

The "/ModemID" command will tell PortInfo to display and log

the results of the modem identification commands (ATI0 -

ATI7).

/NoModem (Professional)

The "/NoModem" causes PortInfo to skip the modem test

completely.

/NoMouse (Professional)

The "/NoMouse" command causes PortInfo to skip the mouse

test. In systems where no mouse is installed, this may save

a few seconds of run time.

/NoWin (All)

The "/NoWin" command will force PortInfo to not look for or

verify the Windows setup.

20

/Plug (All)

The "/Plug" command will cause PortInfo to also look for a

loopback condition on the transmit and receive lines of each

serial port where no other device is detected. This option

can be used with a loopback plug to identify which physical

connector on the back of the computer belongs to which port.

/Win <Path> (All)

To avoid a lengthy search of the hard disk, PortInfo will

normally look for the Windows SYSTEM.INI file in the

"C:\Windows" directory. If Windows is installed in a

different directory or disk drive, then specify the complete

path for PortInfo on the command line here. For example, to

verify the SYSTEM.INI file in "D:\win311", run "PortInfo

/Win d:\win311".

PORTINFO.CFG File

The PORTINFO.CFG file is used to store a set of commands

that you want PortInfo to execute each time. You create a

text file called PORTINFO.CFG containing a list of PortInfo

commands just as they would be entered on the command line.

Then, instead of entering the commands each time you run

PortInfo, you can simply enter the command "PortInfo".

5.11 Network Considerations

Early versions of PortInfo (1.x) would occasionally cause

users to lose their network connections. It is believed that

these problems have been eliminated with version 2.0.

However, if you should have a problem, please contact CTS to

let us know, and then review the following technique for

avoiding the problem.

While searching for serial port and interrupt (IRQ)

conflicts, PortInfo performs a very intensive test of the

computer system interrupts. One result of this testing is

that it is possible to lose the network connection after

running PortInfo. You can restore the network connection by

just logging out and back in, or you can avoid the problem

entirely if you know what IRQ the network card is using.

PortInfo includes the ability to exclude any IRQ or port

address from the test. By ensuring that the network card IRQ

is excluded, you will avoid interference with the network.

If the network card is installed at an address that

conflicts with a normal serial port address, you should also

exclude the address from the test.

21

To exclude one or more IRQs from being tested, you can use

the "/XIi" command line switch. The "/XIi" switch causes

PortInfo to eXclude Irq "i" from being tested.

Example: Exclude IRQ 5 (used by the network card) from a

full system test:

C:> PortInfo /0 /Xi5 <Enter>

NOTE: you can exclude multiple IRQs by using multiple "/XIi"

commands

Example: Exclude a network card at address 2E8 using IRQ 5

from a full system test:

C:> PortInfo /0 /Xi5 /XA2E8 <Enter>

This example demonstrates the use of the eXclude Address

switch "/XAaaa". This switch will prevent PortInfo from

testing a port residing in the address block including

address "aaa".

NOTE: an "address block" is defined for this discussion as

eight consecutive addresses starting with an address ending

in "0" or "8".

The example above excludes testing of the eight addresses

located at 2E8-2EF.

22

5.12 The PortInfo Summary Screen

PortInfo completes operation by displaying a summary of all

information it gathered while testing. The following is a sample

display for a system with four or less serial ports (eight columns are

displayed for more serial ports.) Each item on the display is

discussed in the following sections.

PortInfo Summary Screen

Serial 1     Serial 2     Serial 3   Serial 4

Port Address (Hex)             3F8          2F8          3E8        N/A

Interrupt (IRQ)                  4            3           10

IRQ (Out2) Enabled              No          Yes           No

DOS uses port as              Com1         Com2         Com3        N/A

Device Detected        ExtFAXModem        Mouse     FAXModem    Display

FAX Service Class          0,1,2.0                       0,1

Type of Port                T/Port        16450       16550A

Buffer - Trigger              None         None         On-8

Speed                        19200         1200         2400

Bits / Character                 8            7            8

Parity                        None         None         None

Stop Bits                        1            1            1

Clear to Send                   On          Off          Off

Data Set Ready                  On          Off          Off

Ring Indicator                 Off          Off          Off

Data Carrier Detect             On          Off          Off

Data Terminal Ready            Off           On          Off

Request to Send                Off           On          Off

23

5.12.1 Port Configuration

5.12.1.1 Port Address

This line shows the "base port address" in

hexadecimal for each serial port detected. The

"base port address" is the first address used by

the serial port. Serial ports normally use eight

address locations, so the term "base port address"

is used to refer to the first address used. In the

example displayed above, the COM1 base port

address is 03F8 (hex). The serial port actually

uses eight consecutive addresses starting at 03F8

and ending at 03FF (hex).

5.12.1.2 Interrupt (IRQ)

The Interrupt (IRQ) line of the display shows the

actual hardware interrupt used by the serial port.

This value is determined by testing the serial

port. If a ? is displayed in this field, then

there is a possible conflict with this port that

will cause a problem if you try to use the port.

If a ??? is displayed, then PortInfo was not able

to get the port to generate any interrupts.

5.12.1.3 IRQ (Out2) Enabled

This line shows the current status of the OUT2

line for the serial port. The OUT2 line is a UART

control line used by some communication software

to enable the IRQ. A serial port cannot generate

interrupts when this line is disabled, OR when

another serial port that uses the same IRQ line is

enabled.

If you have two serial ports sharing an IRQ with

an "Enabled" status, or if you have a problem

using one port when the other shows an "Enabled"

status, then you need to disable the OUT2 line on

the inactive port. You can do this with two

different utilities provided in the SPU.

"ResetCom" will restore a serial port to it's

original power-up status, and "IRQ" provides you

with direct control of the IRQ (Out2) line. These

utilities are described below.

24

5.12.1.4 DOS uses port as

This line will show how DOS expects to use this

port. This field will contain the DOS default port

assignments as detected by the system BIOS at

boot-up time. If the letters "N/A" are displayed,

then the port is Not Available for use by commands

like the DOS Mode command. The port assignment can

be changed with the DOS_COM and DOS_SWAP utilities

provided with PortInfo.

If PortInfo detected a problem with the port

assignments, then a < will appear beside the

assignment. The easiest way to fix this problem is

to install the BIOS_FIX (included) in your

AUTOEXEC.BAT file.

5.12.1.5 Device Detected

This line displays the type of device detected on

the serial port - this includes a serial mouse,

modems of several types, and "LoopBack" for the

Registered License options when the "/Plug"

command is used. Modems are identified as

precisely as possible: "Modem" when a modem is

detected, but no other information is available,

"IntModem" when the modem is identified as an

Internal Modem without FAX capability, "ExtModem"

for an external without FAX, "FAXModem" for a

modem with FAX capability, but not identified as

internal or external, "IntFAXModem" or

"ExtFAXModem" for internal and external FAXmodems,

respectively.

5.12.1.6 FAX Service Class

The FAX Service Class, or "FAX class" will display

the reported FAX classes the modem reports

supporting. This information can be used to make

sure that the FAX software is compatible with the

FAX class(es) supported by the modem. As of late

1994, there are three distinct "FAX classes":

Class 1, Class 2, and Class 2.0. Class 0, which

means standard data transfer mode (data and file

transfer) is normally reported as being available

as well

If you are having problems using the FAX modem, be

sure that you are using software designed for the

FAX classes supported by the modem.

25

5.12.1.7 Type of Port

The Type of Port line describes the type of UART

detected by PortInfo. If no UART was detected for

the port, the letters "N/A" will be displayed.

Also notice that the word "Display" appears on the

COM4 port. There are some display cards, including

the Diamond Stealth, the ATI Ultra Graphics, and

others which emulate IBM 8514/A displays that use

the address space normally reserved for COM4.

PortInfo detects the presence of many of these

displays and identifies the port as such. PortInfo

reports Hayes ESP compatible ports as ESP-I or

ESP-II (depending on the ESP version). These ports

are functionally equivalent to a 16550A in non-

enhanced mode. The ESP port detection must be

enabled by using the /ESP command line option with

PortInfo.

Two new UARTs are detected by PortInfo beginning

with version 3.0. The StarTech 16650 is an

"enhanced 16550A" that includes 32 byte buffers,

automatic flow control and a higher data rate.

Unfortunately, this UART is not fully compatible

with the 16550A, so it should be used with

caution. This part is reported as "ST-650".

The second new UART is the 16750 from Texas

Instruments. This UART provides a 64 byte buffer

and automatic flow control and is fully compatible

with the 16550A. This part is reported as the "TI-

750".

NOTE: that there are two types of UARTs which

software cannot tell apart: the 8250A and the

16450. PortInfo chooses to display this UART type

as a "16450" since the 16450 is the most common

UART used today. Other diagnostic software may

identify the UART as an 8250A (or even an 8250,

which is incorrect).

26

5.12.1.8 Buffer - Trigger

One of the unique features of PortInfo is it's

ability to sense not only the status of the FIFO

buffer on a 16550A, but also to detect what

receiver trigger level is being used. This line

will display "None" for those UARTs that do not

have buffers, Off for 16550A compatible UARTs

whose buffers are not enabled, and On-#, where #

will be 1, 4, 8, or 14 to show that the FIFO

buffer receiver trigger is set to (for 16550A

modes; the StarTech 16650 and TI 16750 will report

different trigger levels if they are enabled in

the enhanced mode.)

The receiver trigger level for a 16550A UART

determines when it will issue an interrupt

request. If the trigger is set to 14, the UART

will issue the interrupt after 14 of the 16 spots

in the buffer are filled. This reduces the number

of interrupts generated by the UART, but only

leaves three character times for the PC to service

the interrupt before data is lost. Reducing the

trigger level to 8 increases the number of

interrupts (1 interrupt for every 8 characters

received), but greatly reduces the possibility of

data loss since the PC has 9 character periods

before an overrun will occur. If you experience

data loss at high speed in a communication

program, try exiting to DOS while connected and

run PortInfo on the port. If the Buffer - Trigger

shows "Off", then the software may not support the

16550A UART. You can try enabling the buffer with

the trigger set to one (1) with the "Buffer"

utility and then exit to your communication

software. This change may cause the communication

software to lock up. If it does, your only

recourse is to contact the software manufacturer

for an update, or change to another communications

application.

If PortInfo displays "On-14", try using the

"Buffer" utility to reset the trigger level to

eight (8). This should reduce or eliminate the

overrun problem. Making this change should not

adversely affect the communications software, but

may eliminate the overrun problems.

27

5.12.2 Port Settings

The following port settings are normally controlled by

the communication software.

5.12.2.1 Speed (BPS)

This line shows the current port data rate in bits

per second (BPS). normally control this value in

the communication program, but you can change this

value with the DOS Mode command, or with the

COM_BPS utility provided with PortInfo. The

advantage of the COM_BPS utility is that it allows

setting data rates up to 115,200 bps, and supports

ports not recognized by DOS.

NOTE: to convert to characters per second, you

normally divide the BPS value by 10. That is, a

serial port sending characters at 19,200 bps is

sending 1,920 characters (or bytes) per second.

5.12.2.2 Bits / Character

The bits per character line displays the number of

data bits used for each character. Allowable

values are 5, 6, 7, and 8. You can control the

bits per character with the COM_FMT utility

provided with PortInfo. This setting is normally

controlled by the communications software, but is

provided here for control of non-standard ports

not supported by the DOS Mode command.

5.12.2.3 Parity

The Parity line shows the current parity bit

setting for the port. The possible values are

None, Even, Odd, Mark, and Space. You can control

the port format with the COM_FMT utility provided

with PortInfo.

5.12.2.4 Stop Bits

This line shows the number of stop bits sent with

each character. Valid values for the number of

stop bits are 1, 1.5, and 2. You can control the

number of stop bits with the COM_FMT utility

provided with PortInfo.

28

5.12.3 Hardware Signals

The serial port hardware signals are used by different

modems and devices in different ways. The RTS line is

often used for hardware flow control, or it may be used

to provide power to a portable modem. It's also

possible for a cable, connector, or a serial port board

to provide a means of looping back, or connecting lines

to each other. If you see a hardware line listed as "On

- DTR" or "Off - RTS", then this means that the line

indicated is in the state shown, and it *may* be

connected to the line indicated. These results can be

affected by the devices connected to the serial port.

You can disconnect the device and cables to determine

where the loopback condition occurs.

Common uses for each line are discussed below.

5.12.3.1 Clear to Send

The Clear To Send (CTS) line is commonly used by

modems and other devices for "hardware flow

control". Flow control is used to prevent data

from being sent faster than it can be processed.

The CTS line is input only - it is controlled by

the device attached to the serial port. The modem

can use the CTS line to start and stop the flow of

data sent by the computer when your software

supports hardware flow control.

The CTS line is normally "Off" when the serial

port has nothing connected to it, or if it is

connected to a device like a serial mouse or a

scanner. When the serial port is connected to a

Hayes compatible modem, and the modem is turned

on, CTS should be "On" at all times except when

the modem is connected to a remote system, and the

modem has signalled the software to stop sending

information. If you detect that CTS is not "On"

for the modem when there is no connection, then

you may have one of the following conditions: 1)

The cable between the serial port and the modem

does not contain the CTS line. This means you will

NOT be able to use hardware flow control with the

modem; 2) The modem is not a true Hayes-compatible

modem; 3) The modem is not operating correctly; or

4) The serial port is not working correctly.

29

5.12.3.2 Data Set Ready

The Data Set Ready (DSR) is a line which

historically is used by modems (data sets) to

signal when they are connected to a remote modem

and ready to send data. This line is input only -

it is controlled by the device attached to the

serial port.

Most modems today allow you to configure how this

line is used. The default for Hayes and compatible

modems is to turn this line "On" when the modem is

powered on. On some modems, you can also configure

this line to be "Off" at all times except when

connected with a remote modem.

5.12.3.3 Ring Indicator

The ring indicator line is normally used by modems

to signal that a ring was detected on the phone

line. This line is input only - it is controlled

by the device attached to the serial port.

PortInfo will normally detect this line as "Off"

with a modem unless you manage to get a ringing

signal at the exact moment PortInfo checks the

status of the line.

5.12.3.4 Data Carrier Detect

The Data Carrier Detect (DCD) line is normally

used by a modem to signal that the modem is

connected to a remote modem and ready to exchange

information. This line is input only - it is

controlled by the device attached to the serial

port.

Many modems can be configured so that DCD is "On"

all of the time with a command (the &C command is

used by Hayes and true compatible modems) or with

a switch on the modem.

30

5.12.3.5 Data Terminal Ready

The Data Terminal Ready (DTR) line is a line

output by the serial port to the connected device.

Historically, the DTR line was used by old "dumb

terminals" to signal the modem that they were

ready to receive data.

The DTR line typically has two different uses

today with modems. Most common is to use the DTR

line to end a connection. The modem can look for a

signal from the communication software to force a

hangup. In some cases, this will also hang up the

modem.

The second use for this line is to provide power

to the "line powered" portable modems introduced

over the last couple of years. Some portable

modems have no power supply or batteries, and

actually use the telephone line, the DTR line, and

the RTS line (see below) to supply their power. If

you have one of these modems, then make sure that

DTR is high if you try to use the modem. Most

communication software does this automatically,

but some utilities that send modem commands from

the DOS prompt may not. You can use the "DTR"

utility that comes with PortInfo to control the

DTR line.

5.12.3.6 Request to Send

The Request to Send (RTS) line is another line

output by the serial port to the connected device.

This line typically is used for hardware (RTS/CTS)

flow control. When this line is "On", the serial

port is requesting that the modem send data.

A second use for this line is to provide

additional power to the "line powered" modems as

discussed in the paragraph above. You can use the

"RTS" utility provided with PortInfo to control

the RTS line.

31

6.0 The Other Utilities

There are a number of additional utilities provided with

PortInfo. These utilities are very specific in nature, and are

provided to give you a standard user interface and source of

support. You can also use these utilities to test the PortInfo

utility.

6.1 BIOS_FIX

BIOS_FIX updates the BIOS data area to accurately install

each of the serial ports detected in the computer, resets

and initializes the port to the default DOS values. Install

BIOS_FIX in the AUTOEXEC.BAT file so it is run each time the

computer is restarted - this will ensure that the serial

ports are always ready for communications.

If you have registered the SPU, then BIOS_FIX can also check

the Windows SYSTEM.INI file each time the computer is

rebooted. This will make sure that nothing has changed in

the system setup that may cause a problem in Windows.

6.2 BUFFER

BUFFER controls the internal buffer in a 16550A or

compatible serial port. You can use this utility to turn the

FIFO buffer "On" with any of the four ( 1, 4, 8, and 14)

valid receiver trigger levels, or to turn the buffer off. In

general, application software which can support a 16550A

UART will control the buffer correctly.

The primary use of this utility is to: 1) turn off a 16550A

buffer which was not turned off by an application; and 2) to

provide a way to adjust the buffer setting for a software

application which has the trigger level set too high.

If you have old software which does not support a 16550A,

and you are experiencing an occasional overrun error (this

often appears as a retry during a file transfer with an

"error-free" connection), then you can try turning the

buffer on with different trigger levels. This is an

experimental process, and it may not work. In fact, it's

possible the communication software will "lock up" if you do

this.

From the DOS prompt, type "BUFFER" or "BUFFER /?" to display

the command line options and examples.

NOTE: If no trigger level is specified on the command line,

BUFFER will set the receiver trigger level to 14.

32

6.3 COM_BPS

COM_BPS sets a serial port data rate (in bits per second, or

BPS) to any of the common standard values supported by the

serial port. You can use this to replace the MODE command

provided with DOS on ports that DOS does not detect. The

following data rates are supported by COM_BPS: 110, 300,

600, 1200, 2400, 4800, 7200, 9600, 14400, 19200, 38400,

57600, and 115200 bits per second (bps). From the DOS

prompt, type "COM_BPS" or "COM_BPS /?" to display the

command line options and examples.

NOTE: most communication applications control this parameter

directly.

6.4 COM_FMT

COM_FMT sets the character format on a serial port to the

specified value. You can use this to replace the MODE

command provided with DOS on ports that DOS does not detect.

The following chart shows the three character "format" value

used on the command line and the associated description:

Format         Description

8N1          Eight data, No parity, 1 stop bit

8N2          Eight data, No parity, 2 stop bits

7E1          Seven data, Even parity, 1 stop bit

7E2          Seven data, Even parity, 2 stop bits

7O1          Seven data, Odd parity, 1 stop bit

7O2          Seven data, Odd parity, 2 stop bits

7M1          Seven data, Mark parity, 1 stop bit

7M2          Seven data, Mark parity, 2 stop bits

7S1          Seven data, Space parity, 1 stop bit

7S2          Seven data, Space parity, 2 stop bits

From the DOS prompt, type "COM_FMT" or "COM_FMT /?" to

display the command line options and examples.

NOTE: most communication applications control this parameter

directly.

6.5 DOS_COM

The BIOS of the computer system creates a list of detected

serial ports when it boots. Unfortunately, many computers

will only detect the standard serial ports COM1 and COM2.

Some computers will look for the conventional COM3 and COM4

ports as well. You can use the PortInfo summary screen line

"DOS uses port as" to determine whether the BIOS has

detected the serial port.

33

DOS will only use ports that are detected in the BIOS list

of serial ports. For example, DOS must know a COM3 port

exists if you want to redirect LPT1 to COM3, where you have

a serial printer. If PortInfo shows "DOS uses port as N/A",

then you need to help it out and add COM3 to the list of DOS

ports. You use DOS_COM to do this. From the DOS prompt, type

"DOS_COM /?"  to display the command line options and

examples.

NOTE: Also see the BIOS_FIX and DOS_SWAP utilities for

alternative ways of changing the BIOS port assignments.

6.6 DOS_SWAP

DOS_SWAP exchanges two port addresses in the list of serial

ports used by DOS. From the DOS prompt, type "DOS_SWAP" or

"DOS_SWAP /?" to display the command line options and

examples. You can use the PortInfo summary screen line "DOS

uses port as" to determine whether how the BIOS has assigned

the serial ports it detected.

6.7 DTR

DTR controls the Data Terminal Ready line for the port

specified. You can raise (turn "On") or lower (turn "Off")

the DTR line with this utility. See the discussion on the

DTR line in the section dealing with the PortInfo summary

screen for applications of this utility. From the DOS

prompt, type "DTR" or "DTR /?" to display the command line

options and examples.

6.8 Find_COM

NOTE: Find_COM is a bonus utility provided with the

Professional Licenses only.

Find_COM is a utility which will locate, display, and

optionally install the serial ports into the BIOS data area.

Ports are installed into the BIOS first using the standard

Com1/Com2 addresses, then using the conventional Com3/Com4

address, and finally in ascending address sequence until a

total of 4 serial ports are installed. Many communication

applications use the BIOS data area to identify which serial

ports are available for use.

34

Find_COM carefully searches through the computer system I/O

address space, performing a passive test to determine if a

serial port exists at a particular address block. If the

passive test determines that a serial port may be installed,

then a more detailed test is performed.

Certain areas of the system I/O space are not tested by

FIND_COM under any circumstances. The following address

areas are permanently excluded:

Port Addresses Excluded:

0000 - 0107         PS/2 POS registers

0170 - 0177         Secondary Fixed Disk Controller

01F0 - 01F7         Primary Disk Controller

0320 - 032F         Original Fixed disk controller

03B0 - 03DF    Video

0370 - 0377         Secondary Floppy Disk Controller

03F0 - 03F7         Primary Floppy Disk Controller

If you know that equipment is installed using a particular

I/O address space, then you can avoid testing those address

spaces with the "/XAaaa-bbb" address switch as described

below.

Command Line format:

FIND_COM [/Options] <Enter>

Options:

no parameter: default search

/B   loads serial ports located by COM_FIND into the BIOS

data area. The standard Com1-Com4 addresses are loaded

first, then ports are loaded in ascending address

sequence (up to 4 total ports installed).

/XAaaa or /XAaaa-bbb

excludes the I/O address block at "aaa" or the range of

address blocks including addresses aaa-bbb from

testing. This is used to avoid testing a particular I/O

address that may conflict with a board in the system.

NOTE: Multiple address blocks can be excluded from testing

(see example 5)

/?   Displays the help screen

35

Examples:

1.   FIND_COM <Enter>

Find_COM will search all allowed address locations for

serial ports and display the results to the terminal

screen. No ports are installed into the BIOS data area.

2.   FIND_COM /B <Enter>

The /B switch enables the BIOS port installation.

Find_COM will search all allowed address locations for

serial ports and display the results to the terminal

screen. Located serial ports are installed into the

system BIOS in the following sequence (as found) until

a total of four ports are installed:

3F8, 2F8, 3E8, 2E8, and then in ascending address

sequence starting from address 108.

3.   FIND_COM /XA300 <Enter>

The /XA switch tells FIND_COM to exclude the 8-byte

address block containing the following address. 8-byte

address blocks start with addresses ending in "0" or

"8", so the following are all equivalent:

/XA300, /XA301, /XA302.... /XA307

Find_COM will search all allowed address locations for

serial ports and display the results to the terminal

screen. The address block from 300-307 is not tested.

No ports are installed into the BIOS data area.

4.   FIND_COM /XA302-320 /B <Enter>

The /XA switch in this example tells FIND_COM to

exclude the address block from 300-327 from the search.

NOTE: the address "302" falls in the 300-307 address

block, and the address 320 falls in the 320-327 address

block. Therefore, the address range from 300 - 327 is

excluded from the search.

Find_COM will search all allowed address locations for

serial ports and display the results to the terminal

screen. The address block from 300-327 is not tested.

The detected serial ports are installed in the BIOS as

described in example 2.

36

5.   FIND_COM /XA300 /XA360-36F /B <Enter>

This example shows use of multiple block exclusion.

FIND_COM does not test the addresses 300-307 or

360-36F. All other allowed addresses are searched, and

located ports are installed in the BIOS data area as

described in example 2.

6.9 HANGUP

HANGUP performs a series of actions to help force the modem

to disconnect from the phone line. Some communication

software will not cause the modem to hang up the phone line

if the modem configuration is not correct. HANGUP bypasses

the modem configuration problem by trying a number of

different techniques.

From the DOS prompt, type "HANGUP" or "HANGUP /?" to display

the command line options and examples.

6.10 IRQ

IRQ controls the interrupt request line (Out2) from the

serial port specified. Generally, a serial port should have

IRQs disabled (IRQ Status "Off" on the PortInfo screen) at

all times except when the serial port is actually being

used. The major exception to this rule is when a serial

mouse is attached to the serial port. The mouse driver will

enable the interrupt and keep it enabled at all times.

If you want to try sharing an interrupt between a mouse and

a modem, you can in some cases manage this with IRQ. For

example, if you have a mouse on COM1 and a modem on COM3,

both using IRQ 4, you can run IRQ to disable interrupts on

COM1. Then run your communication program. When you are

finished with the communication program, you can (but

normally you don't have to) run IRQ to re-enable the COM1

interrupt.

NOTE: this will NOT work reliably with a communication

application that is "mouse aware". If you need to share an

interrupt with a mouse-aware application, then you need to

use a utility similar to the CTS "MouseOff" and "MouseOn"

utilities.

From the DOS prompt, type "IRQ" or "IRQ /?" to display the

command line options and examples.

37

6.11 RESETCOM

RESETCOM resets a serial port to the DOS default condition.

Interrupts are disabled, the port is set to 2400 bits /

second with a character format of 8 data bits, no parity,

and 1 stop bit, and the Request to Send (RTS) and Data

Terminal Ready (DTR) lines are set low. If the port is a

16550A or compatible, the buffers (FIFOs) are also disabled.

There are a few communication applications which do not

properly restore a serial port after completion. ResetCOM

can be used to completely reset the serial port to a well-

defined state.

From the DOS prompt, type "RESETCOM" or "RESETCOM /?" to

display the command line options and examples.

6.12 RESETMOD

RESETMOD performs a series of actions to help avoid having

to power-cycle the modem - which is particularly useful if

you have an internal modem that requires rebooting the

system. RESETMOD (short for ResetModem) performs a full

reset of the serial port, and goes farther by then issuing a

sequence of modem commands in an attempt to re-establish

control over the modem.

From the DOS prompt, type "RESETMOD" or "RESETMOD /?" to

display the command line options and examples.

6.13 RTS

RTS controls the Request to Send line for the port

specified. You can raise (turn "On") or lower (turn "Off")

the RTS line with this utility. See the discussion on the

RTS line in the section dealing with the PortInfo summary

screen for applications of this utility. From the DOS

prompt, type "RTS" or "RTS /?" to display the command line

options and examples.

39

7.0 PortInfo FAILURE and WARNING Messages

The different types of failures and conflicts detected by

PortInfo are separated into different categories. Each FAILURE

and WARNING message can be referenced by number to the list

below. Steps you can take to correct each of the problems are

described.

The general rule is to pay the most attention to the first

FAILURE message that displays. Once a FAILURE has been detected,

it should be corrected before looking at other problems that may

be reported.

7.1 Port Failures

7.1.1 Message PI-1000

Symptoms: The serial port UART has failed the "local

loopback" test. This test performs a check of the UART

to verify it is performing basic functions. This port

is not.

Solution: This error normally indicates that the UART

is faulty and needs to be replaced - however, it can

also occasionally indicate an address conflict between

two ports. To verify that the serial port UART is

faulty, remove the port from the system and run

PortInfo again. If PortInfo still shows a serial port

at the same address, then the problem is an address

conflict. If no port appears at that address any

longer, however, then the serial port or modem needs

replacing.

7.2 Interrupt Conflicts

An interrupt conflict message indicates that one or more of

the serial ports is not always generating interrupts. This

can cause a number of "interesting" problems that sometimes

appear and disappear for no apparent reason. Finding the

cause of the problem requires both a tool like PortInfo to

identify that the problem exists, and patience on your part

in taking a systematic approach to fixing the problem.

Please follow the directions carefully.

40

7.2.1 Message PI-2010

Symptoms: You may have regular or sporadic lockups

using either of the ports at this address.

Solution: As the message indicates, PortInfo is

detecting two separate interrupts when testing a single

serial port address. The typical cause of this is two

different serial ports at the same address (eg, two

separate UARTs defined as COM1). To fix this problem,

it is necessary disable, remove, or reassign one of the

serial ports to another address location.

7.2.2 Message PI-2020

Symptoms: You will likely get a system lockup, a high

rate of CRC errors (file transfer retries) if using a

modem, a very jerky motion on a serial mouse, or a

combination of these symptoms if you do not resolve

this problem.

Solution: If the software allows use of another port or

IRQ, then you can change the port settings to avoid

this IRQ. A second approach is to temporarily remove or

disable the serial port, and then to run a GOOD

"interrupt snooper" utility like our IRQInfo to

determine which other device is using the IRQ. Then you

can decide which one of the two devices will actually

use the IRQ.

7.2.3 Message PI-2100

Symptoms: You have a serial port that works sometimes,

but not all of the time. You may have any or all of the

symptoms listed for any message in the PI-2000 to PI-

2999 range.

Solution: The usual causes for this error message are

multiple serial ports at the same address, or another

device using the interrupt. Use the solutions described

for PI-2001 and PI-2002.

41

7.2.4 Message PI-2110

Symptoms: Most applications will lock up when trying to

use this port.

There are three common causes for this message.

Troubleshooting this problem requires a careful and

methodical approach. The basic idea is to reduce the

system to a state where the conflict is no longer

observed, and then add boards back in one at a time to

determine which board is involved with the conflict.

Often (but not always) the board you have most recently

added to the system is the culprit.

Diagnosing and repairing this problem often involves

opening the computer system and removing / installing

boards. Be sure to follow the instructions provided

with the computer system when adding and removing

boards.

PortInfo is most useful at diagnosing problems if you

run it after installing each board. By making sure that

the system is ok after each board is added, you can

then quickly identify which board is causing the

problem.

7.2.4.1 You have multiple ports at the same

address

This is a common problem with many computers.

There is often a forgotten or unknown serial port

installed in the computer, and adding another port

at the same address location can cause all sorts

of interesting problems. The best way to approach

this problem is to remove all of the add-in boards

that have serial ports from the system. This

includes internal modems as well. Then run

PortInfo to verify that the system is no longer

displaying this error message. If the error still

exists, then it is likely due to one of the other

problems discussed here. If the problem did go

away, then add each of the boards back into the

computer one at a time to determine which board

causes the problem.

After you have identified which board causes the

problem, you should change the serial port address

(or Com# setting) to a selection which not used on

the system.

42

7.2.4.2 You have another device using the same

interrupt

Many different devices compete for interrupts with

the serial ports, including other serial ports

(including internal modems), bus mice, network

cards, sound cards, and so on to name several.

While well-behaved serial ports and software will

allow serial ports to share an IRQ, you generally

cannot share an IRQ between a serial port and

another device. Bus mice often use IRQs 2,3,4,5,

or 12, and sound cards commonly use IRQs 5 and 7.

Try removing these cards from the system and then

run PortInfo again to see if the problem still

appears. If the problem still exists, then leave

the boards out and continue diagnosing the problem

following the steps in (1). When you have the

system to the point where the error is no longer

reported, then begin adding the boards back to the

system one at a time, running PortInfo after

you've added each board to determine which board

is involved in the conflict.

7.2.4.3 The system will not let you share this

interrupt

There are some computer system motherboards which

simply will not let any other device generate

interrupts for IRQs used on the motherboard. For

example, if you have a serial port on one of these

motherboards set to use Com1 / IRQ 4, no other

device will be able to use IRQ 4.

Your only solution, if you need to share these

interrupts to make everything fit, is to disable

the serial ports on the motherboard, and purchase

an add-in serial port card. CTS offers several

options, so be sure to give us a call.

43

7.2.5 Message PI-2200

This is an informational message regarding the Telcor

Systems T/Port card. A newer ROM exists for the T/Port.

If you are experiencing problems with the T/Port, then

you should contact Telcor Systems Inc. for more

information.

NOTE: PortInfo is not able to test the IRQ used by the

T/Port when this message is displayed.

7.2.6 Message PI-2300

Symptoms: If you use well-behaved software, and if the

hardware allows sharing interrupts, then everything

will be fine. If not, then you can have problems

ranging from serial ports which work intermittently to

complete system lockups.

Solution: Make sure the hardware will allow the

interrupt to be shared among multiple devices, and that

the software will make the IRQ inactive on the board it

uses after it has completed use. You may not use two

devices at the same time on the same interrupt.

7.2.7 Message PI-2400

Symptoms: You can use your mouse or your modem, but not

both!

Solution: The mouse and modem are sharing the same

interrupt (IRQ). The mouse must be on an IRQ by itself

to work correctly. The normal solution is to move the

modem to a different (preferably unused) IRQ. If you do

not have a GOOD interrupt identification utility, we

suggest our IRQInfo product (available at a discount to

all SPU customers).

44

7.2.8 Message PI-2500

Symptoms: This message is displayed only during a '/LT'

(long test). This indicates that something in the

computer may be generating "noise" on one or more of

the IRQ lines. This report is provided primarily for

the more experienced "techie" who is having problems

running down an intermittent failure.

Solution: In some cases, this is due to a port

addressing conflict (eg, two Com1 ports) that is not

detectable by other means. Other causes may be a poor

serial card design, a noisy power supply or other

problem with the system. They have also been reported

for PCMCIA modems that are working acceptably. If the

spurious interrupts are reported for all ports, and the

ports appear to be working without problems, then

ignore this message. If the report only appears for one

of the serial ports, then there may be a definable

problem with this port. Try to test for a possible

address conflict by disabling or removing the known Com

port and retest with PortInfo. If the port is still

present, then there was a conflict. If the problem goes

away, but there is no other port present, then the

report may be indicative of a pending failure in the

board.

7.3 Port Conflicts

Port conflicts normally indicate that there are two serial

ports located at the same address. This is commonly done

when installing new serial cards or modems into an older

machine without knowing for certain what equipment is

already installed.

45

7.3.1 Message PI-3000

Symptoms: System lockup or failure of a communication

application to successfully use a modem.

Solution: You most likely have a conflict between a

serial port and another device, possibly another serial

port, at the same address location. You may also have a

serial port that is jumpered to use two IRQs at the

same time. Remove or disable the port which fails the

test, and run PortInfo again. If another port is

reported at the same address, then you have a conflict

between two serial ports. If no other serial port is

detected at that address, then you probably have a

conflict with another device. Common conflicts involve

local area network cards. Be sure to verify that your

serial card is jumpered correctly.

7.3.2 Message PI-3100

Symptoms: The system may lock up, or the display may

show meaningless characters ("garbage").

Solution: You cannot use a serial port at the normal

COM4 address location if this message appears. The

display adaptor on the video card is installed at the

address normally used for Com4.

7.3.3 Message PI-3200

Symptoms: The system may lock up, or the display may

show meaningless characters ("garbage").

Solution: There is a serial port installed at Com4 that

is being interfered with by the video card. Video

display cards that emulate an 8514/A display use the

same address as the normal Com4 port (02E8h). The port

at this address must be removed or reassigned to

another "non-standard" address.

46

7.3.4 Message PI-3500

Symptoms: This is a message displayed when a

"ComxBase=" statement exists in the Windows SYSTEM.INI

file that references a serial port that is at a "non-

standard" address and was not defined for PortInfo or

BIOS_FIX.

Solution: Add a "CTSCOMn=/A" environment variable as

described by the message to the AUTOEXEC.BAT file prior

to the BIOS_FIX utility.

7.4 Mouse Conflicts

PortInfo looks for conflicts between the mouse and the

serial ports. Most conflicts are detected by the interrupt

conflict tests. However, PortInfo also checks for possible

future conflicts.

7.4.1 Message PI-4000

This is a WARNING message that the bus mouse is using

one of the IRQs normally used by the Com1 or Com2

serial ports. If you are not using this port for any

connections, then you will not have any problems. If

you do use Com1 and Com2 for your communications and

they are using the standard IRQs (Com1 / IRQ 4, Com2 /

IRQ 3), then you need to change the IRQ used by the

serial port or the bus mouse to resolve the conflict.

47

7.5 DOS Port Assignments

The DOS port assignments are determined by a data table that

is created by the computer system BIOS when the system is

powered up or reset. The BIOS detects the ports installed in

the computer and places them in a data area for use by DOS

and application software.

Unfortunately, different BIOS implementations work

differently, so there are often inconsistencies in how the

serial ports are used. Many systems will only look for two

comm ports, and no system looks for more than four. The BIOS

also doesn't follow conventions in assigning a port at a

particular address to the proper Com port number. To further

confuse the issue, many applications do not use this data

area because of the inconsistencies and address the ports

directly by address. This leads to the situation where two

different applications use the same device through

"different" ports.

Windows operation is also affected by the port assignments.

The assignment by the BIOS is the first priority that

Windows uses. So, if the BIOS is not installing the ports

properly, then Windows may have communication problems.

The best general rule is to always install ports in

sequential order...that is, do not skip a port number. The

first port should use the normal Com1 address, the second

Com2, and so on.

With the release of Version 3, PortInfo will now display a

single message suggesting use of BIOS_FIX if any of the

ports are not recognized by the BIOS or are being identified

out of sequence.

7.5.1 Message PI-5000-5040

Obsolete - Removed from Version 3+

7.5.2 Message PI-5050

Symptom: The BIOS is not detecting one or more of the

serial ports properly. See the discussion above for

affects this can have on computer operation.

Solution: Use the BIOS_FIX utility in the AUTOEXEC.BAT

file to ensure that the serial ports are properly

installed in the bios and initialized.

48

7.6 Other CTS Software Detected

The CTS Utility SwapIRQ can affect the reporting of IRQs

used by some diagnostic software. PortInfo is "aware" of

SwapIRQ and notifies you that it is still active.

7.6.1 Message PI-6000

Symptoms: Just an informational message to let you know

that SwapIRQ was detected and temporarily disabled by

PortInfo. The PortInfo reports will not be affected by

the presence of SwapIRQ.

Solution: You should disable SwapIRQ when the

application has finished with it by using the "/D"

command line switch.

7.7 Windows Communication Setup

The registered versions of PortInfo and BIOS_FIX include a

test of the Windows communication setup parameters.

Normally, PortInfo is used to test the system and correct

any identified problems. BIOS_FIX can then be installed in

the AUTOEXEC.BAT file to "monitor" the system. BIOS_FIX

detects the port addresses and IRQs used by the ports and

then compares them to the way Windows will use the ports. If

the hardware or software configuration changes, then

BIOS_FIX will detect the change and alert you the next time

the computer is re-booted.

7.7.1 Message PI-7000

Symptoms: Just an informational message to let you know

that PortInfo did not check the SYSTEM.INI file when

only a single serial port was tested.

Solution: Run "PortInfo /0" to test all ports.

7.7.2 Message PI-7050

Symptoms: Another informational message to alert you

that PortInfo did not check the SYSTEM.INI file because

one or more serial ports are not generating interrupts.

Solution: Resolve the hardware problem with the serial

port interrupt as described in other PortInfo messages.

49

7.7.3 Message PI-7100

Symptoms: Yet another informational message to let you

know that PortInfo did not check the SYSTEM.INI file.

This message will appear when PortInfo cannot locate

the SYSTEM.INI file.

Solution: PortInfo looks (by default) in the directory

"C:\Windows" for the SYSTEM.INI file. If it is not

located in that directory, then you must specify the

drive and directory with the "/Win <path>" command as

described elsewhere in the User Guide and on the

PortInfo help screen.

7.7.4 Message PI-7200

Symptoms: PortInfo has detected that changes are needed

in the SYSTEM.INI file.

Solution: Make the listed changes to the SYSTEM.INI

file using a standard editor.

7.7.5 Message PI-7300

Symptoms: The port assignments made by the BIOS are

inaccurate.

Solution: Use the SPU utility BIOS_FIX in the

AUTOEXEC.BAT file.

8.0 PCMCIA (Card Modem) Considerations

PCMCIA modems and devices present a new challenge for diagnostic

software. At this time, there are a number of problems inherent

in this technology that are primarily related to getting

everything to work together. That is, the compatibility of the

different BIOS with the PCMCIA devices and drivers still has a

ways to go before the devices truly become easy to use.

PortInfo is helpful in determining 1) how to configure a new

PCMCIA device; and 2) to determine if the device is available and

operating. However, conflicts with other devices can be hidden

from PortInfo by the PCMCIA drivers.

One particular concern on portable computers is the various power

management features designed into the BIOS. In many cases, you

will need to disable the power management features when using the

modem. Common problems are ports that "disappear" after the PC

enters a power-saving mode and an inability to re-awaken the

computer by detecting activity on the serial port. I have

personally experienced modems that "disappear" while the computer

50

is running. In many cases, the only solution I have found to date

is to reboot the computer. Rest assured that we are continuing to

study this area for better solutions.

51

9.0 Customer Support Policy

Customer support is provided via telephone, FAX, and electronic

mail. We provide from 30 to 90 minutes of support (depending on

the license you obtained) for any registered user within the

following guidelines:

1. We are in the office and happy to assist with problems during

our normal business hours of 8-12am and 2-6pm (US east coast

time). We always answer the phone whenever we are in (other

hours). If you are unable to catch us "live", please leave a

detailed message, and we will reply via phone, FAX, electronic

mail (E-mail), or regular mail.

2. Messages sent to us on CompuServe E-mail or via the Internet

will usually receive a reply within 24 hours.

3. We will be happy to assist with SPU installation problems

before you register. However, we cannot provide detailed

technical assistance until we've actually received your

registration. We can register you over the phone via credit card

if you need immediate assistance.

Please be prepared to answer the following questions before you

call for assistance:

*  The version number of your CTS Serial Port Utilities. You

can view this information in the program title by starting

any of the utilities.

*  The brand name and model number of your computer.

*  What equipment you have in your computer. A good diagnostic

program such as our IRQInfo program can be very helpful in

determining what other equipment is installed.

*  The version of DOS you are using (type "VER" at the DOS

prompt).

*  A complete description of the problem.

*  Can you duplicate the series of steps that results in the

problem?

*  Did an error message appear? What did it say?

You may contact CTS via voice phone at 770-263-8623 during normal

business hours (Eastern Time Zone), or 24 hours / day via FAX at

770-263-0124, on CompuServe at 76662,2315, and on the Internet at

"support@comminfo.com". Visit our World Wide Web page at

"http://www.comminfo.com" for the latest versions of CTS shareware

products.

53

10.0 About Computer Telecommunication Systems

Computer Telecommunication Systems, Inc. was formed in 1990 to

provide new communication software products, custom programming

services, and consulting on personal computer asynchronous

communications. We currently offer (as of July, 1995) the

following products and services:

10.1 OEM and Custom Programming Services

Do you need a special version of PortInfo as a customer

support tool? How about a fully customized modem or serial

port installation program? We've done it before, and we're

happy to do it again!

Serial Port and Modem Installation Software: a multipass

installation program that "pre-tests" a user system,

determines port and IRQ availability, suggests likely

changes to the user setup, and then provides the user with a

proper switch settings. After the user installation is

complete, the installation is tested to verify that all is

well. Tell us about your hardware configuration and user

interface requirements!

Custom "Pre-communication" testing: Have a sensitive

communication application that your customers manage to

break easily? A PortInfo "pre-test" can verify that the

ports are set properly, interrupts are not being shared, and

that the modem is powered up and ready to connect.

Transparent to the user, but a great aid in reducing your

customer support calls by catching the problems before they

happen.

Please call us to discuss your special communications

software requirements.

10.2 IRQInfo

IRQInfo is our new general-system test utility that

identifies what interrupts (IRQs) are being used by what.

Unlike most other applications of this type, IRQInfo

actually tests the system to detect which IRQs are being

used (many similar applications make "assumptions" about

what IRQs are being used by the serial and printer ports,

disk drives, etc.) IRQInfo actually tells you when it isn't

certain about an IRQ - for example, it may not be possible

to always detect which IRQ the printer uses - so we make

sure that you know we detected the port, but not the IRQ -

and then we tell you what IRQ that port would normally use.

54

IRQInfo successfully detects interrupts used by sound cards,

SCSI controllers, hard and floppy disk drives, serial and

parallel ports, all standard system equipment, scanners,

network cards...just about everything. We think it is the

best tool available for identifying used and "free" IRQs.

10.3 SwapIRQ

A small utility that allows software to use IRQs that are

not supported directly by the software. For example, a

communication application that supports COM3 on IRQ 4 can

now use COM3 on any IRQ. Or, a game that requires a sound

card on IRQ 7 can now use a sound card that generates

interrupts on IRQ 5. Contact CTS for more information.

10.4 PortFix

A small DOS TSR utility that prevents serial port lockups

with DOS software using a faulty serial port chip shipped

with many PCI-bus machines and some add-in cards in late

1993 and early 1994.

10.5 AT Command Test

AT Command Test (ATCT) is a software application that

performs a comprehensive test of the AT command set of any

modem claiming to be "Hayes Compatible". ATCT tests most of

the standard modem commands, all result codes, and many

other characteristics. ATCT provides three different reports

of command set compatibility with nine different Hayes modem

product lines.

10.6 AT Command Test for EIA-602

AT Command Test for EIA-602 (ATCT'602) is a software

application that performs a comprehensive test of the AT

command set of any modem claiming to be compliant with the

ANSI/EIA/TIA-602 command set standard. ATCT'602 tests all of

the standard modem commands and result codes, and many other

characteristics.

ATCT and ATCT'602 are being used today by modem

manufacturers as a design aid, as a qualification tool for

purchasing modems by major corporations, and as a diagnostic

tool for testing modems returned to corporate support

services prior to obtaining warranty service. Please contact

CTS for more information.

55

10.7 New Products

We are constantly developing new software products that you

may be interested in, including several that have been

suggested by our customers. Be sure to check with us at 1-

800-380-2666 if you have questions about any new products we

may have (or that you would like to have!)

56

The appendices provide background and tutorial information for

the CTS Serial Port Utilities. They have been removed from the

shareware version to reduce file size, but are provided in the

printed User Guide.