SP3000N Installation Steps：

#### 1. Removing the computer cover

#### 2. Setting the modem jumpers

SP3000N

Modem jumper settings for SP3000N (Jumper JP1)

|  | Jumper A | Jumper B | Jumper C | Hex Address |
| --- | --- | --- | --- | --- |
| COM1 | ON | ON | ON | 3F8 to 3FF |
| COM2 | OFF | ON | ON | 2F8 to 2FF |
| COM3 | ON | OFF | ON | 3E8 to 3EF |
| COM4 | OFF | OFF | ON | 2E8 to 2EF |
| Plug and Play | OFF | OFF | OFF |  |

Modem jumper settings for SP3000N (Jumper JP2)

| IRQ3 | ON | OFF | OFF | OFF | OFF | OFF |
| --- | --- | --- | --- | --- | --- | --- |
| IRQ4 | OFF | ON | OFF | OFF | OFF | OFF |
| IRQ5 | OFF | OFF | ON | OFF | OFF | OFF |
| IRQ9 | OFF | OFF | OFF | ON | OFF | OFF |
| IRQ10 | OFF | OFF | OFF | OFF | ON | OFF |
| IRQ15 | OFF | OFF | OFF | OFF | OFF | ON |
| Plug and Play | OFF | OFF | OFF | OFF | OFF | OFF |

Jumper

Jumper

ON

OFF

#### 3. Inserting the internal modem

#### 4. Connecting to the telephone line

#### 5. Connecting to your telephone

#### 6. Verifying your connection

Completed modem installation for SP3000N

#### Windows 95 （OSR2）Modem Driver Installation （PnP）

The screen message will be as follows after Windows 95 was run. ‘New Hardware Found’ ” Micronet Shuttle 3000N 56K Modem”

- Then on the screen the message will show “Update Device Driver Wizard” click “Next” to install “Micronet Shuttle 3000N 56K Modem’s” driver .

- Press “Other location” then “Browse” buttons, put the driver diskette into the floppy drive. Next find out the directory “w95nt40” in the driver diskette. Press “Ok” twice then “Finish” once.

- Note: The original Windows 95 (OSR2) disc is need while setup the configuration.

- “Wave Device ……” message appears then when you see the message “Update Device Driver Wizard” shows on the screen. Install the driver again . The procedures as in step 2 .

- The new modem can work right after all the configurations were set up.

SP3000E Installation Steps：

#### Connecting to a computer or terminal

SP3000E

Figure 1-1. Back of modem

- Plug the male DB-25 connector on your RS-232C cable into the RS-232 connector on the back of the modem (see Figure 1-1).

- Plug the other end of the cable into the appropriate RS-232C connector on your computer or terminal. If you need assistance, refer to the manual that accompanied your equipment or contact your computer dealer.

#### Connecting to the telephone line

- Take one end of the modular telephone cable supplied with your modem and plug into the LINE jack on the back of the modem (see Figure 1-1).

- Plug the other end of the telephone cord into the telephone jack on the wall.

#### Connecting to your telephone

- Connect the telephone's modular cord into the PHONE jack on the back of your modem (see Figure 1-1).

- Lift your telephone's handset and listen for a dial tone.

#### Connecting to an AC outlet

- Insert the supplied power cable into the round AC-IN connector next to the POWER switch on the back of the modem (see Figure 1-1).

- Plug the end with the power pack into either a 110 or 220 VAC outlet.

- Turn the POWER switch to ON position, then the MR LED goes on.

- Turn on the computer or terminal connected to the modem.

#### Using a external MIC and speaker

#### Verifying your connection

#### Windows 95 （OSR2）Modem Driver Installation （PnP）

The screen message will be as follows after Windows 95 was run. ‘New Hardware Found’ ” Micronet Shuttle 3000E 56K Modem”

- Then on the screen the message will show “Update Device Driver Wizard” click “Next” to install “Micronet Shuttle 3000E 56K Modem’s” driver .

- Press “Other location” then “Browse” buttons, put the driver diskette into the floppy drive. Next find out the directory “w95nt40” in the driver diskette. Press “Ok” twice then “Finish” once.

- Note: The original Windows 95 (OSR2) disc is need while setup the configuration.

- “Wave Device ……” message appears then when you see the message “Update Device Driver Wizard” shows on the screen. Install the driver again . The procedures as in step 2 .

- The new modem can work right after all the configurations were set up.

#### BASIC COMMAND SET

| Modem Command | Description |  |
| --- | --- | --- |
| +++ | Keep data connection and move from data mode to command mode When your modem enters the data mode, typing +++ allows it to escape to the command mode while preserving the connection to remote modem. |  |
| A | Answer command Force the modem to go off-hook in answer mode without waiting for a ring. Use this command to manually answer an incoming call. |  |
| A/ | Re-execute last command Repeat the last command line it executed. Not preceded with AT nor followed by pressing the Enter key. This command is useful when you want the modem to redial a telephone number that was busy. |  |
| Bn | U.S./ITU-T answer sequence - ITU-T V.25 answer sequence. ( default) - answer tone. |  |
| D | Dial command Puts the modem into the originate mode, allowing it to automatically dial a telephone number. The telephone number to be dialed, the symbols # and * and the dial modifiers P, R, S=n, T, W, @, !, ; and, can follow the D command. |  |
|  | P | Pulse(rotary) dial (default) |
|  | T | Tone dial |
|  | , | Two-Second pause; link to S8 register |
|  | ; | Return to Command mode after dialing |
|  | ! | Flashes the switch hook |
|  | / | Delays for 125 msec. Before proceeding with dial |
|  | W | Wait for second dial tone (X3 or higher); linked to S6 register |
|  | @ | Dials, waits for quiet answer, and continues (X3 or higher) |
|  | R | Originates call using answer (reverse) frequencies |
|  | #,* | Extended touch tone pad tones |
|  | L | Dials the last-dialed number |

| D | $ | Displays a list of Dial commands |
| --- | --- | --- |
|  | Sn | Dials the phone number string stored in the NVRAM at position n (n=0-3). Phone are stored with the &Zn=s command |
| En | Echo characters in command mode Determines whether the modem echoes to your computer screen the keyboard commands you type during command mode - Do not echo commands - Echo commands (default) |  |
| Fn | Echo transmitted data to Terminal (DTE) in Data Mode - Transmitted data is echoed to DTE - DO not echo (default) |  |
| Hn | Hook switch - Go on-hook (hang up modem) - Go off-hook |  |
| In | Displays the followings information - Four-digit product code - Results of ROM checksum - Results of RAM self test - Product type - Current modem settings - Nonvolatile memory(NVRAM) settings - Link diagnostics - Product configuration - Reserved - Plug and Play information - Reserved - Call statistics |  |
| Ln | Speaker volume - Low - Low - Medium(default) - High |  |
| Mn | Speaker status - Speaker always off - Speaker on until carrier is detected(default) - Speaker always on - Speaker ON after dial, until CONNECT |  |

| On | RETURN to data mode after using escape characters to switch to command node - Return to data mode - Perform equalizer retrain sequence, then return to data mode. A retrain causes the modem to optimize its operating characteristics to obtain the best data transmission. This command works at speeds of 2400 bps or faster. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qn | Modem responses Determines whether the modem returns responses after you execute, or try to execute, a modem command. Modem responses are described in Chapter 2. - Send responses to local computer (default) - Do not send response - Displays result codes only in Originate mode - Displays result codes only in Ring mode. |  |  |  |  |  |  |  |
| Sn? | Display the contents of S-Register n Refer to Chapter 6 for more information. |  |  |  |  |  |  |  |
| Sn=v | Change the value of Register n to v Refer to Chapter 6 for more information |  |  |  |  |  |  |  |
| Vn | Response format If modem responses are enabled (Q0 in effect), determines whether the responses are shown in numeric or word format. - Numeric response format - Word response format (default) |  |  |  |  |  |  |  |
| Xn | Response set Determines the response set used. |  |  |  |  |  |  |  |
|  | Result Codes | X0 | X1 | X2 | X3 | X4 | X5 | X6 |
|  | 0/OK |  |  |  |  |  |  |  |
|  | 1/CONNECT |  |  |  |  |  |  |  |
|  | 2/RING |  |  |  |  |  |  |  |
|  | 3/NO CARRIER |  |  |  |  |  |  |  |

| Xn | 4/ERROR |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5/CONNECT 1200 |  |  |  |  |  |  |  |
|  | 6/NO DIAL TONE |  |  |  |  |  |  |  |
|  | 7/BUSY |  |  |  |  |  |  |  |
|  | 8/NO ANSWER |  |  |  |  |  |  |  |
|  | 10/CONNECT 2400 |  |  |  |  |  |  |  |
|  | 13/CONNECT 9600 |  |  |  |  |  |  |  |
|  | 18/CONNECT 4800 |  |  |  |  |  |  |  |
|  | 20/CONNECT 7200 |  |  |  |  |  |  |  |
|  | 21/CONNECT 12000 |  |  |  |  |  |  |  |
|  | 25/CONNECT 14400 |  |  |  |  |  |  |  |
|  | 43/CONNECT 16800 |  |  |  |  |  |  |  |
|  | 85/CONNECT 19200 |  |  |  |  |  |  |  |
|  | 91/CONNECT 21600 |  |  |  |  |  |  |  |

| Xn | 99/CONNECT 24000 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 103/CONNECT 26400 |  |  |  |  |  |  |  |
|  | 107/CONNECT 28800 |  |  |  |  |  |  |  |
|  | 151/CONNECT 31200 |  |  |  |  |  |  |  |
|  | 155/CONNECT 33600 |  |  |  |  |  |  |  |
|  | 180/CONNECT 32000 |  |  |  |  |  |  |  |
|  | 184/CONNECT 36000 |  |  |  |  |  |  |  |
|  | 188/CONNECT 40000 |  |  |  |  |  |  |  |
|  | 192/CONNECT 44000 |  |  |  |  |  |  |  |
|  | 196/CONNECT 48000 |  |  |  |  |  |  |  |
|  | 200/CONNECT 49333 |  |  |  |  |  |  |  |

| Xn | 204/CONNECT 50666 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 208/CONNECT 52000 |  |  |  |  |  |  |  |
|  | 212/CONNECT 53333 |  |  |  |  |  |  |  |
|  | 216/CONNECT 54666 |  |  |  |  |  |  |  |
|  | 220/CONNECT 56000 |  |  |  |  |  |  |  |
|  | 224/CONNECT 57333 |  |  |  |  |  |  |  |
|  | 228/CONNECT 58666 |  |  |  |  |  |  |  |
|  | 232/CONNECT 60000 |  |  |  |  |  |  |  |
|  | 236/CONNECT 61333 |  |  |  |  |  |  |  |
|  | Functions |  |  |  |  |  |  |  |
|  | Adaptive Dialing |  |  |  |  |  |  |  |

| Xn | Wait for 2nd Dial Tone (W) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Wait for Answer (@) |  |  |  |  |  |  |  |
|  | Fast Dial |  |  |  |  |  |  |  |
|  | requires @ in dial string; replaces NO CARRIER |  |  |  |  |  |  |  |
| Yn | Elects power-on/reset default configuration This command tells the modem which profile to use the next time the modem is turned on or reset. - Load Profile 0 (default) - Load Profile 1 - Generic Template (&F0) - Hardware Flow Control (&F1) - Software Flow Control (&F2) |  |  |  |  |  |  |  |
| Zn | Reset modem Resets the modem and loads the configuration profiles stored with the &Wn command. Refer to Chapter 3 for more information. - Resets modem to NVRAM profile selected by Y command - Resets modem to NVRAM profile 0 - Resets modem to NVRAM profile 1 - Resets modem to factory default profile 0 (&F0) - Resets modem to factory default profile 1 (&F1) - Resets modem to factory default profile 2 (&F2) |  |  |  |  |  |  |  |
| &$ | Display a list of ampersand (&) commands |  |  |  |  |  |  |  |
| &An | Modify responses Determines whether the modem sends MNP responses. - ARQ result codes disabled - Append /ARQ to word CONNECT responses when an error-correction connection is made - Append additional V.32 indicator in result codes for calls at or above 4800bps. - Append Protocol of call : /LAPM, /MNP or NONE and V.42bis/MNP5 |  |  |  |  |  |  |  |

| &Bn | DTE interface speed - Change serial port speed to match bps rate of connection - Do not change serial port speed; serial port speed is independent of the connection speed. (default) - In answer mode, fix serial port speed for ARQ calls and change it for non-ARQ calls. |
| --- | --- |
| &Cn | Carrier Detect signal status Controls the Data Carrier Detect (DCD) signal. - Forced on continuously - Follows status of remote carrier signal (default) |
| &Dn | Data Terminal Ready signal status Determines how your modem reacts to Data Terminal Ready (DTR) signal - DTR override, DTR is always on - On-line command mode on DTR low. Modem enters command mode on DTR transition to low state. Returns on-line returns high. - Computer must send DTR for the modem to accept commands, and dropping DTR terminates a call. (default) |
| &Fn | Fetch the factory default - Load standard factory configuration : X1, &A1, &B0, &H0, &I0,&R1 - Load factory configuration 1 Hardware Flow control : X4, &A3, &B1, &H1, &I2, &R2 - Load factory configuration 2 Software Flow control : X4, &A3, &B1, &H2, &I2, &R1 |
| &Gn | Guard tones Guard tones are used internationally, but are not used in the USA. - Disabled (default) - Generate 550Hz guard tone, requires B0 setting. - Generate 1800Hz guard tone, requires B0 setting. |

| &Hn | Flow control This command can be used to control the flow of data between your modem and computer. Flow control is necessary when the speed between your computer and modem is different than the speed between your modem and the remote modem. - Disable flow control; - Enable hardware flow control (CTS) - Enable software flow control XON/XOFF. - Enable hardware and software flow control. |
| --- | --- |
| &In | XON/XOFF flow control pass through - Disable XON/XOFF flow control. (default) - Process XON/XOFF signals to your modem and remote system. - Process XON/XOFF signals to your modem only. |
| &Kn | Enable/disable V.42bis and MNP Class 5 data compression - Disable data compression - Data compression is enabled if the DTE data rate is higher than the link rate, and the remote DCE either supports MNP 5 or V.42bis (default) - Enable data compression - Disable MNP 5, but enable V.42bis |
| &Mn | Set operating mode Selects the type of asynchronous data connection the modem will try to establish. A modem that uses V.42 can not communicate with one using MNP, since the two error correction protocols are different. However , this command ensures that your modem will be can automatically communicate with modems that use V.42bis, MNP, or no error-correction protocol. - Normal mode. - Reserved - Reserved - Reserved - auto-reliable mode(default) - or MNP reliable mode |

| &Nn | DCE Link Rate Sets connect speed. If connection cannot be established at this speed, the modem will be hang up. - Variable rate (default) - Modem Attempt a 300 bps connection - Modem Attempt a 1200 bps connection - Modem Attempt a 2400 bps connection - Modem Attempt a 4800 bps connection - Modem Attempt a 7200 bps connection - Modem Attempt a 9600 bps connection - Modem Attempt a 12000 bps connection - Modem Attempt a 14400 bps connection - Modem Attempt a 16800 bps connection - Modem Attempt a 19200 bps connection - Modem Attempt a 21600 bps connection - Modem Attempt a 24000 bps connection - Modem Attempt a 26400 bps connection - Modem Attempt a 28800 bps connection - Modem Attempt a 31200 bps connection - Modem Attempt a 33600 bps connection - Modem Attempt a 32000 bps connection (X2) - Modem Attempt a 36000 bps connection (X2) - Modem Attempt a 40000 bps connection (X2) |
| --- | --- |

| &Nn | - Modem Attempt a 44000 bps connection (X2) - Modem Attempt a 48000 bps connection (X2) - Modem Attempt a 49333 bps connection (X2) - Modem Attempt a 50666 bps connection (X2) - Modem Attempt a 52000 bps connection (X2) - Modem Attempt a 53333 bps connection (X2) - Modem Attempt a 54666 bps connection (X2) - Modem Attempt a 56000 bps connection (X2) - Modem Attempt a 57333 bps connection (X2) - Modem Attempt a 58666 bps connection (X2) - Modem Attempt a 60000 bps connection (X2) - Modem Attempt a 61333 bps connection (X2) |
| --- | --- |
| &Pn | Pulse make/break ratio - ratio US and Canada setting (default) - ratio UK and Hong Kong setting |
| &Rn | Receive data hardware flow control Enables options for received data hardware flow control. - RTS signal is ignored - Modem sends data to DTE only if RTS is ON. (default) |
| &Sn | Data Set Ready signal status - DSR signal always ON (default) - DSR signal is ON during handshaking and OFF in test or idle mode. DSR is OFF when the remote carrier signal is lost |

| &Tn | Begins test modes - Ends testing - Initiates Analog Loopback - Reserved - Initiates Local Digital Loopback - Enables Remote Digital Loopback - Prohibits Remote Digital Loopback - Initiates Remote Digital Loopback - Initiates Remote Digital with self-test and error detector - Initiates Analog Loopback with self-test and error detector |
| --- | --- |
| &Un | DCE Link Rate Floor Sets Minimum DCE connection data rates. - Disabled (default) - 300 bps - 1200 bps - 2400 bps - 4800 bps - 7200 bps - 9600 bps - 12000 bps - 14400 bps - 16800 bps - 19200 bps - 21600 bps - 24000 bps - 26400 bps - 28800 bps - 31200 bps - 33600 bps - 32000 bps(X2) - 36000 bps(X2) - 40000 bps(X2) - 44000 bps(X2) - 48000 bps(X2) - 49333 bps(X2) - 50666 bps(X2) - 52000 bps(X2) - 53333 bps(X2) - 54666 bps(X2) - 56000 bps(X2) - 57333 bps(X2) - 58666 bps(X2) - 60000 bps(X2) - 61333 bps(X2) |

| &Wn | Write S register values to one of two profiles The following commands and registers can be stored in nonvolatile memory : Commands : Bn, Cn, En, Ln, Mn, Qn, Vn, Yn, Xn, &Cn, &Dn, &Gn, &Ln, &Pn........ Register : S0, S2, S6, S7, S8, S9, S10, S11, S12, S18, S25, S26, - Write S register values to profile 0 - Write S register values to profile 1 |
| --- | --- |
| Yn | Break handling This command enables various methods of handling break sequences when a break is received from DTE or remote DCE. - Destructive option. When modem receives break signal from DTE, it clears data from transmit buffer, but does not pass break to DCE . Both local and remote modems resets data (default) - Expedited destructive option. When modem receives break signal from DTE, it clears data from transmit buffer and sends break to DCE, also resets data compression tables. - Expedited nondestructive option. When modem receives break signal from the DTE, it does not clear transmit buffer, but sends remote DCE out of sequence. |
| &Zn=Ax | Stored up four telephone numbers in nonvolatile memory n is the memory location (0-3) A is T(for touch tone) or P (for pulse) x is the phone number to be stored To dial a stored number, use the ATDS=n command Example : AT &Z0=T 1 818 555 2121 stores in memory position 0 the telephone number 1 818 555 2121, which will be Touch Tone (T) dialed. |

SP3000N SP3000E

56000bps Modem