Execute VIDEOID to identify the type of video adapter(s). The information is

written both to the screen and also to a text file named VIDEOID.TXT

The first six steps, identifying the installed video adapter(s) are based on

routines from Richard Wilton's "Programmer's Guide to PC and PS/2 Video

Systems" (Microsoft Press, ISBN 1-55615-103-9) with minor modifications to

identify 64k/256k EGAs.

If an EGA or VGA is detected, a seventh step dumps the first 216 bytes of

video RAM to show the manufacturer's copyright notice.

If a VGA is detected, step 8 attempts to identify the chipset manufacturer

and the amount of video memory, if possible. These routines are primarily

based on John Bridges' VGAKIT with modifications and additions. VideoId adds

detect routines for 8514/a and the Hercules Graphics Station and additional

memory tests, where available.

detect  memory  memory test results

------- ------- ---------------------

IBM 8514/a                 VIDEOID         ???k

Hercules Graphics Station  VIDEOID VIDEOID                  1meg

Ahead Systems type A       vgakit          ???k

Ahead Systems type B       vgakit          ???k

Tseng 3000                 vgakit          ???k

Tseng 4000                 vgakit  VIDEOID      256k, 512k, 1meg

Everex                     vgakit  vgakit       256k, 512k

Trident                    vgakit          ???k

ATI                        vgakit  vgakit       256k, 512k

Video7                     vgakit  vgakit       256k, 512k

Paradise                   vgakit  VIDEOID      256k, 512k

Chips & Technologies       vgakit  vgakit       256k, 512k

Oak Technologies           vgakit  vgakit       256k, 512k

The Herc GS "1meg" result is plugged. The Paradise memory routine seems to

be working on a variety of Paradise cards and clones. The Tseng 4000 routine

is new in version 1.3, and we need reports of test results.