TABLE OF CONTENTS

io.library/io_AllocExtParPort

io.library/io_AllocExtSerPort

io.library/io_AllocJoyPort

io.library/io_AllocParPort

io.library/io_AllocSerPort

io.library/io_FreeExtParPort

io.library/io_FreeExtSerPort

io.library/io_FreeParPort

io.library/io_FreeSerPort

io.library/io_ReadExtParA

io.library/io_ReadJoy1A

io.library/io_ReadJoy2A

io.library/io_ReadParA

io.library/io_SetExtParDir

io.library/io_SetExtParDirA

io.library/io_SetJoy1Dir

io.library/io_SetJoy1DirA

io.library/io_SetJoy2Dir

io.library/io_SetJoy2DirA

io.library/io_SetParDir

io.library/io_SetParDirA

io.library/io_WriteExtParA

io.library/io_WriteJoy1A

io.library/io_WriteJoy2A

io.library/io_WriteParA

io.library/io_AllocExtParPort                   io.library/io_AllocExtParPort

NAME

io_AllocExtParPort -- Allocate extra (Busy, Pout, Sel)

parallel bits for use with

io.library.

SYNOPSIS

result = io_AllocExtParPort()

int io_AllocExtParPort( void );

FUNCTION

Reserve the use of the parallel bits (POUT, SEL, BUSY) to

io.library.

INPUTS

none

RESULT

result - NULL for success, -1 for error while opening misc.resource,

-2 for error while opening ciaa.resource or ciab.resource

-3 for Busy, Pout, Sel parallel bits already in use.

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocParPort()

io.library/io_AllocExtSerPort                   io.library/io_AllocExtSerPort

NAME

io_AllocExSerPort -- Allocate extra (DTR, CTS, etc)

serial bits for use with

io.library.

SYNOPSIS

result = io_AllocExtSerPort()

int io_AllocExtSerPort( void );

FUNCTION

Reserve the use of the serial bits to

io.library (see resources/misc.h ).

INPUTS

none

RESULT

result - NULL for success, -1 for error while opening misc.resource,

-2 for error while opening ciaa.resource or ciab.resource

-3 for serial bits already in use.

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocSerPort()

io.library/io_AllocJoyPort                         io.library/io_AllocJoyPort

NAME

io_AllocJoyport -- Allocate JoyPort bits for use with io.library.

SYNOPSIS

result io_AllocJoyPort()

D0

int io_AllocJoyPort( void );

INPUTS

none

FUNCTION

Reserve the use of JoyPort2 (pin: 5, 6, 9) to io.library.

RESULT

NULL for success, -1 for error while opening potgo.resource,

-2 for error while opening ciaa.resource or ciab.resource.

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocParPort                         io.library/io_AllocParPort

NAME

io_AllocParPort -- Allocate parallel bits for use with

io.library.

SYNOPSIS

result = io_AllocParPort()

int io_AllocParPort( void );

FUNCTION

Reserve the use of the parallel bits (D0-D7) to

io.library.

INPUTS

none

RESULT

result - NULL for success, -1 for error while opening misc.resource,

-2 for error while opening ciaa.resource or ciab.resource

-3 for D0-D7 parallel bits already in use.

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocExtParPort()

io.library/io_AllocSerPort                         io.library/io_AllocSerPort

NAME

io_AllocSerPort -- Allocate serial bits for use with

io.library.

SYNOPSIS

result = io_AllocSerPort()

int io_AllocSerPort( void );

FUNCTION

Reserve the use of the serial bits to

io.library.

INPUTS

none

RESULT

result - NULL for success, -1 for error while opening misc.resource,

-2 for error while opening ciaa.resource or ciab.resource

-3 for serial bits already in use.

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocExtSerPort()

io.library/io_FreeExtParPort                     io.library/io_FreeExtParPort

NAME

io_FreeExtParPort -- Free parallel bits allocated by

io_AllocExtParPort().

SYNOPSIS

io_FreeExtParPort()

void io_FreeExtParPort( void );

FUNCTION

Free parallel bits (POUT, SEL, BUSY) allocated

by io_AllocExtParPort().

INPUTS

none

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocExtParPort()

io.library/io_FreeExtSerPort                     io.library/io_FreeExtSerPort

NAME

io_FreeExtSerPort -- Free serial bits allocated by

io_AllocExtSerPort().

SYNOPSIS

io_FreeExtSerPort()

void io_FreeExtSerPort( void );

FUNCTION

Free serial bits ([...]) allocated

by io_AllocExtSerPort().

INPUTS

none

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocExtSerPort()

io.library/io_FreeParPort                           io.library/io_FreeParPort

NAME

io_FreeParPort -- Free parallel bits allocated by

io_AllocParPort().

SYNOPSIS

io_FreeParPort()

void io_FreeParPort( void );

FUNCTION

Free parallel bits (D0-D7) allocated

by io_AllocParPort().

INPUTS

none

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocParPort()

io.library/io_FreeSerPort                           io.library/io_FreeSerPort

NAME

io_FreeSerPort -- Free serial bits allocated by

io_AllocSerPort().

SYNOPSIS

io_FreeSerPort()

void io_FreeSerPort( void );

FUNCTION

Free serial bits ([...]) allocated

by io_AllocSerPort().

INPUTS

none

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_AllocSerPort()

io.library/io_ReadExtParA                           io.library/io_ReadExtParA

NAME

io_ReadExtParA -- Read the logic state for parallel bits

(BUSY, POUT, SEL).

SYNOPSIS

void io_ReadExtParA(pin)

D0

void io_ReadExtParA(int);

INPUTS

pin - Number of the pin (0 for BUSY, 1 for POUT, 2 for SEL)

FUNCTION

Read the logic state for parallel bits.

BUSY is pin 11, POUT is pin 12, SEL is pin 13.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetExtParDirA()

io.library/io_ReadJoy1A                               io.library/io_ReadJoy1A

NAME

io_ReadJoy1A -- Read the logic state for Joyport 1.

SYNOPSIS

void io_ReadJoy1A(pin)

D0

void io_ReadJoy1A(int);

INPUTS

pin - Number of the pin (0 for pin 9, 1 for pin 5, 2 for pin 6)

FUNCTION

Read the logic state of Joyport 1 Input/Output bits (pin 5,6,9).

RESULT

none

BUGS

none

NOTES

none

SEE ALSO

io.library/io_SetJoy1DirA(), io.library/io_WriteJoy1A()

io.library/io_ReadJoy2A                               io.library/io_ReadJoy2A

NAME

io_ReadJoy2A -- Read the logic state for Joyport 2.

SYNOPSIS

void io_ReadJoy2A(pin)

D0

void io_ReadJoy2A(int);

INPUTS

pin - Number of the pin (0 for pin 9, 1 for pin 5, 2 for pin 6)

FUNCTION

Read the logic state of Joyport 1 Input/Output bits (pin 5,6,9).

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetJoy2DirA(), io.library/io_WriteJoy2A

io.library/io_ReadParA                                 io.library/io_ReadParA

NAME

io_ReadParA -- Read the logic state for parallel bits (D0-D7) .

SYNOPSIS

void io_SetParA(pin)

D0

void io_SetParA(int);

INPUTS

pin - Number of the pin (0 for D0, 1 for D1, ..., 7 for D7)

FUNCTION

Read the logic state for parallel bits.

D0 is pin 2, D1 is pin 3, ..., D7 is pin 8.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetParDirA()

io.library/io_SetExtParDir                         io.library/io_SetExtParDir

NAME

io_SetExtParDir -- Set the direction for parallel bits (BUSY,

POUT, SEL).

SYNOPSIS

void io_SetExtParDir( UBYTE )

D0

void io_SetExtParDir( signals );

INPUTS

signals - an UBYTE with the signals to set.

FUNCTION

Set the direction for parallel bits.

BUSY is pin 10, POUT is pin 11, SEL is pin 12.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetExtParDirA()

io.library/io_SetExtParDirA                       io.library/io_SetExtParDirA

NAME

io_SetExtParDirA -- Set the direction for parallel bits

(BUSY, POUT, SEL).

SYNOPSIS

void io_SetExtParDirA(pin, dir)

D0   D1

void io_SetExtParDirA(int, int);

INPUTS

pin - Number of the pin (0 for BUSY, 1 for POUT, 2 for SEL)

state - 0 for input, 1 for output

FUNCTION

Set the direction for parallel bits.

BUSY is pin 11, POUT is pin 12, SEL is pin 13.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetExtParDir()

io.library/io_SetJoy1Dir                             io.library/io_SetJoy1Dir

NAME

io_SetJoy1ParDir -- Set the direction for Joyport 1.

SYNOPSIS

void io_SetJoy1Dir( UBYTE )

D0

void io_SetJoy1Dir( signals );

INPUTS

signals - an UBYTE with the signals to set.

FUNCTION

Set the direction for parallel bits.

Bit 0 is pin 9, bit 1 is pin 5, bit 2 is pin 6.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetJoy1DirA()

io.library/io_SetJoy1DirA                           io.library/io_SetJoy1DirA

NAME

io_SetJoy1DirA -- Set the direction for Joyport 1.

SYNOPSIS

void io_SetJoy1DirA(pin, dir)

D0     D1

void io_SetJoy1DirA(int, int);

INPUTS

pin - Number of the pin (0 for pin 9, 1 for pin 5, 2 for pin 6)

dir - 0 for input, 1 for output

FUNCTION

Set the direction for Joyport 1 Input/Output bits (pin 5,6,9).

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetJoy1Dir()

io.library/io_SetJoy2Dir                             io.library/io_SetJoy2Dir

NAME

io_SetJoy2ParDir -- Set the direction for Joyport 2.

SYNOPSIS

void io_SetJoy2Dir( UBYTE )

D0

void io_SetJoy2Dir( signals );

INPUTS

signals - an UBYTE with the signals to set.

FUNCTION

Set the direction for parallel bits.

Bit 0 is pin 9, bit 1 is pin 5, bit 2 is pin 6.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetJoy2DirA()

io.library/io_SetJoy2DirA                           io.library/io_SetJoy2DirA

NAME

io_SetJoy2DirA -- Set the direction for Joyport 2.

SYNOPSIS

void io_SetJoy2DirA(pin, dir)

D0     D1

void io_SetJoy2DirA(int, int);

INPUTS

pin - Number of the pin (0 for pin 9, 1 for pin 5, 2 for pin 6)

dir - 0 for input, 1 for output

FUNCTION

Set the direction for Joyport 1 Input/Output bits (pin 5,6,9).

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetJoy2Dir()

io.library/io_SetParDir                               io.library/io_SetParDir

NAME

io_SetParDir -- Set the direction for parallel bits (D0-D7) .

SYNOPSIS

void io_SetParDir( UBYTE )

D0

void io_SetParDir( signals );

INPUTS

signals - an UBYTE with the signals to set.

FUNCTION

Set the direction for parallel bits.

bit 0 is D0 (pin2), bit 1 is D1 (pin 3), ..., 7 is D7 (pin 9).

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetParDirA()

io.library/io_SetParDirA                             io.library/io_SetParDirA

NAME

io_SetParDirA -- Set the direction for parallel bits (D0-D7) .

SYNOPSIS

void io_SetParDirA(pin, dir)

D0     D1

void io_SetParDirA(int, int);

INPUTS

pin - Number of the pin (0 for D0, 1 for D1, ..., 7 for D7)

state - 0 for input, 1 for output

FUNCTION

Set the direction for parallel bits.

D0 is pin 2, D1 is pin 3, ..., D7 is pin 8.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetParDir()

io.library/io_WriteExtParA                         io.library/io_WriteExtParA

NAME

io_WriteExtParA -- Set the logic state for parallel bits

(BUSY, POUT, SEL).

SYNOPSIS

void io_WriteExtParA(pin, dir)

D0   D1

void io_WriteExtParA(int, int);

INPUTS

pin - Number of the pin (0 for BUSY, 1 for POUT, 2 for SEL)

state - 0 for logic state 0 (0 volts),

1 for logic state 1 (5 volts).

FUNCTION

Set the logic state for parallel bits.

BUSY is pin 11, POUT is pin 12, SEL is pin 13.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_WriteExtPar()

io.library/io_WriteJoy1A                             io.library/io_WriteJoy1A

NAME

io_WriteJoy1A -- Set the logic state for Joyport 1.

SYNOPSIS

void io_WriteJoy1A(pin, state)

D0     D1

void io_WriteJoy1A(int, int);

INPUTS

pin - Number of the pin (0 for pin 9, 1 for pin 5, 2 for pin 6)

state - 0 for logic state 0 (0 volts),

1 for logic state 1 (5 volts).

FUNCTION

Set the logic state for Joyport 1 Input/Output bits (pin 5,6,9).

RESULT

none

BUGS

none

NOTES

none

SEE ALSO

io.library/io_SetJoy1DirA(), io.library/io_ReadJoy1DirA()

io.library/io_WriteJoy2A                             io.library/io_WriteJoy2A

NAME

io_WriteJoy2A -- Set the logic state for Joyport 2.

SYNOPSIS

void io_WriteJoy2A(pin, state)

D0     D1

void io_WriteJoy2A(int, int);

INPUTS

pin - Number of the pin (0 for pin 9, 1 for pin 5, 2 for pin 6)

state - 0 for logic state 0 (0 volts),

1 for logic state 1 (5 volts).

FUNCTION

Set the logic state for Joyport 1 Input/Output bits (pin 5,6,9).

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetJoy2DirA(), io.library/io_ReadJoy2A

io.library/io_WriteParA                               io.library/io_WriteParA

NAME

io_WriteParA -- Set the logic state for parallel bits (D0-D7) .

SYNOPSIS

void io_SetParA(pin, state)

D0     D1

void io_SetParA(int, int);

INPUTS

pin - Number of the pin (0 for D0, 1 for D1, ..., 7 for D7)

state - 0 for logic state 0 (0 volts), 1 for output

FUNCTION

Set the logic state for parallel bits.

D0 is pin 2, D1 is pin 3, ..., D7 is pin 8.

RESULT

none

BUGS

none known

NOTES

none

SEE ALSO

io.library/io_SetParDirA()