VGA-/ Multiscan-monitor-adaptor

===============================

This is a description to built a monitor adaptor / cable for all

monitors with RGB input (VGA and Multiscan monitors).

It's not a scan-doubler!

All parts are explained within the IFF-picture.

The adaptor is simultaneously to decouple the syncs and to avoid

vertical stripes on the screen.

- Use it when the amiga wont't start with the monitor connected

(screen stays grey and the amiga [all models] restarts in an

endless loop).

- Use it if you have vertical stripes on the screen although you

own a high quality monitor [AGA models].

The adaptor is simultaneously to decouple the syncs and to avoid

vertical stripes on the screen.

I put all parts in the plug at the amiga side, thus the new monitor

cable is simultaneously the adaptor.

The plugs inside with all parts and the cable(s) was fixed by

hot (melt-)adhesive.

The C-Mos TTL 74 HCT 08 is used because of the 50 Ohm output

impedance required at the sync lines (the RGB-lines are at 75 Ohm).

In the Commodore adaptor is also a 74 HCT 08 used.

The decoupling of the syncs is necessery because the amiga isn't

able to drive the impedance of 50 Ohms and less of this value

correctly. This is recognizable when the amiga restarts in an

endless loop with the monitor connected and seems to be start

normaly without the monitor connected. Most monitors will still

work because they have an impedance over 50 Ohms at the sync

inputs. Then you don't need a decoupling of the syncs.

Pay attention to choose the right cables (impedances) e.g. if you

split into 5 lines after the plug at the amiga side to connect to

a BNC input at the monitor. (Three cable of 75 Ohm impedance for

R G B, two cables of 50 Ohm impedance for horizontal and vertical

sync.)

A low voltage computer cable isn't suited very well. This will

cause interferences over the lines.

There are special video cables available holding small coaxial cable

of right impedances inside.

The two inputs of a lattice of the TTL are wired together for a

sync input. Connect the four unused inputs to ground. Don't

connect the unused outputs.

The Resistor of 150 Ohm is to avoid vertical stripes on the screen.

I think all AGA-Amigas are producing such stripes. They are

particulary visible at all resulutions with 27 kHz and more

(DblPal, DblNtsc, Muliscan).

Don't use the resistor with a genlock, it wouldn't work correctly.

If you have interferences on the screen after starting the amiga

(the machine is just cold) increase the value of the resistor.

Are the stripes also present with a resistor of 150 Ohm, decrease

the value.

- Ber