TABLE OF CONTENTS

iec.library/--background

iec.library/ACPtr

iec.library/CIOut

iec.library/Listen

iec.library/Second

iec.library/Talk

iec.library/TkSA

iec.library/UnListen

iec.library/UnTalk

iec.library/--background                             iec.library/--background

Disclaimer: information contained herein is not wholly proven reliable.

I didn't find enough official documentation about IEC interface and DOS

standards, so I examined the original ROM disassemble, some C64 disk

utilities and some Commodore 64 Emulators.

If you find something wrong, or own better specifications, please contact me.

Fabrizio Farenga

(f.farenga@agora.stm.it)

THE SERIAL BUS

The IEC serial bus is a daisy chain arrangement designed to let the

computer communicate with devices such as the VIC-1541 DISK DRIVE and

the VIC-1525 GRAPHICS PRINTER. The advantage of the serial bus is that

more than one device can be connected to the port. Up to 5 devices can be

connected to the serial bus at once.

There are three types of operation over a serial bus: CONTROL, TALK, and

LISTEN. A CONTROLLER device is one which controls operation of the serial

bus. A TALKER transmits data onto the bus. A LISTENER receives data from

the bus.

The computer is the controller of the bus. It also acts as a TALKER

(when sending data to the drive, for example) and as a LISTENER (when

loading a program from the disk drive, for example). Other devices may be

either LISTENERS (the printer), TALKERS, or both (the disk drive). Only

the computer can act as the controller.

All devices connected on the serial bus will receive the whole data

transmitted over the bus. To allow the computer to route data to its

chosen destination, each device has a bus ADDRESS. By using this device

address, the computer can control access to the bus. Addresses on the

serial bus range from 4 to 31.

The computer can COMMAND a particular device to TALK or LISTEN.

When the computer commands a device to TALK, the device will begin

putting data onto the serial bus. When the computer commands a device

to LISTEN, the device addressed will get ready to receive data (from the

computer or from another device on the bus). Only one device can TALK

on the bus at once; otherwise, the data will collide and the system

will crash in confusion. However, any number of devices can LISTEN at the

same time to one TALKER.

A SAMPLE TRANSMISSION

The basic operations, during a trasmission from computer to drive are the

following:

- OPEN TRASMISSION

- SEND DATA

- CLOSE TRASMISSION

This can occur when we want to send a single command to the drive, and we

don't ask for reply. As example we could send the "I" command to the

drive unit 8 to reset it.

In Commodore Basic v2.0 we should use something like:

1 OPEN 1,8,15

2 PRINT #1,"I"

3 CLOSE 1

- In the first line we ask to open a logic file number (1), to the device

number 8 (the drive) on the command channel (15).

- In the second line we send to the logic file 1 (after the previous OPEN,

it refers to the device number 8) the "I" character.

- In the third line we CLOSE the logic file number 1.

Now the drive will be reset.

Doing the same operations, using the iec.library functions and 'C'

language is easy:

/* Step 1 */

Listen(8);

Second(CMD_OPEN+15)

UnListen();

/* Step 2 */

Listen(8);

Second(CMD_DATA+15)

CIOut('I');

UnListen();

/* Step 3 */

Listen(8);

Second(CMD_CLOSE+15)

UnListen();

Step 1  the device 8 (we suppose a 1541 drive) will become a LISTENER.

We send it a secondary address to perform an open command on the

channel 15 (command channel). Then we end the opening session,

ordering to the device to UNLISTEN.

Step 2  the device 8 will become a LISTENER again. We notify by using

the secondary address that we are going to send data. Using the

CIOut() function, we send the 'I' character, then close the data

trasmission.

Step 3  we end up operations sending a secondary address that includes

the CMD_CLOSE command.

Please refer to the example sources to obtain further information.

Note: don't issue any command to the 1541 if it is connected but turned

off. Due to an odd bug, you'll not get the "DEVICE NOT PRESENT

ERROR" and you'll lock up the Amiga! If the 1541 is physically

disconnected, you'll get the proper error.

iec.library/ACPtr                                           iec.library/ACPtr

NAME

ACPtr -- Input byte from serial port.

SYNOPSIS

char = ACPtr()

D0

LONG ACPtr(void)

FUNCTION

This is the function to use when you want to get information from a

device on the serial bus, like a disk drive. This function gets one

data byte from the serial bus using full handshaking. A -1 is

returned when EOF or an error is encountered. To prepare for this

function the Talk() function must be called first to command the

device on the serial bus to send data through the bus. If the input

device needs a secondary command, it must be sent by using the TkSA()

function before calling this function.

How to Use:

1) Command a device on the serial bus to prepare to send data to

the computer. (Use the Talk() and TkSA() functions.)

2) Call this function

3) Print or otherwise use the data.

INPUTS

RESULTS

char - character read (0-255)

EXAMPLE

/*GET A BYTE FROM THE BUS*/

UBYTE c;

c=ACPtr();

printf ("Byte received - %d\n",c);

BUGS

SEE ALSO

Talk(), TkSA()

iec.library/CIOut                                           iec.library/CIOut

NAME

CIOut -- Output byte to serial port.

SYNOPSIS

CIOut(CHAR)

D0

void CIOut(char)

FUNCTION

This function is used to send information to devices on the serial

bus. A call to this function will put one data byte onto the serial

bus using full serial handshaking. Before this function is called,

the Listen() function must be used to command a device on the serial

bus to get ready to receive data (if a secondary address is needed,

this must also be sent by using the Second() function). Device must

be listening or the status word will return a timeout. This function

always buffers one character (the function holds the previous

character to be sent back). So when a call to UnListen() is issued

to end the data transmission, the buffered character is sent with an

End Or Identify (EOI) set, then UNLISTEN is sent to the device.

How to Use:

1) Use Listen() function (and Second() if needed).

2) Call this function to send the data byte.

INPUTS

char - character to transmit (0-255)

RESULTS

EXAMPLE

/*SEND A BYTE TO THE BUS*/

ACPtr(0xff);

BUGS

SEE ALSO

Listen(), Second()

iec.library/Listen                                         iec.library/Listen

NAME

Listen -- Command devices on the serial bus to LISTEN.

SYNOPSIS

Listen(device)

D0

void Listen(UBYTE)

FUNCTION

This function will command a device on the serial bus to receive

data. A device number between 0 and 31 must be provided before

calling the routine. LISTEN will OR the number bit by bit to convert

it to a listen address, then transmit this data as a command onto the

serial bus. The specified device will then go into listen mode, and

be ready to accept information.

INPUTS

char - character to transmit (0-255)

RESULTS

EXAMPLE

/*SEND LISTEN TO DEVICE NUMBER 8*/

Listen(8);

BUGS

SEE ALSO

Second(), UnListen()

iec.library/Second                                         iec.library/Second

NAME

Second -- Command devices on the serial bus to LISTEN.

SYNOPSIS

Second(secondary_address)

D0

void Second(UBYTE)

FUNCTION

This function is used to send a secondary address to an I/O device

after a call to the LISTEN function has been made, and the device is

commanded to LISTEN. The function cannot be used to send a secondary

address after a call to the TALK function. A secondary address is

usually used to give setup information to a device before I/O

operations begin. When a secondary address is to be sent to a

device on the serial bus, the address must first be ORed with one

of the low level command codes: CMD_DATA, CMD_CLOSE or CMD_OPEN.

The secondary address can be any number from 0 to 14. These refer

to channels used to communicate with the disk drive. Channel 15 is

reserved as command channel.

INPUTS

secondary_address - a valid secondary address (0-15)

RESULTS

EXAMPLE

/*OPEN DEVICE #8 WITH COMMAND (SECONDARY ADDRESS) #15 */

Listen(8);

Second(CMD_OPEN|15);

BUGS

SEE ALSO

Listen()

iec.library/Talk                                             iec.library/Talk

NAME

Talk -- Command serial bus device to TALK.

SYNOPSIS

Talk(device)

D0

void Listen(UBYTE)

FUNCTION

To use this function a device number between 0 and 31 must be

provided. When called, it ORes its argument bit by bit to

convert this device number to a talk address. Then data is

transmitted as a command on the serial bus.

INPUTS

device - a valid device number (8-31)

RESULTS

EXAMPLE

/*SEND TALK TO DEVICE NUMBER 8*/

Talk(8);

BUGS

SEE ALSO

TkSA(), UnTalk()

iec.library/TkSA                                             iec.library/TkSA

NAME

TkSA -- Send secondary address after TALK.

SYNOPSIS

TkSA(secondary_address)

D0

void TkSA(UBYTE)

FUNCTION

This function transmits a secondary address onto the serial bus for

a TALK device. The function sends the number as a secondary address

command over the serial bus. This function can only be called after

a call to the TALK function. It will not work after a LISTEN. A

secondary address is usually used to give setup information to a

device before I/O operations begin.

When a secondary address is to be sent to a device on the serial

bus, the address must first be ORed with one of the low level

command codes: CMD_DATA, CMD_CLOSE or CMD_OPEN.

The secondary address can be any number from 0 to 14. These refer

to channels used to communicate with the disk drive. Channel 15 is

reserved as command channel.

INPUTS

secondary_address - a valid secondary address (0-15)

RESULTS

EXAMPLE

/*OPEN DEVICE #8 WITH COMMAND (SECONDARY ADDRESS) #2 */

Talk(8);

TkSA(CMD_OPEN|2);

BUGS

SEE ALSO

Talk()

iec.library/UnTalk                                         iec.library/UnTalk

NAME

UnTalk -- Command serial bus to UNTALK

SYNOPSIS

UnTalk()

void UnTalk(void)

FUNCTION

This function transmits an UNTALK command onto the serial bus. All

devices previously set to TALK will stop sending data when this

command is received.

INPUTS

RESULTS

EXAMPLE

/*SEND UNTALK COMMAND TO ALL DEVICES */

UnTalk();

BUGS

SEE ALSO

Talk()

iec.library/UnListen                                     iec.library/UnListen

NAME

UnListen -- Command serial bus to UNLISTEN

SYNOPSIS

UnListen()

void UnListen(void)

FUNCTION

This function commands all devices on the serial bus to stop

receiving data from the computer (i.e., UNLISTEN). Calling this

function results in an UNLISTEN command being transmitted onto the

serial bus. Only devices previously commanded to LISTEN are

affected. This function is normally used after the computer has

finished sending data to external devices. The sending of UNLISTEN

commands listening devices to get off the serial bus so it can be

used for other purposes.

INPUTS

RESULTS

EXAMPLE

/*SEND UNLISTEN COMMAND TO ALL DEVICES */

UnListen();

BUGS

SEE ALSO

Listen()