WARNING:

Use these files for your own risc!

HISTORY:

I tested the Super83b by Craig Arnoldt and Super72_1942 by Trevor Andrews

but the workbench screen depth gadget didn't appeared because the high

frequency of horizontal used (about 26kHz). So I solved make some changes

and I discovered that 1942 has capacity to sync 24.62kHz x 72Hz (scan rates

of Super72 are 23.21 to 24.62kHz and 71 to 72Hz) but causes out of sync when

the Overscan program is used to adjust the screen position.

The solution: the increase horizontal frequency to 25.99kHz.

CONTENTS:

Super72NEW.lha contains:

Super72 driver 40.4 version (Please DON'T change it by 39.9 version!)

Overscan 40.2 version (Please DON'T change it by 39.2 version!)

Super72NEW.doc (Please DON'T change my doc!)

INSTALLATION:

1) Copy the Super72 monitor WITH ITS ICON into the devs/monitors drawer. I

think that you will not have problems to do this...

2) Copy the Overscan and its icon into the prefs drawer.

3) Reset the best world computer (What?...My Wintel? "-No, no. It isn't a

real multitasking machine. Think...")

4) Select Super72 in Overscan program, edit and center the screen clicking

on the arrows. Click on "OK" and "SAVE" buttons.

5) Now you are be able to change to Super72 on ScreenMode and test it using

the SuperHires laced mode.

PROBLEMS:

If you have problems with bad syncs, edit the Super72 tool types, decreases

the TOTCLKS to 0x88 or 0x87 and save it.

If you desire to get less flicker, decreases TOTROWS to 0x156.

You have always to reset to use these changes.

These changes must be released only by this way. I don't advise the use of

MonED program with 1940/42 monitors to work on Super72 driver!

OS2.01+ USERS:

I didn't test it under OS3.0. Try it...

MAGIC WORKBENCH USERS:

Copy all tool types lines to your Magic Workbench Super72 monitor icon.

PRICE$:

This "program" is Giftware so, if you like it I accept gifts like...a 060

accelerator board to my A1200 2MB (yet).

CONTACT & IDEAS:

Mauro Sergio Lourenco

E-mail: lourenco@urbi.com.br

Local:Niteroi-RJ-Brazil 20-Oct-96