TABLE OF CONTENTS

opal.library/ActiveScreen24

opal.library/AmigaPriority

opal.library/AppendCopper24

opal.library/AutoSync24

opal.library/BitPlanetoOV

opal.library/ClearDisplayBottom24

opal.library/ClearPFStencil24

opal.library/ClearPRStencil24

opal.library/ClearQuick24

opal.library/ClearScreen24

opal.library/CloseScreen24

opal.library/Config24

opal.library/CreateScreen24

opal.library/DisablePRStencil24

opal.library/DisableZDStencil24

opal.library/DisplayFrame24

opal.library/DisplayThumbnail24

opal.library/DrawEllipse24

opal.library/DrawLine24

opal.library/DualDisplay24

opal.library/DualPlayField2

opal.library/EnablePRStencil24

opal.library/EnableZDStencil24

opal.library/FadeIn24

opal.library/FadeOut24

opal.library/FreeScreen24

opal.library/FreezeFrame24

opal.library/ILBMtoOV

opal.library/LatchDisplay24

opal.library/LoadImage24

opal.library/LowMemUpdate24

opal.library/LowMem2Update24

opal.library/LowMemRGB24

opal.library/OpenScreen24

opal.library/OVPriority

opal.library/OVtoBitPlane

opal.library/OVtoILBM

opal.library/OVtoRGB

opal.library/PaletteMap24

opal.library/ReadPFPixel24

opal.library/ReadPixel24

opal.library/ReadPRPixel24

opal.library/RectFill24

opal.library/Refresh24

opal.library/RegWait24

opal.library/RGBtoOV

opal.library/SaveIFF24

opal.library/SaveJPEG24

opal.library/Scroll24

opal.library/SetControlBit24

opal.library/SetCoPro24

opal.library/SetDisplayBottom24

opal.library/SetHires24

opal.library/SetLoadAddress24

opal.library/SetLores24

opal.library/SetPFStencil24

opal.library/SetPRStencil24

opal.library/SetRGB24

opal.library/SetScreen24

opal.library/SetSprite24

opal.library/SingleDisplay24

opal.library/SinglePlayField24

opal.library/StartTransition24

opal.library/StopTransition24

opal.library/StopUpdate24

opal.library/UpdateAll24

opal.library/UpdateCoPro24

opal.library/UpdateDelay24

opal.library/UpdatePalette24

opal.library/UpdatePFStencil24

opal.library/UpdateRegs24()

opal.library/WriteFrame24

opal.library/WritePFPixel24

opal.library/WritePixel24

opal.library/WritePRPixel24

opal.library/WriteThumbnail24

opalreq.library/OpalRequester

opal.library/ActiveScreen24

NAME

ActiveScreen24 -- Provides a pointer to the currently displayed OpalVision screen.

SYNOPSIS

OScrn = ActiveScreen24 (void);

D0

struct OpalScreen *OScrn;

FUNCTION

This function provides a pointer to the currently displayed OpalVision screen. If there is no OpalVision

display active then a null value is returned.

This call is useful for writing background colour cycling or coprocessor effects programs to affect the

currently open screen.

INPUTS

None

RESULT

OScrn	-A pointer to the currently open OpalVision screen, or NULL.

CONSIDERATIONS

Caution must be exercised when dealing with screens owned by another task. Bitplane access should be

avoided unless running cooperative tasks with mutually exclusive bitplane access.

SEE ALSO

OpenScreen24()

opal.library/AmigaPriority

NAME

AmigaPriority -- Gives Amiga graphics priority over OpalVision display.

SYNOPSIS

void AmigaPriority (void);

FUNCTION

This function clears the OVPRI bit of all CoPro instructions which gives Amiga graphics priority over

OpalVision graphics. If a dual display has not been set, only Amiga graphics will be visible.

INPUTS

None

RESULT

None

CONSIDERATIONS

If an Opal display bottom has been set, the coprocessor instructions will not be

modified for that region of the display.

SEE ALSO

OVPriority()

DualDisplay24()

opal.library/AppendCopper24

NAME

AppendCopper24 -- Attaches user copper lists to existing display copper lists.

SYNOPSIS

void AppendCopper24 (CopLists);

A0

UWORD **CopLists[12];

FUNCTION

Up to 12 different Amiga copper lists are used to update the OpalVision memory. This function allows

user copper lists to be attached to the end of each of the lists to enable split screen 24bit displays and

other copper effects.

Each copper list must be terminated with $FFFFFFFE followed by 30 free bytes for linkage code.

After attaching copper lists, the LastWait field in the OpalScreen structure must be initialised with the

last vertical position wait in the attached copper lists.

The VStart field in the OpalScreen structure contains the scan line of the first displayed line for the

screen. To convert a display 'y' coordinate to a vertical copper wait instruction, use VWait = y + VStart.

INPUTS

CopLists	- Pointer to an array of 12 copper list pointers to be joined to the current display copper lists.

RESULT

None

CONSIDERATIONS

All copper lists must reside in chip ram.

SEE ALSO

opal.library/AutoSync24

NAME

AutoSync24 -- Enables auto horizontal synchronisation.

SYNOPSIS

void  AutoSync24 (Sync);

D0

BOOL Sync;

FUNCTION

Enables the OpalVision's auto synchronisation mode (see "Horizontal Synchronisation").  This mode will

be automatically disabled when frame buffer updates are occurring, and re-enabled when they cease.

INPUTS

Sync =	0 = Disable auto syncing,  1 = Enable auto syncing.

RESULT

None

CONSIDERATIONS

SEE ALSO

opal.library/BitPlanetoOV

NAME

BitPlanetoOV -- Converts standard bitplane data to OpalVision format.

SYNOPSIS

void BitPlanetoOV (OScrn, SrcPlanes, SrcWidth, Lines, TopLine, SrcDepth)

A0	  A1	     D0	       D1     D2       D3

struct OpalScreen *OScrn;

UBYTE **SrcPlanes[];

long SrcWidth;

long Lines;

long TopLine;

long SrcDepth;

FUNCTION

Converts bit plane data from the supplied bitplanes into OpalVision memory format and stores this in the

OpalScreen supplied.

The source data will be clipped if it is wider than the destination screen, or will be padded out if it is

narrower.

INPUTS

OScrn 	= Destination OpalScreen.

SrcPlanes 	= A pointer to an array of pointers to source Bitplanes.

SrcWidth	= Byte width of source planes (must be even).

Lines 	= Number of lines to convert.

TopLine 	= Starting line to place destination data.

SrcDepth	= The number of bitplanes in SrcPlanes.

RESULT

None

CONSIDERATIONS

All bitplanes must start on a word boundary, and SrcWidth must be even.

SEE ALSO

OVtoBitPlane()

opal.library/ClearDisplayBottom24

NAME

ClearDisplayBottom24 -- Clears the OpalVision display bottom setting.

SYNOPSIS

void ClearDisplayBottom24 (void)

FUNCTION

Remove the OpalVision display bottom previously set with a call to SetDisplayBottom24().

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

SetDisplayBottom24()

opal.library/ClearPFStencil24

NAME

ClearPFStencil24 -- Clears the PlayField Stencil of the specified screen.

SYNOPSIS

void ClearPFStencil24 (OScrn);

A0

struct OpalScreen *OScrn;

FUNCTION

Clears the playfield stencil (least significant bit of green bank 0) of all of the pixels in the specified

screen.

INPUTS

OScrn	= OpalScreen structure.

RESULT

None

CONSIDERATIONS

This will only have a visible effect if Dual Playfield mode has been set up using DualPlayField24().

SEE ALSO

SetPFStencil24()

DualPlayField24()

SinglePlayField24()

opal.library/ClearPRStencil24

NAME

ClearPRStencil24 -- Clears the Priority Stencil of the specified Screen.

SYNOPSIS

void ClearPRStencil24 (OScrn);

A0

struct OpalScreen *OScrn;

FUNCTION

Clears the priority stencil (least significant bit of blue bank 0) of the all of the pixels in the specified

screen.

INPUTS

OScrn	= OpalScreen structure.

RESULT

None

CONSIDERATIONS

This will only have a visible effect if dual OpalVision/Amiga display mode has been set up using

DualDisplay24().

SEE ALSO

SetPRStencil24()

DualDisplay24()

SingleDisplay24()

opal.library/ClearQuick24

NAME

ClearQuick24 -- Clears OpalVision frame buffer memory.

SYNOPSIS

void ClearQuick24 (void)

FUNCTION

This function clears the frame buffer memory as quickly as possible by enabling a write to all banks of

memory. This function will also zero all bitplanes in memory (see ClearScreen24()). This operation will

take 1 frame to clear any resolution non-interlaced display, and 2 frames for an interlaced display.This

function  acts on the current display screen and cannot be used for virtual screens.

This function is called by OpenScreen24.

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

ClearScreen24()

opal.library/ClearScreen24

NAME

ClearScreen24 -- Clears all bitplanes in a screen.

SYNOPSIS

void ClearScreen24 (OScrn)

A0

struct OpalScreen *OScrn;

FUNCTION

Clear all bitplanes contained in the OpalScreen structure which may be a virtual screen or the display

screen.

This function clears the bitplane memory without updating the frame buffer (unless frame buffer updates

are enabled).

INPUTS

OScrn = Pointer to the Opal screen to be cleared.

RESULT

None

CONSIDERATIONS

SEE ALSO

ClearQuick24()

opal.library/CloseScreen24

NAME

CloseScreen24 -- Stop current display and free resources.

SYNOPSIS

void CloseScreen24 (void);

FUNCTION

This function closes the current displayed screen if it was opened by the current task.

A screen opened by another task can be closed if it was opened with the CLOSEABLE24 flag. Backdrop

and other low priority programs should use the following procedure to open screen.

OScrn = OpenScreen24 (CLOSEABLE24);

.

.

.

.

if (OScrn!=NULL)

{ WaitPort (OScrn->UserPort);

CloseScreen24();

Mesg = GetMsg (OScrn->UserPort);

ReplyMsg (Mesg);

}

The task will be sent a message when another task is trying to open a screen or close down the one

already open. Note that the screen MUST be closed before replying to the message.

An alternative method to create a backdrop is to update the frame buffer, latch the 24 bit display using

LatchDisplay24() and then call CloseScreen24(). The contents of the frame buffer will remain visible

until another task calls OpenScreen24().

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

OpenScreen24()

opal.library/Config24

NAME

Config24 --     Returns the OpalVision hardware configuration.

SYNOPSIS

Config = Config24 (void);

D0

long Config;

FUNCTION

Returns flags indicating the hardware configuration of the 24bit display card, future flags  will give

details on the existence of OpalVision modules such as the Video Roaster Chip and the frame grabber

genlock module.

Current return flags are:

OVCF_OPALVISION	- Display board is an OpalVision card.

OVCF_COLORBURST	- Display board is a ColorBurst.

INPUTS

RESULT

CONSIDERATIONS

SEE ALSO

opal.library/CreateScreen24

NAME

CreateScreen24 -- Creates an arbitrarily sized virtual OpalScreen.

SYNOPSIS

OScrn = CreateScreen24 (ScreenModes,Width,Height)

D0	    D1    D2

struct OpalScreen *OScrn;

long Width;

long Height;

long ScreenModes;

FUNCTION

This function can create an arbitrarily sized OpalScreen in Fast Ram. The bitplanes for the screen are

allocated and an OpalScreen structure initialised, this is the virtual screen equivalent of OpenScreen24().

Once this screen has been opened, all drawing, file and Memory conversion functions can be applied to

this screen, however it cannot be directly displayed. This allows large super bitmap screens to be

allocated in fast ram for manipulation, or to be partially copied to a primary OpalScreen in chip ram for

display (to allow for scrolling).

NOTE: Virtual screens are now  displayable using the LowMemUpdate() function, virtual screens are

therefore recommended when doing one off frame buffer updates (such as the Show24 command) as it

significantly reduces the chip ram requirements

INPUTS

Width	      = Width in pixels of the screen to be opened.

Height	      = Height in pixels of the screen to be opened.

ScreenModes = ScreenModes are identical to those of OpenScreen24().

RESULT

OScrn	= Is a pointer to the new OpalScreen structure or NULL if there is insufficient memory

to open the screen size specified.

CONSIDERATIONS

This function allocates memory with no MEMF_ bits set, the program FastMemFirst should be executed

to force all planes to be loaded into fast ram, under AmigaDOS 1.3 or previous.

SEE ALSO

FreeScreen24()

OpenScreeen24()

opal.library/DisablePRStencil24

NAME

DisablePRStencil24 -- Disables the use of the priority stencil in dual display mode.

SYNOPSIS

void DisablePRStencil24 (void);

FUNCTION

This function clears the PRISTENCIL bit of all CoPro instructions.

INPUTS

None

RESULT

None

CONSIDERATIONS

If an Amiga display bottom has been set using SetDisplayBottom24(), the CoPro instructions will not be

modified for that part of the display.

The priority stencil will only have an effect when a Dual Display is enabled by calling DualDisplay24().

SEE ALSO

EnablePRStencil()

DualDisplay24()

SingleDisplay24()

opal.library/DisableZDStencil24

NAME

DisableZDStencil24 -- Disable the Zero-Detect method of playfield control.

(NOTE: New for V4.0).

SYNOPSIS

void DisableZDStencil24 (void)

FUNCTION

This function disables the zero-detect dual playfield method

enabled using the EnableZDStencil24() function. Once this has

been called, the OpalVision display will revert back to a

single playfield display.

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

StartTransition24()

StopTransition24()

EnableZDStencil()

opal.library/DisplayFrame24

NAME

DisplayFrame24 -- Sets the currently displayed frame within the frame buffer memory.

SYNOPSIS

void DisplayFrame24 (Frame);

D0

long Frame;

FUNCTION

Depending on the resolution of the displayed OpalVision screen, a number of screens can be stored in the

frame buffer memory. The number of frames available for the screens resolution are given in the

MaxFrames field in the OpalScreen structure.

DisplayFrame24() allows each individual frame to be displayed separately where Frame is in the range

0...MaxFrames. Using a combination of WriteFrame24 and DisplayFrame24, it is possible to store

several images in frame buffer memory and to perform simple page flip animation.

INPUTS

Frame	= Frame number to display (0...MaxFrames).

RESULT

None

CONSIDERATIONS

The display frame and the write frame, must reside in the same field area in the frame buffer memory.

(See "Memory Segment Diagram"). Due to this DisplayFrame24() has the side effect of changing the

write frame if the new display frame is in a different field.

SEE ALSO

WriteFrame24()

opal.library/DisplayThumbnail24

NAME

DisplayThumbnail24 -- Displays a file's thumbnail.

SYNOPSIS

ReturnCode = DisplayThumbnail24 (OScrn, FileName, x, y);

D0				 A0     A1	  D0 D1

long ReturnCode;

struct OpalScreen *OScrn;

char *FileName;

long x;

long y;

FUNCTION

This function displays the imbedded thumbnail in the file described by FileName if it exists.

The x coordinate is rounded down to the nearest multiple of four in low resolution mode and to the

nearest multiple of 8 in high resolution mode. The y coordinate is rounded down to the nearest even line

in interlaced mode.

INPUTS

OScrn	= Pointer to an OpalScreen structure.

FileName= The file name of the picture file with the thumbnail.

x	= The x coordinate of the  screen position to display the thumbnail.

y	= The y coordinate of the screen position to display the thumbnail.

RESULT

ReturnCode	= OL_ERR Codes described in Opallib.h.

OL_ERR_NOTHUMBNAIL is returned if no thumbnail exists in the file.

CONSIDERATIONS

Thumbnails must always be displayed in low resolution non interlaced mode. For an example of

displaying thumbnails in a high resolution and interlaced screen, see the example program

"DisplayDir.c"

SEE ALSO

WriteThumbnail24()

SaveIFF24()

opal.library/DrawEllipse24

NAME

DrawEllipse24 -- Draw an ellipse of given dimensions.

SYNOPSIS

void DrawEllipse24 (OScrn,  cx,  cy,  a,  b)

A0	    D0   D1   D2  D3

struct OpalScreen *OScrn;

long cx;

long cy;

long a;

long b;

FUNCTION

Draws an ellipse in the supplied screen.

NOTE: set a=b for circles.

INPUTS

OScrn 	= Destination OpalScreen.

cx 	= Centre x-Coordinate of ellipse.

cy 	= Centre y-Coordinate of ellipse.

a 	= horizontal radius of ellipse (must be >0).

b 	= vertical radius of ellipse (must be >0).

RESULT

CONSIDERATIONS

The ellipse will only be rendered in the region specified by the clip region in the screen structure.

SEE ALSO

opal.library/DrawLine24

NAME

DrawLine24 -- Draws a line into an OpalScreen.

SYNOPSIS

void DrawLine24 (OScrn, x1, y1, x2, y2)

A0     D0  D1  D2  D3

struct OpalScreen *OScrn;

long x1;

long y1;

long x2;

long y2;

FUNCTION

Draws a line in the specified screen structure which may be a virtual or display screen.

In 24bit mode the colour of the line is specified by the Pen_R, Pen_G and Pen_B fields in the OpalScreen

structure. In 15bit mode, Pen_R and Pen_G specify the colour of the line, while in 8bit only Pen_R is

used.

INPUTS

OScrn	= The OpalScreen structure in which to draw.

x1, y1	= The starting co-ordinates of the line.

x2, y2	= The ending co-ordinates of the line.

RESULT

CONSIDERATIONS

The line will only be rendered in the region specified by the clip region in the screen structure.

SEE ALSO

opal.library/DualDisplay24

NAME

DualDisplay24 -- Sets up an Amiga/OpalVision dual display.

SYNOPSIS

void DualDisplay24 (void);

FUNCTION

This function clears the DUALDISPLAY bit of all CoPro instructions, enabling a Dual

Amiga/OpalVision display. The priority of the Amiga/OpalVision graphics can be set with OVPriority()

and AmigaPriority().

INPUTS

None

RESULT

None

CONSIDERATIONS

If an Amiga display bottom has been set, using the SetDisplayBottom24() the CoPro instructions will not

be modified for that region of the display.

SEE ALSO

SingleDisplay24()

AmigaPriority()

OVPriority()

opal.library/DualPlayField2

NAME

DualPlayField24 -- Sets up an OpalVision 24bit dual playfield.

SYNOPSIS

void DualPlayField (void)

FUNCTION

This function sets the DUALPLAYFIELD bit of all CoPro instructions, allowing a dual 24 bit overlay

mode. To determine which bank is displayed for each pixel, the playfield stencil needs to be set

accordingly.

INPUTS

None

RESULT

None

CONSIDERATIONS

If an Amiga display bottom has been set using SetDisplayBottom24(), the coprocessor instructions will

not be modified for that region of the display.

SEE ALSO

SinglePlayField24()

opal.library/EnableZDStencil24

NAME

EnableZDStencil24 -- Enable the Zero-Detect method of playfield control.

(NOTE: New for V4.0).

SYNOPSIS

void EnableZDStencil24 (void)

FUNCTION

The v2.0 EPROM for the OpalVision mother board allows a new

method of performing playfield transitions. This method

is enabled by setting both DualPlayfield and and HiresDisplay

bits, which is performed by this function.

Once the zero-detect stencil is enabled, the Zero-Detect output

from the Amiga is used as a key to switch between the two banks.

This means that wherever the Amiga display is 0 (background

colour) bank 0 will be displayed, and when the Amiga display

is non-0 bank 1 is displayed. The transition can then be

performed using standard graphics library functions and viewports

or intuition screens, the Amiga display only needs to be

1 bitplane deep.

The zero-detect stencil can be hires, although the 24bit display

will always be lores, this can still give a much finer edge on

transitions.

INPUTS

None

RESULT

None

CONSIDERATIONS

Under ECS or AGA the zero-detect logic can be modified using

registers within the Amiga custom chips, this gives even

more flexibility to the zero-detect logic.

Performing transitions using the zero detect logic is much faster

then using the playfield stencil.

SEE ALSO

StartTransition24()

StopTransition24()

DisableZDStencil()

opal.library/EnablePRStencil24

NAME

EnablePRStencil24 -- Enables the use of the priority stencil in dual display mode.

SYNOPSIS

void EnablePRStencil24 (void);

FUNCTION

This function set the PRISTENCIL bit of all CoPro instructions.

INPUTS

None

RESULT

None

CONSIDERATIONS

If an Amiga display bottom has been set using SetDisplayBottom24(), the CoPro instructions will not be

modified for that part of the display.

The priority stencil will only have an effect when a Dual Display is enabled by calling DualDisplay24().

SEE ALSO

DisablePRStencil()

DualDisplay24()

SingleDisplay24()

opal.library/FadeIn24

NAME

FadeIn24 -- Fades display in from black.

SYNOPSIS

void FadeIn24 (Time);

D0

long Time;

FUNCTION

Fade the current display from black to true colour.

The Time parameter specifies the amount of time in 1/100 seconds the fade should take and is

independent of PAL or NTSC refresh rates.

INPUTS

Time = Time in 1/100 seconds to complete fade.

RESULT

None

CONSIDERATIONS

This function cannot be used in 15bit mode.

SEE ALSO

FadeOut24()

opal.library/FadeOut24

NAME

FadeOut24 -- Fade display to black.

SYNOPSIS

void FadeOut24 (Time)

D0

long Time;

FUNCTION

Fade the current display from true colour to black.

The Time parameter specifies the amount of time the fade should take and is independent of PAL or

NTSC machines.

INPUTS

Time = Number of 1/100ths seconds in which to complete the fade.

RESULT

None

CONSIDERATIONS

This function cannot be used in 15bit mode.

SEE ALSO

FadeIn24()

opal.library/FreeScreen24

NAME

FreeScreen24 -- Frees a virtual OpalScreen.

SYNOPSIS

void FreeScreen24 (OScrn);

A0

struct OpalScreen *OScrn;

FUNCTION

This function deallocates all memory associated with a virtual screen. This is the virtual screen

equivalent of CloseScreen24.

INPUTS

OScrn	= A pointer to the virtual screen to be freed

RESULT

None

CONSIDERATIONS

SEE ALSO

CreateScreen24()

CloseScreen24()

opal.library/FreezeFrame24

NAME

FreezeFrame24 -- Freezes the currently displayed screen.

SYNOPSIS

void FreezeFrame24 (Freeze);

D0

BOOL Freeze

FUNCTION

This function freezes the current display. If freeze is TRUE, the display is held static. The display is

returned to normal when the value for freeze is FALSE.

Freeze freezes everything on the display including Amiga graphics.

INPUTS

Freeze	= TRUE (1) = Freeze, FALSE (0) = Unfreeze

RESULT

None

CONSIDERATIONS

This functions is available only while a Scan Rate Converter is present.

SEE ALSO

opal.library/ILBMtoOV

NAME

ILBMtoOV -- Converts interleaved bitmap to OpalVision format.

SYNOPSIS

void ILBMtoOV (OScrn, ILBMData, SrcWidth, Lines, TopLine, SrcPlanes)

A0    A1        D0        D1     D2       D3

struct OpalScreen *OScrn;

UBYTE *ILBMData;

long SrcWidth;

long Lines;

long TopLine;

long SrcPlanes;

FUNCTION

Converts interleaved bitmap memory into OpalVision memory format and stores this in the OpalScreen

supplied.

The source data will be clipped if it is wider than the destination screen, or will be padded out if it is

narrower.

This function is provided to simplify the task of writing a custom IFF loader.

INPUTS

OScrn 		= Destination OpalScreen.

ILBMData 	= interleaved planes of source data.

SourceWidth 	= Width of source ILBM data.

Lines 		= Number of lines to convert.

TopLine 	= Starting line to place destination data.

SrcPlanes	=The number of planes contained in the ILBM data.

RESULT

None

CONSIDERATIONS

SEE ALSO

OVtoILBM()

opal.library/LatchDisplay24

NAME

LatchDisplay24 -- Locks OpalVision  display

SYNOPSIS

void LatchDisplay24 (Latch)

BOOL Latch;

FUNCTION

LatchDisplay24 sets or clears the Latch bit in the control line register. If this bit is set,  the OpalVision

display will remain active regardless of whether there is a valid control line in the Amigas' output. If

CloseScreen24() is called after the latch bit is set, all memory and resources will be freed but the display

will still be active, even if the Amiga is reset.

OpalHotKey uses this technique, images are loaded and updated into the buffer, latched and then the

screen is closed. If any register information needs to be changed, such as changing display priority a

display screen is opened using the CONTROLONLY24 flag which enables registers to be changed

without effecting the contents of the frame buffer.

INPUTS

Latch	= 0 = Free display, 1 = Latch display.

RESULT

None

CONSIDERATIONS

SEE ALSO

opal.library/LoadImage24

NAME

LoadImage24 / LoadIFF24 -- Loads an Image file.

SYNOPSIS

ReturnCode = LoadImage24 (OScrn,FileName,Flags)

D0			  A0    A1	 D0

long ReturnCode;

struct OpalScreen *OScrn;

long Flags;

char *FileName;

FUNCTION

Load an IFF or JPEG file.

This is a general purpose image loading routine which will automatically detect and load IFF and JPEG

files. As this is a general loader, the name of this function has been renamed to LoadImage24(), which is

used as a synonym for the previous function name LoadIFF24() to maintain backward compatibility.

The IFF portion of this loader will load IFF 24bit, Fast Format 24 bit, Palette mapped (up to 256 colours),

Hold and Modify and Extra half brite files.

All palette mapped files will be loaded in the 8 bit palette mapped mode unless the CONVERT24 flag is

set, in which case they will be converted to a non palette mapped 24 bit display.

There are several different forms in which LoadImage24 can load an image. The way in which it

functions is dependant on the specified screen and the flags. If the screen pointer is NULL then

LoadImage24 will open a screen itself, the screen it opens will be a display screen unless

VIRTUALSCREEN24 is set in which case a virtual screen will be created.

If the passed screen structure is not NULL and the image being loaded is the same resolution, then it will

be loaded into that screen. If this is not the case then the screen will be closed and a new screen of the

same resolution as the file will be opened. However if KEEPRES24 is set, the file will be loaded into the

supplied screen regardless of its resolution.

LoadImage24 returns one of two things. If the files was loaded successfully, a pointer to the screen into

which it was loaded is returned. If an error occurred, then an error code will be returned. To determine

which of these messages has been returned, the value can be compared to OL_ERR_MAXERR, if it is

lower than this value then the result is an error code, if it is greater than this number then it is a screen

pointer. If the image is not IFF or JPEG, OL_ERR_FORMATUNKOWN is returned.

Flags:

FORCE24		-  Convert palette mapped files to 24 bit.

KEEPRES24 		-  Keep the same screen resolution.

CLOSEABLE24 		-  Opened screen will be closeable.

LOADMASK24		-  Load mask plane if present (IFF only).

VIRTUALSCREEN24	-  Load image into a virtual screen.

The JPEG loader is a baseline loader as specified in the draft standard ISO/IEC Bis 10918-1 it supports

only 8 bit quantization tables and Huffman entropy compression. It can load files with source colour

space of Y Cb Cr, RGB and Grey scale. It does not support non interleaved files, progressive, hierarchical

or lossless modes.

INPUTS

FileName 	= Filename of image to be display (including path).

Flags 		= see above.

RESULT

ReturnCode = > OL_ERR_MAXERR Return code is a pointer to an Opal screen structure.

ReturnCode = < OL_ERR_MAXERR, Return code indicates error.

CONSIDERATIONS

This function only loads an image, it does not update the frame buffer. To do this you must call

Refresh24() or LowMemUpdate24().

SEE ALSO

SaveIFF24()

SaveJPEG24()

Refresh24()

LowMemUpdate24()

opal.library/LowMemUpdate24

NAME

LowMemUpdate24 -- Low chip ram usage OpalVision update.

SYNOPSIS

RetScrn = LowMemUpdate24 (OScrn, Frame);

D0			  A0	 D0

struct OpalScreen *RetScrn;

struct OpalScreen *OScrn;

long Frame;

FUNCTION

Updates the frame buffer from a virtual screen. This function can update an entire image of any

resolution while only using a small amount of chip ram. This routine uses an 8bit screen to update each

memory segment separately, the CPU is used to copy the bitplane data from the virtual screen to chip

ram. The 8bit plane display screen opened to perform the update is returned, and should be subsequently

closed.

The Frame input sets the first memory segment to be updated, this will normally be 0. This can be set to

6 for example to update a lores screen to bank1  instead of bank0.

NOTE:  OScrn must be a pointer to a virtual screen.

INPUTS

OScrn	= The virtual OpalScreen to be displayed

Frame	= Memory segment to start update (0..11).

RESULT

RetScrn	   >= OL_ERR_MAXERR Return code is a pointer to an Opal screen structure.

RetScrn   < OL_ERR_MAXERR, Return code indicates error.

CONSIDERATIONS

SEE ALSO

LowMem2Update24()

opal.library/LowMem2Update24

NAME

LowMem2Update24 -- Low chip ram usage OpalVision update.

SYNOPSIS

RetScrn = LowMem2Update24 (OScrn, Frame);

D0			   A0     D0

struct OpalScreen *RetScrn;

struct OpalScreen *OScrn;

long Frame;

FUNCTION

Updates the frame buffer from a virtual screen. This function can update an entire image of any

resolution while only using a small amount of chip ram. This routine uses an 8bit screen to update each

memory segment separately, the CPU is used to copy the bitplane data from the virtual screen to chip

ram. The 8bit plane display screen opened to perform the update is returned, and should be subsequently

closed.

The Frame input sets the first memory segment to be updated, this will normally be 0. This can be set to

6 for example to update a lores screen to bank1  instead of bank0.

This function is similar to LowMemUpdate24() although it only updates the frame buffer memory, it does

not modify the display modes,  CoPro bits or palette information. This is very useful for performing

transitions  between two images in lores, the first image can be written into bank1 and displayed using

LowMemUpdate24(OScrn,6), the second image is then updated transparently into bank0 using

LowMem2Update24(OScrn,0). The dual display stencil can then be used to perform the transition

between bank1 and bank0. Note that bank0 is written to last , as only bank0 contains the dual display

stencil.

NOTE:  OScrn must be a pointer to a virtual screen.

INPUTS

OScrn	= The virtual OpalScreen to be displayed

Frame	= Memory segment to start update (0..11).

RESULT

RetScrn	   >= OL_ERR_MAXERR Return code is a pointer to an Opal screen structure.

RetScrn   < OL_ERR_MAXERR, Return code indicates error.

CONSIDERATIONS

SEE ALSO

LowMemUpdate24()

opal.library/LowMemRGB24

NAME

LowMemRGB24 -- Low chip ram usage OpalVision update from an RGB array.

SYNOPSIS

RetScrn = LowMemRGB24 (ScreenModes, Frame, Width, Height, Modulo, RGBPlanes);

D0			D0	    D1	   D2	  D3	  D4      A0

struct OpalScreen *RetScrn;

long ScreenModes,Frame,Width,Height,Modulo;

UBYTE *RGBPlanes[3]

FUNCTION

Updates the frame buffer from RGB byte planes. This function can update an entire image of any

resolution while only using a small amount of chip ram. This routine uses an 8bit screen to update each

memory segment separately, the RGB data is converted into bitplane format one segment at a time and

transferred into the framebuffer. The 8bit plane display screen opened to perform the update is returned,

and should be subsequently closed.

The Modulo parameter allows interleaved RGB data to be updated as well, in this case RGBPlanes would

be initialised 'Width' bytes apart, and modulo would be set to 2*Width.

This function is useful for image processing programs such as ADPro and Imagemaster which store

images in byte planes.

INPUTS

ScreenModes	= See OpenScreen24, these flags enable the resolution and format of the

displayed image to be set.

Frame 		= The memory segment to start the update, (0..1)

Width		= Width of the RGB Array (in pixels).

Height		= Height of the RGB Array.

Modulo		= Modulo to be added after each line in the RGB Array.

RGBPlanes	= Pointers to the three byte planes required (R,G,B).

RESULT

RetScrn	   >= OL_ERR_MAXERR Return code is a pointer to an Opal screen structure.

RetScrn   < OL_ERR_MAXERR, Return code indicates error.

CONSIDERATIONS

SEE ALSO

opal.library/OpenScreen24

NAME

OpenScreen24 -- Allocates all resources and displays an OpalVision screen.

SYNOPSIS

OScrn = OpenScreen24 (ScreenModes)

D0		      D0

struct OpalScreen *OScrn;

long ScreenModes;

FUNCTION

This function creates a display screen and allocates all the resources required to display the screen. The

Frame buffer memory is cleared and updates are disabled to the frame buffer memory.

The screen is positioned according to Amiga preferences, however if the vertical starting position defined

in preferences is too high up, preferences will be modified to set the vertical starting position to the

highest possible, the preferences will be restored when the screen is closed.

The screen will be opened as single playfield, single display mode with OVPriority.

If the CONTROLONLY24 flag is set, the screen will be opened without any bitplanes,  this enables the

copro or palette information of a 'latched on'  to be modified without losing the contents of the frame

buffer.

ScreenModes:

INTERLACE24	- Open an interlaced screen.

HIRES24 	- Open a Hires screen.

OVERSCAN24	- Open an overscan screen.

PLANES15	- 15 bit true colour display.

PLANES8		- 8 bit true colour/palette mapped display.

CLOSEABLE24	- Screen can be closed by another task.

PALMAP24	- Open a palette mapped screen.

CONTROLONLY24	- Open a bitplaneless screen.

Screen Sizes:

Hires	Interlaced	Overscan	PAL	NTSC

No	No		No		320x256 320x200

Yes	No		No		640x256	640x200

No	Yes		No		320x512	320x400

Yes	Yes		No		640x512	640x400

The size of the screen opened will be as specified above unless there is not enough chip ram available, in

which case the maximum amount of lines possible will be displayed. If there is insufficient chip memory,

to hold half of the scan lines, then OpenScreen will be aborted and NULL returned. If the PLANES8 or

PLANES15 flag is not set, a 24 bit screen will be opened.

INPUTS

ScreenModes	=  See above.

RESULT

OScrn	=  A pointer to an OpalScreen structure or NULL if unsuccessful.

CONSIDERATIONS

SEE ALSO

CloseScreen24()

LatchDisplay24()

opal.library/OVPriority

NAME

OVPriority -- Give OpalVision graphics priority over Amiga graphics.

SYNOPSIS

void OVPriority (void);

FUNCTION

This function sets the OVPRI bit of all coprocessor instructions which gives OpalVision graphics priority

over Amiga graphics. If a dual display has not been set, only OpalVision graphics will be visible.

INPUTS

None

RESULT

None

CONSIDERATIONS

If an Amiga display bottom has been set using SetDisplayBottom24(), the coprocessor instructions will

not be modified for that region of the display.

SEE ALSO

AmigaPriority24()

DualDisplay24()

opal.library/OVtoBitPlane

NAME

OVtoBitPlane -- Convert OpalVision bit plane data to standard bitplanes.

SYNOPSIS

void OVtoBitPlane (OScrn, BitPlanes, DestWidth, Lines, TopLine)

A0	  A1	     D0		D1     D2

struct OpalScreen *OScrn;

UBYTE **BitPlanes[];

long DestWidth;

long Lines;

long TopLine;

FUNCTION

Converts OpalVision bitplane format to standard bitplane data. The destination data will be non-

interleaved 24, 15 or 8 planes depending on the type of display OScrn is. Note that the 15bit display

mode is actually stored internally as 16 bitplanes which in turn causes this function to return 16 planes

instead of 15.

DestWidth specifies the width of the destination bitplanes, if the width is less than the source planes, they

will be clipped. If the destination width is larger, the remaining bytes on each scan line will be skipped.

The OpalScreen can be any size, and reside in fast or chip ram.

The array of bitplane pointers passed to this function must contain 8, 16 or 24 entries depending on the

screen type.

INPUTS

OScrn 		= OpalScreen structure describing source data.

BitPlanes	= Array of bitplane pointers to take the destination data.

DestWidth	= Width in bytes of destination planes (must be even).

Lines 		= Total number of scan lines to convert.

TopLine 	= Starting line for conversion within the OpalScreen.

RESULT

None

CONSIDERATIONS

All bitplanes must be on a word boundary and the destination width must be even.

SEE ALSO

BitplanetoOV()

opal.library/OVtoILBM

NAME

OVtoILBM -- Converts OpalVision bit planes to interleaved bitmap format.

SYNOPSIS

void OVtoILBM (OScrn, ILBMData, DestWidth, Lines, TopLine)

A0    A1        D0         D1     D2

struct OpalScreen *OScrn;

UBYTE *ILBMData;

long DestWidth;

long Lines;

long TopLine;

FUNCTION

Converts bitplane information from the supplied OpalScreen, starting at the scan line indicated by

TopLine, into interleaved bitmap format.

If the source OpalScreen is wider than the destination width, the planes will be clipped. If the OpalScreen

is narrower, the extra bytes on each line will be skipped. The OpalScreen can be any size, and reside in

fast or chip ram.

The memory pointed to by ILBMData must be large enough to hold

DestWidth * lines * (8 or 16 or 24 depending on screen type) bytes.

INPUTS

OScrn 		= OpalScreen structure describing source data.

ILBMData 	= Pointer to buffer to hold destination ILBM data.

DestWidth	= Width of destination ILBM planes. (must be even)

Lines 		= Total number of scan lines to convert.

TopLine		= Starting line for conversion within the OpalScreen.

RESULT

None

CONSIDERATIONS

ILBMData must start on a word boundary, and DestWidth must be even.

SEE ALSO

ILBMtoOV()

opal.library/OVtoRGB

NAME

OVtoRGB -- Converts OpalVision bitplane data to three planes of RGB.

SYNOPSIS

void OVtoRGB (OScrn, RGBPlanes[], Top, Left, Width, Height)

A0     A1           D0   D1    D2     D3

struct OpalScreen *OScrn;

UBYTE **RGBPlanes[];

long Top;

long Left;

long Width;

long Height;

FUNCTION

This call converts bitplane data from the OpalScreen into three planes, one containing Red, one Blue and

the last Green, each of these has one byte per pixel. This is useful for making 'brush' cut-outs, or for

subsequent scaling of data.

This function is more flexible than the other memory conversion routines in that it can convert a

rectangular region of bitplane memory positioned anywhere within the source screen.

The OpalScreen can be any size, and reside in fast or chip ram.

INPUTS

OScrn		= OpalScreen structure containing source bitplanes.

RGBPlanes	= Pointer to an array of 3 plane pointers.

Top		= x coordinate of top left hand corner to start conversion.

Left 		= y coordinate of top left hand corner to start conversion.

Width 		= Width in pixels of region to cut.

Height 		= Number of lines to cut.

RESULT

CONSIDERATIONS

The destination planes must be on a word boundary.

SEE ALSO

RGBtoOV()

opal.library/PaletteMap24

NAME

PaletteMap24 -- Enable/Disable palette mapping.

SYNOPSIS

PaletteMap24 (PaletteMap)

D0

BOOL PaletteMap;

FUNCTION

If PaletteMap = TRUE, turn on palette mapping, else turn palette mapping off. This function always

operates on the active display screen.

INPUTS

PaletteMap = True to turn palette mapping on, to turn it off.

RESULT

None

CONSIDERATIONS

This function cannot be used in 15bit mode.

SEE ALSO

SetPalette24()

opal.library/ReadPFPixel24

NAME

ReadPFPixel24 -- Returns the state of a give playfield stencil pixel.

SYNOPSIS

Result = ReadPFPixel24 (OScrn,  x,  y)

D0			A0	D0  D1

long Result;

struct OpalScreen *OScrn;

long x;

long y;

FUNCTION

This function returns 1 if the corresponding playfield stencil pixel is set, or 0 if it is cleared. If the

coordinates are outside of the clip boundary then -1 is returned.

INPUTS

OScrn 	= OpalScreen to be read.

x 	= x Coordinate of pixel to read.

y 	= y Coordinate of pixel to read.

RESULT

Result = 0 if pixel clear , or 1 if pixel set, -1 if pixel is out of range.

CONSIDERATIONS

SEE ALSO

WritePFPixel24()

ClearPFStencil24()

SetPFStencil24()

opal.library/ReadPixel24

NAME

ReadPixel24 -- Returns colour information for a given pixel.

SYNOPSIS

Error = ReadPixel24 (OScrn,  x,  y)

D0		     A0	     D0  D1

long Error;

struct OpalScreen *OScrn;

long x;

long y;

FUNCTION

Return the colour (bit plane) information for a given pixel. If the OpalScreen is in palette mapped mode,

the actual bit plane data (and not the corresponding palette value) will be returned.

The returned value is placed in Red, Green and Blue in the OpalScreen structure while in 24bit modes. In

15bit mode, the colour is returned in Red and Green, while in 8bit mode, the colour is returned in Red.

The GetPen macros can be used to extract the components from the returned value.

Use the macros GetCol24(), GetCol15(), GetCol8() or GetCol8P() for 8 bit palette mapped to return the

pixel value.

If the coordinates are outside the screen's clipping region, Error will be -1, else Error = 0.

This function can operate on any sized screens in chip or fast ram.

INPUTS

OScrn 	= OpalScreen to be read.

x 	= x Coordinate of pixel to read.

y 	= y Coordinate of pixel to read.

RESULT

Error 	= 0 if no error occurred, or -1 if pixel was out of the clipping region.

CONSIDERATIONS

SEE ALSO

WritePixel24()

opal.library/ReadPRPixel24

NAME

ReadPRPixel24 -- Returns the state of a give priority stencil pixel.

SYNOPSIS

Result = ReadPRPixel24 (OScrn,  x,  y)

D0			A0	D0  D1

long Result;

struct OpalScreen *OScrn;

long x;

long y;

FUNCTION

This function returns 1 if the corresponding priority stencil pixel is set, or 0 if it is cleared. If the

coordinates are outside of the clip boundary then -1 is returned.

INPUTS

OScrn 	= OpalScreen to be read.

x 	= x Coordinate of pixel to read.

y	= y Coordinate of pixel to read.

RESULT

Result 	= 0 if pixel clear , or 1 if pixel set, -1 if pixel is out of range.

CONSIDERATIONS

SEE ALSO

WritePRPixel24()

ClearPRStencil24()

SetPRStencil24()

opal.library/RectFill24

NAME

RectFill24 -- Draws a solid rectangle.

SYNOPSIS

void RectFill24 (OScrn, Left, Top, Bottom, Right)

A0	D0    D1   D2      D3

struct OpalScreen *OScrn;

long Left;

long Top;

long Bottom;

long Right;

FUNCTION

Draws a solid rectangle with the colour specified by Pen_R,Pen_G & Pen_B in the OpalScreen structure.

The Rectangle is clipped if all or part of it lies outside the clipping region.

INPUTS

OScrn	= OpalScreen to be rendered into.

Left	= x coordinate of top left-hand corner of the rectangle.

Top	= y coordinate of top left-hand corner of the rectangle.

Bottom	= x coordinate of the bottom right-hand corner of rectangle.

Right	= y coordinate of the bottom right-hand corner of rectangle.

RESULT

None

CONSIDERATIONS

SEE ALSO

opal.library/Refresh24

NAME

Refresh24 -- Refreshes the frame buffer.

SYNOPSIS

void Refresh24 (void)

FUNCTION

Initiates DMA of the currently displayed OpalScreen to the framebuffer. This function will update the

framebuffer in the minimum number of frames required, stop DMA (updates) and return.

This function should be called after any drawing routine, and other routines such as LoadIFF24 which

modify memory, to make the frame buffer (and hence display) consistent with the image in Amiga

memory.

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

UpdateDelay24()

StopUpdate24()

UpdatePFStencil24()

UpdateAll24()

opal.library/RegWait24

NAME

RegWait24 -- Wait for register update to complete.

SYNOPSIS

void RegWait24 (void);

FUNCTION

This function waits for register information to be updated to the OpalVision before returning, or returns

immediately if no updates are pending.

This function is important for synchronizing your program with the OpalVision's update scheme. After

any direct modification of OpalVision registers or after a call to a library function which modifies

registers, this function should be called to allow the update to occur, If this function is not called register

data may be lost.

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

opal.library/RGBtoOV

NAME

RGBtoOV -- Converts three planes of RGB to OpalVision bitplane data.

SYNOPSIS

void RGBtoOV (OScrn, RGBPlanes[], Top, Left, Width, Height)

A0     A1           D0   D1    D2     D3

struct OpalScreen *OScrn;

UBYTE **RGBPlanes[];

long Top;

long Left;

long Width;

long Height;

FUNCTION

This call converts three source planes, one containing Red, one Blue and the last Green into OpalVision

bitplane format. This function is useful for pasting clipped regions (using OVtoRGB) back into

OpalVision memory, or for pasting back data after scaling.

Unlike the other conversion routines, this function is clipped if it is outside of the clipping region, this

enables it to be used as a drawing function rather than a conversion function.

The OpalScreen can be any size, and reside in fast or chip ram.

INPUTS

OScrn		= OpalScreen structure containing destination bitplanes.

RGBPlanes	= Pointer to an array of 3 plane pointers.

Top		= x coordinate of top left hand corner to start conversion.

Left		= y coordinate of top left hand corner to start conversion.

Width		= Width in pixels of region to cut.

Height		= Number of lines to cut.

RESULT

None

CONSIDERATIONS

SEE ALSO

OVtoRGB()

opal.library/SaveIFF24

NAME

SaveIFF24 -- Save an OpalScreen as an IFF file.

SYNOPSIS

Error = SaveIFF24 (OScrn, FileName, ChunkFunction, Flags)

D0                 A0     A1        A2             D0

long Error;

struct OpalScreen *OScrn;

char *FileName;

long (*ChunkFunction)();

long Flags;

FUNCTION

SaveIFF24 will save any sized OpalScreen in normal IFF file format. The chunks written will include

CAMG	- Containing the resolution  (Hires/Interlace/Overscan)

CMAP	- Colour map if in 8bit mode.

CLUT	- Colour lookup tables if in true colour mode.

OVTN	- OpalVision 24bit thumb-nail for display in OpalPaint,

OpalShow and other system software.

BODY	- Standard 24 bit ILBM data using byte run encoding.

If ChunkFunction is not NULL, the function that it points at will be called after the file has been opened

(and FORM ILBM has been written) and before any other chunks have been written. ChunkFunction is

used to insert your own chunks into the IFF file before any of the above chunks. The DOS File Handle for

the open file will be passed to the function on the stack (in standard C calling convention) the chunk

function must return 0 or an error code.

Flags:

OVFASTFORMAT 	- Save as OpalVision fast format.

NOTHUMBNAIL	- Inhibit writing thumb-nail chunk.

SAVEMASK24		- Saves mask plane if one exists.

INPUTS

OScrn		= OpalScreen to be saved.

FileName	= Filename of file to be written (including full path).

ChunkFunction	= Pointer to code to be executed after file is opened.

RESULT

Error	= 0 if no error code, >0 if error occurred.

CONSIDERATIONS

SEE ALSO

LoadImage24()

SaveJPEG24()

opal.library/SaveJPEG24

NAME

SaveJPEG24 -- Save an OpalScreen as a JPEG JFIF file.

SYNOPSIS

Error = SaveJPEG24 (OScrn, FileName, Flags, Quality)

D0		    A0	   A1	     D0     D1

long Error;

struct OpalScreen *OScrn;

char *FileName;

long Flags;

long Quality;

FUNCTION

SaveJPEG24 will save any sized OpalScreen in the JPEG JFIF file format. JPEG is a compression

standard which enables a large amount of compression to be gained on continuous tone images with

minimum loss in image quality. It should be stressed that this compression method is based on

continuous tone images and compression of images with sharp edges may suffer more degradation. For

more details see the JPEG draft  standard ISO/IEC Dis10918-1.

This generates a base line JPEG file using interleaved components, Huffman entropy compression and 8

bit quatization tables. A thumbnail will also be written into the APP0 marker of the JFIF file unless the

NOTHUMBNAIL flag is set.

The quality factor is a percentage value (0...100) which defines the allowable amount of loss in the

compressed image. A factor of 100 corresponds to a quantization table of all 1's and hence has no

quantization loss. A value of 50 corresponds to the quantization tables suggested by the draft standard as

being acceptable for good image quality. A reasonable default value to use is 75, using this level for

continuous tone scanned images a compression factor of  between 15:1 and 20:1 is typical.

Flags:

NOTHUMBNAIL	- Inhibit writing thumb-nail chunk.

INPUTS

OScrn	= OpalScreen to be saved.

FileName= Filename of file to be written (including full path).

Flags	= See above.

Quality	= (0...100) This determines the amount of loss allowed in the compression of

the image. 100 % corresponds to minimum loss.

RESULT

Error		= 0 if no error code, >0 if error occurred.

CONSIDERATIONS

SEE ALSO

LoadImage24()

opal.library/Scroll24

NAME

Scroll24 -- Scrolls currently displayed OpalVision image.

SYNOPSIS

void Scroll24 (DeltaX, DeltaY)

D0     D1

long DeltaX;

long DeltaY;

FUNCTION

This function scrolls the currently displayed image by DeltaX pixels horizontally, and DeltaY lines

vertically, by modifying the video load address register in the OpalVision.

DeltaX and DeltaY are signed values, to enable scrolling in all directions.

This function also clears the ADDLOAD bit on the first CoPro instruction if it is not already cleared.

INPUTS

DeltaX	= Number of pixels to scroll horizontally.

DeltaY	= Number of lines to scroll vertically.

RESULT

None

CONSIDERATIONS

For the Scroll to function correctly, update DMA to the framebuffer must be turned off by calling

StopUpdate24().

SEE ALSO

SetLoadAddress24()

opal.library/SetControlBit24

NAME

SetControlBit24 -- Modifies a bit in the control line register.

SYNOPSIS

void SetControlBit24 (FrameNumber, BitNumber, State)

D0	   D1	      D2

long FrameNumber;

long BitNumber;

BOOL State;

FUNCTION

Sets or clears a bit in the control line register. See "The Opal Control Line Register" for details.

There are 14 different versions of the control line register used to update the maximum of 12 different

memory segments. These differ by the state of the bank and field write enable bits. The frame number

variable specifies which one of these registers should be updated, for bits such as AUTO or COL/CoPro a

global change may be required (i.e. changing all 12 control lines).

Frame Number		Description

0		Red Bank0, Field0 Update

1		Green Bank0, Field0 Update

2		Blue Bank0, Field0 Update

3		Red Bank0, Field1 Update

4		Green Bank0, Field1 Update

5		Blue Bank0, Field1 Update

6		Red Bank1, Field0 Update

7		Green Bank1, Field0 Update

8		Blue Bank1, Field0 Update

9		Red Bank1, Field1 Update

10		Green Bank1, Field1 Update

11		Blue Bank1, Field1 Update

12		Field 0 Display only

13		Field 1 Display only

INPUTS

FrameNumber	= The OpalVision update frame number to modify. One frame corresponds to

one bank update (maximum 12 frames, 2 noupdate lists).

BitNumber	= Bit number within control line to modify (4...19).

State 		= State to be written into bit (Boolean).

RESULT

None

CONSIDERATIONS

These bits should be modified with caution.

SEE ALSO

Control Line Register

opal.library/SetCoPro24

NAME

SetCoPro24 -- Modifies a single instruction in the CoPro list.

SYNOPSIS

void SetCoPro24 (InstructionNumber, Instruction);

D0		    D1

long InstructionNumber;

long Instruction;

FUNCTION

This function modifies a single CoPro instruction and initiates an update to the OpalVision CoPro. Note

that this function is much faster than calling UpdateCoPro24().

INPUTS

InstructionNumber	= The CoPro instruction number (0...289)

Instruction		= 8Bit CoPro instruction. See "The CoPro"

RESULT

None

CONSIDERATIONS

InstructionNumber should be less than LastCoProIns in the OpalScreen structure.

SEE ALSO

UpdateCoPro24()

opal.library/SetDisplayBottom24

NAME

SetDisplayBottom24 -- Sets the lower limit of the OpalVision screen.

SYNOPSIS

Result = SetDisplayBottom24 (BottomLine);

D0

long BottomLine;

BOOL Result;

FUNCTION

This function specifies the lower limit of the OpalVision screen. Below this point Amiga only graphics

will be displayed. Once a display bottom has been set, the region below that line will always contains

Amiga graphics regardless of whether the frame buffer is being updated or not. This is useful for

displaying Amiga gadgets on the screen.

INPUTS

BottomLine	-Specifies the last line of OpalVision graphics.

RESULT

Result 		= 1 if operation successful, 0 if operation failed.

CONSIDERATIONS

This function uses the CoPro to enable Amiga graphics on the bottom section of the screen. To ensure

that the display is not corrupted, only CoPro instructions up to the line specified by LastCoProIns in the

OpalScreen structure should be modified.

SEE ALSO

ClearDisplayBottom24 ()

opal.library/SetHires24

NAME

SetHires24 -- Enable a hires display for a section of the screen.

SYNOPSIS

Result = SetHires24 (TopLine, Lines);

D0       D1

long TopLine;

long Lines;

FUNCTION

Sets the HIRESDISP bits on CoPro instructions starting at TopLine for 'Lines' number of lines. Both

TopLine and Lines must be specified as a non-interlaced scan line (i.e. must be divided by 2 if an

interlaced screen).

INPUTS

TopLine		-Specifies the first line to start setting HIRESDISP bits.

Lines		-Number of lines to modify.

RESULT

None

CONSIDERATIONS

SEE ALSO

SetLores24 ()

opal.library/SetLoadAddress24

NAME

SetLoadAddress24 -- Updates the OpalVision load address register.

SYNOPSIS

void SetLoadAddress24 (void)

FUNCTION

This function uses the Load Address value in the displayed OpalScreen structure to update the load

address register in the OpalVision.

This function is useful for scrolling and distortion effects.

The modulo for a scan line is given in the OpalScreen structure and is independent of the display

resolution.

INPUTS

None

RESULT

None

CONSIDERATIONS

Load Address only has an effect when a CoPro instruction having its ADDLOAD bit cleared is executed.

Therefore a combination of CoPro instructions and the address load register are required to produce the

effect.

SEE ALSO

Scroll24()

opal.library/SetLores24

NAME

SetLores24 -- Enable a Lores display for a section of the screen.

SYNOPSIS

Result = SetLores24 (TopLine, Lines);

D0       D1

long TopLine;

long Lines;

FUNCTION

Clears the HIRESDISP bits on CoPro instructions starting at TopLine for 'Lines' number of lines. Both

TopLine and Lines must be specified as a non-interlaced scan line (i.e. must be divided by 2 if an

interlaced screen).

INPUTS

TopLine		-Specifies the first line to start clearing HIRESDISP bits.

Lines		-Number of lines to modify.

RESULT

None

CONSIDERATIONS

SEE ALSO

SetHires24 ()

opal.library/SetPFStencil24

NAME

SetPFStencil24 -- Sets the PlayField Stencil of the specified Screen.

SYNOPSIS

void SetPFStencil24 (OScrn);

A0

struct OpalScreen *OScrn;

FUNCTION

Sets the playfield stencil (least significant bit of green bank 0) of all of the pixels in the specified screen.

INPUTS

OScrn	= OpalScreen structure.

RESULT

None

CONSIDERATIONS

This will only have an effect if Dual Playfield mode has been set up using DualPlayField24().

SEE ALSO

ClearPFStencil()

DualPlayField24()

SinglePlayField24()

opal.library/SetPRStencil24

NAME

SetPRStencil24 -- Sets the Priority Stencil of the specified Screen.

SYNOPSIS

void SetPRStencil24 (OScrn);

A0

struct OpalScreen *OScrn;

FUNCTION

Sets the priority stencil (least significant bit of blue bank 0) of all pixels in the specified screen.

INPUTS

OScrn	= OpalScreen structure.

RESULT

None

CONSIDERATIONS

This will only have an effect if dual OpalVision/Amiga display mode has been set up using

DualDisplay24().

SEE ALSO

ClearPRStencil()

DualDisplay24()

SingleDisplay24()

opal.library/SetRGB24

NAME

SetRGB24 -- Updates a single palette entry to the OpalVision palette registers.

SYNOPSIS

void SetRGB24 (Entry, Red,  Green,  Blue);

D0    D1    D2      D3

long Entry;

long Red;

long Green;

long Blue;

FUNCTION

This function updates a single palette entry in the OpalVision palette registers.

INPUTS

Entry	- The entry selected for update (0-255).

Red	- Red value (0-255).

Green	- Green value (0-255).

Blue	- Blue value (0-255).

RESULT

None

CONSIDERATIONS

This function will only have a visible effect when in palette mapped mode.

SEE ALSO

PaletteMap ()

SetPalette ()

opal.library/SetScreen24

NAME

SetScreen24 -- Fills screen with a specified colour.

SYNOPSIS

void SetScreen24 (OScrn)

A0

struct OpalScreen *OScrn;

FUNCTION

This function is similar to ClearScreen24, but fills the screen with the colour contained in Pen_R,Pen_G

& Pen_B in the OpalScreen structure.

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

ClearScreen24()

opal.library/SetSprite24

NAME

SetSprite24 -- Allows Amiga sprites to be displayed over OpalVision graphics.

SYNOPSIS

void SetSprite24 (SpriteData, SpriteNumber);

A0	      D0

USHORT *SpriteData;

long SpriteNumber;

FUNCTION

This function allows Amiga hardware sprites to be displayed in OpalVision graphics.

Sprites are displayed during both display and update cycles and due to this the sprite data is written into

the frame buffer memory along with the video data. This may be undesirable in some cases, so the sprite

may be removed before starting updates using Refresh24() or UpdateDelay24() by calling SetSprite24()

with SpriteData = NULL

SpriteData  is a pointer to a data definition of a spite as passed to the SetPointer() function in the

intuition library. The SpriteNumber is the hardware sprite number to be used to display the sprite, this

will normally be 0 to modify the mouse pointer sprite.

N.B. 	Passing -1 for the SpriteData will use the currently active Amiga Sprite in the system. For

example SetSprite24((USHORT *) - 1,0) will allow the currently active mouse pointer to be displayed

over the 24 bit image.

INPUTS

SpriteData	- pointer to the sprite data.

SpriteNumber	- Amiga hardware sprite number (0...7).

RESULT

None

CONSIDERATIONS

All sprites other than the mouse pointer sprite should be allocated by the user using GetSprite () as sprite

0 is normally used for the mouse pointer and another sprite is required by the opal library for normal

screen updates.

Sprites will only be visible during display cycles if Amiga priority is set and dual display mode is active.

SEE ALSO

opal.library/SingleDisplay24

NAME

SingleDisplay24 -- Sets up an Amiga/OpalVision single display.

SYNOPSIS

void SingleDisplay24 (void);

FUNCTION

This function sets the DUALDISPLAY bit of all CoPro instructions, allowing an OpalVision or Amiga

only display.

INPUTS

None

RESULT

None

CONSIDERATIONS

If Amiga display bottom has been set, using the SetDisplayBottom24() the CoPro instructions will not be

modified for that region of the display.

SEE ALSO

DualDisplay24()

opal.library/SinglePlayField24

NAME

SinglePlayField24() -- Sets up an Amiga or OpalVision single playfield.

SYNOPSIS

void SinglePlayField24 (void)

FUNCTION

This function clears the DUALPLAYFIELD bit of all coprocessor instructions, Allowing only one of

OpalVision playfield to be displayed.

INPUTS

None

RESULT

None

CONSIDERATIONS

If an Amiga display bottom has been set, the coprocessor instructions will not be modified for that region

of the display.

SEE ALSO

DualPlayField24()

opal.library/StartTransition24

NAME

StartTransition24 -- Manages buffer to perform play field transitions.

(NOTE: New for V4.0).

SYNOPSIS

OScrn = StartTransition (VScrn)

struct OpalScreen *OScrn,*VScrn;

FUNCTION

The function sets up the framebuffer to perform a transition. VScrn

is a pointer to a VIRTUAL screen containing the image that you wish

to perform the transition to. VScrn will be updated to bank 0 of the

frambuffer (in lores if the image is hires), and the original contents

of the framebuffer will be in bank 1.

To perform your transition you'll need to enable either the playfield

stencil or the zero detect mode using DualPlayField24() or

EnableZDStencil24(). You can then perform the transition by animating

the stencil plane. Once the transition is completed, StopTransition24

should be called.

INPUTS

VScrn - A pointer to an OpalScreen, which MUST be a virtual screen.

RESULT

OScrn - A pointer to the display screen used for the update, this

will be an 8bit screen regardless of the resolution of

VScrn.

If an error occured, then OScrn will be < OL_ERR_MAXERR.

CONSIDERATIONS

There are alot of issues to be aware of if you wish to perform

transitions, the combination of StartTransition24() and

StopTransition24() will handle all of these for you, it is

strongly suggested that you use these functions.

SEE ALSO

StopTransition24()

EnableZDStencil()

DualPlayField24()

opal.library/StopTransition24

NAME

StopTransition24 -- Manages buffer to perform play field transitions.

(NOTE: New for V4.0).

SYNOPSIS

OScrn = StopTransition (VScrn)

struct OpalScreen *OScrn,*VScrn;

FUNCTION

This function should be called after a transition is performed

using StartTransition24(). This will update bank 1 with either

the remaning portion of the virtual screen or a second copy of

the virtual screen, depending on whether it is lores or hires.

The display will also be initialised to the correct format for

the virtual screen.

INPUTS

VScrn - A pointer to an OpalScreen, which MUST be a virtual screen.

RESULT

OScrn - A pointer to the display screen used for the update, this

will be an 8bit screen regardless of the resolution of

VScrn.

If an error occured, then OScrn will be < OL_ERR_MAXERR.

CONSIDERATIONS

There are alot of issues to be aware of if you wish to perform

transitions, the combination of StartTransition24() and

StopTransition24() will handle all of these for you, it is

strongly suggested that you use these functions.

SEE ALSO

StartTransition24()

DisableZDStencil()

SinglePlayField24()

opal.library/StopUpdate24

NAME

StopUpdate24 -- Stops updates to the frame buffer memory.

SYNOPSIS

void StopUpdate24 (void);

FUNCTION

This function stops updates to the OpalVision frame buffer memory initiated with a call to

UpdateDelay24(). This allows changes to be made to the Amiga memory without affecting the

OpalVision frame buffer memory thus offering inherent double buffering. Stopping updates will also

reduce the DMA load on the Amiga.

INPUTS

None

RESULT

None

CONSIDERATIONS

This function may take up to 1 frame as it must wait for the current frame update to be completed.

SEE ALSO

UpdateDelay24()

Refresh24()

opal.library/UpdateAll24

NAME

UpdateAll24 -- Resets the internal update structure so all required banks are updated.

SYNOPSIS

void UpdateAll24 (void);

FUNCTION

Resets the internal update structure so that all required banks are updated.  This function is useful after a

call to UpdatePFStencil24() to reinitialise the internal state of the library so that all the required segments

are updated correctly on subsequent calls to Refresh24() or UpdateDelay24().

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

UpdatePFStencil24()

opal.library/UpdateCoPro24

NAME

UpdateCoPro24() -- Writes CoPro list for the current display screen to Video coprocessor

SYNOPSIS

void UpdateCoPro24 (void);

FUNCTION

Encodes the entire CoPro instruction list from the displayed screen structure and initiates a coprocessor

update.

This function also updates the Load Address Register.

INPUTS

None

RESULT

None

CONSIDERATIONS

Modifying the coprocessor list in the screen structure does not have any effect on the display until

UpdateCoPro24 () is called.

The CoPro list will not be updated in the OpalVision until the next vertical blanking period.

SEE ALSO

SetCoPro24()

RegWait24()

opal.library/UpdateDelay24

NAME

UpdateDelay24 () -- Sets the delay between consecutive frame buffer updates.

SYNOPSIS

void UpdateDelay24 (FrameDelay);

D0

long FrameDelay;

FUNCTION

This function allows a variable frame delay between consecutive frame buffer updates. Setting a frame

delay of zero enables continuous full speed updates.

This function also initiates continuous updates to the OpalVision frame buffer memory which will

continue until either Refresh24() or StopUpdate24() is called.

Setting a delay increases free bus DMA bandwidth to increase performance of the CPU and other DMA

devices.

INPUTS

FrameDelay	= Number of Frames to pause between frame buffer updates.

RESULT

None

CONSIDERATIONS

UpdateAll24() and UpdatePFStencil24() determine which memory segments will be updated during an

update sequence.

SEE ALSO

StopUpdate24()

UpdateAll24()

UpdatePFStencil24()

Refresh24()

opal.library/UpdatePalette24

NAME

UpdatePalette 24 -- Loads all 256 entries of Red, Green and Blue values in the OpalScreen structure onto

the OpalVision  palette registers.

SYNOPSIS

void UpdatePalette (void);

FUNCTION

Updates the OpalVision palette registers with the palette values in the OpalScreen structure.

This also updates the Pixel Read mask and the Command Register and uses the Palette Load Address as

an offset for the palette update.

INPUTS

None

RESULT

None

CONSIDERATIONS

Updates will have no effect in non palette mapped modes.

SEE ALSO

SetRGB24 ()

PaletteMap24 ()

opal.library/UpdatePFStencil24

NAME

UpdatePFStencil24() -- Updates playfield stencil at highest possible rate.

SYNOPSIS

void UpdatePFStencil24(void);

FUNCTION

Enables updates to only the segments containing the playfield stencil (green segments). The speed of

update is three times that of a normal 24bit update. This enables quick playfield transitions.

This function does not update the playfield stencil as such, but modifies the internal state of the library so

that subsequent calls to Refresh24() or UpdateDelay24() will only update the segments containing the

playfield stencil. The internal state of the library can be returned to normal by calling UpdateAll24().

To use the playfield stencil in 8bit mode, the green bank contains the stencil and therefore must be

updated. The most convenient way to do this is to write the frame for the first playfield in the red

segment of bank 0 using WriteFrame24(0) and the second playfield into the red segment of bank 1 using

WriteFrame24(3). UpdatePFStencil24() will call WriteFrame24(1) when in 8bit mode to switch to the

green segment. Placing you playfields in segments other than the green segment will give you the full

256 colours rather than 128.

INPUTS

None

RESULT

None

CONSIDERATIONS

SEE ALSO

UpdateDelay24 ()

DualPlayField24()

SinglePlayField24()

Refresh24()

UpdateAll24()

opal.library/UpdateRegs24()

NAME

UpdateRegs24 () -- Updates the hardware registers for the current display screen

SYNOPSIS

void UpdateRegs24 (void)

FUNCTION

Updates the Pixel Read mask, Command register and Palette Load Address registers in the OpalVision

with the values from the current display screen structure (See "Registers").

INPUTS

None

RESULT

None

CONSIDERATIONS

Changing register values in the screen structure does not take effect until an update has been initiated.

Register updates are not completed until the next vertical blanking period.

SEE ALSO

RegWait24()

opal.library/WriteFrame24

NAME

WriteFrame24 -- Sets the current frame to be written to within the frame buffer memory.

SYNOPSIS

void WriteFrame24 (Frame);

D0

long Frame;

FUNCTION

Depending on the resolution of the displayed OpalVision screen, a number of screens can be stored in the

frame buffer memory. The number of frames available for the screens resolution are given in the

MaxFrames variable in the OpalScreen structure.

WriteFrame24() allows each individual frame to be written separately where Frame is in the range

0...MaxFrames. Using a combination of WriteFrame24 and DisplayFrame24, it is possible to store

several images in frame buffer memory and to perform simple page flip animation.

INPUTS

Frame	= Frame number to display (0...MaxFrames).

RESULT

None

CONSIDERATIONS

The display frame and the write frame, must reside in the same field area in the frame buffer memory.

(See "Memory Segment Diagram"). Due to this WriteFrame24() has the side effect of changing the

display frame if the new write frame is in a different field.

SEE ALSO

DisplayFrame24()

opal.library/WritePFPixel24

NAME

WritePFPixel24 () -- Set or clear a pixel in the playfield stencil.

SYNOPSIS

Result = WritePFPixel24 (OScrn, x, y);

D0			 A0     D0 D1

struct OpalScreen *OScrn;

long x;

long y;

long Result;

FUNCTION

Sets or clears a pixel in the playfield stencil depending on the state of Pen_R in the screen structure. If

Pen_R is = 0, the pixel will be cleared else it is set. The SetPFPen macro can be used to initialize Pen_R.

INPUTS

OScrn	= A pointer to an OpalScreen structure.

x	= x coordinate of pixel.

y	= y coordinate of pixel.

RESULT

Result	= -1 if the pixel is outside the clip boundary else 0.

CONSIDERATIONS

This function only has visible effect in dual playfield mode.

SEE ALSO

ReadPFPixel24()

UpdatePFStencil()

DualPlayField24()

SinglePlayfield24()

opal.library/WritePixel24

NAME

WritePixel24 () -- Write a pixel into an OpalScreen.

SYNOPSIS

Result = WritePixel24 (OScrn, x, y);

D0	               A0     D0 D1

struct OpalScreen *OScrn;

long x;

long y;

long Result;

FUNCTION

Writes a pixel in the specified OpalScreen using the current pen value in that structure. The macro

SetPen can be used to initialize pen values correctly.

INPUTS

OScrn	= A pointer to an OpalScreen structure.

x	= x coordinate of pixel.

y	= y coordinate of pixel.

RESULT

Result	= -1 if the pixel is outside the clip boundary else 0.

CONSIDERATIONS

SEE ALSO

ReadPixel24()

opal.library/WritePRPixel24

NAME

WritePRPixel24 () -- Set or clear a pixel in the priority stencil.

SYNOPSIS

Result = WritePRPixel24 (OScrn, x, y);

D0		         A0     D0 D1

struct OpalScreen *OScrn;

long x;

long y;

long Result;

FUNCTION

Sets or clears a pixel in the priority stencil depending on the state of Pen_R in the screen structure. If

Pen_R is = 0, the pixel will be cleared else it is set. The macro SetPRPen can be used to initialize the

state of Pen_R.

INPUTS

OScrn	= A pointer to an OpalScreen structure.

x	= x coordinate of pixel.

y	= y coordinate of pixel.

RESULT

Result	= -1 if the pixel is outside the clip boundary else 0.

CONSIDERATIONS

This function only has visible effect when the priority stencil is enabled.

SEE ALSO

ReadPRPixel24()

EnablePRStencil24()

DisablePRStencil24()

opal.library/WriteThumbnail24

NAME

WriteThumbnail24 -- Writes an IFF thumb-nail chunk into a file.

SYNOPSIS

ReturnCode = WriteThumbnail24 (OScrn, File);

D0				A0    A1

struct OpalScreen *OScrn;

BPTR File;

long ReturnCode;

FUNCTION

This function generates a 24 bit thumb-nail for the given OpalScreen and writes an IFF OVTN thumb-

nail chunk into the given file.

INPUTS

OScrn	= OpalScreen to generate the thumb-nail for.

File	= File Handle of the file to write thumb-nail to.

RESULT

ReturnCode = 0 if all ok, or an OpalVision Error code  if en error occurred.

CONSIDERATIONS

SEE ALSO

SaveIFF24()

LoadImage24()

opalreq.library/OpalRequester

NAME

OpalRequester -- The OpalVision file requester.

SYNOPSIS

ReturnCode = OpalRequester (OReq);

D0			    A0

struct OpalReq *OReq;

FUNCTION

This is the entry point for the OpalVision requester. OReq is a pointer to a properly initialised OpalReq

structure defined in the above sections. The requester will be displayed and handled completely by the

library, when the user has selected a file or hit cancel, this function will return the selected file and

directory name in OReq. OKHit will be cleared if the user hit the Cancel gadget and set otherwise.

INPUTS

OReq	= A pointer to a correctly initialised OpalReq structure.

RESULT

ReturnCode = 0 if all ok

=  OR_ERR_OUTOFMEM if there is not enough memory to display requester

=  OR_ERR_INUSE if the requester is currently in use.

CONSIDERATIONS

For this release the intuition screen used to display the requester

must be hires interlaced, and have atleast 2 bit planes.

SEE ALSO