Making a Scanner Driver

=======================

A scanner driver is made as an Amiga Device.

The Device I/O request

----------------------

The scanner device uses a special I/O request:

struct ScannerIO

{

struct IOStdReq       IOScan;

struct ScannerOptions option;

}

where struct IOStdReq is a standard IO request as defined in exec/io.h and

struct ScannerOptions is defined in the include file scanner.h

Before opening the device BetaScan places a pointer to a string containing

the name of the IO unit where the scanner is connected (for instance

scsi.device) in the IOScan.io_Data field. Then the open device is called:

OpenDevice(scannerName,unit,(struct IORequest *)ScannerIO,0)

where unit is the unit number of the IO unit.

The open routine must do the necessary initializations and then set the fields

in the structure option. This is to tell BetaScan what the scanner is able to

do. You must set the  following fields:

so_vendor;

so_model;

so_version;

so_revision;

so_colorMode;

so_opticResolution;

so_opticResStep;

so_interResolution;

so_interResStep;

double so_docWidth;  /* Paper size in mm */

double so_docLength;

All other fields are optional. Do not touch if the facility is not supported

by your scanner. The structure is filled with zero before the device is opened.

IO Commands

-----------

The device must respond to the following commands:

CMD_START   - Start scanning

The scanning parameters must have been set before (SCANCMD_SET). The

IOScan.io_Data field contains a pointer to a structure

struct ScanInformation

{

UWORD sv_imageWidth;           /* image width in pixels               */

UWORD sv_imageHeight;          /* image height in pixels              */

UWORD sv_bytesPerLine;         /* bytes per line read (excl. color)   */

UWORD sv_Flags;                /* data information flags (set to 0)   */

UWORD sv_xResolution;          /* actual horizontal resolution        */

UWORD sv_yResolution;          /* actual vertical resolution          */

}

Most scanners must do some roundings so that your scanner parameters are

not exactly the actual ones. Fill in the actual parameter values in this

structure.

The field sv_bytesPerLine is normally the same as sv_imageWidth (except for

B/W scanning).

CMD_STOP    - Stop scanning

This command is always send (even if the driver has send all scan lines or

has reported an error).

CMD_READ    - Read one or more scan lines

The IOScan.io_Data field contains a pointer to a data buffer and the

IOScan.io_Length the buffer length.

Copy scan line data to this buffer. The first byte of of a scan line must

be the color: 0 = red, 1 = green and 2 = blue (0 if grey scale or B/W).

The bytes 1,2,...,sv_bytesPerLine+1 are the actual scanned data.

Note that you have to separate the colors (if this is not already done by

the scanner). In this way it is possible to handle 3-pass scanning.

Return the actual number of bytes returned (an integral multiple of

sv_bytesPerLine+1).

SCANCMD_SET - Set scanning parameters (command value defined in scanner.h)

The field IOScan.io_Data contains a pointer to the structure (found in

scanner.h):

struct ScanParameters

{

ULONG  sp_ColorNum;            /* 0: halftone, >0: number of colors  */

double sp_x0;                  /* Scanning frame in mm               */

double sp_y0;

double sp_x1;

double sp_y1;

UWORD  sp_xResolution;         /* Horizontal resolution              */

UWORD  sp_yResolution;         /* Vertical resolution                */

WORD   sp_brightness[3];       /* Brightness for red, green and blue */

WORD   sp_contrast;            /* Contrast                           */

WORD   sp_shadow;              /* shadow adjust                      */

WORD   sp_highlight;           /* highlight adjust                   */

WORD   sp_midtone;             /* midtone adjust                     */

WORD   sp_halftonePattern;     /* halftomePattern                    */

WORD   sp_exposureTime;        /* Exposure time                      */

double sp_gamma;               /* gamma value                        */

ULONG  sp_flags;

}

It is up to you if you want to send the values directly to the scanner or

store them until the CMD_START is send.

Actual development

------------------

To avoid making all the nasty device stuff and to facilitate the testing you

can use the files in the ScannerDev directory (SAS/C 6.57).

By using these files you only have to make a single file with the following

routines:

void openScanner(char* name,int unit,struct ScannerOptions* option,BYTE* status)

This is called when the scanner device is opened.

input:

name:   name of the IO unit

unit:   number of the IO unit

option: pointer to the option structure

status: the return value (io_Error)

void closeScanner(void)

Close the scanner

void setParameter(struct ScanParameters* param,BYTE* status)

Set scanning parameters

input:

param:  pointer to a parameter structure

status: the return value

void startScanning(struct ScanInformation* inform,BYTE* status)

Start the scanning

input:

inform: a pointer to a ScanInformation structure to be filled by you.

status: the return value

void stopScanning(void)

Stop the scanning

void readScanLine(UBYTE* data,ULONG* length,BYTE* status)

Read one ore more scan line(s)

input:

data:   pointer to the data buffer

length: pointer to an ULONG containing the buffer length

fill with actual number at return

status: the return value

An example file is in ScanDev directory (in fact divided into two: ScanmakerE3.c

and Scsi.c). To make a test version execute the command:

smake -f smakeTest

The resulting executable is called scanner. The command template for this is:

scanner [options]

where options are one or more of these:

color=bw|gray|rgb24                 default is: color=rgb24

frame=x0,y0,x1,y1                   upper left and lower right corner in mm

default is: frame=25,50,75,100

resolution=r                        resolution in dpi

default is: resolution=300

brightness=b                        brightness in %

default is scanner-default

contrast=c                          contrast in %

default is scanner-default

shadow=s                            default is scanner-default

highlight=h                         default is scanner-default

midtone=m                           default is scanner-default

pattern=p                           default is scanner-default

time=t                              exposure time

default is scanner-default

gamma=g                             default is: gamma=1.0

compress=on|off                     default is: compress=on

file=<name>                         default is: file=ram:scanner.ilbm

Example commands:

scanner

scanner color=grey frame=0,0,10,10 compress=off file=dh0:test.ilbm

To make a device execute

smake -f smakeDevice

The linker will make 4 warnings about absolute addressing. Don't care.