Program: Epson Stylus Colour 400/440/500 driver

Author:  Peter Hutchison

Date:    13/2/99

Version: 1.7

Requirements: Any 1MB Amiga, WB 1.3 or better

Introduction

------------

This is my sixth driver to be released . This is a colour printer driver for

the 400 series of the Epson Stylus Colour printers. It should also work with

the older Stylus 500.

Important Note

--------------

According to the brochure, the 400 is 'Designed exclusively for Windows using

Epson printer driver technology'. I have used ESC/P2 commands to drive this

printer but may not work in some cases or give unexpected results.

No details of emulation modes are listed.

Features

--------

Colour: CMYK (Colour AND Black)

Resolution: 180 to 1440 dpi (using Microdot).

Supports Microweave in Letter Quality mode.

Aspect:  Portrait and Landscape

Output:  B&W and Colour Text,

B&W, greyscale and colour graphics

Optimised for Speed

Compiled with SAS C 6.58

Now includes version for 68000 and 68020.

Densities

---------

0  180 x 180

1  180 x 180

2  360 x 360

3  360 x 360

4  360 x 360

5  720 x 720

6  720 x 720

7  720 x 720

Paper: 10in for most sizes, 14in for US LEGAL

Recommend densities 0-4 for Word Processing output.

Supported Printers

------------------

Epson Stylus Color II, 500, 400, 440.

Installation

------------

If using TurboPrint or Studio, please disable them as this driver only works

with Workbench's printer.device.

Use the Install_Driver script to install the drivers. For the Printer preferences

program set Single for Paper Type and DIN A4 for Paper Format and set lines to 66,

and margins to around 2 and 80. Select Save.

For PrinterGfx, select Floyd-Steinberg dithering, Integer scaling, Positive

Image, Horz. aspect and Colour and Ignore limits.

To save on ink use Threshold of around 3 and set density to about 4.

If you are experiencing problems with this driver send me email (see below).

The latest version can be downloaded from Aminet or my Web page below, it

also appears on Amiga Format's CD.

History

-------

1.0 First release (developed from 600 driver).

1.1 Removed ESC P command from end of gfx mode string.

1.2 Rewrote init strings. Used ESC! instead of a load of text formatting commands,

Removed ESC P command from render.c as well.

1.3 Removed ESC p 0 command from dospecial.c. Added missing comma in data.c

1.4 Updated init strings in Dospecial.c and Render.c

Added ESC! to render.c and replaced m & n with real values.

In Dospecial.c added ESCr0 to set colour to black.

1.5 In Render.c, changed margins to use 0 and 255 (max) instead of 1 and 80.

The 'P' could be coming from the Right margin command which is repeated

during a print esp in FW. Added LF to end of every graphics row.

Also added page lenght command to Render.c.

1.6 Combined init strings in Render.c. Removed extra LF at end of gfx row.

1.7 Removed left/right margin commands in Render.c

Giftware

--------

This driver is Giftware, you are not obliged to send anything but if you

want to contribute to further driver development for this printer and others

then send me either money (5UKP) or old software (Send me a list first and they

must be originals):

Email: pjhutch@blizzard.u-net.com

Web:   http://www.blizzard.u-net.com

or

Peter Hutchison

75 Lower Skircoat Green

Copley Lane

Halifax

W Yorks

England

HX3 0TG