port-handler v41.3

COPYRIGHT AND DISCLAIMER

port-handler v41 is copyrighted 1998/99 by Stephan Rupprecht. All

rights reserved. This program is freeware, so no financial donations

required. Redistribution allowed if the package is left unchanged.

The author is not responsible for any damage caused by the use or

misuse of this documentation and/or the program(s) it describes.

port-handler is partly based on:

ComHandler.c (C) 1989, 1993 by Ralph Babel, Amiga Guru Book p.643ff

Port-Handler.asm (C) 1986 John A. Toebes, VIII (AmigaLibDisk #20)

REQUIREMENTS

Any Amiga� (compatible) computer with at least OS2.04.

DESCRIPTION

port-handler v41 replaces the original l:port-handler that manages

the AmigaDOS devices SERx:, PAR: and PRT:. In opposite to the original

handler my version allows you to create new dos devices that can

manage the ports of an i/o-board (pre v41 port-handlers didn't support

third-party parallel devices). The following mountlist entry illustrates

how to create a new device called HPAR: that accesses hyperPAR.device

(HyperCOM io-boards by VMC) for reading and writing.

HPAR:

EHandler = L:port-handler

Stacksize = 1024

Priority = 5

Device = hyperPAR.device

Unit = 0

Flags = 0

Baud = 0

#

Please note the 'Baud' field which must be set to 0 for parallel

devices!

INSTALLATION

All you need is to copy the file "port-handler" to your "L:" drawer.

PROBLEMS

Mount v40 and later crashs if you try to mount a EHandler twice!

SPECIAL THANKS GO TO

Stephan Riedelbeck for testing port-handler on his system and debugging.

Jens Gerrit Schr�der for giving me the idea of rewriting the handler.

Thomas Richter for his help with the BCPL magic.

HISTORY

41.0  first public release

41.1  port-handler is a single file now. Thanks to Th. Richter for his

help.

41.2  Compiled/Optimized with GNUC. included dosdrivers for all HyperCOM

boards.

41.2a No changes, just added some new dosdrivers for the ioblix board by

RBM.

41.3  reduced filesize a bit :) A warning about mount v40+ has been

added to the doc. prt:raw didn't work, baud rate and control

information following the device name were ignored (eg. ser:9600/8n1).

AUTHOR

Stephan Rupprecht

Apfeldweg 1

D-44359 Dortmund

Germany

email:

stephan.rupprecht@gmx.de