Triple-Play2.doc

A MIDI-Interface Projekt by Alfred Faust

*******************************************************************************

DISCLAIMER

FIRST OF ALL:

I AND THE ORIGINAL AUTORS ARE NOT HELD LIABLE IF  THERE  ARE  ANY

DAMAGES OR HARD ERRORS WHEN YOU MAKE AND USE THIS TRIPLE-PLAY.

MAKING MONEY WITH THIS PROJEKT IS NOT ALLOWED.

IT IS ADAPTED FROM THE ORIGINAL "TRIPLE-PLAY MIDI-INTERFACE" FROM

"BLUE RIBBON INC." AND FROM A PCB BY RYAN OSMAN.

USE IT AT YOUR OWN RISK !!!

THAT MUST BE ... I'M A POOR MAN ... ;(

This projekt is freeware. That means you can use it without to pay anything. But

I will be very enjoyed, if you send me a mail, when you use it.

It comes with NO warranties.

********************************************************************************

That's a rework of my TriplePlay-Projekt, that was only a rework of the  PCB  of

Ryan Osmans PCB.

As I have tested the original Triple-Play from Ryan Osman with my  MIDI-Hardware

(DX11), it won't work correct. But this was only at the "IN" and the "THRU". The

three "OUT"-channels of this Interface are worked  fine.  So  I've  altered  the

MIDI-IN  and  THRU. Now it works correct. I hope this little alteration can help

other people, with similar problems.

At the first I will also say you a fact, before you try this all. Have you  ever

used  Bars&Pipes  with  16  channels  and  many  notes  ? If your AMIGA have had

problems  with  this,  it  is  not  probable,  that  it  can  handle  the   full

possibilities of 3x16 channels.

I think a 68040 processor with 40 Mhz is the  minimum  of  the  work  with  this

TriplePlay - MIDI - Interface.

But try it. Bars&Pipes have so much possibilities ...

What components you need:

-------------------------

Logical IC:

The label of  the  logical  IC's  are  some  different  by  the  different

companys. But at all: there is a international standard. The last 4 numers

in the label are the same everywhere.

For example:

The IC "4052" we need : MOTOROLA  MC14052B

PHILIPS   HEF4052B

TOSHIBA      4052B

You can see: the last 4 numbers are the same.

Don't use the 74... types (example 74HC4052). This IC's are for only +5  V

power and they will be destroyed by our +12 V .

We need: 1x  4052 (4 channel Multiplexer/Demultiplexer)

1x  4069 (6 gates - (Hex) Inverter)

Optocoupler:

1x CNY 17 or similar

Si-Diode:  2x 1N4148 or similar

Resistors: 1x 220  Ohm

4x  10 KOhm

Capacitors are not needed.

5 DIN-sockets for the MIDI-connections.

1 SUB-D female plug 25 pin

short piece 9 wire-cable (shielded)

What kind of connectors at the PCB you are use, is your choice.

A small METAL case (100x70X30);

-----

!!!It is very important for shielding radiation!!!

Making the PCB

--------------

For making the PCB you need:

- transparentpaper (for the exposuremask)

- a piece of 6 x 10 cm fotopositv PCB-material

- a UV-lamp (a normal flourescent lamp does the same)

- developer for the exposured PCB: NaOH

- corrosive for the developed PCB: FeCl3 or other

First:

Print out the exposure-mask to the transparent paper. It is possible with  a

normal computerprinter in black/white. Use maximal contrast. The best result

you get with a laser-printer.

If you use a inkjet-printer, it is possible, that the  black  color  is  not

real black. It may be, that than there are some difficulties. You can also

make a fotocopy with maximum contrast. The result is like a printout with a

laser-printer.

The #?.ps - files you can print out using "ghostscript" or  "post",  if  you

have no postscript-printer. Then all will be printed in the right messures.

The #?.iff - files  you  can  print  out  with  any  graphic  print  program

("graphic-publisher"  of  "Turboptint", or "Studio" or other). A DTP-Program

is also a good coice. In this way you must a little  experiment  up  to  the

measure-lines matchs the lenght that shown.

The dimensions of the #?.iff-picture are 21 X 8 cm.

I've made the PCB with ProDraw3. If you have this program you can  load  the

ProDraw-file . Then is the printing easier.

If you can't make PCB's by yourself, I can help you. But I'm not a

electronic concern ... ;-)

my e-mail: alfred.j.faust@gmx.de

Second:

Prepare the developer and the corrosivebath.

!!!! CAUTION !!!

The developer and the corrosive are very corrodive.

!!!! USE GLOVES !!!

NaOH :  50g to 1l water

FeCl: 400g to 1l water

Lay the printed mask down  the  PCB-material.  (remove  the  protectionsfoil

before).  Use  the mirrored one with the printcoulor to the PCB. That means,

that it is now unmirrored. Press it down with a glasplate. Than lay it under

the UV-lamp.

exposure time with UV-lamp : 3 min

exposure time with flourecent-lamp : 7-8 min

(a small distance between lamp and PCB is necessary)

After the exposure put the PCB immediately in the developer. In a short time

you  will now see appearing the line wires. When all spaces between the line

wires are free, take the PCB out of the developer and bring it under  water.

Then let it dry accurate. Don't use a cloth, it wipes the line wires.

********************************************************************************

NB.: I tryed to make the mask with a  inkjet-printer.  The  ink  isn't  real

black. So the UV-light wasn't exact shielded at the black lines. So the

developer also destroyed the lines and the PCB was lost.

So I experimented a little. My result:

Make a low concentrated developerbath : 20g NaOH to 1l water

Exposure a short time: UV-lamp only 40 seconds !!!

flourecent-lamp only 90 seconds !!!

Hold the exposured PCB only so long in the developer  bath  up  to  all

lines are clearly to see, then immediately put it under flowing water.

You can put the PCB again in the developer if it is not OK,  and  again

under the water.

Be careful ! In the other way you will lose your work.

But this is only for that reason if you use a inkjet-printer.

********************************************************************************

If the PCB is dry, put it in the  corrosivebath  .  The  time  it  takes  to

cauterize  is  between  10  min and some hours. Look often at the PCB in the

corrosivebath. If you heat up the corrosivebath to 60C  the  cauterize-time

will be very short. Often move your PCB in the bath, that will also minimize

the time.

If all line wires clean cauterized, put the PCB under water to wash  up  all

corrodive.

Third:

Drilling:

- use a drill 1 mm

Forth:

Soldering:

On the PCB you can see how all components shall be connected. Also you  will

see the connections to the PCB. (MIDI-Output, and serial connection)

Use a electric soldering iron with no more than 20 W.

Look accurate before soldering the IC.

___   ___

1 =|    O    |= 16

=|         |=

=|         |=

=|   4052  |=   Top View

=|         |=

=|         |=

=|         |=

8 =|_________|= 9

___   ___

1 =|    O    |= 14

=|         |=

=|         |=

=|   4069  |=   Top View

=|         |=

=|         |=

7 =|_________|= 8

___   ___

1 =|    O    |= 6

=|  CNY17  |=   Top View

3 =|_________|= 4

Use only  so  much  solder  tin  as  necessary.  Use  only  solder  tin  for

electronics.

There are two small jumpers to sold in. At the PCB these are marked with

"JP". Don't forget these!

For the MIDI-Ports use normal DIN-sockets:

View from the back (soldering) - side:

---------------

|             |

|      2      |

|   5     4   |

|             |

|  3       1  |

|             |

|             |

|      S      |

|             |

---------------

S = Shield of the Plug (must be connected to Pin 2)

ATTENTION !

THERE ARE DIFFERENT ORDER OF THE PINS AT THE PCB FOR THE MIDI-PORTS

MIDI-IN      :   5 2 4

THRU         :   4 2 5

MIDI-OUT 1-3 :   4 2 5

Most there are numbers of the pins at the DIN-sockets. Look accurate please.

For the connection to out use soldering pins, or sold in a PCB-plug.

The numbers of the pin-connections of the serial-port-plug you  see  at  the

top of the PCB.

1+7  2  3  4  9  10  20

CONNECT ONLY THIS PINS. SEE, THAT PIN 1 + PIN 7 AT THE SERIAL-PORT-PLUG  ARE

CONNECTED !

LOOK VERY ACCURATE AND COUNT THE PINS FOR SECURITY THREE TIMES ORE MORE.

MOST FOR PIN 9 and 10. THE REASON IS: PIN 9 HAVE +12V PIN 10 HAVE -12V !

IT WILL BE VERY DANGEROS IF YOU EXCHANGE THESE TWO PINS. THE  IC's  WILL  BE

DAMAGED.

At last build the PCB and  the  DIN-sockets  in  the  metal  case.  For  the

connection  to  the serial-port I made a short cable with the female plug at

the one side.

Before you connect or disconnet to your AMIGA:

!!!!!   S W I T C H     O F    T H E     P O W E R    !!!!!

IT IS VERY EASY TO DESTROY THE PORT IC's, WHEN YOU MAKE ANY

CONNECTION OR DISCONNECTION AT YOUR AMIGA, IF THE POWER IS ON.

The time you loose with a new boot is very good invested in the life of your

AMIGA.

BE WARNED ....

And now good luck and many fun with the Triple-Play-MIDI-Interface ....  :))

############################################################################

The "Driver Tools"  I've  also  put  in  this  package.  They  replaces  the

MIDI-OUT.ptool.

Before you can use the TriplePlay.ptools you must remove the  MIDI-Out.ptool

in the Toolbox (in the menu of the Toolbox "Remove Tool").

After  that  "Install  Tool..."  -  the three TriplePlay.ptools. Use them as

MIDI-Out-Tools.

Each of them can handle 16 Channels.

That means: Each MIDI-OUT-Port has 16 MIDI-Channels. 3x16 = 48

So you have now real up to 48 MIDI-Channels.

All other functions the same like in the MIDI-Out-Tool.

Alfred Faust

Richbergstr.15

D-34639 Schwarzenborn

Germany

Phone: +49 5686/930276

email: alfred.j.faust@gmx.de or

j.k.dax@t-online.de