---------------------------------------------------------------------------

_________ __

/___  ___// /

CHRISTIAN BUCHNER                     / /   / /

/ /   / /__   _______

PROUDLY PRESENTS               / /   / ___ \ / /__/_/

/ /   / /  / // /

/_/   /_/  /_//_/____

____                      __    ____                       __     __

/ __ \                    / /   / __ \                     / /__  / /

/ /_/ /                   / /   / /_/ /                    / //_/_/ /_

/ _  _/ ____/\ __  __ __  / /   / _  _/ ____/\ /\___   __  / /__ /  __/

/ / \ \ / __/ // / / // _\/ /   / / \ \ / __/ // ___ \ / _\/ // / / /

/ /__/ // /_/ // /_/ // /_/ /   / /__/ // /_/ // /  / // /_/ // / / /

/______//_____//_____//_____/   /______//_____//_/  /_//_____//_/ /_/

THE CHEAPEST WAY TO USE YOUR SERIAL PORT AT LIGHT SPEED

---------------------------------------------------------------------------

BaudBandit Documentation, Version 1.4a, 4th of September 1992, 16:00

---------------------------------------------------------------------------

WHAT'S HOT, WHAT'S NEW?

This  release  introduces  a  modified  GvpPatch  witch  will  now work for

BaudBandit device. This is GvpPatch V1.4 - can anyone send me V1.5?

Version  1.4  should fix most problems that occured with previous versions.

The  nasty  transmission  errors  have been fixed, as well as various other

bugs.  Refer to "Version.Doc" for more accurate information.

Ah,  I  nearly  forgot  to  mention that you will automagically "receive" a

lovely  text every time BaudBandit is opened.  It will be displayed on your

terminal  screen, but it does not affect BaudBandit's function.  Instead it

will affect the number of registrations, I hope. ;-)

The  documentation  has  been  reworked  to clarify some misunderstandings.

Read it again, perhaps you will discover some important details.

I have got an EMAIL address now (see below).

I also have got a lot of new ideas concerning serial communication (see the

section about future plans).

WHAT IS THE BAUD BANDIT?

The BaudBandit is a replacement for the original serial.device allowing you

to  drive  your serial port at baud rates up to 115200 Baud with a standard

68000 Processor in full multitasking.  Without any doubt the BaudBandit can

be  called  "a little revolution" within the telecommunication scene on the

Commodore Amiga.

When  Commodore  developed  the  serial.device some years ago, they were of

course  not able to foresee the development of very cheap modems using data

compression  at incredibly high baud rates like the US Robotics HST and the

new  v32bis  modems.   So  the  serial.device  soon became too slow for the

requirements of modern telecommunication standards.

The BaudBandit is especially designed for use with high speed modems and it

supports  full bidirectional RTS/CTS handshake and receive buffering.  This

device will no longer bring your computer to a standstill when transferring

a  file at 14400 baud or more.  Compared to the serial.device, this program

provides a speed increase of about 500% percent.

HOW TO USE THE DEVICE

Simply  copy  the BaudBandit.device to your DEVS:  directory.  Now you have

to  configure  your  software  to  the  new device.  Most telecommunication

programs offer a configurable device name and unit.  Here you have to enter

"BaudBandit.device".  The device name is case sensitive, so be sure to type

it right as shown above!

The  easiest  way  to install the BaudBandit is to run the program QuickFix

early in your startup sequence.  Then you do not have to change any of your

configurations.   QuickFix  will patch the OpenDevice() vectors so that any

attempt  to open the serial device will open the BaudBandit device instead.

The  patch  may  be  removed at any time with a CTRL-C or the break command

from an AmigaShell.

If  you use an A2232 multiserial card, you may install the program A2232Fix

in  your  startup  sequence.  Do NOT run QuickFix in this case.  When using

A2232Fix,  you do not need to change any of your configurations.  The patch

will  replace  oldser.device  with the faster BaudBandit.device by patching

the  OpenDevice()  vectors.   This allows you to access the internal serial

port  via  A2232's serial.device, but this time using my faster BaudBandit.

Please note that the BaudBandit will NOT speed up the A2232 ports.

WHAT ABOUT THE COMPATIBILITY?

The  code  is designed to work under OS1.2/1.3 as well as OS2.0.  It should

run on all processors from 68000 to 68040.

In  order  to  get  the  best speed results, I dropped some features of the

original serial.device which are usually not needed in telecommunication.

I  completely  dropped  the  xOn/xOff protocoll, as well as the termination

array  and  parity  checks.   Breaks are currently not supported but may be

included in later versions.  Characters must consist of 8 bits.  The device

only supports plain 8N1 RTS/CTS transfer.  But this is all you need!

The  BaudBandit.device  should be able to replace the serial.device in most

situations.   It  has  been  tested  with  Fido  mailers  like TrapDoor and

terminal  programs like Term, NComm and Magicall.  Up- and downloads are no

problem  and especially at high speed transfers you should notice that your

computer  will NOT stand still anymore.  FAXing is possible, too.  This has

been tested with GPFax and other software.

HARDWARE REQUIREMENTS

The  device  supports  a very common handshake method for high speed modems

using  the  RTS/CTS  wires.  The RTS/CTS wires prevent a buffer overflow on

the  computer  and  on  the  modem  side.   This  handshake method is often

referred to as "flow control".

THIS  IS  IMPORTANT:  In order for the device to work properly, you have to

connect  these  wires to your computer, even if your modem does not support

this  handshake method.  Without the RTS/CTS wires connected to your serial

port, the device will not be able to send anything!

WHAT IS INCLUDED IN THIS PACKAGE?

This  archive  contains  an  fully  functional  version of the device and a

complete  documentation.  The device displays a little text on the terminal

screen  reminding you that it is shareware.  If you want to use the program

regularly,  you  have  to  register.

Please  note  that  the  contents  of  the  distribution archive may not be

altered in any way.  Any modification of the program is prohibitted.

WHY IS IT SHAREWARE?

The  development  of  this  programm  has taken many weeks of hard work and

frustration.   Previous  versions  of  BaudBandit were not as reliable as I

wanted  them  to  be,  but  now  I  hope  to  present  a reliable and fully

functional program which will be of great use for many people.

Please understand that I do not want to give this program away for free.  I

ask  you  to pay a small shareware donation.  The more people register, the

more  support  this  product  will  get.  The shareware fee will help me to

support my customers and to continue my work on this project.

HOW TO REGISTER

Before  you decide to register, please check whether this program satisfies

all your requirements or not.  You are allowed to evaluate this program for

two weeks.  If you continue using it, you have to register.

Fill  in the registration form and send it to me.  I think, it is justified

to ask for a shareware donation of DM 20.  This is very cheap for a project

like  BaudBandit  and  it is not even half the price the authors demand for

programs like TrapDoor or Foozle.  There are many ways of sending the money

to me, so don't tell me that you did not know how to do it.

a) Enclose it cash.

b) Send me a cheque over DM 20.

c) Let your bank transfer it to me

d) Transfer it with a postal money order.

Please do not transfer any money before having sent the registration form!

The registration form must be mailed to the following address:

Christian Buchner

Ganghoferstrae 2

8034 Germering

Deutschland

----------------------------------------------------

And always remember: USING it means PAYING it, okay?

----------------------------------------------------

I can not FORCE you, but I may PLEASE you.

(End of File)