DB25.DOC

Information on hardwiring two Amiga's together

By Rich Helms

Serial Interface Connector Specification

--------------------------------------------------

The 25 pin D type connector with sockets (DB25S=female) is used to

interface to RS232C standard signals.  Signal names correspond to

those used in the appendices of the hardware manual when possible.

A500 has a male DB25 socket, A1000 has a female DB25 connector.

WARNING:  Pins 14, 21 and 23 carry power on the A1000. On the A500 and

A2000, pins 9 and 10 carry power. Do not connect to these pins

inadvertantly as they can permanently damage external equipment.

Also, pins 15-18, 23-25 carry non-standard signals and should not be

inadvertantly connected. NEVER use a fully wired 25 line cable!

Full Null Modem Cable

---------------------

System A	    System B

1 ----------------- 1 Frame Ground

2 -----3     3----- 2 TX data		      Cross RX & TX

3 -----2     2----- 3 RX data

4 ---*-8     8-*--- 4 Request to Send	      Tie RTS & CTS cross to CD

5 ---*	       *--- 5 Clear to Send

6 ----20     20---- 6 Data set ready	      Cross DSR to DTR

7 ----------------- 7 Signal Ground

8 -----4     4----- 8 Carrier Detect	      Cross CD to tied RTS & CTS

20 ----6     6----- 20 Data Terminal Ready    Cross DTR to DSR

Tricked Null Modem Cable

------------------------

1 ----------------- 1 Frame Ground

2 -----3     3----- 2 TX data		      Cross RX & TX

3 -----2     2----- 3 RX data

4 ---*	       *--- 4 Request to Send	      Tie RTS & CTS

5 ---*	       *--- 5 Clear to Send

6 ---*	       *--- 6 Data set ready	      Tie DSR, CD & DTR

8 ---*	       *--- 8 Carrier Detect

20 --*	       *--- 20 Data Terminal Ready

7 ----------------- 7 Signal Ground

If using a long shielded cable, connect pin 1 to the shield on one end but

not on the other.

UNDER NO CIRCUMSTANCES IS PIN 1 TO BE CONNECTED TO PIN 7 OR ANY OTHER

GROUND OTHER THAN THE SHIELD ON ONE END OF THE CABLE.

Minimum Null Modem Cable

------------------------

1 ----------------- 1 Frame Ground	      This line is optional

2 -----3     3----- 2 TX data		      Cross RX & TX

3 -----2     2----- 3 RX data

7 ----------------- 7 Signal Ground

Serial Interface Connector Pin Assignment (J6) 1000 (500)

--------------------------------------------------

RS-232-C

pin  name  name  dir	std  notes

1000  500	     (A500 notes)

1   FGND  FGND	 y   frame ground -- do not tie to logic ground

[Common physical equipment ground]

2   TXD   TXD    O	 y   transmit data

[Data sent to DCE]

3   RXD   RXD    I	 y   receive data

[Data received from the DCE]

4   RTS   RTS    O	 y   request to send

[Turns on modem's transmit carrier]

5   CTS   CTS    I	 y   clear to send

[Indicates the modems transmit carrier is on]

6   DSR   DSR    I	 y   data set ready

[Indicates the modem is on]

7   GND   GND	 y   signal ground -- do not tie to frame ground

[Ground for signal carrying circuit]

8   CD    CD     I	 y   carrier detect

[Asserted when the modem hears a carrier]

9   ---   +12V	     (+12V on A500)

10   ---   -12V	     (-12V on A500)

11   ---   AUDO	     (Audio out A500)

12   ---   ---

13   ---   ---

14   -5V   ---	     50 ma maximum   *** WARNING -5V ***

15   AUDO  ---    O	     audio output from left (channels 0, 3) port,

intended to send audio to the modem

16   AUDI  ---    I	     audio input to right (channels 1, 2) port,

intended to receive audio from the modem;

this input is mixed with the analog output

of the right (channels 1, 2). It is not

digitized or used by the computer in any

way.

17   EB    ---    O	     716 KHz clock that supports 68000 peripheral

transfers, intended for modem interface;

this is the buffered version of the E

clock from the 68000.

18   INT2* AUDI   I	     asserting this OPEN COLLECTOR signal will

generate a level 2 interrupt to the 68000

if it is enabled.

(Audio into A500)

19   ---   ---

20   DTR   DTR    O	 y   data terminal ready

[Asserted by terminal when ready to use modem]

21   +5V   ---	     100 ma maximum  *** WARNING +5V ***

22   ---   RI 	 y   (Ring Indicator on A500)

[For electrically detecting a phone call

(auto-answer modems)]

23   +12V  ---	     50 ma maximum   *** WARNING +12V ***

24   C2*   ---    O	     3.58 MHz intended for modems that need a

colorburst clock.

25   RESB* ---    O	     Amiga system reset

Pin Layout

While looking into the connector:

The male RS232C is usually for the DTE (Data Terminal Equipment)

The A500 and A2000 use a male plug.

1   2   3   4   5   6   7   8   9  10  11  12  13

*   *   *   *   *   *   *   *   *   *   *   *   *

*	 *   *	 *   *	 *   *	 *   *	 *   *	 *

14	15  16	17  18	19  20	21  22	23  24	25

The female is usually for the DCE (Data Communications Equipment - modem)

The A1000 uses a female socket.

13  12  11  10   9   8   7   6   5   4   3   2   1

*   *   *   *   *   *   *   *   *   *   *   *   *

*	 *   *	 *   *	 *   *	 *   *	 *   *	 *

25	24  23	22  21	20  19	18  17	16  15	14