- SpeedRamsey.asm - A fifteen minutes hack -  1993 by Holger Lubitz -

*** Sets the skip bit in the Rev G Ramsey ***

Intro

Well, I had played around with the skip bit earlier - but it took some

posts to comp.sys.amiga.hardware by people eagerly waiting for this to

convince me to release it.  Isn't Usenet great ?  :)

The  reason  I didn't release it immediately is that the fastest RAM I

could get had only 70 ns.  The skip mode needs 60 ns or faster RAM for

proper  function.   I have only been able to test this by removing all

banks but one from my Amiga.  If I have all four banks installed, this

doesn't  work.   Probably  the additional capacitive load is enough to

slow down a signal by 1 ns, causing this to break.  I don't know.

However,  if you got 60 ns DRAM, you should be fine even with all four

banks  populated.  In my Amiga, running SpeedRamsey reduces the memory

latency  as measured by AIBB 6.1 from 8.1 to 7.1, for a speed increase

of roughly 10%.

How to use it

Just start it.  From Version 1.2 upwards, you can either start it from

WB  or from the CLI (no interesting icon yet, sorry).  You get version

information,  an  author  notice  and a short message telling you what

SpeedRamsey  did,  if  anything.  If you don't have a Rev G ramsey, it

will complain about that and do nothing.

How it works

This  tiny  little  program  is  a  very  simple  tiny little program.

However,  you  need  to  know about Ramsey to understand what it does.

Ramsey  appears  only  in  the  supervisor  data  space.   It  has two

byte-wide registers, one at $DE0003 and one at $DE0043.  The first one

is read-write, and this is where you configure Ramsey.  The second one

is read-only, it offers version information.

Currently  there  are  only two versions of Ramsey.  The Rev G Ramsey,

which  is  the  revision  you  find  in all Amiga 4000s, allows you to

select a mode called skip mode.  More on this later.

The older Rev D Ramsey used in the A3000 doesn't support the skip mode

this  program  is setting.  It uses the very same bit to select memory

width.   But  because  nobody  ever  used  1Mx1 DRAM in the A3000, and

probably  also  because  bits  in  the only configuration register are

somewhat  precious, the bit got reassigned for the skip mode in Rev G.

This makes it important to check for the right Rev of Ramsey.  I don't

even  want to think about the crash you'd get by setting this bit on a

Rev D Ramsey.  Trying it out is certainly not recommended.

So,  after  we  checked  for Rev G by comparing with $0F (a $0D in the

version  register  signals a Rev D Ramsey, by the way), we fiddle with

the  bit and return from the supervisor mode, having set a return code

for  the  rest  of  the program, which puts out an appropriate message

and,  if started from the CLI, sets the return code (to 0 for bit set,

5 for bit reset, 15 for no Rev G Ramsey found).

Why it works and what skip mode really means

The  skip  mode  speeds  up  RAM  access from five clocks each to four

clocks  each  by  skipping  the  last  cycle.   Some  people call this

60-ns-mode  because  you need 60 ns DRAM or faster to ensure that this

works and doesn't crash your Amiga.

My own experience

You  might  succeed in running this with slower DRAM, depending on how

many  banks  you've  populated  and  who manufactured the SIMMs you're

using.   Some  seem  to have higher safety margins than others.  In my

machine, setting the skip bit with only one bank populated (I am using

Micron 1Mx32 SIMMs with 70 ns access time) works.  Setting it with all

four banks populated crashes my machine (see Intro).

You  can't  damage your Amiga, but be prepared for a cold reboot if it

doesn't  work.  Just try it - but don't have anything important in the

background when you do so.  You have been warned.

Frequently asked questions

* Is the Ramsey setting permanent ?

(here was a slightly incorrect answer in earlier doc revisions because

it wasn't really thought out. Sorry for that)

No,  the  setting  survives  a reset, but has to be set manually after

each  power  on.   You  might want to put it in your User-Startup once

you're  sure  that  your memory is fast enough.  Since SpeedRamsey was

designed  to  toggle  the bit and doesn't support parameters to set or

reset it (yet), you have to trick around a bit.  Here's how:

speedramsey >nil: ; toggles the bit, sets warn if reset

if warn

speedramsey >nil: ; if reset, toggles it again

endif

I agree that this is not nice, but it works for now.

It  isn't  really  possible  to  make  the setting permanent (in other

words,   survive  a  cold  reboot).   However,  the  functionality  of

SpeedRamsey  might  get included in the OS one day, who knows (I could

imagine setting the bit, testing if the RAM still works, and resetting

it  if  it  doesn't,  but  you  can  probably only do that from ROM or

Chipram - ah, wait, now THEREs an idea - maybe for a future version of

SpeedRamsey, right now I'm very busy updating the docs :)

* Can I damage my machine by using SpeedRamsey ?

No.  At least not as far as I know.  If it crashes, just switch it off

and on again.

* Does this work on my (insert specific A4000 model) ?

This  should  work on any A4000 with 60 ns DRAM.  It doesn't matter if

you  have  a 030 or 040 model.  Only one person with 4 banks and 60 ns

DRAM reported a crash.  I'd really like to find out why.

* I have only slower DRAM - should I try it anyway ?

See  question  2.   Just  try it out, you can't damage your machine by

doing so.  If it works, fine (use a RAM test program just to be sure),

if it doesn't switch it off and archive SpeedRamsey for later use when

you've gotten faster DRAM.

Distribution

Permission  for  non-commercial distribution of the unmodified archive

is  hereby  granted.   It  must contain SpeedRamsey itself, its source

code   SpeedRamsey.asm,   this  doc  file  SpeedRamsey.doc,  the  icon

SpeedRamsey.info  and the questionnaire SpeedRamsey.survey, and _only_

those  (_no_  stupid  .displaymes  in archives with my software!) Fred

Fish is explicitly allowed to include this in his library.

If you release anything based on this source, please give credit.

Bugs, Suggestions, etc.

You   can   contact   me  electronically  by  using  my  eMail-Address

hal@caty.north.de.   If  you don't have eMail yet, get access to it or

lose.  After all, we are in the nineties now, aren't we ?

History

1.0  (12-Aug-1993) First Usenet release

The original SpeedRamsey.  Size of program:  72 bytes.

1.1  (13-Aug-1993) Second Usenet release

Mainly updated docs because of questions I got by eMail.

Also  made  program slightly shorter, SpeedRamsey now checks for Rev G

and  only  sets  the  bit on Rev G.  It used to do so on any Non-Rev-D

Ramsey, but who knows what this bit will do in the future ?  Optimised

code slightly when changing this.

The  bit  is  now  flipped  instead of set.  Running SpeedRamsey again

resets  the  system  to  the  old  state.   However, you still have no

indication  what  happened  other  than  checking  by  running AIBB or

similar.  I really need to put in messages soon :)

Size of program:  68 bytes.  (Isn't assembler great ?)

(14-Aug-1993) Not released

Well, I just _had_ to include the messages.  Now SpeedRamsey tells you

what  it  did.   It  even tells you if you haven't got a Rev G Ramsey.

Also added WB startup code that opens an own window to write into.

Size  of  program  with text and startup stuff:  Still only 396 bytes,

but pretty large compared to 1.1 - additional comfort has its price :)

(15-Aug-1993) Not released

Slight changes to the documentation

1.2  (16-Aug-1993) Third (and last) Usenet Release

Updated FAQ list.  Some more doc changes.  Added questionnaire because

one  person  reported a crash with 60 ns DRAM.  Please fill it out and

return  it,  regardless  if SpeedRamsey worked for you or not.  I need

that  information  to  find  out under which conditions SpeedRamsey is

safe and under which it is not.

1.2a (17-Aug-1993) First Aminet Release

Corrected  FAQ  list.   The  first  answer was not totally correct and

could  be  misunderstood,  too.   Also offers a workaround for putting

SpeedRamsey into the User-Startup now.

SpeedRamsey now contains version information, which increased its size

slightly.  Size of program: 408 bytes.

To do list

Parameters for explicit setting/resetting of the skip mode bit

function  check  if  possible  (I'll  have  to  look into this)

maybe setting/resetting the other Ramsey bits.