Implementation Manual for

YAM (Yet Another Modem)

by Chuck Forsberg

(Converted for MS-DOS and Lattice-C)

by Olen Pederson

1. FILE TRANSFER PROTOCOL

The transmission of pathnames in batch mode differs from that

used in MODEM7.  Yam sends the pathname as a null terminated

string in a standard Ward Christensen type packet with a

sector number of 0.

The sender may append the following additional information

starting one byte after the null that terminates the

pathname:

sprintf(p, "%lu %lo %o", f.stsize, f.stmtime, f.stmode);

where stsize is the exact length of the file in bytes,

stmtime is the modification date in seconds since 1970, and

stmode is the file mode, as defined for Unix V7/Sys3.  If

the mod time is unavailable, a 0 value will cause the

receiver to ignore it.  If the file mode is unavailable, a

0  value will cause the receiver to ignore it and use a

default  file mode.

Disk specifiers (B:) are not sent.  Directory names of full

pathnames are not (generally) sent.  If this information

(including the trailing null provided by sprintf) exceeds

the 128 byte standard sector length, a long sector must be

sent.

The sender waits for an initial NAK before sending the

pathname.  An explicit NAK is sent by the receiver at

intervals when is ready to receive a pathname.  Upon

receiving and acknowledging the pathname packet, the

receiver opens the file and then sends the initial NAK for

the data in the file.  A null pathname terminates batch

transmission.

The remainder of the pathname sector is set to nulls.

2. YAM IMPLEMENTATION

Yam consists of the files:

yam.h		header and globals

yamsys.h	installation specific data

yam1.c		First time initialization

Main command decoding

yam2.c		Christensen protocol handler

yam3.c		Conversation with data capture

File xmsn w/o error checking.

yam4.c		CIS A Protocol handler (see MYSYSYTEM )

yam5.c		Misc modem related functions, such as

baudrate

user specific modem init, etc.

yam7.c		Disk i/o open,close, directory

Wild card expansion, O/S related stuff

yam8.asm	CRCK calculation routine.

yam9.asm	CRC calculation routine.

yam10.c		USQ and some time critical functions

yam11.asm	MS-DOS assembly language interface

yamcfg.t	default start up config file

yamphone.t	phone list

yamhelp.t	help file

Yam utilizes a circular buffer for file receive.  Characters

are placed into this buffer by the modem input routine.

This routine places data at bufcq.  Data is removed from the

buffer by using another pointer, bufcdq(console) or

bufpdcq(printer). This method allows the buffer to be filled

faster than it is emptied, as long as it eventually gets a

chance to empty out.

Design goals: A general program performing the functions of

MODEM and XMODEM, easily portable to non-timesharing

computer systems. YAM facilitates communications and s/w

exchange via timesharing, RCP/M, and Bulletin Board systems.

Coding style is/was aimed at portability and clarity. File

transfer routines are written without timing assumptions

allowing error free high speed operation even if view

selected

The MS-DOS implementation of YAM is for the IBM pc and its

lookalikes.  Because of the slow screen output functions of

the IBM-PC, the modem input routine was written as an

interrupt procedure.  This has the added advantage of

allowing transfers of data at high baud rates without

loosing characters.  The IBM version has been tested at 9600

baud with no loss of data.

If your machine uses a different type of UART chip, or a

built-in modem card, then the yamsys.h, yam11.asm and

yam5.c  files will need work.

Special effort was made to not use any of the cute calls for

special graphics capability of the IBM pc.  This results in

a no-nonsense but terse program which can easily be modified

to run on ANY MS-DOS compatible machine.

There are versions of Yam available for the Z89 (with aux

board), Cromemco TUART, TRS-80 II, and Apple (currently with

Z19 console).

There will be multiple versions of yam5.c depending on the

target system.  The file yam7.c will vary between different

operating systems.  Most of the rest of the files should

remain fairly constant.

Compilation and proper operation of this version of YAM is

based on version 2.15e of the Lattice 'C' compiler and

version 2.1 of MS-DOS.  Other versions of YAM may be

available from the BDS C Users Group.

3. PROPAGATING YAM

Often the question arises, "How do I get YAM (or whatever)

into this neat new machine with the *%!@$@ format disks?

One possibility would be to convert the yam-target.com image

into a hex file.  Then run copy on the target machine with

input from the serial port    and output to a file.

Once you have a working yam running on target machine, other

files come across effortlessly with the batch file transfer

option.

4. IMPORTANT #define's

CDO

Define a test for carrier drop out so that XYAM will

automatically exit T(erm) function when the

connection is broken.

CICHAR  Unadorned unconditional console input.  This must

not trap on any control characters needed for host

communications.

CONOUT(c)

Unadorned output of c to the local display.  YAM

checks that the local display is ready before

executing CONOUT(c).  CONOUT(c) must return

immediately regardless of what is in c.

HELPFILE

This defines the file name (usually YAMHELP.T:")

which contains a short summary of commands and

other  useful information to jog your memory when

you forget a command.

IBMPC

If defined, will pull in some IBM-PC specific

routines.

IOTYPE

This defaults to char for i/o mapped systems with

256 ports.  This may be defined to unsigned or long

for systems with more ports (8086 or memory mapped

i/o).

KBDBRK

This defines the keyboard character that translates

to a break in yam3.  It should not be a normally

used character.  Found in yam3.c.

LOGRX

Log all files received from remote systems in the

file LOGRX.  Information stored includes user

number, disk, filename, baudrate, and the name of

the remote system given in the last call command

(LASTCALR in xyam).

LOGTX

Log all files transmitted to remote systems in file

LOGTX.  Information same as above.

LOGFILE

Must be defined if either of the above are.

LPOUT(c)

Likewise, unadorned output of c to the printer.

LPOUT(c) must return immediately regardless of what

is in c.

LPREADY

This define returns non zero if the line printer is

ready to accept a character.  It must always return

0 if LPOUT(c) would not return immediately.  If

POREADY is slow, enabling the printer at high baud

rates may cause characters to be lost.

MOCHAR(c)

This outputs the character c to the modem.  This

macro need not check whether the modem is ready for

a  character as that has already been done.

MODOUT(c) must return immediately regardless of what

is in c.

MYSYSTEM

Defining this causes yam2 to be compiled with a call

to yam4.c for handling the Compuserve A protocol for

file transfer (see 2 mode).  Unless you know what

you are doing, you should use the following even if

it is a bloody lie as xftran may not accept

something original.  If this is included, you will

need yam4.c, otherwise not.

#define MYSYSTEM "#CPMHeath/Zenith,CC,HC,PA,PL\015"

PATHSEP

This is defined as a string which defines the

pathname separators.  This supports those who have

inserted the patch for the DOS SWITCHAR by defining

the PATHSEP as "\/".

PHONES

This defines the file (usually "YAMPHONE.T") which

contains a list of names, phone numbers, and

baudrates. The names must be in lower case.

RESTRICTED

Certain commands and options are disabled. Existing

files cannot be deleted.

RXNONO

Define this as a string of characters, any of which

would cause your terminal and/or cbios grief if not

suppressed by term function.

STATLINE

Define this if your terminal has an extra (viz.,

25th) line usable for status information which does

not scroll up nor interfere with scrolling.

lpstat() writes to the entire status line, clearing

out any previous contents.  pstat() writes starting

at the 48th position, not disturbing any information

written by lpstat().  Lpstat and pstat routines will

need to be appropriately modified.

TERMINIT

This string should unconditionally initialize and

clear the screen.

USQ

Allows the type command to automatically recognize

squeezed files and display them in ASCII form.  Adds

about 4k to program size.

YAMCONFIG

This defines the file (usually "YAMCONFIG.T") which

contains configuration data which YAM will use on

startup to configure itself.

5.  TUNING PARAMETERS

Yam uses a number of tuneable parameters for portability.

The values used depend strongly on the speed of functions

and operating system calls used to interrogate the keyboard

and modem port.  They are also affected by the clock speed

and code quality of the compiler used.  These parameters

have been added on a somewhat ad hoc basis, so one should

grep the source files to determine the extent of their

effects.

CLKMHZ

This is a ``master'' parameter.  Use it to calibrate

the timeout in readchar (see yam5 or equivalent).

LOOPS

Timing value in sleep.

LOWWATER

When receiving text from the host in term function,

term transmits XOFF (DC3) when the free buffer space

gets down to LOWATER.

6.  Recent Revisions/Enhancements to YAM

YAM has undergone a nearly complete re-write as of this

revision.  Some of the major changes have been the extensive

addition of comments and the re-formating of the code so it

is all written in a consistent manner.  As the conversion

from CPM to MS-DOS progressed it became apparent the

conditional compile approach for different operating systems

was unwieldy.  As a result, defines for conditional compiles

were removed.  Nearly all the modules suffered major changes

as the code quality had degraded over the years of several

different authors adding features.  Some commands were

deleted because they either didn't work or served no useful

purpose.   So, here we start again anew.

7.  Wish list

Kermit protocol handler

Terminal emulation

##############################################################################################################