AS8, a Z8 Crossassembler

========================

by:	H.-G. Willers

Gartenstrasse 11

D-8047 Karlsfeld

WEST GERMANY

This document dated 11-jan-90

Intro

=====

This document briefly describes the features

of my crossassembler for the Z8 microprocessor,

called AS8.

This assembler is based on some code found in

a crossassembler for the Z80 microprocessor as

published on the DECUS tape 11-SP-18.

The code was polished up (e.g. adding a hashtable

for the symbols instead of linear search) and

many bugs were fixed.

The resulting code acts as a skeleton for my

assemblers like this one: AS8.

The source code was compiled, using:

o  Mark Williams Let's C (Vers. 3.0.2)

o  Microsoft QuickC (Vers. 1.01)

o  System 5.3 UNIX C for 68020

o  Interactive UNIX 386 3.2 2.1

To compile and link under Mark Williams C the file

'time.c' contains some hacks to simulate the missing

library functions.

The code is provided on an 'as is' basis.

No responsibility for any malfunction of the

assembler per se or any assembled program can

be taken. You use it at your own risk.

Please send comments and improvements to the

author at the above address.

Commandline Syntax

==================

AS8 is invoked by the following commandline:

as8 [-nl] sourcefile.ext

The options mean:

-l	generates a listing named sourcefile.l

-n	do not generate an object file

In the process of producing a name for the

object file or the listing file the extension

of the sourcefile is stripped. Any possible

restrictions of filename use in a given

operating system may apply.

Source Syntax

=============

The syntax of a source line for AS8 is as follows:

label:	opc	src, dst	/ comment

The 'label' is optional; if used, it must be followed

by a colon.

'Opc' is the memonic representation for the

instruction to be used.

The source and destination operands are written

as source first, then destination.

Comments are introduced by the character '/'.

Multiple statements per line are separated by ';',

for example:	nop ; nop ; nop

Symbols

=======

A symbol in as8 is a entity with a label set by

a label or a direct assignment. The number of

characters per symbol is 33 as distributed.

It can be changed by adjusting the defined

constant 'NCPS' to the desired value.

Symbols can consist of the characters of

the ASCII character set, but must begin with either

a-z, A-Z, or the characters '.' or '_' (see

the routine 'isidc(c)' ).

Local Labels

============

There are 9 local labels: 1 - 9. references in the

forward direction are indicated by a 'f', appended

to the label number, references backwards are

indicated by a trailing 'b'.

For example:

1:	nop

sob	w16,1b		/ label 1 backwards

...

call	2f		/ label 2 forward

...

2:	ret

Each local label is reassigned to its new value

when reused.

Number Representation

=====================

The default radix is decimal. The allowed digits

range from 0 - 9.

Base 16 numbers (hex) are denoted by a leading '0x'.

For example: 0x20 is the same as 32.

Base 8 numbers (octal) are denoted by a leading '0'.

For example:  012 is the value of the ascii character CR.

Base 2 numbers are preceeded by a '0B'.

For example:  0B01010101 is the same as 0x55.

Literals

========

Literal constants are enterd by the character ' ,

followed by the desired ASCII character.

Expressions

===========

Expressions are made up from numbers (or symbols)

and arithmetic operators.

Expression evaluation is done from left to right.

This order can be changed by parenthesis with '[ .. ]'.

The binary arithmetic operators are + , -, *, and %.

The operator '%' in AS8 means / ( because comments

are introduced by '/' ).

The bitwise logical operators are

&   bitwise AND

|   bitwise inclusive OR

^   bitwise exclusive OR

>   right shift

<   left shift

The following unary operators are supported:

-   two's complement

!   one's complement

Pseudo Operators

================

.byte	symbol

Generates one byte consisting of the value of 'symbol'.

Example:

.byte	0x20		/ one byte (ascii space)

.word	symbol

Generates one word consisting of the value of 'symbol'.

The word is generated with the byte ordering of the Z8:

nameley the upper 8 bits first (big endian byte ordering).

Example:

.word	0x1000		/ generates the bytes 0x10 and 0

.ascii	string

Generates the named string as a sequence of ascii characters.

The first character of the string is taken as a terminator.

The string ends, when the terminator is found again.

Example:

.ascii	"Hi there!"	/ ascii string

.asciz	string

Generates the named string as a sequence of ascii characters.

The string is followed by a null-character. The first character

of the string is taken as a terminator.	The string ends, when

the terminator is found again.

Example:

.asciz	"Hi there!"	/ ascii string with trailing zero

.blkb	count [, value]

Space for 'count' bytes is left free. If the optional 'value'

is not given, null bytes are taken for filling; bytes

with the 'value' are taken otherwise.

Examples:

.blkb	100, 0x20	/ 100 (decimal) bytes of content 0x20

/ are filled up

.blkb	0x1000		/ 4096 bytes of content 0

.page

The listing is advanced by one page.

.title	string

The title line for the listing is set. The string starts

with a terminator, which also end the string.

Example:

.title	"simple demo"

.hlist

Sets the radix for number output in the listing to

hex.

.olist

Sets the radix for number output in the listing to

octal.

.list

Starts listing generation. This is the default state.

.nlist

Stops listing generation.

.end	symbol

The entrypoint of the loaded program is set to 'symbol'.

The start record of the produced intel-hex-tape is set

to this value.

Example:

.end	_main		/ program starts at _main

Location Counter

================

The location counter is set by the assignment of '.' to

the desired value.

Example:

.	=	0x1000		/ sets the location counter

/ to 0x1000

Addressing Modes

================

(See file z8tst.z8 for clarification)

Register:		r0 - r255

Working Register:	w0 - w15

Register Indirect:	e.g. ...  (r0)

Working Register Ind.:	e.g. ...  (w6)

Indexed:		e.g.	mov  r100(w0),w1

Immediate:		e.g.	add  $1,w0

Direct, relative	indicated by a label

Instruction Description

=======================

AS8	ZILOG      switch in 'xasm'	base opcode

---	-----      ----------------

ccf	ccf		S_OP0		0xef

di	di		S_OP0		0x8f

ei	ei		S_OP0		0x9f

reti	iret		S_OP0		0xbf

nop	nop		S_OP0		0xff

rcf	rcf		S_OP0		0xcf

ret	ret		S_OP0		0xaf

scf	scf		S_OP0		0xdf

srp	srp		S_OP2		0x31

clr	clr		S_OP3		0xb0

com	com		S_OP3		0x60

da	da		S_OP3		0x40

dec	dec		S_OP3		0

pop	pop		S_OP3		0x50

push	push		S_OP3		0x70

rl	rl		S_OP3		0x90

rlc	rlc		S_OP3		0x10

rr	rr		S_OP3		0xe0

rrc	rrc		S_OP3		0xc0

sra	sra		S_OP3		0xd0

swap	swap		S_OP3		0xf0

call	call		S_OP4

jmp	jp  DA		S_OP5		0x8d

jcs	jp  c,DA	S_OP5		0x7d

jcc	jp  nc,DA	S_OP5		0xfd

jeq	jp  z,DA	S_OP5		0x6d

jne	jp  nz,DA	S_OP5		0xed

jpl	jp  pl,DA	S_OP5		0xdd

jmi	jp  mi,DA	S_OP5		0x5d

jov	jp  ov,DA	S_OP5		0x4d

jnov	jp  nov,DA	S_OP5		0xcd

jge	jp  ge,DA	S_OP5		0x9d

jlt	jp  lt,DA	S_OP5		0x1d

jgt	jp  gt,DA	S_OP5		0xad

jle	jp  le,DA	S_OP5		0x2d

juge	jp  uge,DA	S_OP5		0xfd

jult	jp  ult,DA	S_OP5		0x7d

jugt	jp  ugt,DA	S_OP5		0xbd

jule	jp  ule,DA	S_OP5		0x3d

br	jr  DA		S_OP6		0x8b

bcs	jr  c,DA	S_OP6		0x7b

bcc	jr  nc,DA	S_OP6		0xfb

beq	jr  z,DA	S_OP6		0x6b

bne	jr  nz,DA	S_OP6		0xeb

bpl	jr  pl,DA	S_OP6		0xdb

bmi	jr  mi,DA	S_OP6		0x5b

bov	jr  ov,DA	S_OP6		0x4b

bnov	jr  nov,DA	S_OP6		0xcb

bge	jr  ge,DA	S_OP6		0x9b

blt	jr  lt,DA	S_OP6		0x1b

bgt	jr  gt,DA	S_OP6		0xab

ble	jr  le,DA	S_OP6		0x2b

buge	jr  uge,DA	S_OP6		0xfb

bult	jr  ult,DA	S_OP6		0x7b

bugt	jr  ugt,DA	S_OP6		0xbb

bule	jr  ule,DA	S_OP6		0x3b

sob	djnz  wr,RA	S_OP7		0x0a

add	add		S_OP8

adc	adc		S_OP8		0x10

sub	sub		S_OP8		0x20

sbc	sbc		S_OP8		0x30

or	or		S_OP8		0x40

and	and		S_OP8		0x50

tcm	tcm		S_OP8		0x60

tm	tm		S_OP8		0x70

cmp	cmp		S_OP8		0xa0

xor	xor		S_OP8		0xb0

mov	ld  dst,src	S_OP9

incw	incw		S_OP10		0xa0

decw	decw		S_OP10		0x80

inc	inc		S_OP11		0x20

dec	dec		S_OP12

movp	ldc     'move from/to program memory'

movd	lde     'move from/to data memory'

movpi	ldci    'move from/to prog. mem autoincr.

movdi	ldei	'move from/to data mem autoincr.'

Error Messages

==============

Errors in the source code are flagged in the listing

and in the terminal output by a one letter indicator.

This is a list of the error indicators:

a	assembly error

o branches out of range

o 'srp' with wrong value

o illegal addressing modes

e	argument not even (e.g. working reg. pair)

m	symbol multiply defined

o	undefined opcode name

p	phase error: labels do not have the

same values in pass1 and pass2 of assembly

q	questionable syntax:

o digit without ':'

o .ascii or .asciz directive with no args

o '[' and ']' not paired

r	types of arithmetic expression do not

match

t	value of a byte to be output is

out of range

u	symbol undefined

z	object buffer overflow

Disclaimer

==========

The original source code (Z80 crossassembler as found

on DECUS 11-SP-18) is to the best of my knowledge in

the public domain.

My software for AS8 (opcode handling, misc. changes)

is hereby released into the public domain.

All registered trade marks belong to their owners.