WWIV v4 Modem Documentation

Copyright (c) 1988-1991 by Wayne Bell

Because so many people have problems with their modems, and having the

BBS use them, I am writing a separate .DOC file to attempt to better

describe modems, and how they need to be configured in order to run

optimally with WWIV.  Additionally, this describes how WWIV v4.20

handles the modem, and how to add support for additional modem types.

The first thing you need to be aware of is that just about every modem

manufactured for the IBM claims to be "100% Hayes compatible."  This

doesn't actually mean that it is 100% compatible, but only that it is

relatively close (would you buy a modem that is advertised as 99%

compatible?).  With 9600 and faster modems, Hayes compatibility means

very little.  Each modem handles high speed differently, and special

support may be needed for each brand modem.

For external modems, one important feature is your modem cable. In order

to work correctly with WWIV, the following pins need to be connected

from the modem to the computer (all are pin numbers on the DB-25

connected to the modem): 2,3,7,8,20.  If any of these are not connected,

the modem probably won't work correctly, so you'll need a new modem

cable. Fortunately, just about all straight serial cables manufactured

correctly connect these pins.  If your modem cable has sealed connectors

on the ends, don't tear it apart; it is probably fine.  However, If you

made up your own cable (or had someone else make you up one), you might

want to check it out.

For both external and internal modems, the first important setting you

need to have on your modem is the carrier detect.  Most modems have two

settings for carrier detect (or CD).  Either CD is ALWAYS ON, or CD

tracks the state of the carrier.  Using the second setting, WWIV is

correctly able to tell if a remote caller is connected by checking the

CD lead.  But, for some damned fool reason, most modem manufacturers set

it to default to the always on state.

On some modems, there will be a physical switch that you will need to

check (and probably change).  On some modems (such as Hayes external),

you'll need to pop off the face-plate and use a pencil or something to

flip the DIP switch.  Other external modems may have the DIP switches

located on the back panel.  So, pull out your modem manual, and check

which switch affects the CD or DCD lead, and have it set so that it

ISN'T ALWAYS ON.  Internal modems that have DIP switches place the

switches on the modem card.

With some modems, there are "logical" (changed via the keyboard)

switches that determine the CD status.  The keyboard command for the

Prometheus 2400G, for example, is &C1.  The WWIV modem configurations

default to &C1 for all 2400 or faster modems.

The second important setting is the data terminal ready line (DTR),

which is like the CD lead.  The DTR lead is what WWIV uses to send a

'hang up' command to the modem.  Usually, the DTR lead is ON until WWIV

turns the DTR lead OFF, at which time the modem should hang up.  Most

modems can be set so that the modem will completely ignore the DTR

status.  Also, by some quirk in the Universe, it is also usually set by

the manufacturer to ignore DTR status instead of correctly looking at

it.

So, if you have physical switches on your modem, whip out your manual

again, and find which switch you need to check so that the modem will

hang up when DTR is dropped.

For the modems without physical switches, you need to find the command

for that setting.  For the 2400G, the correct command is &D2, which is

described as, "Modem goes on-hook, assumes command state and disable

auto-answer with ON-to-OFF transition."  Fortunately, (again), the WWIV

modem configurations default to setting &D2 for all 2400 or faster

modems.

Contained in the WWIV software ZIP file is a file named MODEMS.MDM.

This file contains setups (or configurations) for most modems.  The .MDM

file is what INIT reads to create a modem data file, which is what the

BBS uses.  If none of the standard configurations in the MODEMS.MDM seem

to work for your speed/ make/type modem, you have several options:

1.  You can 'tweak' an existing configuration in the MODEMS.MDM file

2.  You can build your own configuration and add it to the end of the

MODEMS.MDM file

3.  You can create your own .MDM file (file name can be named any legal DOS

file name, but MUST have the .MDM extension

4.  Write me for help

Modem configurations are placed in the BBS data directory, and the data

itself is stored in text format (for easy editing with any ASCII

editor). Only the file MODEMS.MDM can contain more than one

configuration.  Any other *.MDM file can contain only one configuration,

and does not require a "FILE:" line, as the file name (before the .MDM)

is the name.  Don't worry, that will be explained later. Also, the

MODEMS.MDM file is the only file that can contain auto-selection lines.

Described below are examples of the above options.  Additional

information can be also be found in the WWIVINIT.DOC file.

WWIV defaults to the optimal setup for most non-MNP 300, 1200 and 2400

baud modems.  These are the "H0300", "H1200" and "H2400" configurations,

which are described below.  Some modems are a bit slow to respond after

they send the RING result code, and do not properly answer the phone

when it rings (despite WWIV saying that it has answered the phone).  If

the BBS does not answer the phone when it should, try one of the "S"

configurations - "H1200S" or"H2400S". This should take care of the

problem.  The only difference between the "S" configurations and the

normal ones is that the "S" configurations have a different ANSR:

string, which as a "~" (tilde) at the beginning.  The tilde causes about

an 0.5 second delay, giving the modem some time before the "ATA" is

sent.  If this still doesn't work, try editing the appropriate

configuration file, and adding an additional tilde on the beginning.

If one of the 300, 1200 or 2400 configurations does not work for your

modem, you have two choices: 1) read about the WWIV modem configuration

file, located below, and modify (tweak) the appropriate section to get

it to work, or 2) contact me via one of the methods in the 'READ.ME'

file, and see if I can get it to work for you (or if I already have an

updated configuration file that supports your modem).  If you have a

good understanding of how modems work, I'd suggest trying to figure it

out on your own first.

In any case, if you have to modify a part of the default configuration

file to get your modem to work, please send the updated configuration

section (be sure to update the modem name and description also) to me,

so that I may include that configuration in future releases of WWIV.

This will help other sysops who purchase the same model modem.

Some MNP and 9600+ modems require special configuration.  Fortunately,

most of the high-speed modems on BBS's (that I know of) are USRobotics

modems, and WWIV already knows how to handle the USRobotics high-speed

modems.

If WWIV does not currently support the type of high-speed modem that you

have, you can try building your own configuration section for it.

However, unless you are VERY familiar with how the modem works, I'd

suggest not trying that.  Even if you are able to use your high-speed

modem perfectly with your terminal program, the same configuration will

NOT work for the BBS.  The reason is, in the terminal program, you (the

user) are sitting there, and are able to interpret the result codes and

fix things if problems occur.  There is nobody around to help a BBS if

it gets confused.  Also, the BBS needs very specific information on the

modem (including flow control, buffering, baud rates, result codes,

etc.) in order to work properly for the variety of callers and modems

that connect with it.

If you are unable to get your own configuration to work (or don't want

to try), you'll pretty much have to contact me directly.  Unless you are

trying this during the first few weeks after the release of v4.20, there

is a good chance I will already have the configuration file for your

modem, and can forward it (e-mail) it to you.  When contacting me, BE

SURE to include as much information about your modem as possible.

Telling me only that it's supposed to be a Hayes compatible modem just

won't do it.  I need manufacturer, model number, maximum speed and what

the modem returns when you enter 'AT'<enter> and 'AT0'<enter> when in a

terminal program.

If I find I DON'T already have a configuration file for it, then I will

need a fair amount of information on the modem to get it to work.  In

all likelihood, if you call up technical support for your modem

manufacturer, and ask them to send me (a developer) a copy of the

manual, they probably won't mind.  With a manual, I can probably work it

out from there (at no cost to you).  If you send me YOUR copy of the

manual, don't expect me to mail it back.  I just don't have the time to

be running to the post office every few days.

If you have a modem type that is not currently supported, I REALLY WOULD

like to get a configuration setup for that modem and support it in WWIV.

Please do not think that I am unwilling to support all types of modems.

I do want to support them, and will put in the time necessary to get

your modem working. But remember: I will need quite a bit of info on

your modem type.

/****************************************************************************/

Now, more about the WWIV modem configuration file.

Creating a new section of the modem configuration file does not require

any programming knowledge; it only requires in-depth, intimate knowledge

of how the modem works.  I would not suggest that anyone try to set up

their own modem configuration section from scratch; rather, take one of

the existing configurations, and tweak it as necessary.

Modem configurations are stored in text format (for easy editing with

any ASCII editor), in *.mdm files in the data directory.  The file

'modems.mdm' contains the default configurations (multiple

configurations).  Any other *.mdm file can contain only one

configuration, and does not require a "FILE:" line, as the file name

(before the .mdm) is the name.  Don't worry, that'll be explained later.

Also, the 'modems.mdm' file is the only file that can contain

auto-selection lines.

Let's start out with a sample configuration (for a 2400 baud modem), and

explain the various parts of it (this configuration is from the

MODEMS.MDM file).

##############################################################################

#

# Hayes compatible 2400 baud modems.

#

FILE: "H2400"

NAME: "Hayes compatible 2400"

NOTE: "If the BBS doesnt answer the phone, try the H2400S configuration."

SETU: "AT&C1&D2Q0V1{"

INIT: "ATC1E0F1H0M0Q0V1X4S0=0S2=1{"

ANSR: "ATA{"

PICK: "ATH1{"

HANG: "ATH0{"

DIAL: "ATDT"

SEPR: ""

DEFL: MS=2400 CS=2400 EC=N DC=N AS=N FC=N

RESL: "OK"              "Normal"        NORM

RESL: "RING"            "Ring"          RING

RESL: "NO CARRIER"      "No Carrier"    DIS

RESL: "ERROR"           "Error"         ERR

RESL: "NO DIAL TONE"    "No Dial Tone"  NDT

RESL: "NO DIALTONE"     "No Dial Tone"  NDT

RESL: "BUSY"            "Busy"          DIS

RESL: "NO ANSWER"       "No Answer"     DIS

RESL: "RINGING"         "Ringing"       RINGING

RESL: "VOICE"           "Voice"         DIS

RESL: "CONNECT"         "300"           MS=300   CS=300   CON

RESL: "CONNECT 1200"    "1200"          MS=1200  CS=1200  CON

RESL: "CONNECT 2400"    "2400"          MS=2400  CS=2400  CON

As you can see, the modem configuration is made-up of three groups:

The STRING lines (FILE, NAME, NOTE, SETU, INIT, ANSR, PICK, HANG, DIAL,

SEPR), the DEFAULT SETTINGS (DEFL), and the RESULT CODE lines (RESL).

The "FILE" line gives the "file name" of the configuration.  If you took

the configuration above (minus the FILE: line), copied it from the

modems.mdm file, and put it in a "H2400.MDM" file in the data directory,

everything would work exactly the same.  The "FILE" entry just allows

you to put multiple configurations into the 'modems.mdm' file, and not

clutter up your data directory with lots of *.mdm files that you don't

need.

Each of the strings allows control characters and delays to be

specified.  A "{" is converted to a carriage-return.  A "~" is converted

into an 0.5 second delay.  Any string such as "^J" is converted into the

specified control character (here, control-J, or a linefeed).  Any

string such as "\^" or "\~" quotes the character after it, allowing you

to put a ^ or a ~ into the string.  Also, each string is terminated by

either a single or double quote.  If you need to actually have a ' or "

in the string, you will need to 'quote' the quote, as in \' or \".

"NAME:" gives a textual name/description of the modem, which is

displayed in INIT section 5, along with the file name, which is used to

help select the correct configuration.

"NOTE:" A note that is printed out for the user when the specific modem

configuration is selected, giving any caveats for the configuration.

Multiple NOTE: lines may be specified, however you will need to end each

line with "^M^J" (carriage-return/line-feed), so it looks nice for the

user.

"SETU:" gives the setup string for the modem.  This string is sent out

once each time the BBS is loaded and run.

"INIT:" gives the initialization string for the modem.  This is sent out

to the modem when the BBS is first run (but after the SETUp string), and

after each caller hangs up.

"ANSR:" gives the string sent to the modem to tell it to answer the

phone (such as after the RING result code is received).

"PICK:" gives the string to pick up the modem (take the phone off-hook),

to prevent users from calling and getting a ring (thus thinking the BBS

is down).

"HANG:" gives the string t hang up the phone (after the PICK string is

sent).

I believe every modem in the world will probably use the same strings

for ANSR, PICK, and HANG.  (Except, some modems (as noted earlier) are

slow, and need the answer string to be "~ATA{" (pause 1/2 second before

sending the answer string).

"DIAL:" gives the dial prefix.  If your phone line does not support

tones, then changing it to "ATDP" will use pulse dialing.  Putting

"ATB1DT" will cause a USR Dual Standard to dial out with the HST

protocol (instead of V.32, since "B0" is in the init string).

"SEPR:" gives separation characters for the result codes.  This will be

discussed later, under the high-speed modem section.

"DEFL:" gives the default modem setup.  It gives, essentially, modem

result configuration info.

"RESL:" gives modem result code info.  There are two strings, then the

result configuration info.

The first string is the actual "result code" received from the modem.

Here, verbose result codes are used.  (All result codes are converted to

uppercase).

The second string is the description of the result code, presented to

the user.

The result configuration info gives information describing the modem

result code.  The possible strings in this area are:

NORM          normal state of modem

RING          phone is ringing

RINGING       remote phone is ringing

ERR           error encountered

DIS           disconnected (No connection)

NDT           no dial tone (same as DIS)

CON           connection established

MS=           modem speed

CS=           com port speed

AS=           asymmetrical baud rates (Y/N)

EC=           error correcting (Y/N)

DC=           data compression (Y/N)

FC=           flow control (Y/N)

For 2400 or slower modems, you'll pretty much just have "MS=2400 CS=2400

CON" (or whatever the baud rate is).  The other flags are used for

high-speed modems.

So, how do you modify this information?  You pretty much take it as-is,

and modify it.  Say your modem had a result code indicating connection

at 600 baud.  You'd then just add a line saying:

RESL: "CONNECT 600"     "600"           MS=600   CS=600   CON

And that would be it.  If you needed additional initialization strings

sent to the modem, then you'd just add that into either the setup or

init strings (depending on if you needed it sent only once, or for each

caller).  Beware, though: Most modems only have a 40 character buffer,

so if you make the strings too long, it might not work.

Thus, modifying a 2400 or lower modem configuration should be fairly

simple (if you know what you need it to do).  9600's, however, are much

more complicated.

If/when you need to modify a modem configuration, you should first take

the configuration you are starting with, and copy it out of the

'modems.mdm' file into a separate file.  If, say, you need to tweak the

H2400 configuration for your Prometheus modem, you'd copy the H2400

configuration (minus the FILE: "H2400") into a file, say, "PROMETH.MDM"

in your data directory.  Then you'd modify the NAME: and NOTE: lines for

the new modem type.  Then, when you're done, you can easily distribute

the new configuration (as the file PROMETH.MDM) to other people who

might need it (and also send a copy back to me, so I can include it in

future versions of the modems.mdm file).

/****************************************************************************/

The configuration for the latest USRobotics high-speed modems is:

##############################################################################

#

# Newest version of USR HST/V.32bis/DS modems

#

# This really returns the optimal information.

#

FILE: "USRC31"

NAME: "USR Courier (Newest, With rectangular LEDs/V.32bis)"

CONF: "ATZ{~~~~~~~AT{~~AT&W{"

SETU: "ATC1E0F1H0M0Q0V1X6&A3&B2&C1&D2{~~AT&H1&I0&K2&N0&R2&S0S0=0S2=1{"

INIT: "ATB0H0M0{"

ANSR: "ATA{"

PICK: "ATH1{"

HANG: "ATH0{"

DIAL: "ATB1DT"

SEPR: "/"

DEFL: MS=38400 CS=38400 EC=N DC=N AS=N FC=Y

RESL: "OK"              "Normal"        NORM

RESL: "RING"            "Ring"          RING

RESL: "NO CARRIER"      "No Carrier"    DIS

RESL: "ERROR"           "Error"         ERR

RESL: "NO DIAL TONE"    "No Dial Tone"  NDT

RESL: "BUSY"            "Busy"          DIS

RESL: "NO ANSWER"       "No Answer"     DIS

RESL: "RINGING"         "Ringing"       RINGING

RESL: "VOICE"           "Voice"         DIS

RESL: "CONNECT"         "300"           MS=300   CS=300   CON

RESL: "CONNECT 1200"    "1200"          MS=1200  CS=1200  CON

RESL: "CONNECT 2400"    "2400"          MS=2400  CS=2400  CON

RESL: "CONNECT 4800"    "4800"          MS=4800  CS=4800  CON

RESL: "CONNECT 7200"    "7200"          MS=7200  CS=7200  CON

RESL: "CONNECT 9600"    "9600"          MS=9600  CS=9600  CON

RESL: "CONNECT 12000"   "12000"         MS=12000 CS=12000 CON

RESL: "CONNECT 14400"   "14400"         MS=14400 CS=14400 CON

RESL: "ARQ"                             EC=Y     CS=38400

RESL: "HST"             '/HST'          AS=Y

RESL: "V32"             '/V.32'         AS=N

RESL: "MNP"             '/MNP'          EC=Y

RESL: "LAPM"            '/LAPM'         EC=Y

RESL: "MNP5"            '/MNP5'         DC=Y

RESL: "V42BIS"          '/V.42bis'      DC=Y

RESL: "NONE"                            EC=N

RESL: "SYNC"

As you can see, much of it is the same.

One of the differences is that the setup string is divided into two

portions, to get around the (stupid) 40 character buffer in the modem.

Another major difference is the "CONF:" line.  This configuration line

is sent out from the INIT program, when the specific modem configuration

is selected.  It is sent out at the maximum baud rate the modem runs at.

For the USRobotics here, it sets the baud rate in the NRAM to 38400

baud.  This is important, as this stored baud rate is used by the modem

when an error-correcting connection is made.  Modems such as the

Compucom (which does not have any NRAM) will not need a CONF: line.

A subtle, but important, change is that the SEPR: is now set to "/". The

USRobotics result codes are sent out in strings such as "CONNECT

9600/ARQ/V32/LAPM/V42BIS".  The SEPR: "/" tells the BBS to split up that

big result code into several "partial result codes", at the separator

character.  Thus, the above result code will be split into "CONNECT

9600", "ARQ", "V32", "LAPM, "V42BIS", which is much easier for the BBS

to handle.  Each one of the partial result codes is then processed

independently, in order.  All of the partial result codes are processed

before any connected/disconnected decision is made by the BBS.

Now, the "DEFL:" line here says that, by default, the state of the modem

is (in order) running at 38400 bps, non-error-correcting, no data

compression, not asymmetrical, and flow control IS needed.  When a

result code is received, all the modem settings are assumed to be at the

DEFL state.  The result configuration info for each partial result code

modifies each field independently, starting from the DEFL state.

Each of the "CONNECT" partial result codes simply sets both the modem

speed (MS=) and the com port speed (CS=) to the baud rate specified,

same as for 2400 baud modems.

Suppose we got the result code "CONNECT 14400/ARQ/HST/HST/V42BIS".

We start out at the default:

Partial result code             modem settings after processing

At beginning, DEFL              MS=38400 CS=38400 EC=N DC=N AS=N FC=Y

"CONNECT 14400"                 MS=14400 CS=14400 EC=N DC=N AS=N FC=Y

"ARQ"                           MS=14400 CS=38400 EC=Y DC=N AS=N FC=Y

"HST"                           MS=14400 CS=38400 EC=Y DC=N AS=Y FC=Y

"HST"                           MS=14400 CS=38400 EC=Y DC=N AS=Y FC=Y

"V42BIS"                        MS=14400 CS=38400 EC=Y DC=Y AS=Y FC=Y

Which means:

MS=14400        The user's throughput is 14400 bps

CS=38400        The BBS should talk to the modem at 38400 bps.

(Due to the &B2 setting)

EC=Y            Error correction is active (LAPM/MNP2-4)

DC=Y            Data compression is active (V.42bis/MNP5)

AS=Y            Asymmetrical baud rates.  This is because, with the HST

connection, it is only 14400 bps in one direction.  The

other direction is only 450 bps.

FC=Y            Flow control is still active.

The "CON" at the end of each of the "CONNECT" lines means that the user

is connected.  Even though the "CON" is processed in the first partial

result code, the remaining partial result codes are still processed

Note that, for the HST, V32, etc, partial result codes, the description

of the partial result code is in single-quotes (vs double-quotes for the

CONNECT descriptions).  If the description is in double-quotes, then any

previous descriptions is OVERWRITTEN.  If the description is in

single-quotes, then the description is APPENDED to any previous partial

description.  For the result code above, the description would end up

being "14400/HST/HST/V.42bis"

For the ARQ partial result code, there is no description, and so the

description (so far) is unmodified by the "ARQ".

How do you make your own modem configuration file?  A lot of it will

probably be the same as other configurations (ANSR, PICK, HANG, DIAL,

the first few RESL), however you will certainly have to completely

re-write the SETU and INIT strings.  You will also almost certainly have

to modify the DEFL line, and the RESL lines for the connecting result

codes.

There are also length limits on the strings:

file: 8 chars

name: 80 chars

note: 4096 chars TOTAL (for however many NOTE: lines you have)

conf: 160 chars

setu: 160 chars

init: 160 chars

ansr: 80 chars

pick: 80 chars

hang: 80 chars

dial: 80 chars

sepr: 9 chars

result code from modem: 40 chars

result code description: 30 chars

Also: the sepr: string may contain multiple chars.  It is not really a

"string" as a "list of chars".  If you know C, this string is passed to

strtok().

/****************************************************************************/

Auto-selection: WWIV maintains a database for automatic modem type

detection in the MODEMS.MDM file.  The data comprise a sort of "state

machine".  When trying to detect a certain type of high-speed modem, it

starts out in state 1.  If/when it gets to state 0, then it has detected

the modem type.  If it passes through the entire list without passing

any of the rules, then it determines that it cannot determine the modem

type.

The first column is the "start" state.  The second is the "end" state.

If the current state is equal to the "start" state, then the string in

the fourth column is sent to the modem.  If the response is equal to the

string in the fifth column, then the state transition is made, and the

current state is set to the "end" state, and the currently detected

modem type is set to the string in the third column.

Note that the auto-selection data is used ONLY if the modem responds at

9600 baud or higher.

Also note that it is not important for you to put in auto-selection data

to detect your modem type.  The auto-selection stuff is only a nicety,

and you can always go into init section 5 and pick whichever modem data

you want.