#[33mImage Processing Operations#[31m

The AVT system offers a number of specialized image processing operations

the are optimized for use with slow scan images; These are generally

available in the RoboSynth window, and when you press the "Lcor" button

that is actually IN the RoboSynth window - that opens yet another

process window with some very, VERY nice goodies in it.

Let's stick to the RoboSynth window for the moment, however.

In here you will find Four transmit convert buttons (discussed in the

file headed #[33mRobotColor.doc#[31m, along with the post-receive tint

control). You will also find a button headed "Luma". This button takes the

contents of the Red, Green, and Blue buffers and creates a Black and white

image according to the forumla used for NTSC composite images: 59% from the

green buffer, 30% from the red buffer, and 11% from the blue buffer. This

has two effects; One obvious (it makes it black and white) and the other

not so obvious: it makes a smoothed image from a color one that may have

been quite rough.

You will also notice two sets of arrow buttons. One pair has small "b"'s in

them, and the other has small "c"'s. In the center of these four arrows

is a small square buttton labled "H".

The Arrows are for stepping the brightness and contrast up and down,

and the "H" button generates a histogram.

Like most functions in the AVT system, if you are in B&W, then the contrast

and brightness functions, and the histogram, only affect that particular

buffer - red, green OR blue. If you are in color, all three of the red,

green and blue are affected, and the color image is redrawn for you.

If you press the "Lcor" button, a new window opens and you have a number

of Image processing tools available to you that have to do with single

line error correction (hence the name - Lcor = Line CORection).

The first thing to notice is the small "keypad" of arrow keys, with

the square button in the center. The up and down arrows (and the up and

down cursor keys on the kbd, if you prefer) move a small cursor up and

down along side the image you are working on. If you press the square

button in the center of the keypad, then this action is multiplied - the

small white cursor moves in ten line steps instead of 1 line steps.

The right and left arrows (and the right and left on the keyboard) actually

shift the line you are on to the right or left - you can correct many of the

positioning errors that are inherent in the old, error-prone protocols

like Robot composite color, and 8 second B&W.

In addition, the action of the center shift button is duplicated by the

ENTER key on the numeric keypad - note that this is NOT the "RETURN" key

on the main keyboard.

That does something else, of course, as you will see in a moment.

Look at the right edge of the window. You will see three small buttons.

Two have small 'c's adjacent to arrows in them, and one has a line with

two arrows pointing at it.

The 'c' buttons are line-copy buttons. They cause the line below or above

the line that the cursor is on to be copied into the current line.

The line with the two arrows pointing at the line in the center is a

line synthsize initiator. This causes the line that the cursor is

positioned upon to be completely replaced with a line that was

created from the average of the line above it and the line below it.

Try each of these three options on each line you want to be working on,

see which works best for you.

You'll also see a button labled "Shft". This is for use with the

Line-sequential modes such as Volker-Wrasse or WA7WOD's line sequential

modes (25.5, 50,102). If you receive an image that seems to go to some

wild (and WRONG) color combination in the middle of the image, bring up

the window, move the cursor to the scan line where the problem occurs,

and press the Shft button once or twice. That should clear it up.

You may find that you might have to do that multiple times in order to

completely clean up the image, but it's quite easy to do.

The RETURN key on the keyboard does this too, when this window is open.

In addition, the two copy and the synthesize options are available from

the three keys at the top right of the numeric keypad. The 9 is copy

down, the 6 is copy up, and the 3 is synthsize.

Remember that you can use the Amiga programs "Butcher" and PixMate" to

do REALLY heavy-duty image processiing work for you; But these operations

are specific to SSTV, and are unlikely to be included in a standard

image processing suite.