#[33mReceiving SSTV:#[31m

The most inportant thing you need to know in order to receive SSTV is

what mode is the other party sending?

There are many, many formats of SSTV supported by the AVT system, and

unfortunately it is difficult to help you on this one from the manual.

If you are a HAM, you can always simply ASK the other party. If you are

an SWL, then you must listen very carefully and try to figure it out

from the conversation of the parties involved in the active transmission

and reception fo these modes.

A small bit of help is this; For many of the Robot Color modes, they often

simply say, "this will be 36" or "here comes a 12". Most modes are referred

to by their speed, and very often no mention is made of whether it will

be a color or black and white shot. Again, you kind of need to listen

carefully to see what's going on.

To actually initiate receive is very easy. You press the appropriate

button on the AVT's on-screen control panel for the speed, and then the

mode, and then press the recieve button.

Any receive mode, including FAX, can be aborted by pressing the left mouse

button during receive. However, in FAX, you must hold the button down for

a period of time greater than 6 lines of FAX video at the scane rate

you are using. Simply put, hold the button for at LEAST 6 seconds and the

program will quit receiving.

Some receive modes wait until they "hear" an incoming signal before

actually DOING anything. The AVT modes are an example of this. Until an

Initiate sequence is receieved from the station that is the source of the

image, nothing will happen.

When receiving FAX, you can "phase" the image by pressing the left mouse

button for a SHORT period of time during reception of a signal. Phasing the

FAX image essentially means you are aligning it to match the left edge of

your screen.

This version of the program does not autostart when it hears a FAX signal;

instead, it begins receive immediately when you press the receive button.

It is a virtual certainty that you will begin reception with an image that

is NOT properly horizontally aligned; If this is so, tap the left mouse

button until the image is positioned correctly.

It may take several tries to learn how to do this effectively, but it is

time well spent.

Each receive mode has a set of characteristics unique to itself; It will

take you some time to become proficient at recognizing the various types

of signals from the way that they sound, but that's part of the fun.

With any other system besides the AVT system, you wouldn't have the option;

instead, many times SSTV images would fly by and there would be no WAY

for you to receive them. You made the right choice!