#[33mUsing the 'Scope Simulation#[31m

The AVT system has a very useful Scope simulation available in the main

control panel. There are two controls that are directly associated with

the scope, both also in the control panel window, right next to it.

The first is the "tune" button. This activates the scope and locks the

mouse pointer. When you are done using the scope, simply press the left

mouse button and it will turn off.

The second is the "Trig" button. This causes the scope to re-trigger

when a negative edge, transiting the frequency region from black to

sync is detected. The net effect of this is that incoming SSTV of the

older format lines up left-to-right on the scope display, starting with

the sync pulse and continuing accross the display with the video

information for each scan line.

Generally, you use the scope to tune in other SSTV stations, but you

can also use it to detect the shift of RTTY signals, tune FAX, and other

interesting signals.

The scope is a two dimensional display, with time going positive from

left to right, and frequency going higher from bottom to top. There are

three red marks at the left edge of the dispaly area that indicate the

exact position where the standard black, white, and sync frequencies

are located. For your information, black is 1500 Hz, white is 2300 Hz,

and sync is 1200 hz. Additionally, there are marks on either side

of the sync mark, that show a '1' and a '0'. These are at 1100 and

1300 hz, which are the two frequencies that the Robot scan converters

use to send information as to what mode is coming. THis is done in the

main vertical sync pulse.

#[33mTuning with another AVT station#[31m

When you are working with another station that has the AVT system,

you can ask them to open their SET window and press the BLK button.

This causes them to transmit a pure black tone - while you are

receiving this signal, ensure that the TRIG button is OFF (no light

showing) and press the tune button; tune your SSB receiver for a line

exactly on the graticule that is marked "black". once you have done

this, you are tuned exactly correctly - no guesswork.

#[33mTuning with a station that is running a new Robot unit#[31m

Here, ask the other station to send you a grey scale, or failing that,

any other video. While receiveing this; make sure the TRIG button is

ON (light showing) and tune your SSB receiver so that the tips of the

sync pulses are exactly positioned next to the red arrow marked SYNC.

#[33mTuning a station running other types of units#[31m

When tuning stations with older hardware, it is best to tune the

radio so that the black and white levels are correctly aligned -

not the sync. Some of the older Robot 400s, for instance, have

separate adjustments for sync, black, and white frequecies. We have

seen stations transmitting video where the black and white frequencies

were good, but the sync was too high or too low. In this situation, it

is best to tune for the video levels, because the object, after all,

is to receive an image. The newer Robot units like the 400c, 450c, and

1200c, do not have adjustments that can go out of (or be SET out of)

whack; instead, they use primitive digital synthesis techniques which

are quite reliable - the sync, black, and white frequencies are

always correct.

#[33mSweep Rates#[31m

The Scopes sweep rate is set automatically when you press a recieve

mode. For instance, if you press the top left button, the 8 second

speed, and then the green memory, the AVT is set up to receive 8

second Robot-format black and white images. These inages have

scan line times of about 60 ms, and the scope is set for a sweep

rate of about 59 ms. Another example is pressing the CMP (composite)

button and the 72 second speed button. This sets the scope for

approximately 305ms per scan. For use with unusual signals (found while

SWL'ing, for instance) press one of these speed buttons to control the

sweep rate of the scope. 8 second, black and white is the

fastest sweep rate, and 72 second, color composite is the slowest

sweep rate.

#[33mThe Filter Button#[31m

There is another button which affects the scope; This is the FILTER

button on the main control panel. This button runs a 2-pole

software low pass filter simulation in such a way as to have the

same effect one would have if you filtered the incoming video

with an op-amp circuit. You will notice that the incoming waveform is

smoothed considerably when the filter is on. Since the scope draws it's

waveforms by placing one, and only one, dot along the horizontal axis for

each measurement it takes, you may find that wildly varying frequencies

(like white noise) produce a display that makes little sense. If you

turn on the filter, you will see the low-frequency components that make

up the white noise. Experiment with this. Remember, though, that the

filter button affects not only the scope, but the receieved video as well;

If you notice that all your incoming video looks very smooooth, check the

filter button.