#[33mTransmitting:#[31m

Tranmission is similar to reception; If you have not yet read the section

on receiving SSTV, please do so now.

In general, to transmit, you need to set the AVT to the correct

transmission mode, and press the TX button. The only exception to

this are the Robot composite color modes; if you wish to transmit in one

of these modes, you must first open the RoboSynth window and select

the correct speed - the system will create a transmit record

(we refer to this process as 'compiling', here at Black Belt Systems)

and then return you to the main control panel.

The AVT uses the Amiga's sound hardware to transmit SSTV and FAX images.

The Amiga has a four channel, highly sophisticated digital sound

synthesis system that runs using DMA and Interupt control within the

computer, and requiring no external hardware at all.

In the SET window, you will find five (or six, if you own and a500

or a2000) controls that affect the transmitted signal.

#[33mChannel selection#[31m

There are four channel buttons; these choose which of the four Amiga

sound channels you wish to utilize in the production of SSTV and FAX

AFSK signals. The default is Channel 1.

#[33mLow Pass Filter#[31m

The low pass filter button controls, in a500's and a2000's, whether

the AMiga's internal filter is on or off. it also controls the Amiga's

power indicator - so if you see that the Amiga's power indicator is off

or dim, check this button before panicking. On an unmodified a1000, this

button ONLY controls the power indicaator - sorry about that. An issue

of Amazing Computing detailed how to modify the a1000 so that the low

pass filter could be turned on and of just like the a500 and the a1000.

#[33mMultiple Frame Tranmission#[31m

If you press the MEM button, the memory control window will open, and

you can select some additional transmit operations. First, you can

press the arrow buttons to change the transmit repeat count. For instance,

you might want to set it to four. Once that is done, you can press the

TxRp button, and the AVT will send, in the currently selected mode,

four frames of the image in the current memory. If you press the Sq

button, another window will open. here, you can set up a sequence of

frames to be transmitted that are in different memories. For instance,

you could put a picture of your dog in memory 1, your cat in memory 2,

and yourself in memory 3. Set the Begin value to 1, anf the end value

to 3. now close the wondow, and press the TxSeq button. Out they go,

one after another. You can also receive sequential images the same

way - just set up the begin and end values, and press RxSeq.

#[33mTransmission of individual modes#[31m

#[33mRGB frame sequential transmission#[31m

This is very simple. Press the speed you wish to use (8,17a,17b,34),

then poress the 111 buitton, and the TX button.

#[33mFAX Transmission#[31m

Just press TX. Note that if you are running with a 1 megabyte machine,

you will only be able to transmit data from the screen itself, not

the FAX memory (since you won't be able to HAVE a fax memeory). This

measn that the effective resolution of your FAX images is 640x400.

If you do have extended memory, and you did not get a requester

indicating that you were unable to allocate the memory, then the

FAX transmission is senbt in 1024x1200, in 16 grey levels. You need to

ensure that you have selected the correct fax mode as well from the

two available - Weather fax (120 lpm) and news fax (60 lpm).

#[33mBlack and white transmission#[31m

Press the R, G, or B buttons, select the speed from either the Robot

mode group (8,12,24, and 36) or the standard/AVT mode group

(8,17a,17b, or 34), and press the TX button.

#[33mAVT transmission#[31m

Select the speed from the std/AVT mode group (24, 90, or 94), press

the AVT button, then press TX

#[33mRobot Color transmission#[31m

Press the RoboSynth button, select the appropriate speed from the four

in the Robosyth window (NOT the std control panel) and wait for the

conversion to complete. Once complete, the RoboSynth window will close,

then you press teh TX button.

#[33mStandard line sequential color transmission#[31m

Press the 25.5, 51, or 102 buttons, then press TX.

#[33mVolker-Wrasse line sequential color transmission#[31m

Press the 25.5, 51, or 102 buttons. Then press Tx. You must ensure that

the V-W button in the SET window is selected (red light showing) or these

buttons select only standard line sequential modes.