A KA9Q NOS device driver for OH1MQK's "Dual-DDLC" card

The Dual-DDLC-card ("the card") has 2 or 4 synchronous

serial channels capable of operating up to (about) 4 Mbps each.

Dual-port memory access constraints limit the speed of all

channels together to about the said 4 Mbps.

However, that should be enough for most uses :-)

The card has synchronous interfaces (see cards technical

documentation) which contains INPUTS of Rx and Tx CLOCKS, Rx and

Tx data, RTS, CTS, and a couple of general purpose signals (1 input,

1 output). (Alike any decent synchronous modem interface.)

The card is completely MEMORY MAPPED device, and does not

have any IO-space locations.  It can be configured to be in any

address within the low 1MB of PC/XT/AT/AT386 memory space at incre-

ments of 16kB.  The card can be configured to use 16kB or 32kB of

memory space.

The card uses an on-board SRAM as a buffer for incoming, and

outgoing packets - alike most Ethernet boards.

The card has no access-speed limitations (unlike 8530 et.al.),

and thus has no need to do fancy delays while accessing it.

Due to PC's INT-high-active nature, the card can't share

INT with another card via PC-Bus INT signal.

Usage of the  ATTACH  command:

attach ddlc INIT IO-BASE INT-NRO [BOARD-SIZE]

attach ddlc CHANNEL MODE LABEL MTU SPEED DUPLEX [IP-ADDR]

Where:

INIT		"init" - lowercase special label.

Indicates board definition statement. Must be

done before the actual channel attach is done.

IO-BASE		Base address of the D-DDLC-board.  20-bit

hex address in the form of  HHHHH.

For example:  A0000

INT-NRO		INT-vector number of the board.

*	NR-OF-CHIPS	1 or 2

Number of equiped chips on this board.

If 1, then it must be for channels 0 and 1.

*	BOARD-SIZE	"16"/"32"/"16kB"/"32kB"

Tells the size of the card (of its on-board

memory size)  -- comes equiped always with

32kB, but upper half can be discarded, and

address granularity stepped down to 16kB.

( Params marked with `*' are automatically figured out from the board )

The  "attach init" -command must be used before the card

can get attachements of actual channels.

In the source files (ddlc.h) is definition of maximum of two

DDLC cards (DDLCMAX), but as that is up to 8 interfaces,

it should be enough...

CHANNEL		0..7

Number of channel to be attached.

0..3 on the first card, 4..7 on the second.

(And so on, if DDLCMAX is increased.)

If the card has only 1 chip, only the first

two channels of that card are usable.

MODE		"ax25" or "raw" (alias "hdlc")

Lower-case special labels.

AX.25 mode and bare HDLC-encapsulation (wire

line) supported.

LABEL		"dd0"

As usual, label for the channel.

MTU		1500 (or whatever)

As usual, MTU size.

SPEED		56000/9600/1200/250000/2000000/...

Informative text for status display.

interface speed IS NOT configurable

by the software.  Both clocks come

from the modem.

DUPLEX		"full" / "half" / "fullduplex" / "halfduplex" /

(Recognize by the first character; f or h)

Tells wether or not the channel has a full

or a half-duplex radio-/wire-modem.

IP-ADDR		44.109.04.20

Optionally define interface IP number(s).

Defaults to global  Ip_addr.

Quite normal form of attach-command.