R:910902/1018 @:WA8BXN.OH.USA.NA Kirtland, OH #:6091 Z:44026

R:910902/0535z @:NT8V.MI.USA.NA PORT HURON, MI. #:684 Z:48060

SYSOP v2.0 - by Thomas Dwyer III (N8MHF)	July 24, 1991

What it is:

SYSOP is a program which simplifies the authentication process

when becoming the remote sysop of an MSYS BBS.  When the SysRq

key on the Enhanced keyboard is pressed, the \Sysop command is

sent to MSYS.  This program then waits for MSYS to supply:

Usage stats: Users=27 Msgs rcvd=25 Msgs sent=1 Max=27

Note that the numbers in the above line will be different with

each access attempt.  This program parses the line and builds

the correct password, which is then sent to MSYS.

How to use it:

SYSOP takes a string as an argument.  The first character of

the string must be a digit [1-8] of the com port that SYSOP

should use for communication with MSYS.  The rest of the string

is the password specified in the MSYS.RMT file on the remote

system.  If SYSOP is run with no arguments, or if the first

character of the argument is not a digit in the range [1-8],

a syntax message is printed and the resident code is not

installed.

Once installed, SYSOP can be removed from memory by running it

and specifying "0" as the port number.

It is possible to change the COM port that SYSOP uses to talk

to MSYS after it has been installed by simply running SYSOP

and specifying the new port number.  It is not necessary to

supply the password on the command line in this case.

Notes:

SYSOP depends on interrupt 0x15 being called via the keyboard

interrupt.  If this does not happen in your system, SYSOP will

not work.  If you have F11 and F12 keys on your keyboard, SYSOP

will probably work with your machine.

The addresses for the com ports COM1 through COM8 are hard

coded in the program.  Here is the part of the listing file

that has these addresses.  Those of you (if any) who have

problems getting ports 3-8 working on your machines might

have to use DEBUG to alter this address table.

0438

0438		p_list	label	word	; here are the addresses

0438  03F8		dw	3f8h	; for COM1 through COM8

043A  02F8		dw	2f8h

043C  03E8		dw	3e8h

043E  02E8		dw	2e8h

0440  02F0		dw	2f0h

0442  03E0		dw	3e0h

0444  02E0		dw	2e0h

0446  0260		dw	260h

0446

During the time SYSOP is active (ie, you pressed SysRq), the

scroll-lock light will be lit.  When SYSOP has finished with

the password transmission, the scroll-lock light will turn

off.  SYSOP will wait until MSYS responds with the line SYSOP

is looking for, or until the ESC key is pressed.

Compatibility:

SYSOP was written for and tested with MSYS version 1.11

Remote machine was running Kermit, Yapp, Telix, and others.

Fixed Bugs:

When pressing SysRq a second time while the first invocation

is still active, the PC would hang.  This is fixed.

There was no provision for SYSOP to check whether it was

previously installed, and running it several times would cause

a new copy to become resident.  This is now fixed.

There was no provision to remove SYSOP from memory once installed.

This is now fixed.

Known Bugs:

SYSOP requires one and only one space between the command and the

argument.  This may or may not be fixed in the next version.

There is no provision to change the password string that SYSOP

uses once it has been installed.  This will be fixed in the next

version.  The current workaround is to uninstall SYSOP and then

install it again with the new password.

Unknown Bugs:

You tell me!  Send mail to:

Internet:	tomiii@mtu.edu

Bitnet:		tomiii@mtus5.bitnet

Packet Radio:	N8MHF@W8YY.#UPMI.MI.USA.NA

Snail Mail:	Thomas Dwyer III

311 Scallon Ave.

Hancock, MI  49930

USA