IdentCDR - Documentation

1. Purpose

This small tool gives you some information about CD-R media; the main feature is

the brand recognition, done through ATIP (Absolute Time In Pre-groove).

It also gets some additional data, like maximum capacity of the CD-R and maximum

allowed recording capacity (which is always a bit less than the first one).

2. Requirements

First of all, a SCSI CD-R burner. IdentCDR can work with CD-ROMs, but will give

you very little information compared to a full blown CD-R burner. CD-R burners

know much more about a CD-R media than a reader.

Reqtools.library and a 68020 or better are mandatory since version 1.2 .

Last but least, a SCSI host adapter which can correctly process SCSI-Direct

commands (you should already have it, because otherwise you wouldn't be able to

use your CD-R for burning).

2. Installation

Smash the executable and the its icon anyware you want.

3. Configuration - using it.

- From Workbench:

You can specify your CD-R using tooltypes: either

DOSVOLUME=xxx:

like

DOSVOLUME=CDR:

in case you mounted your CD-R with a filesystem. Otherwise you must specify

both the exec device name and unit number.

DEVICE=xxxx.device

UNIT=nnn

like

DEVICE=gvpscsi.device

UNIT=2

IdentCDR also supports some switches: DEBUG, DUMP and INFOS.

DEBUG:

prints some more information about the values read from the CD-R. Only

for experts.

DUMP:

will make IdentCDR also print all its output to a text window.

INFOS (new in 1.4!):

prints all burning information about a data-CD, like volume name,

creator, program used for burning, etc... (hi Andrzej !). This way

you'll see that MakeCD places your registration number (not the key!)

in every CD created with it. Keep in mind that all this informations

belong to the *first session* only !!!

All these options will be used for the first identification run each time

IdentCDR is launched. After this first one, you can change the device name

through a reqtools requester (the other options will stay like first specified).

- From Shell: pretty much the same:

IdentCDR CD1:

-or-

IdentCDR gvpscsi.device 4

The actual template is: NAME/A,U=UNIT/N,DEBUG/S,DUMP/S,INFOS/S .

IdentCDR requires a minimum stack of 16384 bytes (16K) !

See the WB section for explanation of each switch.

4. Some tech tips.

One of the information displayed by IdentCDR is the 'Disk capacity

(overburn ?)'. I don't really know if this value is to be considered as the

maximum capacity; the MMC standard says it's the "last possible start time of

lead-out", whatever this means. Opposite to this, there's the 'true recordable

free space', that is the common value read by a CD burning software (like

MakeCD).

5. Acks.

Thanks to all those who contacted me asking for improvements. You know who you

are!

The NewIcon is taken from an unknown author, it was originally for OptyCDPlayer.

6.

Version history:

1.4	Now prints some information about the volume name, creator and such.

Thanks to Andrzej Boreczko for some help and encouraging me.

Can (and will) correctly disinguish between CD-R and CD-RW; thanks to

Stephen White for hinting this. This feature will work on CD burners

only, no CD-ROM.

Added display on the titlewindow of how many slots are available.

Added new entries: 60 slots, about 56-58 entries (= different brands),

Fixed a typo in the entries database.

1.3	Some more entries: 54 slots, about 50 entries (TDK and Mitsubishi have

two slots) .

Reworked command-line interface.

New commands (both from CLI and Workbench): DUMP and DEBUG.

(actually, DEBUG was already there in 1.2, but it was hidden).

1.2	Added a small reqtools GUI.

Added reading tooltypes from icon.

Added recording length discovery/display.

Now checks for 'prohibited' values. See the URL found in the .readme

for explanations.

Finally found out how to get exec device informations from a volume

specification (like CD1:). BPCL sucks!!!

No longer displays obtained values on the Shell. Is this a problem for

anyone ?

Executable is indeed shorter than 1.1, because in 1.1 I linked some

useless math libraries; IdentCDR is long about a tenth of the Windoze

counterpart. But if anybody wants it, I can supply with a 300K version

(18K of code and 282K of useless junk, just to make you happy).

Really knows 41 brands, because FUJI uses two slots.

1.1	Fixed memory leak (46 bytes not freed).

New method for those drives who can't use READ_TOC_PMA_ATIP.

Some more sanity checks to ensure returned data is valid.

Currently knows 42 brands (are these all ?).

1.0 	Initial release