Hi There!!

This disc contains two sample source files (and thier

execuatables) to help you include Trojan Phazer compatiblity in your

programs.

The files on this disc are:-

GunDemo1.s - Source for a simpler gun driver that will 'steal' a

frame from the game/program that is running.

LoseFrame  - The object file produced by the above source.

GunDemo2.s - Source for a driver based in the VBL interrupt that

will not steal a frame from your program.

FrameGun   - The object produced by the above source.

Both of these programs create a black screen with a small

white cross in the middle. This cross is the calibration point for the gun.

Fire carefully at the cross to calibrate the gun, then firing

will just plot a dot on the screen where the gun was aimed.

Method of Operation

-------------------

Both of the programs first set up the machine and a screen with

the LPEN bit (bit 3) set in the BPLCON0 register.

GunDemo1

--------

This sits in a main loop and tests the trigger bit,

if pressed it white outs the screen and waits for the frame end.

Once the frame has ended it looks at the lightpen latch register

(VPOSR) and calculates the screen X and Y.

GunDemo2

--------

This sits in an empty main loop and tests the trigger

in the VBL interrupt routine. If trigger pressed then sets a flag

(Scan_frame) and whites out to show current frame is a gun read frame.

At the next VBL the routine resets the palette and reads the

gun position from the appropriate register (and corrects it).

Calibration

-----------

Calibrating the gun is very important for two main

reasons :-

1 .... Each gun/tv/computer combination has slightly different

timings due to component tolerances etc. This means that

a gun has to be set for the individual system.

2 .... Every individual holds the gun at different angles etc.

Correct calibration makes the gun 'accurate' for a person.

Calibration involves firing at a predefined point (usually in

the centre of the screen) and then storing the actaul distance from this

point that the gun registerd. The X and Y difference are then used to

alter the gun XY position every time it is fired from then on.

On the Atari ST the calibrate routine actually modifies an

interrupt routine entry point as well as storing XY offsets. It

does this because the gun would not work correctly at the screen edges

otherwise.

If you have any difficulties in getting the guns to work properly

ring me (David Phillips) on 0554 777993.

Good Luck!!!

D.K.Phillips (Software Manager)

Trojan Software

Unit 7

Dafen Park

Dafen

Llanelli

SA14 8LX