Edit 2.00 - Template Documentation for MT32*.TEM                      01/04/91

The files MT32*.TEM are the Edit 2.00 templates for the Roland MT-32.  They

include the following:

MT32PAT.TEM     MT-32 patch editing template

MT32TIM.TEM     MT-32 timbre editing template

MT32SYS.TEM     MT-32 system area editing template (not yet available)

The MT-32 has lots of memory areas; some are PROM, some are RAM.  The PROM

areas cannot be read or written, but the RAM areas can.  RAM areas include

patches, timbres and the system area.  A timbre defines a basic sound, while

a patch points to a timbre.  When you select a preset number, you are really

selecting a patch; that patch in turn points to a timbre.  There are two very

distinct areas of RAM memory used for patches and timbres.  One area defines

all 128 patches and all 64 user-modifiable timbres.  These areas are known as

"patch memory" and "timbre memory", respectively.  Additionally, each channel

has its own memory area for a single patch and timbre; these are known as

"temporary" areas.  When a patch is selected (either by the front panel or by

a MIDI program change message), the specified patch and its associated timbre

are copied into the temporary area for that channel.

This program will only write data to the "patch memory"and "timbre memory"

areas.  This means that, after writing a timbre change or patch change, you

will need to send a program change message to "activate" your modifications.

Note that the front panel will only allow you to select the PROM timbres,

not the user-modifiable RAM timbres.  Therefore, you will need to send a

MIDI program change command with your MIDI controller.  Eventually, Edit

will have the capability to send most simple MIDI commands, but not in this

release.

This may all sound VERY confusing... I know it was to me, and I wrote the

darned thing <grin>.  So, in order to try to make it easier for you to use

Edit with the MT-32, the following is a step by step procedure to show you

how to use RAM-based timbres.

1. Power up your MT-32... this will reset all RAM and set the device ID to 17

2. Double check the device ID by pressing the Master Volume button and holding

it while pressing the Sound button.  You should see:

** Unit number :  17

If not, use the Select/Volume knob to set the unit number to 17.

3. The first thing we want to do is update one of the RAM timbres.  Load Edit

(type EDIT20 and return).

4. When the first screen comes up, press F7 to load a template, and key in

MT32TIM into the template name prompt.

5. After the template is loaded, press F9 to load a patch, and key MT32WHO

into the patch name prompt.

6. You should now see the timbre named "Doctor Who".  This is a little timbre

I whipped up to emulate the initial sound of the Doctor Who theme song

(the funky electronic buzz with the neat filtering).  Note that the timbre

number is 0; this number will be important later on.  We can update any

of the 64 memory timbres by entering a number from 0 to 63 here, but for

now leave it at 0.

7. Now press Alt-F10 to send the patch.  At this point, all we've done is to

load a memory timbre.  No patch points to it yet, so it doesn't really

affect anything.

8. Now we want to make channel 1 point to the newly loaded timbre.  Press the

Part 1 button.  The display should read as follows:

1|Slap Bass1|vol> 80

9. This would be sufficient for our purposes, but let's do a little extra

something... press the Sound Group button and the display changes:

1>Bass   |Slap Bass1

This display shows the timbre group as well as the specific timbre now.

"Bass" indicates the Bass group of the PROM timbres, while "Slap Bass1"

is the specific timbre.

10. Now, load the MT-32 patch template.  Press F7 and type MT32PAT into the

template name prompt.

11. After the template is loaded, press F9 to load a patch, and key MT32MEM

into the patch name prompt.

12. You will now see a patch loaded.  Especially note the following:

Patch Number....: [  0]

Timbre Group....:  2

Timbre Number...:   0

The patch number corresponds to the MIDI program number.  We are going to

change program number 0.  The timbre group is the key to the whole thing;

timbre groups 0 and 1 point to PROM group A and PROM group B, respectively,

but timbre group 2 indicates the memory timbres.  By specifying group 2,

we can now access any timbres we have uploaded.  The timbre number matches

the timbre number from the MT32TIM template which we discussed in step 6.

13. Press Alt-F10 again to send the patch template.  At this point, we have

pointed patch 0 to the Doctor Who timbre we uploaded to memory timbre 0.

However, as I mentioned earlier, these changes don't take effect until we

issue a MIDI program change command.  Use your MIDI controller to send a

program change command on MIDI channel 2 (remember that the MT-32 channels

1-8 correspond to MIDI channels 2-9).  Change to program 0.  If you want,

you can use Cakewalk to do this... exit Edit (press F3, then press Enter

on the exit prompt) - we're done with it for now - and load Cakewalk.  On

the track screen, simply cursor over to the channel column (Ch) and set it

to 2 (press the + key twice), cursor one more over to the patch field (Pat)

and set it to 0 (press the + key once).  Or use your MIDI controller to

send program change 0 for channel 2 (0xC1 0x00).  The following display

should appear on the MT-32:

1>Memory |Doctor Who

"Memory" indicates that we have successfully pointed the patch to a memory

timbre, and "Doctor Who" is the timbre.  Now, hammer on a nice loud C3 (or

C5 in Cakewalk-ese) on channel 2 and listen to the sound.

Congratulations!  You have just created a new sound for the MT-32!  Now you can

experiment with various parameters and just go wild.  One note of caution: many

(and I do mean MANY) of the timbres of the MT-32, especially those from third

party sources, have parameters that fall outside of the ranges specified in the

MT-32 manual.  These work, but cause all kinds of unpredicted results; they

also make Edit act a little weird.  Edit will still send these out-of-range

parameters, but will not allow you to enter them that way.  I'm trying to come

up with a generic way of turning off range-checking, and will stick it in as

soon as I do.

Also included in this ZIP file should be SQ3-1.SYX through SQ3-9.SYX; these are

some timbres as loaded by the Space Quest III game.  Use them for fun and as

examples of how to mess the MT's mind.  Have at it!