MIDI

Except for  the Real Time  commands page  in  Send Mode, MIDI  uses the hex

numbering system. All midi data displayed on  the screen is in hex, and all

midi data entered at  the keyboard must be done in hex.  The program is not

case sensitive. Most information in  synth manuals about sys-ex messages is

given in hex  but be careful, SOME of  it is not. There are  several memory

resident  programs on  the market  that convert  between hex,  decimal, and

binary, which can be useful while  working with MIDI. Below is a conversion

chart that can be used also.

Note:  If a  resident program  is  called  up while  in Monitor  Mode, upon

returning to  MIDI you may  have to  hit  "Esc", which will  reset the midi

interface.

MIDI checks  to see if  it has  an  MPU-401 or equivalant  interface. If it

doesn't find one, it will inform you and give you the option of running the

program anyway. If  you go ahead and run the  program without an interface,

the  only  thing  that  doesn't  work  is  receiving  and transmitting data

(although it will appear to transmit).

Note: MIDI checks files before they are  transmitted. If the file is not in

standard "system-exclusive" format, an error message will be returned.

---------------------------------------------------------------------------

MONITOR MODE

Monitor Mode  is used to look  at the midi data  coming from an instrument.

The  Mask Bytes  option enables  you  to  have the  program ignore  and not

display data you  don't want to see. To  remove or enter a mask,  press the

"/" key. The arrow  keys can be used to move the cursor  to an open mask or

to a specific mask  to delete. Delete by pressing "X" or  Esc. You can mask

up to five different  bytes. If you mask a data byte,  that byte will never

be displayed. If you mask a status byte (which is what this is really for),

when  that byte  is received,  it and  it's associated  data bytes  are not

displayed.  This  is  very  useful  for  "uncluttering"  the  display  with

information you don't  want to see. Pressing "E" or  "8" is a shorthand way

of entering "FE" or "F8". If you make a mistake in entering the first digit

of the byte, the  Backspace key can be used to start  again. If the mistake

is made on the second digit, you will have to press the "/" key, arrow over

to the byte, and then reenter it. To clear all masks, press "C".

Note: The "/" key is called "?" on  the program screen - you do not have to

press Shift when hitting this key.

The "Esc" key resets the midi interface.

To  make the  display of  data more  readable, any  byte higher  than 7F is

displayed in reverse video or a different color (to identify it as a status

byte), and  2 spaces are  inserted before it.  Pressing "L" will  display a

list of MIDI status bytes and what they mean. The "K" key clears the screen

and also clears the history buffer.

-------------

Pressing "H" calls  up the history buffer, which  holds 30,000 bytes. After

the buffer  is filled, it  is still  updated,  so it always  holds the last

30,000 bytes.  The History Buffer can  be written to disk  as an ASCII file

which can then be printed.

---------------------------------------------------------------------------

SEND MODE

Send Mode  is used to find  out how equipment will  respond to various midi

data. The loop  repeat can be used to constantly  send a small sequence for

trouble shooting purposes. Use spaces for delays. Each space is about 0.028

(1/36th) sec. You  can enter up to two  lines of data. The "G"  key retypes

the last line(s) that were entered. The  "U" key allows you to send a "dump

request" and then view the response on the monitor screen. Just type in the

request, complete  with F0 at  the beginning  and  F7 at the  end, and then

press "U".  The request will  be sent, and  you will be  immediately put in

Monitor Mode to view  the response, with the first two masks  set to FE and

F8. (This is  because many synths & drum machines  keep sending these bytes

while doing  a bulk dump). The  response can then be  viewed in the history

buffer and saved to disk if desired.

It takes two  hex characters to make a  complete byte. If an odd  number of

hex characters is entered, the last  single character won't be sent. If you

press "U" with an odd number of  characters, the entry area will be blanked

and you won't be put in Monitor  Mode. If this happens, press "G" to retype

the entry, correct it, and then press "G" again.

-------------

The Real Time commands screen has  more options for sending data, including

MIDI clock and associated commands. The instructions on the screen are self

explanatory. This screen is particularly useful for checking sequencers and

drum  machines for  proper operation   and finding  out what  commands they

support.

Note: If you're  using Song Position Pointer, send the  pointer, and then a

"continue" command  (not start). The  start command always  means "start at

the beginning".

---------------------------------------------------------------------------

PATCH MODE

Patch  Mode consists  of several  subsections for  dealing with  saving and

transmitting bulk  patch or sequence data  that is in the  standard "System

Exclusive"  format. When  you first  enter Patch  Mode the  screen shows an

alphabetical listing, by extension or name, of all the files on the disk or

current subdirectory. The maximum number of  files that can be displayed on

a single  page is  114. The  maximum number  of pages  allowed is four (456

files). If  there are more, they  will not be displayed  and the "number of

pages" area at the  upper right corner of the screen will  have a star (*).

As files are added, deleted, or renamed, the directory will be updated. You

will notice  that the names of  programs that start with  "MIDI" (like this

one) are  not displayed. This  is done on  purpose to save  space for voice

bank listings. Do not name any files  with "midi...". If you do, they won't

be displayed.

The  "Receive"   option  will  save   System  Exclusive  dumps   from  midi

instruments. When you press "R", you are given the choice of initiating the

dump  from the  instrument or  sending a  "dump request".  Some common dump

requests are listed on the screen  along with instructions for entering the

request. Since it takes  two hex characters to make a byte,  if you type an

odd number of digits and press Enter, the request will not be sent, and you

will be instructed to  try again. If you elect not to  send a dump request,

the computer is  immediately put in sys-ex receive mode  and is waiting for

data. As data comes in, small  musical notes will appear between the braces

in the upper left  portion of the screen. When the notes  are gone, the "W"

key should  be pressed to  write the data  to the disk.  This is a separate

operation so that  separate bulk dumps from one  or several instruments can

be saved to  one file. Just keep initiating bulk  dumps until you are done,

then press the "W" key. This can be  done up to a maximum of 131,070 bytes.

A switch in  the SPX-90 has  to be changed  which makes the  "thru" jack an

"out" jack in order to use the special SPX-90 dump request.

The  "Transmit" option  transmits the  system exclusive  files back  to the

instruments.  You have  a "block-at-a-time"  choice here  in case  you have

saved several instruments to one file  and need time between each "send" to

connect  the proper  instrument to   the computer.  If the  instruments are

different,  they can  ALL be  fed the  file non  stop. Each instrument will

respond only to it's particular sys-ex message.

The "Rename" and "Delete" options are self explanatory.

The "File Size" command lists sizes of sys-ex dumps of various instruments.

On some instruments, like the Yamaha  DX7II, you have the option of sending

from as little as one bank to  as much as "everything". The numbers on this

screen are for "everything".

Pressing  "X" changes  the file  sort order.  The sort  order is for MIDI's

display  only -  the files  are not  permanently sorted. Occasional sorting

with a  file sort utility  will help  speed  up the operation  of MIDI when

changing subdirectories.

-------------

The (V)iew command  has two different meanings. If the  highlight is over a

file, pressing "V" will display the contents of the file in "debug" format.

This is  useful for looking  at voice names  in a bulk  dump file, or  just

being curious about what's there. At the bottom right corner of the display

is a "counter" that indicates the byte number of the last byte displayed on

the screen.

Any filenames with  an extension of "*s*" are  subdirectories. Pressing "V"

when a subdirectory is highlighted will take you to that subdirectory. This

is  useful  for  keeping  patches  for  different instruments separate. The

(B)ack command simply takes you "back" one subdirectory level.

Note: If you name a subdirectory with an extension, MIDI will not know that

it is a subdirectory. This doesn't hurt anything, you just won't be able to

move to it.

The (C)hange command allows you to  change default drives. If you select an

invalid  drive, there  will be  no error  message -  you just  won't change

drives.

---------------------------------------------------------------------------

DOS COMMAND LINE ARGUMENTS

MIDI supports command line arguments for  Monitor (M), Send (S), Patch (P),

black & white (B), and display sorted by name instead of extension (X).

For  example: To  immediately enter  Patch Mode,  type "MIDI  P" at the DOS

prompt.  For black  & white  display, files  sorted by  name, and initially

entering Monitor Mode, type "MIDI BXM".  The parameters can be in any order

and invalid parameters are ignored.

---------------------------------------------------------------------------

MIDI is shareware.

Contributions, suggestions, and bug reports are appreciated. Any additional

info  for the  "Bulk Dump"  and  "File  Size" screens  will be  included in

subsequent  updates. A  contribution will  insure that  you are informed of

updates.

If you give MIDI to other people, be  sure you them this text file with the

program.

Eddy Reynolds

11214 Olde Mint House Lane

Tomball, Texas 77375

BDQ855 on the Source

Enjoy !!

---------------------------------------------------------------------------

Major differences between this and previous versions:

1)  Added command line arguments.

2)  Use of resident programs is supported.

3)  Color is now default. Use "B" as a command line option for black &

white or monochrome.

4)  Patch Mode supports multiple pages of files (up to 456 files), and

changing drives or subdirectories.

5)  A movable highlight for selecting files in Patch Mode.

6)  View  option in Patch  Mode allows viewing files and checking  the

file size, creation date and time.

7)  The program "runs" without a midi interface installed.

---------------------------------------------------------------------------

NUMBER CONVERSION

~~~~~~~~~~~~~~~~~

******   Status Bytes   ******

~~~~~~ ~~~~~

HEX  DECIMAL  BINARY        HEX  DECIMAL  BINARY       HEX  DECIMAL  BINARY

~~~  ~~~~~~~  ~~~~~~        ~~~  ~~~~~~~  ~~~~~~       ~~~  ~~~~~~~  ~~~~~~

80   128   10000000         AB   171   10101011        D6   214   11010110

81   129   10000001         AC   172   10101100        D7   215   11010111

82   130   10000010         AD   173   10101101        D8   216   11011000

83   131   10000011         AE   174   10101110        D9   217   11011001

84   132   10000100         AF   175   10101111        DA   218   11011010

85   133   10000101         B0   176   10110000        DB   219   11011011

86   134   10000110         B1   177   10110001        DC   220   11011100

87   135   10000111         B2   178   10110010        DD   221   11011101

88   136   10001000         B3   179   10110011        DE   222   11011110

89   137   10001001         B4   180   10110100        DF   223   11011111

8A   138   10001010         B5   181   10110101        E0   224   11100000

8B   139   10001011         B6   182   10110110        E1   225   11100001

8C   140   10001100         B7   183   10110111        E2   226   11100010

8D   141   10001101         B8   184   10111000        E3   227   11100011

8E   142   10001110         B9   185   10111001        E4   228   11100100

8F   143   10001111         BA   186   10111010        E5   229   11100101

90   144   10010000         BB   187   10111011        E6   230   11100110

91   145   10010001         BC   188   10111100        E7   231   11100111

92   146   10010010         BD   189   10111101        E8   232   11101000

93   147   10010011         BE   190   10111110        E9   233   11101001

94   148   10010100         BF   191   10111111        EA   234   11101010

95   149   10010101         C0   192   11000000        EB   235   11101011

96   150   10010110         C1   193   11000001        EC   236   11101100

97   151   10010111         C2   194   11000010        ED   237   11101101

98   152   10011000         C3   195   11000011        EE   238   11101110

99   153   10011001         C4   196   11000100        EF   239   11101111

9A   154   10011010         C5   197   11000101        F0   240   11110000

9B   155   10011011         C6   198   11000110        F1   241   11110001

9C   156   10011100         C7   199   11000111        F2   242   11110010

9D   157   10011101         C8   200   11001000        F3   243   11110011

9E   158   10011110         C9   201   11001001        F4   244   11110100

9F   159   10011111         CA   202   11001010        F5   245   11110101

A0   160   10100000         CB   203   11001011        F6   246   11110110

A1   161   10100001         CC   204   11001100        F7   247   11110111

A2   162   10100010         CD   205   11001101        F8   248   11111000

A3   163   10100011         CE   206   11001110        F9   249   11111001

A4   164   10100100         CF   207   11001111        FA   250   11111010

A5   165   10100101         D0   208   11010000        FB   251   11111011

A6   166   10100110         D1   209   11010001        FC   252   11111100

A7   167   10100111         D2   210   11010010        FD   253   11111101

A8   168   10101000         D3   211   11010011        FE   254   11111110

A9   169   10101001         D4   212   11010100        FF   255   11111111

AA   170   10101010         D5   213   11010101

******   Data Bytes   ******

~~~~ ~~~~~

HEX  DECIMAL  BINARY        HEX  DECIMAL  BINARY       HEX  DECIMAL  BINARY

~~~  ~~~~~~~  ~~~~~~        ~~~  ~~~~~~~  ~~~~~~       ~~~  ~~~~~~~  ~~~~~~

0     0   00000000         2B    43   00101011        56    86   01010110

1     1   00000001         2C    44   00101100        57    87   01010111

2     2   00000010         2D    45   00101101        58    88   01011000

3     3   00000011         2E    46   00101110        59    89   01011001

4     4   00000100         2F    47   00101111        5A    90   01011010

5     5   00000101         30    48   00110000        5B    91   01011011

6     6   00000110         31    49   00110001        5C    92   01011100

7     7   00000111         32    50   00110010        5D    93   01011101

8     8   00001000         33    51   00110011        5E    94   01011110

9     9   00001001         34    52   00110100        5F    95   01011111

A    10   00001010         35    53   00110101        60    96   01100000

B    11   00001011         36    54   00110110        61    97   01100001

C    12   00001100         37    55   00110111        62    98   01100010

D    13   00001101         38    56   00111000        63    99   01100011

E    14   00001110         39    57   00111001        64   100   01100100

F    15   00001111         3A    58   00111010        65   101   01100101

10    16   00010000         3B    59   00111011        66   102   01100110

11    17   00010001         3C    60   00111100        67   103   01100111

12    18   00010010         3D    61   00111101        68   104   01101000

13    19   00010011         3E    62   00111110        69   105   01101001

14    20   00010100         3F    63   00111111        6A   106   01101010

15    21   00010101         40    64   01000000        6B   107   01101011

16    22   00010110         41    65   01000001        6C   108   01101100

17    23   00010111         42    66   01000010        6D   109   01101101

18    24   00011000         43    67   01000011        6E   110   01101110

19    25   00011001         44    68   01000100        6F   111   01101111

1A    26   00011010         45    69   01000101        70   112   01110000

1B    27   00011011         46    70   01000110        71   113   01110001

1C    28   00011100         47    71   01000111        72   114   01110010

1D    29   00011101         48    72   01001000        73   115   01110011

1E    30   00011110         49    73   01001001        74   116   01110100

1F    31   00011111         4A    74   01001010        75   117   01110101

20    32   00100000         4B    75   01001011        76   118   01110110

21    33   00100001         4C    76   01001100        77   119   01110111

22    34   00100010         4D    77   01001101        78   120   01111000

23    35   00100011         4E    78   01001110        79   121   01111001

24    36   00100100         4F    79   01001111        7A   122   01111010

25    37   00100101         50    80   01010000        7B   123   01111011

26    38   00100110         51    81   01010001        7C   124   01111100

27    39   00100111         52    82   01010010        7D   125   01111101

28    40   00101000         53    83   01010011        7E   126   01111110

29    41   00101001         54    84   01010100        7F   127   01111111

2A    42   00101010         55    85   01010101