Polyphonic Music Compiler 1.10

Polyphonic Music Player

Copyright (c) 1989 GrigaSoft Productions

_____________________________________________

/                                             \

-*> FOR UPDATES FROM VERSION 1.00 SEE THE FILE: <*-

-*>                UPDATE.TXT                   <*-

-*> ALL UPDATED MATERIAL THAT IS DOCUMENTED IN  <*-

-*>  UPDATE.TXT IS MARKED '(UPDATED)' IN THIS   <*-

-*>                  MANUAL                     <*-

\_____________________________________________/

IF YOU HAVE ANYTHING YOU'D LIKE TO SEE DONE

WITH THIS PACKAGE (SPECIFIC SONG CODINGS,

NEW APPLICATION IDEAS, ETC.) PLEASE CONTACT

THE AUTHOR (EDWARD GRIGASSY, (512)385-1368)

[call collect if you have to!]. For more

personal info see section 7.5 of this maunal.

INTRODUCTION

------------

Welcome to the world of polyphonic music!  Recently several

programmers working seperately have discovered and implemented

the amazing ability to play three (count 'em) independent voices

on the cheap IBM PC/AT/Clone speaker. Why is this amazing? Well,

for years any IBM "Music" program was laughed at by experts simply

because all the software could do was drone out a simple (not to

mention, dull) melody. Lately the techniques to play three notes

at a time, allowing true chords and harmony, has been developed.

This package exemplifies the continued refining of these

techniques.

CREDITS

-------

Steve Muenter: wrote the original assembly algorithm

for playing three notes at once,

Ted Holden: informed me of this algorithm and his program

VMUSIC inspired all my work

TABLE OF CONTENTS

-----------------

Subject                                                 Section

-------                                                 -------

What's included ......................................... 1.0

What is Polyphonic Music? ............................... 2.0

MakePoly Compiler ....................................... 3.0

Explanation .......................................... 3.1

Use .................................................. 3.2

Requirements ...................................... 3.21

Starting MakePoly / Command Line .................. 3.22

Text File Format / Syntax ......................... 3.23

PlayPoly Player ......................................... 4.0

Explanation .......................................... 4.1

Use .................................................. 4.2

Requirements ...................................... 4.21

Starting PlayPoly / Command Line .................. 4.22

Other Polyphonic Music Programs ......................... 5.0

AutoHarp ............................................. 5.1

LoopPoly ............................................. 5.2

SoundEff ............................................. 5.3

TimeCHK .............................................. 5.4

Using the Polyphonic C library .......................... 6.0

The Library Routines ................................. 6.1

Using the Polyphonic Music Library ................... 6.2

How to use the Library Routines ...................... 6.3

Author's Note ........................................... 7.0

Program Development .................................. 7.1

Program Features ..................................... 7.2

Program Info ......................................... 7.3

Future Enhancements .................................. 7.4

Author's Corner ...................................... 7.5

1.0    WHAT'S INCLUDED

---------------

Included in this current version of Polyphonic Music are the

following files:

MAKEPOLY.EXE    -- The textfile to polyfile compiler

PLAYPOLY.EXE    -- The polyfile player

LOOPPOLY.EXE    -- The polyfile player which repeats until

a key is pressed

AUTOHARP.EXE    -- The executable version of an example of

how to use the Polyphonic Music C interface.

SOUNDEFF.EXE    -- A sound effect program which uses the Poyphonic

Music C interface.

TIMECHK.EXE     -- The executable version of an example of

how to use the processor speed-checking functions

of the Polyphonic Music library.

AUTOHARP.C      -- The source code of AUTOHARP.EXE.

SOUNDEFF.C      -- The source code of SOUNDEFF.EXE.

TIMECHK.C       -- The source code of TIMECHK.EXE.

POLYCOMP.BAT    -- An example compilation batch file for

a C program calling the Polyphonic Music functions

POLY.H          -- The header file for the Polyphonic Music C

library

POLYCHRD.H      -- The header file for defining the array constants

of lots of chords

POLY.LIB        -- The Polyphonic Music C interface library.

NPOLY_S.OBJ     -- The UPDATED C extended function poly() (small

memory model)

NTRIT_S.OBJ     -- The UPDATEED C extended function tritone() (small

memory model)

POLYFUNC.OBJ    -- The other C functions in the Polyphonic Music

library

SIMPTEST.PLY    -- A polyfile song that is probably the first three-

voice song ever heard on an IBM (see 7.1 -

PROGRAM DEVELOPEMENT)

TRIPTEST.SNG    -- An example song file that displays use of

simulated triplets

POLY.DOC        -- The documentation file you are reading

*.SNG, *.PLY    -- Various song files to play

2.0    WHAT IS POLYPHONIC MUSIC?

-------------------------

The Polyphonic Music programs are an innovative system

for allowing three voice music to be played on the IBM PC and

compatibles. The package provides several uses of Polyphonic

Music for the user. These are:

o A command line controlled jukebox, in which the user simply

compiles (MakePoly.exe) and plays (PlayPoly.exe) his/her

songs,

o A batch file enhancer, which can play three part music while

waiting for a keypress, and

o A C programming language enhancer, which allows three voice

music to be played within a C program until a keypress is

detected.

[See section 7.4 - Future Enhancements for uses soon to be

implemmented!]

The Polyphonic Music programs use two techniques to play

music. The first is the ability to compile a text file with

commands similar to the IBM BASIC PLAY statement into a ".ply"

file format which can be directly played from the command line,

batch files, or even from within your C programs themselves.

The second technique is only usable within C programs. It involves

defining the music to be played within the program (in array form).

This second method may be preferrable if the song is short or

if the programmer doesn't wish to access external files.

3.0    MAKEPOLY COMPILER

-----------------

3.1  EXPLANATION

-----------

The MAKEPOLY.EXE file, when run, compiles text (ascii)

files into files in a ".PLY" format which can then be directly

read by other Polyphonic Music programs. This program is necessary

to translate a file of ascii music commands into the raw

frequencies that the computer can play.

3.2  USE

---

3.21    REQUIREMENTS

------------

The MAKEPOLY.EXE program will run in MS-DOS (and

compatibles) 2.0 or later. It needs less than 80K of

memory depending on the size of the text file being

compiled (usually needs 15K to 64K). It can be run from

either a floppy diskette or a hard disk. To work

properly, a text file of music commands (see 3.23 - TEXT

FILE FORMAT / SYNTAX) is required. (For more information

on errors, see 3.24 - MAKEPOLY ERRORS.)

3.22    STARTING MAKEPOLY / COMMAND LINE

--------------------------------

To run MakePoly, make sure you are in the directory

where MAKEPOLY.EXE resides. Enter the following command

at the DOS prompt:

MAKEPOLY <file1> [<file2>]

where:

<file1> = the name of the text file which contains

the Polyphonic Music commands (see 3.23 -

TEXT FILE FORMAT / SYNTAX).

This is the file to be compiled.

<file2> = the name of the output ".ply" file which

contains the raw music (frequencies, and

durations) which can then be played by

other Polyphonic Music routines, such as

PLAYPOLY.EXE (this name is optional)

Note: If no extension is given on <file1> and/or <file2>,

MAKEPOLY assumes ".SNG" (song) for <file1>,

and ".PLY" (poly or play) for <file2>.

Also, if the <file2> parameter is not given,

MAKEPOLY will attempt to add the ".PLY" extension

to <file1>.

Examples:

to compile a file named "SIMPTEST.SNG":

MAKEPOLY simptest

to compile a file named "ODE"

MAKEPOLY ode. ode.ply

3.23    TEXT FILE FORMAT / SYNTAX

-------------------------

MAKEPOLY compiles a file in a special format.

The file is made up of ascii text with certain ascii

characters representing certain music commands.

The commands can be in upper or lower case.

The commands have been derived directly from recent

versions of the IBM BASIC "PLAY" command (although a few

new commands have been added).

Here is a list of the commands that MAKEPOLY will

compile:

On  --  Sets the octave of subsequent note output. Any

integer n is allowed, but values between 0 and

6 are recommended. The value 3 is the octave

containing middle C (the only C written without

ledger lines in the Treble Clef). Octaves n < 3

are lower in pitch, while those with n > 3

are higher in pitch. Each octave contains

the notes from C to B (ascending).

(note: if not specified MAKEPOLY assumes 3)

Example: O4  -> sets subsequent note output to

the octave above middle C

Ln  --  Sets the default note length. The integer n

can be one of the following:

1  -- representing a whole note

2  -- half note

4  -- quarter note

8  -- eigth note

16 -- sixteenth note

32 -- thirty-secondth note

64 -- sixty-fourth note

(note: n can be other values between 1 and 64, see

7.2 - PROGRAM FEATURES (discussion on

triplets) for details)

The number n is the length any subsequent note

which doesn't have an explicit length following it

will be assigned. (This is useful when a piece

comprises largely of one type of note length.)

(note: if not specified MAKEPOLY assumes 4)

Example: L4  -> sets default note length to quarter

notes

Tn  --  (UPDATED)

Changes the current tempo for all subsequent notes.

See the file: UPDATE.TXT

This command, as well as the K command, can be

embedded in any voice. The tempo of all three voices

is changed at the point the Tn command is found.

>,< --  Raise current octave up, down. The command > raises

the current octave set by the O command, while the

command < lowers it. The current octave is raised

or lowered by 1.

Vn  --  Set subsequent notes to be played by voice n.

The integer n can ONLY have the values 1, 2, or 3.

All the following notes will be programmed to play

in voice n, until another Vn is found with a different

value for n.

(note: an error occurs when you attempt to play

notes before setting which voice to play them

on; no voice is assumed as default)

Any combination of the three voices is allowed,

including ommission of 1 or more of them.

Example: V1  -> sets up voice 1 to play the

subsequent notes

K   --  Check the keyboard for a keypress. This command

will abort the playing of the song if a key has

been pressed. The playing routine will only abort

in the place where the K command is specified.

This command can be in any place in the song and

it causes the player to check if a key pressed,

and abort if so, at that point in the song.

Example: V1EEKEF  -> check for a keypress

after playing two 'E' notes

(if no key pressed, play 'EF')

*..*    (UPDATED)

Allow a user comment. All text in between two

asterixes is ignored by the compiler.

This is useful if the music scorer needs to mark a

section of music or explain something about what he

is scoring.

Example: *voice1:*  V1O3 AB   -> The words "voice1:"

are ignored (skipped

over) by MAKEPOLY

Notes are entered in the following manner:

<N>[#,+,-][n][.[.[. etc.]]]

where:

<N>  = the name of the note. This may be one of

the following letters ONLY:

A,B,C,D,E,F,G  -- musical notes

P,R            -- rests (pauses, silences)

[#,+,-]  = optional note modifier.

#,+ -- makes the note "sharp"

-   -- makes the note "flat"

(note: B# (or B+) = C, E# (or E+) = F,

and C- = B, F- = E)

[n] = the optional note length. Can be of the possible

values described under the L command. If n is not

included, MAKEPOLY assumes the default note length

set by the last L command.

[.[.[. etc.]]] = the optional dots on the note. Each dot

modifies the note's length by adding

1/2 of the current value. There is no

limit to the number of dots modifying

any given note.

Examples:

e       -- play note 'e'

f#4     -- play 'f sharp' for a quarter note length

duration

a+8.    -- play a dotted 'a sharp' eighth note

b-      -- play 'b flat' for default duration

specified by last L command

4.0    PLAYPOLY POLYPHONIC MUSIC PLAYER

--------------------------------

4.1 EXPLANATION

-----------

The PLAYPOLY.EXE file, when run, plays a ".ply" file

in polyphonic (three voice) sound. The ".ply" file is a special

file format that cannot be adjusted or modified directly; it

is only for use by the Polyphonic Music playing routines.

4.2 USE

---

4.21    REQUIREMENTS

------------

The PLAYPOLY.EXE program will run in MS-DOS (and

compatibles) 2.0 or later. It needs less than 80K of

memory depending on the size of the ".ply" file being

played (usually 15K to 64K). It can be run from

either a floppy diskette or a hard disk. To work

properly a ".ply" file must be specified. This file

must be in a specific machine-readable format.

4.22    STARTING PLAYPOLY / COMMAND LINE

--------------------------------

To run PlayPoly, make sure you are in the directory

where PLAYPOLY.EXE resides. Enter the following command

at the DOS prompt:

PLAYPOLY <file1> [tval,/c,/s]

where:

<file1> = the name of the ".ply" file which contains

the Polyphonic Music in raw form. Such a

file can only be created by the MakePoly

compiler.

This is the file to be played.

One of the following optional 2nd parameters may be used:

tval    = the tempo you would like to set PlayPoly

to play at. If not given, PLAYPOLY assumes

1024. This can be any positive number ranging

up to 65535. The lower the number, the

faster the tempo. Included in the Polyphonic

Music package is a program called TIMECHK.EXE

which will approximate a good tempo value for

your system (see 5.4 - TIMECHK).

(UPDATED)

/C      = the song's starting tempo is determined

by a processor speed determining routine.

/S      = same as /C, except the status line "PLAYPOLY.EXE:

Checking processor speed..." is not displayed.

Note: If no extension is given on <file1>,

PLAYPOLY assumes ".PLY" (for "poly" or "play").

Examples:

to play a file named "SIMPTEST.PLY":

PLAYPOLY simptest

to play a file named "ODE.JOY"

PLAYPOLY ode.joy

to play a file named "INVENT13.PLY" at tempo = 512

PLAYPOLY invent13 512

to play a file named "INVENT13.PLY" and set the tempo

according to the processor speed

PLAYPOLY invent13 /c

5.0    OTHER POLYPHONIC MUSIC PROGRAMS

-------------------------------

5.1 AUTOHARP

--------

As you may have noticed, several other mysterious programs

have reared their ugly heads in this package. The first one,

AUTOHARP is an interesting attepmt to breathe creativity into the

PC speaker.

Run it by entering AUTOHARP at the DOS prompt (in the drive/

directory where AUTOHARP.EXE is located). The keyboard layout

screen comes up.

To use this program, play your favorite chord progression

by pressing the keys representing the chords you want. The top

row of the keyboard (numbers) will play all your major chords.

The next row plays minor chords, and the third plays diminished

chords. By pressing <Shift> along with any key used plays a

seventh of the chord in question. For instance, to play a G7 chord

(that's G DOMINANT 7, to be specific), press <Shift '-'>.

One other note: the tempo of the chord was set in the

POLYCHRD.H include file and its a good speed for an IBM PCXT but

may be way too fast for a quick processor. If you wamt to change

the tempo modify the source code and recompile it.

Side note: the way in which all those chords were defined is

kind of nasty (they're defined in POLYCHORD.H). A much better

solution for chord playing would be to have some sort of template for

the form of each chord, and then make a key change appropriately.

A discussion of the procedures called in AUTOHARP.C can be found

in 6.1 - THE LIBRARY ROUTINES.

5.2 LOOPPOLY

--------

This program is identical to PLAYPOLY.EXE in every way except

that at the end of a song it plays it over if a key has not been

pressed. This is useful in batch files while waiting for user input.

One other note, the ascii value of the key pressed which

ended the program is returned in the DOS variable "errorlevel", and

can be accessed in batch files.

5.3 SOUNDEFF

--------

This program just uses the Polyphonic Music library function

"tritone()" to sound as much like an airplane (with a propellor)

(or something else very peculiar) as it can.

5.4 TIMECHK

-------

TIMECHK.EXE is a very useful tool in the Polyphonic Music world.

Not only will it benchmark your computer, but it gives you an

accurate tempo value to set playback of your ".ply" files.

You run TIMECHK.EXE by typing TIMECHK at the DOS prompt and

pressing enter. After a few seconds of processor intensive checking

routines, TIMECHK returns the number of hundredths of a second that

your processor completed these tasks in. Also it evaluates an

approximate Polyphonic Music tempo value from this count.

Just for comparison, I ran this on some of the machines around

the office:

Hund. of Secs.      Poly. Tempo

IBM original PC                45                 465

Turbo XT clone                 20                1024

IBM original AT                14                1462

IBM PC + 286 charge card       13                1706

IBM PS2 model 50                8                2560

286 machine                     6                3413

386 machine                     3                6820

The best use for this value is in making a batch file for

your player files. For instance, if you had the 386 machine, you

could call PLAYPOLY with a batch file PLAY.BAT that looks like

the following:

PLAYPOLY %1 6820

Note: If your computer is so fast that the hundredths of a

second value is 2 or lower, you may have to experiment with large

tempos (7000 or above) to find the correct value for your computer.

A discussion of the C functions used in the source code

of TIMECHK can be found in section 6.1 - THE LIBRARY ROUTINES.

6.0    USING THE POLYPHONIC C LIBRARY

------------------------------

6.1 THE LIBRARY ROUTINES

--------------------

The Polyphonic library consists of several functions that

can be directly accessed within your C program source code and

can be then linked in to your executable modlues. Since not all

C compilers are the same, I've included both the .LIB file

(POLY.LIB) containing all the enhancement functions, and its

component .OBJ files.

All the functions are defined in the header file POLY.H, and

are supported in POLY.LIB. Within this package only near pointers are

accounted for. Any LARGE structures will probably not work. You can

order the Large memory model versions of these functions (see the

text file included in this package: REGISTER.TXT).

Here are descriptions of the Polyphonic Music functions:

-------------------------------------------------------------------

#include <poly.h>

int playpoly(plyfilename, defaulttempo)

char *plyfilename;                         -- DOS path to play

int defaulttempo;                          -- beginning tempo

Object module: polyfunc.obj+poly.obj

Description:

The playpoly routine, when passed a legal filename (of the

proper ".ply" type), and a beginning tempo, will play the three-

voice song defined by the data in the ".ply" file.

The playpoly routine can only read files of the type created by

the MAKEPOLY.EXE program.

If a file extension is not specified in the string plyfilename,

a ".ply" is assumed.

The default tempo must be in the range of 1 (very fast) to 65535

(very slow). This value is the starting tempo of the music

file when played. (UPDATED)

This routine will allocate memory and read in the file. If successful,

it will close the file and play the entire file until completion.

Then the allocated memory is freed.

A keypress may abort the playing if such an action was provided for

in the text version of the song (see 'K' command, section 3.23 -

TEXT FILE FORMAT / SYNTAX).

Return value:

playpoly() returns a 0 (zero) if no errors occured during play.

If an error does occur, an appropriate message is written to stderr,

and a value of -1 (0xFF) is returned.

See Also: looppoly(), polytempo()

Example:

#include <stdio.h>

#include <poly.h>

main()

{

printf(" 'Ode to Joy'. Press any key to stop repititions.... ");

while (!kbhit())

playpoly( "ode", polytempo(10,250) );

printf("\r\n");

}

-------------------------------------------------------------------

#include <poly.h>

int looppoly(plyfilename, defaulttempo)

char *plyfilename;                         -- DOS path to play

int defaulttempo;                          -- beginning tempo

Object module: polyfunc.obj+poly.obj

Description:

The looppoly routine, when passed a legal filename (of the

proper ".ply" type), and a beginning tempo, will play the three-

voice song defined by the data in the ".ply" file. It will play

the song reptitively until a key has been pressed.

The looppoly routine can only read files of the type created by

the MAKEPOLY.EXE program.

If a file extension is not specified in the string plyfilename,

a ".ply" is assumed.

The default tempo must be in the range of 1 (very fast) to 65535

(very slow). This value is the starting tempo of the music

file when played. (UPDATED)

This routine will allocate memory and read in the file. If successful,

it will close the file and play the entire file until completion.

If a key has not been pressed, it will start playing again.

When a key press is detected, the allocated memory is freed.

The keyboard is only checked every time the song ends!

A keypress may abort the playing DURING the song if such an action

was provided for in the text version of the song (see 'K' command,

section 3.23 - TEXT FILE FORMAT / SYNTAX).

Return value:

looppoly() returns the ascii/scan value (getch()) of the keypress

detected. If an error occurs, an appropriate message is written to

stderr, and a value of -1 (0xFF) is returned.

See Also: playpoly(), polytempo()

Example:

#include <stdio.h>

#include <poly.h>

main()

{

printf(" 'Ode to Joy'. Press any key to stop repetitions.... ");

looppoly( "ode", polytempo(10,250) );

printf("\r\n");

}

-------------------------------------------------------------------

#include <poly.h>

unsigned timetest(sizechk)

int sizechk;                       -- number of nodes to test

Object module: polyfunc.obj+poly.obj

Description:

The routine timetest() is used to determine the current processor's

speed. The function itself does several processor intensive tasks

on an array of size sizechk, and returns the amount of hundredths

of seconds required by the processor.

The sizechk parameter determines the size of a mallocated array

and thus the complexity and length (in time) of the test. The larger

this value is, the longer the test will take, the more memory it

will require, and the more accurate the return value will be.

Return value:

timetest() returns the number of hundreths of a second required to

complete several processor intensive routines.

On fast machines, sizechk values below 300 can often return 0.

See also: processval(), polytempo()

Example:

#include <stdio.h>

#include <poly.h>

main()

{

int numnodes;

printf("How many nodes to test? ");

scanf("%d", &numnodes);

printf("\r\n");

printf("Test completed in %d hundreths of a second.\r\n",

timetest(numnodes));

}

-------------------------------------------------------------------

#include <poly.h>

unsigned processval( times, nodes )

int times;                           -- number of times to run test

int nodes;                           -- number of nodes to use

Object module: polyfunc.obj+poly.obj

Description:

This routine runs the timetest() routine the number of times sent

in the 'times' parameter using arrays of 'nodes' size. It then

returns the average of all the tests to give a processor dependent

value which can be used to portably move your programs from computer

to computer, adjusting program speed by means of a ratio of processor

value to your original value.

This technique was used in the PlayPoly routines.

Return value:

The amount of hundreths of a second, on average, the processor took

to do the timetest() function 'times' times, with 'nodes' nodes.

See also: timetest(), polytempo()

Example: SEE the included file TIMECHK.C for an example of this

routine.

-------------------------------------------------------------------

#include <poly.h>

unsigned polytempo( times, nodes )

int times;                           -- number of times to run test

int nodes;                           -- number of nodes to use

Object module: polyfunc.obj+poly.obj

Description:

This routine runs the timetest() routine the number of times sent

in the times parameter using arrays of nodes size. It then returns

an approximate value which is the optimum polyphonic tempo for

the current processor to play at. This function allows portability

of all the PlayPoly routines that require a tempo value, as the

processor which is about to do the playing can be tested for speed

first.

This technique was used in the PlayPoly routines.

Return value:

The approximate optimum tempo for the PlayPoly routines to play at.

(note: this function involves division and if the processval()

function returns a 0 because of a very fast processor,

this function will cause a division by zero error)

See also: timetest(), processval()

Example: SEE the included file TIMECHK.C for an example of this

routine.

-------------------------------------------------------------------

#include <poly.h>

void poly( song )  (UPDATED)

int *song;                  -- array of ".ply" format music to play

Object module: npoly_s.obj

Description:

This routine plays the array of integers pointed to by song.

This routine drives the PC speaker. The integer array is in a

special format. Each value in the array means a specific command

to the function.

THE FIRST VALUE IN THE ARRAY IS THE TEMPO TO BEGIN PLAY AT.

The constant values below are defined in the POLY.H file.

The value that accompanies each comand is ORed (|) into the

low 13 bits of the integer.

A value of:         Means:

TCHANGE             modify the starting tempo (see UPDATE.TXT

with value being equivalent to the n in

the updated version of the Tn command)

V1                  set the frequency of voice 1 to value

V2                  set the frequency of voice 2 to value

V3                  set the frequency of voice 3 to value

KBCHECK             check the keyboard buffer to see if a key

has been pressed

DUR                 play the three voices (set by V1..V3) for

a duration of DUR * Current_Tempo

STOP                quit playing, return to calling program

A song is played by setting the tempo, setting the three voices, and

setting the duration (which then plays the three voices), until a

STOP value is encountered.

Return value:

There is no return value.

See also: tritone(), playpoly()

Example: SEE the included file AUTOHARP.C and POLYCHRD.H for an

example of this routine.

-------------------------------------------------------------------

#include <poly.h>

void tritone(tempo, v1, v2, v3, duration)

int tempo;                        -- the tempo to play at

int v1, v2, v3;                   -- the three voice frequencies

int duration;                     -- the duration of the 3 voices

Object module: ntrit_s.obj

Description:

This function plays three notes simultaneously on the speaker.

The tempo value is usually an static variable which applies

throughout the program and is processor dependent. For best

results in frequencies, use the defined values in the POLY.H

file.

This function can be used for music or sound effects.

Return value:

There is no return value.

See also: poly(), playpoly(), polytempo()

Example: SEE the included file SOUNDEFF.C for an example of this

routine.

6.2 USING THE POLYPHONIC MUSIC LIBRARY

----------------------------------

In order to use the Polyphonic C routines, insert them as you

normally would into your source code (for an example, see any of the

".c" files included in this package).

Next, compile your source code to object modules (".obj" files).

In other words, compile WITHOUT linking.

Finally, if you are using Microsoft C, link your object modules

to the POLY.LIB library (as well as to the standard library).

This step may not work with other brands of C compilers, so I've

included the .OBJ files included in POLY.LIB. If you can't use

the POLY.LIB or it doesn't work, link your C source files to

the necessary object files (each function's necessary object files

are listed before their description in section 6.1 - THE LIBRARY

ROUTINES). This usually involves using a '+' command with your

linker.

For example: (taken from the file POLYCOMP.BAT)

cl /c myprog.c         --> compiles without linking (Microsoft C)

link myprog,,,+poly;   --> links myprog.obj with standard library

and also uses poly.lib (Microsoft Linker)

myprog.exe             --> runs myprog

If you use just the object files:

cl /c myprog.c                   --> say myprog calls playpoly()

link myprog+polyfunc+poly_s;     --> link necessary object files

myprog.exe                       --> run program

6.3 HOW TO USE THE LIBRARY ROUTINES

-------------------------------

PlayPoly/LoopPoly:

These functions need a legitimate ".ply" file to play.

First, type in the music you want to play in the text format

outlined in section 3.23. Make sure you place the 'K' command

wherever necessary (to abort on keypress). It is suggested that

you use the 'K' command in rests or at the end of phrases or

measures, since it may take a small time interval to scan the

keyboard (but usually this is not noticable). Also, be generous

with them because your program user probably does not want to

have to wait until the end of several measures for the program

to continue.

Second, compile the text file to a ".ply" file format using the

included program MAKEPOLY.EXE.

Third, use the path of the file in a call to the playpoly()/

looppoly() functions (see section 6.1 - THE LIBRARY ROUTINES).

Fourth, compile and link your C program as described above (section

6.2).

Poly:

This function needs a legitimate array whose members are

a list of integers in the ".ply" format.

There are two ways to make this integer array. One is to load the

array from a ".ply" file into memory (see PlayPoly/LoopPoly above

for how to obtain the file). The second way is to hard code the

music into your source code. Using this method has the advantage

that the music takes up no external files, it is stored within the

executable program itself.

The way this second method is implemented is as follows:

(UPDATED)

1. Define an integer array,

2. Fill it with the ".ply" values making up your

song (this involves a certain amount of derivation),

3. Call poly() with the array pointer as a parameter.

The way in which you fill the array with ".ply" values (step 2

above) is as follows:

1. Use the defined values created in the POLY.H file.

2. Set the tempo by plugging a tempo value into the

array (an unsigned integer in the range of 1 to 65535).

3. Set the voices by plugging (Vn | voicefreqency) into the

array (where 'n' is '1', '2', or '3'). You can use

the frequency values defined in POLY.H. They are set

up by nameofnote_octave, e.g. C sharp in the middle

octave (the one with middle C in it) = CSHA_M. Set these

defines according to your usage. Note: multiplying these

numbers by 2 raises one ocatve, and dividing by 2 lowers

one octave.

4. Set the duration of the notes by plugging (DUR |

notelength) into the array. This will play the last

defined voice values for the duration given. If any voice

was not defined it will use the last frequency given.

If there was never any value defined for any one voice

it will play 0 (silence) for that voice.

5. Check for keypresses at any point by plugging (KBCHECK)

into the array

6. Continue steps 3 - 5 (use TCHANGE | changeval as described

in UPDATE.TXT to change the tempo in the middle of the

song) until the song is completely hard coded.

7. Plug the last value (STOP) into the array

For an example of this, see the header file POLYCHRD.H which defines

the play of several three-note chords as if they were songs.

Tritone:

Use this function exactly as you would Poly, except send

the VALUES of tempo, v1, v2, v3, and duration as parameters, NOT

as an array. This function does not check for a keypress and

a STOP value is not necessary since it stops once the duration

expires.

TimeTest/ProcessVal/PolyTempo:

Use these functions to test the speed of a processor. They

should be self-explanatory.

7.0    AUTHOR'S NOTE

-------------

7.1 PROGRAM DEVELOPMENT

-------------------

The idea for this program first came the time I excitedly

downloaded PIANOMAN.ARC from my favorite bulletin board. I

impatiently loaded up the program waiting to see what one

unknowingly deceiving uploader penned as "Plays polyphonic

music!". When it loaded up I was sorely disheartened, as the

pianoman was as far from a piano as a computer could be (well,

next to having no speaker at all).

Then one day I noticed a file that said VMUSIC.ARC "plays

notes simultaneously". I had to check it out. And sure enough, it

did what it claimed. It did not "simulate" three voices by playing

three notes real fast through one voice. All three voices were there!

This interested me and I played a little with the program.

The time came when I wanted to use the possibility of three

voices in a program I was writing. VMUSIC couldn't help me because

it was unable to be accessed from C easily and quickly (not to

mention it was copyrighted!), so I called up Ted Holden, the

writer of VMUSIC. He didn't give me any code, but he referred me

to the work of Steve Muenter. This was the guy who wrote the low-

level technique for three voices. So, I got a copy of this assembly

code and I hooked it up to C.

I thought that a music compiler (putting the music into

raw form for quick playing) would be a good idea, so I wrote

MakePoly. I wrote it so that the three voices could be implemented

into C.

Basically this is the end result. The C enhancement

routines provided in this package expand on the original concept

and applications of three-voice music. And there is even more in

the works (from Mr. Holden as well as myself -- see 7.4 - PROGRAM

ENHANCEMENTS).

Every thing in this package was written in either C or

assembly (here's your free plug, Microsoft (5.1 to be exact)).

* Three voice technology first started with Steve Muenter (as

far as I know). He created a cryptic BASIC interface with a

function similar to the poly() function in this package.

His program played the song contained in the file SIMPTEST.PLY.

The form it is in now is not very far from the original form

in was in!

Ted Holden crystalized this idea by allowing imitation BASIC

commands in ascii text files to be played. *

7.2 PROGRAM FEATURES (UPDATED)

----------------

Both MAKEPOLY/PLAYPOLY and VMUSIC use the Basic-like commands

to read and play three-voice music. They both are good music

writing/composition tools.

In the latest version of VMUSIC which is propogating around

the nation's bulletin boards, Richard Pierce scored lots of Bach

pieces and offered some comments to Ted Holden. In one of the

README files Ted responds directly to Richard Pierce's suggestions.

I'd like to do the same (my comments delimited by **'s):

"There  are some obvious ways (to me, at least) vmusic could

be enhanced. This is contradictory to the concept of vmusic  as,

really, a toy program (with no intention on my part to denegrate

it or the authors efforts). Many of the suggestions I have would

be satisfied by MIDI sequencer programs. I doubt that the author

had any inkling that vmusic would be put to such  efforts  as  I

have here. Be that as it may...

Check!> Tempo

Implementation  of  the  Basic "play" command for tempo 't'

would be very useful. Several  of  the  pieces  performance

style  could  hve been immeasurably helped by being able to

implement even incremental changes of tempo, much less true

accelerando and ritardando.

** Incremental tempo changes are supported in this version...

(see UPDATE.TXT) **

Check!> Triplets

True  triplet  notes  (where  three notes are played in two

beats) could have made some of the pieces far  easier,  and

would  have  encouraged me to try others. One way that this

could be noted in the music source would be some convention

that would group the triplets together, such as:

l8

v1o4 (efe)3 (ded)3 c4

v2o3 c<b    >cd     e4

where  the triplet eighth notes like "efe" are played three

against the normal two eighth notes "c<b" in a  normal  3/4

time  measure.  The  extension  of  this to sixth groupings

"(......)6" and even seventh groupings  "(.......)7"  would

enable  many  other  pieces  to be simply coded (the latter

being true for the first movement of Bach's Partita #6).

** MAKEPOLY will allow "simulated" triplets. The manner in which

this is accomplished is the following:

Every note length in the <textfile> is first converted to

a real time value. Specifically,

Note Length:     1   2   4    8   16   32   64

- is converted to -

Real Time Duration:  64  32  16   8   4    2    1

(the formula is RTD = 64 / NL)

So, if you time your triplets in real time, and then set

your note length accordingly, you can fake them.

For instance, say you want a triplet for eighth notes.

This would be equivalent in real time to 2 eighth notes, i.e.

real time = 8+8 = 16, so your three notes would need to add up

to 16. Let's use 5, 5, and 6. Since NL = 64 / RTD, and 64/5 =

12.8, and 64/6 = 10.67, use the notelengths, 13, 13, and 11,

which add up to (when applied through the original formula, and

rounded) 16. This works only because MAKEPOLY rounds the RTD

derived from the formula. Try it!

This example is realized in the file TRIPTEST.SNG.  They sound

like triplets to me! **

More voices

Simply said...

** It is possible. I tried four (have capability for up to five)

but the noise level was too much. I'll continue work on this! **

Platform independence

It would be nice to be able to  detect  the  speed  of  the

machine,  then set the tempo and pitch parameters automati-

cally (or at least scale them) so that the same piece would

sound the same on different hardware.

Check!> ** PLAYPOLY currently offers this capability (see the command line

parameter /C, section 4.22 - STARTING PLAYPOLY / COMMAND LINE)

for playing songs which don't change tempo internally. This

option has not been fully tested to see whether it reproduces

pitches perfectly or just scales them (most likely the pitches

are only scaled). A better speed detector would be nice

(one that plays silence for instance!) **

Syntax checking

A   more  intelligent  parser  would  help  debugging.  For

example, if  vmusic  had  some  knowledge  of  the  measure

length,  it could detect missing or excessive notes in each

measure.

** MAKEPOLY simply plays silence for a voice when a voice's

info runs out. If you know what the piece sounds like, you

should be able to detect slips in the voices. Although, an

idea is in the works which, when implemented, will inform

the user that one voice is shorter than another.

Also: use the comment delimiters to section off pieces of

the music. This allows focusing in on certain parts and

you can then easily determine what's wrong. Never thought

I'd be debugging an opera! **

Note Style

Implementing the Basic  "play"  command  for  setting  note

style,  such as legato, portamento and staccato, would make

coding easier.

** One possible alternative is to play half of the value of the

note and then rest the other half (staccato). I beleive these

styles are possible. Explain what exactly the terms mean, and I'll

have a go at them in upcoming versions **

Check!> Equal voices

In some of the  more  complex  pieces  (such  as  the  trio

sonata),  the  second and third voices get lost beneath the

more predominate first voices. All voices should have equal

weight.

** 'This version of MAKEPOLY doesn't account for this but I have

some ideas of ways to attempt this. Hopefully the next

version will have conquered this problem.' - I said last (1.00)

version.

And it has... see UPDATE.TXT for details on equal voices ** ."

7.3 PROGRAM INFO

------------

MakePoly was written entirely in Microsoft C 5.1.

In its finished form it is rougly 20K of source code.

PlayPoly was written in Microsoft C 5.1, and linked to

and external function written in Microsoft Macro Assembler 5.1.

Both programs use dynamic memory for note storage, so the

only limitation to song length is the amount of available memory.

(Large memory model only, the small memory model has an upward

limit of about 64K).

Updated versions will be produced even if the amount of user

feedback is minimal (in other words, PLEASE FEED BACK! Your comments

are greatly wanted/apprieciated!).

7.4 FUTURE ENHANCEMENTS

-------------------

Other than the enhancements mentioned in the discussion with

Richard Pierce in section 7.2 - PROGRAM FEATURES, I personally

am dedicated to see the following enhancements come to the world

of three voice IBM music:

* Functions to add three voice capability to:

BASIC

PASCAL, and any other requested languages

(Note: the poly() function, in its present form, may

may very well work for Pascal. I have yet to

try it, but I ask anyone who uses Pascal to

feel free to explore! I will attempt this

if and when I get Turbo Pascal 5.5!)

* The ability to work appearantly simultaneously

while other functions are doing their thing (this

is the "DOS-world-possibility" mentioned in section

7.2)

* The "REPEATED MEASURE" ability for MAKEPOLY

* For AUTOHARP type programs, a sensitive assembly key-

board parser which plays a chord until a key is released

(not just repeating the key).

* A simple sequencer-type program that let's you

edit a ".ply" file without knowing how to read music!

(the keyboard would have to have assembly-type instructions

as described for AUTOHARP above so the program could

sense when keys were pressed and released.)

* What would be really great is to be able to tune

the player to concert pitch.

7.5 AUTHOR'S CORNER

---------------

Let me just tell you a little about myself...

I am 19, a Computer Science Major at the University of Texas.

I also work for the Department of Verteran's Affairs, working with

(what else) PC's. I am a programmer, and a serious musician.

I play the guitar primarily, but I also have a great love for

piano and the drums (trapset). I can't deny but that the Lord has been

very instrumental and creative in my life. He has given me peace, joy,

and love I never knew before I knew Him (and believe me, I searched!).

Not only that but He has specifically answered my prayers and spoken

through me. I give every waking moment to Him, and He satisfies me

more than the world ever did!

So if you have any comments, questions, advice, suggestions, or

obscene remarks about what is present in this package, PLEASE

contact me!

Edward ("Eddie") Grigassy

2304A Riverside Farms Rd.

Austin, TX  78741

(512)385-1368

On Compuserve: 73207,3140

Here is the address of Ted Holden, author of VMUSIC (obtainable

on Compuserve {go ibmnew, music section}):

Ted Holden

HT Enterprises

1947 Storm Dr.

Falls Church, Va. 22043

703 760-9713

And here is the address of Richard Pierce, the arranger of the

many Bach music files (playable through MAKEPOLY and the other

Polyphonic Music play routines presented here) included in the latest

VMUSIC package (some were reproduced here):

Richard Pierce

17 Sartelle Street

Pepperell, Ma 01463

Technical Stuff:

----------------

If you enjoyed, used, or even loved this package, please consider a

contribution of $20 or of any size. This package has been designated as

ShareWare by the author (E. Grigassy); it serves as a demo for the

more powerful later versions of Polyphonic Music, the Large

memory models of the poly.lib functions. (see REGISTER.TXT), and

the registration releasing full rights to the registeree

(is that a word?).

Registration Information:

Polyphonic Music is distributed as User-Supported software.  You are free

to copy and distribute this software freely, however, if you find it of

use to you, you are encouraged to register your copy.  Registering your

copy of the software helps the author continue to provide professional

quality software at very reasonable prices.

The Basic Registration is $5.00 and includes royalty-free use of library

functions, unlimited technical support, and information about low-cost

upgrades.

All materials are shipped on 5.25-inch floppy diskettes. If you require

3.5-inch floppies instead, there is an additional $2.00 charge covering

the cost of the diskette itself.

Non-U.S. orders need to include $5.00 extra to cover additional shipping

and handling charges.  Checks and money orders must be drawn on a U.S.

bank.  Please send all payments payable in U.S. Dollars.

Print the registration form, REGISTER.TXT, or include on a piece of

paper your name, address, phone number, and requested items, and send

it along with your payment to:

Edward Grigassy

GrigaSoft Productions

2304A Riverside Farms Rd.

Austin, TX  78741

Microsoft(R), MS(R) are registered trademarks of Microsoft Corporation.

IBM(R) is a registered trademark of the International Business Machines

Corporation.

All versions of VMUSIC (vm0389.exe is latest as of writing) are Copyright

HT Enterprises / Ted Holden

The files: PLAYPOLY.EXE, MAKEPOLY.EXE, LOOPPOLY.EXE, AUTOHARP.EXE,

TIMECHK.EXE, AUTOHARP.C, TIMECHK.C, POLY.H, POLYCHRD.H,

POLY.LIB, POLY_S.OBJ, TRIT_S.OBJ, POLYFUNC.OBJ, and

any of the original source code not mentioned above

are Copyright (c) 1989 Edward P. Grigassy / GrigaSoft

Productions. None of these files or procedures may

be used in any commercial software nor may they be

bought or sold without the explicit written permission

of Edward P. Grigassy / GrigaSoft Productions.

Polyphonic Music (TM) is a trademark of GrigaSoft Productions.

Disclaimer:

-----------

Edward Grigassy / GrigaSoft Productions is not responsible for any

damage or loss of property caused by the software in this package. This

package is provided AS IS and no guarantee is made by the author/owner

for the safety of the user's property.

All the documentaion included in this package is subject to change at any

time and in no way reflects any commitment on the part of the author/owner.