Applied Computing Enterprises

ACE_UTU

November 2, 1987

This readme file contains some basic operational information about the

program ACE_UTU.  This program is designed to communicate with and control

the functions of a Kantronics UTU via your Amiga's serial port.  It may

work with other terminal units functionally equivalent to the UTU, although

I can not guarantee it, as I have not tested the code on other

manufacturer's hardware.  Should you try it successfully, I would really

be interested in finding out your successes and failures.

This version is for DEMO only.  It is fully operational except that

It will not write data out to the terminal unit on command.  The program

is currently written in AMIGABASIC, although it will be translated into

compiled "C" in the near future.  This has the advantage of reduced size

and increased speed as well as more functionality (by having full access

to the ROM Kernel).

Any person who decides to purchase the AMIGABASIC

version will receive the upgraded "C" verion upon completion for the

cost of media and shipping.

Should you care to purchase the program, because you find it worthwhile,

send $39.95 to

Applied Computing Enterprises,

924 Westbury Court,

Sarnia, Ontario

Canada   N7T 7R7

This will bring you the terminal unit program on 3.5" diskette with a

printed manual.

If you have any comments or suggestions for enhancements, drop me a line

and I'll see what I can do.  This program was tailored for the features

that I felt that I could use.  You may some additional ones.

ACE_UTU Program

This program is designed to compliment the features of the Kantronics

UTU.  Its control codes are thus designed for this unit.  You should

find the menu options self explanatory for the most part if you are

familiar with the terminal unit's operation.

The following is a brief functional description of the meaning of each

menu item.  You can try them from there to determine their operation and

use.  The full manual describes each in much greater detail.

When the program starts up it will create a three window display.  The top

window displays the received data from the terminal unit.  The lower display

will be used to display/load buffers for transmitting.  The display in the

upper right is simply a time/date window for those of you who do not have

enough PD clocks already.

The first operation of the program is to set the TU characteristics

correctly.  The UTU comes up with the echo disabled, which we would like

enabled.

Menu Items:

1) Options -- UTU Menu

This will activate the terminal unit menu mode and allow you

to interact with it to set modes like RTTY, CW etc.

-- Load/Display Xmit Buffers

There are 5 built in transmit buffers that you can load and

save for future use (ie CQ, QRZ etc).  These options allow you

to load and display the buffer contents.

-- Load autoid buffer

This will allow you to load the call of the station you are

working.  With this, the program will construct your beginning

and ending of transmission text with your ID, time and leading

RYRY if you want it (usually only RTTY).

-- Save/Recall Xmit Buffers

Will load and save the 5 brag buffers to/from disk for future

use, or to overwrite what you currently have in the buffers.

-- Set program defaults

Allow you to set basic program options such as printer on/off

status, auto load of brag buffers on startup, autoid etc.

These characteristics will be created every time you startup

the program

2) Mode    -- Transmit Volatile

Allows you to create a typeahead buffer of information while

reading incoming data.  You initiate transmission with F1 key.

You can continue to load the buffer as the computer unloads it

to the TU.  When it catches up to your typing, it will

transmit as you type in real time.

-- Transmit Buffer 1-5

Transmit one of the preloaded brag (transmit) buffers.

-- Receive

Force TU back to receive mode.

-- Transmit Real Time

Transmit data to TU as you type it without any typeahead

buffering.

3) Speed

This option allows you to select the speed setting of the TU

with simple mouse selection.

4) Printer

Selects printer status.  With printer on, all data that is

received will be echoed to the printer for hard copy.

5) ID      -- ID on/off

Will enable or disable the autoid which is "his call" de

"your call".

-- RY/Time enabled/Disabled

Will send series of RYRY at beginning of transmission.  Will

also prefix and suffix your transmission with a time of day

string.

6) Exit Program

Exit the program and cleanup the windows.

Have fun, and feel free to send me your comments.  Thanks to Kathy for

including this demo on her PD disk.

Sincerely

Allan Ebert

VE3AXV