TABLE OF CONTENTS

ateobus.library/--background--

ateobus.library/AC_GetSpaceBase

ateobus.library/AC_AddIntServer

ateobus.library/AC_RemIntServer

ateobus.library/AC_IrqLine2Level

ateobus.library/AC_IrqLevel2Line

ateobus.libraru/AC_DisableIrq

ateobus.library/AC_EnableIrq

ateobus.library/AC_QueryValue

ateobus.library/AC_StoreValue

ateobus.library/AC_GetVersion

ateobus.library/AC_GetGlobalPrefs

ateobus.library/--background--                 ateobus.library/--background--

Version 3.0 / April 2, 1999

The main use is this library is to control the At�oBus trough the

At�oBusServer (started by the StartAteoBus command)

The At�oBusserver has several functions:

- Storage of values

- Serving interrupt request comming from the At�oBus

- Giving base adresse of the differents bus space

In the future, this library and it associe server will also support

ISA Plug&Play card on the bus

mail bug report to: arlequin@ateo-concepts.com

ateobus.library/AC_GetSpaceBase               ateobus.library/AC_GetSpaceBase

NAME

AC_GetSpaceBase - get the adress base of the specified space

SYNOPSIS

base = AC_GetSpaceBase( flags )

D0                   D0

void* AC_GetSpaceBase( ULONG )

FUNCTION

This routine will return the adress of the specified space

The return value will be the 'Amiga' adress see as 0x0 from the At�oBus

side for the specified space

Please use it instead of using define adress. Adress space can change and

this is the only safe methods

INPUTS

The flags are combinaison of:

ATEOBUS_SPACE (bit 0)

ISA_SPACE     (bit 1)

IO_SPACE      (bit 4)

MEMORY_SPACE  (bit 5)

For example is you want to know the ISA IO base adress, you have to call

this routine with ISA_SPACE(bit1) and IO_SPACE(bit4) set

RESULTS

The routine will return the requested adress or NULL if this space is not

supported

NOTE

SEE ALSO

ateobus.library/AC_AddIntServer               ateobus.library/AC_AddIntServer

NAME

AC_AddIntServer -- add an interrupt server the At�oBus server chain

SYNOPSIS

ok = AC_AddIntServer(intNum, interrupt)

D0                D0-0:4  A1

ULONG AC_AddIntServer(ULONG, struct Interrupt *);

FUNCTION

This function adds a new interrupt server to a given server chain.

The node is located on the chain in a priority dependent position.

If this is the first server on a particular chain, interrupts will

be enabled for that chain.

Each link in the chain will be called in priority order until the

chain ends or one of the servers returns with the 68000's Z condition

code clear (indicating non-zero).  Servers on the chain should return

with the Z flag clear if the interrupt was specifically for that

server, and no one else. (Take care with High Level Language servers,

the language may not have a mechanism for reliably setting the Z flag

on exit).

Servers are called with the following register conventions:

D0 -> D7 - scratch

A0 - scratch

A1 - server is_Data pointer (scratch)

A2 -> A4 - scratch

A5 - jump vector register (scratch)

all other registers must be preserved

INPUTS

intNum - the At�oBus interrupt line number (0 through 10) given by

AC_IrqLine2Level() .

interrupt - pointer to an Interrupt structure.

By convention, the LN_NAME of the interrupt structure must

point a descriptive string so that other users may

identify who currently has control of the interrupt.

The LN_TYPE must be set to NT_INTERRUPT. If not this function

will fail

RESULTS

NULL is the routine fails, no NULL overwise

WARNING

Some compilers or assemblers may optimize code in unexpected ways,

affecting the conditions codes returned from the function.  Watch

out for a "MOVEM" instruction (which does not affect the condition

codes) turning into "MOVE" (which does).

BUGS

Version 1.1 of the ateobus.library does not organize the chain in

priority order and has only two place for each irq line.

SEE ALSO

AC_RemIntServer()

ateobus.library/AC_RemIntServer               ateobus.library/AC_RemIntServer

NAME

AC_RemIntServer -- remove an interrupt server from a server chain

SYNOPSIS

ok = AC_RemIntServer(intNum, interrupt)

D0                   D0      A1

ULONG AC_RemIntServer(ULONG,struct Interrupt *)

FUNCTION

This function removes an interrupt server node from the given

server chain.

INPUTS

intNum - the At�oBus interrupt line (0..10) given by AC_IrqLine2Level()

interrupt - pointer to an interrupt server node

NOTE

This routine doesn't take care of the interrupt structure. This structure

is just a way for passign the int server pointer. (You can use a different

interrupt structure for adding and removing a serveur as long as the is_Code

and is_Data field are the same).

SEE ALSO

AC_AddIntServer(), AC_IrqLevel2Line(), AC_IrqLine2Level()

ateobus.library/AC_IrqLine2Level             ateobus.library/AC_IrqLine2Level

NAME

AC_IrqLine2Level -- Convert the ISA Irq line number to the At�oBus Irq Level

(V3)

SYNOPSIS

IrqLevel = AC_IrqLine2Level( IrqLine )

D0                           D1

UBYTE AC_IrqLine2Level( UBYTE IrqLine)

FUNCTION

This function convert the IrqLine of the ISA bus (3->15) to the IrqLevel

you must give to AC_AddIntServer() or to AC_RemIntServer()

INPUTS

IrqLine - the ISA like Irq number

RESULTS

The IrqLevel you must give to AC_AddIntServer() or to AC_RemIntServer()

SEE ALSO

AC_AddIntServer(), AC_RemIntServer(), AC_IrqLevel2Line()

ateobus.library/AC_IrqLevel2Line             ateobus.library/AC_IrqLevel2Line

NAME

AC_IrqLevel2Line -- Convert the At�oBus Irq level to an ISA Irq line number

(V3)

SYNOPSIS

IrqLine = AC_IrqLevel2Line( IrqLevel )

D0                          D1

UBYTE AC_IrqLevel2Line( UBYTE IrqLevel )

FUNCTION

This function convert the At�oBus Irq Levelto the ISA Irq line number.

INPUTS

IrqLevel - the At�oBus Irq Level

RESULTS

The ISA Irq line number

SEE ALSO

AC_AddIntServer(), AC_RemIntServer(), AC_IrqLine2Level()

ateobus.libraru/AC_DisableIrq                    ateobus.libraru/AC_DisableIrq

NAME

AC_DisableIrq -- Control the At�obus interrupts (V3)

SYNOPSIS

ok = AC_DisableIrq( Irq, Flags )

D0                  D0   D1

UBYTE AC_Disable( BYTE Irq, ULONG Flags )

FUNCTION

This function control the At�obus interrupts

if Irq is equal to -1, this function will prevent the At�obus to interrupt

the Amiga. When Irq = -1, the Flags are ignored.

If the bit AC_IRQLEVELB is set in Flags, the Irq will refer to the At�obus

level, is this bit is reset, the Irq will refer to the ISA Line number.

Callign this function with AC_IRQLEVELB reset and Irq = 5, will disable the

interrupt server for the ISA interrupt line number 5

INPUTS

Irq - the Irq line or level, according to Flags

Flags - bit AC_IRQLEVELB, if set, the function works with interrupt level,

if reset, the function works with line.

RESULTS

ok - Error code, if =0 the Level/Line is not supported by the At�oBus.

WARNING

Be careful with this function! If you disable a specific line/level where an

interrup can occur, you will freeze the Amiga!

In ateobus.library V3.0, the Enable/Disable function by Line/Level doesn't works!

ateobus.library/AC_EnableIrq                      ateobus.library/AC_EnableIrq

NAME

AC_EnableIrq -- Control the At�oBus interrupts (V3)

SYNOPSIS

ok = AC_EnableIrq( Irq, Flags )

D0                 D0   D1

UBYTE AC_EnableIrq( BYTE Irq, ULONG Flags )

FUNCTION

This function control the At�oBus interrupts

if Irq is equal to -1, this function will enable the At�obus to interrupt

the Amiga. When Irq = -1, the Flags are ignored.

If the bit AC_IRQLEVELB is set in Flags, the Irq will refer to the At�obus

level, is this bit is reset, the Irq will refer to the ISA Line number.

Callign this function with AC_IRQLEVELB reset and Irq = 5, will enable the

interrupt server for the ISA interrupt line number 5

INPUTS

Irq - the Irq line or level, according to Flags

Flags - bit AC_IRQLEVELB, if set, the function works with interrupt level,

if reset, the function works with line.

RESULTS

ok - Error code, if =0 the Level/Line is not supported by the At�oBus.

WARNING

In ateobus.library V3.0, the Enable/Disable function by Line/Level doesn't works!

NOTE

If you enable a line where not interrupt will occur you can just loose time!

ateobus.library/AC_QueryValue                   ateobus.library/AC_QueryValue

NAME

AC_QueryValue -- Query the server about a specified

SYNOPSIS

Value = AC_QueryValue( Number )

D0                     D1

ULONG AC_QueryValue(ULONG Number)

FUNCTION

This function query the server about a given memory case.

INPUTS

The memory case number.

RESULTS

The content of the memory case.

SEE ALSO

AC_StoreValue()

ateobus.library/AC_StoreValue                   ateobus.library/AC_StoreValue

NAME

AC_StoreValue -- Store the Value in Query the server about a specified

SYNOPSIS

ok = AC_StoreValue( Value, Number )

D0                  D0     D1

ULONG AC_StoreValue(ULONG Value, ULONG Number )

FUNCTION

This function store the Value on the Number memory case.

INPUTS

The memory case number and the Value to Store.

RESULTS

Succes (NULL) or Error code (Positive value).

SEE ALSO

AC_StoreValue()

ateobus.library/AC_GetVersion                   ateobus.library/AC_GetVersion

NAME

AC_GetVersion -- get the version of the At�oBus server

SYNOPSIS

version = AC_GetVersion()

D0

ULONG AC_GetVersion( void )

FUNCTION

This function will return the version of the At�oBus server

INPUTS

None

RESULTS

Version of the At�oBus server (100 x version + revision)

SEE ALSO

ateobus.library/AC_GetGlobalPrefs           ateobus.library/AC_GetGlobalPrefs

NAME

AC_GetGlobalPrefs -- get a copy of the AteoBusPrefs structure

SYNOPSIS

Pref = AC_GetGlobalPrefs( )

D0

struct AteoBusPrefs *AC_GetGlobalPrefs( void )

FUNCTION

This function will return you a ptr to the AteobusPrefs structure. Be

careful, this strcuture is read only!!!!

INPUTS

None

RESULTS

A ptr to the AteoBusPrefs structure

WARNING

The AteoBusPrefs structure is read only!

WARNING 2

This fucntion is not yet implemented!

SEE ALSO