CHIPSET.EXE is a small program that simply checks to see if your PC

has one of the memory controllers required to use The Last Byte

memory manager.

To see if your PC has one of the supported memory controllers,

simply enter the following command at the DOS prompt:

A>CHIPSET

To see the entire list of what devices are supported, add a question

mark to the command line, as in:

A>CHIPSET ?

If your PC has one of these devices, then The Last Byte (tm) memory

manager will work on your PC, and you will be able to load device

drivers, TSRs, DOS buffers and more into upper memory above 640k.

If your PC uses another kind of chipset, it may not be supported for

one of the following reasons:

(1) No shadow ram hardware:

Examples: VLSI Technologies 82C100 series

Western Digital chipsets

(2) No shadow ram read/write mode; read-only and write-only modes only.

Examples: ACC Microelectronics 2036 and 2121, and all

chipsets from Headland Technologies (previously

known as G2)

(3) Only the CPU (and not DMA) can access the shadow ram.  (Several

device drivers and TSRs use disk buffers that DOS accesses via

DMA.)

Examples: VLSI Technologies VL82C286 & VL82C386 TOPCAT

(4) Shadow ram control not based on one enable bit per segment.

Examples: Zymos chipsets

However, some of these chips have hardware support for LIM 4.0 EMS,

and thus many of their motherboards come with a EMS device driver

designed specifically to take advantage of this capability.  So in

many cases, you can install this driver first, and then The Last Byte

can use it to manage upper memory.

A free demo version of The Last Byte may be downloaded using the

filename TLB-Vnnn.ZIP, where the "nnn" is replaced by the latest

version number.  The demo version is fully functional except that it

limits you to 32k of memory and no more than two device drivers

and/or TSRs loaded into upper memory.  If you can't find a copy of

The Last Byte, contact us and we'll be happy to send you a copy.

Key Software Products

440 Ninth Avenue

Menlo Park, CA 94025

(415) 364-9847