Z.DOC:  What we know so far about the DSZ transfer engine

____________________________________________________________________________

PROGRAM:      DSZ.COM   External Protocol Driver, Borland Turbo-C Version

PROGRAM:      DSZ.EXE   External Protocol Driver, Microsoft-C LIB Version

PROGRAM:      GSZ.EXE   External Protocol Driver, graphic display Version

----------------------------------------------------------------------------

PURPOSE:         High-performance File Transfer Engines

----------------------------------------------------------------------------

DESCRIPTION:         DSZ is a State-of-the-Art File Transfer Engine

that functions as a  DOS Console Terminal  or,

as an  External Transfer Protocol Driver  for

sending and receiving Files.  GSZ is functionally

equivalent to DSZ.EXE with the addition of graphics.

----------------------------------------------------------------------------

FEATURES:     T   Zmodem Batch-T  DOS Telecommunications Console

A   Zmodem Batch-Z  with RLE compression for  ASCII

W   Zmodem Batch-W  with sliding window for TELENET

R   Zmodem Batch-R  with automatic  Crash  Recovery

M   Zmodem Batch with MobyTurbo enhanced throughput

Z   Zmodem Batch with 32-bit CRC  error correction

B   Ymodem Batch with 16-bit CRC  error correction

Y   Xmodem-1k /CRC /CheckSum /-G  error correction

X   Xmodem    /CRC /CheckSum /-O  error correction

G   Ymodem Batch-G  for hardware  error correction

----------------------------------------------------------------------------

PERFORMANCE:         Observed Throughput  ( avg ZIP File Size = 100 Kb )

CCITT V.22bis 2400 Bps STD Modem

233 +/- 5 c.p.s.  ZMODEM-m transfer ZIP Files,  CONNECT 2400

230 +/- 5 c.p.s.  ZMODEM-Z transfer ZIP Files,  CONNECT 2400

213 +/- 5 c.p.s.  ZMODEM-Z transfer ZIP Files through Telenet 2400

108 +/- 5 c.p.s.  ZMODEM-Z transfer ZIP Files through Telenet 1200

CCITT V.22bis 2400 Bps "MNP" OR V.42/V.42 bis Modem

280 +/- 5 c.p.s.  YMODEM-G transfer ZIP Files,  CONNECT 2400 MNP

275 +/- 5 c.p.s.  ZMODEM-m transfer ZIP Files,  CONNECT 2400 MNP

223 +/- 5 c.p.s.  YMODEM-G transfer ZIP Files through Telenet 2400 MNP

113 +/- 5 c.p.s.  YMODEM-G transfer ZIP Files through Telenet 1200 MNP

throughput values will be about 2% less for V.42/V.42 bis transfers

CCITT V.32/HST 9600 Bps "MNP" OR V.42/V.42 bis Modem

1140 +/- 15 c.p.s.  YMODEM-G transfer ZIP Files,  CONNECT 9600 MNP

1130 +/- 15 c.p.s.  ZMODEM-m transfer ZIP Files,  CONNECT 9600 MNP

1050 +/- 15 c.p.s.  ZMODEM-Z transfer ZIP Files,  CONNECT 9600 MNP

throughput values will average 2% less for V.42/V.42 bis transfers

CCITT V.32 bis/HST 14400 Bps "MNP" OR V.42/V.42 bis Modem

1728 +/- 20 cps  YMODEM-G transfer ZIP Files,  CONNECT 14400 MNP

1690 +/- 20 cps  ZMODEM-m transfer ZIP Files,  CONNECT 14400 MNP

1650 +/- 20 cps  ZMODEM-Z transfer ZIP Files,  CONNECT 14400 MNP

throughput values will average 2% less for V.42/V.42 bis transfers

______________________________________________________________________________

CHARACTERISTICS

DSZ.COM :         Compiled with Borland Turbo-C run-time Library.

the COM Driver uses less memory than EXE Driver

the COM Driver executes when both are on d:\path

DSZ.EXE :         Cross-compiled using the Microsoft "C" Library.

the EXE Driver does not support  MS-DOS "SHARE"

the EXE Driver uses table-driven look up of CRCs

the EXE Driver uses different run-time routines

use the EXE driver @9600 and up with slow CPU's

_______________________________________________________________________________

AUTHOR:         Charles A. "Chuck" Forsberg

Omen Technology, Inc.

ADDRESS:         17505-V Northwest Sauvie Island Road

Portland, Oregon  USA  97231

REGISTRATION:         Enables advanced features of DSZ, and provides

the author with funding for new development.

For VISA/MC Registrations, call (503) 621-3406

($5.00 fee charged for credit card registration)

US & CANADA:         Mail $20.00 to  Omen Technology Inc

P.O. Box 4681

Portland, Oregon USA 97208-4681

Price includes 5-1/4" disk. Add $5.00 for 3-1/2" disk.

Also add $5.00 for shipping & handling unless

your full payment accompanies your order.

OVERSEAS:         Mail $25.00 to  Omen Technology Inc

P.O. Box 4681

Portland, Oregon USA 97208-4681

Price includes 5-1/4" disk. Add $5.00 for 3-1/2" disk.

Also add $5.00 for shipping & handling unless

your full payment accompanies your order. (U.S. funds)

______________________________________________________________________________

DSZ  COMMAND  LINE  SYNTAX

DSZ [parameters] [numeric parameters] command [-options] [File]

SYNTAX:         The DSZ command line is lengthy, cAsE SeNsItIvE

and position dependent. Although the parameters

and options provide advanced features and enable operation with

a variety of modems and transmission mediums, all configuration

of the driver is done from the DOS command line or environment.

To realize the full potential of DSZ, one should make Batch Files

to act as receivers and transmitters.  Using numeric parameters

and command -options, receivers and transmitters are fine tuned.

NOTE:           Some features are not enabled in unregistered

copies of DSZ.

USAGE:

DSZ [port n] [speed s] [ha x] [est 0 b] [d/D] [pB8192] command [-options]

ENVIRONMENT  VARIABLES  (** = registration required)

** SET DIRRX=d:\path                   specifies a directory to receive to

** SET DIRRX=d:\path\                  specifies absolute receive directory

SET DSZFILES=File1 File2 File3      specifies Filename(s) to send batch

** SET DSZLOG=d:\path\DSZ.LOG          specifies log File name and location

** SET DSZOPT=ty8g                     specifies global command line options

SET DSZPORT=2                       specifies serial port for operations

SET MAXBYTES=1024000                specifies maximum bytes per transfer

SET ZONE=420                        specifies minutes behind GMT for PST

-----------------------------------------------------------------------------

COMMAND  LINE  PARAMETERS

CON             specifies redirection to the DOS console  CON:

use this parameter when operating in CTTY mode

alternatively, use  >CON  >PRN or >NUL  at the

end of a command line to redirect the display.

use  >filespec  to  "capture"  console data to

a disk File - press ALT-X (after RXD modem LED

stops glowing) to end capturing data.

the default device is CON:, unless redirected

NOTE:          The CON parameter must be the first parameter

on the command line except when used with

PcAnywhere.

port n          specifies the COMn: serial communications port

n = 1,2,3,4  for IBM PC/XT and PC/AT computers

n = 10,11,12,13  for IBM PS/2 computer systems

alternatively,  SET DSZPORT=n  in AUTOEXEC.BAT

the default communications port n = 1 for COM1

speed s         opens the COMn: serial port with Bps Rate = s

s =  300, 1200, 2400, 4800, 9600, 19200, 38400,

57600, 115200

or, use  MODE COMn:,s,N,8,1,-  in AUTOEXEC.BAT

the default Bps Rate is initialized by MS-DOS

ha x            enables hardware and/or software handshaking x

ha bo   enables XON/XOFF and RTS/CTS handshake

ha on   specifies RTS/CTS hardware handshaking

ha slow specifies RTS/CTS, drops RTS when writing

to the disk.

ha cts  specifies CTS (5) hardware handshaking

ha dsr  specifies DSR (6) hardware handshaking

ha dcd  specifies DCD (8) hardware handshaking

the default handshaking is XON/XOFF for Zmodem

and Ymodem Batch or, no handshaking for Xmodem

Zmodem MobyTurbo requires  ha on   or  ha cts

est 0 b         estimates Batch Transfer times for Bps Rate b

Use this parameter to achieve better estimates

of Batch Transfer times when the port is setup

for a fixed speed and the modem Bps is lower.

syntax:         est filesize Baudrate

the default estimate is based upon the speed s

a filesize of 0 causes DSZ to use the actual DOS

file size when calculating the transfer time.

NOTE:          The estimate parameter must follow port, speed,

and handshake parameters, when used.

d               disables checking DCD (8) for a CARRIER - this

is useful for operating in terminal mode before a

CARRIER is present, and for direct connections.

the default check DCD (8) for a CARRIER is ON.

D               enables  checking DCD (8) for a CARRIER and it

will drop DTR (20) if CARRIER is not detected:

useful for a BBS or other HOST installation.

This is DSZ's default mode when not specified

otherwise.

A               Use with DSZ.EXE and PcAnywhere to suspend

PcAnywhere operations during file transfers.

The [A] parameter must appear first on the

command line.  Start all DSZ transfers on the

PcAnywhere Host first.

F               Use with 16550 UARTS to leave the FIFO buffer

enabled after DSZ exits. The [F] parameter

must preceed the port statement on the command

line.

restrict        Since DSZ has the ability to transfer a full

path specification, bbs sysops will want to

place this command in their DSZ Zmodem/Ymodem

receivers to lock out intentionally misdirected

files. The [restrict] command restricts incoming

pathnames to the current D:\path and prohibits

the overwriting of any existing files. It also

causes incomplete files to be deleted and blocks

transfers having file names of AUTOEXEC.BAT and

COMMAND.COM.  The restrict command should be

placed immediately preceeding the rz statement

on the command line.

o               Used in "tty" terminal mode to disconnect by

dropping DTR before exiting to DOS with the

F1 function key.  Requires that your modem be

capable of responding to toggling of the DTR pin.

ie:  DSZ t o

pB8192          sets aside 8k of RAM as a Disk Buffer for DSZ

Legal values are 1024 through 16384 in DSZ.EXE

Use 1024 through 4096 with DSZ.COM.

Use this parameter to minimize Disk Read/Write

activity for operations at or below 2400 Bps.

the default Disk Buffer is 1024 bytes for DSZ

Do not use the pB buffer at speeds above 2400 bps

except on fast drives and ramdisks.

NOTE:          Certain device drivers, memory managers, caches and

TSR programs can interfere with DSZ operations

and may have to be removed if interrupt conflicts

surface.

----------------------------------------------------------------------------

COMMAND  LINE  NUMERIC  PARAMETERS

The Driver is rich with timing adjustments and

alternative configuration options.

p               Use  DSZ d p t  to display numeric parameters

z p             Use  DSZ d z p t  to display  "zmodem"  params

p z p           Use  DSZ d p z p t  to display all DSZ numeric

parameters and their default values.

NOTE:          Numeric parameters are specified by p_ values,

and "zmodem" numeric parameters by  z p_ .

DSZ supports many protocols, "zmodem" being

just one of them.

Numeric parameters are generally specified at

the end of a parameter list, before a command.

To get an accurate report of parameters on the

command line, use  p  or  z p  last, as in:

EXAMPLE:         DSZ D pB8192 p z pL1024 pw4096 p sz -k *.*

which displays an accurate report of the  pB_

numeric parameter and the pL_ and pw_ "zmodem"

numeric parameters.

p_      NUMERIC PARAMETERS

pB8192          creates an 8 Kilobyte Disk Buffer for DSZ.

Use this parameter to minimize Disk Read/Write

activity for operations at or below 2400 Bps.

the default Disk Buffer is 1 Kilobyte for DSZ

use values between 1024 and 16384 (1-16k).

NOTES:         The pB8192 parameter is applicable to both the

.COM and .EXE Driver in circa 1990 and later

versions. Previously it was not applicable to

the DSZ.EXE driver or recommended above 2400 bps.

The pB8192 parameter may be experimented with

at >2400 bps speeds when used in conjunction

with fast hard disks or a non-conflicting ramdisk.

Maximum buffer size in DSZ.COM is 4096 bytes.

pO0             controls Xmodem "OverThruster" File transfers

EXAMPLES OF OVERTHRUSTER CONFIGURATION

pO-512  CompuServe XMODEM-Ovth  DSZ pO-512 ro Filename.ext

Negative numbers up to a system dependent value less

than the protocol block size increase throughput.

Note that the number is negative (-)

Tune the  pO_  parameter for a specific Host computer.

Try  pO-512  for  CompuServe  as the standard setting.

Retry 0:  Received dup Sector - reduce parameter pO_ .

pO60    XMODEM/-1k/-Ovth        DSZ pO60 rx -o Filename.ext

Positive numbers up to a system dependent value less

than the protocol block size increase throughput.

Tune the  pO_  parameter for a specific Host computer.

Try  pO60  for 2400 Bps MNP as the standard setting.

Retry 0:  Received dup Sector - reduce parameter pO_ .

pO60    YMODEM/-1k/-Ovth        DSZ pO60 rb -o

Parameter pO_ is useful for tuning a CRC-16 receiver to

one  particular  Host  and  transmission  medium.

NOTES:

The idea behind -Ovth is for the receiver to transmit an

ACK for Block a few ticks before all 1,024 bytes come in.

Fiddling with the pO_ parameter, positive/negative values

show different effects with different modems and mediums.

"OverThruster" is an  "advance ACK"  configured receiver.

Varying the  pO_ parameter  eeks out the last few  c.p.s.

possible for CRC-16 based, Xmodem/-1k and Ymodem-K Batch.

This is when you get down to configuring a receiver for a

particular Host... like, she's got to have it.

Because  -o closes the "artificial window" between sender

and receiver, errors encountered during transfers will

cause the transfer to catastrophically abort.

Whenever hardware error correction is available, the -g

"Gonzomode" is the fastest way to receive compressed files.

OverThruster is only available for Xmodem transfers.

pS15            timeout in seconds to wait after receiving the

XOFF (CTRL-S) character or, after CTS went low.

the default wait for handshake is  15  seconds.

NOTE:          Use CTRL-Break to unlock Receiver Console after

receiving an XOFF (CTRL-S).

pG_             Controls the number of bytes loaded into the

16-byte FIFO buffer on 16550 UARTs. DSZ fills

all 16 bytes by default.  Reducing this value

may help some cheap MNP modems better handle flow

control.  Setting this parameter pG0 effectively

disables the UART buffer entirely.

NOTE:          Once data has been handed off to the UART it is

sent on to the modem regardless of the status of

the CTS pin.  Some poorly designed (read cheap)

modems cannot accept a few additional bytes after

they exert FLOW control.

pQ_             Forces a user specified error code to be returned

to DOS upon completion of a file transfer. Normally

a successful transfer returns an error code of 0.

ie:  DSZ pQ-1 sz myfile.zip forces an error code

of 1 at the end of the transfer.

pV_             When set to a value of 1, Forces DSZ.COM and GSZ

to use direct video when writing to the screen

instead of BIOS calls. Speeds up displays. Typical

syntax:  GSZ pV1 rz  Default value is 0, and is not

supported in DSZ.EXE

pW_             Wait for <enter> key to be hit before exiting after

an error (registered copies only). DSZ pW1 sz myfile.

Default value is 0 (exit immediately).

pY_             Control byte for the FCR register in 16550 UARTS.

Setting pY129 may improve DESQview operation when

multiple DSZ sessions are active.  It is suggested

that a 16550 UART applications manual be consulted

before tinkering with the FCR register.

pa2000          CPU calibration for Xmodem and Ymodem  8088-05

pa4000          CPU calibration for Xmodem and Ymodem  8086-08

pa6000          CPU calibration for Xmodem and Ymodem 80286-08/1

pa8000          CPU calibration for Xmodem and Ymodem 80286-10/1

pa10000         CPU calibration for Xmodem and Ymodem 80386-16/1

pa12000         CPU calibration for Xmodem and Ymodem 80386-20/1

NOTES:         the default pa_ timing is 2000 for a PC/XT i.d.

the default pa_ timing is 6000 for a PC/AT i.d.

NOTE WELL: -->  it is unlikely that tinkering with these default

timings will provide any additional throughput.

pd1             enables sending  File Date Header information,

the default Batch transfer timestamp method.

pd0             disables sending File Date Header information,

and uses current (receiver) date for new Files.

DSZ pd0 sz myfile.zip may provide a marginal

increase in observed throughput of small files.

pv0             disables DSZ "trace" mode, the default setting

pv1             rings console BEL at the end of File transfers

pv2             enables DSZ "trace" mode, displays Sector info

pz0             time zone in minutes behind GMT world standard

pz300           time zone in minutes behind GMT for  EST  Zone

pz360           time zone in minutes behind GMT for  CST  Zone

pz420           time zone in minutes behind GMT for  PST  Zone

alternatively, SET ZONE=nnn  in  AUTOEXEC.BAT

where nnn = minutes behind GMT

z p_      ZMODEM NUMERIC PARAMETERS

z pb0             disable sending of BREAK upon transfer error

z pb1             enable sending of BREAK upon transfer error,

useful for clearing some high-speed modems.

z pB2             duration in tenths of a second of the BREAK

signal - default is 200 milliseconds.

z pC11            number of repetitions of a ZCOMMAND request

Unclear if supported in current versions of DSZ.

see ZCOMM or PRO-YAM docs for more information.

z pe0             disables escape of ASCII control characters,

the default setting.

z pe1             enables  escape of ASCII control characters,

useful on certain networks.

z pl0             disables wait for ACK for each Zmodem sector,

the default setting (free flowing).

z pl4096          enables  wait for ACK after every 4096 Bytes,

transforms Zmodem to -4K Block, CRC-32 Batch.

z pL0             enables auto-config of  Zmodem  Sub-Packets (default)

z pL128           configures Zmodem for  128 Byte Sub-Packets

z pL256           configures Zmodem for  256 Byte Sub-Packets

z pL384           configures Zmodem for  384 Byte Sub-Packets

z pL512           configures Zmodem for  512 Byte Sub-Packets

z pL1024          configures Zmodem for 1024 Byte Sub-Packets

NOTES:         Larger Zmodem Sub-Packet sizes give better

performance on clear transmission lines.

Smaller Zmodem Sub-Packet sizes give better

performance on noisy transmission lines.

it is convenient to set sub-packet sizes to

one-fourth the effective Zmodem "window", as:

DSZ z pL256 pw1024 sz [Filespecs]

DSZ z pL384 pw1536 sz [Filespecs]

DSZ z pL512 pw2048 sz [Filespecs]

DSZ z pL1024 pw4096 sz [Filespecs]

z pn0             defines the number of ASCII NULL characters

sent before each  ZDATA  frame - useful for

FastLink modems, whatever those are.

z pp50            timeout in tenths of a second for a receiver

to wait for gaps within a packet. Default = 5 sec

z pP_             a single lower case "p" following the speed

parameter will cause DSZ to report the type of

serial port UART detected.

0 = 8250/16450     1 = 16550     2 = 82510

Forcing a value of "1" with z pP1 and placing this

ahead of the speed parameter will bypass DSZ's

auto UART detect mechanism and may in certain

instances permit use of "brain damaged UARTs"

z pr0             disables removal of incomplete Files received,

the default setting.

z pr1             enables  removal of incomplete Files received,

useful for Host systems:  (DSZ z pr1 rz )

z ps15            timeout in seconds to wait after receiving the

XOFF (CTRL-S) character or, after CTS went low.

the default wait for handshake is  15  seconds.

NOTE:          Use CTRL-Break to unlock Receiver Console after

receiving an XOFF (CTRL-S).

z pt100           timeout in tenths of a second for a receiver

to wait for a packet or acknowledgement from

the sender.  The default is  10  seconds.

z pT600           timeout in tenths of a second for the sender

to wait for a packet or acknowledgement from

the receiver.  The default is  60  seconds.

z pw0             disable "window"  flowing  ACK from receiver,

the default setting (free-flowing).

z pw2048          enable  "window"  2 Kb for Telenet 1200 Bps,

requesting the receiver to send an ACK every

512 Bytes received.

z pw4096          enable  "window"  4 Kb for Telenet 2400 Bps,

requesting the receiver to send an ACK every

1024 Bytes received.

NOTES:         Larger Zmodem "windows" increase performance

but may cause excessive XOFF delays or loss

of data inside the network - to minimize the

frequency of XOFFs on Telenet, use:

SET? 0:33,35:10,41:1,64:4 <cr>

at the outdial PAD before transmitting Files.

the   z pw_   parameter is applicable to the

Zmodem sender, as in:  DSZ z pw4096 sz Files.

the  -wN option over-rides "z pw_" parameter.

z pW1400          defines the number of characters the  Zmodem

receiver will eat when looking for the start

of a new frame.

NOTE:          Increase  z pW_  to tune error recovery  for

networks with a large "window" of in-transit

packets.   "Garbage Count Exceeded"   errors

indicate that z pW_ is too low.

z pZ0             the number of minutes to adjust a File date

when using  TELINK  or  CLINK  protocols to

transfer a File between time zones.

____________________________________________________________________________

TRANSFER  PROTOCOL  CONFIGURATIONS

COMMANDS        TRANSMITTERS      (** = registration required)

sz File(s)      Send Zmodem Batch [filespecs] to a Receiver.

or

sz @D:FILES.LST Send Zmodem Batch [filespecs] specified with

the ASCII File "FILES.LST" located on d:\path

to a Receiver.  Valid delimiters for Filespecs

within "FILES.LST" are a SPACE, TAB, or CR/LF.

Mind the use of the "@" character.

Alternatively, Files can be specified by using

SET DSZFILES=d:\path\FILE1.ZIP FILE2.ZIP (etc)

in the AUTOEXEC.BAT File.

**     sz -S File(s)   Send Zmodem Batch [filespecs] to a Receiver and

interrogate the RECEIVING program for its serial

number.  The serial number received will then

be saved in the performance log, DSZ.LOG.

sz -m File(s)   Send Zmodem batch [filespecs] to a Receiver,

using MobyTurbo throughput enhancement. Helps

achieve nearly the same throughput as Ymodem-G

with the added benefit of CRC-32 error checking.

RTS/CTS handshaking required (ha on).

**     sz -Z File(s)   Send Zmodem Batch [filespecs] to a Receiver,

using "RLE" compression. This reduces the time

required to transmit ASCII Files; there is no

benefit when sending ZIP type Files. Cannot be

used in conjunction with MobyTurbo mode (-m).

sz -w File(s)   Send Zmodem Batch [filespecs] to a Receiver,

using a  "sliding window" - forcing a Receiver

to transmit an  acknowledgement  to the Sender

every window /4 bytes. This feature is useful

for sending Files over Telenet.

sz -k File(s)   Send Zmodem Batch [filespecs] to a Receiver,

using 1,024 byte packets instead of auto-shift.

May increase average throughput by a few cps on

small files. No effect on files >100 kb.

**     sb -k File(s)   Send Ymodem-1K or Ymodem-1K-G the Filespecs to

a Receiver.  The Receiver determines CRC or -G

error correction method. The Sender determines

128 byte or 1,024 byte (-1K) packet size.

sx -k File      Send Xmodem-1k or Xmodem-1k-G [filespecs] to

a Receiver.  The Receiver determines CRC, Sum,

or -G error correction method. The Sender sets

the Xmodem packet size.  Xmodem-1K-C  is often

incorrectly called  Ymodem/CRC .

sx File         Send Xmodem/CRC or Xmodem/Sum [filespecs] to

a Receiver.  The Receiver determines CRC, Sum,

or -O error correction method.

COMMANDS        RECEIVERS

rz -y           Receive Zmodem Batch and overwrite an existing

File with the same name.

**     rz -y d:\path   Receive Zmodem Batch and overwrite an existing

File with the same name on directory d:\path

REGISTRATION REQUIRED for the d:\path feature.

**                     Alternatively, a Receive pathspec is specified

by using  SET DIRRX=d:\path  in  AUTOEXEC.BAT

rz -m           Receive Zmodem Batch using MobyTurbo mode for

faster transfer times. Specifying -m to the

receiver will force a DSZ transmitter to use

MobyTurbo unless transmitter is restricted -M

also requires RTS/CTS hardware handshaking.

rz -M           Receive Zmodem Batch and force the DSZ transmitter

to NOT use MobyTurbo mode.  Useful with ASCII

transfers in conjunction with RLE compression

(DSZ rz -MZ)

rz -r           Receive Zmodem Batch and overwrite an existing

File with the same name - starting at the last

byte of the existing File ( Crash Recovery )

rz -rr          Receive Zmodem Batch (as above) with the additional

feature of doing a more thorough check of both files

before continuing (Intelligent Crash Recovery)

**     rz -Z           Receive Zmodem Batch using "RLE" compression.

When used on the receiver, will force the DSZ

transmitter to use RLE unless restricted.

**     rb -y           Receive Ymodem Batch and overwrite an existing

File with the same name.

rc -y File      Receive Xmodem/CRC or Xmodem/Sum or Xmodem-1k,

overwriting "File" if it already exists.

NOTES:         ZMODEM uses 32-Bit Cyclical Redundancy Checking

YMODEM uses 16-Bit Cyclical Redundancy Checking

YMODEM-1k-G requires modem hardware error correction

XMODEM uses 16-Bit CRC and falls back CheckSum

XMODEM-1k-G requires modem hardware error correction

NOTES, contd:  both '-G' protocols, Ymodem-1k-G and Xmodem-1k-G

require that the modem do the error correction.

If one of these '-G' streaming protocols encounters

a "hit" the transfer will abort catastrophically.

This is why the '-G' streaming protocols cannot be

used successfully with non-error correcting modems.

COMMANDS        RECEIVERS  -  High Speed MNP, V29, V32, V.32bis  Modems

**     rb -gy          Receive Ymodem Batch-G  and overwrite existing

File with the same name - use with MNP or V.42

modems-

...the fastest transfer protocol available today.

rc -gy File     Receive Xmodem-1k-G  and overwrite an existing

File with the same name - use with MNP or V.42

modems-

ro -y File      Receive Xmodem-O  "Overthruster" and overwrite

an existing File.  Useful for GEnie Downloads,

and Compuserve XMODEM Downloads.

____________________________________________________________________________

COMMAND  LINE  OPTIONS

TRANSMITTER OPTIONS

ZMODEM:       sz [-MmZSwNfkt8g]  [PREFIX=p] [ONAME=n]  Filespecs

YMODEM:       sb [-fkt8g]  [PREFIX=p] [ONAME=n]  Filespecs

XMODEM:       sx [-kt8g]  Filespec

-M     disables MobyTurbo mode when given to either the transmitter

or the receiver, and overrides -m on the receiver. Option only

available with the DSZ Zmodem protocol.

-m     enables MobyTurbo mode when given to either the transmitter

or the receiver except when either transmitter or receiver is

restricted -M.  This is the fastest way to transfer .ZIP files

when using Zmodem. (Moby is a Zmodem option only and requires

RTS/CTS  handshaking (DSZ ha on rz    or    DSZ ha on sz )

-S     Interrogates the receiving program for its serial number and

when obtained, stores this information in the performance log.

-Z      enables ZMODEM 12-Bit Lempel-Ziv compression of File contents.

...this is the fastest way to transmit ASCII Files, and is

much faster than MNP or V.42bis compression. (DSZ Zmodem only)

-wN     Configure a Zmodem "window" of N bytes, over-riding a ZMODEM

z pw_ numeric parameter if also present.  The default window

is 3072 Bytes for the -w option.

-f      enables (Batch) send Files to specified  d:\path  of receiver.

-k      enables 1 Kb XMODEM or YMODEM Blocks or Zmodem initial packet.

RECEIVER OPTIONS

ZMODEM:       rz [-mMr+npyt8g]  [d:\pathspec]  [Filespecs]

YMODEM:       rb [-go+npyt8g]  [d:\pathspec]  [Filespecs]

XMODEM:       rc [-gyt8g]  Filespec

YMODEM/Overthruster:     ro [-yt8g]  Filespec

-m      enables MobyTurbo mode enhanced throughput. Use w/Zmodem only.

-M      disables MobyTurbo mode. Use w/Zmodem only.

-r      enables automatic Crash Recovery for Zmodem File transfers.

-rr     enables Intelligent Crash Recovery for DSZ Zmodem transfers.

-g      enables full-flowing XMODEM or, YMODEM-1K-G File transfers.

-o      enables "OverThruster" mode for YMODEM-1K-O File transfers.

CONSOLE TERMINAL OPTION for TRANSMITTERS and RECEIVERS

-yt8g           This option will return the Sender or Receiver

to terminal mode after a File Transfer. Add this

option to Transmitter/Receiver command line or,

**                     Use  SET DSZOPT=yt8g  in AUTOEXEC.BAT to make

all transfer commands return to the terminal mode

after Receiving or Sending File(s).

**                     Use  SET DSZOPT=rt8g  in AUTOEXEC.BAT to make

all transfer commands return to the terminal mode

after Receiving or Sending File(s) with Crash

Recovery enabled.

Zmodem Terminal mode Receiver

RT.BAT  = DSZ port 1 speed 2400 d pB8192 t -my8g

enter RT to go on-line and ALT-X to go off-line (the DOS prompt).

Zmodem-Z Console Transceiver

SZ.BAT  = DSZ port 1 speed 2400 D pB8192 sz -mSZty8g %1 %2 %3

enter SZ [Filename]s] at the DOS prompt and use CTRL-Break to abort.

Substitute correct port and speed as necessary.

after sending Files  the Transmitter  becomes a Console Receiver.

enter ALT-X to go off-line (the DOS prompt) and RT to go on-line.

FILE HANDLING OPTIONS

-p      transfer File if and only if it does not exist on d:\path .

-r      receiver:  recover data to an existing File

-r      sender:  command receiver to recover data to existing File

-+      receiver:  append data to an existing File

-+      sender:  command receiver to append data to an existing File

sz -+ ONAME=MASTER.LOG *.LOG   transmits all .LOG Files from

the sender's directory and appends them to File  MASTER.LOG

on the receiver's directory.

-f      transmit a full pathspec with the filespec to the receiver.

sz -f \path\Filespec    sends File to \path\ on remote Dir.

unless restricted.  Sysops should restrict to prevent files

from landing on the wrong directory.

sz -y PREFIX=\path\ *.LOG   transmits all .LOG Files from

the sender's directory and overwrites on remote's \path\ .

-n      transfer File if newer or non-existent on remote directory.

-N      transfer File if newer or longer or non-existent on remote.

-y      receiver:  overwrite data over an existing File w/same name

-y      sender:  command receiver to overwrite an existing File

-Y      transfer File if and only if it exists on remote directory.

MISCELLANEOUS OPTIONS

-a      requests the receiver to adjust Files to ASCII standards.

-b      inhibits the receiver from altering binary information.

-e      inhibits Zmodem sender from transmitting control characters.

-S      interrogates the receiver for his DSZ serial number.

-s      logs "questionable" aborts from a Zmodem receiver.

-u      deletes the Filespec(s) sent after transfer is completed.

-v      enables viewing of data being received during a transfer.

-q      disables XMODEM and YMODEM "block" status line displays.

-zT     specify T minutes behind GMT (Greenwich Mean Time) in place

of pz parameter value, if also present.

____________________________________________________________________________

CONSOLE  (TERMINAL MODE) CONFIGURATION

COMMANDS

t -y8g          DSZ "tty"  8-N-1  Terminal Mode with automatic

Zmodem Batch Downloading -  Files received are

written to the current directory and overwrite

an existing File with the same name.

t -r8g          DSZ "tty"  8-N-1  Terminal Mode with automatic

Zmodem Batch Downloading -  Files received are

written to the current directory and append to

an existing File with automatic Crash Recovery.

t -gy8g         DSZ "tty"  8-N-1  Gonzole Receiver with Zmodem

Downloading -  Existing files are overwritten,

hit F3 to begin Ymodem-1K-G Batch Downloading.

NOTES:         Use  DEVICE=ANSI.SYS  in  CONFIG.SYS  to make

a DOS ANSI Console DEVICE.

Use ANSI.COM in AUTOEXEC.BAT to make PC Mag's

ANSI-IBM Console RESIDENT.

Use DEVICE=FCONSOLE.DEV in CONFIG.SYS to make

a FANSI-ZMODEM Console Receiver.  This DEVICE

provides a scroll-back screen buffer that can

be saved to disk, enhanced keyboard and video

performance, and full ANSI X3.64 emulation.

CONTROLS

Fkey-1       -  exits to DOS (not Shell), with

no loss of CARRIER.

Fkey-3       -  begin to receive Ymodem Batch;

Zmodem receive is automatic.

NOTE:          Use  DSZ t -gy8g  to enable YMODEM-1K-G  auto

downloading via Fkey-3.

ALT-X       -  exits to DOS (not Shell), with

no loss of CARRIER.

ALT-N       -  aborts DSZ and returns to DOS,

but does not drop the CARRIER.

ALT-P       -  toggles 8th bit parity off/on,

note 8g is required for ANSI.

ALT-H       -  toggles local echo on, autoLF,

and echo/LF off (default). Use

this for PC to PC connections.

ALT-F       -  cancels a File being received;

a Batch Transfer continues with

the next file in sequence.

ALT-B       -  cancels a Batch transfer being

received, returns to terminal.

CTRL-S       -  (XOFF) suspends receiving data

CTRL-Q       -  (XON)  resumes  receiving data

CTRL-X       -  (CAN)  cancels  receiving data

CTRL-Break   -  cancels any File Transfer then

returns to terminal mode.  Use

to unlock the local console if

an XOFF (CTRL-S) is received.

SHIFT-PtScr   -  sends screen display to printer

CTRL-PtScr   -  trace console display on printer

EXAMPLES:

Zmodem-T Console Receiver

RT.BAT  = DSZ port 1 speed 2400 ha on d pB8192 t -myZ8g

Zmodem-R Console Receiver

RZ.BAT  = DSZ port 1 speed 2400 ha on d pB8192 t -rmyZ8g

Zmodem-Z Console Transceiver

SZ.BAT  = DSZ port 1 speed 2400 ha on D pB8192 sz -mSZty8g %1 %2 %3

Zmodem-W Console Transceiver

SW.BAT  = DSZ port 1 speed 2400 ha on D pB8192 sz -mZwty8g %1 %2 %3

CONSOLE  TERMINAL  OPERATION

enter RT to go on-line and ALT-X to go off-line (the DOS prompt).

enter RZ to go on-line with Zmodem CRC-32 Crash Recovery enabled.

enter SZ [Files] at the DOS prompt and use CTRL-Break to abort.

enter SW Filespecs at the DOS prompt to transmit "window" Zmodem.

Upon completion of a transfer the console is returned to terminal

mode, acting effectively as a "transceiver".  Loading the program

from a virtual (RAM) disk, the transition from DOS console to TTY

is so fast that it feels as if "RT" is a DOS "RTTY" mode.

To dial enter ATDT1234567 <cr>. To hang up enter three "+++" then

wait for your modem's "OK" to appear and type  ATH <cr>.

There is no software dialer inside DSZ.  With hardware dialing it

really sings.  Software dialing is available from many integrated

communications packages:  the DSZ "console transceiver" is closer

to the DOS console and the Modem device than integrated packages.

Rather like a Formula II car with no fenders or bumpers but with

a very powerful Zmodem engine behind you.

A DSZ console transceiver operator should be aware that his modem

responds to typed commands beginning with the prefix "AT" and his

console (display / keyboard) responds to "escape sequences", thus

"emulating" an American National Standards Institute terminal.

With FANSI CONSOLE installed, RT delivers efficient displays and

a screen recall buffer.  With CED installed, operations from the

DOS prompt are facilitated.  RT.BAT and associated receivers and

transmitters can be fully integrated with DOS by using "CED syn".

To capture data enter  RT >capture.dat   and punch  ALT-X  after

your modem RXD light stops glowing ( no display during capture ).

enter  RT >>capture.dat  to append data to an existing File.

CONSOLE STATUS DISPLAY

enter  DSZ d pv2 p z p t  for a detailed console display.

DSZ (TurboC) Copyright 01-08-89 by Omen Technology INC

"The High Reliability Software"

B =   1024   O =      0   Q =      1   S =     15   W =      0   a =   6000

d =      1   v =      2   z =      0

b =      0   B =      2   C =     11   e =      0   l =      0   L =      0

n =      0   p =     50   P =      1   r =      0   s =     15   t =    100

T =    600   w =      0   W =   1400   Z =      0

COM1 2400 bps  Carrier Detect DISABLED  Handshake Off S/N 12345

DSZOPT = 'Zyt8g' Command line = 'd pv2 p z p t '

Talking 8g. Type F1 to exit.

STATUS:         the NUMERIC PARAMETERS and ZMODEM PARAMETERS

are listed by  letter = value.

the pv2 parameter enables detail during xfers,

including status of Zmodem-W "window" pacing.

the port  COM1:  device was opened (default).

the "d" parameter used above has DISABLED the

check for DCD (pin 8) Carrier Detect.

the hardware Handshake (RTS/CTS) is disabled,

XON/XOFF handshaking is in effect for Zmodem

and Ymodem Batch transfers by default.

environment options DSZOPT=Zyt8g are present.

DSZ Terminal mode was invoked with  -y option

from the environment enabling File over-write.

DSZ Terminal mode was invoked with -8g option

from the environment enabling high ASCII (the

default "parity" of  8n  "none" is equivalent

to a high-ASCII display filter).

CONSOLE INDICATORS & ERROR CODES

2048 ZMODEM CRC-32  XOFF

indicates that the sender has received an XOFF

from the receiver or a network medium through

which he is transmitting.

Sector 280 35k YMODEM-1k-g  FLOW

indicates that CTS (pin 5) is low, suspending

transmission to the modem or device until the

line goes high. A NORMAL OCCURRENCE when used

in conjunction with high speed error-correcting

modems!

Output Restraint RELEASED

indicates that the sender or receiver has not

received an XON or CTS (8) high in 15 seconds.

NOTES:         Use the NUMERIC PARAMETERS pS15 and z ps15 to

change the default 15 second timeout for XOFF.

Use the NUMERIC PARAMETERS pS15 and z ps15 to

change the default 15 second timeout for FLOW.

ERROR RECOVERY

indicates that sender and receiver are trying

to continue the File transfer after an error.

ZMODEM Crash Recovery at

indicates that the "-r" option was active and

the receiver is recovering data in a Filespec.

Not allowed to overwrite

indicates that the "-y" option was not active

and the receiver has attempted to over-write

an existing Filespec.

File skipped by receiver request

indicates that the "-y" option was not active

for the receiver and the sender has attempted

to transfer an existing Filespec.

No such file or directory

indicates that the Path/File specified on the

command line was nonexistent.

Unknown Machine Type XX

indicates the pa6000 timing parameter may need

to be manually set for your computer. (RARELY)

Garbage Count Exceeded

indicates the z pW1400 timing parameter should

be manually set for this transmission medium.

Serial Input Error: Line Status Register XX

indicates a general communications failure, as:

01      Receive buffer full - remote not responding to

XOFF software or RTS hardware flow control.

02      Data overrun - receiving errors were caused by

interrupt latency effects.  Upgrade to a 16550AFN

UART and/or eliminate TSRs and DEVICEs.  This is

the most common type of DSZ error.  The problem

is definitely on the receiving end.

08      Framing error - indicates Bps/Parity mismatch

or very high line noise.

10      Break interrupt - also a Bps/Parity mismatch

or very high line noise.

18      Framing error &/or Break interrupt - defective

UART in the serial I/O port.

No Carrier Detect

indicates that the ENABLED check for a CARRIER

on DCD (pin 8) has found NO CARRIER present.

DSZ will exit abruptly.

DSZ FINISHED! exit code = 0

indicates the errorlevel returned to DOS after

exit from Terminal Mode or completion of File

transfer.  Errorlevel = 1 for abnormal exit.

____________________________________________________________________________

TRANSMITTER  CONFIGURATIONS  FOR  TELENET

Send Zmodem Batch-W  Telenet 2400 (CONNECT 2400)

SW.BAT  = DSZ port 1 speed 2400 pB8192 sz -Zw4096 %1 %2 %3

use this command when you are connected 2400

to your local PAD and a Host is CONNECT 2400

from the 2400 out-dial PAD.

Send Zmodem Batch-W  Telenet 2400 (CONNECT 1200)

S3.BAT  = DSZ port 1 speed 2400 est 0 1200 pB8192 sz -Zw3072 %1 %2 %3

use this command when you are connected 2400

to your local PAD and a Host is CONNECT 1200

from the (2400 or 1200) out-dial PAD.

Send Zmodem Batch-W  Telenet 1200 (CONNECT 1200)

S2.BAT  = DSZ port 1 speed 1200 pB8192 sz -Zw2048 %1 %2 %3

use this command when you are connected 1200

to your local PAD and a Host is CONNECT 1200

from the 1200 out-dial PAD.

NOTES:           Larger Zwindows increase transfer performance but

may cause excessive XOFF delays from the network.

Downloading is more efficient than uploading and

Host systems can usually transmit without "-w".

TELENET  CONFIGURATION  PROCEDURES

PROCEDURE:           to configure Telenet for ZMODEM CRC-32 Batch

1)      ATDT7780825 <cr>        dial local Telenet PAD

2)      @D <cr>                 link 8-N-1 Telenet PAD

3)      D3 <cr>                 TERMINAL= buffered 12:1

4)      C D/TXHOU/24,id,pw      request remote PAD link

5)      ATZ <cr>                clear   out-dial modem

6)      @ <cr>                  request Telenet command mode

7)      SET? 5:1 <cr>           enable  XOFF from PAD to you

8)      RST? 5:1 <cr>           enable  XOFF from PAD to host

9)      CONT <cr>               return  to out-dial modem

10)      AT*E1 <cr>              enable  auto-MNP seeking

11)      ATDT9611604 <cr>        execute auto-dial of number

NOTES:           ZMODEM "escapes" network commands and can compress

data before sending in a continuous flow.  Packets

adjust in size depending upon Bps and error rates.

The receiver checks each packet with a 32-bit CRC,

and requests retransmission of data from an exact

point should an error occur.

When an error does occur, a receiver discards the

data flowing in until the requested frame start is

found.  This "garbage count" is set to 1400 bytes

by default and can be made larger if required.

When sending, the network will accept a continuous

stream of data until an artificial "window" fills,

at which time it sends an XOFF (CTRL-S) to stop or

slow down (pace) the transmitter.

The frequency of XOFF delays is a function of the

network loading and the size of the real "window".

Zmodem-W File transfers are free-flowing however,

the receiver must return an acknowledgment after

every  window.  When the Zmodem window is equal

to the real network window, fewer  XOFF delays

and over-run errors are experienced.

The idea behind ZMODEM-W is for the receiver and

transmitter to pace themselves rather than having

the network set the pace by causing XOFF delays.

The Zmodem "-w" option configures a File transfer

for a 3 Kb "window" (by default) and can be reset

up  -w4096  or down  -w2048  for precision pacing.

A "window" ie: -w4096  usually indicates that free

flowing (no -w option) is possible.

PROCEDURE:           to configure Telenet for YMODEM/-1K/CRC/-G Batch

1)      ATDT7780825 <cr>        dial local Telenet PAD

2)      @D <cr>                 link 8-N-1 Telenet PAD

3)      D3 <cr>                 TERMINAL= buffered 12:1

4)      C D/TXHOU/24,id,pw      request remote PAD link

5)      ATZ <cr>                clear   out-dial modem

6)      @ <cr>                  request Telenet command mode

7)      SET? 5:1 <cr>           enable  XOFF from PAD to you

8)      RST? 5:1 <cr>           enable  XOFF from PAD to host

9)      SET? 0:33,35:10,64:4    increase artificial window

10)      CONT <cr>               return  to out-dial modem

11)      AT*E1 <cr>              enable  auto-MNP seeking

12)      ATDT9611604 <cr>        execute auto-dial of number

NOTES:           YMODEM sends binary data in 128 byte or 1K blocks,

and uses a Filespec Header for  Batch  capability.

The YMODEM receiver performs a 16-bit CRC test for

each data block, returning an ACK or NAK code to a

sender after each block is received and tested.

When  -w4096  is possible there is  YMODEM-1K-G to

consider.  This is a balls-out protocol, no checks,

designed for MNP type modems.  Telenet features MNP

on all 2400 Bps Lines.  Even if your modem is not

MNP, enabling  the  outdial  modem's  MNP  provides

hardware error correction whenever a Host uses MNP.

Achieving a RELIABLE connect with a Host will often

enable his high-speed "-G" protocols.  But even so,

just firing up a  YMODEM-G Receive  in response to

a YMODEM-1K Sender causes him to go into "-G" mode.

From a  DSZ t -g  Console Receiver, just punch F3.

Without error-correction on one or both (PAD) ends,

using YMODEM-G can be risky.  To be sure, one line

hit or extraneous character in the data stream will

cause a transfer to abort ( often spectacularly ).

Using  YMODEM-G  pushes the ability of the network

to process continuous data at high speeds. Setting

SET 0:33,35:10  (equivalent to SET 4:10)  commands

the network to accumulate data into larger bundles

before sending them on. This reduces the frequency

of XOFF flow control delays when you upload... but

there are unpleasant side-effects on your console.

Parameter 0:33,35:2 can be SET from 35:1 to 35:80

Parameter 0:33,64:0 can be SET from 64:0 to 64:8

Parameter 64:4 is called "advance ACK" and defines

an "artificial window".  Changing the default 64:0

to increase the artificial window size may improve

efficiency... but there is some risk of data loss.

Tweaking the Telenet default  SET?  0:33,35:2,64:0

affects Zmodem-W and Ymodem-G transfer performance.

Note that any SET? commands can always be modified

after the out-dial modem has established a CONNECT.

CAUTION:           Note that the RST? command should not be attempted

after the out-dial modem has established a CONNECT.

PROCEDURE:           to configure Telenet for XMODEM/-1K/CRC/Sum/-G

1)      ATDT7780825 <cr>        dial local Telenet PAD

2)      @D <cr>                 link 8-N-1 Telenet PAD

3)      D1 <cr>                 TERMINAL= video display

4)      C D/TXHOU/24,id,pw      request remote PAD link

5)      ATZ <cr>                clear   out-dial modem

6)      @ <cr>                  request Telenet command mode

7)      SET? 1:0,7:8 <cr>       change  <cr>@<cr> to BREAK

8)      CONT <cr>               return  to out-dial modem

9)      AT*E1 <cr>              enable  auto-MNP seeking

10)      ATDT9611604 <cr>        execute auto-dial of number

NOTES:           XMODEM sends pure binary data in 128 byte or 1 Kb

packets.  The XMODEM receiver performs a checksum

or 16-bit CRC test for each data block, returning

an ACK or NAK code to the waiting sender.

Because the network medium causes a delay between

sender and receiver, the turnaround time required

to process an ACK or NAK is considerable.  XMODEM

with 1 Kb packets (often called Ymodem/CRC), uses

7 less ACK/NAK delays for every 1,024 bytes sent.

As in YMODEM-G, the XMODEM-G receiver requests an

XMODEM/-1K sender to transmit free-flow.  However

unlike YMODEM-G, XMODEM does not support XON/XOFF

and can garble data by blowing it into Telenet.

CAUTION:           Because the network command <cr>@<cr> in any data

stream would cause an escape to net command mode,

change the <cr>@<cr> to a break signal to prevent

accidental abort of an XMODEM File transfer.

Note further that unless  CTRL-Break "send BREAK"

functions on your console, using SET 1:0,7:8 will

prevent subsequent access to net command mode!

PROCEDURE:           to configure out-dial Racal-Vadic Modem for MNP

1)      ATZ <cr>                clear   out-dial modem

2)      CTRL-E <cr>             request Racal-Vadic mode

3)      O <cr>                  request Option Groups menu

4)      2 <cr>                  request Option Group 2 menu

5)      19 <cr>                 request Option 19 menu

6)      1 <cr>                  enable  auto-MNP seeking

7)      0 <cr>                  request Option Group 2 menu

8)      0 <cr>                  request Option Groups menu

9)      4 <cr>                  execute changed options

10)      D9611604 <cr>           execute auto-dial of number

11)      R9 <cr>                 execute auto-redial of number

NOTES:           Telenet's  Racal-Vadic mode presents menus in place

of "AT" commands. They are useful for exploring the

capabilities of out-dial hardware and for using the

built-in auto-dialer.

The R-V auto-dialer is faster than the "Hayes" ATDT

dialer and provides more information about the call.

The R-V hardware redialer will make up to 9 redials.

MNP seeking remains enabled until a CONNECT or ATZ.

Whether using the above procedure for "Racal-Vadic"

or simply using  AT*E1  from the "Hayes" mode, note

that MNP must be re-enabled after completing a call.

Note some 1200 Bps out-dial modems do not have MNP.

PROCEDURE:           to disconnect from TeleNet and turn off all timers

1)      I <cr>                  request Hayes mode

2)      ATZ <cr>                clear   out-dial modem

3)      @ <cr>                  request Telenet command mode

4)      D <cr>                  request remote PAD unlink

5)      +++                     request modem command mode

6)      ATH <cr>                disable carrier

____________________________________________________________________________

EXAMPLES of DSZ USAGE with CCITT V.22bis STANDARD 2400 BPS MODEMS

Zmodem Console Receiver

RT.BAT  = DSZ port 1 speed 2400 d pB8192 t -8g

Zmodem-Z Console Transmitter

ST.BAT  = DSZ port 1 speed 2400 sz -Zt8g %1 %2 %3

Standard 2400 Bps CCITT V.22bis  Modems can achieve 228 +/- 5 c.p.s.

throughput on ZIP Files using the DSZ Zmodem Driver.

TRANSMITTERS    (typical batch files for use alone or as external

drivers with another communications program)

Send Zmodem Batch CRC-32/RLE

SZ.BAT  = DSZ port 1 speed 2400 pB8192 sz -Z %1 %2 %3

Send Zmodem Batch Mobyturbo

SZ.BAT  = DSZ port 1 speed 2400 ha on pB8192 sz -m %1 %2 %3

Send Ymodem Batch/CRC-16

SB.BAT  = DSZ port 1 speed 2400 pB8192 sb -k %1 %2 %3

Send Xmodem-1k/CRC/CheckSum

SY.BAT  = DSZ port 1 speed 2400 pB8192 sx -k %1

Send Xmodem/CRC/Sum

SX.BAT  = DSZ port 1 speed 2400 pB8192 sx %1

RECEIVERS

Receive Zmodem Batch CRC-32

RZ.BAT  = DSZ port 1 speed 2400 pB8192 rz

Receive Zmodem Batch Mobyturbo

RZ.BAT  = DSZ port 1 speed 2400 ha on pB8192 rz -m

Receive Ymodem Batch/CRC-16

RB.BAT  = DSZ port 1 speed 2400 pB8192 rb

Receive Xmodem/-1k/CRC/Sum

RY.BAT  = DSZ port 1 speed 2400 pB8192 rc %1

NOTES:         Use  SET DSZOPT=y  in AUTOEXEC.BAT to enable

overwriting existing Files when received.

Use  SET DSZOPT=r  in AUTOEXEC.BAT to enable

Crash Recovery of existing Files.

Use  SET DSZOPT=yt8g  in AUTOEXEC.BAT to make

all of the above commands return to "tty" mode

after Receiving or Sending File(s).

Use  SET DSZOPT=rt8g  in AUTOEXEC.BAT to make

all of the above commands return to "tty" mode

with Crash Recovery enabled.

____________________________________________________________________________

EXAMPLES of DSZ USAGE with 2400 bps V.22bis MNP/V.42/42bis MODEMS

NOTE: Port speed must always be higher than modem speed with MNP or V.42

Zmodem Console Receiver

RT.BAT  = DSZ port 1 speed 9600 ha on est 0 2800 d t -mg8g

Zmodem-Z Console Transmitter

ST.BAT  = DSZ port 1 speed 9600 ha on est 0 2800 sz -mZktg8g %1 %2 %3

Ymodem-G Console Transmitter

SB.BAT  = DSZ port 1 speed 9600 ha on est 0 2800 sb -ktg8g %1 %2 %3

The MultiTech MT224EH is a CCITT V.22bis MNP  level  5  modem,

capable of 280 +/- 5 c.p.s. throughput for ZIP Files using  DSZ.

The 224E should be configured to disable "compression" for ZIP

File transfers; use  AT&E14&W0  to do this.  Note that RTS/CTS

handshaking should be ENABLED -- and that XON/XOFF handshaking

is DISABLED (to "flow through") for proper Modem operation.

This Modem features "speed conversion".  The serial port UART

is "locked" at 9,600 Bps and the 224E adjusts  automatically

to a 1200 or 2400 Bps Host (with or without MNP).

TRANSMITTERS for use with the MultiTech & similar 2400 bps modems:

Send Zmodem Batch CRC-32/RLE/MOBYTURBO

SZ.BAT  = DSZ port 1 speed 9600 ha on est 0 2800 sz -mZ %1 %2 %3

Send Ymodem Batch/CRC-16/-G

SB.BAT  = DSZ port 1 speed 9600 ha on est 0 2800 sb -k %1 %2 %3

Send Xmodem-1k/CRC/Sum/-G

SY.BAT  = DSZ port 1 speed 9600 ha on sx -k %1

Send Xmodem/CRC/Sum/-O

SX.BAT  = DSZ port 1 speed 9600 ha on sx %1

RECEIVERS for use with the MultiTech and similar 2400 bps MNP modems:

Receive Zmodem Batch CRC-32

RZ.BAT  = DSZ port 1 speed 9600 ha on est 0 2800 rz -m

Receive Ymodem Batch/CRC-16

RB.BAT  = DSZ port 1 speed 9600 ha on est 0 2400 rb

Receive Ymodem Batch-G

RGB.BAT = DSZ port 1 speed 9600 ha on est 0 2800 rb -g

Receive Xmodem/-1k/CRC/Sum

RY.BAT  = DSZ port 1 speed 9600 ha on rc %1

Receive Xmodem/-1k/-G

RG.BAT  = DSZ port 1 speed 9600 ha on rc -g %1

Receive Xmodem/-1k/-O

RO.BAT  = DSZ port 1 speed 9600 ha on pO60 ro %1

NOTES:         Use  SET DSZOPT=y  in AUTOEXEC.BAT to enable

overwriting existing Files when received.

Use  SET DSZOPT=r  in AUTOEXEC.BAT to enable

Crash Recovery of existing Files.

Use  SET DSZOPT=ytg8g  in AUTOEXEC.BAT to make

all of the above commands return to "tty" mode

after Receiving or Sending File(s).

Use  SET DSZOPT=rtg8g  in AUTOEXEC.BAT to make

all of the above commands return to "tty" mode

after Receiving or Sending File(s) with Crash

Recovery enabled.

EXAMPLES of DSZ USAGE with CCITT V.32 MNP/V.42/V.42bis MODEMS

Zmodem Console Receiver

RT.BAT  = DSZ port 1 speed 19200 ha on est 0 11000 d t -g8g

Zmodem-Z Console Transmitter

ST.BAT  = DSZ port 1 speed 19200 ha on est 0 11000 sz -Zktg8g %1 %2 %3

Ymodem-G Console Transmitter

SB.BAT  = DSZ port 1 speed 19200 ha on est 0 11000 sb -ktg8g %1 %2 %3

The  USR 14400 HST,  the USR Dual Standard  and the new CCITT

V.32bis modems are capable of 1728 +/- 20 cps throughput using

DSZ to transfer ZIP files

The USR modems should be configured to disable "compression"

for ZIP File transfers.  Use  AT&K3&W  to do this.  Note that

RTS/CTS handshaking MUST be ENABLED - and that XON/XOFF handshaking

MUST be DISABLED for proper Modem operation.

This Modem features "speed conversion".  The serial port UART

is "locked" at 38,400 Bps and the modem adjusts  automatically

to a 1200, 2400, 9600 or 14400 bps Host (with or without MNP).

Owners of the USR HST, Dual Standard, V.32 and V.32bis modems must

replace their  8250  or  16450 UART chip with a  NS16550AFN  UART

to achieve high performance. (about $20 at larger electronics parts

stores - you won't find it at Radio Shack -  get the part number

ending with "AFN" for best performance).

TRANSMITTERS for use with 14400 HST's and V.32bis MODEMS:

Send Zmodem Batch CRC-32/RLE/MOBYTURBO

SZ.BAT  = DSZ port 1 speed 38400 ha on est 0 17000 sz -mZ %1 %2 %3

Send Ymodem Batch/CRC-16/-G

SB.BAT  = DSZ port 1 speed 38400 ha on est 0 17000 sb -k %1 %2 %3

Send Xmodem-1k/CRC/Sum/-G

SY.BAT  = DSZ port 1 speed 38400 ha on sx -k %1

Send Xmodem/CRC/Sum/-O

SX.BAT  = DSZ port 1 speed 38400 ha on sx %1

RECEIVERS for use with 14400 HST's and V.32bis MODEMS:

Receive Zmodem Batch CRC-32

RZ.BAT  = DSZ port 1 speed 38400 ha on est 0 17000 rz -m

Receive Ymodem Batch/CRC-16

RB.BAT  = DSZ port 1 speed 38400 ha on est 0 14400 rb

Receive Ymodem Batch-G

RGB.BAT = DSZ port 1 speed 38400 ha on est 0 17000 rb -g

Receive Xmodem/-1k/CRC/Sum

RY.BAT  = DSZ port 1 speed 38400 ha on rc %1

Receive Xmodem/-1k/-G

RG.BAT  = DSZ port 1 speed 38400 ha on rc -g %1

Receive Xmodem/-1k/-O

RO.BAT  = DSZ port 1 speed 38400 ha on pO60 ro %1

NOTES:           Use  SET DSZOPT=y  in AUTOEXEC.BAT to enable

overwriting existing Files when received.

Use  SET DSZOPT=r  in AUTOEXEC.BAT to enable

Crash Recovery of existing Files.

Use  SET DSZOPT=ytg8g  in AUTOEXEC.BAT to make

all of the above commands return to "tty" mode

after Receiving or Sending File(s).

Use  SET DSZOPT=rtg8g  in AUTOEXEC.BAT to make

all of the above commands return to "tty" mode

after Receiving or Sending File(s) with Crash

Recovery enabled.

____________________________________________________________________________

EXAMPLES of DSZ USAGE with EIA RS232-C  DIRECT CONNECTIONS

Zmodem Console Receiver

RT.BAT  = DSZ port 1 speed 115200 ha on d t -yg8g

Zmodem-Z Console Transmitter

ST.BAT  = DSZ port 1 speed 115200 ha on d sz -mZktyg8g %1 %2 %3

Ymodem-G Console Transmitter

SB.BAT  = DSZ port 1 speed 115200 ha on d sb -ktyg8g %1 %2 %3

Direct connection between computers creates an RS232 "network"

by which two operators can communicate at very high speed.

PC -to- PC  File transfers are useful when two computers have

dissimilar removable media or, when they are far apart.

By using an appropriate Null Modem Cable between serial ports,

two high speed machines can transfer Files at up to 115,200 Bps.

IBM PC SERIAL PORT  UART SPECIFICATIONS

8250       The PC/XT 8250 Universal Asynchronous Receiver

Transmitter (UART) provides nine of the RS232C

lines from a  DB-25p  (25-pin, male) connector.

This is the standard PC serial port controller,

and is frequently  limited to  speeds of  less

than 9600 Bps. Usually adequate for 2400 Bps.

16450       The "AT" 16450 Universal Asynchronous Receiver

Transmitter (UART) provides nine of the RS232C

lines from a  DB-9p (9-pin, male) connector or,

from a  DB-25p (25-pin, male) connector. It is

capable of operating at higher speeds.

16550       The PS/2 16550 Universal Asynchronous Receiver

Transmitter (UART) provides nine of the RS232C

lines from a DB-9p (9-pin, male) connector and

features a 16-byte  FIFO  port  buffer.  It is

capable  of  operating  at  higher speeds than

either the 8250 or 16450 UART.

NOTE:            UART 40-pin DIP devices are pin-compatible and

can be upgraded. The NS16550AFN is recommended

for any MS-DOS computer running the DSZ Driver.

IBM PC SERIAL PORT  PLUG SPECIFICATIONS

The Serial Port is configured as RS232-C Data

Terminal Equipment (DTE) :

pin    DTE direction     signal

( 1)    CHS    nul    chassis ground

( 2)    TXD    out    transmit data

( 3)    RXD    inp    receive data

( 4)    RTS    out    request to send

( 5)    CTS    inp    clear to send

( 6)    DSR    inp    data set ready

( 7)    GND    nul    signal ground

( 8)    DCD    inp    data carrier detect

(9/20)   DTR    out    data terminal ready

NOTE:            The Data Terminal Ready DTR signal is on pin 9

of DB-9p Male connectors and pin 20 of DB-25p.

IBM PC to MODEM CABLE CONFIGURATION: STANDARD DTE-DCE

signal        pin    DTE     pin    DCE        signal

chassis ground    ( 1)    CHS    ( 1)    CHS    chassis ground

transmit data    ( 2)    TXD    ( 2)    TXD    transmit data

receive data    ( 3)    RXD    ( 3)    RXD    receive data

request to send    ( 4)    RTS    ( 4)    RTS    request to send

clear to send    ( 5)    CTS    ( 5)    CTS    clear to send

data set ready    ( 6)    DSR    ( 6)    DSR    data set ready

signal ground    ( 7)    GND    ( 7)    GND    signal ground

data carrier detect    ( 8)    DCD    ( 8)    DCD    data carrier detect

data terminal ready   (9/20)   DTR   (9/20)   DTR    data terminal ready

NOTE:            Connecting a Modem or other  Data Communications

Equipment device requires a shielded, nine-wire,

straight connect cable, with one DB25p Male plug

for the device and either a DB9s or DB25s Female

socket for the computer.

IBM PC to PC CABLE CONFIGURATION:  NULL MODEM DTE-DTE

signal        pin    DTE     pin    DTE        signal

chassis ground    ( 1)    CHS    ( 1)    CHS    chassis ground

transmit data    ( 2)    TXD    ( 3)    RXD    receive data

receive data    ( 3)    RXD    ( 2)    TXD    transmit data

request to send    ( 4)    RTS    ( 5)    CTS    clear to send

clear to send    ( 5)    CTS    ( 4)    RTS    request to send

data set ready    ( 6)    DSR   (9/20)   DTR    data terminal ready

signal ground    ( 7)    GND    ( 7)    GND    signal ground

data terminal ready   (9/20)   DTR    ( 6)    DSR    data set ready

( 8)    DCD    data carrier detect

( 6)    DSR    data set ready

data carrier detect    ( 8)    DCD

data set ready    ( 6)    DSR

NOTES:           Connecting a  Computer  or other  Data  Terminal

Equipment device requires a shielded, nine-wire,

"null modem" cable with  DB9s  or  DB25s  Female

sockets for the two computers.

Standard Practice  to make a null modem cable is

to modify straight-wired cable with an extractor/

insertion tool, switching:  2x3,  4x5,  and  6x9

(or 6x20 for DB25) on one end.  then...

Cut the DCD wire  2" from pin 8 on both ends, and

"strap" it to DSR pin 6 by soldering on both ends.

Take out the old DCD wire from the cable assembly.

...alternatively the state of Data Carrier on pin

eight is ignored by using the  "d"  parameter.

____________________________________________________________________________

EXAMPLES of DSZ CONFIGURATION for PCBOARD with 14400 HST

N   None Specified   ( always prompts )

X   Xmodem/CRC/Sum   ( 84% efficiency )

Y   Xmodem/-1K/CRC   ( 94% efficiency )

O   Xmodem/-1K/y-G   ( MNP efficiency )

B   Ymodem Batch-K   ( 94% efficiency )

G   Ymodem Batch-G   ( MNP efficiency )

Z   Zmodem Batch-Z   ( 96% efficiency )

UPLOADING

PCBOARD  FILE  PCBRX.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d z pr1 rc %3

Rem

IF errorlevel 1 GoTo ERROR

IF not exist %3 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

IF exist %3 Del %3 >Nul

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBRY.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d z pr1 rc %3

Rem

IF errorlevel 1 GoTo ERROR

IF not exist %3 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

IF exist %3 Del %3 >Nul

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBRO.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d z pr1 rc -g %3

Rem

IF errorlevel 1 GoTo ERROR

IF not exist %3 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

IF exist %3 Del %3 >Nul

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBRB.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha bo d est 0 %4 pd0 z pr1 rb -p %3

Rem

IF errorlevel 1 GoTo ERROR

IF not exist %3 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

IF exist %3 Del %3 >Nul

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBRG.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha bo d est 0 %4 pd0 z pr1 rb -gp %3

Rem

IF errorlevel 1 GoTo ERROR

IF not exist %3 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

IF exist %3 Del %3 >Nul

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBRZ.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d est 0 %4 pd0 z pr1 rz -mp %3

Rem

IF errorlevel 1 GoTo ERROR

IF not exist %3 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

IF exist %3 Del %3 >Nul

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

DOWNLOADING

PCBOARD  FILE  PCBSX.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d sx %3

Rem

IF errorlevel 1 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBSY.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d sx -k %3

Rem

IF errorlevel 1 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBSO.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d sx -k %3

Rem

IF errorlevel 1 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBSB.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha bo d est 0 %4 sb -k %3

Rem

IF errorlevel 1 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBSG.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha bo d est 0 %4 sb -k %3

Rem

IF errorlevel 1 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

PCBOARD  FILE  PCBSZ.BAT

@Echo Off

Rem

Rem     %1      COMn: Port Number n

Rem     %2      COMn: Port Baud Rate

Rem     %3      Up/Download Pathspec

Rem     %4      CONNECT (Link) Speed

Rem

IF exist c:\pcb\$DOOR.LOG Del c:\pcb\$DOOR.LOG >Nul

Rem

DSZ port %1 speed %2 ha on d est 0 %4 sz -km %3

Rem

IF errorlevel 1 GoTo ERROR

IF exist c:\pcb\PCBERR.FIL Del c:\pcb\PCBERR.FIL >Nul

GoTo END

:ERROR

Copy c:\pcb\PCBERR.OLD c:\pcb\PCBERR.FIL >Nul

:END

NOTES:           The Xmodem transmitter automatically detects

a CRC-16 or CheckSum receiver and adapts.

These receivers and transmitters drop DTR if

the carrier is lost during a transfer.

PCBOARD  FILE  PCBPROT.DAT

N,I,0,None Specified   ( always prompts ),N,N,N

X,S,128,Xmodem/CRC/Sum   ( 84% efficiency ),N,Y,N

Y,S,1024,Xmodem/-1K/CRC   ( 94% efficiency ),N,Y,N

O,S,1024,Xmodem/-1K/y-G   ( MNP efficiency ),Y,Y,N

B,D,1024,Ymodem Batch-K   ( 94% efficiency ),N,Y,N

G,D,1024,Ymodem Batch-G   ( MNP efficiency ),Y,Y,N

Z,D,1024,Zmodem Batch-Z   ( 96% efficiency ),N,Y,Y

EXAMPLES of DSZ CONFIGURATION for PRODOOR with 14400 HST

;

;       UPLOADING

;

94 B 0 R 0 Ymodem Batch-K  (94%)

DSZ ha both est 0 $C$ pB4096 pd0 z pb1 pr1 rb -p

;

94 Y 0 R 1 Xmodem/-1K/CRC  (94%)

DSZ ha rts pB4096 z pb1 pr1 rc

;

99 G 1 R 0 Ymodem Batch-G  (MNP)

DSZ ha bo est 0 $C$ pB4096 pd0 z pb1 pr1 rb -gp

;

99 O 1 R 1 Xmodem/-1K/y-G  (MNP)

DSZ ha rts pB4096 z pb1 pr1 rc -g

;

96 Z 0 R 0 Zmodem Batch-Z  (96%)

DSZ ha rts est 0 $C$ pB4096 pd0 z pb1 pr1 rz -mp

;

;       DOWNLOADING

;

94 B 0 T 20 Ymodem Batch-K  (94%)

DSZ ha bo est 0 $C$ pB4096 z pb1 sb -k $@$

;

94 Y 0 T 1 Xmodem/-1K/CRC  (94%)

DSZ ha cts pB4096 z pb1 sx -k

;

99 G 1 T 20 Ymodem Batch-G  (MNP)

DSZ ha bo est 0 $C$ pB4096 z pb1 sb -k $@$

;

99 O 1 T 1 Xmodem/-1K/y-G  (MNP)

DSZ ha cts pB4096 z pb1 sx -k

;

96 Z 0 T 20 Zmodem Batch-Z  (96%)

DSZ ha cts est 0 $C$ pB4096 z pb1 sz -kZm $@$

NOTES:           ProDoor provides a Batch Transfer interface to

the DSZ Driver and enhances  PCBoard's Message

and Conference capabilities.

ProDoor implements the "RZ" Command which will

initiate a Zmodem Batch Receive.

Because DSZ sends an  rz <cr>  to the Receiver

at the beginning of a Zmodem Upload, users can

Upload Zmodem to the ProDoor Command Prompt.

____________________________________________________________________________

EXAMPLES of DSZ CONFIGURATION for DOS CTTY with 14400 HST

RX  SX  Xmodem/CRC/Sum   ( 80% efficiency )

RY  SY  Xmodem/-1K/CRC   ( 90% efficiency )

RO  SO  Xmodem/-1K/y-G   ( 99% efficiency )

RB  SB  Ymodem Batch-K   ( 90% efficiency )

RG  SG  Ymodem Batch-G   ( 99% efficiency )

RZ  SZ  Zmodem Batch-Z   ( 94% efficiency )

It is useful for a Remote Sysop to have File Transfer capability

from the  DOS  "Shell"  CTTY  Mode.  Place these DSZ Batch Files

"on the path" of the HOST computer to achieve this.

RECEIVERS

RX.BAT  = DSZ CON port 1 speed 38400 ha on D rc -y %1

RY.BAT  = DSZ CON port 1 speed 38400 ha on D rc -y %1

RO.BAT  = DSZ CON port 1 speed 38400 ha on D rc -gy %1

RB.BAT  = DSZ CON port 1 speed 38400 ha on D rb -y

RG.BAT  = DSZ CON port 1 speed 38400 ha on D rb -gy

RZ.BAT  = DSZ CON port 1 speed 38400 ha on D rz -my

TRANSMITTERS

SX.BAT  = DSZ CON port 1 speed 38400 ha on D sx %1

SY.BAT  = DSZ CON port 1 speed 38400 ha on D sx -k %1

SO.BAT  = DSZ CON port 1 speed 38400 ha on D sx -k %1

SB.BAT  = DSZ CON port 1 speed 38400 ha on D sb -k %1 %2 %3

SG.BAT  = DSZ CON port 1 speed 38400 ha on D sb -k %1 %2 %3

SZ.BAT  = DSZ CON port 1 speed 38400 ha on D sz -mZ %1 %2 %3

NOTES:           The DSZ 'CON' Parameter redirects the display

to the  DOS CON: device  for  CTTY  operations.

Alternatively, use  >NUL  at the end of a line.

Because DSZ sends an  rz <cr>  to the Receiver

at the beginning of a Zmodem Upload, Files can

be uploaded to the DOS Prompt with SZ Filespec

from the Remote Sysop's Console Transmitter.

____________________________________________________________________________

PRO-YAM HOST  TeleGodzilla  BULLETIN BOARD

(DSZ Support)

CONNECT:         (503) 621-3746  1200, 2400, 9600, 14400 MNP 8-N-1

USRobotics 14400 bps Dual Standard

1200 Telenet D/ORPOR/12

DIRECTORY:     Enter DIRD at the Pro-Yam Host prompt to list Files

DOWNLOAD:      sz DSZ.ZIP  -  DSZ.COM and DSZ.DOC Fileset

DOWNLOAD:      sz DSZEXE.ZOO  -  DSZ.EXE Driver File

DOWNLOAD:      sz ZCOMMDOC.ZIP  -  Zmodem Communications Reference

DOWNLOAD:      sz YZMODEM.ZOO  -  Ymodem and Zmodem Protocol Reference

DOWNLOAD:      sz RZSZ.ZOO  -  Zmodem Source Code in "C" Language

DOWNLOAD:      sz *.*  -  Sends all files in the download directory.

===============================================================================

DISCLAIMER:     It is  not our  purpose in creating this document to

attempt to  diminish the work of the program author,

but rather to augment a fabulous program with our small contribution

in the way of an easier  to understand  explanation of the otherwise

complicated syntax, such that others might also enjoy this program's

full potential.   We thank  Chuck Forsberg,  for his untiring effort

in  revolutionizing  data  communications  with his  excellent   DSZ

Program and his Zmodem file transfer protocol.

===============================================================================

Prepared by:   Steve Shelton    co-Sysop of Sands ProBoard

.oOo.

Last Update 08/14/91:   Bill Garfield    co-Sysop of Sands ProBoard

with the help of the Sands ProBoard members

===============================================================================

SANDS ProBoard, Houston, TX (713) 961-1604  2400 MNP/V.42bis & 9600 HST/V.32bis

===============================================================================