PrintDM Version 1.0

by

Softronix

(c) 1991

PrintDM - License and Disclaimer Statement

Copyright (c) 1991 Softronix

All rights reserved.

PrintDM, Print Dot Matrix, is distributed as a user-supported

software product.  It is designed as a companion tool for use with

both EditPCB and EditSCH.  It replaces completely PrintPCB and

PrintSCH.  You may copy and distribute PrintDM and its accompanying

files freely, as long as the files are distributed together

unaltered.  No registration fee is requested for PrintDM.

PrintDM is distributed "as is" and without any express or implied

warranties. Because of the diversity of conditions and hardware under

which this program may be used, no warranty of fitness for a

particular purpose is offered.  The user is advised to test the

program thoroughly before relying on it.  The user assumes the entire

risk of using the program.  Any liability of Softronix is limited

exclusively to product replacement.

1.  Introduction

PrintDM is a user-supported print package and is used to print output

from either EditPCB or EditSCH.  It was developed to provide a low

cost and professional method of producing PCBs and schematics and

distributing these designs to others via floppies or BBSs.

An IBM compatible equipped with 640K and a PrintDM compatible printer

are the only minimum mandatory hardware required to run PrintDM;

however, a hard disk is strongly recommended.

PrintDM supports a wide range of laser and dot-matrix printers

enabling almost anyone with an output device to generate PCB artwork

or schematics.

PrintDM is one part of Softronix's design packages.

1.1  Getting Started

PrintDM is a stand-alone print utility used to generate PCB artwork,

drill plans, or schematics.  PrintDM distinguishes between a PCB or

schematic by its file extension.  A filename ending in ".PCB" is

interpreted as a PCB produced by EditPCB.  A filename ending in

".SCH" is interpreted as a schematic produced by EditSCH.  PrintPCB

or PrintSCH are no longer needed.  The GDHEAD.PS - PostScript support

file - is no longer necessary when printing designs.

The command-line parameters are NOT case-sensitive.  The command-

line syntax for PrintDM is shown below:

PrintDM [ options ] file

-O device

-X number

-Y number

-M mode

-P port

-S scale

-T scale

-R scale

-L number

-F flip

1.1.1 -O device option

This option defines the output device type.  Currently, PrintDM

supported devices are:

FX - Epson-compatible 9-pin dot matrix printers

LQ - Epson-compatible 24-pin dot matrix printers

LJ - HP LaserJet

PS - Postscript Printers

1.1.2  -X and -Y option

This option moves the print origin from 0,0 to a user defined

location.  The units are in mils.  One mil equals one one-thousandth

(1/1000) of an inch.

1.1.3  -M option

This option sets the print resolution.  Currently, PrintDM supports

three resolution modes: Lo, Med, and Hi.  Lo resolution is useful for

quick check plots. Med and Hi resolution are used to generate artwork

suitable for PCB fabrication.  If the PS device is selected, all

plots are generated with a resolution setting of Hi.  If no option is

given, artwork is generated in low resolution mode.

1.1.4  -P option

This option defines the output port location.  Currently PrintDM

supports the following port locations: LPT1, LPT2, COM1, COM2, and

FILE.  PrintDM does not change serial port options (i.e. baud rate).

You must have these options set prior to printing.  If this option is

not used, output will go to LPT1.

If the FILE option is chosen, PrintDM will generate output to a file

called PLOTFILE instead of sending the output directly to the output

device.  This option is useful when you wish to print the artwork

elsewhere or later.

1.1.5  -S option

This option is used to scale artwork.  A floating point number is

legal with this option.

1.1.6  -T option

This option is used to scale trace widths.  A floating point number

is legal with this option.

1.1.7  -R option

This option is used to scale pin and hole diameters.  A floating

point number is legal with this option.

1.1.8  -L option

This option is used to select which layers are printed.  Numbers from

0 to 7 are listed together without spaces to select layers.  If this

option is not used or schematic is printed, all layers are printed.

1.1.9  -F option

This option flips the output of a PCB so transparences can be made

easily.  The Y values of all objects is inverted when this option is

used.

1.2  PrintDM Command-line Examples

An example PCB is included with EditPCB Intro.  Below are some

command-line examples for plotting the file.

PrintDM -O LQ -p FILE example.pcb

This line generates PCB artwork for the example.pcb.  The output is

for an Epson 24-pin printer.  The output is 1X the actual size with

LO resolution.

PrintDM -Ofx -S 2.0 -m HI -L012567 example.pcb

This line generates PCB artwork for the example.pcb.  The output is

for an Epson 9-pin printer and is 2X the actual size. The plot

resolution is HI.  Layers 0,1,2,5,6 and 7 are printed.

1.3  PCB/SCH Readme File Format

When distributing PCBs and schematics created using EditPCB or

EditSCH Softronix recommends a that PCBREAD.ME or SCHREAD.ME file

accompany the design file.  The READ.ME file format is shown below:

Date:             PCB/SCH date of creation/release

Name:             Name of the PCB/SCH design file

Type:             Single or Double-Sided, Silkscreen, solder

mask, etc.

File Origin:      Location of lower left corner of PCB or SCH

Created by:       Version of EditPCB or EditSCH used to create

the PCB.

Description:      A detailed description of the design.

Ex. Command-Line: Example command-lines for producing the

Output.

Comments:         Any additional comments, hints, or

restrictions.

Acknowledgements

PostScript is a registered trademark of Adobe Systems, Inc.

IBM is a registered trademark of International Business Machines

Corporation.

Printer names are trademarks of their respective manufacturers.