DC3D TO GBEAM .B3E CONVERSION PROGAM

SYS(25)=0                                               'SOUND OFF

SYS(26)=1                           'MANIPULATE CURRENT LAYER ONLY

TN=0                             'ADVANCE STORAGE LOCATION / ARRAY

PRECISION 0                        'SET NUMBERS AS ie 12 NOT 12.00

DIM A(600),B(600),C(600),E(600),N(600)

DO WHILE AS<>49 OR AS<>50

INPUT "GET ALL ENTITIES = 1 or SELECT A ENTITY = 2 : ",AS$

AS=ASC(AS$)

IF AS=49 THEN GOSUB ALL

IF AS=50 THEN EXIT DO

LOOP

FOR D=1 TO SYS(9)                              'CHOOSE ENTITY UPTO TOTAL

>GETENTITY                  'DC3D WILL PROMPT YOU TO SNAP TO A ENTITY

GETXYZ G,H,I  'GET XYZ COORDS OF THE CURSOR THEREFORE ENTITY LOCATION

FOR CC=1 TO SYS(1)   'PUT ALL XYZ COORDS OF THIS ENTITY/NODES INTO ARRAY

POINTVAL XX YY ZZ CC

A(CC+TN)=XX

B(CC+TN)=YY

C(CC+TN)=ZZ

S=S+1            'REMEMBER TOTAL NUMBER OF POINTS IN ENTITY / NODES

NEXT CC

FOR EE=1 TO SYS(1)-1                        'PUT ELEMENT INFO INTO ARRAY

Q=Q+1                       'THIS FOR LOOP WRITES ELEMENT/NODE INFO

Z=Z+1

E(Q)=Z

N(Q)=Z+1

NEXT EE

Z=Z+1  'AVOID CONNECTION OF SEPARATE ENTITY'S/ELEMENTS IN .B3E FILE

TN=S             'KEEP POSITION OF LAST WRITE TOO LOCATION IN ARRAY

SYS(1)=0

>PXYZ G,H,I

>CHC 2               'CHANGE COLOUR OF ENTITY TO AVOID RE-SELECTION

IF D=SYS(9) THEN EXIT FOR

DO WHILE YN<>89 OR YN<>121 OR YN<>78 OR YN<>110

INPUT "GET ANOTHER ENTITY  YES = Y or NO = N : ",YN$

YN=ASC(YN$)

IF YN=110 OR YN=78 THEN EXIT DO

IF YN=89 OR YN=121 THEN EXIT DO

LOOP

IF YN=110 OR YN=78 THEN EXIT FOR

NEXT D

GOSUB PARM                               'SAVE AS / DEFINE ELEMENT TYPE

GOSUB HEADER                                      'WRITE HEADER TO FILE

GOSUB NODES                                  'WRITE NODE COORDS TO FILE

GOSUB ELES                                'CONNECT NODES WITH A ELEMENT

GOSUB MESS                            'INFORM USER PROGRAM HAS FINISHED

ALL:

FOR D=1 TO SYS(9)  'THIS DOES SAME AS ABOVE BUT AUTO SELECTS ALL ENTITYS

ENTITY D      'SEE README.DOC - PROBLEMS

FOR CC=1 TO SYS(1)

POINTVAL XX YY ZZ CC

A(CC+TN)=XX

B(CC+TN)=YY

C(CC+TN)=ZZ

S=S+1

NEXT CC

FOR EE=1 TO SYS(1)-1

Q=Q+1

Z=Z+1

E(Q)=Z

N(Q)=Z+1

NEXT EE

Z=Z+1

TN=S

SYS(1)=0

NEXT D

GOSUB PARM

GOSUB HEADER

GOSUB NODES

GOSUB ELES

GOSUB MESS

PARM:

INPUT "SAVE AS - File will be saved as C:\GBEAM\PRO2\ --- .b3e: ", NAME$

DO WHILE TY<>48 OR TY<>49

INPUT "ELEMENTS ARE OF WHAT TYPE - BEAM=1 or TRUSS=0 - : ", TY$

TY=ASC(TY$)

IF TY=48 OR TY=49 THEN EXIT DO

LOOP

FILE$="C:\GBEAM\PRO2\" + NAME$ + ".b3e"

RETURN

HEADER:

IF S<=9 OR Q<=9 THEN SP$="  "

IF S>=10 AND S<=99 OR Q>=10 AND Q<=99 THEN SP$=" "

IF S>=100 AND S<=999 OR Q>=100 AND Q<=999 THEN SP$=""

OPEN "A",1,FILE$

PRINT #1,NAME$

PRINT #1,"  "

PRINT #1,SP$,S,"         TOTAL NUMBER OF NODES                  "

PRINT #1,SP$,Q,"         TOTAL NUMBER OF ELEMENTS                 "

PRINT #1,"   0          LARGEST DIFFERENT'S BETWEEN 2 NODES      "

PRINT #1,"                                                       "

PRINT #1,"X/Y/Z COORDINATES OF NODES"

CLOSE #1

RETURN

NODES:

PRECISION 4

OPEN "A",1,FILE$

FOR CC=1 TO S

PRINT #1,A(CC),B(CC),C(CC)

NEXT CC

CLOSE #1

RETURN

ELES:

PRECISION 0

OPEN "A",1,FILE$                                   'OPEN FILE AND APPEND

A$=""

B$="      "

D$="    "

E$="       "

F$="        "

R$="0.0000E+00"

PRINT #1,"  "

PRINT #1,"TYPE   ELEMENT NODES   ROTATION ANGLE   MATERIAL   GEOMETRY"

FOR RR=1 TO Q

IF RR<=9 THEN C$="   "

IF RR>=10 AND RR<=99 THEN C$="  "

IF RR>=100 AND RR<=999 THEN C$=" "

PRINT #1,A$,CHR$(TY),B$,E(RR),C$,N(RR),D$,R$,E$,1,F$,1

NEXT RR

CLOSE #1

RETURN

'NOTE LEAVE M+G AS 1 UNTIL SPACING DEVELOPMENT THEN ADD DO / LOOP

MESS:

WINDOW 6,61

LOCATE 2,2

PRINT "All Element  TYPE - MATERIAL - GEOMETRY  will be the same"

LOCATE 3,3

PRINT "Element config can altered within GBEAM in the usual way "

LOCATE 5,2

PRINT "     ******* PROGRAM COMPLETE  Press anykey *******      "

ANYKEY

WCLOSE

END