PROGRAM:  COLUMN xx

REFERENCE

AISC Manual of Steel Construction

American Institute of Steel Construction

Seventh Edition

APPLICATION

By the use of this program the AISC column formulas can be

used to rapidly determine the size of compression member in

either of two formats: (1) as a single column by varying one of

the parameters (K, L, or r) until a safe size is found; or (2) by

the generation of a table of KL/r values versus stress from

which safe values may be selected.  Similar tables for commonly

used grades of structural steel are published in the AISC Manual.

In either case the value of the program goes beyond the rapid

solution of commonly used structural steels because the AISC

equations have been rearranged to separate the critical stress,

the factor of safety (FS), and the allowable stress.  The

program, therefore, is especially useful in applications where

critical stress is used of itself or in combination with other

factors to determine the acceptance criteria.  Futher more,

because the material properties (modulus of elasticity and yield

strength) are entered by the user, tables similar to those given

in the AISC Manual may be generated for other than the commonly

used grades of structural steel.

By the AISC Specification an allowable stress is determined

by either of two equations depending upon certain criteria

involving material properties and geometry. The equations are

given in section 1.5.1.3.1 (equation 1.5-1) and section 1.5.1.3.2

(equation 1.5-2).  The critical / allowable stress, as

calculated by the program, is identified with the appropriate

AISC equation by the words "short" or "slender".  Short is the

reference to equation 1.5-1 and slender is the reference to

equation 1.5-2.

The program may be verified simply by duplicating any

of the tables in the AISC specification.  A value of 29,000,000

should be used for the modulus of elasticity.

AUTHOR'S COMMENTS ON PROGRAM & FILE NUMBER

The initial release of this program is being made to

the ASME bullentin board system in August 1993 as file

"COLUMN00.ZIP".  The compressed file is comprised of

COLUMN00.EXE

COLUMN00.DOC

The "00" which forms the file name indicates the release

or revision and is unique to each file name.

Page 1

This program "COLUMNxx" is being used to announce the

availability of other programs written by this author. The list

of programs, appearing below, will be updated periodically

and so indicated by a revision to the ZIP and DOC file names.

If there is any interest in the program list, the ASME BBS

( 608 ) 233-3378, should be checked periodically to see if

there has been any revision in the file "COLUMNxx.ZIP".

Although the author encourages distribution of this

program to other BBS 's, he can only update the ASME BBS.

Comments on the program "COLUMN" and other programs

to be released in the future are welcome until about January 2002

at which time I expect to retire for a second time.  With the

preparation of the program "COLUMN" I shall begin to fulfill a

career long ambition to solve problems of interest to me and

write the corresponding computer programs.  Problems which

interest me are those for which handbook solutions are not

available ( "COLUMN" is the exception ) and the transformation

from theory to practice is more time consuming than most

engineering departments can afford.  But once the problem is

solved and the program written, the practical utility of the

program will be of value indefinitely.

C. Carroll Brice

August 18, 1993

PROGRAM LIST

1.    File name:  SECTION0.zip   ( 0 designates initial release)

Calculates properties of cross sectional areas formed

by rectangles, arcs, rings, sectors, circles and triangles.

The results include the composite area, the location of the

centroidal axis, the centroidal moment of inertia, the angle

angle of the principal axis, and the principal moments of

inertia.

The graphic images of the composite areas are displayed

on screen and may be printed.  The input data and results

may be saved to file for subsequent editing.

Documentation includes instructions for use and

derivation of all equations.

Availability:  Full program, disk plus documentation, $250.00

Demonstration program, file name SECTION0.ZIP,

down-load from the ASME BBS, free

2.    File name:  ( to be determined )

Calculates shear center and shear stress.

Page 2

3.    File name:  ( to be determined )

Calculates multiple spring supports reactions

for a structure with multiple loads.

SHARE-WARE

This program, "COLUMN", is share-ware.  Distribution of

this program to other persons and other BBS's is encouranged

provided that the execute file and document file are keep together.

The share-ware concept is an attempt  to provide quality

software at  low cost. Both the user and author benefit from this

plan. The user will benefit by being able to "test  drive" soft-

ware thoroughly before purchasing it.  The author  benefits by

being able  to enter the commercial software  market  while

avoiding  the  high  cost  of commercial distribution.

This concept  helps many independent authors  and small

companies that otherwise would be discouraged from developing and

promoting their ideas. It can only work  with your support. If you

obtain a user supported program from a friend and are still using

it after a few weeks, then  it is obviously worth something  to

you, and a registration fee should be forwarded to the author.

For this program the cost of registration is the price of a

postage stamp.  Return of this registration form will help

establish the interest in small engineering programs as offered

here.

Page 3

Note:  This form can also be printed from the program.

REGISTRATION FORM

Name of Program:  COLUMN               Files:  COLUMN00.EXE

COLUMN00.DOC

User Name:____________________________________________________

Address:_________________________________________________

Address:_________________________________________________

City: _______________________State:_________Zip__________

Share-Ware Price:  No charge

QUESTIONNAIRE

1.    Would enhancements to this program be of value to you

such as

(a)  Secondary members as per section 1.5.1.3.3.

Yes     or     No

(b)  Your comments    ( please describe )

2.    Describe any other problems for which you would like

to find share-ware programs on the ASME BBS.

3.    Where did you obtain this program?  Other BBS, Share-Ware

distributor, Friend, Other

Return this registration form with questionnaire to

C. Carroll Brice, PE

3 Sandy Drive

Morrisville, Pennsylvania  19067

Page 4